3GPP TSG-CT WG4 Meeting #130	C4-254123

Sophia Antipolis, France, 13th – 17th October 2025

Source:	KDDI, vivo, Samsung

Title:	Pseudo-CR on KI#1 update for adding a NOTE to the HSS/UDM related failures scenario

Spec:	3GPP TR 29.867 v0.2.0

Agenda item:	21.4

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

At the SA Plenary in September, the SID for Study on Security Aspects for IMS resiliency (FS_IMSRE_SEC, SP-251237.zip) was approved. The Objective section of this SID includes the coordination with CT4.

KI#1 captures the scenarios for P-CSCF, I/S-CSCF and HSS/UDM related failures, and the P-CSCF-related failures are proposed as KI definitions. However, the HSS/UDM related failures may be caused by authentication signaling and the scenario needs to be discussed in SA3.

3. Conclusions

We propose adding a NOTE to the HSS/UDM related failures scenario of KI#1, clarifying that it will be discussed under SA3.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867 v0.2.0.

# * * * 1st Change * * * *

5.1.1	Description of the scenario

When a failure occurs in IMS, depending on the type of failure, a number of UEs may experience failed SIP Registration attempts, leading to mass retransmissions in certain configurations. This surge in SIP Registration requests might lead to congestion of IMS nodes, impacting both voice services and data services, making failure recovery increasingly challenging.

Furthermore, if an operator providing voice services to 40 million subscribers sets the SIP Registration retransmission interval to 40 minutes following a severe IMS failure that affects the whole network, nearly 1 million SIP Registration requests per minute would flood the IMS, exacerbating network congestion.

The expected IMS congestion scenarios that can trigger SIP Registration from UEs:

-	P-CSCF related failures: This scenario includes failures of the P-CSCF itself, network failures between PGW/UPF and P-CSCF, and network failures between PCRF/PCF and P-CSCF. In the event of a failure in this scenario, the P-CSCF does not respond to the SIP message or returns a response with error code. For no response to the SIP message, some UEs may repeatedly attempt SIP Registration, while others cannot determine whether there is an abnormality in the IMS or if there is an issue with data connectivity in the IMS PDN.

-	I/S-CSCF related failures: This scenario includes failures of I-CSCF itself, S-CSCF itself, network failures between the I-CSCF and S-CSCF and network failures between the P-CSCF and I-CSCF. In the event of a failure in this scenario, the I-CSCF and S-CSCF may not respond to the SIP message. Consequently, the P-CSCF will send a SIP error, including a Retry-After message to the UEs. Upon receiving the message, the UE will initiate a SIP Registration request after a delay based on the timer value included in the Retry-After message. This results in mitigating the load and avoiding the overload.

-	HSS/UDM related failures: This scenario includes failures of the HSS/UDM itself, network failures between UDM and S-CSCF, and network failures between the HSS/UDM and I-CSCF. In the event of a failure in this scenario, the HSS/UDM may not respond to the Diameter message from the S-CSCF and I-CSCF, may respond with a delay, or may return a response with error code. Consequently, the P-CSCF will send a SIP error, including a Retry-After message to the UEs. Upon receiving the message, the UE will initiate a SIP Registration request after a delay based on the timer value included in the Retry-After message. This results in mitigating the load and avoiding the overload.

NOTE:	The HSS/UDM related failures may be caused by authentication signaling, which needs coordination with SA3.

Figure 5.1.1-1: IMS failures scenarios for EPS

Figure 5.1.1-2: IMS failures scenarios for 5GS

Several standard solutions exist for different IMS failure scenarios, but not resolving the described issues:

-	P-CSCF Restoration (3GPP TS 23.380 [2]): Mechanism to ensure service continuity by redirecting UEs to an alternative P-CSCF using triggers such as PCO/ePCO, SIP 504 responses, or PDN disconnection. The issues in the solution are as follows:

-	In the scenario where multiple P-CSCF instances fail and the P-CSCF instance selected by the UE is unavailable, the UE continues retrying SIP registration request until it finds an available P-CSCF instance.

-	The GGSN/PGW sends the P-CSCF addresses to the UEs almost simultaneously, and the UEs sends the SIP registration almost simultaneously, which may lead to network congestion.

-	Overload Control (3GPP TS 23.228 [3]): Mechanism to mitigate IMS congestion by limiting SIP Registration attempts. Especially it is commonly used to apply traffic restrictions from the UEs at the P-CSCF, protecting the I-CSCF and S-CSCF, which serve as backend components of the P-CSCF. For example, when Overload Control on the P-CSCF, it is assumed that P-CSCF will send a SIP error, including a Retry-After message as defined in 3GPP TS 24.229 [3] to the regulated UEs. Upon receiving the message, the UE will send a SIP Registration request after a delay based on the timer value included in the Retry-After message. Whether this solution has issues needs to be studied.

-	The solution is designed to protect IMS, so it does not stop but delays SIP Registration requests from UEs to IMS. As a result of UEs repeatedly (potentially with increasing delays) retransmitting SIP Registration.

-	The solution is designed to limit UE access attempts (e.g., SIP Registration request) during the IMS high load to prevent overload and congestion, addressing the scenarios involving IMS failures or precaution measurement to prevent IMS congestion caused by frequent SIP registrations.

-	Congestion caused by SIP Registration can be mitigated by the Overload Control. However, once this mechanism is triggered, it inevitably impacts new registrations and possibly existing voice services to some extent.

-	UAC (3GPP TS 23.501 [5], TS 24.501 [6], TS 38.300 [7]): Mechanism to protect IMS from overload and congestion by setting access categories for IMS registration-related signalling in NG-RAN, triggered by OAM requests or NG-RAN itself. The potential issues in the solution to be studied are as follows:

-	Since there is no clear correlation between the node selection logic of IMS and IMS-related NFs (PGW/PCRF) and the base station's cell or PLMN, it is not possible to precisely apply access control and barring to the UEs affected by the failure. Therefore, to impose the access control and barring that cover 100% of the affected UEs, it is necessary to set UAC excessively, which results in suspending voice services for UEs that are originally unrelated.

-	The solution depends on the 3GPP RAN implementation. Therefore, it is not effective for VoWi-Fi.

-	UAC can only be applied to the RAN operated in house, so IMS failures in roaming-out or IMS failures in MVNO operators cannot be addressed.

-	It is assumed that addressing SIP layer issues between the UE and IMS using protocols between the UE and RAN presents technical challenges. Ideally, the issue should be handled using a protocol closer to the SIP layer.

-	The solution is designed to limit UE access attempts (e.g., SIP Registration request) during the IMS overload and congestion, it does not address the scenarios involving IMS failures or precaution measurement to prevent IMS congestion caused by frequent SIP registrations.

-	From the perspective of actual operation, the following issues are identified:

-	It is necessary to impose access control and barring via UAC on NG-RANs affected by the IMS overload/congestion. This linkage between the area affected by IMS overload and congestion and NG-RAN depends on the operator’s design. However, node bypassing frequently occurs due to various reasons such as maintenance work. As a result, maintaining the uniqueness of the relationship between IMS and RAN is impossible. Therefore, presetting and fixing the linkage configuration would be meaningless.

-	Access control and barring in UAC may take time to become effective depending on the area and scale. When a failure occurs in IMS, within just a few seconds, tens to hundreds of thousands of UEs that failed to SIP Registration requests might emerge, so it does not have an immediate effect.

-	The solution is effective for UEs in specific areas, such as Tracking Areas. However, when considering a nationwide large-scale failure, the effectiveness of the restrictions is expected to be limited. For UEs under restrictions, they will continue retransmitting requests until they succeed. As a result, in the case of a nationwide failure, despite the restrictions imposed, the traffic load on the core network may not be reduced as expected.

-	Dynamic extension of registration expiration timers (3GPP TR 29.866 [8], KI#4/Sol#10): This solution help mitigate HSS overload by adjusting the retransmission timer interval of the UE. The potential issues in the solution to be studied are as follows:

-	This solution cannot stop SIP Registration attempts from the UE itself during the overload.

-	If all HSS instances become unavailable and the HSS cannot send the message, this solution cannot be executed.

-	UE's usage setting (3GPP TS 24.501 [6]): Mechanism defining the preference of voice over data services (or vice versa) for voice-capable UEs. The potential issues in the solution to be studied are as follows:

-	The majority of UEs are configured with a fixed "voice centric" or "data centric", making them dependent on the UE's implementation.

-	During an IMS failure, UEs with a "voice-centric" configuration initiate attach/registration requests, while those with a "data-centric" configuration do not. Nevertheless, periodic SIP registration continues, meaning the SIP registration cannot be stopped during failures.

-	It is technically possible to handle it via firmware updates such as OTA, depending on the UE's implementation, However, the impact of regression testing is significant, making it unusable during failures.

As a result, the UE fails to receive the expected response.

# * * * End of Changes * * * *