3GPP TSG-CT WG4 Meeting #131	C4-254138

Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.573 | CR | 0235 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to N32 security capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | n32KeepaliveTimer is defined as integer in Table 6.1.5.2.3 but DurationSec in the OpenAPI, leading to ambiguity about units. Additionally, several data types used by normative procedures (PlmnId, PlmnIdNid, Uinteger) are missing from the data type tables, which causes confusion for implementers. This CR corrects inconsistencies in the definition of the n32KeepaliveTimer IE and updates the data type tables in clause 6.1.5.1 to include missing reused types. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the data type of n32KeepaliveTimer in Table 6.1.5.2.3 (SecNegotiateRspData) with the OpenAPI definition by specifying DurationSec. - State that the unit of n32KeepaliveTimer is seconds. - Correct the description of n32KeepaliveTimer IE in SecNegotiateRspData. Add missing reused data types (PlmnId, PlmnIdNid, Uinteger) to Table 6.1.5.1-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of misinterpretation of n32KeepaliveTimer semantics and inconsistent timer handling between implementations. Data types are missing from the reused data types table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.1, 6.1.5.2.2, 6.1.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 6.1.5.1	General

This clause specifies the application data model supported by the API.

Table 6.1.5.1-1 specifies the data types defined for the N32 interface.

Table 6.1.5.1-1: N32 specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SecNegotiateReqData | 6.1.5.2.2 | Defines the security capabilities of a SEPP sent to a receiving SEPP. |
| SecNegotiateRspData | 6.1.5.2.3 | Defines the selected security capabilities by a SEPP. |
| SecurityCapability | 6.1.5.3.3 | Enumeration of security capabilities. |
| SecParamExchReqData | 6.1.5.2.4 | Request data structure for parameter exchange |
| SecParamExchRspData | 6.1.5.2.5 | Response data structure for parameter exchange |
| ProtectionPolicy | 6.1.5.2.6 | The protection policy to be negotiated between the SEPPs. |
| ApiIeMapping | 6.1.5.2.7 | API URI to IE mapping on which the protection policy needs to be applied. |
| IeInfo | 6.1.5.2.8 | Protection and modification policy for the IE |
| ApiSignature | 6.1.5.2.9 | API URI of the service operation |
| N32fContextInfo | 6.1.5.2.10 | N32-f context information |
| N32fErrorInfo | 6.1.5.2.11 | N32-f error information. |
| FailedModificationInfo | 6.1.5.2.12 | Information on N32-f modifications block that failed to process. |
| N32fErrorDetail | 6.1.5.2.13 | Details about the N32f error. |
| CallbackName | 6.1.5.2.14 | Callback Name. |
| IpxProviderSecInfo | 6.1.5.2.15 | Defines the security information list of a RI. |
| IntendedN32Purpose | 6.1.5.2.16 | Defines the intended N32 establishment purpose. |
| RiErrorInformation | 6.1.5.2.17 | RI error information. |
| ExtRedirectResponse | 6.1.5.2.18 | Extension of the redirection response |
| RedirectResponseAddInfo | 6.1.5.2.19 | Additional information in the redirection response |
| HttpMethod | 6.1.5.3.4 | Enumeration of HTTP methods. |
| IeType | 6.1.5.3.5 | Enumeration of types of IEs (i.e kind of IE) to specify the protection policy. |
| IeLocation | 6.1.5.3.6 | Location of the IE in a HTTP message. |
| N32fErrorType | 6.1.5.3.7 | Type of error while processing N32-f message. |
| FailureReason | 6.1.5.3.8 | Reason for failure to reconstruct a HTTP/2 message from N32-f message. |
| N32Purpose | 6.1.5.3.9 | Usage purpose of establishing N32 connectivity |
| N32ReleaseIndication | 6.1.5.3.10 | N32-f connection or N32-f context release instructions. |

Table 6.1.5.1-2 specifies data types re-used by the N32 interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N32 Handshake service based interface.

Table 6.1.5.1-2: N32 re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Fqdn | 3GPP TS 29.571 [12] |  |
| SupportedFeatures | 3GPP TS 29.571 [12] | Used to negotiate the applicability of the features defined in table 6.1.7-1. |
| DurationSec | 3GPP TS 29.571 [12] |  |
| Uinteger | 3GPP TS 29.571 [12] | Unsigned integer |
| PlmnId | 3GPP TS 29.571 [12] | PLMN ID |
| PlmnIdNid | 3GPP TS 29.571 [12] | PLMN ID and Network ID |

* * * Next Change * * *

##### 6.1.5.2.2	Type: SecNegotiateReqData

Table 6.1.5.2.2-1: Definition of type SecNegotiateReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sender | Fqdn | M | 1 | This IE shall uniquely identify the SEPP that is sending the request. This IE is used to identify and store the negotiated security capability against the right SEPP. |  |
| n32HandshakeId | string | C | 0..1 | This IE shall be present if the initiating SEPP support N32 handshake identifier and the N32 handshake identifier may be used to correlate the N32-f connection to the parent N32-c context for TLS security (see clause 5.3.3.2). When present, this IE shall contain the N32 handshake identifier to be used by the responding SEPP, if TLS security is negotiated, for: - TLS protected message forwarding procedure over N32-f towards the initiating SEPP - subsequent N32-c signalling request related to this N32-c connection (e.g. to request to tear down the N32-f TLS connection) The n32HandshakeId shall encode a 64-bit integer in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N32 handshake Id shall appear first in the string, and the character representing the 4 least significant bit of the N32 handshake Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{16}$' Example: "0600AD1855BD6007". |  |
| supportedSecCapabilityList | array(SecurityCapability) | M | 1..N | This IE shall contain the list of security capabilities that the requesting SEPP supports. To tear down the N32-f TLS connection and to reset N32-c context (as identified by the n32HandshakeId IE, if present), this IE shall set SecurityCapability as "NONE". |  |
| 3GppSbiTargetApiRootSupported | boolean | C | 0..1 | This IE should be present and indicate that the 3gpp-Sbi-Target-apiRoot HTTP header is supported, if TLS security is supported for N32f message forwarding. When present, it shall indicate if TLS security using the 3gpp-Sbi-Target-apiRoot HTTP header is supported: - true: supported - false (default): not supported (NOTE 1) |  |
| plmnIdList | array(PlmnId) | O | 1..N | A list of PLMN IDs associated with the SEPP, which is sending the request. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]) |  |
| snpnIdList | array(PlmnIdNid) | O | 1..N | A list of SNPN IDs associated with the SEPP, which is sending the request. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]) |  |
| targetPlmnId | PlmnId | O | 1 | When present, this IE shall contain a PLMN ID of the target SEPP. See clause 5.2.2 step 1. |  |
| targetSnpnId | PlmnIdNid | O | 0..1 | When present, this IE shall contain a SNPN ID of the target SEPP. See clause 5.2.2 step 1. |  |
| intendedUsagePurpose | array(IntendedN32Purpose) | O | 1..N | This attribute notifies the list of requested usage purpose the N32 is established for. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.7 is supported |  |
| senderN32fFqdn | Fqdn | O | 0..1 | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection towards a specific FQDN. (NOTE 2) | SNDN32F |
| senderN32fPortList | array(Uinteger) | O | 1..N | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection using a specific port number. The N32-f ports list shall contain one port number per security capability encoded in the supportedSecCapabilityList IE and it shall be ordered in the same order as the security capabilities list. For example, if TLS is the first security capability in the supportedSecCapabilityList, then the first N32-f port in the senderN32fPortList shall be for TLS. (NOTE 3) | SNDN32F |
| n32KeepaliveTimer | DurationSec | O | 0..1 | This IE may be present if the initiating SEPP wishes to indicate for how long (in seconds) it maintains the N32-f connection in the absence of incoming traffic (see clause 5.3.3.5). |  |
| NOTE 1: The attribute name does not follow the naming conventions specified in 3GPP TS 29.501 [5]. The attribute name is kept though as defined in the current specification for backward compatibility reason. NOTE 2: If the senderN32fFqdn IE is absent, the receiving SEPP establishes the N32-f connection towards the sending SEPP using the N32-c FQDN and/or local configuration. NOTE 3: If the senderN32fPortList IE is absent, the receiving SEPP shall use a locally configured port if any, otherwise the default HTTPs port number, i.e., TCP port 443 for "https" URIs as specified in IETF RFC 9113 [7]. |  |  |  |  |  |

* * * Next Change * * *

##### 6.1.5.2.3	Type: SecNegotiateRspData

Table 6.1.5.2.3-1: Definition of type SecNegotiateRspData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sender | Fqdn | M | 1 | This IE shall uniquely identify the SEPP that is sending the response. This IE is used to identify and store the negotiated security capability against the right SEPP. |  |
| selectedSecCapability | SecurityCapability | M | 1 | This IE shall contain the security capability selected by the responding SEPP. When the request is for tearing down the N32-f TLS connection, the responding SEPP shall add SecurityCapability as "NONE". |  |
| n32HandshakeId | string | C | 0..1 | This IE shall be present, if the N32 handshake identifier to was received in the request and the responding SEPP supports the N32 handshake identifier. When present, this IE shall contain the N32 handshake identifier to be used by the initiating SEPP for: - TLS protected message forwarding procedure over N32-f towards the responding SEPP - subsequent N32-c signalling request related to this N32-c connection (e.g. to request to tear down the N32-f TLS connection) The n32HandshakeId in the response may have a different value than the n32HandshakeId received in the request. The n32HandshakeId shall encode a 64-bit integer in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N32 handshake Id shall appear first in the string, and the character representing the 4 least significant bit of the N32 handshake Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{16}$' Example: "0600AD1855BD6007". |  |
| 3GppSbiTargetApiRootSupported | boolean | C | 0..1 | This IE should be present and indicate that the 3gpp-Sbi-Target-apiRoot HTTP header is supported, if TLS security is negotiated for N32f message forwarding and the initiating SEPP indicated support of this header. When present, it shall indicate if TLS security using the 3gpp-Sbi-Target-apiRoot HTTP header is supported: - true: supported - false (default): not supported (NOTE 1) |  |
| plmnIdList | array(PlmnId) | O | 1..N | A list of PLMN IDs of a single PLMN associated with the SEPP, which is sending the response. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]). If different PLMNs are represented by different PLMN IDs supported by a SEPP, then the SEPP shall select the PLMN as specified in clause 5.2.2 step 2a. |  |
| snpnIdList | array(PlmnIdNid) | O | 1..N | A list of SNPN IDs of a single SNPN associated with the SEPP, which is sending the response. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]). If different SNPNs are represented by different SNPN IDs supported by a SEPP, then the SEPP shall select the SNPN as specified in clause 5.2.2 step 2a. |  |
| allowedUsagePurpose | array(IntendedN32Purpose) | O | 1..N | This attribute notifies the list of allowed usage purpose the N32 is established for. IntendedN32Purpose shall not include attribute "cause". |  |
| rejectedUsagePurpose | array(IntendedN32Purpose) | O | 1..N | This attribute notifies the list of rejected usage purpose the N32 is established for. Shall only be present if any of the requested usage purpose is rejected. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.7 is supported |  |
| senderN32fFqdn | Fqdn | O | 0..1 | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection towards a specific FQDN. (NOTE 2) | SNDN32F |
| senderN32fPort | Uinteger | O | 0..1 | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection using a specific port number. (NOTE 3) | SNDN32F |
| n32KeepaliveTimer | DurationSec | O | 0..1 | This IE may be present if the responding SEPP wishes to indicate for how long (in seconds) it maintains the N32-f connection in the absence of incoming traffic (see clause 5.3.3.5). |  |
| NOTE 1: The attribute name does not follow the naming conventions specified in 3GPP TS 29.501 [5]. The attribute name is kept though as defined in the current specification for backward compatibility reason. NOTE 2: If the senderN32fFqdn IE is absent, the receiving SEPP establishes the N32-f connection towards the sending SEPP using the N32-c FQDN and/or local configuration. NOTE 3: If the senderN32fPort number is absent, the receiving SEPP shall use a locally configured port, if any, otherwise the default HTTPs port number, i.e., TCP port 443 for "https" URIs as specified in IETF RFC 9113 [7]. |  |  |  |  |  |

* * * End of Changes * * *

* * * For Information * * *

# A.2	N32 Handshake API

openapi: 3.0.0

info:

version: '1.4.0-alpha.2'

title: 'N32 Handshake API'

description: |

N32-c Handshake Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

servers:

- url: '{apiRoot}/n32c-handshake/v1'

variables:

apiRoot:

default: https://example.com

description:  apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

externalDocs:

description: 3GPP TS 29.573 V19.4.0; 5G System; Public Land Mobile Network (PLMN) Interconnection; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.573/

[Skipped for clarity]

SecNegotiateRspData:

description: Defines the selected security capabilities by a SEPP

type: object

required:

- sender

- selectedSecCapability

properties:

sender:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

selectedSecCapability:

$ref: '#/components/schemas/SecurityCapability'

n32HandshakeId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

3GppSbiTargetApiRootSupported:

type: boolean

default: false

# The attribute name does not follow the naming conventions specified in 3GPP TS 29.501. The attribute name is kept though as defined in the current specification for backward compatibility reason.

plmnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

snpnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

allowedUsagePurpose:

type: array

items:

$ref: '#/components/schemas/IntendedN32Purpose'

minItems: 1

rejectedUsagePurpose:

type: array

items:

$ref: '#/components/schemas/IntendedN32Purpose'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

senderN32fFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

senderN32fPort:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

n32KeepaliveTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

[Skipped for clarity]