3GPP TSG-CT4 Meeting #131	C4-254166

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0909 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on epsInterworkingIndication alt1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The implementation of epsInterworkingInd in the SmContextCreateData is currently leading to interoperability issues. In nsmf_pdusession_createsmcontext_request, epsInterworkingInd is not required. The AMF has two options when constructing SmContextRequest - Do not set epsInterworkingInd (i.e., omit the IE entirely) - Explicitly set epsInterworkingInd to "NONE" The current specification states that: When present, this IE shall indicate whether the PDU session may possibly be moved to EPS or EPC/ePDG and whether N26 interface to be used during EPS interworking procedures. The potential ambiguity here is that if the PDU session cannot be moved to EPS, an implementer may interpret the specification as meaning the IE does not need to be included in the SmContextCreateData at all. It has been observed during interoperability testing that some AMF implementations do not provide epsInterworkingInd even when a combined P-GW-C+SMF is selected. As a result, the P-GW-C+SMF fails to complete the feature testing for EBI allocation, since the epsInterworkingInd is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To avoid such ambiguity, it is proposed to clarify when to include the epsInterworkingInd . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The differences in interpretation may lead to continuous interoperability failures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Change * * * *

6.1.6.2.2	Type: SmContextCreateData

Table 6.1.6.2.2-1: Definition of type SmContextCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| roamingUeInd | boolean | O | 0..1 | This IE may be present, when the requestType IE indicates that the request refers to an emergency PDU session and the supi IE is present. When present, this IE shall indicate whether the UE is a roaming UE or not: - true: the UE is a roaming UE. - false: the UE is a non-roaming UE. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UICCless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the PDU Session ID. |  |
| dnn | Dnn | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the requested DNN; the DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. (NOTE 9) |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. (NOTE 9) |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure. In this case, it shall contain the requested S-NSSAI for the serving PLMN. This corresponds to an S-NSSAI from the allowed NSSAI. This IE shall also be present during an EPS to 5GS idle mode mobility or handover with I-SMF/V-SMF involved using the N26 interface. In this case, it shall contain the S-NSSAI configured in the AMF for EPS interworking. This IE shall also be present during a V-SMF insertion or an inter PLMN V-SMF change procedures. When present, it shall contain the S-NSSAI for the serving PLMN which shall be used by the (target) V-SMF instead of the one that it receives in the SmContext from the old V-SMF or the H-SMF. This IE shall also be present during an inter PLMN I-SMF insertion or change procedures, if the Serving PLMN ID is not the same as the PLMN ID of the SUPI and a mapped S-NSSAI value of the HPLMN is present in the Allowed NSSAI for a non-roaming UE. When present, it shall contain the S-NSSAI for the serving PLMN which shall be used by the (target) I-SMF instead of the one that it receives in the SmContext from the old I-SMF or the SMF. This IE shall also be present in Indirect Network Sharing deployment. It shall contain the S-NSSAI for the hosting operator corresponding to the participating operator's S-NSSAI. See clause 5.18.5 of 3GPP TS 23.501 [2]. |  |
| altSnssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure if the NF service consumer supports the network slice replacement, and the network slice indicated in the sNssai IE is requested to be replaced. In this case, the NF service consumer shall send the S-NSSAI and the alternative S-NSSAI. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a roaming PDU session, or a non-roaming PDU session if the Serving PLMN ID is not the same as the PLMN ID of the SUPI and a mapped S-NSSAI value of the HPLMN is present in the Allowed NSSAI, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain the requested S-NSSAI for the HPLMN. This corresponds to an S-NSSAI from the Mapping of Allowed NSSAI corresponding to the SNSSAI value included in the sNssai IE. This IE shall be present also in Indirect Network Sharing deployment. It shall contain the S-NSSAI for the HPLMN corresponding to the hosting operator's S-NSSAI. See clause 5.18.5 of 3GPP TS 23.501 [2]. |  |
| altHplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a roaming PDU session during the PDU session establishment procedure if the NF service consumer supports the network slice replacement, and the network slice indicated in the hplmnSnssai IE is requested to be replaced. In this case, the NF service consumer shall send the HPLMN S-NSSAI and the alternative HPLMN S-NSSAI. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| servingNfId | NfInstanceId | M | 1 | This IE shall contain the identifier of the serving NF (e.g. serving AMF). |  |
| guami | Guami | C | 0..1 | This IE shall contain the serving AMF's GUAMI. It shall be included if the NF service consumer is an AMF. |  |
| serviceName | ServiceName | O | 0..1 | When present, this IE shall contain the name of the AMF service to which SM context status notifications are to be sent (see clause 6.5.2.2 of 3GPP TS 29.500 [4]). This IE may be included if the NF service consumer is an AMF. |  |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. This IE shall contain the selected PLMN ID representing the participating operator in the case of Indirect Network Sharing (see clause 5.18.1 of 3GPP TS 23.501 [2]). |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from UE, this IE shall be set based on the Request type IE received (see clause 9.11.3.47 of 3GPP TS 24.501 [7]). |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present and reference the N1 SM Message binary data (see clause 6.1.6.4.2), except during an EPS to 5GS Idle mode mobility or handover using N26. |  |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. |  |
| perLadnDnnSnssaiInd | boolean | C | 0..1 | This IE shall be present, if the AMF enforces the LADN Service Area per LADN DNN and S-NSSAI (see clause 4.3.2.2.1 of 3GPP TS 23.502 [3]). When present, it shall indicate that the PDU Session is subject to LADN per LADN DNN and S-NSSAI and be set as follows: - True: the PDU Session is subject to LADN per LADN DNN and S-NSSAI. - False (default): the PDU Session is not subject to LADN per LADN DNN and S-NSSAI. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| smContextStatusUri | Uri | M | 1 | This IE shall include the callback URI to receive notification of SM context status. |  |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios, including Indirect Communication with Delegated Discovery, if the AMF and V-SMF do not support the ACSCR feature. This IE shall be present in HR roaming scenarios during a PDU session establishment procedure and EPS to 5GS mobility procedures, if the AMF and V-SMF support the ACSCR feature. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected H-SMF. The API URI shall be formatted as specified in clause 6.1.1. (NOTE 8) |  |
| hSmfId | NfInstanceId | O | 0..1 | This IE may be present when hSmfUri is present. If present, this IE shall carry the NF instance ID of the selected H-SMF. (NOTE 2) |  |
| hsmfSetId | NfSetId | O | 0..1 | This IE may be present when the hSmfId IE is present, if SMF Set is supported by the H-SMF indicated by the hSmfId. When present, this IE shall contain the NF Set ID of the H-SMF identified by the hSmfId. (NOTE 11) |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, including Indirect Communication with Delegated Discovery, if the AMF and I-SMF do not support the ACSCR feature. This IE shall be present for a PDU session with an I-SMF during a PDU session establishment procedure and EPS to 5GS mobility procedures, if the AMF and I-SMF support the ACSCR feature. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected SMF. The API URI shall be formatted as specified in clause 6.1.1. (NOTE 8) | DTSSA |
| smfId | NfInstanceId | O | 0..1 | This IE may be present when smfUri is present. If present, this IE shall carry the NF instance ID of the selected SMF. (NOTE 2) | DTSSA |
| smfSetId | NfSetId | O | 0..1 | This IE may be present when the smfId IE is present, if SMF Set is supported by the SMF indicated by the smfId. When present, this IE shall contain the NF Set ID of the anchor SMF identified by the smfId. |  |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| pduSessionsActivateList | array(PduSessionId) | C | 1..N | This IE shall be present, during an EPS to 5GS Idle mode mobility using the N26 interface, if the UE indicated PDU session(s) to be activated in the Registration Request. When present, it shall indicate all the PDU session(s) requested to be re-activated by the UE. |  |
| ueEpsPdnConnection | EpsPdnCnxContainer | C | 0..1 | This IE shall be present, during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain an MME/SGSN UE EPS PDN connection including the EPS bearer context(s). |  |
| hoState | HoState | C | 0..1 | This IE shall be present during an EPS to 5GS handover using N26 interface or during a N2 handover with I-SMF insertion/change/removal procedure, to request the preparation of a handover of the PDU session. When present, it shall be set as specified in clauses 5.2.2.2.3 or 5.2.2.2.5. |  |
| additionalHsmfUri | array(Uri) | O | 1..N | This IE may be present in HR roaming scenarios. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional H-SMFs discovered by the AMF for the given DNN, hplmnSnssai and for this PDU session. If provided, the V-SMF shall use these additional H-SMF(s) if the V-SMF is not able to receive any response from the H-SMF identified by hSmfUri. The API URI shall be formatted as specified in clause 6.1.1. |  |
| additionalHsmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalHsmfUri is present. If present, this IE shall carry the NF instance ID(s) of H-SMF(s) as stated in additionalHsmfUri IE, in exactly the same order. (NOTE 2) |  |
| additionalHsmfSetIdList | map(NfSetId) | O | 1..N | This IE may be present when the additionalHsmfId IE is present. When present, this IE shall carry the SMF Set ID per additional H-SMF which supports SMF Set. The NF instance ID of the additional H-SMF is the key of the map. (NOTE 10, NOTE 11) |  |
| additionalSmfUri | array(Uri) | O | 1..N | This IE may be present for a PDU session with an I-SMF. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional SMFs discovered by the AMF for the given DNN, Snssai and for this PDU session. If provided, the I-SMF shall use these additional SMF(s) if the I-SMF is not able to receive any response from the SMF identified by smfUri. The API URI shall be formatted as specified in clause 6.1.1. | DTSSA |
| additionalSmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalSmfUri is present. If present, this IE shall carry the NF instance ID(s) of SMF(s) as stated in additionalSmfUri IE, in exactly the same order. (NOTE 2) | DTSSA |
| additionalSmfSetIdList | map(NfSetId) | O | 1..N | This IE may be present when the additionalSmfId IE is present. When present, this IE shall carry the SMF Set ID per additional anchor SMF which supports SMF Set. The NF instance ID of the additional anchor SMF is the key of the map. (NOTE 10) |  |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |  |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if pcfId IE is present. When present, this IE shall contain the NF Set ID of the PCF indicated by the pcfId IE. |  |
| nrfUri | Uri | O | 0..1 | This IE may be present to indicate the NRF to use for PCF selection within the same network slice instance. When present, the SMF shall use the NRF URI to select the PCF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.8 is supported. |  |
| anchorSmfFeatures | SupportedFeatures | C | 0..1 | This IE shall be present, during the establishment of a HR PDU session or a PDU session with an I-SMF, if the information is known by the AMF. When present, this IE shall include the features of the Nsmf_PDUSession service (see clause 6.1.8) that are supported by anchor SMF. The absence of this IE shall not be interpreted as indicating that the anchor SMF does not support any feature. |  |
| additional AnchorSmfFeatures | array(SupportedFeatures) | C | 1..N | This IE shall be present, during the establishment of a HR PDU session or a PDU session with an I-SMF, if the additionalHsmfUri IE or the additionalSmfUri IE is present and the information is known for all the additional anchor SMFs. When present, this IE shall include the features of the Nsmf_PDUSession service (see clause 6.1.8) that are supported by the additional anchor SMFs, in exactly the same order. The absence of this IE shall not be interpreted as indicating that the additional anchor SMFs do not support any feature. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| backupAmfInfo | array(BackupAmfInfo) | C | 1..N | This IE shall be included if the NF service consumer is an AMF and the AMF supports the AMF management without UDSF for the following cases: - First interaction with SMF. - Modification of the BackupAmfInfo. |  |
| traceData | TraceData | C | 0..1 | This IE shall be included if trace is required to be activated (see 3GPP TS 32.422 [22]). |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 3) |  |
| epsInterworkingInd | EpsInterworkingIndication | C | 0..1 | This IE shall be present when a PGW-C+SMF is selected to serve the PDU Session. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS or EPC/ePDG and whether N26 interface to be used during EPS interworking procedures. The AMF may derive the value of the indication from different sources, like UE 5GMM capabilities (e.g. "S1 mode supported"), UE subscription data (e.g. "Core Network Type Restriction to EPC" and "Interworking with EPS Indication" for the DNN) and configurations. |  |
| indirectForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of indirect data forwarding. When present, it shall be set as follows: - True: indirect data forwarding is applicable - False: indirect data forwarding is not applicable |  |
| directForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of direct data forwarding. When present, it shall be set as follows: - True: direct data forwarding is applicable - False: direct data forwarding is not applicable |  |
| targetId | NgRanTargetId | C | 0..1 | This IE shall be present in the following cases: - during an EPS to 5GS handover preparation using the N26 interface, when the hoState IE is set to the value "PREPARING"; - during N2 based handover procedure with I-SMF or V-SMF insertion/change/removal, when hostate IE is set to the value "PREPARING". When present, it shall contain the Target ID identifying the target RAN Node ID and TAI. In case of EPS to 5GS handover, the TAI is received in the Forward Relocation Request from the Source MME. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Registration Request from the UE. When present, it shall be set to the value received from the UE. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True", if - the NF service consumer (e.g. the AMF) has verified that the CIOT feature is supported by the SMF (and for a home-routed session, that it is also supported by the H-SMF); and - Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clauses 4.3.2.2.1 and 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with the value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| n3gPathSwitchSupportInd | boolean | C | 0..1 | This IE shall be present if AMF supports non-3GPP access path switching while maintaining two N2 connections for non-3GPP access, the selected SMF supports non-3GPP path switching and if the UE supports non-3GPP access path switching for a MA-PDU session. When present, it shall be set as follows: - true: non-3GPP access path switching is supported - false (default): non-3GPP access path switching is not supported | N3GPS |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the I-SMF. | DTSSA |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. | DTSSA |
| n2SmInfoExt1 | RefToBinaryData | C | 0..1 | This IE shall be present if more than one N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). | DTSSA |
| n2SmInfoTypeExt1 | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfoExt1" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfoExt1" attribute. | DTSSA |
| smContextRef | Uri | C | 0..1 | This IE shall be present during an I-SMF or V-SMF insertion if available and during an I-SMF or V-SMF change or removal. When present, this IE shall contain the URI of the SM Context resource in the SMF or of the SM context resource in the source I-SMF or V-SMF during an I-SMF or V-SMF insertion or during an I-SMF or V-SMF change/removal respectively. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.3.2). (NOTE 6) | DTSSA |
| smContextSmfPlmnId | PlmnIdNid | C | 0..1 | This IE shall be present during an inter-PLMN mobility procedure if the smContextRef IE is present. It may be present otherwise, if the smContextRef IE is present. When present, this IE shall carry the PLMN ID of the SMF which hosts the SM Context resource identified by smContextRef IE. For an SNPN, the NID together with the PLMN ID shall identify the SNPN. (NOTE 7) | DTSSA |
| smContextSmfId | NfInstanceId | O | 0..1 | This IE may be present if smContextRef is present. When present, this IE shall carry the NF instance ID of the SMF which hosts the SM Context resource identified by smContextRef IE. (NOTE 2) | DTSSA |
| smContextSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the old V-SMF or the old I-SMF or the SMF as identified by the smContextSmfId. | DTSSA |
| smContextSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this SmContext) in the old V-SMF or the old I-SMF or the SMF. | DTSSA |
| smContextSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the SM context resource. | DTSSA |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to request the activation of the user plane connection of the PDU session, in the following cases: - during Service Request procedure with an I-SMF insertion / change / removal, or with a V-SMF change (see clause 5.2.2.2.6); - during Registration procedure with an I-SMF insertion / change / removal, or with a V-SMF insertion / change / removal (see clause 5.2.2.2.7), if this PDU session is requested to be activated by the UE. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is available in AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present if the APN rate control status is available in AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 5.2.8.2.5 in 3GPP TS 23.502 [3]. | CIOT |
| extendedNasSmTimerInd | boolean | C | 0..1 | This IE shall be present with the value "True" if the UE supports CE mode B and use of CE mode B is not restricted according to the Enhanced Coverage Restriction information in the UE context in the AMF. When present, it shall indicate whether extended NAS SM timers shall be used for the UE as specified in 3GPP TS 24.501 [7], as follows: - True: extended NAS SM timers shall be used - False (default): normal NAS SM timers shall be used. | CIOT |
| dlDataWaitingInd | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), if the same indication is received from the MME in the Context Response message. When present, it shall be set as follows: - true: DL data needs to be sent to the UE; - false (default): no DL data needs to be sent to the UE. | CIOT |
| ddnFailureSubs | DdnFailureSubs | C | 0..1 | This IE shall be present to subscribe the notification of the DDN Failure if the Availability after DDN failure event is subscribed by the UDM, see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. | CIOT |
| smfTransferInd | boolean | C | 0..1 | This IE shall be present during an SMF Context Transfer procedure, LBO or no Roaming, no I-SMF. When present, it shall be set as follows: - True: SMF Context Transfer - False (default): Not an SMF Context Transfer | CTXTR |
| oldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, it shall indicate old SMF instance identifier. | CTXTR |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. This IE shall also be present, without smfTransferInd set, if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. | CTXTR EnEDGE |
| wAgfInfo | WAgfInfo | C | 0..1 | This IE shall be present, if received from the W-AGF. When present, it shall contain information about the N3 terminations of the W-AGF. The SMF may use this information when selecting the UPF. |  |
| tngfInfo | tngfInfo | C | 0..1 | This IE shall be present, if received from the TNGF. When present, it shall contain information about the N3 terminations of the TNGF. The SMF may use this information when selecting the UPF. |  |
| twifInfo | twifInfo | C | 0..1 | This IE shall be present, if received from the TWIF. When present, it shall contain information about the N3 terminations of the TWIF. The SMF may use this information when selecting the UPF. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if the NG-RAN is not changed in case of I-SMF/V-SMF change or insertion during CM-CONNECTED registration procedure after EPS to 5GS handover (see clause 5.2.2.2.7) or I-SMF/V-SMF selection per DNAI (see clause 5.2.2.2.12). When present, it shall be set as follows: - true: NG-RAN is not changed; - false: NG-RAN is changed. | DTSSA |
| samePcfSelectionInd | boolean | C | 0..1 | This IE shall be present, if the AMF received the PCF Selection Assistance info from the UDM indicating that the same PCF is required. (NOTE 4) When present, it shall be set as follows: - True: the SMF is indicated to select the same PCF instance for the PDU session. - False (default): the SMF is not indicated to select the same PCF instance for the PDU session. |  |
| targetDnai | Dnai | C | 0..1 | This IE shall be present, if this information was received earlier in Notify SM Context Status. I-SMF or SMF shall use this information for I-UPF / L-PSA / PSA selection, see clauses 4.3.5.1, 4.3.5.2 or 4.23.5.4 of 3GPP TS 23.502 [3]. V-SMF shall use this information for V-UPF selection, see clause 6.7.3.2 of 3GPP TS 23.548 [74]. | EnEDGE HR-SBO |
| nrfManagementUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFManagement Service (see clause 6.1.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfDiscoveryUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFDiscovery Service (see clause 6.2.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfAccessTokenUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the Access Token Service (see clause 6.3.2 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfOauth2Required | map(boolean) | C | 1..N | This IE should be present if the nrfUri IE, nrfManagementUri IE or nrfDiscoveryUri IE is present and if the information is available. When present, this IE shall indicate whether the NRF requires Oauth2-based authorization for accessing its services. The key of the map shall be the name of an NRF service, e.g. "nnrf-nfm" or "nnrf-disc". The value of each entry of the map shall be encoded as follows: - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of NRF services. |  |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the SM context resource and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. | DTSSA |
| pvsInfo | array(ServerAddressingInfo) | C | 1..N | This IE shall be present, if the AMF received this information from AUSF during User Plane Remote Provisioning of UEs procedure (see clause 5.30.2.10.4 of 3GPP TS 23.501 [2]). When present, this IE shall contain the remote Provisioning Server(s) information. | ENPN |
| onboardingInd | boolean | C | 0..1 | This IE shall be present, if the UE is registered for onboarding in an SNPN (see clause 5.30.2.10.4 in 3GPP TS 23.501 [40] and clause 4.2.2.2.4 in 3GPP TS 23.502 [3]). - false (default): The UE is not registered in an SNPN for the purpose of onboarding; - true: The UE has registered in the SNPN for the purpose of onboarding. | ENPN |
| oldPduSessionRef | Uri | C | 0..1 | This IE shall be present if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2). | EnEDGE |
| smPolicyNotifyInd | boolean | O | 0..1 | When present, this IE shall indicate whether the SM Policy Association Establishment and Termination events shall be reported for the PDU session by the PCF for the SM Policy to the PCF for the UE: - true: SM Policy Association Establishment and Termination events shall be reported - false (default): SM Policy Association Establishment and Termination events is not required (NOTE 5) | SPAE |
| pcfUeCallbackInfo | PcfUeCallbackInfo | C | 0..1 | This IE shall be present when the smPolicyNotifyInd IE is present with value true. When present, this IE shall contain the callback information (callback URI and binding information, if available) of the PCF for the UE to receive SM Policy Association Establishment and Termination events notification from the PCF for the SM Policy. (NOTE 5) | SPAE |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | This IE may be present if the AMF supports the 5GSAT feature and the AMF is aware that there is a satellite backhaul towards the 5G AN serving the UE. When present, this IE shall indicate the category of the satellite backhaul used towards the 5G AN serving the UE. | 5GSAT |
| upipSupported | boolean | C | 0..1 | This IE shall be present during the PDU session establishment procedure, if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality. It may be present otherwise. When present, this IE shall be set as follows: - true: User Plane Integrity Protection with EPS is supported; - false (default): User Plane Integrity Protection with EPS is not supported. | UPIPE |
| disasterRoamingInd | boolean | O | 0..1 | This IE may be set when the UE is registered for Disaster Roaming service. When present, this IE shall be set as follows: - true: the UE is registered for Disaster Roaming service - false (default): the UE is not registered for Disaster Roaming service |  |
| anchorSmfOauth2Required | boolean | O | 0..1 | When present, this IE shall indicate whether the H-SMF or SMF for a PDU session with an I-SMF requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the anchor SMF's Nsmf_PDUSession service. (NOTE 2) |  |
| smContextSmfOauth2Required | boolean | O | 0..1 | This IE may be present during an I-SMF insertion, change or removal. This IE may be present when V-SMF changes within the same PLMN. When present, this IE shall indicate whether the SMF hosting the SM Context requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the Nsmf_PDUSession service on the SMF hosting the SM Context. (NOTE 2) |  |
| geoSatelliteId | GeoSatelliteId | O | 0..1 | The AMF may include this IE during a PDU Session Establishment procedure (see clause 4.3.2.2.1 of 3GPP TS 23.502 [3]) for support of 5G satellite backhaul as defined in clause 5.43 of 3GPP TS 23.501 [2]. When present, this IE shall contain a GEO satellite ID. | 5GSATB |
| servingSatelliteId | SatelliteId | O | 0..1 | The AMF may include this IE during a PDU Session Establishment procedure (see clause 4.3.2.2.1 of 3GPP TS 23.502 [3]) to support UE-Satellite-UE communication for the NR satellite access as defined in clause 5.4.14 of 3GPP TS 23.501 [2]. When present, this IE shall contain UE's serving satellite ID. | 5GSAT-ACCESS |
| hrsboAllowedInd | boolean | O | 0..1 | This IE may be present in HR roaming scenarios. When present, this IE shall Indicate whether Session Breakout for HR Session in VPLMN is allowed: - true: Allowed. - false (default): Not allowed. | HR-SBO |
| estabRejectionInd | boolean | O | 0..1 | This IE may be present with the value true if the AMF has determined that the PDU Session Establishment shall be rejected, e.g. according to the ODB configuration of the UE retrieved from the UDM. Presence of this IE with the value false shall be prohibited. | PSER |
| estabRejectionCause | EstablishmentRejectionCause | C | 0..1 | This IE shall be included if the estabRejectionInd is present with the value true. When present, this IE shall indicate the cause of the PDU session establishment rejection. | PSER |
| sliceAreaRestrictInd | boolean | C | 0..1 | This IE shall be present and set to true if the AMF determines that the PDU Session is subject to area restriction for the S-NSSAI, i.e. if the S-NSSAI is part of the partially allowed NSSAI or if the support of the S-NSSAI is restricted to a NS-AoS. See clauses 5.15.17 and 5.15.18 of 3GPP TS 23.501 [2]. Presence of this IE with the false value shall be prohibited. | SAR |
| qosMonitoringPdSupported | QosMonitoringPdSupported | C | 0..1 | This IE shall be present if the QME feature is supported by AMF and SMF and if the information is available. When present, this IE shall indicate whether the NG-RAN supports the QoS monitoring for packet delay feature. See clause 5.45.1 of 3GPP TS 23.501 [2]. | QME |
| qosMonitoringCongestionSupported | QosMonitoringCongestionSupported | C | 0..1 | This IE shall be present if: - the QME feature is supported by the AMF; - the QME feature and the EMECI feature are supported by the SMF; and - the information is available. When present, this IE shall indicate whether the NG-RAN supports the QoS monitoring for congestion feature. See clause 5.45.1 of 3GPP TS 23.501 [2]. | QME |
| availBitRateMonSupported | AvailBitRateMonSupported | C | 0..1 | This IE shall be present if the AVABIT feature is supported by AMF and SMF and if the information is available. When present, this IE shall indicate whether the NG-RAN supports the available bitrate monitoring feature. See clause 5.45.1 of 3GPP TS 23.501 [2]. | AVABIT |
| ueLevelMeasConfig | UeLevelMeasurementsConfiguration | C | 0..1 | This IE shall be present if the signalling based UE Level Measurement task is activated. When present, this IE shall contain UE Level Measurement configuration data for UE (see clause 4.1.2.18 of 3GPP TS 32.422 [22]). |  |
| pgwChangeInd | boolean | C | 0..1 | This IE shall be present if the AMF performs PDN connection restoration at an alternative PGW-C/SMF during a EPS to 5GS mobility procedure. See clause 31.5 of 3GPP TS 23.007 [45]. When present, this IE shall indicate whether it is the restoration of the PDN connection at an alternative PGW-C/SMF. - true: it is for the restoration of the PDN connection at an alternative PGW-C/SMF Presence of this IE with the value false shall be prohibited. |  |
| localOffloadingMgtAllowedInd | boolean | C | 0..1 | This IE shall be present with the value true, if Local Offloading Management is allowed based on subscription data and the UE is within a Local Offloading Management service area. The presence of this IE with the value false shall be prohibited. | ISMF-LOM |
| NOTE 1: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. NOTE 2: If the SMF is aware that Oauth is enabled for the indicated next hop SMF, e.g. when the anchorSmfOauth2Required IE and/or the smContextSmfOauth2Required IE are received with the value true or when received a "401 Unauthorized" response code from next hop SMF, the SMF shall use the NF instance Identifier to acquire the access token for the Nsmf_PduSession service on the indicated SMF. NOTE 3: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 4: If present, this attribute shall be used together with the PCF ID received from the AMF for selecting the same PCF instance for the PDU session. NOTE 5: If the AMF has received the callback information of the PCF for the UE together with the information of the PDU sessions (i.e. Slice and DNN combination) that are applicable for notification of SM Policy Association events (from PCF or from old AMF in UE Context), during the PDU session establishment procedure, the AMF shall identify whether the non-roaming or local breakout PDU session to be created is applicable for SM Policy Association events, i.e, whether the slice and DNN combination of the PDU session is listed in the received PDU session information from the PCF for the UE. If the PDU session is applicable for notification of SM Policy Association events, the AMF shall include the smPolicyNotifyInd IE with the value "true" and the callback information of the PCF for the UE in the request. The SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy during SM Policy Association Establishment. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. NOTE 6: See NOTE 2 of Table 6.1.6.2.3-1. NOTE 7: If the PLMN ID of the SMF holding the SM context received in smContextSmfPlmnId attribute is different from the PLMN ID of the target V-SMF/I-SMF/anchor SMF, the target V-SMF/I-SMF/anchor SMF shall retrieve the SM Context from the SMF via the SEPP. In this case, the smContextSmfPlmnId attribute may also be used for the discovery and selection of the local SEPP. NOTE 8: The smfUri and hSmfUri attributes need not be included in Create SM Context request in procedures other than PDU session establishment procedure and EPS to 5GS mobility procedures if the NF Service Consumer (e.g. AMF) and I-SMF/V-SMF support the "ACSCR" feature. See clause 6.1.8. NOTE 9: When the AMF supports the "subscription-based routing to a target core network" feature as specified in clause 5.48 of 3GPP TS 23.501 [2] and it receives the targetPlmnId as part of SmfSelectionSubscriptionData, it shall include the targetPlmnId as Operator Identifier in the Dnn to establish the PDU session in the SMF in that PLMN. The V/I-SMF shall forward the Dnn to the (H-)SMF in the PduSessionCreateData and should include the Dnn as one of query parameters if it needs to perform a service discovery to select an alternative (H-)SMF, e.g., when the binding indication is not received from the failed (H-)SMF. If the PDU session is a HR PDU session, the V-SMF shall include the target-plmn-list set to the PLMN ID derived from UE's SUPI in the discovery request message. NOTE 10: Additional (H-)SMF instances belonging to different SMF Sets shall be allowed only during establishment of a new PDU session, i.e. when Create SM Context is invoked for an existing PDU session (e.g. I-SMF/V-SMF insertion or change) or PDN connection (e.g. EPS to 5GS mobility), only alternative SMF instances within the same SMF Set of the (H-)SMF can be reselected. NOTE 11: If an H-SMF Set ID is received, the V-SMF may further delegate the re-selection of the H-SMF instance within the H-SMF set selected by the AMF as specified in clause 4.17.10a of 3GPP TS 23.502 [3] and clause 6.10.12 3GPP TS 29.500 [4], during the establishment of a HR PDU session, by including the 3gpp-Sbi-Discovery-target-nf-set-id set to the H-SMF Set ID and all the other discovery factors necessary for the H-SMF instance reselection in the Create Request it sends towards the HPLMN. In this scenario, the V-SMF should support the 3gpp-Sbi-Response-Info header to determine whether it may retry or not a request that is not successful towards an alternative H-SMF instance of the H-SMF set (see NOTE 3 of Table 6.1.6.2.3-1). |  |  |  |  |  |

* * * End Change * * * *