3GPP TSG-CT WG4 Meeting #131	C4-254169

Sophia-Antipolis; 13th – 17th October 2025

Source:	Ericsson

Title:	UE ID with LCS User Plane Connection

Agenda item:	18.40

Document for:	DISCUSSION

# 1. Background

In 3GPP release 18 LCS User Plane Connection feature was introduce to off load the control plane traffics, which allows UE and LMF to exchange LCS session requests/responses via user plane instead of control plane.

If the LCS-UPP connection feature is supported, the LMF and UE can establish an LCS-UPP connection, e.g. when the LMF received a location request towards the UE and determine the UE supports LCS-UPP or when the UE triggers LCS-UPP connection establishment with an uplink UPP-CM establishment request. When subsequent the LMF received a location request to a UE, the LMF will determine the existing LCS-UPP connection and send/receive the N1 LCS messages with the UE (e.g. LPP/LCS-AP) for UE positioning procedure via the LCS-UPP connection.

The LMF determine whether there is an existing LCS-UPP connection with the UE based on the UE ID used for the LCS-UPP establishment and the UE ID in the location request. Thus the UE ID is mandatory in location request when the LMF supports LCS-UPP connections. Currently the AMF may provide SUPI or GPSI to the LMF in the location request and in the UPConfig operation (for UE triggered LCS-UPP establishment). Because the LMF doesn't have the mapping relationship between the SUPI and GPSI for the UE, to ensure that the LMF can successfully associate the LCS-UPP connection and location requests for the same UE, there is a requirement that the AMF needs to provide consistent UE ID for the same UE if LCS-UPP connection is supported, i.e. always SUPI or always GPSI.

# 2. Discussion

There are two problems in above requirements:

- When the LCS-UPP connection establishment and the location requests for the same UE are handled by the same AMF, the above requirements can be implemented by the AMF, i.e. the AMF will remember which UE ID was provided to the LMF for this UE and always use this UE ID. However, after the UE is moved to a new AMF after UE mobility, the new AMF doesn't know which UE ID to be used and may provide a different UE ID in subsequent location request. If this happen, the LMF cannot determine that there is already one existing LCS-UPP connection established with the UE, so the LMF may either try to establish another LCS-UPP connection with the same UE (which leads to uncertain result) or perform the UE positioning via control plane (i.e. the LCS-UPP feature becomes useless).

Option-1.1: To enforce that the SUPI shall be mandatory to use for the UE, if LCS-UPP feature is supported, with GPSI as optional addition. With this option, the LMF can always receive the SUPI of the UE when LCS-UPP feature is supported, thus the LMF can always successfully associate the subsequent location requests and the existing LCS-UPP connection for the same UE with the SUPI.

Option-1.2: To indicate the UE ID used in old AMF to the new AMF for each LMF in the LCS-UPP context of the UE context. With this option, the new AMF shall read the UE ID used for this LMF from the UE context (if available) and always use the UE ID received in the UE context in subsequent location request.

- A UE might have multiple GPSIs and different AF may use different GPSIs to request UE positioning. When the LCS-UPP connection establishment was triggered by a location request from AF1 with GPSI-1, the LCS-UPP connection will be recorded in the LMF with GPSI-1 for the UE. If subsequently AF2 request UE location with GPSI-2 for the UE, the AMF cannot correctly handle the request towards the LMF,

  - if the AMF provide GPSI-1 to the LMF (following the requirement as in the background), the LMF can successfully associate the LCS-UPP connection. However, the GPSI-1 will be included in location report provided to the NF consumer, i.e. AF2, but AF2 cannot handle it because it only recognizes GPSI-1; or

  - AMF provide GPSI-2 to the LMF (not follow the requirement), then the LMF will not detect the existing LCS-UPP connection. The LMF will try to establish new LCS-UPP connection towards the same UE and lead to uncertain result (mostly failed)

To resolve that, there are also two options:

Option-2.1 (Same as Option-1.1): To enforce that the SUPI shall be mandatory to use for the UE, if LCS-UPP feature is supported, with GPSI addition if requested by the AF. With this option, the LMF can always receive the SUPI of the UE when LCS-UPP feature is supported, thus the LMF can always successfully associate the subsequent location requests and the existing LCS-UPP connection for the same UE with the SUPI.

Option-2.2: To provide both GPSIs in the location request and indicate which one is used for LCS-UPP and which one is for the request. e.g. When GPSI-1 was used for LCS-UPP connection establishment, the AMF needs to provide both GPSI-1 (as the identify for LCS-UPP) and GPSI-2 (for the location request) to the LMF, which requires big impacts on the interface and AMF/LMF service logic.

Alternative 1 (using Option-1.1/2.1 resolves both problems) provide a consistent and reliable way for association of LCS-UPP connection and subsequent location requestion in the LMF, with no (least) impacts on the interfaces and AMF/LMF service logic.

Another alternative (taking both Option-1.2 and Option-2.2 together because they resolve different problems) requires decent impact on SBI interfaces and big impacts in AMF/LMF service logic (providing two GPSIs in one location request is very ugly protocol design).

# 3. Conclusion and proposal

It is recommended to adapt Option-1.1/2.1 as the way forward, i.e. when LCS-UPP connection feature is supported, the AMF always provide the SUPI in the location requests and/or service operations for uplink UPP-CM message towards the LMF. The LMF shall always use the SUPI to mark the LCS-UPP connection and associate subsequent location requests and LCS-UPP connection using the SUPI of the UE.

Related CR0380 of TS 29.572 and CR1266 of TS 29.518 to implement the conclusion.