3GPP TSG-CT WG4 Meeting #131	C4-254178

Sophia-Antipolis; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0379 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Default Notification Subscription for UP Notify |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-09-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For LCS-UPP connection, the AMF maintains the context for each LCS-UPP connection in the UE context, and the AMF needs to subscribe for UP Notify on the serving LMF for each LCS-UPP connection. When the UE moves from one AMF to another during mobility, the new AMF then needs to explicitly subscribe to UP Notify for all the connections and the old AMF needs to unsubscribe. As an improvement, the AMF may register the default notification subscription for UP Notify and the LMF can find the default notification callback URI to deliver the UP Notify to avoid all these explicit subscribe / unsubscribe service operations, as proposed in CR1250 of TS 29.510. Currently when AMF explicitly subscribes to LMF for UP Notify, the AMF provides a Notification correlation ID and will be provided by the LMF to the AMF in UP Notify to allow the AMF to determine which subscription the notification is correlated. For default notification subscription, the notification correlation ID is not available. To allow the AMF to determine which LCS-UPP connection context the UP NOTIFY, the LMF needs to provide additional information, i.e. the UE ID (SUPI) and the LMF Identification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Add UE ID and serving LMF ID in the UP Notification 2/ Add description on sending notification to default notification subscription 3/ Update OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMF cannot correlate the UP NOTIFY to the corresponding LCS-UPP connection, the default notification subscription cannot work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.8.2, 6.1.6.2.49, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 29.510 CR1250 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduce backward compatible feature to following APIs: TS29572_Nlmf_Location.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.8.2	Notification of LCS-UP connection

This procedure notifies the NF Service Consumer (i.e. AMF) about status or modification of a secure LCS-UP connection for a target UE.

Figure 5.2.2.8.2-1: UPNotify

1.	The LMF shall send a POST request to the upNotifyCallBackUri provided by the NF consumer in a previous UPConfig service operation (see clause 5.2.2.8) or UPSubscribe service operation (see clause 5.2.2.7). The request body shall include a notification correlation ID, LCS-UP connection status and may include a target LMF identifier.

If the upNotifyCallBackUri is not previously provided by the NF consumer explicitly, the LMF may discover the NF consumer (i.e. the serving AMF) in NRF and send a post request to the callback URI of the default notification subscription for Notification Type ("LCS_UP_NOTIFY") if registered by the NF consumer. The request body shall include the LCS-UP connection status, the UE id, the LMF identifier and may include a target LMF identifier.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, one of the appropriate HTTP status code listed in Table 6.1.5.2.3.1-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

* * * Next Change * * * *

##### 6.1.6.2.49	Type: UpNotifyData

Table 6.1.6.2.49-1: Definition of type UpNotifyData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notifCorrelationId | CorrelationID | C | 1 | This IE shall be present if the Notification Correlation ID was previously received from the NF consumer. When present, this IE shall carry the Notification correlation ID previously received from the NF consumer. |
| supi | Supi | C | 0..1 | This IE shall be present when the notification is sent to a default notification subscription (see clause 5.2.2.8.2) and when the SUPI of the UE was during the establishment of the LCS-UPP connection. When present, this IE shall indicate the SUPI of the UE. |
| gpsi | Gpsi | C | 0..1 | This IE shall be present when the notification is sent to a default notification subscription (see clause 5.2.2.8.2) and when the GPSI of the UE was during the establishment of the LCS-UPP connection. When present, this IE shall indicate the SUPI of the UE. |
| lmfId | LMFIdentification | C | 0..1 | This IE shall be present when the notification is sent to a default notification subscription (see clause 5.2.2.8.2). When present, this IE shall carry the LMF identifier of the serving LMF. |
| upConnectionStatus | UpConnectionStatus | M | 1 | UP Connection Status |
| targetLMFId | LMFIdentification | O | 0..1 | This IE may be present if the UpConnectionStatus is set to “MOVE”. When present, this IE shall indicate the Target LMF identifier. |

* * * Next Change * * * *

# A.2	Nlmf_Location API

openapi: 3.0.0

************************ Text Sipped for Clarity *****************

UpNotifyData:

description: UP Subscription

type: object

required:

- upConnectionStatus

properties:

notifCorrelationId:

$ref: '#/components/schemas/CorrelationID'

upConnectionStatus:

$ref: '#/components/schemas/UpConnectionStatus'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

lmfId:

$ref: '#/components/schemas/LMFIdentification'

targetLMFId:

$ref: '#/components/schemas/LMFIdentification'

************************ Text Sipped for Clarity *****************

* * * End of Changes * * * *