3GPP TSG-CT WG4 Meeting #131	C4-254194

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0916 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Populating Service-level-AA container to (h-)SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in the CT4 LS out to SA5, C4-253415, CT4 answered SA5 question: "The UE supporting UAS services provides CAA-level UAV ID included in the Service-level-AA container IE to the AMF/SMF during the registration procedure/PDU session establishment procedure as described in clause 4.22 of 3GPP TS 24.501, Authentication and authorization of UAV. Accordingly, the AMF/SMF can identify the UE is an UAV UE or requesting UAV services." And also according to the requirements in clause 5.2.3.2 of TS 23,256: The procedure assumes that the UE/UAV has already registered on the AMF. 0. Steps 1 - 5 as in TS 23.502 [3] figure 4.3.2.2.1-1. The UAV includes the Service Level Device Identity (e.g. the CAA-Level UAV ID of the UVA) and may include the Authentication Server Address (i.e. the USS address or the USS address for the corresponding geographical area based on the UAV current location) and optionally Authentication Data (i.e. the UUAA Aviation Payload) in the PDU Session Establishment request. The SMF determines that it needs to invoke UAS NF/NEF service operation for UUAA Authentication/Authorization of the PDU session establishment request based on that the provided DNN/S-NSSAI combination is dedicated for aerial services (have aerial service indicator set) and that the Service Level Device Identity (CAA-Level-UAV ID) is included in the request. If the provided APN/DNN is dedicated for aerial services but Service Level Device Identity (CAA-Level UAV ID) is not provided, the SMF shall reject the establishment of the PDU Session and steps 1 - 9 are not performed. The SMF identifies the UAS NF/NEF based on local configuration or by NF discovery procedure using DNN/S-NSSAI and/or UE provided identity e.g. USS address. The UUAA-SM procedure may be performed during the UE-requested PDU session establishment procedure, or a PDU session modification for the re-authentication purpose as well as a PDU session establishment/modification for C2 authorization/communication, therefore the Service-level-AA container is included in PDU Session Establishment Request/accept/reject messages, PDU session Modification Command/Request message, PDU session Release command, and Service-Level Authentication Comand/Complete messages.(See 24.501) The Service-level-AA container should be transferred transparently between the (H-)SMF and the I/V-SMF via n1SmInfoFromUe and n1SmInfoToUe IEs, for a Home Routed PDU session or a PDU session with I-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Service-level-AA container IE into the n1SmInfoFromUe and n1SmInfoToUe. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UUAA-SM procedure can not be properly performed and UAS service can not be offered for HR PDU session and a PDU session with I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR doesn't introduce any impact to openAPI files |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

##### 6.1.6.4.4	n1SmInfoFromUe, n1SmInfoToUe, unknownN1SmInfo

n1SmInfoFromUe, n1SmInfoToUe and unknownN1SmInfo shall encode one or more NAS SM IEs, including the Type and Length fields, as specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type.

Clause 5.2.3.1 specifies the information that shall be included in these payloads.

n1SmInfoFromUe and n1SmInfoToUe may encode the 5GS NAS IEs listed in tables 6.1.6.4.4-1 and 6.1.6.4.4-2.

Table 6.1.6.4.4-1: n1SmInfoFromUE content

| 5GS NAS IE | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Message type | 9.7 | All NAS SM messages |
| PDU session type | 9.11.4.11 | PDU Session Establishment Request |
| SSC mode | 9.11.4.16 | PDU Session Establishment Request |
| Maximum number of supported packet filters | 9.11.4.9 | PDU Session Establishment Request PDU Session Modification Request |
| Integrity protection maximum data rate | 9.11.4.7 | PDU Session Modification Request (NOTE 3) |
| SM PDU DN request container | 9.11.4.15 | PDU Session Establishment Request |
| Extended protocol configuration options | 9.11.4.6 | PDU Session Establishment Request PDU Session Authentication Complete PDU Session Modification Request PDU Session Modification Complete PDU Session Modification Command Reject PDU Session Release Request PDU Session Release Complete |
| EAP message | 9.11.2.2 | PDU Session Authentication Complete |
| Requested QoS rules | 9.11.4.13 | PDU Session Modification Request |
| Requested QoS flow descriptions | 9.11.4.12 | PDU Session Modification Request |
| 5GSM cause | 9.11.4.2 | PDU Session Modification Request PDU Session Release Request PDU Session Release Complete (NOTE 2) |
| 5GSM capability | 9.11.4.1 | PDU Session Establishment Request PDU Session Modification Request (NOTE 1) |
| Mapped EPS bearer contexts | 9.11.4.8 | PDU Session Modification Request |
| Remote UE context connected | 9.11.4.29 | Remote UE Report |
| Remote UE context disconnected | 9.11.4.29 | Remote UE Report |
| Non-3GPP delay budget | 9.11.4.37 | PDU Session Modification Request |
| URSP rule enforcement reports | 9.11.4.38 | PDU Session Establishment Request PDU Session Modification Request |
| Non-3GPP device identifier connection information | 9.11.4.41 | PDU Session Modification Request |
| Service-level-AA container | 9.11.2.10 | PDU Session Establishment Request PDU Session Modification Request Service-level Authentication Complete (NOTE X) |
| NOTE 1: The 5GSM capability IE shall be encoded as received from the UE. It may contain UE capabilities that the V-SMF (or I-SMF) only needs to transfer to the H-SMF (or SMF), e.g. support of reflective QoS, or support of multi-homed IPv6 PDU session, and/or capabilities to be interpreted and used by the V-SMF (or I-SMF). NOTE 2: The 5GSM cause IE shall be encoded as received from the UE. This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Modification Command Reject message, in which case it shall be sent as a separate n1SmCause IE over N16/N16a and not within the n1SmInfoToUE binary data. NOTE 3: This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Establishment Request, in which case it shall be sent as separate maximum integrity protected data rate IEs over N16/N16a and not within the n1SmInfoFromUE binary data. NOTE X: The Service-level-AA container included in the Service-level Authentication Complete message has no IEI defined, however, the (h-)SMF can derive it is Service-level-AA container using the length. |  |  |

Table 6.1.6.4.4-2: n1SmInfoToUE parameters

| 5GS NAS IE | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Message type | 9.7 | All NAS SM messages |
| RQ timer value | 9.11.2.3 | PDU Session Establishment Accept PDU Session Modification Command |
| EAP message | 9.11.2.2 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Authentication Command PDU Session Authentication Result PDU Session Release Command |
| Allowed SSC mode | 9.11.4.5 | PDU Session Establishment Reject |
| Extended protocol configuration options | 9.11.4.6 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Authentication Command PDU Session Authentication Result PDU Session Modification Reject PDU Session Modification Command PDU Session Release Reject PDU Session Release Command |
| 5GSM cause | 9.11.4.2 | PDU Session Establishment Accept PDU Session Modification Command (NOTE 1) |
| Mapped EPS bearer contexts | 9.11.4.8 | PDU Session Establishment Accept PDU Session Modification Command |
| ATSSS container | 9.11.4.22 | PDU Session Establishment Accept PDU Session Modification Command |
| N3QAI | 9.11.4.36 | PDU Session Establishment Accept PDU Session Modification Command (NOTE 2) |
| Service-level-AA container | 9.11.2.10 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Modification Command PDU Session Release Command Service-level Authentication Command (NOTE X) |
| NOTE 1: This IE indicates the 5GSM cause the H-SMF (or SMF) requires the V-SMF (or I-SMF) to send to the UE. The V-SMF (or I-SMF) shall transfer the received value to the UE without interpretation. This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Establishment Reject message, in which case it shall be sent as a separate n1SmCause IE over N16/N16a and not within the n1SmInfoToUE binary data. NOTE 2: N3QAI is configured in SMF based on the S-NSSAI and DNN for PIN service (see clause 5.44.3.3 of 3GPP TS 23.501 [2]). NOTE X: The Service-level-AA container included in the Service-level Authentication Command message has no IEI defined, however, the I/V-SMF can derive it is Service-level-AA container using the length. |  |  |

The Message Type shall be present and encoded as the first 5GS NAS IE in any n1SmInfoFromUe, n1SmInfoToUe and unknownN1SmInfo binary data, to enable the receiver to decode the 5GS NAS IEs.

NOTE:	The Information Element Identifier (see clause 11.2.1.1.3 of 3GPP TS 24.007 [8]) of a 5GS NAS IE uniquely identifies an IE in a given message.

* * * End of Changes * * * *