3GPP TSG-CT WG4 Meeting #131	C4-254211

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.504 | CR | 0336 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NF Group ID Map Result |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a discrepancy in the definition of NfGroupIdMapResult, between the table in clause 6.2.6.2.2 and the OpenAPI Annex: - In clause 6.2.6.2.2, it is defined as an object containing an attribute called "nfGroupIds" which, in turn, is defined as a map of NfGroupId - OpenAPI defines this type simply as a map of NfGroupId (i.e. the "nfGroupIds" attribute does not exist) According to 3GPP guidelines, the definition in OpenAPI takes precedence. However, given the blunt deviation between both definitions, there is a clear risk that implementors might take one or another approach, making implementations not inter-operable. The approach taken in this correction gives precedence to the OpenAPI definition, so there is no actual protocol change proposed by this CR, but it is proposed to apply the correction since Rel-16 in order to: - align both protocol definitions and avoid any risk of inter-operability - specify clearly (without ambiguity) how the payload of the GET response message is defined, to resolve any inter-operability dispute between vendors |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define the response body in clause 6.2.3.2.3.1 as a map, rather than referring to a explicit data type definition - Mark as "void" the definition of NfGroupIdMapResult type - Re-define the response body in OpenAPI as an "inlined" map rather than using the existing data type - Correct an unrelated editorial issue in clause 6.2.6.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API specification is contradictory between the MS Word tables and the formal OpenAPI definition, causing inter-operability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.3.1, 6.2.6.1, 6.2.6.2.2, 6.2.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections, with impacts on the following APIs: - TS29504_Nudr_GroupIDmap.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *First Change * * * *

6.2.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nf-type | array(NFType) | M | 1..N | see 3GPP TS 29.510 [14] |
| subscriberId | SubscriberId | M | 1 | Represents the Subscription Identifier SUPI or GPSI or IMPI or IMPU (see 3GPP TS 23.501 [4] clause 5.9.2 and clause 5.9.8) or Routing Indicator pattern: ^(imsi-[0-9]{5,15}¦nai-.+¦msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦impi-.+¦impu-.+¦rid-[0-9]{1,4}¦.+)$ |

NOTE 1:	The format of the query parameter subscriberId is in line with the yaml and thus does not follow the lower-with-hyphen format specified in 3GPP TS 29.501 [8].

NOTE 2:	If the UDR does not support a certain alternative in the regular expression pattern of the subscriberId query parameter (e.g. "rid-xxxx"), it returns an HTTP 404 error response.

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(NfGroupId) | M | 1..N | 200 OK | Upon success, a response body containing the NF Group IDs for the requested NF types shall be returned. The response shall contain a map (list of key-value pairs where NFType serves as key) of NfGroupIds. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be set to one of the following application errors: - USER_NOT_FOUND |
| NOTE: In addition, common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 are supported. |  |  |  |  |

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the structured data types defined for the Nudr_GroupIDmap service API. For simple data types defined for the Nudr_GroupIDmap service API see table 6.2.6.3.2-1.

Table 6.2.6.1-1: Nudr_GroupIDmap specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| RoutingIdResult | 6.2.6.2.3 | Routing Indicators for the requested NF type and NF Group ID |
| SubscriptionData | 6.2.6.2.4 | Information of a subscription to notifications to UDR GroupIDmap service, included in subscription requests and responses |
| GroupIdMapNotify | 6.2.6.2.5 | Data sent in notifications from UDR to entities subscribed to UDR GroupIDmap service |
| SubscriberId | 6.2.6.3.2 | Represents the Subscription Identifier SUPI or GPSI or IMPI or IMPU or Routing Indicator |

Table 6.2.6.1-2 specifies data types re-used by the Nudr_GroupIDmap service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_GroupIDmap service API.

Table 6.2.6.1-2: Nudr_GroupIDmap re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [10] | Common data type used in response bodies |
| SupportedFeatures | 3GPP TS 29.571 [10] |  |
| NFType | 3GPP TS 29.510 [14] |  |
| NfGroupId | 3GPP TS 29.571 [10] |  |
| IdentityRanges | 3GPP TS 29.571 [10] |  |

* * * Next Change * * * *

##### 6.2.6.2.2	Void

* * * Next Change * * * *

##### 6.2.6.2.3	Type: RoutingIdResult

Table 6.2.6.2.3-1: Definition of type RoutingIdResult

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| routingIndicators | array(string) | M | 1..N | An array of Routing Indicators. pattern (of each item): '^[0-9]{1,4}$' |

* * * Next Change * * * *

# A.3	Nudr_GroupIDmap API

(... text not shown for clarity ...)

responses:

'200':

description: >

Expected response to a valid request; it shall contain a map (list of key-value

pairs) where NFType is the key of the map

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

minProperties: 1

(... text not shown for clarity ...)

schemas:

# STRUCTURED TYPES:

(... text not shown for clarity ...)

* * * End of Changes * * * *