3GPP TSG-CT WG4 Meeting #131	C4-254222

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.369 | CR | 0005 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIoT support for T-IDs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-09-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA3 agreed in 3GPP TS 33.369 to store the Temporary ID (T-ID) in the AIoT devices profile. ### “5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory [..] - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile. [..]” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to include the T-ID and T-ID type in the AIoT device profile data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient IoT stage 3 specification not aligned with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.1, 6.1.6.2.2, 6.1.6.2.X (new), 6.1.6.3.2, 6.1.6.3.Y (new), A2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29369_AIoT_Data.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 9113: "HTTP/2".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 9457: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.369: " Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[15]	IETF RFC 9110: "http Semantics".

[16]	IETF RFC 9111: "HTTP Caching".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[33369]	3GPP TS 33.369: "Security aspects of Ambient IoT service for isolated private networks".

# *** Next Change ***

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nadm_DM service based interface protocol.

Table 6.1.6.1-1: Nadm_DM specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AiotDevProfileData | 6.1.6.2.2 | AIoT Device Profile Data |  |
| LastKnownAiotfInfo | 6.1.6.2.3 | Last Known AIoTF Information |  |
| IndividualAfAuthorizationData | 6.1.6.2.4 | Individual AF Authorization Data |  |
| AllowedTargetAiotDevice | 6.1.6.2.5 | Allowed Target AIoT Device |  |
| AfAuthorizationData | 6.1.6.2.6 | AF Authorization Data |  |
| LastKnownAiotfInfoInd | 6.1.6.3.2 | Last known AIOTF Information |  |
| AfId | 6.1.6.3.2 | AF ID |  |
| AllowedServiceOperation | 6.1.6.3.3 | Allowed Service Operation |  |
| TidData | 6.1.6.2.X | Temporary ID (T-ID) data |  |
| Tid | 6.1.6.3.2 | Temporary ID (T-ID) of the AIoT device |  |
| TidType | 6.1.6.3.Y | The handling type of the T-ID |  |

Table 6.1.6.1-2 specifies data types re-used by the Nadm_DM service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nadm_DM service based interface.

Table 6.1.6.1-2: Nadm_DM re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [17] | see 3GPP TS 29.500 [4] clause 6.6 | SupportedFeatures |
| AiotDevPermId | 3GPP TS 29.571 [17] | AIoT device permanent identifier |  |
| AiotArea | 3GPP TS 29.571 [17] | Contains the AIoT Area |  |
| NfInstanceId | 3GPP TS 29.571 [17] | Represent a NF instance. |  |
| IpAddr | 3GPP TS 29.571 [17] | Represent the IP address |  |
| Fqdn | 3GPP TS 29.571 [17] | Represent Fully Qualified Domain Name |  |
| ProblemDetails | 3GPP TS 29.571 [17] | Used in error responses to provide more detailed information about an error. |  |

# *** Next Change ***

##### 6.1.6.2.2	Type: AiotDevProfileData

Table 6.1.6.2.2-1: Definition of type AiotDevProfileData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aiotDevPermId | AiotDevPermId | M | 1 | Globally unique AIoT device permanent identifier |  |
| lastKnownAiotfInfo | LastKnownAiotfInfo | O | 0..1 | Indicate the last known AIOTF that serves the AIoT device, or unknown. |  |
| tidData | TidData | O | 0..1 | Temporary ID (T-ID) Data |  |

# *** Next Change ***

##### 6.1.6.2.X	Type: TidData

Table 6.1.6.2.X-1: Definition of type TidData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tidType | TidType | O | 0..1 | The handling type of the T-ID as specified in TS 33.369 [33369]. |  |
| tidCurrent | Tid | O | 0..1 | Current Temporary Identifier (TIDn) |  |
| tidPrevious | Tid | O | 0..1 | Previous Temporary Identifier (TIDn-1) |  |

# *** Next Change ***

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AfId | string | Identifier used to identify the AF. |  |
| Tid | string | pattern: "^[A-Fa-f0-9]{32}$" |  |

# *** Next Change ***

##### 6.1.6.3.Y	Enumeration: TIdType

Table 6.1.6.3.Y-1: Enumeration TidType

| Enumeration value | Description |
| --- | --- |
| "CONCEALED_FROM_STORED" | Concealed type derived from stored T-ID |
| "CONCEALED_FROM_PERMANENT" | Concealed type derived from AIoT device permanent identifier |
| "STORED_NO_UPDATE" | Stored type, no update required |
| "STORED_UPDATE_WITH_COMMAND" | Stored type, update with command |
| "STORED_UPDATE_NO_COMMAND" | Stored type, update with inventory |
| "NONE" | No T-ID is used, permanent ID is used directly |

# *** Next Change ***

# A.2	Nadm_DM API

openapi: 3.0.0

info:

version: '1.0.0-alpha.3'

title: 'Nadm_DM'

description: |

Nadm Data Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.369 Ambient IoT Data Management Services, version 0.3.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.369/'

servers:

- url: '{apiRoot}/nadm-dm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nadm-dm

- {}

paths:

/aiot-device-profile-data/{aiotDevPermId}:

get:

summary: get AIoT Device Profile Data

operationId: Get AIoT Device Profile Data

tags:

- AIoT Device Profile Data Retrieval

parameters:

- name: aiotDevPermId

in: path

description: AIoT Device Permanent ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AiotDevProfileData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify AIoT Device Profile Data

operationId: Modify AIoT Device Profile Data

tags:

- AIoT Device Profile Data Update

parameters:

- name: aiotDevPermId

in: path

description: AIoT Device Permanent ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AiotDevProfileData'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/af-authorization-data:

get:

summary: get AF Authorization Data

operationId: Get AF Authorization Data

tags:

- AF Authorization Data Retrieval

parameters:

- name: af-id

in: query

description: AF ID

required: false

schema:

$ref: '#/components/schemas/AfId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AfAuthorizationData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nadm-dm: Access to the Nadm_DM API

nadm-dm:aiot-device-profile-data:read: Access to read AIoT Device Profile Data

nadm-dm:aiot-device-profile-data:modify: Access to update AIoT Device Profile Data

nadm-dm:af-authorization-data:read: Access to read AF Authorization Data

schemas:

#

# STRUCTURED TYPES

#

AiotDevProfileData:

description: Contains the AIoT Device Profile Data.

type: object

required:

- aiotDevPermId

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

lastKnownAiotfInfo:

$ref: '#/components/schemas/LastKnownAiotfInfo'

tidData:

$ref: '#/components/schemas/TidData'

description: >

Contains Temporary ID Data.

LastKnownAiotfInfo:

description: Contains the Last Known AIoTF Info.

type: object

required:

- lastKnownAiotfInfoInd

properties:

lastKnownAiotfInfoInd:

type: boolean

lastKnownAiotfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

lastKnownAiotfAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

lastKnownAiotfFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

IndividualAfAuthorizationData:

description: Contains the AF Authorization Data for an invidual AF.

type: object

required:

- afId

properties:

afId:

$ref: '#/components/schemas/AfId'

allowedArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

allowedServiceOperations:

type: array

items:

$ref: '#/components/schemas/AllowedServiceOperation'

allowedTargetAiotDevices:

type: array

items:

$ref: '#/components/schemas/AllowedTargetAiotDevice'

AllowedTargetAiotDevice:

description: Contains the permanent AIoT Device ID or the filtering information

type: object

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

AfAuthorizationData:

description: Contains the AF Authorization Data.

type: object

required:

- afAuthData

properties:

afAuthData:

type: object

description: >

'Map carrying the AF Authorization Data.

The key of the map is the AF ID of the corresponding Authorization Data'

additionalProperties:

$ref: '#/components/schemas/IndividualAfAuthorizationData'

minProperties: 1

TidData:

description: Contains the Temorary ID (T-ID) Data.

type: object

properties:

tidType:

$ref: '#/components/schemas/TidType'

description: >

Contains the handling type of the Temporary ID.

tidCurrent:

$ref: '#/components/schemas/Tid'

description: >

Contains the currect Temporary ID of the AIoT device.

tidPrevious:

$ref: '#/components/schemas/Tid'

description: >

Contains the previous Temporary ID of the AIoT device.

#

# SIMPLE TYPES

#

AfId:

description: Indicates the identifier used to identify the AF.

type: string

Tid:

description: Temporary ID of the AIoT device.

type: string

pattern: '^[A-Fa-f0-9]{32}$'

#

# ENUMS

#

AllowedServiceOperation:

description: The Allowed Service Operation.

anyOf:

- type: string

enum:

- INVENTORY

- READ

- WRITE

- PERMANENT_DISABLE

- type: string

TidType:

description: The type of the Temporary ID.

anyOf:

- type: string

enum:

- CONCEALED_FROM_STORED

- CONCEALED_FROM_PERMANENT

- STORED_NO_UPDATE

- STORED_UPDATE_WITH_COMMAND

- STORED_UPDATE_NO_COMMAND

- NONE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

# *** End of Changes ***