3GPP TSG RAN Meeting #109                                 RP-252093

Beijing, China, Sep 15-18, 2025

Agenda Item:	8.2.1.3

Source:	CMCC

Title:	Consideration on 6G service-based RAN

Document for:	Discussion, Decision

1	Introduction

5G revolutionarily adopts the service-based architecture (SBA) as the foundation of the core network, enabling independent scaling, independent evolution, and on-demand deployment of network functions, while continuously driving the enhancement and optimization of service-based functions and frameworks. Based on SBA, differentiated network slices can be flexibly designed to generate dedicated networks for various industries as needed.

Looking ahead to 2030, on one hand, new capabilities such as sensing, computing, and AI will be introduced as network functions (NFs) and NF services; on the other hand, the core network UPF, RAN, and even UE will be service-oriented, becoming both producers and consumers of services.

In this contribution, we would like to provide our thinking on the 6G service-based RAN.

2	Discussion

2.1	Motivation from integrated gNB to service-based RAN

From the perspective of 5G ToB implementation, industry applications often require networks to provide customized capabilities in computing, AI, communications, and more. However, due to the strong coupling between integrated BBU functions, industry-specific features cannot be agilely embedded within the base station itself. Typically, this is achieved by attaching industry-specific functions externally to the base station, which severely impacts the performance of industry-specific networks. For example, some industry scenarios require the network to provide guarantees of 4ms@99.99% or even 1ms@99.99%, but the current capabilities of 5G networks can only achieve 10ms@99.99%. The external attachment of functions fails to meet the performance requirements of these industries. Additionally, the accumulation of multiple functions results in high implementation costs for industry-specific networks, where customer investment does not yield proportional gains, leading to low deployment willingness.

Observation 1: Currently, the high degree of coupling between integrated base station functions makes it difficult to introduce new features and limits flexible adaptation to differentiated and customized scenario requirements.

Service-based RAN will help address the issues mentioned above. Unlike an integrated gNB, service-based RAN is defined by two key characteristics:

a) Highly cohesive and loosely coupled functional services: this ensures that each function can be reused independently and flexibly across diverse scenarios.

b) Lightweight API-based interactions: the APIs can be invoked by any consumer, including newly introduced NFs, so new NFs can be deployed more rapidly.

Proposal 1: Support service-based RAN for 6G to address the adaptation challenges of RAN to differentiated and customized scenario requirements.

2.2	Consideration on 6G service-based RAN

In order to reduce the complexity of network operation and maintenance management, some design principles of the 5G core network service-based architecture [1] should be extended to the service-based RAN, such as:

-	Separate the User Plane (UP) functions from the Control Plane (CP) functions, allowing independent scalability, evolution and flexible deployments e.g. centralized location or distributed (remote) location.

-	Modularize the function design, e.g. to enable flexible and efficient network slicing.

-	Wherever applicable, define procedures (i.e. the set of interactions between network functions) as services, so that their re-use is possible.

-	Enable each Network Function and its Network Function Services to interact with other NF and its Network Function Services directly or indirectly via a Service Communication Proxy if required.

-	Support "stateless" NFs, where the "compute" resource is decoupled from the "storage" resource.

-	Support capability exposure.

- Proposal 2: The fundamental principles of service-based design in the core network should also be applied to the RAN.

5G NR achieves the separation of CU-CP and CU-UP, with the primary goal of addressing diverse and differentiated service requirements, enabling more flexible networking and more efficient resource management. After the separation, the CU-CP can be centrally deployed to manage a large number of connections and signaling, while the CU-UP can be deployed locally (e.g., providing low-latency guarantees for uRLLC services) or centrally (e.g., conserving transmission resources for eMBB services) based on business needs. For future application scenarios, such differentiated functional or deployment requirements will become even more pronounced. Therefore, we believe that the 6G RAN architecture will still need to support the separation of CP and UP functions.

- Proposal 3: Separate the UP functions from the CP functions should be supported in 6G.

Base stations have evolved over decades in a monolithic integrated manner, with tightly coupled logical functions, leading to issues such as complex RRC structures and difficulties in introducing new user plane features. To reduce the complexity of network design and operation, it is necessary to modularize RAN functions. Only by building modular functions with high cohesion and loose coupling can functional reuse be maximized. Therefore, we believe modularization is an essential step toward achieving a service-based RAN.

Given that the 6G core network will highly likely define new 6G capabilities - such as computing, sensing, and AI - as independent Network Functions (NFs) and NF services, we argue that these capabilities should also be introduced on the RAN side as independent NFs and NF services. On one hand, this will better support coordination between RAN and CN capabilities; on the other hand, it will avoid coupling between new capabilities and traditional communication functions, enabling independent evolution of features.

- Proposal 4: Modularize the function design and define at least an independent RAN NF to support new capabilities such as computing and AI.

Services are defined based on external requirements. For the RAN, requirements can originate from CN NFs (such as AMF/UPF), RAN nodes (such as gNB/CU-CP/CU-UP/DU), or from the UE. Relevant interface protocols include NGAP, XnAP, E1AP, F1AP, PDU Session user plane protocol, and NR user plane protocol, among others. While redefining all these interface procedures as services is unlikely, it is feasible to start with NGAP procedures by defining (some) NGAP procedures as services before gradually considering the servitization of other interfaces.

- Proposal 5: For the RAN service definition, (some) NGAP procedures can be defined as services first.

Exposure of information is often one of the earliest services provided by a network element. Through an Exposure Service, the RAN can expose structured information (e.g., UE-related information) to other core network NFs as a capability. In addition to exposing information to internal network NFs, RAN information can also be exposed to third-party applications via the NEF to facilitate the optimization of business applications. Considering that third-party applications may require faster interaction with the RAN or need more timely information from the RAN (such as wireless channel state information), direct RAN capability exposure without NEF involvement also warrants further study.

As mentioned earlier, new capabilities such as computing and AI in the RAN are best implemented through independent NFs and NF services. These new services may include:

- AI-related services: AI task control services can invoke connectivity, computing, and data services to execute relevant AI tasks. For example, the AI task control function can interact with data collection services to acquire UE-related data and collaborate with connectivity functions to establish connections between the UE and the AI task control service. This enables the UE to serve as an execution node for AI tasks, providing services such as AI model training or inference. AI model management services offer functionalities such as model registration, model selection, and model performance monitoring.

- Data-related services: Existing networks primarily function as data transmission pipelines and cannot meet the end-to-end data processing requirements of future AI applications, which involve data collection, transmission, processing, and storage. Introducing data collection services on the RAN side can effectively address the demands of AI for large-volume, real-time data while avoiding performance impacts on other functional services due to data collection. Data storage services, evolving from UDR/UDSF, can enable more reliable and efficient data storage and read/write operations. Data processing services provide functions such as de-privatization processing and data refinement.

- Computing-related services: Computing control services offer functionalities such as computing resource awareness and allocation, computing node selection, and computing deployment/ suspension/ resumption/ termination. Computing execution services provide capabilities for AI model training or AI model inference.

- Proposal 6: Define at least capability exposure, computing control and execution, data collection and handling as RAN services.

As of now, 3GPP has defined four communication models between services. Model A and Model B are direct communication models, in which the service consumer performs service selection based on local configuration or service producer information (NF profile) obtained from the NRF. Model C and Model D are indirect communication models based on the Service Communication Proxy (SCP). In these models, each NF only needs to focus on its own business processing logic, while delegating functions such as service discovery and routing to the SCP, making the NF more lightweight. To ensure efficient interworking between RAN NFs and CN NFs, the communication model on the RAN side must align with that on the core network side.

- Proposal 7: Communication models on the RAN side need to align with those of the CN.

Statelessness is crucial for enhancing network scalability, resilience, and resource utilization. Through stateless design, the state management of an NF is decoupled from its business logic processing. Since any instance can handle any request, horizontal scaling (scale-out/in) can be achieved by increasing or decreasing the number of service instances. If a service instance crashes, no unique state data is lost. New requests can be routed to other healthy instances, which can reconstruct the context from external storage, enabling rapid failure recovery. However, current RAN does not possess dynamic scaling capabilities.

- Proposal 8: Support "stateless" NFs to enhance the scalability and resilience of the RAN.

3	Proposal

Based on the discussion in the previous sections we propose the following:

Observation 1: Currently, the high degree of coupling between integrated base station functions makes it difficult to introduce new features and limits flexible adaptation to differentiated and customized scenario requirements.

Proposal 1: Support service-based RAN for 6G to address the adaptation challenges of RAN to differentiated and customized scenario requirements.

- Proposal 2: The fundamental principles of service-based design in the core network should also be applied to the RAN.

- Proposal 3: Separate the UP functions from the CP functions should be supported in 6G.

- Proposal 4: Modularize the function design and define at least an independent RAN NF to support new capabilities such as computing and AI.

- Proposal 5: For the RAN service definition, (some) NGAP procedures can be defined as services first.

- Proposal 6: Define at least capability exposure, computing control and execution, data collection and handling as RAN services.

- Proposal 7: Communication models on the RAN side need to align with those of the CN.

- Proposal 8: Support "stateless" NFs to enhance the scalability and resilience of the RAN.

4	References

TS 23.501, System architecture for the 5G System (5GS)