3GPP TSG RAN Meeting #109							    RP-252458

Beijing, China, September 15-18, 2025

Source:	Qualcomm Incorporated

Title:	Views on Ambient IoT SI in Rel-20

Agenda Item:	9.2.3

Document for: 	Discussion and Decision

# Background

In this document, we discuss following aspects of Rel-20 A-IoT.

- Positioning/Proximity determination

- Max target distance for active device

# Positioning/proximity for Rel-20 SI

## Positioning

Positioning has been an important case in Ambient-IoT case study in SA1[1]. However, it has not been included in the scope of Rel-20. In RAN#108, it was concluded that the SID for Rel-20 Ambient IoT capture following statement on positioning [2] expecting further discussion on positioning and decision during RAN1#109 about its inclusion in SI scope.

2. RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item

Rel-18 RAN level study Ambient IoT study, captured in TR 38.848[3], includes following representative use cases rUC3 and rUC7 as positioning use case.

| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide |
| --- | --- |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |

We think identified use cases are important and interesting use cases which could be addressable by Rel-20 active devices.

Along with use case categorization, TR 38.848[3] captures following target accuracy.

The design target of absolute positioning accuracy when performed by the cellular network (including assisting nodes when present) is: - 1~3 meters @ 90% indoor location. - Several tens of meters @ 90% outdoor location.

At this point, whether such requirements could be met or not may depend on scenarios/techniques being considered. We think following aspects should be considered.

- Given very limited time for Rel-19 SI, only simple and basic techniques, such as single point ranging based on measurement of e.g., RSRP, RTT, etc, should be considered.

- Feasible target accuracy should be decided considering low complexity of potential techniques.

- The considered techniques should be applied to both T1 and T2 and be preferred to be transparent to devices.

Observation 1: Following aspects needs to be considered in the study of positioning techniques for Device 2b and C in Rel-20 SI.

- Considering the limited time in the Rel-20 SI, only simple techniques such as single-point ranging based on e.g., RSRP, RTT etc. is to be considered.

- Accuracy target is to be decided accordingly.

- Techniques should be applied to both T1 and T2 and preferred to be transparent to the device.

## Proximity determination

TR 38.769 captures the study on proximity determination for A-IoT device [4].

### 6.9.4 Proximity determination Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item. Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader. Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side. Proximity determination based on device-side measurements is not considered.

Unlike positioning, proximity determination is a binary determination for the relative distance between the reader and A-IoT devices. It is useful to identify one device very close to the reader, e.g., a few tens of centimetres or up to one meter, as similar as NFC (near field communication) use cases supported either by a handheld UE/reader in T2 scenario or by a fixed BS/reader in T1 scenario, e.g., to detect badge at the gate or to charge fee at the charging station.

For passive device 1 and active device 2b/C, the transmit power and the antenna penalty could be very different. The reader needs to know the transmit power/antenna penalty per device type to estimate the path loss based on the D2R power measurement/detection as the solution 2. While solution 1 does not require path loss estimation, it is based on R2D/D2R power reduction. Reducing R2D power can be challenging, particularly for the BS/reader. To control D2R power variation, it may also be necessary to adjust or manage the CW power for device 1 D2R backscattering, or to utilize power control for active device 2b/C.

For active device 2b/C, measuring the R2D signal strength, such as RSRP, could be possible. The reader may indicate an RSRP threshold, allowing the device to determine whether it is within the proximity range.

Observation 2:

- For passive Device 1 and active Device 2b/C, solution1/2 in TR38.769 are feasible for proximity determination. However, further concerns must be addressed before considering the schemes to be adopted in Rel-20 WI:

  - For solution 1, it may not be easy/accurate to reduce/control the R2D/D2R transmit power, due to reader hardware limitations, to limit the proximity range, e.g., a few tens of centimeters or up to one meter.

  - For solution 2, the reader needs to be aware of the transmit power/antenna gain penalty associated with each device type, and to use D2R measurement/detection for proximity determination.

- For active Device 2b/C, the measurement of R2D signal strength by the device may be utilized, and the reader configures the RSRP threshold to determine device proximity.

Based on the discussion in Section 2.1 and Section 2.2, we make the following proposal.

Proposal 1: Update SID scope as follows.

RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios …. - Define link budget calculation for coverage. - Study the feasibility of positioning/proximity techniques for Device 2b/C considering - Simple single-point ranging like technique e.g., based on RSRP (including reader side or device side measurement), RTT measurements, etc. - Target accuracy to be decided accordingly - Applicability to both T1 and T2 RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. - RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item

# Max target distance for active device

SID for Rel-20 A-IoT study [2] capture following guideline on maximum distance between Reader and Device.

RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

The SID description indicates to include an achievable cell edge data rate assuming the maximum distance between Reader and Device of 50 to 500m, which indicates that system design should target the range between 50 to 500m. But we have seen in RAN1#122 that there were a few companies considering much larger target distance >>500m in the outdoor scenarios. This is clearly not in line with current SID guidelines and could require much more RAN1 efforts and potentially result in a quite different design than what the current scope is implying.

Proposal 2: RAN confirms that the target distance between Reader and Device 2b/C for Rel-20 study for outdoor scenario is to be decided between 50 and 500m. Don’t consider a larger range. Independently, the type of energy harvesting may further limit the achievable range of operation.

# Conclusion

In this contribution, we made following proposals.

Observation 1: Following aspects needs to be considered in the study of positioning techniques for Device 2b and C in Rel-20 SI.

- Considering the limited time in the Rel-20 SI, only simple techniques such as single-point ranging based on e.g., RSRP, RTT etc. is to be considered.

- Accuracy target is to be decided accordingly.

- Techniques should be applied to both T1 and T2 and preferred to be transparent to the device.

Observation 2:

- For passive Device 1 and active Device 2b/C, solution1/2 in TR38.769 are feasible for proximity determination. However, further concerns must be addressed before considering the schemes to be adopted in Rel-20 WI:

  - For solution 1, it may not be easy/accurate to reduce/control the R2D/D2R transmit power, due to reader hardware limitations, to limit the proximity range, e.g., a few tens of centimeters or up to one meter.

  - For solution 2, the reader needs to be aware of the transmit power/antenna gain penalty associated with each device type, and to use D2R measurement/detection for proximity determination.

- For active Device 2b/C, the measurement of R2D signal strength by the device may be utilized, and the reader configures the RSRP threshold to determine device proximity.

Proposal 1: Update SID scope as follows.

RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios …. - Define link budget calculation for coverage. - Study the feasibility of positioning/proximity techniques for Device 2b/C considering - Simple single-point ranging like technique based on e.g., RSRP (inc. reader side or device side measurement), RTT measurements, etc. - Target accuracy to be decided accordingly - Applicability to both T1 and T2 RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. - RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item

Proposal 2: RAN confirms that the target distance between Reader and Device 2b/C for Rel-20 study for outdoor scenario is to be decided between 50 and 500m. Don’t consider a larger range. Independently, the type of energy harvesting may further limit the achievable range of operation.

# Reference

- TR22.840 Study on Ambient power-enabled Internet of Things

- RP-251884 New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

- TR38.848 Study on Ambient IoT (Internet of Things) in RAN

- TR38.769 Study on solutions for Ambient IoT (Internet of Things) in NR