3GPP TSG-RAN Meeting #109	RP-252586

# Beijing, China, September 15-18, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | 0062 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.941: Rel-19 CLTA-related agreements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, ZTE, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on technical content Endorsed during RAN4#116 in R4-2512670, in this Company CR we provide formal CR to properly close Core objective of the NR_BS_RF_req_evo WI (i.e., OTA test enhancement: Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition.). This is done in parallel to the NR_BS_RF_req_evo WID revision, which is fixing Core/Perf aspects of the impacted specification (TR 37.941), as previously TR 37.941 was marked as belonging to Perf part of NR_BS_RF_req_evo WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 6.4.4 with Rel-19 CLTA-related agreements for OTA co-location requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 CLTA-related agreements for OTA co-location requirements would not be properly captured within Core deadline of Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Technical content is the same as in R4-2512670, which was Endorsed in RAN4#116. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 6.4	Co-location measurements

### 6.4.1	General

Co-location measurements are required for the OTA co-location requirements which are based on assuming the BS is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

CLTA is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. Basic principle of the CLTA are outlines in clause 6.4.2. For more details on the co-location test antenna characteristics, refer to TS 37.145-2 [4] clause 4.15.2.2, or TS 38.141-2 [6] clause 4.12.2.2. The CLTA is suitable for testing BS implemented with a planar antenna array.

The alignment between the BS under test and the co-location test antenna is depicted in figure 6.4.1-1. For more details on the co-location test antenna alignment and test arrangement, refer to TS 37.145-2 [4] clause 4.15.2.3, or TS 38.141-2 [6] clause 4.12.2.3. The same physical alignment applies to in-band and out-of-band co-location requirements.

Co-location requirements are not applicable to FR2.

Figure 6.4.1-1: Alignment of BS and CLTA

### 6.4.2	Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of BS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such that a suitable practical BS antenna can be found for each BS.

The CLTA is hence defined as follows:

-	The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the BS composite antenna ±30 %.

-	For in-band requirements, the CLTA supports the same frequency range and polarizations as the BS composite antenna.

-	For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3 °.

-	The polarization should match the in-band CLTA polarization.

-	The CLTA has a horizontal half-power beamwidth of 65 ° ± 10 °.

-	The front faces of the BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction.

-	The centre of the vertical radiating regions of the CLTA and the BS composite antenna need to be aligned.

### 6.4.3	Standard test antenna

Co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is not covered by the present release of this technical report.

### Co-location measurements for higher frequency bands

#### Background on CLTA alternatives discussion

Rel-19 work item on NR base station RF requirement evolution for FR1/FR2 and testing (NR_BS_RF_req_evo work item) has captured one objective related to the OTA co-location reference antenna. One of the triggers for this work, was concern on the availability of suitable CLTA test antennas for operating bands in the upper region in FR1 frequency range. Therefore, it was seen beneficial to further investigate alternatives of the co-location test antennas, with the assumption that Rel-15 definition of the co-location scenario based on co-location reference antenna and conformance test based on co-location test antennas is still the baseline.

- Investigation of the core definition and the co-location reference antenna relates to if the scenarios defined and the co-location reference antenna definition still represent a reasonable (worst case) scenario for high frequency FR1 bands.

- Investigation into the conformace and the CLTA relates to the practical test methodology of the test antenna definition, its suitability and its availability.

Furthermore, the work on CLTA alternatives was backed by the statement already captured in the TS 38.141-2 [6] and TS 37.145-2 [4] specification, saying:

“Translation of the requirements to other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified MU.”

Core objective of the NR_BS_RF_req_evo work item aimed to investigate whether the BS/IAB OTA co-location reference antenna definition required to be improved for FR1, and to investigate if solutions alternative to the CLRA are available. Coformance-related objective was to investigate whether the CLTA-based test approach could be improved, and potentially improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1. Conformance testing methodology was investigated to explore possible CLTA alternatives.

#### BS-to-BS isolation analysis

The BS-to-BS isolation between two co-located base stations have been evaluated from OTA measurement and OTA co-loation simulation results. The coupling loss have been evaluated for intra-band and inter-band scenarios, considering the coupling loss as sub-Array-to-Sub-Array (SA-to-SA) coupling loss, Array-to-Sub-Array (A-to-SA) coupling loss and Array-to-Array (A-to-A) coupling loss.

Coupling loss for different scenarios is listed in Table 6.4.4.2-1 and Table 6.4.4.2-2.

Table 6.4.4.2-1: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical Separation | SA-to-SA | M: 54 | M: 53 | M: 56 |  |
| Horizontal Separation | SA-to-SA | M: 31 | M: 34 | M: 46 |  |
|  | A-to-SA |  | M: 35 S: 55±15 | M: 51 M: 69 | M: 55 S: 70±15 |
|  | A-to-A |  |  |  | S: 77±15 |
| M: measurement results S: simulation results. |  |  |  |  |  |

Table 6.4.4.2-2: Inter band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |
| --- | --- | --- |
|  |  | 3.5 GHz to 7 GHz |
| Horizontal separation | A-to-SA | S: 57±10 |
|  | A-to-A | S: 54±10 |
| M: measurement results S: simulation results. |  |  |

The outcome of coupling loss evaluation indicates that the assumption taken from the conducted co-location scenario where BS-to-BS port coupling loss of 30 dB for BS type 1-C at high FR1 frequencies is not relevant anymore.

From the combined result from measurements and simulations it can be concluded that the general trend indicates coupling loss increase for bands in the upper region of FR1.

#### Impact on the co-location requirements

#### 6.4.4.3.1	General

The co-location requirements are requirements which are based on a given BS-to-BS co-location scenario, for BS type 1-C and 1-H it is assumed that this scenario has an MCL of 30dB. The intention with these requirements is to ensure that BSs serving different bands can be co-sited at the same site and/or tower.

Table 6.4.4.3.1-1: Overview of NR co-location requirements

| Requirement | Reference to clauses in NR BS specification TS 38.104 | Description |
| --- | --- | --- |
| Transmitter spurious co-location emission | BS type 1-C and BS type 1-H in Clause 6.6.5.2.4. BS type 1-O in Clause 9.7.5.2.5. | This requirement is defined to protect BS operating in other 3GPP band(s) co-located at the same site. |
| Receiver out-of-band blocking | BS type 1-C and BS type 1-H in Clause 7.5.3. BS type 1-O in Clause 10.6.2.2. | This requirement is defined to guarantee UL operation when a BS is co-located to another BS operating at another 3GPP band(s). |
| Transmitter OFF power | BS type 1-C in Clause 6.4.1.2. BS type 1-H in Clause 6.4.1.3. BS type 1-O is Clause 9.5.2.2. | This requirement is defined to protect other BSs in the same network in neighbouring cells. |
| Transmitter intermodulation | BS type 1-C in Clause 6.7.2. BS type 1-H in Clause 6.7.3. BS type 1-O in Clause 9.8.2. | This requirement guarantees transmitter robustness with respect to an external interferer. |

Originally all requirements were derived for non-AAS BS (BS type 1-C). In Rel-13, hybrid AAS BS was introduced (In NR specifications referred to as BS type 1-H) and in Rel-15 all OTA AAS BS was introduced (In NR specifications referred to as BS type 1-O). For BS type 1-C and BS type 1-H, all requirements are defined at the transceiver ports (also known as the Transceiver Array Boundary (TAB) connector(s)). For BS type 1-O all co-location requirements are defined based on a concept of a co-location reference antenna. In conformance testing a practical implementation of this antenna is defined and is called as the Co-Location Test Antenna (CLTA).

Looking at the details related to the technical background for each individual requirement it can be noticed that the BS-to-BS coupling loss assumed is of different type depending on requirement. In Table 6.4.4.3.1-2, some further details for each requirement are presented.

Table 6.4.4.3.1-2: Requirement derivation assumptions

| Requirement | Frequency aspect | Description |
| --- | --- | --- |
| Transmitter spurious emission | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking related to induvial receiver branches, A-to-SA is considered when the BS type 1-C and 1-H requirement level is derived. The BS type 1-O requirement is based on receiver sensitivity so whilst the means of interference to each receiver branch is from A-SA the requirement is based on the A-A system performance |
| Transmitter OFF power | Intra band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on to neighbouring BS and not side-by-side. |
| Transmitter intermodulation | Intra band | This requirement is defined to guarantee than unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to induvial transmitter branches, A-to-SA is considered when the BS type 1-C and 1-Hrequirement level is derived. The BS type 1-O requirement is based on TRP emissions so whilst the means of interference to each Tx branch is from A-SA coupling the requirement is based on the spurious output from the entire array. |

It is worth to notice that the analysising BS-to-BS coupling for all requirements listed in Table 6.4.4.3-2, may require a numer of different scenarios A complete technical background for all requirements would require a coupling evaluation including the following cases:

- Intra band A-to-A for relevant band combinations

- Inter band A-to-SA for relevant band combinations

- Intra band A-to-A for relevant bands considering coupling to neighbouring BS

- Intra band A-to-SA for relevant bands

The relevance of each of the assumed MCL values to each of the BS types and how they are used in deriving the core and conformance requirements also needs to be clearly stated. Analysing the results presented in Table 6.4.4.2-1 and Table 6.4.4.2-2 would indicate that assuming BS-to-BS port coupling loss equal to a value larger than 30 dB would be reasonable for bands in the upper region of FR1.

In the following clauses, impact on individual requirements is further elaborated.

#### 6.4.4.3.2	Transmitter OFF power requirement

FR1 NR BS transmitter OFF power is defined in terms of PSD in 1MHz and derived according to the following approach:

Transmit OFF power < kT0 + NF – 6dB + MCL + 10log(1MHz)

< -174 + 5 – 6 + MCL+60

< -115dBm/MHz + MCL

Considering the worst case is that BSs are assumed to be co-located, and for BS type 1-O the requirement is specified at the output of the CLRA the minimum coupling loss between co-located BSs would have no impact on the definition of core requirement. For the existing BS type 1-C the transmitter OFF power -85dBm/MHz, is specified at the antenna connectors and it is based on the assumption of MCL between two co-located BS as 30 dB so the core requirement would be affected by a change in the minimum coupling loss assumption.

#### 6.4.4.3.3	Co-location spurious emission requirement/Protection of BS receiver of own or different BS

The protection of own or different BS is based on desensitizing a victim receiver by 0.8dB with the assumption the victim receiver has the same performance as the system under test.

For BS type 1-O the interference power level is specified at the co-located antenna output and is given by:

Spurious Emission LevelWA_1-O ＜ kT0 + NF + 10*log10(100kHz) －7dB

= -174dBm/Hz + 5dB + 50dB－7dB = -126dBm

Note the figure in the requirement is scaled based on 8 TRX so is actually -126 +9 = -117dBm

The Spurious Emission Level of BS type 1-C si specified at the antenna port and is adjusted by the MCL:

Spurious Emission LevelWA_1-C ＜ kT0 + NF + 10*log10(100kHz) －7dB + MCL

= -174dBm/Hz + 5dB + 50dB－7dB + MCL

Where:

- MCL refer to the minimum coupling loss between co-located BSs, therefore the BS-to-BS isolation analysis would have the impact on the definition of the BS type 1-C co-location spurious emission requirement/Protection of BS receiver of own or different BS. MCL is assumed as 30dB in the existing specification.

- The BS of different classes will have different Noise figures. Generally, in most 3GPP spec, it is assumed that the    WA NF = 5dB, MR NF = 10dB, and LA NF = 13dB.

#### 6.4.4.3.4	Transmitter intermodulation requirement

For FR1 transmitter intermodulation requirement, interfering signal level is equal to Rated total output power (Prated,t,AC) in the operating band. For BS type 1-O it is specified at the input to the co-located antenna, for BS type 1-C it is specified at the antenna conector and as such is affected by – MCL, therefore the BS-to-BS isolation analysis would have the impact on the definition of BS type 1-C transmitter intermodulation requirement. MCL refers to the minimum coupling loss which is assumed to be 30dB in the existing specification.

#### 6.4.4.3.5	Co-located OOB Blocking requirement

For FR1 co-located OOB Blocking requirement, it s assmed that the co-located interfering system is of the same class as the DUT, interfering signal level at the interfereing co-located system for WA, MR and LA is calculated based on the following assumption:

- 46dBm

- 38dBm

- 24dBm

For BS type 1-O the interferer level is specified at the co-located antenna input.

For BS type 1-C the interferer level is specified at the antenna connector and as such is affected by the MCL:

- 46dBm-MCL

- 38dBm-MCL

- 24dBm-MCL

Where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP, therefore the BS-to-BS isolation analysis would have the impact on the definition of BS type 1-C receiver’s co-located OOB blocking requirement.

#### CLTA alternatives - Wideband horn antenna

The use of wideband horn antenna (as an alternative to the OTA co-location testing with CLTA) has a number of potential advantages:

- Wideband horn antennas are easily available and cover wide frequency range.

- Testing with wideband horn antennas is expected to require smaller number of test antennas to cover the whole tested frequency range (e.g., in case of spurious emissions). This may be most beneficial in case where no off-the-shelf single column antennas are available for certain bands, requiring custom-made test antennas.

- Testing may be less time-consuming. For a standard horn antenna, the antenna characteristics is provided in customer documentation (e.g., gain, radiation pattern, etc.), which may differ in case of custom-made CLTA products.

There are some basic differences with wideband horn antennas and CLTAs:

- Wideband horn antennas are intended to be used directed towards the DUT.

- Wideband horn antennas are different size than CLTA (considering that co-location testing with CLTA has certain DUT dimension-related restrictions).

- Wideband horn antennas are receiving different frequencies in different depths of the horn.

- Wideband horn antenna structure is semi-open from two sides and closed from two sides or closed from all four sides.

- Measurements were made with the horn antenna replacing the CLTA using the current CLTA arrangement. The horn antenna was placed next to the DUT keeping the 10cm distance and its boresight was facing the same direction as the DUT. Example measurement setup is shown in figure 6.4.4.4-1.

Figure 6.4.4.4-1: Co-located horn antenna example measurement setup

Based on measurements with CLTA arrangements and different configurations, it was observed that:

- The size of the wideband horn antenna has a significant effect on the coupling loss results.

- There is no immediate correlation between the coupling loss in the near-field, increasing distance from 10 cm (i.e., OTA co-location test setup specific arrangement) to 20cm or even to 50cm doesn’t guarantee higher coupling loss.

- CLTA test setup arrangement (as detailed in e.g., TS 38.141-2 [6]) as such is not optimal for wideband horn antennas due to the structure of the antenna and would need to be further studied.

- There is no straightforward one-to-one translation to use wideband horn antenna as CLTA with the current CLTA arrangement. This could be considered as a potential study area in future.

Single column passive antennas may not be readily available (for the purpose of CLTA test antenna) for all of the required frequency bands. They typically meet the CLTA requirements only at relatively narrow frequency ranges (e.g., 410 MHz – 467.5 MHz). Currently multiple custom-made antennas may be needed to cover OTA co-location testing in the required frequency range. This could make testing in higher and lower frequencies become unnecessarily complex and time-consuming. Therefore, a new or additional method for the OTA co-location testing may be needed to simplify testing.

------------------------------ End of modified section -------------------------