3GPP TSG-RAN WG1 Meeting #122	R1-2506644

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0577 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of TEI19 for LTE-based 5G Broadcast [5GB CASMuting] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of CAS muting for LTE-based broadcast |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a statement suppresssing PSS and PBCH transmission in certain radio frames. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for CAS muting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4, 6.11.1.2, 6.11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.6.4	Mapping to resource elements

The block of complex-valued symbols for each antenna port shall

-	for an MBMS-dedicated cell, be transmitted during 4 consecutive radio frames fulfilling , starting in each radio frame fulfilling , and

-	otherwise, be transmitted during 4 consecutive radio frames, starting in each radio frame fulfilling .

The block of complex-valued symbols shall be mapped in sequence starting with  to resource elements  constituting the core set of PBCH resource elements. The mapping to resource elements  not reserved for transmission of reference signals shall be in increasing order of first the index, then the index  in slot 1 in subframe 0 and finally the radio frame number. The resource-element indices are given by

where resource elements reserved for reference signals shall be excluded. The mapping operation shall assume cell-specific reference signals for antenna ports 0-3 being present irrespective of the actual configuration. The UE shall assume that the resource elements assumed to be reserved for reference signals in the mapping operation above but not used for transmission of reference signal are not available for PDSCH or MPDCCH transmission. The UE shall not make any other assumptions about these resource elements.

For an MBMS-dedicated cell configured with repetition, the physical broadcast channel shall be repeated as described in clause 6.6.4.1.

For an MBMS-dedicated cell configured with CAS muting, the physical broadcast channel shall only be transmitted in the first 4KCAS frames, starting in frames fulfilling nf mod 16NCAS=0 where NCAS∈2, 4, 8, 16 and KCAS∈4, 5, 6, …, 63 are given by the higher-layer parameter cas-MutingConfig.

If a cell is configured with repetition of the physical broadcast channel

-	symbols mapped to core resource element  in slot 1 in subframe 0 within a radio frame  according to the mapping operation above, and

-	cell-specific reference signals in OFDM symbols  in slot 1 in subframe 0 within a radio frame  with  according to the mapping operation above

shall additionally be mapped to resource elements  in slot number  within radio frame  unless resource element  is used by CSI reference signals.

For frame structure type 1, , , and  are given by Table 6.6.4-1.

For frame structure type 2,

-	if ,  and  are given by Table 6.6.4-2 and ;

-	if ,  and  are given by Table 6.6.4-2 and , except that repetitions with  and  are not applied.

For both frame structure type 1 and frame structure type 2, repetition of the physical broadcast channel is not applicable if .

Resource elements already reserved or used for transmission of cell-specific reference signals in absence of repetition shall not be used for additional mapping of cell-specific reference signals.

Table 6.6.4-1: Frame offset, slot and symbol number triplets for repetition of PBCH for frame structure type 1.

|  | Frame offset, slot and symbol number triplets |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 | (1,18,3), (1,19,0), (1,19,4), (0,0,4) | (1,18,3), (1,19,0), (1,19,5) |
| 1 | (1,18,4), (1,19,1). (1,19,5), (0,1,4) | (1,18,4), (1,19,1). (0,0,3) |
| 2 | (1,18,5), (1,19,2), (1,19,6), (0,1,5) | (1,18,5), (1,19,2), (0,1,4) |
| 3 | (1,18,6), (1,19,3), (0,0,3), (0,1,6) | (1,19,3), (1,19,4), (0,1,5) |

Table 6.6.4-2: Slot and symbol number pairs for repetition of PBCH for frame structure type 2.

|  | Slot and symbol number pairs |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 | (0,3), (1,4), (10,3), (11,0), (11,4) | (0,3), (10,3), (11,0) |
| 1 | (0,4), (1,5), (10,4), (11,1), (11,5) | (0,4), (10,4), (11,1) |
| 2 | (0,5), (10,5), (11.2) | (0,5), (10,5), (11.2) |
| 3 | (0,6), (10,6), (11.3) | (1,4), (11,3), (11.4) |

#### 6.11.1.2	Mapping to resource elements

The mapping of the sequence to resource elements depends on the frame structure. The UE shall not assume that the primary synchronization signal is transmitted on the same antenna port as any of the downlink reference signals. The UE shall not assume that any transmission instance of the primary synchronization signal is transmitted on the same antenna port, or ports, used for any other transmission instance of the primary synchronization signal.

The sequence  shall be mapped to the resource elements according to

For frame structure type 1, the primary synchronization signal shall be mapped to the last OFDM symbol in slots 0 and 10.

For frame structure type 2, the primary synchronization signal shall be mapped to the third OFDM symbol in subframes 1 and 6. Resource elements  in the OFDM symbols used for transmission of the primary synchronization signal where

are reserved and not used for transmission of the primary synchronization signal.

For frame structure type 3, the primary synchronization signal shall be mapped according to frame structure type 1 with the following exceptions:

-	the primary synchronization signal shall be transmitted only if the corresponding subframe is non-empty and at least 12 OFDM symbols are transmitted,

-	a primary synchronization signal being part of a discovery signal shall be transmitted in the last OFDM symbol of the first slot of a discovery signal occasion.

For an MBMS-dedicated cell, the primary synchronization signal shall be mapped according to frame structure type 1 with following exception:

-	the primary synchronization signal shall be transmitted in slot 0 in subframes fulfilling  only,

For an MBMS-dedicated cell configured with CAS muting, the primary synchronization signal shall only be transmitted in the first 4KCAS frames, starting in frames fulfilling nf mod 16NCAS=0 where NCAS∈2, 4, 8, 16 and KCAS∈4, 5, 6, …, 63 are given by the higher-layer parameter cas-MutingConfig.

#### 6.11.2.2	Mapping to resource elements

The mapping of the sequence to resource elements depends on the frame structure. In a subframe for frame structure type 1 and 3 and in a half-frame for frame structure type 2, the same antenna port as for the primary synchronization signal shall be used for the secondary synchronization signal.

For an MBMS-dedicated cell configured with CAS muting, the secondary synchronization signal shall only be transmitted in the first 4KCAS frames, starting in frames fulfilling nf mod 16NCAS=0 where NCAS∈2, 4, 8, 16 and KCAS∈4, 5, 6, …, 63 are given by the higher-layer parameter cas-MutingConfig.

The sequence  shall be mapped to resource elements according to

Resource elements  where

are reserved and not used for transmission of the secondary synchronization signal.