3GPP TSG-RAN WG1 Meeting # 122	R1-2506684

Bengaluru, India, 25th – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0689 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TEI19 Simultaneous NZP-CSI-RS resource counting with NES [SimCSI_countNES] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TEI19 agreement in RAN1#121: Agreement - For simultaneous CSI-RS reception when operating Network Energy saving UE features 42-1/1a/1b/1c, and 42-2/2a/2b/2c, define the following new UE capability: - Simultaneous NZP-CSI-RS resource counting NES: To allow the UE to indicate that CSI-RS ports within one periodic/semi-persistent CSI-RS resource, as well as the periodic/semi-persistent CSI-RS resource, are counted as one resource even if the periodic/semi-persistent CSI-RS resource is referred multiple times by one or more CSI Reporting Settings with at least one CSI Reporting Setting configured with higher layer parameter csi-ReportSubConfigToAddModList. RAN1#121 agreed on a text proposal reflected in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying the agreed new capability related behaviour for counting periodic or semi-persistent NZP-CSI-RS resources and ports as one resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed TEI item is not supported by the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter [RRC_name-r19], OCPU= OCPU,1 is considered.

For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,2 simultaneous CSI calculations it is said to have NCPU,2 CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,2-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,2-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,2(n), the UE is not required to update the N-M2 requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M2≤N is the largest value such that n=0M2-1OCPU,2(n)≤NCPU,2-L  holds.

For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report is not considered within any of M and M2, the values for OCPU,1  and OCPU,2 are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report.

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured.

-	OCPU=1  for a CSI report with ltm-CSI-ReportConfig or a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index', 'cli-SRS-RSRP', 'cli-RSSI', 'rs-pai-r19', 'none-bm-r19', 'none-csi-r19' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured) or a CSI report with CSI-ReportConfig with higher layer parameter eventType configured.

-	OCPU=Y+1⋅X, for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'tdcp' and with number of delays Y configured by higher layer parameter Y, where the value of X∈{1, 2} is reported by UE capability.

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if max{ µPDCCH, µCSI-RS, µUL} ≤ 3, and if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	if a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to mTRP-CSI-numCPU-r17 and M is defined in clause 5.2.1.4.2,

-	if a CSI-ReportConfig contains a list of LR sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList,

-	OCPU=i=1 LRKsi for periodic CSI reporting, where Ksi is the total number of CSI-RS resources in the CSI-RS resource set for channel measurement corresponding to the i-th sub-configuration.

-	OCPU=i=1NRKsi for aperiodic and semi-persistent CSI reporting, where Ksi is the total number of CSI-RS resources in the CSI-RS resource set for channel measurement corresponding to the i-th sub-configuration, and where the i-th sub-configuration is from NR indicated sub-configurations out of LR sub-configurations contained in a CSI-ReportConfig, where NR≤LR and NR≥1.

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 1<NTRP≤4 resources, OCPU=ceil(X⋅NTRP), where X∈{1, 1.5, 2} is reported by UE capability indication,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18',

-	if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with K CSI-RS resources,  OCPU=8 for K=12 and OCPU=Y1⋅K for K<12, where Y1∈{1, 2, 3} is reported by UE capability indication,

-	if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with a single CSI-RS resource, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter vectorLengthDD, and Y2∈{1, 2, 3} is reported by UE capability indication,

-	if a CSI-ReportConfig is configured with the higher layer parameter codebookType set to 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19' or 'typeII-FePortSelection-r19', with total number of CSI-RS ports P∈{48,64,128}, OCPU=P/32 for capability 1 and OCPU=1 for capability 2, as reported by UE capability indication,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r19',

-	if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with KDOPP CSI-RS resource groups of K CSI-RS resources, with total number of CSI-RS ports P∈{48,64,128},

-	for capability 1, OCPU=8 for KDOPP=12 and OCPU=min(Y1⋅KDOPP⋅P/32,8) for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication,

-	for capability 2, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication,

-	if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with K CSI-RS resources, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{1, 2, 3} is reported by UE capability indication, for both capability 1 and 2 indication,

-	if a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel-r19' and with a list of L sub-configurations,

-	for periodic CSI reporting, OCPU=L for both capability 1 and 2 reported by UE capability indication,

-	for aperiodic and semi-persistent CSI reporting, OCPU=N for both capability 1 and 2 reported by UE capability indication, where N is the number of indicated sub-configurations, with 1≤N≤L,

-	if a CSI-ReportConfig is configured with higher layer parameter reportQuantity set to 'cjtc-Dd', 'cjtc-F' or 'cjtc-P' and NTRP CSI-RS resources or resource sets in the corresponding CSI-RS Resource Setting for channel measurement, as described in Clause 5.2.1.4.1, OCPU=NTRP⋅X, where the value of X≥1 is reported by UE capability for each report quantity,

-	OCPU=2NTRP⋅X, for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd-F' and NTRP CSI-RS resource sets in the corresponding CSI-RS Resource Setting for channel measurement, where the value of X≥1 is reported by UE capability,

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

-	for a CSI report with CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19',

-	if nroftimeinstance-r19 is not configured, OCPU,1=X1 and OCPU,2=X2, where the value of X1and X2  are reported by UE capability.

- 	if nroftimeinstance-r19 is configured, OCPU,1=Y1 and OCPU,2=Y2, where the value of Y1and Y2  are reported by UE capability.

-	OCPU,1=X1 and OCPU,2=X2, for a CSI report configured with the higher layer parameter [RRC_name-r19], where the values of X1 and X2 are reported by UE capability.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,2, for  CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

-	A semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' occupies CPU(s) from the first symbol of KP-th latest consecutive periodic/semi-persistent CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where the value of KP∈{1,2,4} is indicated by UE capability.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation.

where (Z3,Z3') are defined in the table 5.4-2.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19'), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released.

If a CSI-RS resource is referred N times by one or more CSI Reporting Settings not configured with higher layer parameter csi-ReportSubConfigToAddModList, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, if a CSI-RS resource is referred X times by one of the M CSI-RS resources, where M is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times.

If the UE indicates [simultaneous NZP-CSI-RS resource counting_NES] and the CSI-RS resource is configured as periodic or semi-persistent and is referred N times by one or more CSI Reporting Settings with at least one CSI Reporting Setting configured with higher layer parameter csi-ReportSubConfigToAddModList, the CSI-RS resource is counted once and the CSI-RS ports within the CSI-RS resource are counted P, where P is the number of ports configured by nrofPorts.

If the UE does not indicate [simultaneous NZP-CSI-RS resource counting_NES] or the UE indicates [simultaneous NZP-CSI-RS resource counting_NES] and the CSI-RS resource is not configured as periodic or semi-persistent:

-	For a CSI-ReportConfig containing a list of LR sub-configurations provided by higher layer parameter csi-ReportSubConfigToAddModList, if a CSI-RS resource is referred by M sub-configurations among NR triggered sub-configurations for CSI reporting for aperiodic CSI-RS resource, or LR configured sub-configurations for CSI reporting for periodic or semi-persistent CSI-RS resource, the CSI-RS resource is counted M times and the CSI-RS ports within the CSI-RS resource are counted maxs=1MPs, P, where P is the number of ports configured by nrofPorts and Ps is the number of CSI-RS ports in s-th sub-configuration from M sub-configurations derived from the corresponding antenna port subset indicator portSubsetIndicator according to clause 5.2.1.4.2 if configured, otherwise Ps=P .

For a periodic or semi-persistent CSI-RS resource in a CSI-RS resource set for channel measurement linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted KP times, where the value of KP∈{1,2,4} is indicated by UE capability.

<omitted text>