3GPP TSG RAN WG1 #122bis                                                                                          R1-2506877

Prague, Czech, Oct 13th – 17th, 2025

Source:	vivo

Title:	Maintenance on Rel-19 NR NTN

Agenda Item:	8.7.1

Document for:	Discussion and Decision

- Introduction

Note: Maximum one contribution for NR_NTN_Ph3, and potential RAN1 impact from NR_NTN_Ku_bands and NR_IoT_NTN_req_test_enh. Note: For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any: - NR_NTN_Ph3 - RAN1 impact from NR_NTN_Ku_bands and NR_IoT_NTN_req_test_enh

R1-2505282 Maintenance on 3 MHz CBW and on Ku-band for NR-NTN Ericsson From Wednesday session Conclusion TP1 in R1-2505282 is postponed to RAN1#122bis for further checking.

As per the chairman’s guidance, in this contribution, we provide our views on the remaining issues of Rel-19 NR NTN, including the support of 3MHz bandwidth, Ku band/FR2 issues, DL coverage enhancement, and Uplink Capacity/Throughput Enhancement.

- Discussion

2.1 3 MHz CBW

As part of 3GPP Rel-19 WI NR_IoT_NTN_req_test_enh, RAN4 had the objective of supporting less than 5MHz for NTN. RAN4 has introduced new sync raster for operating band (FR1-NTN) for 3MHz channel bandwidth in [1], and reference to the new raster should be added in RAN1 spec.

- TP#1 for 38.211

38.211 - 7.3.2.2 Control-resource set (CORESET) For CORESET 0 on a carrier where the SS/PBCH block is detected at sync raster points defined in Tables 5.4.3.1-2 or 5.4.3.1-3 of [14, TS 38.101-1] and configured by the ControlResourceSetZero IE: - NRBCORESET and NsymbCORESET are defined by Table 13-0 in clause 13 of [5, TS 38.213]; - if NRBCORESET=12 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2; - if NRBCORESET=24 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2 or non-interleaved mapping as defined by clause 13 of [5, TS 38.213], followed by puncturing the 9 highest-numbered resource blocks to obtain the 15 resource blocks forming CORESET 0; - if NRBCORESET=24 on a carrier with a channel bandwidth of 5 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2, followed by puncturing the 4 highest-numbered resource blocks to obtain the 20 resource blocks forming CORESET 0; - L=6; - nshift=NIDcell; - the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1; - the UE may assume the same precoding being used within a REG bundle.

According to the yellow-highlighted part, for a TN carrier with a channel bandwidth of 3 MHz, CORESET 0 can be 12 RB or 24 RB with the 9 highest-numbered resource blocks being punctured. Similarly, for a NTN carrier with a channel bandwidth of 3 MHz, the yellow-highlighted part is also applicable.

However, it is also noted that the cyan-highlighted part is for TN band n100 with 5 MHz CBW consisting of 20 transmissions PRBs, and therefore is not applicable to NTN. As the raster points for NTN with 3 MHz CBW are defined in Table 5.4.3.1-2 of 38.101-5, the following TP is proposed:

Reason for change: Reference to sync raster points for NTN with 3MHz CBW is missing.

Summary of change: Add reference to sync raster points for NTN with 3MHz CBW defined in 38.101-5.

Consequences if not approved: CORESET0 for FR1-NTN with 3MHz CBW is not supported.

TP#1 for 38.211 - 7.3.2.2 Control-resource set (CORESET) ==omitted== For CORESET 0 on a carrier where the SS/PBCH block is detected at sync raster points defined in Tables 5.4.3.1-2 or 5.4.3.1-3 of [14, TS 38.101-1] or Table 5.4.3.1-2 of [16, TS 38.101-5], and configured by the ControlResourceSetZero IE: - NRBCORESET and NsymbCORESET are defined by Table 13-0 in clause 13 of [5, TS 38.213]; - if NRBCORESET=12 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2; - except for NTN, if NRBCORESET=24 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2 or non-interleaved mapping as defined by clause 13 of [5, TS 38.213], followed by puncturing the 9 highest-numbered resource blocks to obtain the 15 resource blocks forming CORESET 0; - if NRBCORESET=24 on a carrier with a channel bandwidth of 5 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2, followed by puncturing the 4 highest-numbered resource blocks to obtain the 20 resource blocks forming CORESET 0; - L=6; - nshift=NIDcell; - the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1; - the UE may assume the same precoding being used within a REG bundle.

Proposal 1. Adopt TP#1 for 38.211 to support 3MHz CBW for NTN.

- TP#2 for 38.213

Table 5.4.3.3-2 in 38.101-1 defines the applicable SS raster entries per operating band for 3 MHz channel bandwidth, meanwhile, the applicable SS raster entries per operating band (FR1-NTN) for 3MHz channel bandwidth are defined in Table 5.4.3.3-1a of 38.101-5. Thus, configurations with index 0 to 9 in Table 13-0 can be reused for NTN.

Table 5.4.3.3-1: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n100 | 15 kHz | Case A | 2303 – <1> – 2307, 4163812 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. NOTE 2: The applicable SS raster entries are GSCN = {6432, 6443, 6457, 6468, 6479, 6493, 6507, 6518, 6532, 6543}. NOTE 3: The following GSCN are allowed for operation in band n46: GSCN = {8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9288, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9472, 9485, 9499, 9513}. NOTE 4: The following GSCN are allowed for operation in band n96: GSCN = {9548, 9562, 9576, 9590, 9603, 9617,9631, 9645, 9659, 9673, 9687, 9701, 9714, 9728, 9742, 9756, 9770, 9784, 9798, 9812, 9826, 9840, 9853, 9867, 9881, 9895, 9909, 9923, 9937, 9951, 9964, 9978, 9992, 10006, 10020, 10034, 10048, 10062, 10076, 10090, 10103, 10117, 10131, 10145, 10159, 10173, 10187, 10201, 10214, 10228, 10242, 10256, 10270, 10284, 10298, 10312, 10325, 10339, 10353}. NOTE 5: The applicable SS raster entries are GSCN = {5032, 5043, 5054} NOTE 6: The applicable SS raster entries are GSCN = {4707, 4715, 4718, 4729, 4732, 4743, 4747, 4754, 4761, 4768, 4772, 4782, 4786, 4793} NOTE 7: The SS raster entries apply for channel bandwidths larger than or equal to 40 MHz NOTE 8: The SS raster entries apply for channel bandwidths smaller than 40 MHz NOTE 9: The following GSCN are allowed for operation in band n102: GSCN = {9548, 9562, 9576, 9590, 9603, 9617,9631, 9645, 9659, 9673, 9687, 9701, 9714, 9728, 9742, 9756, 9770, 9784, 9798, 9812, 9826, 9840, 9853, 9867}. NOTE 10: The SS raster entries apply for channel bandwidths larger than or equal to 10 MHz. NOTE 11: The SS raster entries apply for channel bandwidth equal to 5 MHz NOTE 12: Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

It is noted that configurations with index 10 to 11 in Table 13-0 are defined for TN band n100 with a 5 MHz CBW consisting of 20 transmission PRBs, while Table 13-1 is not applicable to TN band n100. Since NTN does not support the feature of 5 MHz CBW consisting of 20 transmission PRBs, the configurations with index 10 to 11 in Table 13-0 cannot be reused for NTN, while Table 13-1 can still be used. Thus, the following TP is proposed:

Reason for change: Reference to applicable SS raster entries per operating band for FR1 NTN with 3 MHz channel bandwidth is missing.

Summary of change: Add reference to applicable SS raster entries per operating band for FR1-NTN with 3MHz CBW defined in 38.101-5.

Consequences if not approved: CORESET0 and search space 0 for FR1-NTN with 3MHz CBW are not supported.

TP#2 for 38.213 13 UE procedure for monitoring Type0-PDCCH CSS sets If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-0 through 13-10, for operation without shared spectrum channel access in FR1, FR2-1 and FR2-NTN, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Table 13-10A for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15A. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. In Table 13-0, configurations with index 0 to 9 are applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in [8-1, TS 38.101-1] or Table 5.4.3.3-1a of [16, TS 38.101-5], configurations with index 10 to 11 are applicable when an associated SS/PBCH block is located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1], and non-interleaved CCE-to-REG mapping applies for configurations with index 6 to 9. In Table 13-1, the associated SS/PBCH block is not located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1].

Proposal 2. Adopt TP#2 for 38.213 to support 3MHz CBW for NTN.

- TP#3 for 38.214

Similar to the previous discussion, as CORESET 0 puncturing should be supported for NTN, reference to Table 5.4.3.1-2 of 38.101-5 should be added.

Reason for change: Reference to sync raster points for NTN with 3MHz CBW is missing.

Summary of change: Add reference to sync raster points for NTN with 3MHz CBW defined in 38.101-5.

Consequences if not approved: Description of the cell search procedure on a FR1-NTN cell with 3MHz CBW is missing.

TP#3 for 38.214 5.1 UE procedure for receiving the physical downlink shared channel ==omitted== For a cell detected in cell search procedure with synchronization raster defined in Table 5.4.3.1-2 or Table 5.4.3.1-3 of [8, TS 38.101-1], or Table 5.4.3.1-2 of [16, TS 38.101-5], the size of CORESET 0 for the cell in this clause refers to the size of punctured CORESET 0 as defined in clause 7.3.2.2 of [4, TS 38.211] if any.

Proposal 3. Adopt TP#3 for 38.214 to support 3MHz CBW for NTN.

2.2 R19 Ku band/R18 FR2-NTN

In the last meeting, [2] pointed out that after the introduction of the Ku band, some clauses of the RAN1 specifications refer to “for FR2”, while others explicitly state “FR2 and FR2-NTN”, and commented that this inconsistency could lead to misunderstanding. Thus, it is proposed to revise the RAN1 Rel-19 specifications (e.g., by consistently adding FR2-NTN).

However, as clarified in the last meeting, this inconsistency was not caused by the introduction of the Ku band in Rel-19, but rather by the introduction of FR2-NTN in Rel-18.

Observation 1. The inconsistency indicated in [2] is due to the inclusion of FR2-NTN in Rel-18.

Before we discuss how/whether to handle the inconsistency, the following historical discussions for FR2-NTN should be noted.

RAN4

Since FR2-NTN and FR2-TN bands do not fully overlap, RAN4 introduced the following note2 in the specification. The purpose of note2 is to avoid the need for RAN1 and RAN2 specifications to extensively add “FR2-NTN” everywhere, which would otherwise result in a large amount of specification changes.

TS 38.101-5 Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1. Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN (Note 1); 410 MHz – 7125 MHz. FR2-NTN (Note 2); 17300 MHz – 30000 MHz. NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated..

RAN1

RAN1 has previously discussed several meetings and endorsed CRs introducing FR2-NTN with the following:

- “and FR2-NTN” were added in the key fundamental functions where NTN applicability was required.

- For procedures where FR2-1 and FR2-2 have different behaviors (e.g., CORESET0 configuration table, PDCCH monitoring occasions from searchSpaceZero, GSCN offset, etc), FR2-NTN was explicitly added after ‘FR2-1’ to clarify that FR2-NTN reuses the procedure of FR2-1.

[2] points out that there is an inconsistency between “FR2” and “FR2 and FR2-NTN” in clause 4.1 of 38.213. However, in addition to this, similar descriptions already exist in other clauses and other specs. For example, in the descriptions about k_ssb in clause 13 of 38.213, “and FR2-NTN” was added to clarify that FR2-NTN reuses the interpretation of subCarrierSpacingCommon in a same way as FR2-1. However, for Table 13-17, there is no ambiguity, thus FR2 NTN was not added.

For operation with shared spectrum channel access in FR2-2 and for operation without shared spectrum channel access, a UE assumes that the offset in Tables 13-0 through 13-10A is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block, after puncturing if any [4, TS 38.211]. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1, FR2-1 and FR2-NTN, and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10, kSSB is defined in [4, TS 38.211]. ==omitted== Table 13-17: Mapping between the combination of kSSB and controlResourceSetZero and searchSpaceZero in pdcch-ConfigSIB1 to NGSCNOffset for FR2 kSSB; 16×controlResourceSetZero +searchSpaceZero; NGSCNOffset. 12; 0, 1, …, 255; 1, 2, …, 256. 13; 0, 1, …, 255; -1, -2, …, -256. 14; 0, 1, …, 255; Reserved, Reserved, …, Reserved.

Furthermore, it should be noted that ‘FR2-NTN is never mentioned in 38.215, instead, ‘frequency range 1’ and ‘frequency range 2’ are used throughout. However, NTN is still assumed to be supported by default.

38.215

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration, except when measurement is being performed by wake-up receiver (WUR). If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |

For these specifications, there are several options.

- Option 1: Keep the specifications unchanged.

- Option 2: Remove “FR2-NTN” from all RAN1 specifications.

From a specification maintenance perspective, this option would make the applicability of existing features even less clear than the current specification. Alternatively, for some features that have not yet been applicable to NTN (e.g., power prioritization between CA), additional descriptions to exclude NTN may be needed.

- Option 3: Add “FR2-NTN” consistently across all RAN1 specifications.

A lot of spec changes are needed. This option would require that once a feature is introduced for TN, there must be a corresponding discussion on whether it applies to NTN, and explicit addition of “FR2-NTN” in the specifications.

- Option 4: Add “FR2-NTN” only to the clauses that include both “FR2” and “FR2-1 and FR2-NTN”/“FR and FR2-NTN”.

This option may need fewer spec changes compared with option 3, but it still has the same issues as option 3, i.e., additional discussion is needed on whether a new feature introduced for TN can be applied to NTN.

These options may have different pros and cons:

|  | Pros | Cons |
| --- | --- | --- |
| Option1 | - 0 spec changes | - The spec may not be crystal clear |
| Option2 | - N.A | - Requires extensive modification of specifications. - Less clear than option 1. - For some features not applicable to R18/19 NTN(e.g., DC/CA), additional descriptions to exclude NTN may be needed. |
| Option3 | - Spec is clear | - Requires extensive modification of specifications. - Increases maintenance effort whenever a feature is introduced for TN. |
| Option4 | - Spec is clear - Fewer changes than option 3 | - Similar to option 3, increases maintenance effort whenever a new feature is introduced. |

It should also be noted that NTN CA has only just been agreed to be supported by RAN4, and therefore, in R20, there may still be a need to introduce corresponding changes in RAN1.

Considering the above pros and cons, option 1 is preferred as it has less maintenance effort.

Proposal 4. Regarding the term “FR2-NTN” handling, option 1 (no spec changes) is preferred.

2.3 DL coverage enhancement: SIB1 PDSCH repetition

It was agreed that the common PDDCH repetition can also be applied to TN FR1 in RAN#108, but SIB1 PDSCH repetition is only applicable to NTN. Hence, the corresponding part of TS38.214 should be further updated to clarify that, e.g., only UEs reporting the UE FG 65-1-2 PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration support the SIB1 PDSCH repetition. In particular, the TP is shown as follows:

- TP#4 for 38.214

Reason for change: The SIB1 PDSCH repetition is only applicable to NTN, not for TN.

Summary of change: Add the corresponding UE capability for the SIB1 PDSCH repetition.

Consequences if not approved: SIB1 PDSCH repetition is also applicable to TN, which is not agreed by RAN1 and RAN plenary.

TP#4 for 38.214 5 Physical downlink shared channel related procedures 5.1 UE procedure for receiving the physical downlink shared channel ==omitted== If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], or not available for PDSCH without a corresponding PDCCH transmission receptions as described in clause 24 of [6, TS 38.213], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. A UE capable of [PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration], which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0. ==omitted==

Proposal 5. Adopt TP#4 for 38.214 for clarifying the application condition of SIB1 repetition.

2.4 Uplink Capacity/Throughput Enhancement: reference to 212

6.1.2.1 Resource allocation in time domain ==omitted== For a PUSCH transmission with repetition Type A and with OCC operation enabled, an OCC group is defined by Locc consecutive PUSCH repetitions and the integer number of OCC groups M is determined as M=K/Locc, OCC group m includes PUSCH repetition m x Locc, m x Locc+1, .., (m+1) x Locc-1, where m=0,1, .., M-1, where repetition i within the OCC group will be multiplied with the OCC factor wi, where i is the repetition index within the OCC group. The OCC factor is for each repetition index i, based on the orthogonal sequence wni defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 of [4, TS 38.211], where the sequence index n is derived from the DCI format scheduling the transmission for PUSCH or activating configured grant Type 2 PUSCH according to [5, TS 38.212, antenna port mapping to sequence], or provided by OCCsequenceIndexCGType1-r19 for configured grant Type 1 PUSCH, and provided for actual transmission according to clause 6.3.1.2a in [4, TS 38.211], where i=0,1 for OCC length 2 and i=0,1,2,3 for OCC length 4. ==omitted==

The yellow-highlighted text above in the latest 38.214 is not needed. Reference to 38.212 is sufficient. We propose the following TP#2:

- TP#5 for 38.214

Reason for change: The format of the reference to 38.212 is not correct.

Summary of change: Remove ‘antenna port mapping to sequence’.

Consequences if not approved: The format of the reference to 38.212 remains incorrect.

TP#5 for 38.214 6.1.2.1 Resource allocation in time domain ==omitted== For a PUSCH transmission with repetition Type A and with OCC operation enabled, an OCC group is defined by Locc consecutive PUSCH repetitions and the integer number of OCC groups M is determined as M=K/Locc, OCC group m includes PUSCH repetition m x Locc, m x Locc+1, .., (m+1) x Locc-1, where m=0,1, .., M-1, where repetition i within the OCC group will be multiplied with the OCC factor wi, where i is the repetition index within the OCC group. The OCC factor is for each repetition index i, based on the orthogonal sequence wni defined in Table 6.3.2.5A-1 and Table 6.3.2.5A-2 of [4, TS 38.211], where the sequence index n is derived from the DCI format scheduling the transmission for PUSCH or activating configured grant Type 2 PUSCH according to [5, TS 38.212], or provided by OCCsequenceIndexCGType1-r19 for configured grant Type 1 PUSCH, and provided for actual transmission according to clause 6.3.1.2a in [4, TS 38.211], where i=0,1 for OCC length 2 and i=0,1,2,3 for OCC length 4. ==omitted==

Proposal 6. Adopt TP#5 for 38.214 to remove ‘antenna port mapping to sequence’ in the reference.

2.5 Uplink Capacity/Throughput Enhancement: Event of phase continuity and power consistency

Agreement Send LS to RAN4 on requirements for the phase continuity and power consistency: RAN1 has agreed the following: - RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed. LS from RAN4 (R4-2510262) RAN4 would like to thank RAN1 for the LS in R1-2503071. RAN4 has discussed the phase continuity requirements for the duration of one OCC group with PUSCH, and confirmed that the same requirements for DMRS bundling can be applied. Specifically, ≤25° between adjacent slots. The side conditions will also be aligned with DMRS-bundling requirement. It is RAN4 opinion that UE should be able to indicate support for OCC independently for GSO and NGSO deployments.

RAN4 discussed the phase continuity for OCC group, and agreed the following requirements and side conditions on maintaining phase continuity. There is no need for RAN1 to further define events breaking the phase continuity within a OCC group.

Requirements from RAN4 R4-2512535 - 6.4.2.3 Phase continuity requirements for OCC For bands that NTN UE indicates the support of UE capability of [PUSH_OCC2] or [PUSCH_OCC2_OCC4], when the NTN UE has OCC operation enabled [14], the maximum allowable difference between the measured phase value in any slot p-1 and slot p across the slots in an OCC group [14] for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.3-1 for the measurement conditions defined in Table 6.4.2.5-2 of TS 38.101-1 [5]. The phase value for each slot is measured as shown in Annex F.9 of TS 38.101-1 [5]. These requirements apply to PUSCH transmissions with DFT-s-OFDM waveforms. Table 6.4.2.3-1: Maximum allowable phase difference for OCC2 and OCC4 UL channel; Modulation order; Phase difference between any slot p-1 and slot p. PUSCH; Pi/2 BPSK, QPSK, 16QAM, 64QAM; 25 degrees.

Side conditions from RAN4 R4-2512535 The above requirements are applicable when all the conditions listed below are met within the measurement time window: - RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated. - Modulation order does not change. - No network commanded TA takes effect. - The TPMI precoder does not change. - There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE. - UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions. - Doppler conditions are set to zero and delay conditions are set to constant. - UE specific TA and common TA do not update.

Regarding power consistency for DMRS bundling in TN, the aggregate power tolerance requirement specified in 38.101-1 was agreed to be reused [3]. And the aggregate power tolerance requirement is also reused for NTN according to clause 6.3.4 in 38.101-5. On the other hand, RAN4 indicates that the UE transmission power level should remain unchanged within the measurement time window to maintain the phase continuity for OCC. Consequently, there is no need for RAN1 to further define events that would break power consistency.

R4-2206538[3] CR on UE RF requirements for DMRS bundling in TS 38.101-1: 38.101-1 v17.4.0	CR-1034  rev  Cat: B (Rel-17)

| Reason for change: | To support DMRS bundling for PUSCH and PUCCH in Rel-17 coverage enhancements WI, phase continuity and power consistency need to be maintained with certain tolerances. For power consistency, the existing requirement for aggregate power tolerance applies and no dedicated new requirements are to be introduced. For phase continuity, new requirements should be introduced. |
| --- | --- |
| Summary of change: | Introduce the phase continuity requirements for DMRS bundling for PUSCH and PUCCH. |
| Consequences if not approved: | DMRS bundling for PUSCH and PUCCH cannot be supported in Rel-17. |

TS 38.101-1 - 6.3.4.4 Aggregate power tolerance The aggregate power control tolerance is the ability of the UE transmitter to maintain its power in a sub-frame (1 ms) during non-contiguous transmissions within 21 ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in TS 38.213 [8] kept constant. The minimum requirement specified in Table 6.3.4.4-1 apply in the power range bounded by the minimum output power as specified in clause 6.3.1 and the maximum output power as specified in clause 6.2.1. Table 6.3.4.4-1: Aggregate power tolerance TPC command; UL channel; Aggregate power tolerance within 21 ms. 0 dB; PUCCH; ± 2.5 dB. 0 dB; PUSCH; ± 3.5 dB. TS 38.101-5 - 6.3.4 Power control The requirements for Power control defined in 3GPP TS 38.101-1 [5] clause 6.3.4 shall apply for NTN satellite UE.

- TP#6 for 38.214

Reason for change: 38.101-5 defines conditions and requirements for keeping phase continuity and power consistency for OCC, and should be referenced in 38.214.

Summary of change: Add reference to conditions and requirements defined in 38.101-5 for keeping phase continuity and power consistency for OCC.

Consequences if not approved: The conditions and requirements to keep phase continuity and power consistency are not clear.

TP#6 for 38.214 - 6.1.2.1 Resource allocation in time domain <omitted text> The UE shall maintain power consistency and phase continuity within an OCC group of PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, or 0_2, or PUSCH repetition Type A with a configured grant as specified in clause 6.4.2.3 and 6.3.4 of [22, 38.101-5]. For a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, the redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … N∙K -1, is determined according to the first row of Table 6.1.2.1-2.

Proposal 7. RAN1 does not need to further define events breaking the power consistency and/or phase continuity for OCC. Adopt the above TP#6 for 38.214 to clarify that conditions for maintaining phase continuity and phase continuity are defined in 38.101-5.

- Conclusion

In this contribution, we provide our views on remaining issues of Rel-19 NR NTN, including the support of 3MHz bandwidth, ku band/FR2 issues, DL coverage enhancement and Uplink Capacity/Throughput Enhancement. According to the discussions, we have the following proposals:

Observation 1. The inconsistency indicated in [2] is due to the inclusion of FR2-NTN in Rel-18.

Proposal 1. Adopt TP#1 for 38.211 to support 3MHz CBW for NTN.

Proposal 2. Adopt TP#2 for 38.213 to support 3MHz CBW for NTN.

Proposal 3. Adopt TP#3 for 38.214 to support 3MHz CBW for NTN.

Proposal 4. Regarding the term “FR2-NTN” handling, option 1 (no spec changes) is preferred.

Proposal 5. Adopt TP#4 for 38.214 for clarifying the application condition of SIB1 repetition.

Proposal 6. Adopt TP#5 for 38.214 to remove ‘antenna port mapping to sequence’ in the reference.

Proposal 7. RAN1 does not need to further define events breaking the power consistency and/or phase continuity for OCC. Adopt the above TP#6 for 38.214 to clarify that conditions for maintaining phase continuity and phase continuity are defined in 38.101-5.

- Reference

[1] R4-2509781, BigCR for TS 38.101-5 to introduce NTN less than 5MHz, xiaomi

[2] R1-2505282, Maintenance on 3 MHz CBW and on Ku-band for NR-NTN, Ericsson

[3] R4-2206538, CR on UE RF requirements for DMRS bundling in TS 38.101-1, Huawei, HiSilicon