3GPP TSG RAN WG1 #122b																		R1-2506883

Prague, Czech, Oct 13th – 17th, 2025

Source:	vivo

Title:	UE features for evolution of NR duplex operation: SBFD

Agenda Item:	9.3

Document for:	Discussion and Decision

- Introduction

  - For SBFD operation, following agreements were made in RAN1#122 meeting. In this contribution, we will discuss remaining issues for SBFD UE features.

Agreement: For FG 60-1, “n/a” is adopted for “Need of FR1/FR2 differentiation” column Agreement: - FG 60-1 to include support of PDSCH/CSI-RS reception in two DL subbands  Add a new component to report the number of partial PRGs supported Candidate values: {2, 4}  Add a new component to report CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands  Candidate values: {“with factor-of-two relaxation”, “without factor-of-two relaxation”}  Add a note: “FFS: details of counting”  Add a new component of “maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s)”  Add a new component of “Determination of TBS of PDSCH/PUSCH in SBFD symbols” - For FG 60-2, define another FG for Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) - For FG 60-7 and 7a, confirm only one common pattern between DG/Type 2 CG PUSCH and Type 1 CG PUSCH - Introduce two FGs “separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform” and “separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform”. - FFS: Whether/how to capture maximum number of UL muting symbols per slot Agreement: Remove FFS: Other component(s) which may be separate FG from FG 60-3 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; 60-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. Agreement: - Add a new component “Support of valid RO across SBFD and non-SBFD symbols” - Remove “FFS: Other component(s) which may be separate FG” 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) 2. Support of valid RO across SBFD and non-SBFD symbols FFS: Other component(s) which may be separate FG. Agreement: For prerequisite FG(s) for FG 60-4, adopt FG 60-1 only Agreement: - Confirm FG 54-1 as prerequisite FG for FG 60-5 Agreement: For FG 60-7 and 60-7a - No prerequisite FG is defined for FG 60-7 and 60-7a - Remove “FFS: other potential component/FG/additional restriction”

- Discussion

  - SBFD TX/RX/measurement procedures

  - For FG 60-1, regarding CSI-RS reception in two non-contiguous DL subbands, we don’t see the motivation to define specific handling for CSI-RS port counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands, since relaxed CSI processing timeline has been already considered. On the other hand, the relaxed CSI processing timeline is defined based on the assumption that existing CSI-RS port counting and active CSI-RS resource counting for a single CSI-RS resource are reused. Hence, further optimization on the CSI-RS port counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands is not needed. Besides, this optimization would involve functional impact on the specification, which should be discussed in maintenance session.

  - Regarding FG 60-2a, the prerequisite is only FG 60-1, since DL reception and UL transmission with Configuration 2 can be separate.

  - In summary, for SBFD Tx/Rx/measurement operation, following FGs are suggested.

Proposal 1: Support following FGs for SBFD operation.

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Number of partial PRGs supported 11. Supported CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands 12. Maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s) 13. Determination of TBS of PDSCH/PUSCH in SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | Component 7 applies only to UEs supporting FG 23-1-1h Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement Component 10 candidate values: {2, 4} Component 11 candidate values: {“with factor-of-two relaxation”, “without factor-of-two relaxation”} FFS: details of counting | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-2 | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) FFS: Other component(s) FFS: whether component 2 is defined as separate FG | 60-1 | YES | n/a | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-2a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | 60-1 | YES | n/a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |

  - SBFD random access operation

  - For FG 60-5a, the prerequisite should include FG 54-1.

  - For Msg3 repetitions in SBFD symbols, we think it should be a separate FG.

  - For SBFD RACH operation, following FGs are suggested.

Proposal 2: Support followings FGs for SBFD RACH operation.

| 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) | 60-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) 2. Support of valid RO across SBFD and non-SBFD symbols | 60-1 | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-5 | Preamble repetition within additional-ROs for RACH configuration Option 1 | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros 2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-3, 54-1 | YES | n/a | Preamble repetition within additional-ROs for RACH configuration Option 1 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-5a | Preamble repetition within additional-Ros for RACH configuration Option 2 | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros 2. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-4, 54-1 | YES | n/a | Preamble repetition within additional-ROs for RACH configuration Option 2 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | [60-3 or 60-4, 30-6] | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

  - CLI handling

  - For 60-7b and 60-7c, the prerequisite should include 5-19 of Type 1 Configured UL grant. Regarding the maximum number of UL muting symbols, based on our agreements, at most two symbol per UL muting pattern were supported, if there are more than one PUSCH TDMed in one slot, e.g., one DG PUSCH and one CG type 1 PUSCH, the number of UL muting symbols can be more than two since the UL muting pattern for DG PUSCH and CG type 1 PUSCH are separately configured.  There was no any discussion about the maximum number of UL muting symbols per slot, and the motivation to limit maximum number of UL muting symbols per slot artificially is not clear.

  - Following questions on UE features for L1 CLI measurement and reporting were discussed.

- Whether to define the following reporting (related to maximum number of CLI-RSSI measurement resources in various units), and if needed, whether to define new FG to report them:

  - Maximum number of [configured] L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Maximum number of simultaneous L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) across all CCs

  - Maximum number of simultaneous L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Maximum number of aperiodic L1 CLI-RSSI measurement resources across all CCs

  - Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type [#1/#2] (sum of aperiodic/semi-persistent/periodic) per CC

  - Whether/how to define maximum number of configured L3 and L1 CLI-RSSI measurement resources if simultaneous configuration of L3 and L1 CLI-RSSI measurements is supported

- Whether to define the following reporting (related to maximum number of SRS-RSRP measurement resources in various units), and if needed, whether to define new FG to report them:

  - Maximum number of [configured] SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Maximum number of simultaneous SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) across all CCs

  - Maximum number of simultaneous SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Maximum number of aperiodic SRS-RSRP measurement resources across all CCs

  - Whether/how to define maximum number of configured L3 and L1 SRS-RSRP measurement resources if simultaneous configuration of L3 and L1 SRS-RSRP measurements is supported

  - It was agreed in the last meeting in AI 9.3.3 that a UE does not expect to be simultaneously configured with L3 and L1 UE-to-UE CLI measurement and reporting. Thus, there is no need to define maximum number of configured L3 and L1 CLI-RSSI/L1 SRS-RSRP measurement resources.

  - Regarding the other questions, we have the following proposals.

Proposal 3: For 60-7b and 60-7c, the prerequisite should include 5-19. There is no need to capture maximum number of UL muting symbols per slot.

Proposal 4: For L1-CLI-RSSI measurement and reporting, define the following reporting (related to maximum number of CLI-RSSI measurement resources in various units):

  - Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC

Proposal 5: For L1-SRS-RSRP measurement and reporting, define the following reporting (related to maximum number of CLI-RSSI measurement resources in various units):

  - Maximum number of configured L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - Maximum number of simultaneous L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

  - For CLI handling, following FGs are suggested.

Proposal 6: Support following FGs for CLI handling.

| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - One common pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - One common pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-7b | Separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform | Support of separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform | 5-19 | YES | n/a | Separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform is not supported | Per Band | TDD only] | n/a | n/a |  | Optional with capability signalling |
| 60-7c | Separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform | Support of separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform | 5-19 | YES | n/a | Separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8 | L1 CLI-RSSI measurement and aperiodic reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs 5. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC 6. Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | Aperiodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8a | Support of periodic L1 CLI-RSSI reporting on PUCCH | 1. Periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs 5. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC 6. Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC |  | YES | n/a | Periodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8b | Support of semi-persistent L1 CLI-RSSI measurement resource | 1. Support of semi-persistent L1 CLI-RSSI measurement resource | 60-8 | YES | n/a | Semi-persistent L1 CLI-RSSI measurement resource is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource 2. Maximum number of L1 SRS-RSRP measurement resources across all CCs 3. Maximum number of configured L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC 4. Maximum number of simultaneous L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | Aperiodic L1 SRS-RSRP measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

- Conclusion

The FGs for SBFD are summarized as follows.

- SBFD operation

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Number of partial PRGs supported 11. Supported CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands 12. Maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s) 13. Determination of TBS of PDSCH/PUSCH in SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | Component 7 applies only to UEs supporting FG 23-1-1h Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement Component 10 candidate values: {2, 4} Component 11 candidate values: {“with factor-of-two relaxation”, “without factor-of-two relaxation”} FFS: details of counting | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-2 | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) FFS: Other component(s) FFS: whether component 2 is defined as separate FG | 60-1 | YES | n/a | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-2a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | 60-1 | YES | n/a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |

- SBFD RACH

| 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) | 60-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) 2. Support of valid RO across SBFD and non-SBFD symbols | 60-1 | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-5 | Preamble repetition within additional-ROs for RACH configuration Option 1 | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros 2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-3, 54-1 | YES | n/a | Preamble repetition within additional-ROs for RACH configuration Option 1 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-5a | Preamble repetition within additional-Ros for RACH configuration Option 2 | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros 2. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-4, 54-1 | YES | n/a | Preamble repetition within additional-ROs for RACH configuration Option 2 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | [60-3 or 60-4, 30-6] | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

- CLI handling

| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - One common pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - One common pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-7b | Separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform | Support of separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform | 5-19 | YES | n/a | Separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform is not supported | Per Band | TDD only] | n/a | n/a |  | Optional with capability signalling |
| 60-7c | Separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform | Support of separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform | 5-19 | YES | n/a | Separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8 | L1 CLI-RSSI measurement and aperiodic reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs 5. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC 6. Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | Aperiodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8a | Support of periodic L1 CLI-RSSI reporting on PUCCH | 1. Periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs 5. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC 6. Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC |  | YES | n/a | Periodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8b | Support of semi-persistent L1 CLI-RSSI measurement resource | 1. Support of semi-persistent L1 CLI-RSSI measurement resource | 60-8 | YES | n/a | Semi-persistent L1 CLI-RSSI measurement resource is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource 2. Maximum number of L1 SRS-RSRP measurement resources across all CCs 3. Maximum number of configured L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC 4. Maximum number of simultaneous L1 SRS-RSRP measurement resources (sum of aperiodic/semi-persistent/periodic) per CC FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | Aperiodic L1 SRS-RSRP measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

# References