3GPP TSG-RAN WG1 Meeting #122bis	   R1-2506932

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	10.1.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on other aspects for CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN1#122 other aspects of CSI compression including NW and UE data collection for training, performance monitoring, as well as model pairing related issues were discussed and some agreements were reached [1]. In this contribution, we continue our discussion on these aspects.

# Data collection for training

## NW side data collection

### Target CSI quantization

In RAN1#122, following agreements were reached regarding NW side data collection.

Agreement: For NW side data collection of CSI compression, discuss the content and format of the collected data by assuming higher layer based signalling. - Details on signaling is up to RAN2. Agreement: For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantization format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

Analysis on the Target CSI quantization options

Option 2 and Option 3: Among the listed options in the above agreement, Option 0, Option 1, and Option 4 were quite extensively studied and evaluated during Rel-18/19 SI. In turn, details of Option 2 and Option 3 are not clear and limited evaluation results/concrete analysis about these two Options are available from SI. Option 2 and Option 3 aim to reduce SVD/EVD decomposition complexity and overhead, respectively. However, NW side data collection is infrequent and RAN1 agreed that NW side data collection uses higher layer signaling. Therefore, we do not think complexity/overhead of other well-studied options with smaller effort for standard discussions such as Option 0, Option 1, and Option 4 is prohibitive and we tend to think that the motivation of Option 2 or Option 3 is not strong enough to justify lengthy discussions required to clarify their details and considerable evaluation efforts to substantiate their claimed benefits.

Option 0 and Option 4: Option 4 is an enhanced version of Option 0 that supports new set of parameters. In Rel-18/19, it has been discussed whether to adopt Rel-16 eType II-like codebook, especially with new parameter values that support a higher resolution, to quantize and report the Target CSI to NW. Without obtaining high resolution Target CSI as training label, it is challenging for NW to train high performance model. For instance, if the resolution of the Target CSI is also up to PC8 (with ~300bits), the recovery CSI can hardly exceed PC8 even if the CSI feedback payload size is as high as 300bits. Both Option 0 and Option 4 have been evaluated during SI:

- For model training, majority companies show that Option 4 outperforms Option 0 with PC8 by obvious gain (0.7%~5.4% SGCS gain), and is almost close to the upper-bound performance of Float32. On the other hand, the overhead of Option 4 is significantly reduced (94%~97.5%) compared to the unquantized Target CSI of Float32. Considering the training data collection is non-real time (days/months level frequency), the average overhead for training data collection is negligible.

- For model monitoring, majority companies show that Option 4 outperforms Option 0 with PC8 in general, e.g., by 12.2%~76% monitoring accuracy gain under threshold 0.02.

Option 1: Option 1 was also evaluated during SI. Compared to Option 2/3/4, Option 1 has the benefit of less complexity since there is no need to perform matrix projection or SVD/EVD operations. In addition, Option 1 is simple from standard perspective as there is no need to redesign the codebook as opposed to Option 2/3. On the other hand, Option 1 is expected to have larger overhead since it does not perform compression over spatial/frequency domain. However, considering the NW side data collection is carried with higher layer signaling, the overhead may not be a significant issue.

In summary, both Option 1 and Option 4 have small specification effort. Compared to Option 1, Option 4 has a relatively smaller specification effort and overhead. On the other hand, eType II CB is only optionally supported by legacy UEs, thus, a UE with limited processing capabilities may not support eType II-based Option-4. Therefore, we think Option 1 and Option 4 are reasonable solutions and either or both can be considered in the specification.

EVM

In the RAN1#122 meeting, there is a FFS on the EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing Rel-18 study EVM as baseline.

Regarding the benchmark, we can reuse the Rel-18 SI agreement, i.e., FP32 is considered as upper bound.

Agreement (RAN1#111) For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison.

Regarding the metric, two alternatives are provided in below. At least Alt.2 should be considered and Alt.1 can be additionally provided for information.

- Alt.1 (SGCS between FP32 and quantized Target CSI): This option does not need to train and test the model, but only compares the Target CSI of FP32 with its quantized version. For example, for a Target CSI with FP32 (i.e., CSIFP32), a quantization method is adopted to derive CSIQT, and the SGCS is calculated between CSIFP32 and CSIQT.

- Alt.2 (SGCS between model input and model output): This option considers the model is trained with quantized Target CSI (CSIQT), and SGCS is derived by testing the model with model input and model output. For example, for the model trained with CSIQT, SGCS is calculated between FP32 (CSIFP32) as the model input and recovery CSI as the model output (CSIOut).

Moreover, regarding EVM for Option 1 and Option 4, the following aspects should be considered.

- For Option 1, the normalization method should be aligned for evaluation. For example, for a batch of S reported data samples each of which includes the Target CSIs for L layers/Rx antennas, N3 subbands, and 2*N1*N2 Tx ports, a strongest element is selected as the element with largest real/imaginary value over the S, L, N3, 2*N1*N2 floating elements, and the real/imaginary value of other floating elements are quantized to the range of (0, 1) with X (e.g., X=4,8,16) bits relative to the strongest element.

- For Option 4, the following new parameter set was adopted in Rel-18 evaluation and can be used as a reference: L= 8, 10, 12; pv = 0.8, 0.9, 0.95; beta = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

Proposal 1: For the NW side training data collection, for the quantization of Target CSI, consider the following EVM:

- Regarding the benchmark, FP32 is taken as the upper-bound.

- Regarding the metric, consider the following two alternatives, wherein Alt.2 is considered as baseline.

  - Alt.1: SGCS is calculated between the Target CSI of FP32 (CSIFP32) with its quantized version (CSIQT).

  - Alt.2: SGCS is calculated between Target CSI of FP 32 (CSIFP32) as the model input and the recovery CSI (CSIOut) as the model output of the model trained with Target CSI with quantized version (CSIQT).

- Regarding the EVM for Option 1, the normalization method should be aligned. For example, for a batch of S reported data samples each of which includes the Target CSIs for L layers/Rx antennas, N3 subbands, and 2*N1*N2 Tx ports, the real/imaginary value of each floating element is quantized in the range of (0, 1) relative to the strongest element over the S, L, N3, 2*N1*N2 floating elements.

- Regarding the EVM for Option 4, consider the enhanced parameters as: L= 8, 10, 12; pv = 0.8, 0.9, 0.95; beta = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

Evaluation results

The evaluation results are provided for Option 0, 1, and 4. Note that, regarding the analysis of the options above, both Target CSI types of precoding matrix (quantized per layer) and channel matrix (quantized per Rx antenna) are considered in our evaluations. Benefits of supporting both precoding and channel matrix as Target CSI are explained in our companion paper [2].

The following evaluations consider Option 1 at different quantization precision levels, with simulation assumptions detailed in the Appendix. Note that for Option 0/4, the size per subband is calculated by dividing the CSI feedback payload size with subband number.

The specific parameter combinations for Option 0 and Option 4 are given in below.

- Option 0 with PC6. The CSI payload size for per data sample is 279 bits.

- Option 0 with PC8. The CSI payload size for per data sample is 328 bits.

- Option 4 with New Parameter#1 {L=10, p=0.9, beta=0.31, reference amplitude: 6 bits, differential amplitude: 4 bits, phase quantization: 6 bits}: The CSI payload size of for per data sample is 1014 bits.

- Option 4 with New Parameter#2: {L=12, p=0.8, beta=0.5, reference amplitude: 6 bits, differential amplitude: 4 bits, phase quantization: 6 bits}: The CSI payload size for per data sample is 1610 bits.

Table 1 Option 0/1/4 SGCS results for precoding matrix as Target CSI (32 ports, 13 subbands)

|  | Size per subband per layer, Bytes | Alt.1(SGCS between FP32 and quantized Target CSI) | Alt.2 (SGCS between model input and model output, 240bits CSI feedback) |
| --- | --- | --- | --- |
| Option1/FP 32 (upper bound) | 256 | 1 | 0.918 |
| Option1/FP 16 | 128 (-50% OH) | 1 | 0.911 (-0.7%) |
| Option1/Scalar 8 | 64 (-75% OH) | 0.9998 (-0.02%) | 0.910 (-0.8%) |
| Option1/Scalar 4 | 32 (-87.5% OH) | 0.9306 (-6.94%) | 0.898 (-2%) |
| Option0/PC8 | 3.3 (-98.7% OH) | 0.9222 (-7.78%) | 0.887 (-3.1%) |
| Option4/NewParameter#1 | 10.2 (-96% OH) | 0.9452 (-5.48%) | 0.917 (-0.1%) |
| Option4/NewParameter#2 | 15.4 (-94% OH) | 0.9496 (-5.04%) | 0.917 (-0.1%) |

Table 2 Option 1 SGCS results for channel matrix as Target CSI (64 ports, 17 subbands)

|  | Size per RB per Rx, Bytes | Alt.1(SGCS between FP32 and quantized Target CSI) | Alt.2 (SGCS between model input and model output, 240bits CSI feedback) |
| --- | --- | --- | --- |
| Option1/FP 32 (upper bound) | 512 | 1 | 0.6138 |
| Option1/FP 16 | 256 (-50% OH) | 0.9999 (-0.01%) | 0.6055 (-0.83%) |
| Option1/Scalar 8 | 128 (-75% OH) | 0.9998 (-0.02%) | 0.6012 (-1.2%) |

From the evaluation results in the above tables, it can be noted that in Table 1, Option 1 and Option 4 outperform Option 0 on the SGCS performances regardless the KPI calculation method is Alt.1 or Alt.2. In addition, for Option 1, only negligible performance loss is observed as oppose to FP32 even when the quantization granularity is reduced down to Scalar 8 (8 bits for real and 8 bits imaginary) for both precoding matrix and channel matrix.

Proposal 2: For the NW side training data collection, for the quantization of Target CSI, consider to support scalar quantization (Option 1) or eType II like CB with new parameter values (Option 4).

### Need of Pairing ID

In the RAN1#122 meeting, it has been discussed [3] whether to indicate Pairing ID for NW side data collection.

### [Rd2] (Updated) Proposal 2.5a: Pairing ID associated with the exchanged dataset of Option 4-1, model parameters of Option 3a-1, and fully standardized reference model of Direction C, is considered for UE side data collection, model pairing procedure for inference (i.e., applicability inquiry and reporting), inference configuration, and NW-sided data collection. - FFS mandatory or optional - FFS NW-sided data collection - Note: For NW-sided data collection, Pairing ID (optionally configured) may be used, e.g., for NW side model fine-tuning, UE reports logged data along with the corresponding Pairing ID to facilitate NW side data categorization.

The argument to support indicating Pairing ID for NW side data collection is it saves the effort for NW side data management. For example, assuming UE logs the data for a long time before it reports, after NW side receives a batch of reported data, for any specific data, it does not know when it is generated, and what is the NW side additional condition at that historic time, if there is no tag to describe the data.

The counter argument, however, is that, for higher layer report, UE may additionally report the cell ID as well as time stamp, so that NW would be aware of the NW side additional condition.

From our view, considering Pairing ID is anyway indicated to UE for model pairing purpose, it would not cause any harm if it is additionally indicated for NW side data collection purpose.

Proposal 3: For the NW side training data collection, Pairing ID may be optionally indicated to UE.

- UE reports Pairing ID along with the corresponding Target CSI data in the higher layer reporting.

## UE side data collection

In the RAN1#122 meeting, the following agreement was achieved for UE side data collection.

Agreement: To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report. - P-CSI-RS and /SP CSI-RS are supported.

In our view, the main purpose of UE side data collection is UE side model training/update. It may be also useful to include Pairing ID(s) in the CSI-ReportConfig for UE side data collection. As a use case example, if a Pairing ID provided in CSI-ReportConfig is the same as the Pairing ID that is associated with an exchanged dataset#A during inter-vendor training collaboration, this implies that the same model that was trained using dataset#A needs to be updated and, based on UE-side implementation, the UE collected data may be augmented to dataset#A.

In addition, similar to NW side data collection, as Pairing ID is anyway indicated to UE for model pairing purpose, it would not cause any harm if it is additionally indicated for UE side data collection purpose.

Proposal 4: Pairing ID(s) can be optionally indicated in CSI-ReportConfig for UE side training data collection.

# Model pairing

In the RAN1#122 meeting, the following agreement was achieved for model pairing procedure.

Agreement: For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point. - Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3. - FFS whether/how to link the Pairing ID with the inter-vendor collaboration. - FFS whether/how Paring ID(s) are reported in Step 4. - FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only). - FFS CSI compression specific inference configuration or inference related parameters.

There are multiple FFSs in the agreement that need to be addressed.

## Linkage of Pairing ID with the inter-vendor collaboration

The first FFS enquires whether/how to link the Pairing ID with the inter-vendor collaboration. Note that, regardless of the inter-vendor collaboration directions, the shared Pairing ID from NW side to UE side during inter-vendor collaboration is implicitly associated with an encoder model: In Direction C, the Pairing ID is associated with a fully specified encoder (and decoder), in Direction A sub-Option 3a-1, the Pairing ID is associated with a set of parameters corresponding to the encoder, and in Direction A sub-Option 4-1, the Pairing ID is associated with the dataset used to train the encoder. Therefore, in Step 3 of applicability report procedure, NW can signal inference configuration (Option A) or set of inference related parameters (Option B) along with a Pairing ID that has been shared with the UE side during inter-vendor collaboration to enquire whether or not the encoder that is implicitly indicated by the Pairing ID is applicable with the provided inference configuration (Option A) or set of inference related parameters (Option B).

Regarding Direction C, it can be specified with a fixed Pairing ID.

Proposal 5: Support indicating the Pairing ID in Step 3 of applicability report procedure to enquire if the UE part model that is associated with the Pairing ID is applicable with the provided inference configuration (Option A) or set of inference related parameters (Option B).

- The Pairing ID refers to the exchanged dataset/exchanged model parameters/standardized reference model in the inter-vendor training collaboration.

- For Direction C, it can be specified with a fixed Pairing ID.

## Support of Option A and Option B

Regarding the third FFS, we may borrow the procedure for Rel-19 BM. For Rel-19, RAN2 has agreed that both Option A and Option B are supported for BM. In addition, Option B is not supported for CSI prediction due to lack of parameters provided by RAN1.

Agreements … 4 Introduce two conditional mandatory capabilities (with signaling) for AI/ML based BM Option A and Option B, if UE supports FG58-0-1 and/or FG58-1-2/3/4/5 (the details of those feature group depend on RAN1 progress). … 6 UAI is mandatory for both Option A and B

Agreements Other Config for option B 1 RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. 2 For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19. 3 For Option B for BM Case 1/2, inference related parameter set is configured per serving cell.

Agreements 2 The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1.

The major differences between Option A and Option B are summarized as follows.

For Option A, the full inference configurations are all provided within CSI-reportConfig in Step 3, and as long as UE reports ‘applicable’ state in RRCReconfigurationComplete message at Step 4 (for each CSI-reportConfigId), gNB can continue to further activate/schedule the corresponding CSI report with MAC CE/DCI. Therefore, there is no need for UE to wait for another RRCReconfiguration for CSI reporting configuration in Step 5.

For Option B, a set of parameters (referred by ApplicabilitySetConfig) which strongly impacts the AI/ML model applicability is indicated in Step 3 via OtherConfig. For example, for BM, the set of parameters include the pattern of Set A/Set B which impacts the input/output dimension, the associated ID which impacts the data property, number of predicted time instances which impacts the input/output dimension, etc. Afterwards, UE will feedback the ‘applicable’ or ‘inapplicable’ state in RRCReconfigurationComplete message in Step 4 for each ApplicabilitySetConfigId. Then, gNB will further configure CSI-reportConfig in Step 5 to actually enable the functionality.

(UE report at Step 4)

…

(gNB configuration at Step 3 for Option B)

Therefore, compared with Option A, Option B initializes with an applicability check for the UE side model. For Option A, if gNB cannot confirm whether the UE can support a CSI-reportConfig in the first place but blindly configure with the CSI-reportConfig, it may suffer the consequence of UE reporting ‘inapplicable’ with RRCReconfigurationComplete, and thus the CSI-reportConfig is wasted. Furthermore, if gNB wishes to reconfigure AI/ML based CSI reporting, it needs to always configure a new CSI-reportConfig without knowing whether the UE will support it or not in advance. If the UE replies with ‘inapplicable’, then it means the gNB cannot immediately receive the CSI report, and it needs to blindly try another CSI-reportConfig, causing longer CSI report latency. In contrast, for Option B, gNB may configure all candidate combinations of preferred parameters and receive their applicability status in one hand-shake.

The same principle of AI/ML BM should be applied to CSI compression, i.e., both Option A and Option B should be supported. RAN1 should identify the inference related parameters as soon as possible to RAN2 to avoid the same consequence of unsupported Option B for CSI prediction in Rel-19.

Proposal 6: Regarding the model pairing procedure for inference, support both Option A and Option B.

To discuss the second FFS “whether/how Pairing ID(s) are reported in Step 4”, note that the UE report in Step 4 is a reply to the NW enquiry in Step 3 and the inclusion of the Pairing ID in the report depends on whether there is any ambiguity if the report does not include the Pairing ID. We think this depends at least on how the NW enquiry in Step 3 is signaled to the UE.

For instance, if Option A is used and the inference configuration along with a single Pairing ID is included in the CSI-ReportConfig, one CSI-ReportConfigId will uniquely refer to one Pairing ID. Since it is clear for NW that the UE report in Step 4 corresponds to which CSI-ReportConfigId, inclusion of Pairing ID in the report seems unnecessary. For Option B, if one Pairing ID is embedded to per ApplicabilitySetConfig, then similar to Option A, the UE feedback on the applicability of ApplicabilitySetConfigId can also uniquely refer to one Pairing ID.

On the other hand, if NW can enquire about the applicability of multiple encoders (corresponding to multiple Pairing IDs) in the same ApplicabilitySetConfig, then it seems necessary to include Pairing IDs the Step 4 UE report.

In our view, one Pairing ID can be indicated per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B.

Proposal 7:  Regarding the model pairing procedure for inference, one Pairing ID can be indicated per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B.

- Reuse Rel-19 rule where the applicability state reported in Step 4 is per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B.

## Other issues for model pairing

In the RAN1#122 meeting, the following questions were discussed in the FL summary [3].

### [Rd1] Question 4.4: For model pairing procedure, other than Pairing ID, whether associated ID is additionally indicated to UE in Step 3 for representing NW side additional conditions?

### [Rd1] (Closed) Question 4.5: Other than For model pairing procedure, whether Pairing ID can be is additionally indicated to UE for monitoring purpose in monitoring procedure? - If so, is it used for NW side monitoring, or UE side monitoring?

In our view, there is no need to additionally indicate associated ID, since Pairing ID already represents the NW side additional condition.

Regarding whether to indicate Pairing ID for monitoring, considering the monitoring is associated to a specific inference CSI report which is already linked with a Pairing ID, there is no need to further indicate Pairing ID for monitoring.

Proposal 8:  Regarding the model pairing procedure for inference/monitoring:

- No need to additionally indicate associated ID to UE in Step 3.

- No need to additionally indicate Pairing ID for monitoring.

# Performance monitoring

In the RAN1#120bis meeting, the following conclusions on model monitoring were reached.

Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. Conclusion UE-side monitoring is feasible. Observation Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

In this section, we provide our views on NW side and UE side monitoring.

## NW side monitoring

For NW side monitoring, at least target CSI needs to be reported and, therefore, similar to NW side data collection, target CSI quantization format should be discussed. In our view, similar arguments that justify the use of SQ and eType II-like CB with new parameters for NW side data collection also apply to the NW side monitoring. Further, using the same target CSI quantization format for monitoring and data collection significantly reduces the standardization effort and the specification impact. Therefore, we propose the following:

Proposal 9: For NW side monitoring, reuse the same Target CSI quantization format(s) that are supported for NW side data collection.

Regarding report content and medium for NW side data monitoring, the following proposal was provided in the FL summary in RAN1#122 [3]:

### [Rd2] (Updated) Proposal 3.1a: For NW side monitoring, if supported, consider the higher layer signaling for reporting Target CSI. - Regarding the content - Option 1: Paired Target CSI and CSI feedback used for inference - Option 2: Target CSI only (assuming NW obtains CSI feedback via inference CSI report) - Details on signaling is up to RAN2 - Quantization format of Target CSI is the same as for NW side data collection. - Note: other implementation based solutions can also be adopted for monitoring, e.g. eventual KPI based, SRS based, legacy eType II via L1 signaling, etc.

Regarding the report content for NW side data monitoring, a consideration is that the Target CSI and CSI feedback resources should be the same, otherwise NW would not be able to correctly calculate SGCS between the reconstructed CSI and Target CSI. If L1 reporting is agreed for NW side monitoring, such an association between the Target CSI and CSI feedback resources can be maintained by NW side configuration of separate CSI reports. Therefore, only Target CSI is required to be reported for monitoring and NW can obtain CSI feedback via inference CSI report (Option 2). In turn, if L3 reporting is agreed, considering the Target CSI is reported after UE logging for a monitoring time window, Option 2 may not be applicable as NW would not know the feedback Target CSI corresponds to which reported resources during inference; thus, the paired Target CSI and CSI feedback are required to be reported together (Option 1).

Proposal 10: For NW side monitoring, support inclusion of the following content in the report:

- Paired Target CSI and CSI feedback if L3 reporting is supported

- Target CSI if L1 reporting is supported

  - Note: In this case, NW obtains the associated CSI feedback via inference CSI report.

## UE side monitoring

Proxy model based monitoring

Based on RAN1#117 agreement, this type of monitoring relies on the availability of output of the CSI reconstruction model at the UE (Case 2-1) or direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2). Both of Case 2-1 and Case 2-2 are based on a proxy model at the UE. Our understanding is that the proxy model has the following drawbacks:

- Generalization performance of the proxy model. Since the proxy CSI reconstruction model is different from the actual NW part CSI reconstruction model, e.g., with a smaller size/simpler architecture and weaker learning capability, the generalization performance of the proxy model will be typically worse than the actual NW part CSI reconstruction model.

- Additional LCM of the UE side proxy model. Considering the proxy model is used to monitor the performance of actual model, then a subsequent issue is how to monitor and manage the proxy model. Since the proxy model is one-sided model at the UE side, one potential way is that it is up to UE implementation with level x collaboration and, therefore, its performance can hardly be monitored or trusted by the NW side.

In summary, from monitoring perspective, the proxy model (either to mimic the CSI reconstruction part at NW or to directly output the intermediate KPI) can be operated under level x collaboration, and there is no need to consider specification impact on the proxy model.

Proposal 11: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring.

Precoded RS based monitoring

After NW recovers the CSI, it may use the recovered CSI to generate UE-specific precoded CSI-RS. Then, UE can use the precoded RS to calculate the eventual KPI (e.g., CQI, L1-RSRP, hypothetical BLER, etc) and directly report the eventual KPI to NW, e.g., based on “cri-RI-CQI” as legacy. Alternatively, UE may compare the AI/ML-based eventual KPI with a legacy non-AI/ML solution applied with the legacy precoder/PMI.

This option is overhead friendly for UL report, as it may avoid delivery of Target CSI from UE to NW. In addition, it is more reliable than proxy model based monitoring (Case 2-1/2-2). On the other hand, precoded RS based monitoring may reflect the eventual performance, but may not reflect the intermediate KPI (e.g., SGCS) of the model. Note that the eventual performance may not only be impacted by the accuracy of the model but also other factors such as scheduling, MU pairing, channel status, etc. Moreover, this method is applicable only when the channel state does not change rapidly. Otherwise, considering there is a time gap between the time slot when the UE feeds back the Target CSI and the time slot when UE calculates the KPI, the accuracy of the calculated KPI may be degraded if the channel state changes significantly in between.

In summary, compared with proxy model based monitoring, precoded RS based monitoring may be a more reliable method for UE side monitoring, but it cannot work in a standalone manner without NW side monitoring.

Proposal 12: Precoded RS based monitoring should not work as the only option without NW side monitoring.

# Conclusions

In this paper we make the following proposals:

Proposal 1: For the NW side training data collection, for the quantization of Target CSI, consider the following EVM:

- Regarding the benchmark, FP32 is taken as the upper-bound.

- Regarding the metric, consider the following two alternatives, wherein Alt.2 is considered as baseline.

  - Alt.1: SGCS is calculated between the Target CSI of FP32 (CSIFP32) with its quantized version (CSIQT).

  - Alt.2: SGCS is calculated between Target CSI of FP 32 (CSIFP32) as the model input and the recovery CSI (CSIOut) as the model output of the model trained with Target CSI with quantized version (CSIQT).

- Regarding the EVM for Option 1, the normalization method should be aligned. For example, for a batch of S reported data samples each of which includes the Target CSIs for L layers/Rx antennas, N3 subbands, and 2*N1*N2 Tx ports, the real/imaginary value of each floating element is quantized in the range of (0, 1) relative to the strongest element over the S, L, N3, 2*N1*N2 floating elements.

- Regarding the EVM for Option 4, consider the enhanced parameters as: L= 8, 10, 12; pv = 0.8, 0.9, 0.95; beta = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

Proposal 2: For the NW side training data collection, for the quantization of Target CSI, consider to support scalar quantization (Option 1) or eType II like CB with new parameter values (Option 4).

Proposal 3: For the NW side training data collection, Pairing ID may be optionally indicated to UE.

- UE reports Pairing ID along with the corresponding Target CSI data in the higher layer reporting.

Proposal 4: Pairing ID(s) can be optionally indicated in CSI-ReportConfig for UE side training data collection.

Proposal 5: Support indicating the Pairing ID in Step 3 of applicability report procedure to enquire if the UE part model that is associated with the Pairing ID is applicable with the provided inference configuration (Option A) or set of inference related parameters (Option B).

- The Pairing ID refers to the exchanged dataset/exchanged model parameters/standardized reference model in the inter-vendor training collaboration.

- For Direction C, it can be specified with a fixed Pairing ID.

Proposal 6: Regarding the model pairing procedure for inference, support both Option A and Option B.

Proposal 7:  Regarding the model pairing procedure for inference, one Pairing ID can be indicated per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B.

- Reuse Rel-19 rule where the applicability state reported in Step 4 is per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B.

Proposal 8:  Regarding the model pairing procedure for inference/monitoring:

- No need to additionally indicate associated ID to UE in Step 3.

- No need to additionally indicate Pairing ID for monitoring.

Proposal 9: For NW side monitoring, reuse the same Target CSI quantization format(s) that are supported for NW side data collection.

Proposal 10: For NW side monitoring, support inclusion of the following content in the report:

- Paired Target CSI and CSI feedback if L3 reporting is supported

- Target CSI if L1 reporting is supported

  - Note: In this case, NW obtains the associated CSI feedback via inference CSI report.

Proposal 11: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring.

Proposal 12: Precoded RS based monitoring should not work as the only option without NW side monitoring.

# References

Report of RAN1#122, Chairman, August 25 – 29, 2025.

R1-2506931, “Discussion on Inference related aspects for CSI compression,” Huawei, HiSilicon, RAN1#122bis.

R1-2506485, FL summary#5 for other aspects of CSI compression, Moderator (Huawei), RAN1#122.

# Appendix: Simulation Assumptions

Table A.1 Evaluation assumptions for results in Table 1

| Parameter | Value |
| --- | --- |
| Duplex | FDD |
| Carrier frequency | 2 GHz |
| Antenna configuration of gNB | 32 ports: (8,8,2,1,1,2,8) |
| Simulation bandwidth | 10MHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| BS Tx power | 44dBm |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |

Table A.2 Evaluation assumptions for results in Table 2

| Parameter | Value |
| --- | --- |
| Duplex | TDD |
| Carrier frequency | 4 GHz |
| Antenna configuration of gNB | 64 Tx ports: (12,8,2,1,1,4,8) |
| Antenna configuration of UE | 4Rx: (1,2,2,1,1,1,2) |
| Simulation bandwidth | 100MHz, 272PRBs |
| SCS | 30kHz |
| SRS configuration | 10/20/40ms period, 17 hops |
| CSI payload size (for Test 2/3) | 120 bits per layer/Rx antenna |
| CSI feedback periodicity | 20ms |
| gNB maximum Tx power | 51dBm |
| UE maximum Tx power | 23dBm |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |