3GPP TSG RAN WG1 #122bis			R1-2506964

Prague, Czech, Oct 13th – 17th, 2025

Source:	Xiaomi

Title:	Discussion on remaining issues for Ambient IoT in NR

Agenda:	8.4

Document for:	Decision

- Introduction

In RAN1#121 meeting [1], the normative work for Rel-19 Ambient IoT in RAN1 has been announced finished, and the latest version of specification in physical layer of TS 38.291 [2] has also been completed. By checking all the RAN1’s and RAN2’s agreements, one issue was found, which needs to be discussed. This contribution discusses about the remaining issue of Rel-19 A-IoT.

- Discussion

  - Issue 1: CRC attachment for Access Trigger message

For R2D CRC attachment in TS 38.291 [2], the parity bits are computed and attached to the transport block by setting the number of parity bits to 16 bits if the payload size > 24, and setting the number of parity bits to 6 bits otherwise. According to RAN2’s design on Access Trigger message in TS 39.391 [3], the field R2D Message Type in this message with length of 3 bits. Even the payload size of R2D is half length of CRC length, some companies thinks that it is too short to attach CRC-6 due to high overhead. However, it still works based on the CRC attachment principle according to current specification. There is no necessary to change the specification as optimization.

Observation 1: For the new R2D message Access Trigger message with length of 3 bits, RAN1 specification of CRC attachment principle can work.

Proposal 1: RAN1 will no longer conduct CRC optimization in Rel-19 A-IoT (e.g., introduce cases of no CRC attachment) or TP discussions on Access Trigger message.

- Conclusion

This contribution discusses about the remaining issue in Rel-19 A-IoT, and the observation and proposal are concluded as:

Observation 1: For the new R2D message Access Trigger message with length of 3 bits, RAN1 specification of CRC attachment principle can work.

Proposal 1: RAN1 will no longer conduct CRC optimization in Rel-19 A-IoT (e.g., introduce cases of no CRC attachment) or TP discussions on Access Trigger message.

- Reference

- Chairman’s notes, 3GPP RAN1#121 meeting, St Julian’s, Malta, May 19th – 23rd, 2025.

- TS 38.291 (v19.0.0), NR Ambient IoT Physical layer (Release 19), June, 2025.

- TS 38.391 (v1.0.0), NR Ambient IoT Medium Access Control Protocol specification (Release 19), September, 2025.