3GPP TSG RAN WG1 #122bis		R1-2507162

Prague, Czech Republic, October 13th – 17th, 2025

Source:	OPPO

Title:	Discussions on UE features for NR mobility enhancements

Agenda Item:	9.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we will provide our views on the UE features for rel-19 NR mobility enhancements including L1-RSRP based on CSI-RS, LTM event triggering L1 measurement reporting and CSI acquisition of candidate cell.

- Discussions on UE features

For CSI-RS based beam measurement, the UE measures the CSI-RS resource for beam management of one or more candidate cells. Thus, the maximum number of CSI-RS resources for L1-RSRP measurement of each candidate cell shall be up to UE capability. And the UE is configured to measure multiple candidate cells simultaneously. Thus, the maximum number of CSI-RS resources of all candidate cells shall be UE capability too.

Therefore, the FGs 63-1 and 63-2 shall be updated as follows:

Proposal 1: Add two more items in FGs 63-1 and 63-2 as follows:

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource. 5. Maximum number of periodic CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 6. Maximum total number of periodic CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement. | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource 3. Maximum number of semi-persistent CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 4. Maximum total number of semi-persistent CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement. | 63-1 | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |

- Conclusion

In this contribution, we discuss the UE features for Rel-19 mobility with the following proposals:

Proposal 1: Add two more items in FGs 63-1 and 63-2 as follows:

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource. 5. Maximum number of periodic CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 6. Maximum total number of periodic CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement. | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource 3. Maximum number of semi-persistent CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 4. Maximum total number of semi-persistent CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement. | 63-1 | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |