3GPP TSG RAN WG1 #122bis		R1-2507280

Prague, Czech, Oct 13th – 17th, 2025

Source: 	Fujitsu

Title:	Discussion on inference related aspects in CSI compression with AI/ML

Agenda item:	10.1.1.1

Document for:	Discussion

Introduction

In RAN#108 plenary meeting, new WI for the normative support of CSI feedback enhancement (two-sided model) was agreed. In this contribution, we will provide views on inference aspects of CSI feedback enhancement based on WID below [1].

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air, including: - … - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports …

Discussion

Target CSI type

In RAN1#122 meeting, the following agreement was made for targe CSI type.

Agreement For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

For the target CSI type of precoding matrix, there are two alternatives for CSI compression Case 0:

- Alt.1, precoding matrix in spatial-frequency domain.

- Alt.2, precoding matrix represented using angular-frequency or angular-delay domain projection.

In our view, Alt.1 should be supported at least. The reason is as follows.

Alt.1 is easier to implement than Alt.2 since UE/NW side should perform additional pre-processing/post-processing for Alt.2. Besides, the study on Alt.2 was not sufficient in either Rel 18 or Rel-19.

For the codebook subset restriction (CBSR), Alt.2 may have the advantage but Alt.1 could also performed via Fourier transform. For example, the precoding vector over spatial-frequency domain is converted to the angle-frequency domain, and some coefficients over this domain are removed based on CBSR; Finally, the precoding vector is converted back to spatial-frequency domain as the input of AI/ML encoder.

Proposal 1:

- For CSI compression Case 0, RAN1 to consider the precoding matrix in spatial-frequency domain as target CSI.

CQI/RI determination

In the RAN1 #112 meeting [3], the following agreement was reached regarding CQI determination.

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including: - Option 1a: CQI is calculated based on target CSI with realistic channel measurement. - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment. - Option 1c: CQI is calculated based on legacy codebook. - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including: - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment. - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded. - Note1: Feasibility of different options should be evaluated. - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated. - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead.

For Option 1a, CQI is calculated based on target CSI without adjustment. Since the reconstruction error is not considered at UE side, CQI would be overestimated in the CSI report. However, this issue might be solved by Option 1b or open loop link adaptation (OLLA) at NW side.

Among the options, our view is that Option 2 should be deprioritized. The reasons are as follows. It is not expected to have AI/ML-based CSI reconstruction part at UE, neither the real CSI reconstruction part, nor a proxy decoder. It is not practical to assume the true CSI reconstruction part be available at UE. Firstly, for the proprietary issue, the NW vendors don’t want to disclose the design of the CSI reconstruction part. Secondly, running the real CSI reconstruction part at the UE side leads to higher computational complexity, more storage and more power consumption. The proxy decoder at the UE side may not be a good idea either. The concern is mainly about the reliability of the output of the proxy decoder. Specifically, the performance of the proxy decoder cannot be guaranteed, and the reported CQI may not be trustable to the NW. As a result, we should not assume that an AI/ML-based CSI reconstruction part is available at the UE side.

In addition, the two-stage approach in Option 2b is also not recommended. The main concern is the large delay it suffers. Specifically, the channel aging issue may be severe such that the CQI recommended is not meaningful for the NW scheduler.

Since both two alternatives in Option 2 have practical issues, which cannot be solved. We propose to prioritize Option 1.

Proposal 2:

- For CSI compression Case 0, prioritize Option 1a and Option 1b for CQI/RI determination.

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement.

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment.

quantization configuration/codebook

In RAN1#122 meeting, the following agreements were agreed for quantization configuration/codebook.

| Agreement Specify the precoding matrix feedback via two-sided model as follows: For a certain layer l and rank ν, - A precoding matrix is mapped with a latent message zl=z0l,z1l,⋯,zdzl,ν-1l, with l = 1, ..., v where l is the layer index. - The mapping is subject to [at least] a pairing ID configured by high-layer signalling. - FFS dzl,ν is common or specific for different layers or ranks - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further - zl is quantized and mapped to bit sequence before reporting. Agreement For precoding matrix feedback, for a certain layer l of rank ν, the latent message zl contains dzl,ν real values - If scalar quantization (SQ) supported, it refers to the quantization of each real value independently, - The payload size for layer l and rank ν is determined by dzl,ν×Ql,ν, where Ql,ν is the number of bits per scalar. - If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized. - The payload size for layer l and rank ν is determined by dzl,νL×Ql,ν, where L is the length of the vector segment, Ql,ν is the number of bits per segment - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded. Agreement: For precoding matrix feedback, consider the following aspects for payload determination - Support only SQ, or only VQ or both - dzl,ν and / or Ql,ν and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting. - dzl,νand / or Ql,ν is layer-common or layer-specific, rank-common or rank-specific. - If configured, dzl,νand / or Ql,ν and / or L above are configured separately from pairing ID, or configured by Pairing ID. - The configuration may be via other parameters which are used to derive dzl,ν, L and Ql,ν. Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges. Note: dzl,ν, Ql,ν and L are independent on subband and port configurations, - FFS each combination of dzl,ν, Ql,ν and L is applicable to a certain set of subband and port configurations. |  |
| --- | --- |

The formulation of quantization codebook/configuration has been defined by the parameter combination of {dzl,ν ,Ql,ν, L }. In our view, these parameter combinations should be specified and associated with a standardized configuration. For a certain set of sub-band and port configurations, three kinds of combinations could be considered at least which corresponds to low/mediate/high resolutions/payloads.

Proposal 3:

- For quantization codebook/configuration, RAN1 to specify multiple parameter combinations of {dzl,ν ,Ql,ν, L }.

  - Three combinations can be a starting point which corresponds to low/mediate/high resolutions/payloads.

With the consideration of model scalability, one AI/ML model may correspond to multiple parameter combinations of quantization codebooks/configurations. Therefore, it is more feasible to configure {dzl,ν ,Ql,ν, L } separately from pairing ID, e.g., by CSI report configuration.

Proposal 4:

- With the consideration of model scalability, the parameter combination of {dzl,ν ,Ql,ν, L } should be configured separately from pairing ID.

For the quantization codebook exchange, one agreement was made as follows.

Agreement: For quantization codebook used for quantizing the feedback, - For inter-vendor collaboration Direction A, select one of the following: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Alt2: Use a standardized quantization codebook decided by RAN1.

From our perspective, Alt1 should be supported for inter-vendor collaboration Direction A.

Firstly, the distribution of latent space should be different for different UE/NW side conditions. There is a risk of performance degradation for a fixed/standardized quantization codebook in Alt2. Secondly, one advantage of inter-vendor collaboration Direction A is reducing the risk of performance degradation in field since the model parameters are explicitly or implicitly shared from NW side to UE side instead of fixed/standardized parameters. The same logic should be applied to quantization codebook.

Proposal 5:

- For inter-vendor collaboration Direction A, RAN1 to support Alt.1 for quantization codebook.

  - Alt.1: Exchange quantization codebook from NW side to UE side along with exchanged dataset or model parameters.

UCI mapping

CSI Part 1 and Part 2

One issue that needs to be considered for UCI mapping (order) is UCI partition. The CSI compression Case 0 should follow the legacy framework of Part 1 and Part 2. Part 1 has a fixed payload size and Part 2 has a dynamic payload size which is indicated by the fields in Part 1. Since there is no non-zero coefficient (NZC) for AI/ML, the payload size of Part 2 should be only dependent on RI in Part 1 and the standardized configuration for payload sizes.

In the legacy codebook, NW side only configures the maximum payload size and UE side can further perform the down-selection for NZCs if their values are too small. However, for AI/ML-based CSI feedback, given a rank value, we don’t see the necessity of payload size re-determination at UE side.

Proposal 6:

- For CSI compression Case 0, UCI mapping should follow the legacy framework of CSI Part 1 and Part 2, and the output of AI encoder should be included in CSI Part 2.

- The payload size of CSI Part 2 is determined by NW configuration and the rank value.

Group 0/1/2 of CSI Part 2

The other issue for UCI mapping is the order of Part 2, saying the definition of group 0/1/2. In legacy design, wideband related information is included in group 0 and sub-band related information is included in group 1/2. However, due to the black-box effect, we may not explicitly distinguish wideband and sub-band information, even/odd sub-band information for the output of CSI generation part if there is no specific design. Besides, only one group is also a solution for AI/ML-based CSI feedback.

RAN1 should study the necessity and feasibility of grouping the output of CSI generation part for CSI Part 2.

Proposal 7:

- For CSI Part 2, RAN1 to study the necessity and the feasibility of grouping the output of AI encoder.

  - FFS: the number of groups.

Priority rules for CSI reports

CSI collision handling

For calculating priority values , the legacy method could be reused for AI/ML-based CSI feedback. Considering that the AI/ML method could always fall back to the legacy method if the performance is degraded, the AI/ML-based report should have a high priority over the codebook-based report. It could be achieved by configuring a small value of reportConfigID for CSI report.

Proposal 8:

- The legacy calculation of priority values could be reused for AI/ML-based CSI feedback.

- The AI/ML-based CSI report should have a higher priority over the codebook-based CSI report, which could be realized by implementation.

CSI omission

When CSI reporting on PUSCH comprises two parts, UE may omit a portion of CSI Part 2 (group 0/1/2). However, as mentioned before, we may not interpret the physical meaning for the output of AI encoder, and the CSI omission may or may not be feasible.

One way to address this issue is only defining group 0 for AI/ML-based CSI feedback. So, UE could only omit or keep the whole of Part 2. The other way to address this issue is grouping the output of AI encoder based on some rules, such as layer/subband-based method or payload scalability-based method. RAN1 should study the feasibility of grouping the output of AI encoder.

Conclusion

For CSI compress Case 0, we have the following proposals for inference aspects.

Proposal 1:

- For CSI compression Case 0, RAN1 to consider the precoding matrix in spatial-frequency domain as target CSI.

Proposal 2:

- For CSI compression Case 0, prioritize Option 1a and Option 1b for CQI/RI determination.

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement.

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment.

Proposal 3:

- For quantization codebook/configuration, RAN1 to specify multiple parameter combinations of {dzl,ν ,Ql,ν, L }.

  - Three combinations can be a starting point which corresponds to low/mediate/high resolutions/payloads.

Proposal 4:

- With the consideration of model scalability, the parameter combination of {dzl,ν ,Ql,ν, L } should be configured separately from pairing ID.

Proposal 5:

- For inter-vendor collaboration Direction A, RAN1 to support Alt.1 for quantization codebook.

  - Alt.1: Exchange quantization codebook from NW side to UE side along with exchanged dataset or model parameters.

Proposal 6:

- For CSI compression Case 0, UCI mapping should follow the legacy framework of CSI Part 1 and Part 2, and the output of AI encoder should be included in CSI Part 2.

- The payload size of CSI Part 2 is determined by NW configuration and the rank value.

Proposal 7:

- For CSI Part 2, RAN1 to study the necessity and the feasibility of grouping the output of AI encoder.

  - FFS: the number of groups.

Proposal 8:

- The legacy calculation of priority values could be reused for AI/ML-based CSI feedback.

- The AI/ML-based CSI report should have a higher priority over the codebook-based CSI report, which could be realized by implementation.

Reference

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Prague, Czech Republic, June 9-13, 2025

- 3GPP RAN1 chair’s notes, RAN WG1 #113, May 2023.

- 3GPP RAN1 chair’s notes, RAN WG1 #112, March 2023.

- 3GPP RAN1 chair’s notes, RAN WG1 #120bis, April 2025.

- 3GPP RAN1 chair’s notes, RAN WG1 #122, August 2025.