3GPP TSG-RAN WG1 #122bis	R1-2507554

Prague, Czech, Oct 13th – 17th, 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on coverage enhancement

Document for:        	Discussion and Decision

Agenda Item:         	10.4.1 - Coverage enhancement

- Introduction

A work item on coverage enhancements for NR phase 3 with the following objective has been approved [1].

The detailed objectives for coverage enhancement of this work item include: - Specify following PRACH coverage enhancements [RAN1, RAN2] - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure. - UE may receive UL beam information after transmission of MSG1 - UL beam information is to assist the UE decision on Msg3 beam selection. - Note 1: “different Tx beams” is for the purpose of future RAN1 discussions - Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable. - Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable. - Note 4: The PRACH repetitions are transmitted over ROs associated with the same SSB. - Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18 - Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI [RAN1, RAN2] - Specify enhancements to improve PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables [RAN1]

In this paper, we present our perspective for enhancing PRACH and Msg3 transmission, PUSCH repetition scheduled by DCI 0_0 with C-RNTI, and the extension of π/2-BPSK to additional MCS entries.

- Discussion

  - Multiple PRACH transmissions with different beams

In Rel-18, PRACH repetition with same beams was supported. A UE can transmit multiple PRACHs with same beams across PRACH occasions associated with the same SSB. In Rel-20, PRACH repetition with different beams is considered.

PRACH resource configuration

It can be discussed whether PRACH resources should be differentiated between PRACH repetition with different beams and PRACH repetition with same beams. We think such differentiation is necessary because the gNB needs to provide information for Msg3 beam selection only to UEs that transmit PRACHs with different beams. If differentiation of PRACH resources is supported, two possible approaches can be considered: differentiation by ROs or by PRACH preambles. The first approach (by ROs) may require a different RO determination procedure for PRACH repetition with same beams, which could have an impact on the specifications. The second approach (by PRACH preambles) may lead to further fragmentation of PRACH preambles, while the impact on the specifications may be kept relatively small by reusing the existing FeatureCombinationPreambles framework.

Proposal 1:	Support differentiation of PRACH resources between PRACH repetition with different beams and PRACH repetition with same beams.

- PRACH resources can be differentiated by ROs and/or PRACH preambles

  - Interaction between multiple PRACH transmissions and Msg3

With PRACH repetition using different beams, the gNB can provide assistance information for beam selection to UEs that do not support beam correspondence without Tx beam sweeping. The beam information can be indicated via RAR, and the UE can determine the spatial domain filter for subsequent UL transmissions (e.g., Msg3) based on the indicated beam information.

Indication for Msg3 beam selection

One of the issues regarding beam indication is how to indicate the beam information. One approach is implicit indication, where the beam information is conveyed via the RA-RNTI associated with each PRACH occasion. The other approach is explicit indication, where the beam information is provided by a dedicated field in the RAR. In the PRACH repetition procedure of Rel-18, the RA-RNTI are associated with the last valid PRACH occasion in the RO group. Consequently, the implicit approach may require modifications to the existing RAR monitoring procedure. On the other hand, the explicit approach would require introducing a new field in the RAR for beam indication.

Proposal 2:	For beam indication for Msg3 transmission, the following methods can be considered:

- Option 1: Implicit indication using RA-RNTI

- Option 2: Explicit indication using a dedicated field in RAR

Another issue is whether beam information can also be conveyed via DCI 0_0 with TC-RNTI when scheduling Msg3 retransmission. In our perspective, the gNB may not have any useful information for beam selection other than the information that was provided in the RAR, since it does not receive any UL signals from the UE other than PRACH repetition. Therefore, the benefit of indicating beam information via DCI 0_0 with TC-RNTI may be limited.

Proposal 3:	RAN1 to discuss whether to support beam indication via DCI 0_0 with TC-RNTI for Msg3 retransmission.

Coupling of Msg3 beam selection and Msg3 repetition

It may be worth discussing the coupling of Msg3 beam selection and Msg3 repetition which is introduced in Rel-17. Transmitting Msg3 PUSCH multiple times using the indicated best beam could enhance Msg3 coverage. One issue when supporting both features is how to indicate both the beam information and the number of repetitions simultaneously. In Msg3 repetition, the number of repetitions is indicated by the MCS field in the UL grant of the RAR. Therefore, the MCS field cannot be reused for beam indication. A straightforward approach would be to introduce a new field in the RAR for beam indication.

Proposal 4:	RAN1 to study mechanisms for coupling of Msg3 beam selection and Msg3 repetition.

  - PUSCH repetition via DCI 0_0 with C-RNTI

During the random access procedure, both PRACH and Msg3 PUSCH already support repetition. However, Msg5 PUSCH, which is scheduled by DCI 0_0 with C-RNTI prior to dedicated RRC signaling, does not support repetition. Therefore, enabling repetition for Msg5 PUSCH would be beneficial for coverage enhancement.

Mechanisms for Msg5 repetition

We think that the mechanisms used for Msg3 retransmission with repetition, which is scheduled by DCI 0_0 with TC-RNTI, can be reused for Msg5 repetition. These mechanisms are summarized below.

- Indication of number of repetitions

The number of repetitions and the MCS index are indicated by the MCS field in the DCI. If a UE requests Msg3 repetition, the MCS field is repurposed to indicate both the number of repetitions and the MCS index. The 2 MSBs indicate the number of repetitions, while the 3 LSBs indicate the MCS index.

- Determination of redundancy version (RV)

The RV ID is indicated by the RV field in the DCI, and RV cycling follows the rules specified in Table 6.1.2.1-2 of TS 38.214.

- Determination of available slots

The number of repetitions is counted based on the available slots for Type A PUSCH repetition. These available slots are determined by tdd-UL-DL-ConfigurationCommon, ssb-PositionsInBurst, and the TDRA field value in the DCI.

- Frequency hopping

The enable/disable status of frequency hopping is indicated by the frequency hopping flag field in the DCI. Intra-slot frequency hopping is not supported, whereas inter-slot frequency hopping is supported. For inter-slot frequency hopping, the RB offset determination follows the rules specified in Table 8.3-1 of TS 38.213, and the frequency hopping pattern follows the rules specified in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A.

Proposal 5:	The mechanisms for Msg3 retransmission with repetition can be considered as a baseline for Msg5 repetition in the following aspects:

- Indication of number of repetitions and MCS index

- Determination of redundancy version and available slots

- Frequency hopping

UE capability reporting for Msg5 repetition

To ensure that the gNB applies Msg5 repetition only to UEs that support this feature, a mechanism for UE capability reporting before scheduling Msg5 is required. A simple solution is for UEs to report their capability via Msg3 PUSCH transmission. This approach is already specified for Msg4 HARQ-ACK PUCCH repetition in NTN and, therefore, could be implemented with minimal impact on the specification.

Proposal 6:	The UE capability for supporting the Msg5 repetition feature can be reported via Msg3 PUSCH transmission.

  - Extending π/2 BPSK to more MCS entries

In the current specification of TS 38.214, π/2-BPSK is supported only for MCS 0 and MCS 1. By extending π/2-BPSK to additional MCS entries in the MCS tables, higher data rates can be achieved while maintaining low PAPR. We think that the extended MCS entries could be supported for UEs in RRC_CONNECTED mode, as the need for dynamic switching of MCS tables may be low during the random access procedure.

- Summary and proposal

In summary, the followings were proposed:

Proposal 1:	Support differentiation of PRACH resources between PRACH repetition with different beams and PRACH repetition with same beams.

- PRACH resources can be differentiated by ROs and/or PRACH preambles

Proposal 2:	For beam indication for Msg3 transmission, the following methods can be considered:

- Option 1: Implicit indication using RA-RNTI

- Option 2: Explicit indication using a dedicated field in RAR

Proposal 3:	RAN1 to discuss whether to support beam indication via DCI 0_0 with TC-RNTI for Msg3 retransmission.

Proposal 4:	RAN1 to study mechanisms for coupling of Msg3 beam selection and Msg3 repetition.

Proposal 5:	The mechanisms for Msg3 retransmission with repetition can be considered as a baseline for Msg5 repetition in the following aspects:

- Indication of number of repetitions and MCS index

- Determination of redundancy version and available slots

- Frequency hopping

Proposal 6:	The UE capability for supporting the Msg5 repetition feature can be reported via Msg3 PUSCH transmission.

- References

[1] RP-252824, “Revised WID on Coverage enhancements for NR Phase 3”, China Telecom, 2025.