3GPP TSG RAN WG1 #122bis			R1-2507804

Prague, Czech, Oct 13th – 17th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on the inter-vendor training collaborations for two-sided AI/ML models

Agenda Item:	10.1.2

Document for: 	Discussion and Decision

- Introduction

At the RAN Plenary #108 meeting, the New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements [1] was approved. The working scope for the inter-vendor training collaboration for two-sided models is as follows,

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

  - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

This contribution discusses the necessary specification support for the inter-vendor training collaborations.

- Specification support for pairing ID

At the RAN1 #122 meeting, the following was agreed upon about the pairing ID [2].

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange. - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the pairing ID needs further studied. - FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

Pairing ID assignment

One of the remaining issues from the last meeting is the assignment and uniqueness of the pairing ID. This issue applies not only to Option 4-1 but also to all three inter-vendor options. The assignment and uniqueness of the pairing should be designed taking all three options into account, and a unified framework for model pairing with Direction A (Option 3a-1 and 4-1) and Direction C is preferred.

For Direction C (Option 1), because the reference model for Direction C will be specified, a unique ID can be allocated for each specified model. These IDs are naturally globally unique and are assigned in the specifications. To accommodate these IDs into a unified pairing ID framework, a range of pairing ID space can be reserved for Direction C, as demonstrated in the example shown in Table 1.

Table  1: An example of pairing ID allocation for Directions A and C.

| ID | Assignment |
| --- | --- |
| 0 ~ X | Used/Reserved for the specified reference model. |
| X+1 ~ Y | Unique per MNO, notified to UE during dataset or model parameter exchange. |

For Direction A, the pairing ID is assigned by NW and notified to the UE. A key issue is within what scope the UE should treat the two IDs as the same one if their values are the same. Since NW is deployed and operated by MNOs, who master the information about the deployment environments and network device characteristics, MNO can assign the pairing ID and ensure that the UE can use the same model for a pairing ID when the UE is in the same MNO’s network. Therefore, RAN1 should study and specify the UE behaviors under the assumption that the pairing ID is assigned by MNO and it is unique per MNO.

Proposal 1

- Support the following pairing ID assignment for Direction C and Direction A, respectively.

  - For Direction C (Option 1), the ID is specified for each specified reference model.

  - For Direction A, the ID, which is different from the ones for Direction C, is assigned by NW.

    - The ID is notified to the UE during the exchange of datasets or model parameters, as per the Rel-19 study.

    - The details of ID assignment are up to RAN2. RAN1 specifies the UE behaviors based on the assumptions that the ID is unique per MNO.

Pairing ID validation/expiry

After a pairing ID is notified with model parameters or datasets, the UE/UE-side will develop the model that can be paired with NW. These models will be stored in the UE or UE-side servers.

In practical deployment, NW can upgrade its AI/ML implementations based on the evolution of AI/ML technologies or the collection of more data in the deployment scenarios. After that, NW can notify new or existing UEs with a new pairing ID and corresponding model parameters or datasets. After all UEs upgrade their models, the old one and its pairing ID can be abandoned.

However, although the old model and ID will never be used again, the UE side will store them since it does not have the information about whether this model will be used again, and the ID is also occupied to prevent future usage.

To resolve this issue, a mechanism is necessary to release the pairing ID, along with the corresponding models and datasets stored by the UE/UE-side. For example, a validation duration can be introduced, and RAN1 can further study the corresponding UE behaviors.

Proposal 2

- Support the mechanism to release the pairing ID, e.g., with a pairing ID validation duration.

  - RAN1 sends an LS to RAN2 (and/or other related WGs) to specify the detailed mechanism.

  - RAN1 studies the UE behaviors when a pairing ID becomes invalid or is released.

- Specification support for dataset exchange

Target CSI format

At the RAN1 #122 meeting, the agreements on the target CSI format for dataset exchange are the following [2],

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format - FFS: CSI feedback type and format - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

Based on the discussions in 10.1.1.2, we should pursue a unified format for the target CSI, especially between the data collection and dataset format, because both target CSI data are for offline processing, and the collected target CSI is the source of the exchanged dataset. Therefore, we have the following proposal,

Proposal 3

- Target CI format reuse the same format as NW-side data collection for training.

Performance target type

At the RAN1 #122 meeting, the agreements on the performance target for dataset exchange are the following [2],

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, consider the following methods for performance target in the exchanged dataset, - SGCS - FFS: Average SGCS - FFS: SGCS values at X-percentiles - NMSE: - FFS: When the exchanged CSI feedback is the floating-point values at the input of quantization - FFS: When the exchanged CSI feedback is the binary bit sequence at the output of quantization, the binary sequence will be mapped back to the floating-point values via quantization codebook - FFS: Multiple performance targets for different layer when the target CSI type is precoding matrix, different configurations such as antenna ports, subband configuration and payload size configuration

The SGCS is a type of end-to-end performance target. The UE that supports it should also train the CSI reconstruction part. Generally speaking, it is up to the UE implementation to decide whether and how it trains the CSI reconstruction part, as this is a training technique issue. However, when the performance target and the corresponding UE applicability report are involved, there should be a common understanding about the SGCS exchanged between NW and UE. Similar to the UE-side performance monitoring with a proxy model, the UE-trained CSI reconstruction part cannot match the NW implementation in the practical deployment, which introduces bias and error in the performance target.

For this reason, the NMSE type of performance target is more reasonable, as it only considers the UE-side implementations. Therefore, we have the following proposal,

Proposal 4

- Support the NMSE as the performance target.

CSI feedback format

At the RAN1 #122 meeting, the agreements on the CSI feedback for dataset exchange are the following [2]. The down selection between two options will be discussed.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options: - Option 1: The exchanged CSI feedback is the latent message before quantization. - Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization. - Note: Quantization codebook is exchanged in addition if not specified in the spec. - Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

The two options are, in principle, the same because any piece of information exchanged via the network is ultimately quantized into bits. Given that the quantization codebook is specified or exchanged as indicated in the note, Option 2 reflects reality. Therefore, Option 2 is a natural choice for this issue.

Proposal 5

- Option 2 is naturally the CSI feedback format and should be supported. No need to further discuss this issue.

- Conclusion

In this contribution, we have the following proposals,

Proposal 1

- Support the following pairing ID assignment for Direction C and Direction A, respectively.

  - For Direction C (Option 1), the ID is specified for each specified reference model.

  - For Direction A, the ID, which is different from the ones for Direction C, is assigned by NW.

    - The ID is notified to the UE during the exchange of datasets or model parameters, as per the Rel-19 study.

    - The details of ID assignment are up to RAN2. RAN1 specifies the UE behaviors based on the assumptions that the ID is unique per MNO.

Proposal 2

- Support the mechanism to release the pairing ID, e.g., with a pairing ID validation duration.

  - RAN1 sends an LS to RAN2 (and/or other related WGs) to specify the detailed mechanism.

  - RAN1 studies the UE behaviors when a pairing ID becomes invalid or is released.

Proposal 3

- Target CI format reuse the same format as NW-side data collection for training.

Proposal 4

- Support the NMSE as the performance target.

Proposal 5

- Option 2 is naturally the CSI feedback format and should be supported. No need to further discuss this issue.

References

- Moderator (Qualcomm), RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements,” Jun. 2025.

- RAN1 chair (CMCC), “Draft Report of 3GPP TSG RAN WG1 #122”, Aug., 2025.