3GPP TSG RAN WG1 #122			R1-250XXXX

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	10.3

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 10.3 Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

Document for:	Discussion, Decision

## Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

Please refer to RP-251884 for detailed scope of the SI.

[122-R20-A-IoT] Email discussion on Rel-20 A-IoT – xxx

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2505851	Work plan for Rel-20 Ambient IoT SI	LG Electronics

### Evaluations

Including necessary evaluation assumptions of deployment scenarios for coverage and coexistence, evaluations of achievable cell edge data rate and link budget, as well as applicability and necessity of Device 2b and Device C to given scenarios.

R1-2505603	Discussion on evaluation of active Ambient IoT device	ZTE Corporation, Sanechips

R1-2505246	Evaluations	Ericsson

R1-2506158	Evaluations for Active Device in Ambient IoT	InterDigital, Inc.

R1-2505154	Discussion on Evaluation Methodology for R20 A-IoT	FUTUREWEI

R1-2505166	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

R1-2505212	Evaluation for active A-IoT device in outdoor scenario	Huawei, HiSilicon

R1-2505304	Evaluation methodology for A-IoT outdoor deployment scenarios	CATT

R1-2505410	Evaluation methodologies, assumptions and results for R20 AIoT	vivo

R1-2505423	Initial views on Evaluation Methodology for Rel-20 AIoT	Nokia

R1-2505457	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices	Xiaomi

R1-2505513	Discussion on evaluations for Ambient IoT	China Telecom

R1-2505578	Evaluation methodology and assumptions for Rel-20 Ambient IoT	Samsung

R1-2505615	Discussion on evaluation for Ambient IoT	Transsion Holdings

R1-2505751	Discussion on EVM for R20 A-IoT	OPPO

R1-2505793	Evaluation assumptions for outdoor Ambient IoT 	Tejas Network Limited

R1-2505852	Evaluations for Rel-20 Ambient IoT SI	LG Electronics

R1-2505907	On Rel-20 Ambient IoT evaluation assumptions	Apple

R1-2505923	Evaluations for Ambient IoT	NEC

R1-2506091	Discussion on evaluation assumptions	CMCC

R1-2506112	Ambient IoT evaluations for outdoor active devices	Sony

R1-2506121	SON MDT for Intra-NTN Mobility	Jio Platforms

Withdrawn

R1-2506212	Evaluations	Qualcomm Incorporated

R1-2506299	Study on evaluations for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

R1-2506373	Discussion on evaluation methodology for Rel-20 AIoT	IIT Kanpur

R1-2506458	FL summary #1 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

Agreement

For Rel-20 study, Budget-Alt2 (as per TR38.769) is used to derive Rx sensitivity in the evaluation for both R2D and D2R.

Agreement

Proposal: For Rel-20 study, channel model TDL-C with an RMS delay spread of 300 ns delay spread is used for link level simulation for NLOS.

R1-2506459	FL summary #2 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

Agreement

For Rel-20 study, the followings is used to perform coverage evaluation for R2D and D2R, as for Rel-19:

- Step 1: Obtain the required SINR by link-level simulations under different reference data rates

- Step 2: Obtain the receiver sensitivity based on the required SINR if Budget-Alt2 is used

- Step 3: Obtain the MPL/distance based on the receiver sensitivity and link budget calculation

Agreement

For Rel-20 study, the following path-loss models are used in the coverage evaluation:

- UMa NLOS defined in TR38.901

- RMa NLOS defined in TR38.901

  - Note: companies can also report to use RMa LOS defined in TR38.901

- UMi NLOS defined in TR38.901

- Note: this does not have implication on the coverage target (if any)

Agreement

For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows:

- BS antenna height

  - UMa: 25m

  - RMa: 35m

  - UMi: 10m

- Device antenna height

  - 1.5m

Agreement

For Rel-20 study, the following layout assumption is used for evaluation purpose:

- Hexagonal grid, 19 macro sites, 3 sectors per site

  - Note: the same layout assumption is used for UMi

Agreement

For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings:

- For standalone: 43 dBm

- For in-band:

  - 38 or 33 dBm over R2D transmission BW

  - Note1: any necessary PSD limitation can be applied by RAN4

  - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations

Send an LS to RAN4 with the above agreement, and to provide feedback to RAN1 if any.

Comeback for draft LS, considering potential further agreement on candidate R2D transmission BW.

Agreement

For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings:

- Device 2b: {-10, -20} dBm

- Device C: {-3, 0, 5} dBm

Agreement

For Rel-20 study, BS antenna gain assumption in link budget for evaluation purpose:

- 17dBi

  - This assumes in the LLS table (as per TR38.769), the values in “[0h1] Number of antenna elements” and “[0h2] the Number of TXRUs” would be same (as Rel-19)

Agreement

For Rel-20 study, Device antenna gain assumption in link budget for evaluation purpose is 0dBi for both Device 2b and C.

Agreement

For Rel-20 study, BS noise figure assumption in link budget for evaluation purpose is 5dB.

Agreement

For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 180kHz as starting point

  - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

Agreement

For Rel-20 study, R2D receiver bandwidth assumption in link budget and LLS for evaluation purpose is up to company report with considering:

- R2D transmission bandwidth

- Device Rx LO uncertainty (e.g. CFO ppm assumption)

- Note: ED bandwidth (for the purpose of noise/interference power calculation, if any) and BB LPF bandwidth in LLS are assumed to be same as R2D receiver bandwidth in link budget for active device with ZIF/IF receiver

Agreement

For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 15kHz as starting point

  - Other values are not precluded and up to company report

  - Companies to report which of 2SB or 1SB modulation is assumed for D2R transmission bandwidth

Agreement

For Rel-20 study, message size in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether to add value of 800 bits.

Agreement

For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose:

- Device 2b: 15 dB

  - FFS: 40dB as additional assumption for Device 2b with IF-ED

- Device C: 13 dB

Agreement

For Rel-20 study, R2D FEC in LLS assumption for evaluation purpose:

- Companies who report evaluations using FEC should also report evaluations without FEC

Agreement

For Rel-20 study, add new item/row of “Repetition” for both R2D and D2R columns into LLS assumptions for evaluation purpose

- Companies who report R2D evaluations using repetition should also report R2D evaluations without repetition

Agreement

For Rel-20 study, reference data rate in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether to add additional value(s)

R1-2506460	FL summary #3 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

### Study of air interface for Device 2b/C

Please refer to the first paragraph of objective 1 for the given conditions. Including study necessary and feasible changes to the Rel-19 air interface for Device 2b/C.

R1-2505411	Discussion on air interface for Device 2b/C for R20 AIoT	vivo

R1-2506300	Study on air interface for Ambient IoT device 2b/C	NTT DOCOMO, INC.

R1-2506213	Study of air interface for Device 2b/C	Qualcomm Incorporated

R1-2505155	Discussion on Air Interface Enhancements for R20 A-IoT	FUTUREWEI

R1-2505167	Discussion on study of air interface for Device 2b/C	Spreadtrum, UNISOC

R1-2505213	Study of air interface for A-IoT device 2b/C	Huawei, HiSilicon

R1-2505247	Study of air interface for Device 2b/C	Ericsson

R1-2505305	Study of A-IoT enhancement for device 2b/C	CATT

R1-2505424	Initial views on air interface for Device 2b/C for Rel-20 AIoT	Nokia

R1-2505458	Discussion on the air interface for Device 2b/C	Xiaomi

R1-2505514	Discussion on air interface for Device 2b/C for Ambient IoT	China Telecom

R1-2505579	Study on air interface design for active devices	Samsung

R1-2505604	Discussion on air interface of active Ambient IoT device	ZTE Corporation, Sanechips

R1-2505616	Discussion on air interface for device 2b and device C	Transsion Holdings

R1-2505630	Discussion of air interface for Device 2b/C	Lenovo

R1-2505671	Initial Views on AIoT outdoor for active devices	Ofinno

R1-2505752	Discussion on air interface enhancement for device 2b/C	OPPO

R1-2505768	Discussion on air interface for Device 2b/C	Quectel

R1-2505794	Study of air interface for Device 2b and C for outdoor	Tejas Network Limited

R1-2505807	Study of air interface for outdoor active ambient IoT devices	ROBERT BOSCH GmbH

R1-2505853	Air interface for Device 2b/C	LG Electronics

R1-2505908	On Rel-20 Ambient IoT air interface for device 2b/C	Apple

R1-2505924	Study of air interface for active device	NEC

R1-2505956	A-IoT Air Interface for Device 2b/C	Panasonic

R1-2505999	Views on air interface for Device 2b&C for Ambient IoT	HONOR

R1-2506013	Discussion on air interface for active devices	Sharp

R1-2506061	Discussion on air interface for device 2b and device C	ETRI

R1-2506092	Discussion on air interface for Device 2b/c	CMCC

R1-2506113	Air interface for device C and 2b	Sony

R1-2506159	Study of air interface for Device 2b/C in Ambient IoT	InterDigital, Inc.

R1-2506263	Discussion on air interface for active devices for Ambient IoT	TCL

R1-2506313	Discussion on air interface for Devices 2b/C for outdoor	Sequans Communications

R1-2506344	Discussion on A-IoT active device	ASUSTeK

R1-2506356	Discussion on air interface for Device 2b/C	CEWiT

R1-2506372	Discussion on air interface for active device types	IIT Kanpur

R1-2506507

Agreement

For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values:

- Addition of M=1

- Addition of M=32

- Removal of M=24

Agreement

Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects:

- Coding schemes: convolutional code (e.g TBCC), RM code

  - Note: focus on coding schemes already specified in LTE or NR

- FEC code rates: at least 1/2, 1/3

- Interleaver or LTE bit collection scheme for TBCC

Agreement

Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types:

- Block-level repetitions

- Bit-level Type-2 repetitions

- Chip-level repetitions (in case of M=1 with FEC if applied)