3GPP TSG RAN WG1 #122			R1-250XXXX

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	10.5

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 10.5 Study on Integrated Sensing And Communication (ISAC) for NR

Document for:	Discussion, Decision

## Study on Integrated Sensing And Communication (ISAC) for NR

Please refer to RP-251861 for detailed scope of the SI.

[122-R20-ISAC] Email discussion on Rel-20 ISAC – xxx

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2505777	Work plan for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

R1-2505778	TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

### Evaluation assumptions and performance evaluation

Including study of metrics, measurements, and relevant measurement quantization for UAV use case.

R1-2506093	Discussion on Measurement and Evaluation for ISAC		CMCC

R1-2505638	Study on Integrated Sensing And Communication (ISAC) for NR	Tejas Network Limited

R1-2505921	Evaluation assumptions and performance evaluations of ISAC for NR	Nokia, Nokia Shanghai Bell

R1-2505684	Discussion on ISAC Performance Evaluation	NIST

R1-2505168	Discussion on evaluation assumptions and performance evaluation for ISAC	Spreadtrum, UNISOC

R1-2505214	Performance metrics, methodologies, and initial evaluation results for ISAC	Huawei, HiSilicon

R1-2505306	Evaluation assumptions and metrics for R20 ISAC for NR	CATT, CICTCI

R1-2505412	Views on 5G-A ISAC Study	vivo

R1-2505459	Discussion on performance evaluations for ISAC	Xiaomi

R1-2505469	Evaluation assumptions and performance evaluation for ISAC	EURECOM

R1-2505515	Discussion on ISAC evaluation assumptions and performance evaluation	China Telecom

R1-2505580	Discussion on ISAC evaluation assumptions and performance evaluation	Samsung

R1-2505613	ISAC Evaluation assumptions	Tiami Networks

R1-2505652	Evaluation assumptions and performance evaluation of NR ISAC	InterDigital, Inc.

R1-2505654	Views on evaluation assumptions and performance evaluation for ISAC	Sharp

R1-2505753	Discussion of ISAC evaluation in 5GA	OPPO

R1-2505776	Evaluation assumptions and performance evaluation for ISAC	Panasonic

R1-2505833	On ISAC performance evaluations and assumptions	Lenovo

R1-2505909	On Rel-20 Evaluation assumptions and performance evaluation for 5G-A ISAC	Apple

R1-2505950	Discussion on evaluation assumptions and performance evaluation for NR ISAC	LG Electronics

R1-2505975	Discussion on R20 ISAC Study	SK Telecom

R1-2506036	Discussion on ISAC evaluation assumptions and performance evaluation	MediaTek Inc.

R1-2506114	Initial views on ISAC evaluation & performance metrics	Sony

R1-2506131	Discussion on 5G-A ISAC evaluation	ZTE Corporation, Sanechips, Pengcheng Laboratory

R1-2506214	Considerations on UAV gNB-monostatic sensing	Qualcomm Incorporated

R1-2506256	Discussion on ISAC for NR	Ericsson

R1-2506301	Discussion on evaluation assumptions and performance evaluation for ISAC for NR	NTT DOCOMO, INC.

R1-2506476	Summary #1 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2506477	Summary #2 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement

Horizontal/vertical positioning accuracy are agreed as performance metrics for NR ISAC.

  - It is defined as the absolute value of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target.

    - Note: in RAN1 evaluations, there should be only one estimated horizontal/vertical position corresponding to the true position of a sensing target

[FL2][H] Proposal 4.2-1-rev2

- Velocity accuracy is agreed as performance metric for NR ISAC.

  - For single TRP monostatic sensing, radial velocity accuracy is estimated

    - Radial velocity accuracy is defined as the absolute value of the difference between the estimated radial velocity and the corresponding true radial velocity of a sensing target.

    - Note: in RAN1 evaluations, there should be only one estimated radial velocity corresponding to the true radial velocity of a sensing target

    - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic.

  - The accuracy of horizontal/vertical estimates of the true velocity can also be reported as a performance metric for NR ISAC.

[FL2][H] Proposal 4.2-1-rev2

- Velocity accuracy is agreed as performance metric for NR ISAC.

  - Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target.

  - For single TRP monostatic sensing, radial velocity accuracy is estimated

    - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic.

  - The accuracy of horizontal/vertical estimates of the true velocity can also be reported as a performance metric for NR ISAC.

R1-2506478	Summary #3 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Scenario | UMa-AV, Optional RMa-AV |
| Carrier frequency | Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz] |
| System bandwidth | 100 MHz |
| Numerology | SCS = 30kHz |
| BS Layout | Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees |
| Inter-BS (2D) distance | Uma-AV: 500m, optional 1000m, RMa-AV: 1732m |
| BS antenna height | 25m for UMa-AV, 35m for RMa-AV |
| Wrap-round | No wrap-round |

Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Target type | UAV with small size (0.3m x 0.4m x 0.2m) |
| Target distribution when target(s) are dropped | N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m |
| Mobility | horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h |

Note: N=0 will be discussed in a later proposal in relation to false alarm.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Minimum BS-target 3D distance | 10 m |
| Minimum target-target (3D) distance | 10 m |
| Target outdoor/indoor proportion | 100% outdoor |
| LOS/NLOS | LOS and NLOS |
| Orientation | Random in horizontal domain |
| RCS model | RCS model 1 for UAV with small size |

R1-2506479	Summary #4 on evaluations for NR ISAC	Moderator (Xiaomi)

[FL3][H] Proposal 4.4-2-rev1

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) is based on the final sensing results in the simulation area, and should fulfil the following conditions:

- One true target is associated with at most one detected object

- One detected object is associated with at most one true target

- If the absolute value of difference of each of position and velocity between a detected object and a true target is less than a respective threshold, the detected object can be considered as estimate of the true target.

  - The threshold for each metric is up to implementation.

    - The used threshold for each metric should be reported with submitted evaluation results.

- The same association  applies to miss detection, false alarm probability Type 2 and positioning/velocity accuracy

- The same sensing algorithmshould be applied for  false alarm probability Type 1, for the determination of miss detection, false alarm probability Type2 and positioning/velocity accuracy.

Agreement

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions:

  - One true target is associated with at most one detected object

  - One detected object is associated with at most one true target

  - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy

  - Companies to report the method used for association of the detected object(s) and the true target(s)

Agreement

- Missed detection probability is agreed as performance metric for NR ISAC.

  - It is defined as the conditional probability of not detecting the presence of a target when the target is actually present in the simulation area.

Pmd=n=0N-1DnMnN

Where,

      - Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object

      - Mn is the number of true targets in the drop n.

      - N is total number of drops with at least one target per drop

Agreement

- False alarm probability is agreed as performance metric for NR ISAC.

  - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported

  - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported

  - Note: both False alarm probability Types are mandatory

  - KPI values for False alarm probability Type 1 and 2 can be discussed separately.

Definitions:

  - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm.

Pf1=n=0N-1QnN

Where,

    - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0.

    - N is the total number of drops without targets in the simulation area.

  - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm.

Pf2=0≤n<NMn'≠0Dn'Mn'K

Where,

      - Dn' is the number of detected objects but not associated with any true targets in the drop n.

      - Mn' is the total number of detected objects in the drop n.

      - FFS:

        - Option 1: K is number of drops (N)

        - Option 2: K is number of drops with at least one detected object

Note: the number of targets should be reported by companies when providing False alarm probability Type 2

Agreement

- Velocity accuracy is agreed as performance metric for NR ISAC.

  - Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target.

  - For single TRP monostatic sensing,

    - The radial velocity accuracy can be estimated

      - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic.

    - The true velocity accuracy can be estimated.

R1-2506480	Summary #5 on evaluations for NR ISAC	Moderator (Xiaomi)