3GPP TSG RAN WG1 #122-bis                                                                                               R1-2508024

Prague, Czech Republic, October 13th ~ 17th, 2025

Agenda item:	10.1.1.1

Source: 	Moderator (Qualcomm)

Title: 		Final FL summary on AIML inference related aspects

Document for:	Discussion and Decision

# Introduction

Based on the conclusion of the Rel-19 study [1], spatial-frequency CSI compression (with temporal aspects) via two-sided AIML model proceeds to the normative work, with following objective [2] for RAN1.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

The above objectives are further split into three agenda, i.e., inference aspects, inter-vendor collaboration and other aspects including data collection, monitoring and model pairing. This document summarizes the issues regarding agenda item 10.1.1.1 (Inference aspects of two-sided CSI compression) in RAN#122.

## Contact Information

Please provide your contact information.

| Company | Name | E-mail |
| --- | --- | --- |
| Moderator (Qualcomm) | Chenxi Hao | chenxih@qti.qualcomm.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Bingchao Liu | vpourahmadi@lenovo.com liubc2@lenovo.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| Sharp | Pochun Chou | bertrand.pc.chou@sharp-world.com.tw |
| CMCC | Yi Zheng Yuhua Cao | zhengyi@chinamobile.com caoyuhua@chinamobile.com |
| Ericsson | Xinlin Zhang Jingya Li | xinlin.zhang@ericsson.com jingya.li@ericsson.com |
| Futurewei | Zhigang Rong | zrong@futurewei.com |
| MediaTek | Reubengeorge Stephen Pedram Kheirkhah Sangdeh | Reubengeorge.stephen@mediatek.com Pedram.kheirkhah@mediatek.com |
| CEWiT | Shivshankar Advaith | shivshankar@cewit.org.in advaith22@cewit.org.in |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Panasonic | Tetsuya Yamamoto Hidetoshi Suzuki Xuan Tuong Tran (Henry) | yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com xuantuong.tran@sg.panasonic.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| LG Electronics | Minseok Jo (MJ) | ms.jo@lge.com |
| TCL | Pu Yuan Tianqi Wu | Pu.yuan@tcl.com tianqi1.wu@tcl.com |
| NEC | Peng Guan Yi Jiang Shafivulla Sayyed | guan_peng@nec.cn y-jiang_ct@nec.com Sayyed.Shafivulla@india.nec.com |
| Apple | Huaning Niu Yuan Zhu | huaning_niu@apple.com yuan_zhu3@apple.com |
| Vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| InterDigital | Afshin Haghighat | afshin.haghighat@interdigital.com |
| ETRI | Anseok Lee | alee@etri.re.kr |

# Specification of target CSI type

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 1: Support configuring 0≤L≤N/2 beams for target CSI. When L=0, antenna-domain target CSI is used. When L>0, beam-domain target CSI is used, where L beams are used for converting the precoding matrix to beam-domain. |
| Futurewei | Proposal 2: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for inference, support using eigenvector of the channel covariance matrix as target CSI type. Also consider full channel matrix as target CSI type for further performance improvements. |
| Spreadtrum | Proposal 1: support target CSI is the spatial-frequency domain eigen vector. |
| Huawei, HiSilicon | Proposal 1: For the evaluation assumption of channel matrix as Target CSI type, at least consider the following configurations:  SRS period/hopping: For 100MHz BWP with 272PRBs,  SRS period includes at least 10ms, 20ms, 40ms.  SRS measurement is considered with at least 17 hops each with 16 PRBs.  DL/UL reciprocity error modeling: at least consider channel estimation error to DL channel and UL channel, where DL/UL channel matrix are generated by adding DL/UL channel estimation error depending on DL/UL SNR, respectively.  Frequency granularity: For 100MHz BWP with 272PRBs, at least consider the Encoder input as the DL channel matrixes for 17 subbands, where a certain PRB among the 16 RBs is selected per subband for generating the DL channel matrix.  DL Rx number: at least consider 4 Rx, where the DL channel vectors per subband per Rx antenna are considered as individual Tokens. Observation 4: From the evaluation results over different target CSI types, under TDD scenario, with SRS periodicity of 10ms, 20ms, and 40ms,  Channel matrix as Target CSI (with NW implemented fusion) can outperform SRS measurement only with 16.42%~34.75% gains, and 21%~54% SGCS gain.  Channel matrix as Target CSI (with NW implemented fusion) can outperform precoding matrix as Target CSI with 18.26%~30.9% gains, and 17%~31% SGCS gain. Proposal 2: Regarding the Target CSI type, support channel matrix (H) as Target CSI type.  Note: fusion with SRS measurement is NW implementation and is transparent to UE. |
| InterDigital | Proposal 1: Support beam domain processing for determination of Target CSI for AI/ML CSI SF compression Case 0. |
| Google | Proposal 1: For AI/ML based CSI report, support the UE to report the followings for PMI in a CSI report • SD basis indicating W1 based on an eType2 codebook • The compressed W2 based on the AI/ML |
| CATT | Proposal 1: Regarding target CSI type, precoding matrix in spatial-frequency domain should be prioritized for model inference. |
| Vivo | Proposal 1: For inference/monitoring/data collection purpose, consider channel matrix as a CSI type of target CSI for TDD scenario. |
| Xiaomi | Proposal 1: Support precoding matrix in spatial-frequency domain as target CSI type as baseline. |
| TCL | Proposal 2: For Target CSI, support using the precoding matrix as the baseline. • FFS: Study if the full channel matrix can be used in specific scenarios. |
| ZTE | Proposal 3: For the target CSI type for model inference, support precoding matrix with spatial-frequency domain representation as a unified solution for different number of antenna port configurations. Proposal 4: For target CSI type for model inference, further study explicit channel matrix as model input for both FDD and TDD scenarios. |
| Samsung | Proposal#1: Consider the following Target CSI types, by considering performance-complexity tradeoff for the maximum number of ports to be supported • Option 1-1: Precoding matrix in spatial-frequency domain • Option 1-2: Precoding matrix in angle-delay domain |
| Panasonic | Proposal 1: Spatial-frequency domain precoding matrix is supported as target CSI type at least in Rel.20 NR. Proposal 2: The additional support of channel matrix must demonstrate clear and justified advantages / gain. |
| OPPO | Proposal 1: Only support CSI eigenvector in spatial-frequency domain as the target CSI type.  Whether the input of precoding matrix in spatial-frequency domain is transformed into other forms can be left to UE-side encoder implementation. |
| KT | Proposal 1: Study the feasibility of also supporting channel matrix as target CSI type, particularly for scenarios with a large number of transmit antennas. Proposal 2: Support representation of the target CSI in the angular-delay domain, as its sparse structure provides a more efficient approach to target CSI formatting with potential for overhead reduction. |
| Nokia | Proposal 1: Clarify that a precoding matrix as target CSI type for inference is defined as in legacy non-AIML CSI compression, i.e. it is a 2N_1 N_2×υ matrix for PMI sub-band t=0,…,N_3-1. The latent message indicates N_3 such precoding matrices. Proposal 7: Define an uncompressed and unquantized reference format for the target precoding matrices for inference, for example as a special parameter combination of Rel-16 eType-II, with L=N_1 N_2, M_υ=N_3 and with K_0=2N_1 N_2 N_3 unquantized nonzero coefficients per layer. |
| LGE | Proposal #1: Support spatial-frequency domain precoding matrix for target CSI type. |
| NEC | Proposal 1: Support precoding matrix in spatial-frequency domain as target CSI type. |
| Fujistu | Proposal 1:  For CSI compression Case 0, RAN1 to consider the precoding matrix in spatial-frequency domain as target CSI. |
| ETRI | Proposal 1: For AI/ML-based CSI compression, consider the precoding matrix on the spatial-frequency domain as baseline. |
| CMCC | Proposal 1: To consider the target CSI type for inference, monitoring and inter-vendor collaboration training together, there should be at least one common type for target CSI for inference, monitoring and inter-vendor collaboration training. Proposal 2: At least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the target CSI type. |
| Ofinno | Proposal #1: Regarding target CSI type for AI/ML-based CSI compression (Case 0), support precoder matrix in spatial-frequency domain as a baseline. |
| Apple | Proposal 1: Proceed Rel-20 WI with target CSI type as spatial-frequency precoding matrix: • FFS on applying transforming to the target CSI before model input; and • FFS channel-based target CSI in 6GR. |
| Toyota ITC | Proposal 1: For 5G NR Release 20, consider the channel matrix as CSI feedback type, if the gain in system/user throughput is observed in most of the typical 5G NR deployments. Performance evaluations should consider the extra signaling overhead. Proposal 2: Specify - the maximum number of antenna ports - maximum rank - reporting per antenna port, or reporting in the beam-domain of the precoding matrix, prior to deciding whether d_z^(l,ν) is common, or specific for different layers, or ranks. |
| Qualcomm | Proposal 1: For Rel-20 NR CSI feedback via two-sided model, precoder in port-subband domain is mapped with the latent message subject to the pairing ID. - For a certain layer, the precoding matrix of layer l on N_3 subbands and P ports are written as W^l=[W_1^l,W_2^l,⋯,W_(N_3)^l ], where W_k^l contains P×1 complex numbers with unit norm. Proposal 2: For Rel-20 NR CSI feedback via two-sided model, support a single solution for precoder matrix feedback upto 128 ports, and the precoder matrix is represented in port-subband domain. |
| DCM | Proposal 1 - Support the target CSI defined in the spatial-frequency domain (i.e., antenna port-subband). Proposal 2 - Support the restrictions for AI/ML-based CSI compression to achieve similar effects as the CBSR of the legacy codebooks.  Method: Restrict the amplitude of the target CSI in NW-configured directions. Proposal 3 - Support specifying a phase normalization method for the target CSI. |

## Discussion

FL note target CSI type.

Based on the proposals in the contribution, FL observes following preference on the target CSI type.

- Precoder in spatial frequency domain (21)

  - Futurewei, Spreadtrum, Huawei, vivo (with evaluation of upto 128 ports), CATT, Xiaomi, TCL, ZTE, Panasonic, OPPO, Nokia, LGE, NEC, Fujistu, ETRI, CMCC (unified solution), QC (With evaluation for upto 128 ports), DCM, MTK, Apple, Ofinno.

- Channel feedback as further improvement (6)

  - Futurewei, Huawei, ZTE, vivo, KT, Toyota ITC

- Precoder in beam-frequency domain as input together with W1 feedback (2)

  - InterDigital, Google

- Precoder in beam-frequency domain with conditions (1)

  - Ericsson (with evaluation): configuring L=0

- Precoder in beam-delay domain (2)

  - Samsung (with generalization performance): based on number of ports

  - KT (without condition on number of ports)

A good number of companies cite the observations and conclusions in TR 38.843 that, for the complexity range of 1M~100M FLOPs and 0.1M~10M #parameters, sufficiently good performance gain has been observed by spatial frequency (port-subband) domain precoder matrix, though 2 sources see some gain at ultra-low complexity range of 0.1M by angular-delay domain representation. Regarding further complexity reduction, advanced AIML techniques such as teacher-student training and knowledge distillation can be considered rather than adopting two solutions in the standard.

There is one source arguing the advantage of generalization performance, but sufficiently good gain has been identified in Rel-18 study if mixed training using data from different scenarios is considered.

Furthermore, two sources, vivo and Qualcomm provide evaluation on 128 ports. Qualcomm observes similar performance complexity gain offered by port-subband domain over beam-delay domain. Vivo shows a good scalability from 32 ports to 128 ports using common model structure / parameters.

Vivo’s evaluation: A scalable model can be applicable P=32 port and P=128 port, and the performance loss (average -0.58%) between the scalable model and dedicated model is sm Scalability for 32 Tx port and 128 Tx port, with PC1. PC1; 32 Tx port; 128 Tx port. Dedicated structure & Dedicated parameter; 0.703 (baseline); 0.694 (baseline). Common structure & Dedicated parameter; 0.695 (-1.14%); 0.694 (-0.00%). Common structure & Common parameter; 0.691 (-1.71%); 0.686 (-1.15%). Scalability for 32 Tx port and 128 Tx port, with PC3. PC3; 32 Tx port; 128 Tx port. Dedicated structure & Dedicated parameter; 0.762 (baseline); 0.727 (baseline). Common structure & Dedicated parameter; 0.755 (-0.92%); 0.727 (-0.00%). Common structure & Common parameter; 0.754 (-1.05%); 0.720 (-0.96%). Qualcomm’s evaluation on target CSI type for 128 ports:

Besides, OPPO and Qualcomm mentioned that the transformation to beam-delay domain can be absorbed into the framework of port-subband domain representation if considering such pre-processing is an implementation choice. From all these, let us try to adopt a unified solution in RAN1 specification and support upto 128 ports.

Further, Nokia proposed to further restrict the target CSI represented by legacy eType II codebook format with uncompressed and unquantized manner, e.g., L=N1*N2 and Mv=N_sb, for the feasibility of CBSR configuration. DCM also proposed to consider potential solutions for CBSR. This can be discussed in a later stage.

##### Proposal 2.1 (closed):

Support precoding matrix with port-subband domain representation as target CSI type upto 128 ports.

- Note: for a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- FFS: whether further restricting the representation of precoding matrix in a legacy codebook format.

| Companies | Views |
| --- | --- |
| vivo | Support |
| Apple | Support |
| NTT DOCOMO | Support the main bullet, and we don’t think the FFS part is necessary. |
| Samsung | The support for more than 32 ports requires careful consideration in terms of performance complexity trade-off and feasibility. As we did in Rel-19, the performance within a feasible range of complexity, e.g., 1M parameters and 10MFLOPs, should be checked. Port-domain input would imply the model dimension scales with the number of ports. Qualcomm’s results in the port domain illustrates more than 70MFLOPs are required for a single layer inference, which is more than 7x of the cut-off complexity we have considered in Rel-19. A careful check may consider the following three cases Alt1) Angular domain up to 128 ports Alt2) Port domain up to 32 ports and angular-domain up to 128 ports Alt3) Port domain up to 128 ports. |
| OPPO | Support. No need to mention the FFS part. |
| Huawei, HiSilicon | Regarding the precoding matrix type, ok with spatial/frequency domain matrix. But in the last meeting, there is one FFS on the evaluation for channel matrix. Agreement For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc. For this meeting, a larger number of companies have interests on using channel matrix, and more companies bring simulation results. - Precoder in spatial frequency domain (20 21) - Futurewei, Spreadtrum, Huawei, vivo (with evaluation of upto 128 ports), CATT, Xiaomi, TCL, ZTE, Panasonic, OPPO, Nokia, LGE, NEC, Fujistu, ETRI, CMCC (unified solution), QC (With evaluation for upto 128 ports), DCM, MTK, Apple, Ofinno. - Channel feedback as further improvement (6) - Futurewei, Huawei, ZTE, vivo, KT, Toyota ITC Therefore, we believe we should continue discussing the evaluations for channel matrix. Otherwise, we do not think the two-sided model is appealing ofr commercialization with merely ~10% gain. Proposal: For the evaluation for channel matrix, following assumptions are considered: - SRS period/hopping: For 100MHz BWP with 272PRBs, - SRS period includes at least 10ms, 20ms, 40ms. - SRS measurement is considered with at least 17 hops each with 16 PRBs. - DL/UL reciprocity error modeling: at least consider channel estimation error to DL channel and UL channel, where DL/UL channel matrix are generated by adding DL/UL channel estimation error depending on DL/UL SNR, respectively. - Frequency granularity: For 100MHz BWP with 272PRBs, at least consider the Encoder input as the DL channel matrixes for 17 subbands, where a certain PRB among the 16 RBs is selected per subband for generating the DL channel matrix. - DL Rx number: at least consider 4 Rx, where the DL channel vectors per subband per Rx antenna are considered as individual Tokens. |
| ZTE | Support. The last FFS can be separately discussed since the representation restriction is not justified. |
| Futurewei | Support in principle. Agreed with OPPO that there is no need to mention the FFS part. |
| CATT | Support in principle. Regarding the 1st bullet, analogous to legacy eType II codebook, RRC parameter numberOfPMI-SubbandsPerCQI-Subband can be configured to have R PMIs per subband. Therefore, we propose the following modification: a precoding matrix on N_sb subband, with R PMI per subband, and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, R∙Nsb-1l] with Win,kl being a P×1 vector. Regarding the 2nd bullet, it is related to the format of target CSI type and whether CBSR can be configured, this can be discussed separately. |
| Ofinno | Support |
| InterDigital | Due to the large computational overhead, we do not see it practical to support broad usage of precoding matrix with port-subband domain representation as target CSI type upto 128 ports. We believe a better approach may be to have it supported till 32 ports, and then use a beam/angular domain as suggested in Alt2 by Samsung. |
| LG Electronics | Support. |
| Ericsson | We share the same concern with Samsung and InterDigital that the complexity becomes too high for 128 ports. Note that with legacy eType II, the complexity for inference is only around 0.3M FLOPs. We prefer to have beam-domain representation for larger than 32 ports. |
| Lenovo | Support in principle. Could it be clarified that whether this option is also applied to the case that <= 32 ports? |
| TCL | Support |
| Xiaomi | Support., if the FFS is used for the feasibility of CBSR configuration, it can be discussed separately, e.g., discussed within proposal 4.2. In addition, does the subband in the note is CQI subband or PMI subband? It needs to clarify. Wording as given by CATT is fine to us. |
| SPRD | Support. And FFS part is not needed. |
| Panasonic | Support |
| HONOR | Support. In addition, the FFS can be separately discussed. |
| Fujitsu | Support |
| ETRI | Support |
| CMCC | Support. But the last FFS is not needed. |
| Transsion | Support. The last FFS is not needed. |
| NEC | Support. |

Concern on 128 ports: Samsung, Ericsson, InterDigital

Removing FFS: DCM, OPPO, ZTE, CATT, Xiaomi, Spreadtrum, HONOR, CMCC

##### Proposal 2.1a (closed):

Support precoding matrix with port-subband domain representation as target CSI type upto 128 ports.

- Note: for a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- FFS: support to upto 32 ports or 128 ports

- FFS: whether further restricting the representation of precoding matrix in a legacy codebook format.

After further discussion, following is agreed.

##### Agreement 2.1a:

Support precoding matrix with port-subband domain representation as target CSI type.

- Note: for a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- Only subband PMI reporting is supported.

# Payload and quantization related aspects

## Summary of companies’ proposals

| Company | Proposals |
| --- | --- |
| Ericsson | Proposal 2: Support SQ as the only quantization method that is used for quantizing the latent space for both model training and model inference. Proposal 3: For SQ, support rank-common d and rank-specific Q, with candidate values of {d, Q} given by Table 1. d-Q Combination; d; Q; Total payload. Layer 1,2, 3,4; Rank = 1; Rank = 2; Rank = 3; Rank = 4; Rank 1,2,3,4. Layer 1; Layer 1, 2; Layer 1; Layer 2, 3; Layer 1, 2; Layer 3,4. dQ Comb. 1; 16; 4; 4; 2; 3; 2; 3; 64,128,128,160. dQ Comb. 2; 32; 4; 4; 2; 3; 2; 3; 128,256,256,320. dQ Comb. 3; 64; 4; 4; 2; 3; 2; 2; 256,512,512,512. Proposal 4 For inter-vendor training collaboration Direction A, respect the agreement in RAN1#120bis, and support Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters. |
| Futurewei | Proposal 4: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for quantization codebook used for quantizing the feedback, for inter-vendor collaboration Direction A, support Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters associated with an ID. The exchanged quantization codebook can be in the form of the codebook itself or can be parameters of the codebook based on which the codebook can be reconstructed. Proposal 5: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), support using the following for CSI feedback payload size determination. • The ID for pairing encoder at UE side and decoder at NW side • RI • Quantization codebook associated with the ID Proposal 6: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for UCI mapping for inference, support carrying the CSI feedback in two parts. The first part is of a fixed size and should include CQI for the first codeword, RI, and an indication of the size of the second part. The second part should at least include the content of the UE encoder output and CQI for the second codeword, if applicable |
| Spreadtrum | Proposal 2: Support d_z^(l,v) is common to different layers or ranks to achieve greater compatibility for multiple models. Proposal 3: For CSI report in inference, CSI part 1 will include the factor representing the size of part 2, and CSI part 2 will include encoder output. Proposal 5: Only support SQ as the quantization method. |
| Huawei, HiSilicon | Proposal 3: Candidate combinations of {subband pattern, Tx port number, d/Q/L values}, can be represented by the standardized model (Direction C)/model structure (Option 3a-1 Direction A) or delivered dataset (Option 4-1 Direction A) during the inter-vendor training collaboration.  After model pairing procedure, candidate d/Q/L values for the applicable model are aligned. A specific CSI payload size can be indicated by additionally configuring the specific d/Q/L value among the candidate d/Q/L values. Proposal 4: If Target CSI is channel matrix, CSI payload size adaptation can be achieved by reporting partial channel matrix Proposal 5: For precoding matrix feedback, support layer-specific configuration of d_z^(l,ν)and / or Q^(l,ν). - For layer 1/2, one set of {d, Q} is configured, while for layer 3/4, a separate set of {d, Q} is configured. - Layer 1/2 can be configured from a super set of {d, Q} ranging across small/medium/high payload sizes and layer 3/4 can be configured from its subset corresponding to small/medium payload sizes. - Certain configuration(s) of {d, Q} leading to equal CSI payload size between layer 1/2 and layer 3/4 should be supported. Proposal 6: For flexible trade-off between quantization performance and quantization complexity/overhead: - To simplify the design for VQ and SQ: - For SQ, uniform quantization between 0 and 1 is considered for 2bits and 4 bits quantization granularity. - For VQ, a master codebook including the set of codewords (i.e., floating vectors) and fixed length of the vector segment (i.e., L), is considered over all layers/Rx antennas. - Flexible quantization configuration should be supported. - For SQ, multiple quantization granularities are considered. - For VQ, flexible selection of codewords from the master codebook is considered to achieve different quantization granularities (Q values). Proposal 7: For quantization codebook used for quantizing the feedback in inter-vendor collaboration Direction A, support Alt1 (Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters). Proposal 8: At the inference phase, the quantization codebook (e.g., quantization dictionary for VQ) can be updated by gNB with RRC configuration. Proposal 9: For UCI mapping, consider two parts CSI: - CSI part 1 contains CQI, RI, and the CSI payload size information of CSI part 2 if needed. - CSI part 2 contains the bit sequences representing the compressed Target CSI. For precoding matrix: - Consider the priority rule to split the bit sequence per layer into M bit segments with different priorities for UCI omission. - Consider inter-layer first omission: first omit the M-th segment from layer N to layer 1, then the (M-1)-th segment,…, then the 1st segment. |
| Google | Proposal 3: The CSI report configuration for AI/ML based CSI compression is based on the Rel-19 eType2 CSI report configuration • The CSI report configuration can additionally provide a list of model IDs corresponding to different CSI payload size • The UE selects the model according to the maximum payload size for the CSI report to avoid the CSI omission |
| InterDigital | Proposal 4: For both Direction A and Direction C, support hybrid VQ + SQ quantization to reduce the VQ overhead and complexity, while also avoiding the inefficiency of standalone SQ schemes that require high overhead for acceptable accuracy. Proposal 5: For the hybrid VQ-SQ approach add the following definition on payload size. - If hybrid VQ-SQ is supported, it refers to the quantization of a segment of L consecutive real values jointly, and the segment(s) are separately quantized, and independent quantization of a scalar per each segment. - The payload size for layer l and rank ν is determined by (d_z^(l,ν))/L×Q_VQ^(l,ν) + L×Q_SQ^(l,ν), where Q_VQ^(l,ν) is the number of bits per segment, Q_SQ^(l,ν) is the number of bits per scalar. Proposal 6: Support non-uniform scalar quantizer design based on the distribution of latent samples where the UE and NW exchange the non-uniform scalar quantizer parameters. Proposal 7: Support Alt-1 for the quantization codebook for inter-vendor collaboration Direction A, where the quantization codebook from NW-side to UE-side is exchanged along with each exchanged dataset or model parameters. |
| CATT | Proposal 7: Prioritize scalar quantization for CSI compression for model inference. Proposal 8: Support different values of d for different number of Tx ports and subbands, specifically - The value of d can be different for each layer for different ranks, and the value of d can be represented as N times a specified unit. Proposal 9: Support a layer common and rank common combination of L and Q. - For scalar quantization, a common value of Q is configured. - For vector quantization, different quantization codebooks can be employed for different layers and ranks. Proposal 10: For scalar quantization, Q=2 can be used as a starting point with d determined by payload size and Q. Proposal 11: The association of quantization codebooks and the configurations, including the number of Tx ports, subbands and payload sizes, could be indicated during inter-vendor collaboration. Proposal 22: For UCI mapping of AI/ML-based CSI compression, two part CSI mechanism can be reused.  The CSI feedback bit for AI model output can have different priorities according to the corresponding layer and/or subband configuration. |
| Vivo | Proposal 6: Support Alt 1: exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters Proposal 7: For exchanged quantization codebook, multiple L (e.g., 1, 2, 4) can be supported, Proposal 8: A payload-common codebook can be exchanged from NW-side to UE-side along with each exchanged dataset or model parameters. Proposal 9: For RI and payload determination • RI can be determined by UE using target CSI as legacy • The payload size of CSI feedback can be either layer-common or layer-specific. • The payload size can be configured directly in inference report configuration Proposal 10: For CSI compression with UE side model, two parts CSI is suggested for reporting • Part 1 contains RI, CQI and potential payload truncation indication of each layer • Part 2 contains bit sequence of CSI feedback with UE side model Proposal 11: New omission rule, e.g., per layer omission, or payload truncation can be considered |
| Xiaomi | Proposal 2: Support scalar quantization for CSI feedback. Proposal 3: Support layer-common and rank-common Q^(l,v). Proposal 4: Layer-common and rank-specific d_z^(l,v) can be supported. Proposal 5: Support NW to configure the total CSI payload size for each rank or CSI payload size of the first layer for a certain rank v. Proposal 6: Support Alt2, i.e., use a standardized quantization codebook decided by RAN1 if SQ is adopted as quantization method of CSI feedback. Proposal 9: Support Option 3-1, i.e., layer common and rank common two-sided AI/ML model for CSI compression. Proposal 11: For two-sided AI model based CSI feedback, support two parts CSI reporting and CSI part 1 at least including RI and CQI, CSI part 2 including the output of CSI generation part. Proposal 12: The mapping order of CSI part 1 for two-sided AI model based CSI feedback reuses the mapping order of CSI part 1 for legacy eType II codebook based CSI feedback. Proposal 13:CSI part 2 of two-sided AI model based CSI feedback is divided into multiple groups for UCI mapping, and how to divide CSI part 2 into multiple groups need to further investigate. |
| ZTE | - Proposal 9: Support layer common and rank common as the baseline for AI/ML model setting. - Proposal 10: For a layer common and rank common model with scalability over payload size configurations, layer-specific payload size can still be achieved, such as by truncation/masking of the output linear layer output. Proposal 11: For payload determination of AI/ML CSI feedback, support both SQ and VQ to be applicable to different quantization aware training cases.. Proposal 12: For payload determination of AI/ML CSI feedback, support layer-specific CSI payload size to balance accuracy and overhead. Proposal 13: For payload determination of AI/ML CSI feedback, support layer-specific latent message (i.e., d value) which is configured separately from the pairing ID linked to the CSI reporting setting. Proposal 14: For payload determination of AI/ML CSI feedback, support layer-common quantization codebook (i.e., Q and L value), which can be configured by the pairing ID linked to the CSI reporting setting. Proposal 15: For payload determination of AI/ML CSI feedback, support the quantization parameters (e.g., d, Q, or L) of each layer to be directly configured by the network. Proposal 16: For quantization codebook for inter-vendor collaboration Direction A, support Alt1 due to its better flexibility and field performance. Proposal 17: For quantization codebook for inter-vendor collaboration Direction A, not support Alt2 since it cannot ensure future proofness and would cause too much additional specification works in RAN1. Proposal 18: For CSI feedback using two-sided model, for UCI mapping, the UE may omit a portion of the Part 2 CSI according to pre-defined priority rules. Proposal 19: Consider the following methods for CSI omission for potential down-selection: layer-based CSI omission, subband or subband group based CSI omission, truncation/masking of encoder output. |
| CMCC | - Proposal 12: The total payload of AI/ML based CSI compression is dynamic determined by gNB configuration and information in Part 1. - Proposal 13: NW can configure the overhead information for each layer. UE can report RI=v in one specific CSI report, then the total overhead is the sum of the overhead of each layer for RI=v. - Proposal 14: For AI/ML based CSI compression, one CSI report includes CSI part 1 + CSI part 2. CSI part 1 includes CQI for first codeword, RI, and information representing part 2 size, CSI part 2 includes at least the content of CSI generation part output and CQI for second codeword (if RI > 4). - Proposal 15: For precoding matrix mapped with a latent message z^l=[z_0^l,z_1^l,⋯,z_(d_z^(l,ν)-1)^l ], the value of d_z^(l,ν) should be common for different layers and different ranks. - Proposal 16: For quantization codebook used for quantizing the feedback, at least scalar quantization (SQ) should be supported. - Proposal 17: For quantization codebook used for quantizing the feedback, for inter-vendor collaboration Direction A, support Alt1: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Proposal 18: For quantization codebook used for quantizing the feedback, d_z^(l,ν)and Q^(l,ν) should be determined based on NW configuration. - Proposal 19: For quantization codebook used for quantizing the feedback, layer-common and rank-common d_z^(l,ν)and Q^(l,ν) should be the baseline. |
| Samsung | Proposal#3: Support scalar quantization codebooks catered for different dynamic range [z_min,z_max ] output of the latent message z_j^l for j=0,…,d_z^(l,v)-1. - Support range [-1,1] and [0,1] and uniform quantization - Support uniform number of quantization bits, i.e., Q=Q^(l,v) for all l,v. At least Q=2 is supported. - For direction A, network exchanges at least one of the specified quantization codebooks along dataset and/or parameter. - For Direction C, the quantization codebook is determined in relation to last layer activation function and Q. Proposal#4: Support the following scalar quantization codebooks catered for the different dynamic range for elements of the latent message [z_min,z_max ] Proposal#5: Regarding quantization codebooks, support Alt2: Use a standardized quantization codebook decided by RAN1. • For Direction C, the quantization codebook is determined along the specified reference model • For Direction A, the network indicates one of the specified quantization codebooks along dataset/parameter Proposal#6: Following legacy, for two-sided model-based CSI compression, support multiple feedback payload sizes through a number of parameter combinations. Proposal#7: For two-sided model-based CSI compression, follow the legacy principle for feedback size, i.e., PMI feedback overhead roughly doubles from rank=1 to rank=2, and feedback overheads for rank=3 and rank=4 remain in the same level as rank=2. Proposal#8: For two-sided model-based CSI compression, for payload size configuration consider parameter combination including latent message size (d_z^1,1), the number of bits per latent message output (Q) as well as the payload size for rank1 (X1=d_z^1,1×Q). - FFS: Rules to determine payload size for other ranks Proposal#9: For two-sided model-based CSI compression, support two parts CSI. The PMI information is mapped to Part 2 CSI. Support Part 2 CSI dropping. - FFS: whether/how to define groups for Part 2 CSI including priority formula for PMI bits. - FFS: dropping granularity for Part 2 CSI. - FFS: content of Part 1 to determine size of Part 2 |
| KT | Proposal 3: Support Alt1, i.e., exchange of the quantization codebook from NW side to UE side along with each dataset or model parameter update, to ensure alignment between the quantizer and the distribution of the latent message values. Proposal 4: Support specification that payload size is determined from multiple network-configured values, with the UE selecting the appropriate value according to channel conditions. Proposal 5: Support configuration of d_z^(l,υ), Q^(l,υ), and L by pairing ID, given that multiple payload size configurations are already supported under a single pairing ID. Proposal 7: For CSI omission in AI/ML-based CSI compression, study potential specification support to define grouping or hierarchy information for latent vector entries to enable prioritized partial reporting. |
| HONOR | Proposal 1: That d_z^(l,ν) is common or specific for different layers or ranks is depended on the UE’s capability reporting. Proposal 2: Each combination of d_z^(l,ν), Q^(l,ν) and L can be applicable to a certain set of subband and port configurations, e.g., the size of n_subband×n_port are the same or at least similar. Proposal 3: For Quantization methods, SQ is preferred. • FFS: whether/how to support fixed vector quantization codebook Proposal 5: For UCI mapping, support UCI grouping based on portions of each layer, further study the following options: • Option1: Grouping based on output dimension index or segment index. • Option2: Grouping based on quantized bit index. |
| Ofinno | Proposal #2: Regarding target CSI type for AI/ML-based CSI compression (Case 0), support layer-common and rank-specific configuration of d_z^(l,v). Proposal #3: Regarding a formulation of quantization for latent message, support SQ only. Proposal #4: Adopt Alt 1 (NW trained and exchanged) for quantization codebook configuration. Proposal #5: Regarding latent-based CSI quantization, consider a two-part quantization method on latent vector by splitting coarse part and finer part for each component of the latent vector. |
| Lenovo | Proposal 7: Support indication of the length of the compressed CSI other than CRI, RI and CQI within the CSI-Report. Proposal 8: At least support scalar quantization method for the CSI bit generation. Proposal 9: RAN1 should specify the codebook for latent message quantization to allow the case that CSI quantization is not part of the UE-side model encoder. Proposal 10: Quantization codebook parameter, Q^(l,ν) and L is configured by the NW according to UE capability it latent message quantization is not part of the model inference. Proposal 11: For inter-vendor collaboration Direction A, support exchange of quantization codebook from NW-side to UE-side. Proposal 12: Both single part and two parts CSI should be considered for AI based compressed CSI report. |
| OPPO | Proposal 2: Regarding the quantization configuration, support SQ only with L=1, layer-specific and rank-common d_z^(l,v), layer-common and rank-common Q_z^(l,v) Proposal 3: Regarding d_z^(l,v)and/or Q_z^(l,v) configuration, support: NW configure: explicit and/or implicit indication separately to pairing ID UE reported: explicit and/or implicit indication in CSI part 1 Possible combination of NW configure and UE reporting Proposal 4: Regarding inter-vendor collaboration Direction A, support Alt 1. Proposal 5: Suggest candidate N CSI feedback payload values per-layer: {60, 120, 240, 360} for N=4 {60, 120, 180, 240, 300, 360, 420, 480} for N=8 Proposal 10: Support to reuse the legacy UCI multiplexing and resource allocation framework for AI/ML based CSI report. Proposal 11: Support CSI omission for CSI part 2 on PUSCH with per-layer truncation before or after quantization. |
| LGE | Proposal 3: For scalability of payload size and restriction on potential parameter combinations of (d, Q), consider to introduce common/fixed values of Q over rank/layer, where Q is the number of quantization bits and d is the latent message size. Proposal 4: Support rank-specific latent dimension (i.e., d_z^ν) to ensure flexibility of payload size, enabling payload adaptation for different rank, where ν denotes rank. Proposal #6: Study on the CSI encoding method with payload size indication, where part 1 of the CSI report can include either of the following: compression ratio, payload indicator, or the number of multiple information blocks (if applicable). Proposal #8: Regarding quantization codebook for CSI feedback, support Alt1 (NW trained and exchanged). Proposal #9: Further study on following quantization alignment methods. • Via model identification procedure • Via dataset {target CSI, CSI feedback} exchange, where data format for CSI feedback may be specified as i. codeword index of NW-determined codebook with additional signaling for quantization configuration (e.g., codebook exchange) ii. unquantized version with indication of NW’s selection from standardized (3GPP-specified) quantization codebooks |
| Apple | Proposal 6: The total number of possible payload sizes is limited to a small number, e.g., 4. Each payload size corresponds to a single combination of (d_z^(l,v), Q^(l,v)). Proposal 7: The two parameters d_z^(l,v) and Q^(l,v) for a latent message are layer-common and rank agnostic, e.g., 〖d_z^(l,v)=d〗_z and Q^(l,v)=Q. Proposal 8: For AI based CSI compression inference configuration, the NW configures the maximum CSI payload size and a rank restriction. The UE determines the rank and CSI bits per layer and include the CSI size info in CSI part 1. Proposal 9: For AI based CSI compression inference configuration, the quantization alignment can be signaled via pairing ID. Proposal 11: For AI based CSI compression, CSI part 1 includes RI/CQI/number of bits per layer, CSI part 2 includes CSI feedback per each layer in order. Proposal 12: For AI based CSI compression, for CSI omission, the CSI feedback is dropped layer by layer. |
| NEC | Proposal 4: For precoding matrix feedback, support both SQ and VQ for payload determination. Proposal 5: RAN1 to specify parameter combination of {d, Q} or/and {d, L, Q} for the payload size determination. Proposal 6: Multiple (candidate) parameters (e.g., d, Q, L) or parameter combinations can be considered. And the parameters or parameter combination used in inference can be determined based on NW configuration and/or UE reporting, which requires further discussion. Proposal 7: For CSI payload scalability, prioritize those options that enable layer specific or rank specific. Proposal 10: In CSI spatial/frequency compression, support two-part as CSI report format. The CSI Part 1 includes CRI, RI, CQI (for first codeword) and information determining the payload size of the compressed PMI, while CSI Part 2 includes compressed PMI. - Whether/which information determining the payload size of the compressed PMI is included in the CSI Part 1 depends on the outcome of discussion about ‘payload determination’. |
| Nokia | Proposal 2: Select scalar quantization as the only approach for quantizing CSI feedback. Proposal 3: For quantization codebook used in the feedback under inter-vendor collaboration Direction A, select Alt 2: Use a standardized quantization codebook decided by RAN1. Proposal 4: Support fine granularity control of CSI feedback overhead via quantization and truncation. Proposal 5: Control CSI feedback overhead via a configurable dimension d_z^(l,ν) and a configurable overhead budget B^(l,ν), and derive the quantization Q^(l,ν) of each latent vector component from d_z^(l,ν) and B^(l,ν). Proposal 9: For the UCI mapping of an inference-based CSI report, reuse legacy design for Rel-16 eType-II, i.e. two-part CSI with Part 1 containing RI (if reported) and CQI and Part 2 containing the quantized latent message. Proposal 10: Postpone the discussion on omission rules until after the payload size and structure is fully decided. The Part 2 omission procedure for inference-based CSI reports can largely follow legacy operation. |
| Panasonic | Proposal 3: For the specification payload size determination, RAN1 should consider the identified options on AI/ML model setting to adapt to ranks / layers. Proposal 4: For inter-vendor training collaboration Direction A, quantization codebook is exchanged from NW-side to UE-side along with each exchanged dataset or model parameters. Observation 1: For rank > 1 solutions, in addition to each of model size/complexity, the total complexity with multiple models should be taken into account. |
| Transsion Holdings | Proposal 3: Regarding configuration of scalar quantization, the following can be considered: • The number of bits per latent dimension • The minimum and/or maximum range of quantization Proposal 4: Regarding configuration of vector quantization, the following can be considered: • The number of bits per segment • Segment size • The size of codebook Proposal 5: Regarding quantization codebook for inter-vendor collaboration Direction A, support Alt2: • Alt2: Use a standardized quantization codebook decided by RAN1. Proposal 6: Regarding codebook design of vector quantization, the following alternatives can be considered: • Alt-1: A unified codebook is designed for all the segments of latent vector • Alt-2: A dedicated codebook is designed for each segment of latent vector Proposal 7: Regarding CSI omission in CSI spatial/frequency compression, the following alternatives can be considered: • Alt-1: Layer-based priority rule • Alt-2: Segment based priority rule for vector quantization • Alt-3: Scalar of latent space based priority rule for scalar quantization. Proposal 8: In CSI spatial/frequency compression, study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information. |
| Fujistu | Proposal 3: - For quantization codebook/configuration, RAN1 to specify multiple parameter combinations of {d_z^(l,ν) ,Q^(l,ν),L }. - Three combinations can be a starting point which corresponds to low/mediate/high resolutions/payloads. Proposal 4: With the consideration of model scalability, the parameter combination of {d_z^(l,ν) ,Q^(l,ν),L } should be configured separately from pairing ID. Proposal 5:  For inter-vendor collaboration Direction A, RAN1 to support Alt.1 for quantization codebook. - Alt.1: Exchange quantization codebook from NW side to UE side along with exchanged dataset or model parameters. Proposal 6:  For CSI compression Case 0, UCI mapping should follow the legacy framework of CSI Part 1 and Part 2, and the output of AI encoder should be included in CSI Part 2.  The payload size of CSI Part 2 is determined by NW configuration and the rank value. Proposal 7:  For CSI Part 2, RAN1 to study the necessity and the feasibility of grouping the output of AI encoder. o FFS: the number of groups. |
| MTK | Proposal 4. Consider the following options for CSI omission: • Option 1- Layer-wise omission: All feedbacks bits corresponding to a layer will be omitted at a time. • Option 2 - Group-wise omission: group(s) of feedback bits from all layers will be omitted at a time. Proposal 5. Prioritize group-wise omission where group(s) of feedback bits from all layers are omitted if omission is required. Proposal 6. Discuss how to group the CSI feedback bits for potential group-wise omission in the following aspects: • Number of groups • Whether grouping changes for layers and ranks • Impact on inter-vendor collaboration Proposal 7: Support layer-common rank-common AI/ML model with single set of {d_z^(l,ν),Q^(l,ν), L} for non-scalable AI/ML model and multiples set of {d_z^(l,ν),Q^(l,ν), L} for scalable models which are independent of rank and layers, i.e. l and v. Proposal 8. Support at least scalar quantization for quantizing the output of the encoder in the inference stage. Proposal 9: Support separate configuration of {d_z^(l,ν),Q^(l,ν), L} from pairing ID. Proposal 10. Prioritize Alt 2- Use a standardized quantization codebook decided by RAN1 at least for SQ. |
| ETRI | Proposal 5 : For AI/ML-based CSI compression, prioritize to consider the vector quantization and consider the scalar quantization in the framework of vector quantization, if needed. Proposal 6: For AI/ML-based CSI compression, support configuration of the quantization parameters in rank-common and layer-specific manners. Proposal 7: For AI/ML-based CSI compression, consider the trained quantization codebook and its exchanges to enable inter-vendor training collaborations. Proposal 8: For AI/ML-based CSI feedback, adopt type2-r16 UCI mapping as the baseline, investigate a new method for indicating the size of CSI Part 2 in CSI Part 1, and map the CSI Part 2 into a single group or, at most, two groups. |
| Toyota ITC | Proposal 3: Support scalar quantization for the latent message z^l. Proposal 4: The quantization level, Q^(l,ν) should be rank specific and common for all layers for a certain rank. Proposal 5: For inter-vendor collaboration Direction A, support the exchange of quantization codebook from NW-side to UE-side along with each exchanged dataset, or model parameters of a restricted size. |
| Qualcomm | Proposal 3: For Rel-20 NR CSI feedback via two-sided model, consider following principles for payload configuration or determination • Multiple payload configurations considering various uplink coverage, but specify a minimum and necessary number of payload configurations across all layers and ranks • Same model(s) and payload options designed for layer 1 of rank 1 is reused for other layers and other ranks. Proposal 4: Determine the payload options in following procedure: • Step 1: Determine the payload options {d, L, Q} for a single layer o Note: the study should include all the four layers as it is not only used for layer 1 of rank 1, but will be reused for other layers and other ranks. • Step 2: Determine the payload combinations across layers for rank > 1 from the payload options in step 1, without introducing new payload options Proposal 5: Support both VQ with exchanged codebook and SQ with standardized codebook - For a certain pairing ID, VQ is supported if the VQ codebook is exchanged during the inter-vendor collaboration; otherwise, the standardized SQ codebook is used. Proposal 6: For the payload configurations for VQ and SQ - For VQ, support fixed segment size L=2 and codebook size with Q=4 bits per segment and support d_z^(l,ν)∈{32,64,96,128} for payload size of 64, 128, 192 and 256 bits. - For SQ, support Q=2 per scalar and support d_z^(l,ν)∈{32,64,,96,128} for payload size of 64, 128, 192 and 256 bits. - Support common codebooks for all segments / scalars and for all payload options. - The network configures one from {32,64,,96,128} for the payload of rank 1, d_z^(0,1). Proposal 7: Support same payload configurations of each layer of rank 2 as the payload configuration of the layer of rank 1. Proposal 8: If rank-specific payload configuration is considered for rank 3 and rank 4, further study whether layer-specific payload is needed considering the following aspects - Spectral efficiency or system throughput as performance metric - The payload options for each layer should be within the payload options of rank-1 Proposal 9: If the smallest payload option d_z^(0,1)=32 is configured for rank-1, support layer-common and rank-common configurations for each layer of rank > 1. Proposal 10: For Rel-20 NR CSI feedback via two-sided model, RAN1 consider further define the applicability of each payload configuration to a certain range of subband / port configurations. E.g., 64 / 128 bits per layer are only applied to <=32 ports, 256 bit per layer is only applied for >32 ports. Proposal 11: Further model applicability on subband, port and payload configuration can be aligned during applicability enquiry phase. Proposal 12: UCI omission is a corner case, RAN1 should adopt the simplest solution. Proposal 13: For Rel-20 NR CSI feedback via two-sided model, adopt UCI mapping and omission based on layer indices. |
| DCM | Proposal 5 - Support the SQ as the baseline quantization scheme of the output of the encoder. Proposal 6: Given RI, support specifying multiple parameter combinations of (d_z^(l,v),Q^(l,v)) that result in different CSI payload sizes. - Q^(l,v) is configured by the NW and is independent from l,v. Proposal 7 - The specified payload size grows sub-linearly with the RI, i.e., d_z^(0,v)>d_z^(1,v)>d_z^(2,v)… for RI = v. Proposal 8: Reuse the CSI Part 1 and CSI Part 2 of NR CSI for UCI mapping. - In CSI Part 1 - UE reports RI (v) and CQI. - UE reports d_z^(l,v) or the sum of d_z^(l,v) for l=0,…,v-1 in CSI Part 1, and the UE uses the configured Q^(l,v) values to generate the CSI feedback. - Note: Due to the CSI omission, the reported d_z^(l,v) can be smaller than the configured value. - In CSI Part 2 - If necessary, UE reports how many or which scalars in the latent message are reported for each layer. - UE reports d_z^(l,v)×Q^(l,v) bits as the quantized latent message for each layer. Proposal 9 The priority of the CSI payload is decided by the indices of the scalars in the latent message. • A priority of bits corresponding to a scalar can be assigned based on its index, or the index of a group of scalars, and other factors such as the layer index. • The scalars or groups with lower priority can be omitted to adapt the uplink transmission resources. |
| CEWiT | Proposal-5: For AI/ML CSI compression, consider specifying the UCI bits mapping for higher channel rank. |

## Discussion

In this section, proposals and discussions are elaborated based on companies’ proposals. The discussion will be carried out by quantization, payload configuration for rank-1 and rank > 1 separately.

### Quantization type and codebook:

To facilitate discussion, relevant observations on quantization study in R18, and the agreement made in RAN1 #122bis R19, are captured below (TR38.843, section 6.2.2.8).

From the perspective of quantization methods for CSI feedback, - For the quantization awareness for training, it is beneficial to consider quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2) to avoid severe performance degradation. In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ). - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ) in general, where VQ achieves better performance than SQ in some cases while worse in some other cases.

Agreement For inter-vendor collaboration Direction C, - Use standardized quantization codebook For inter-vendor collaboration Direction A Options 4-1, 3a-1 (with or without NW-side target CSI sharing), - Standardize configuration(s) of quantization codebook, e.g., scalar or vector quantization, segment size of VQ, codebook size. - FFS: applicability of the above for Case 2 - Exchange quantization codebook of (the selected) standardized configuration(s) from NW-side to UE-side along with each exchanged dataset or model parameters. - FFS: whether quantization codebook may be different across different payload size configurations

Additionally, two companies, Samsung and Qualcomm provide more evaluation results

Samsung’s observation: - VQ has gain only if the codebook is learned together with model, but sequential training assumes fixed codebook for UE side training wherein the true benefit of learnable codebook may not be realized. Thus, fixed SQ codebook is sufficient. ↓X →Y; Case 1 SQ; Case 2-1 SQ; Case 2-2 SQ; Case 1 VQ; Case 2-1 VQ; Case 2-2 VQ. Case 1 SQ; 17.3%~83.2 %; 23.1%. Case 2-1 SQ; 0.46% ~4%. Case 2-2 SQ; -2.9%~-6.4%; 0.7%~3.8%. Case 1 VQ; 3.0%~21.6%; 10.7%~30%. Case 2-1 VQ; -1%~-4.5%. Case 2-2 VQ. Qualcomm did an extensive evaluation on possible payload variations of VQ and SQ. - The results show that learnable VQ outperforms SQ, and the performance SQ with learnable codebook and standardized codebook are comparable.

The key information above is

- For VQ, the major benefit comes from learnable / exchanged codebook.

- VQ is generally comparable to SQ, but both have advantages at different cases.

- Benefit of exchanging codebook is acknowledged by the Rel-19 study, but evaluations show marginal benefit for SQ, major benefit is for VQ.

Companies views on quantization type are summarized as follows:

- SQ only or SQ preferred / prioritized / at least (5 + 12):

  - Ericsson (only), Spreadtrum (only), Ofinno (only), OPPO (only), Nokia (only),

  - CATT (prioritize), CMCC (at least), SS, KT (L value), CMCC (at least), HONOR (preferred, with evaluation showing SQ > VQ with L=4), Lenovo (at least), LGE (d and Q), Apple (d and Q), MTK (at least), Toyota ITC, DCM (baseline)

- Both VQ and SQ (6+5): 6 is for who explicitly mention VQ

  - Huawei, QC, NEC, vivo (L=1,2,4), Transsion Holdings, ETRI (SQ is under VQ),

  - ZTE (Q and L values), KT (L value) , Fujistu (d, Q and L), CATT (L and Q), MTK (d, Q and L values)

- Hybrid VQ + SQ

  - IDC: VQ for coarse quantization, SQ for residual error

- Two-part quantization

  - Ofinno: first SQ for coarse granularity, second SQ for finer granularity

Companies views on quantization codebook are summarized as follows:

- Alt1 (exchanged): 10 + 8 = 18, where 10 is for SQ only / preferred

  - Ericsson, CMCC, KT, Ofinno, Lenovo, OPPO, LGE, Apple, , Fujistu, Toyota ITC, QC (for VQ), Futurewei, Huawei, IDC, vivo, ZTE, ETRI, Panasonic

- Alt2 (specified): 4 + 2 = 6

  - Xiaomi (if SQ), Nokia (SQ only), MTK (SQ only), QC (for SQ), Lenovo (if quantization not part of encoder), Transsion Holdings

- Hybrid:

  - SS (2 CBs, NW indicates one or two)

- Do not support Alt2: 1

  - ZTE

With all above study and summary, there is dependency on the quantization type and codebook determination. The decision of one issue would impact the choice of the other. For example, if only SQ is supported, the benefit of the learnable codebook is not well justified; if codebook is learnable and exchanged, there is no strong motivation to down-select VQ given that it has benefit in some cases

With such, it might be clear by merging the quantization type and codebook discussion and consider following three possibilities

- Alt1a: Support VQ with learnable codebook exchanged from NW to UE side

- Alt1b: Support SQ with learnable codebook exchanged from NW to UE side

- Alt2: Support SQ with standardized codebook

Although Alt1b has more supporters based on companies’ proposals, Alt1a does have a valid point of potential higher performance with VQ. Also, Alt1b can be absorbed in to the framework of Alt1a by exchanging a codebook with common codewords on L entries. Alt2 camp also has a valid point that standardized codebook is sufficient for SQ, and it would ensure a baseline performance which could be more robust from generalization perspective.

To sum, from performance perspective, let us consider giving the NW more flexibility by exchanging the codebook from NW to UE. With this spirit, either VQ, or SQ, or a standardized codebook, is all upto NW’s decision. From specification impact perspective, let us try to consider limit the scope of VQ so as to ease the discussion on payload scalability. Thus, FL propose the following as a unified framework

##### Proposal 3.1 (closed):

Support NW optionally exchanges codebook to the UE side.

- For a certain pairing ID, if NW exchanges a codebook, it is either SQ or VQ, not both.

  - To simplify the payload configuration discussion, the number of bits per segment in VQ is L times of the number of bits per scalar in SQ,  i.e., Qsql,ν=Qvql,ν/L, and for VQ,  support L=2 only.

- If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID.

| Companies | Views |
| --- | --- |
| Mod | @all, Learned and exchanged codebook gives flexibility to the NW. Thus, either VQ, or SQ, or a standardized codebook, is all upto NW’s decision, FL don’t see strong motivation to down-select any of them considering they all have advantages in some cases. The subbulet of the first bullet aims to limit the effort spent on VQ. SQ and VQ only different whether grouping L entries for quantization, they should share the common payload. |
| vivo | We are fine with this direction. One clarification question: Does the second sub-bullet imply that only one standardized SQ codebook can be specified? There seems no benefit to specify multiple standardized SQ codebooks. |
| Apple | Fine with the direction. But supporting L=2 only for VQ may need more discussion. Intuitively, VQ with larger L has higher complexity but performance is supposed to be better. |
| NTT DOCOMO | We are generally fine with the main bullets. For the sub-bullet, we don’t think it is necessary to connect the design of SQ and VQ, since they have different principles. The parameters of SQ and VQ can be separately optimized if both are supported. |
| Samsung | Re: SQ vs. VQ. We have illustrated in our contribution that there is no gain from VQ over scalar quantization as the model would learn how to decorrelate the outputs of the encoder when trained over SQ. Moreover, from UE-side perspective, training on network defined VQ is analogous to training over fixed VQ where the latter is proven to perform worse than training over fixed SQ. This implies there is a risk of performance degradation, if the UE-side trains its actual decoder with UE-side data (which is basic assumption in Direction A). We first need to clarify what QCB exchange would mean? It could mean one of the following two things Option#1: Indication of one of the specified codebooks Option#2: Exchange of mapping table between codewords and quantized values/vectors We are fine with Option#1. Option#2 requires to define format for the quantized values and vectors? We believe the spec. may define a set of few QCBs tailored for dynamic ranges (last layer activation functions) [Zmin and Zmax] of elements of the latent message. |
| OPPO | Fine with this direction. For the second bullet, it seems that only one SQ codebook is specified? |
| Huawei, HiSilicon | Not clear on meaning of the subbullet: if SQ and VQ are both supported, then the parameters of Q and L are separately configured. Why do they have a linkage? For one inference session, it is not likely to be configured with both SQ and VQ. BTW, considering companies supporting SQ only are more than VQ, we think we may first agree to at least support SQ, and FFS VQ. |
| ZTE | For SQ, we fell it’s better to down select between exchanged codebook and standardized codebook. If exchanged SQ codebook is supported, the additional benefit to support standardized SQ codebook is not clear while the spec effort is double. For the sub-bullet, we agree with Apple to keep it open now. Larger L values (e.g., L=16 as evaluated before) can in general bring less quantization error since the correlations among different entries in the latent message can be better captured. |
| Futurewei | We are generally fine with the direction. However, we don’t see it is necessary to limit L=2 for VQ considering the overhead and performance tradeoff. Suggest removing the sub-bullet “To simplify the payload …”. |
| MediaTek | Based on the extensive evaluation, companies have observed limited gain from VQ even for larger L. Such an aggressive segmentation with L=2 further diminish that marginal gain. So, it is somehow questioning why VQ is still worthy to be considered if L=2. |
| Mod | @Huawei, as I mentioned in the first comment row, the linkage between SQ and VQ is to reduce the additional spec effort for VQ. With that, SQ and VQ payload are same, we can focus on one of them in the payload configuration part, we don’t bother define different payloads for them. From NW perspective, a NW will choose from either SQ or VQ. @Apple, @Futurewei @ZTE, do you have evaluations showing substantial gain of L > 2? Increasing L will complicate the codebook exchange for VQ, i.e., you need 2L×Q codepoints. Given that a good number of companies want to support SQ only, let use try to reduce the additional effort of VQ. @Samsung, the majority companies prefer exchanging the codebook, so the best we can do is to absorb standardized CB as one of the NW options. Codebook format can be further discussed in 10.1.2 just as the format of target CSI and the CSI feedback. @ZTE, I think you can choose to learn the codebook and exchange, if you don’t see any benefit of the standardized option. |
| CATT | We agree with HW’s suggestion, considering companies supporting SQ only are more than VQ, we think we may first agree to at least support SQ, VQ can be FFS. Also for SQ, quantization aware training can be performed with fixed standardized quantization codebook, and according to Rel-18 study evaluation results, It can be observe that under quantization-aware training, adopting a fix scalar quantization can acquire satisfying performance. Therefore, standardized SQ can be a baseline solution. |
| Ofinno | Agree with HW’s comment |
| LG Electronics | First of all, we are fine with both VQ and SQ. The reason why we mentioned only the parameters d and Q, but not L is just that they are common in both VQ and SQ. Thus, our position could be properly captured as follows. Companies views on quantization type are summarized as follows: - SQ only or SQ preferred / prioritized / at least (5 + 1211): - Ericsson (only), Spreadtrum (only), Ofinno (only), OPPO (only), Nokia (only), - CATT (prioritize), CMCC (at least), SS, KT (L value), CMCC (at least), HONOR (preferred, with evaluation showing SQ > VQ with L=4), Lenovo (at least), LGE (d and Q), Apple (d and Q), MTK (at least), Toyota ITC, DCM (baseline) - Both VQ and SQ (6+56): 6 is for who explicitly mention VQ - Huawei, QC, NEC, vivo (L=1,2,4), Transsion Holdings, ETRI (SQ is under VQ), - ZTE (Q and L values), KT (L value) , Fujistu (d, Q and L), CATT (L and Q), MTK (d, Q and L values), LGE For the proposal, similar view with Apple and DCM. We support only the main bullets, i.e., Support NW optionally exchanges codebook to the UE side. - For a certain pairing ID, if NW exchanges a codebook, it is either SQ or VQ, not both. - To simplify the payload configuration discussion, the number of bits per segment in VQ is L times of the number of bits per scalar in SQ, i.e., Qsql,ν=Qvql,ν/L, and for VQ, support L=2 only. If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID |
| Ericsson | Agree with Huawei, we can agree on SQ first and FFS VQ. |
| Lenovo | We are generally fine with the two main bullets. But we share similar concern as HW on the subbullet, why we need such link between SQ and VQ? Additionally, based on the results of 10.1.2 discussion on having standardized SQ/VQ, we believe there is a case that - For a certain pairing ID, NW exchanges an identifier for selecting from the standardized SQ or VQ. |
| TCL | Do not support the first sub bullet that links the size of the VQ segment to the size of the SQ. |
| Xiaomi | Given more companies to prefer or prioritize SQ, we think SQ is firstly agreed to support. We share similar view with Samsung that there is no gain from VQ for separating training two-sided model especially when UE-side data distribution is not consisted with NW-side data distribution. We do not see the benefit of VQ. |
| SPRD | Agree with HW and CATT view, we can study SQ firstly. For the downselection between learnable codebook and standardized codebook, given that the standardized codebook can achieve acceptable performance and significantly reduce the air interface overhead caused by exchanging codebooks. Therefore, we support the standardized codebook for SQ. |
| Panasonic | We are fine with the direction to support the optional exchange of quantization codebook from NW to UE-side and fallback to standardized SQ codebook when NE does not exchange the codebook. |
| HONOR | We are fine with this proposal in principle. However, the exchanged codebook may be up to the NW’s implementation, so why there is a linkage between SQ and VQ, where it seems that no heavy spec effort is required additionally. |
| ETRI | Support. And for the sub-sub bullet, we don’t see much reason to restict L=2 at this stage. |
| CMCC | We can agree on SQ first and FFS VQ. |
| Transsion | Agree with DOCOMO and Huawei, the number of bits per segment in VQ and the number of bits per scalar can be configured separately. |

Following is an executive summary of companies’ views on VQ vs. SQ and exchangeable codebook. The main controversial part lies in whether include VQ in the scope or not. How to specify VQ can be the next level discussion.

- Concerns on VQ: Samsung, [Huawei?], CATT, [Ofinno?], [Ericsson?], Xiaomi, Spreadtrum, CMCC

- Concerns on restricting VQ scope: DCM, Apple, [Huawei?], ZTE, Furturewei, MTK, LGE, Lenovo, TCL, HONOR, ETRI

Based on the discussion, two alternatives are proposed regarding the way forward of this issue. 4 companies have concern on including VQ in the scope, while other 4 companies have concerns if VQ is not in the scope. FL recommend continuing discussion on these two alternatives, and we can revisit later in this meeting or next.

##### Discussion 3.1a (closed):

Alt1: Support NW optionally exchanges codebook to the UE side.

- For a certain pairing ID, if NW exchanges a codebook, it is either SQ or VQ, not both.

  - To simplify the payload configuration discussion, the number of bits per segment in VQ is L times of the number of bits per scalar in SQ,  i.e., Qsql,ν=Qvql,ν/L, and for VQ,  support L=2 only.

  - FFS L values for VQ

  - Note: proceed payload configuration focusing on SQ. The same d is applied to VQ, and Qvql,ν=Qsql,ν×L.

- If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID.

Concern: Nokia, Samsung, Xiaomi, Ofinno

Alt2: Support NW optionally exchanges codebook to the UE side.

- If NW exchanges a SQ codebook, the exchanged SQ codebook is used for this pairing ID.

- For a certain pairing ID, if NW exchanges a codebook, it is either SQ or VQ, not both.

  - To simplify the payload configuration discussion, the number of bits per segment in VQ is L times of the number of bits per scalar in SQ,  i.e., Qsql,ν=Qvql,ν/L, and for VQ,  support L=2 only.

- If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID.

- FFS whether / how VQ is supported

Concern: QC, Futurewei, LGE, vivo

There are also discussions on codebook granularity, companies’ views can be summarized as follows.

- Multiple codebooks or master/subset codebooks are needed if multiple L or Q values are specified (Huawei, Ericsson, vivo)

- If common Q and L, whether common or specific codebooks are used

- Specific to layer / ranks: ZTE

- Payload common: vivo

- Layer common: ZTE

- Layer common rank common: CMCC

- Dataset common, including all layers, all ranks and scalars / segments: QC

If different Q values are used for different payload, then it seems straightforward that different codebooks will be used. Other than that, specific codebooks to layers, ranks, segments, scalars or payloads are lack of justification. Thus, following is proposed regarding codebook granularity.

##### Proposal 3.2 (closed):

For payload option with same Ql,ν for SQ or same L,Ql,ν for VQ, a common codebook is used for each entry or each segment, regardless of layer or rank or latent dimension d.

| Companies | Views |
| --- | --- |
| vivo | Support |
| Apple | Support |
| NTT DOCOMO | Support. |
| Samsung | Agree with Ql,v=Q for SQ. |
| OPPO | Agree. |
| Huawei, HiSilicon | OK for same Q. |
| ZTE | Support |
| Futurewei | Support |
| MediaTek | Support |
| CATT | support |
| Ofinno | Fine |
| LG Electronics | Generally fine with the direction. However, it would be better to first decide whether candidates for Q values are common regardless of layer or rank (i.e., Ql,v=Q). And then, we can discuss whether common codebooks are used. When NW trains and exchanges a codebook, specific codebooks can be used for more flexibility. |
| Ericsson | We are open to discuss, but we have some questions on UE compleixty. See below for an example. To keep the total payload the same or in the same ballpark for rank2 and rank4, a rank-common Q will require a rank-specific d. Consider a case where {d, Q} = {64, 2} is for each layer for rank 2, while {d, Q} = {32, 2} is for each layer for rank 4. Then, in our understanding, for layer 2 of rank2 and layer 2 of rank4, UE needs to run two separate models as the d values are different. Is our above understand correction, if so, will the above increase UE complexity compare to a rank-specific Q and rank-common d? With rank-common d, UE only needs to run the model once for a given layer, irrespective of the rank. |
| Lenovo | Support |
| TCL | Agree |
| Xiaomi | Support |
| SPRD | Support |
| Panasonic | Support |
| HONOR | Agree with Ql,ν=Q for SQ. For VQ, the distribution of different segments may vary greatly, so further study may be needed. |
| Fujitsu | Support |
| ETRI | Support |
| CMCC | Support. |
| Transsion | Agree with HONOR that for VQ, we are open to study separate codebooks for different segments. |

To simplify the complexity and specification effort of VQ, let us define a look-up table for the exchanged codebook, and NW exchanges the codewords values sequentially. The detailed format for these codewords can be further discussed in either 10.1.1.1 or 10.1.2. Also, let us define an invariant payload size for VQ and SQ with same average bits per scalar, to ease the discussion on payload determination.

##### Proposal 3.1b (closed)

Support NW optionally exchanges codebook to the UE side.

- For a certain pairing ID, if NW exchanges a codebook, it is either SQ or VQ, not both.

  - To simplify the payload configuration discussion, the number of bits per segment in VQ is L times of the number of bits per scalar in SQ,  i.e., Qsql,ν=Qvql,ν/L, and for VQ,  support L=2 only.

  - Consider a look-up table for exchanging the codebook as following. NW exchange the entries of x0, x1,… sequentially.

| codepoints | codewords |
| --- | --- |
| 0 | x0=[x0,0,x0,1,…,x0,L-1] |
| 1 | x1=[x1,0,x1,1,…,x1,L-1] |
| … | … |
| 2Q-1 | x2Q-1=[x2Q-1,0,x2Q-1,1,…,x2Q-1,L-1] |

  - FFS the format of the codewords.

  - If NW supports SQ, FFS whether NW only exchanges the first entry of x0, x1…, or exchanges the same values for xi,0=xi,1=…=xi,L-1.

  - FFS L value for VQ

  - Note: Discuss payload configuration focusing on SQ. The same d is applied to VQ, and Qvql,ν=Qsql,ν×L.

- If NW does not exchange the codebook, a standardized SQ codebook is used for this pairing ID.

- Note: For payload parameter combinations with same Ql,ν for SQ or same L,Ql,ν for VQ, a common codebook is used for each entry or each segment, regardless of layer or rank or latent dimension d

| Companies | views |
| --- | --- |
| MTK | As the proposal are mentioning optional exchange for both SQ and VQ, we request further clarification for VQ case. How can UE assume an aligned codebook if VQ codebook is not exchanged? |
| Mod | @MTK, as stated in the second bullet, NW not exchange any codebook, only choice is the standardized SQ codebook. |
| Xiaomi | For SQ, assume Q is fixed. We think the codeword could be fixed as well. It is not necessary to define different codeword for SQ. Could the proponent for supporting to exchange codebook for SQ illustrate what is benefit of exchange codebook for SQ. We think the look-up table is only used for VQ. |
| Mod | @Xiaomi, I think you mean that for SQ a single codebook should be standardized, as opposed to exchanged. |
| LG Electronics | Regarding the 1st sub-bullet and the last (note), is it necessary to connect the designs of SQ and VQ? From our perspective, the parameters of SQ and VQ should be separately optimized as they have different principles. Of course, we agree to limit the total number of payload parameter combinations. However, whether we need a constraint like Qvql,ν=Qsql,ν×L is questionable. In addition, we may focus on VQ, since SQ can be regarded as a just subset of VQ. |
| Futurewei | We share the same concern as LG Electronics. There is no technical reason to restrict that Qvql,ν has to be L times of the number of bits per scalar as the approaches are very different; thus, each will be designed and optimized separately based on performance expectation. We acknowledge to avoid many total payload (parameter) configurations, and we suggest the following for the first sub-bullet: To simplify the payload configuration discussion, consider supporting the number of bits per segment in VQ between [Qvq,1, Qvq,2, …, Qvq,k], FFS: value of k. |
| Samsung | Do not support. We have a strong concern on supporting vector quantization. Significant performance degradation is observed (refer to the TR) when UE side trains its encoder on fixed VQ provided by the network. We should prioritize the discussion on scalar quantization. |

Further discussion is needed regarding whether / how to support VQ. The discussion is dependent on the results of 3.3c.

### Payload scalability

The plan of this section is to first determine the payload options for rank 1 reporting by selecting from potential values of {d, Q} or {d, L, Q}. The second step is to discuss the payload scalability over layers and high ranks. For the second step, we need to at least identify principles and methodologies in order for down-selection from alternatives.

Companies are encouraged to provide evaluations results for this section, but we only have two sources in this meeting.

Evaluation from Ericsson: (rank-common d, rank-specific Q) Rank; Payload; Layer 1; Layer 2; Layer 3; Layer 4. Rank = 1; 64 128 256; {16, 4} 0.7151 {32, 4} 0.7796 {64, 4} 0.8530. Rank = 2; 128 256 512; {16, 4} 0.7151 {32, 4} 0.7796 {64, 4} 0.8530; {16, 4} 0.5634 {32, 4} 0.6563 {64, 4} 0.7509. Rank = 3; 128 256 512; {16, 2} 0.6226 {32, 2} 0.7328 {64, 2} 0.8201; {16, 3} 0.5484 {32, 3} 0.6419 {64, 3} 0.7355; {16, 3} 0.4465 {32, 3} 0.5400 {64, 3} 0.6628. Rank = 4; 160 320 512; {16, 2} 0.6226 {32, 2} 0.7328 {64, 2} 0.8201; {16, 2} 0.4771 {32, 2} 0.5961 {64, 2} 0.7085; {16, 3} 0.4465 {32, 3} 0.5400 {64, 2} 0.6201; {16, 3} 0.3826 {32, 3} 0.4716 {64, 2} 0.5553. (rank-specific d, rank-common Q) Rank; Payload; Layer 1; Layer 2; Layer 3; Layer 4. Rank = 1; 64 128 256; {32, 2} 0.7328 {64, 2} 0.8201 {128, 2} 0.8894. Rank = 2; 128 256 512; {32, 2} 0.7328 {64, 2} 0.8201 {128, 2} 0.8894; {32, 2} 0.5961 {64, 2} 0.7085 {128, 2} 0.8161. Rank = 3; 96 192 384; {16, 2} 0.6226 {32, 2} 0.7328 {64, 2} 0.8201; {16, 2} 0.4771 {32, 2} 0.5961 {64, 2} 0.7085; {16, 2} 0.4162 {32, 2} 0.4896 {64, 2} 0.6201. Rank = 4; 128 256 512; {16, 2} 0.6226 {32, 2} 0.7328 {64, 2} 0.8201; {16, 2} 0.4771 {32, 2} 0.5961 {64, 2} 0.7085; {16, 2} 0.4162 {32, 2} 0.4896 {64, 2} 0.6201; {16, 2} 0.3486 {32, 2} 0.4177 {64, 2} 0.5553. (rank-specific d, rank-specific Q) Rank; Payload; Layer 1; Layer 2; Layer 3; Layer 4. Rank = 1; 64 128 256; {32, 2} 0.7328 {64, 2} 0.8201 {128, 2} 0.8894. Rank = 2; 128 256 512; {32, 2} 0.7328 {64, 2} 0.8201 {128, 2} 0.8894; {32, 2} 0.5961 {64, 2} 0.7085 {128, 2} 0.8161. Rank = 3; 112 224 448; {16, 2} 0.6226 {32, 2} 0.7328 {64, 2} 0.8201; {16, 2} 0.4771 {32, 2} 0.5961 {64, 2} 0.7085; {16, 3} 0.4465 {32, 3} 0.5400 {64, 3} 0.6628. Rank = 4; 160 320 640; {16, 2} 0.6226 {32, 2} 0.7328 {64, 2} 0.8201; {16, 2} 0.4771 {32, 2} 0.5961 {64, 2} 0.7085; {16, 3} 0.4465 {32, 3} 0.5400 {64, 3} 0.6628; {16, 3} 0.3826 {32, 3} 0.4716 {64, 3} 0.6020. Evaluations from Qualcomm:

FL note on rank 1 payload discussion

Companies views on payload scalability for a single layer (rank1) are summarized as follows:

- Varying d: Ericsson, QC

- Multiple L: vivo

- Fixed Q: Ericsson (rank1), Xiaomi, OPPO, LGE, QC

- Payload options:

  - {60, 120, 240, 360}: OPPO

  - {60,120,180,240, 300, 360, 420, 480}: OPPO

  - {64, 128, 192, 256}: QC

  - {64, 128, 256}: Ericsson

- Each payload is a single combination of {d, Q}: Apple

For rank1, from the proposals, it seems a slightly stable opinion is to consider fixed quantization resolution Q, but varying latent dimension d to support variable payload size options. For possible values of quantization resolution of SQ, from Ericsson and QC evaluation, Q=2 seems better than other choices. For possible values of quantization resolution of VQ, according to QC evaluation, Q=4 and L=2 yields the best performance. Thus, following is proposed for discussion regarding the payload of rank 1.

##### Proposal 3.3 (closed):

For payload configurations of rank 1, support the following

- Fixed Q=2 for SQ, and fixed Q=4 with L=2 for VQ

- N=4 options of d={32, 64, 96, 128} corresponding to payload size of {64, 128, 192, 256}.

| Companies | Views |
| --- | --- |
| Mod | The intention of this proposal is to determine how should we determine the payload scalability for rank 1 at least, e.g., whether we scale d, or scale Q, or scale combination of {d, Q}. The proposed values are based on the evaluations from Ericsson and Qualcomm. The principle will be considered as guidance for payload scalability for other layers / ranks, e.g., if layer 4 payload is half of layer 1, then whether we use a lower d, or lower Q, or both may use this proposal as a baseline. |
| Apple | Fixed Q=2 for SQ is fine. L, Q may need further study for VQ. But maybe we can fix average bits per latent number for VQ to be the same as that of SQ. E.g., Q/L=2 for VQ, detailed Q, L FFS. For payload size options, we may need to decide first whether we want to strictly follow legacy eTypeII. If yes and consider parameter set 1, 2 and 3 of legacy eTypeII, the payload size may need to be expanded to {32, 48, 64, 96, 128, 192, 256} for d={16, 24, 32, 48, 64, 96, 128}. Unfortunately, N seems large in this case. If no, then maybe the list only needs to be expanded by one value to payload size {32, 64, 96, 128, 256} corresponding to d={16, 32, 48, 64, 128}. Maybe we can even drop d=48 to make N still 4. |
| NTT DOCOMO | We think some general principles, like the ones in Proposal 3.4 and 3.6, should be discussed first, and then we can come back to discuss the detailed numbers. |
| Samsung | Agree with two points, - Q=2 for SQ - N=4 payload size configurations For the list of values of payload sizes (d), we may come back after agreeing on the principles for payload size determination for rank>1. |
| OPPO | Generally fine to this direction. But in our view, it is not proper to use the fixe Q=2 for SQ. In our simulations in R18/R19, for smaller payload size (e.g, 64, 128), Q=2 may be the best choice. While for larger CSI feedback payload size, lager Q (e.g., Q=3, Q=4) would be better to achieve a higher SGCS performance for AI model. Therefore, it needs more discussions. Specifically, if the codebook is exchanged from NW-side to UE-side, the flexibility of supporting multiple kinds of Q values should be considered, it is up to NW’s implementation. |
| Huawei, HiSilicon | Main text should add ‘for precoding matrix’. Fixed Q=2 for SQ is fine; but may be an additional value to adjust per layer payload size between rank 1/2 and rank 3/4, as Option 2 in proposal 3.6. For VQ, we think the value of L is too small – that makes a short codeword, then the benefit of VQ quantization shrinks. At least L=4 or 6 should be considered. But for the value set of d, we should first make some principles before going to the detailed parameters. E.g., the range of the max and min value should be comparable with the max and min value of eType II CB, respectively. |
| ZTE | We are OK for SQ, and open to study larger L, d value for VQ for better performance-complexity tradeoff. |
| Futurewei | We are fine with fixed Q=2 for SQ. However, as we commented on Proposal 3.1, limiting L=2 for VQ is too restrictive. The list of values of “d” can be discussed later. |
| MediaTek | We are fine with Q=2 for SQ as it has been observed for SQ changing the payload size by d results better performance than adjusting Q |
| Mod | @apple, @DCM, I don’t see a strong need to discuss high rank first. Rank 1 is the very first step, just like all other CSI codebooks we did. Discussion on other ranks will consider rank 1 as the staring point. My intention is that for other ranks, the payload of each layer should choose from the N=4 payload configurations. @Huawei, @ZTE, @Futurewei, and @all, let us not debate L values here. Please comment in proposal 3.1 regarding L. And again, do we see substantial gain with L=4 or 6? I think the complexity in exchanging and computation cost of 28 or 212 codepoints is very high. The main message is here are 1) fixing Q, 2) N=4 configs, and 3) d candidate values. |
| CATT | We are OK for Fixed Q=2 for SQ. VQ can be FFS. Latent dimension and payload sizes can also be FFS. We agree with HW that the range of payload sizes should be somehow comparable with respect to eType II codebook. |
| Ofinno | We have a similar view with OPPO based on the simulation results (for larger payload with Q=3,4) |
| LG Electronics | Similar view with DCM. In our view, it seems to be premature to discuss the detailed values. Let’s discuss general principle and/or methodology first. |
| Lenovo | We are fine with fixed Q=2 for SQ based on the simulation results in Rel-18/19. But for VQ, we can have further discussion. |
| TCL | Agree with the SQ part. |
| Xiaomi | For the first subbullet, we are fine with Q=2 for SQ. Since it is much more controversial for VQ, we suggest to removed it or FFS. For the second subbullet, before giving these detail values, we suggest to firstly discuss the principle for total payload size for one layer or across layers. |
| SPRD | OK with Fixed Q=2 for SQ. |
| Fujitsu | Generally fine with this direction. The detailed value of L and d may need further investigation. |
| ETRI | We have similar view with NTT DOCOMO. Prefer to discuss more general principles at this stage. |
| CMCC | Agree with this direction with Fixed Q=2 for SQ and Fixed N=4 configs. |
| Transsion | We are fine with fixed Q=2 for SQ. |

Following is an executive summary of companies’ opinions. Further discussion is needed focusing on proposal 3.3a.

FFS values: Samsung, Apple

Concerns on VQ: Apple, Huawei, ZTE, Futurewei, CATT, Lenovo, Xiaomi, Fujistu

Discuss general principles first: DCM, LGE, ETRI

Other Q values: OPPO, Ofinno, Huawei, Ericsson

##### Proposal 3.3a (closed):

For payload configurations of rank 1, support the following

- Fixed Q=2 for SQ, and fixed Q_vq=Q*L VQ

- N=4 options of latent dimension d is supported and FFS these values. d={32, 64, 96, 128} corresponding to payload size of {64, 128, 192, 256}

##### Proposal 3.3b (closed)

Down-selection to N payload parameter combinations (PC) of {d, Q}

- Candiate values for d: 32, 64, 96, 128

- Candidate values for Q: 1, 2, 3, 4

- Candidate values for N: 4, ?

- Note: These N payload PCs of {d, Q} are considered as the pool for the payload PC for each layer and each rank.

| Companies | views |
| --- | --- |
| MTK | In our view, it would be more efficient to have a separate candidate selection for each quantization scheme or make a down selection first on quantization scheme. Specifically, regarding value Q, based on the evaluation Q=2 seems the best choice for SQ but may not be sufficient for VQ, or Q=1 is infeasible for VQ, or Q=4 is unnecessarily large for SQ, etc. As our goal is to agree on few candidates, we would suggest to not waste our choices by considering both SQ and VQ together in candidate combination selection. |
| Mod | As stated in proposal 3.3b, the Q choice of VQ is derived from the Q choice of SQ to simplify the discussion. So, let use proceed the discussion assuming the Q value of SQ (or equivalently, the average bits per scalar, regardless of VQ or SQ). |
| Xiaomi | For SQ, we are fine with Q=2. Regarding these candidate values for d and N, we are general fine and open to further discuss. |
| Samsung | We discussed that the payload size of v>1 should be determined based on the payload size of rank 1. Thus, we propose the following changes. To keep the same level of total payload sizes as in eType II in the ballpark of 50-800bits, RAN1 may consider dz1,1=192 for Q=2. Down-selection to N payload parameter combinations (PC) of {d, Q} - Candidate values for d dz1,1: 32, 64, 96, 128, 192 - Candidate values for Q: 1, 2, 3, 4 - Candidate values for N: 4, ? - Note: These N payload PCs of {dz1,1, Q} are considered as the pool for the payload PCs to determine payload sizes for each layer and each rank. Example of per layer payload determination for different rank (v) values, is provided below for Q=2. dz1,1; v=1; v=2; v=3; v=4. 32; {64}; {64,64}; {64, 32, 32}; {32, 32, 32, 32}. 64; {128}; {128, 128}; {128, 64, 64}; {64, 64, 64, 64}. 96; {192}; {192, 192}; {192, 96, 96}; {96, 96, 96, 96}. 128; {256}; {256, 256}; {256, 128, 128}; {128, 128, 128, 128}. 192; {384}; {384, 384}; {384, 192, 192}; {192, 192, 192, 192}. |

##### Proposal 3.3c (closed)

Down-selection to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption

- Candidate values for L: L=2, d

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

After discussion, following was agreed by clarifying that 1) configuration is a separate discussion, 2) rank-common rank-specific model is assumed, and 3) performance tradeoff with overhead and complexity is considered.

##### Agreement 3.3c

When the Target CSI is precoder matrix, in order to down-select to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν based on performance, overhead and complexity trade-off results, consider the following candidate values:

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption for SQ.

- Candidate values for L: L=2, 4,  dl,ν

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

- Note: The above is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: Rank-common and layer-common model with payload scalable design is considered.

- Note: The above does not mean support of both SQ and VQ.

- Note: The number of parameters combinations configurations (Y) does not exceed 8.

FL note on payload scalability for rank > 1:

Companies’ views on payload scalability across layers and ranks are summarized below.

- General principles / methodology:

  - Rank 4 is comparable to rank 2: SS, Ericsson

  - Layer 1/2 payload (high / medium / low) is super set of layer 3/4 payload (medium / low): Huawei

  - The overall payload options for rank > 1 should be drawn from the same set of payload configurations for rank 1: Qualcomm

  - Spectral efficiency or throughput should be used as performance metric: Qualcomm

- Scalability of latent dimension d:

  - Layer common and rank common d: Ericsson

  - Layer-common and rank-specific d: Xiaomi, Ofinno, LGE

  - Layer-specific and rank-common d: OPPO, DCM (in decreasing order)

  - Layer specific / rank-specific d: CATT

  - Layer-specific or rank-specific d can be considered if following principles met: QC

    - Drawn from the same set of d for layer 1

    - Layer-common rank-common should be supported for the smallest payload

- Scalability of quantization resolution Q:

  - Layer-common and rank common Q: Xiaomi, OPPO, LGE, QC, DCM, CATT

  - Specific Q for different ranks / layers: Ericsson

    - Resulting in payload of {16x2=32, 16x48, 16x4=64, 32x2=64, 32x3=96, 32x4=128, 64x2=128, 64x3=192, 64x4=256} total 9 payload options.

  - Layer-group-specific {d,Q}: Huawei

    - {d1,Q1} for layer 1/2, {d2,Q2} for layer 3/4

    - Layer 1/2 are from a superset of high/medium/low, layer 3/4 are from subset of medium/low

    - Layer-common can be considered

  - Rank-specific and layer-common Q: Toyota ITC

- Non scalable d or Q

  - Spreadtrum, CMCC, Apple, MTK

- Other proposals but high level

  - Layer-common or layer-specific payload: vivo

  - Layer-specific payload: ZTE

  - Ranks-specific payload: SS

  - Rank-common layer-specific payload: ETRI

  - Layer-specific and rank-specific payload: NEC

The views are diverged and high-level. The options can be categorized by layer-common/specific, or rank-common/specific, but there are lack of more concrete proposals or examples. Moreover, companies are encouraged to provide evaluations and justifications for their proposal. Towards this, maybe it is beneficial to first agree on general principles or methodologies as guidance for the discussion.

More specifically, Samsung and Ericsson mentioned that one principle is that total payload of rank 4 should be comparable to rank 2 to avoid waste of UL resource allocation. Also, some chipset vendors e.g., Apple, Spreadtrum, MTK mentioned that layer-common rank-common payload should be considered to ease implementation complexity. But it conflicts with the principle of comparable payload of rank 2 and rank 4. QC proposed that the payload option for high rank or high layers should be drawn from the payload options of rank 1. In this way, the model developed for a certain layer with N payload options can be reused for other layers / ranks, while still maintain the comparable payload of rank 2 and rank 4.

##### Proposal 3.4 (closed):

For payload scalability of layers of rank > 1, consider following general principles

- Rank-common and layer-common model design is considered as the baseline implementation for discussion purpose.

- Limiting the total number of (per-layer) payload parameter configurations across all layers and all ranks

  - E.g., choose from the set of payload parameter configurations adopted for rank 1

- At least for medium to high per-layer payload regime, the total payload of rank 2, rank 3 and 4 are comparable to each other.

- Spectral efficiency or system throughput is considered as the performance metric.

| Companies | Views |
| --- | --- |
| Mod | @all, The first bullet is to allow the simplest model implementation considering model storage and loading capability. A single model to support all configurations, layers and ranks seems to be the most typical case. The payload scalability discussion should take this into account. Regarding the second bullet, the layer-common and rank-common model would need to adapt to all the payload parameter configurations across layers and ranks either via specific adaptation layers or truncation / masking. More options increase the training challenges and may degrade the performance for each configuration. The simplest way is to choose payload configurations from those agreed for rank 1. The third bullet is to reuse the principle of eType II, but the payload of rank 4 may not need to be exactly same as rank 2. Then intention is just to avoid linearly increasing the payload based on rank. Low payload regime needs further consideration because the principle brings additional payload options for the first layer (i.e., lower payload than the lowest option of rank 1). The intention of last bullet is a methodology of how to make the decision among the layer-specific/common and rank-specific/common alternatives. Per-layer SGCS does not account for inter-layer interference. |
| vivo | If we agree the rank-common and layer-common model directly, is the last bullet still necessary? Could we first agree on the rank-common and layer-common model without including the last bullet? Additionally, for the third bullet, do we need to state this principle explicitly? Would it be enough for NW to provide a maximum payload across all ranks, while the specific payload (selected from candidate payload configuration for a layer) used for each rank/layer can be reported by the UE? |
| Apple | We are fine with the baseline model to be rank-common and layer-common. Is the last bullet necessary? |
| NTT DOCOMO | We feel that the 3rd bullet does not help provide a guideline since its meaning is ambiguous. |
| Samsung | Agree with the assumption rank-common and layer-common model. We also agree with the third bullet to follow the principles of eType II. |
| OPPO | Support of rank-common and layer-common as the baseline implementation. The last bullet is not necessary. For different layers, layer 2, 3 and 4 can use different payload sizes to layer 1 to limit the total payload, but with the same candidate payload values. |
| Huawei, HiSilicon | Main text should add ‘for precoding matrix’. First bullet: OK with the direction. One question: If we adopt different d values (e.g., with truncation operation) between rank 1/2 and rank 3/4 for adaptation, does it still belong to the layer-common/rank common? Second bullet: OK with the direction. To clarify, the ‘payload parameter’ here refers to the value of d, right? Not Q/L (which does not impact model scalability). Third bullet: OK with the direction, but we think this principle may be too tight especially for rank 3, could be updated to: the total payload of rank 3 and 4 do not exceed the total payload of rank 2 |
| ZTE | For the 2nd bullet, we think lower layer can be allocated with more payload size since it contributes more to the beam-forming gains. For the 3rd bullet, the intention is clear, but we are not sure whether we need to strictly following the payload determination rule for eType II. In eType II, the restriction on the number of non-zero coefficients is primarily due to that further increasing the number of non-zero coefficients (i.e., larger than 2K0) only provides marginal beam-forming gains, but would largely increase the reporting overhead. However, for payload determination of AI/ML CSI feedback, more payloads generally provide better performance according to Rel-19 evaluations. |
| Futurewei | We support rank-common and layer-common as baseline implementation. |
| MediaTek | We support rank-common and layer-common model design. We also support limitting the number of the candidate payload sizes to keep training burden reasonable. However, we have concern regarding the third bullet if the intention is following eT2 principle in restricting payload size for rank 3 and 4 to be at the same level of rank 2. Such a restriction needs further study. |
| Mod | @all, let me clarify the intention again. 2nd bullets mean that the payload parameters {d,Q}or {d, L, Q} across all layers and all ranks should drawn from a common limited set, e.g., the N=4 payload parameters of rank 1. With that, the model implemented from layer 1 can be reused for other layers so as to honor the 1st bullet (layer-common and rank-common model). However, the above does not prevent a layer-specific or rank-specific payload. For example, if the payload sizes of rank 1 are 64, 128, 192, 256, then when payload 128 is configured for rank 1, the payload for layers of rank 4 can be {64, 64, 64, 64}. @Apple, @vivo, rank-common and layer-common model does not mean that the payload is rank-common and layer-common. The last bullet is used to evaluate different payload options for high rank, SGCS only provides per-layer accuracy, but does not account for inter-layer interference. |
| CATT | Generally fine with the direction |
| Ofinno | Fine in principle as a baseline |
| LG Electronics | For the sub-bullet, how to limit the total number of payload parameter configurations should be openly discussed. For the 3rd bullet, we have a similar view with vivo and DCM. In addition, as far as I know, we did not have enough discussion for the last bullet to be agreed upon. |
| Ericsson | We are fine with the proposal. |
| Lenovo | We agree to support rank-common and layer-common model as the baseline, and generally fine with the second and the third bullet. |
| TCL | Generally OK. |
| Xiaomi | We are general fine with the direction. For the second bullet, since Q and L is fixed as given in proposal 3.2, it is sufficient to limit the total number of latent message sizes, i.e., d. |
| Panasonic | We agree to vivo’s comments. We are fine with the baseline model to be rank-common and layer-common, and it could be agreed first without including the last bullet. The 3rd bullet would not be necessary. |
| HONOR | Agree with the bullets 1-3. |
| Fujitsu | We should decouple the model design and the payload configuration. For example, a layer-specific payload configuration can be supported by a payload-scalable model or multiple models with different payload sizes. We support layer-specific configuration. For example, configure more payload bits to low layers for its higher contribution to the channel capacity. We think 3rd bullet is fine, but 4th bullet is unnecessary. |
| ETRI | We are OK with the direction. One question from our side is regarding d. In our understanding, d is part of model implementations. Does the layer-common and rank-common model implementation means the layer-common and rank-common d? |
| CMCC | Support this proposal. Taking Proposal 3.3 together, we understand rank-common and layer-common model design mean different layers have same model sets corresponding to N = 4 configs, rather different layers have same model. |
| Transsion | We are general fine with the direction. |

Based on the discussion, proposal 3.4 is updated to the following by clarifying the definition of rank-common and layer-common model, and the layer-specific / rank-specific payload parameters.

##### Proposal 3.4a (closed):

For determining the payload parameters dl,ν,Ql,ν or dl,ν,L,Ql,ν  scalability of layers of rank > 1 across all layers and ranks, consider following general principles

- Rank-common and layer-common model with payload-scalable design is considered as the baseline implementation for discussion purpose.

  - Note: layer-specific / rank-specific (if supported) payload parameters are supported by this rank-common and layer-common model with payload-scalable design.

- Limiting the total number of (per-layer) payload parameters dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and all ranks

  - E.g., for the payload parameters for rank > 1, choose from the set of payload parameters adopted for rank 1, etc

- At least for medium to high per-layer payload regime, the total payload of rank 2, rank 3 and 4 are comparable to each other.

  - E.g., taking eType II payload as a reference for the definition of comparable payload.

- Spectral efficiency or system throughput is considered as the performance metric.

After further discussion, following is agreed

##### Agreement 3.4a:

For determining the payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and ranks, consider following general principles

- Rank-common and layer-common model with payload-scalable design is considered as the baseline implementation for discussion purpose.

  - Note: layer-specific / rank-specific (if supported) payload parameters are supported by this rank-common and layer-common model with payload-scalable design.

- Limiting the total number of (per-layer) payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and all ranks

- At least for medium to high per-layer payload regime, the total payload of rank 2, rank 3 and 4 are comparable to each other.

- Spectral efficiency or system throughput with tradeoff with reporting overhead is considered as the performance metric.

For the detailed options, it seems that it is slightly stable that the payload parameters for each layer of rank 2 is the same as that of rank 1. For rank 3 and rank 4, most companies consider payload scalability by choosing different latent dimension, while Huawei and Ericsson consider payload scalability via changing quantization granularities.

##### Proposal 3.5 (closed):

For each layer of rank 2, support the same payload parameters as rank 1.

| Companies | Views |
| --- | --- |
| vivo | Firstly, we prefer to share common candidate payload parameter sets{e.g.,d=32,64,…} across all the layers and rank as layer common and rank common model is widely used. And in a report, a payload parameter can be selected from candidate payload parameter sets{e.g.,d=32,64,…} for a layer and reported by UE . |
| Apple | Support |
| NTT DOCOMO | Same comments as Proposal 3.3. After the decision for Proposal 3.4 and 3.6, we can further decide this issue. |
| Samsung | Fine. |
| OPPO | Ok. But why we only limit this proposal for rank 2? |
| Huawei, HiSilicon | Main text should add ‘for precoding matrix’. OK with the principle. To clarify: the ‘payload parameter’ here refers to the value of d, right? Not Q/L (which does not impact model scalability). |
| Futurewei | Ok. |
| MediaTek | We support proposal for rank up to 4. It is not clear why it should be limited to rank 2. How UE or NW will determine payload among available options are a separate discussion. |
| Mode | @DCM, @OPPO, @MTK, let us proceed progressively, rank-by-rank. It’s obviously that rank 3/4 options are controversial, rank-specific layer-specific payload are preferred by other companies who want to avoid of wasting UL resource allocation. @Huawei, payload parameter configuration = {d,Q} and {d,Q,L}. Quantization is part of the model or not is upto implementation. In our discussion, let us accommodate these two implementations. One specific payload configuration of {d,Q} would imply a specific payload implementation. |
| CATT | Similar to vivo, we prefer to have common candidate payload parameters across layers and ranks. For each layer and each rank, the payload option can be selected and configured from the common candidate payload parameters. If companies want to add constraints to limit the payload configuration for different ranks, e.g., rank 2 has the same payload as rank 1, or rank 3 and rank 4 has less payload compare to rank 1 and rank 2, we are OK to further discuss how to limit the choose of subset payload options from the common candidate payload parameters in each rank. |
| Ofinno | Based on Mod’s comment, it is fine to have the same parameter for each layer on rank 2 |
| LG Electronics | We have the same comment as DCM. |
| Lenovo | Support |
| Lenovo | Support |
| Xiaomi | Support |
| SPRD | We are a little confused about "the same payload parameters as rank 1". Does this mean that for each layer of rank 2, the selected payload parameters are the same and exactly the same as the configurable payload parameters of rank 1? Or is it that each layer's selected payload parameter is the same as the configurable payload parameter at rank 1, but there is no requirement for the same payload parameter to be selected between each layer? |
| HONOR | Fine |
| Fujitsu | OK |
| CMCC | Fine. |
| Transsion | Support |

##### Proposal 3.6 (closed):

If layer-specific or rank-specific payload option is supported, consider following alternatives for rank 3 and rank 4 for potential down-selection

- For rank 3, consider alternatives of {a, a, a}, {a, a, b}, {a, b, b}.

- For rank 4, consider alternatives of {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c}.

- Note: a, b, and c represent a payload parameters configuration of a corresponding layer.

- Note: a, b and c may or may not be same in rank 3 and rank 4

Regarding choices of a, b and c, consider following options

- Option 0: Choose from the set of payload parameters configurations adopted for rank 1

- Option 1: Different latent dimensions for a, b and c other than those adopted for rank 1 can be considered, but use same quantization resolution Q.

- Option 2: Different quantization resolution Q for a, b and c other than those adopted for rank 1 can be considered, but use same latent dimensions d.

| Companies | Views |
| --- | --- |
| vivo | Same view as proposal 3.5, we prefer to share common candidate payload options, and each layer and rank independently selects the payload from the common candidate values For example: candidate payload parameter sets includes {32, 64, 128}, and the maximum payload is 256 for all ranks - The UE can select any value from {32,64,128}for layer 1 and rank 1 as long as the payload less than 256 - The UE can select any value from{32,64,128}for layer 1 and any value from {32,64,128}for layer 2 as long as the payload less than 256 - The UE can select any combination from{32+32+32, 32+64+128，64+64+64, 64+64+128, …} for rank 3 as long as the payload less than 256 - … |
| xApple | Option 0 regarding a, b and c values. |
| NTT DOCOMO | We don’t think Option 2 is a good approach based on the experiences from performance evaluations. Option 1 can be considered, while Option 0 can serve as a guideline, i.e., minimizing the overall number of parameter combinations with Option 1. |
| Samsung | Option 0 is the cleanest method. If payload size for rank1 is determined as X1, then for rank>2, the total payload size can be 2X1 (following eType II’s principle) . The parameters {a, b, c} may define as the ratio of payload distribution or proportions, e.g., {1/2, 1/2, 1/2 , 1/2} or { 1, 1, 1/4 1/4} with the sum resulting to 2. Payload parameters could be a bit ambiguous does it mean all parameters such as Q, L, d or just payload size proportions of layers. We suggest the following change. - Note: a, b, and c represent the ratio of per-layer payload size to the total payload size a payload parameters configuration of a corresponding layer. |
| OPPO | Prefer Option 0 for simplicity. |
| Huawei, HiSilicon | Main text should add ‘for precoding matrix’. OK to discuss with the a, b, c combination part. Our initial view: for rank 3, {a, b, b} and {b, a, a} are fine; but for rank 4, {a,a,a,a} should be a baseline; in this way, the a value for rank 4 can be half of the per layer payload size for rank 2. For the three options, we think we do not need to couple it with the candidate set of payload sizes in rank 1 (since there will be anyway smaller per layer payload than rank 1 to ensure doubled total payload size for rank 3/4). We can simply make two orthogonal options: Option 1: Same quantization resolution Q with rank 1, but different d value than rank 1 of the same PC. => E.g., rank 1 is 64 bits with d=16 and Q=4 (SQ); rank 2 is {64 bits, 64 bits} with the same d, Q to rank 1; rank 3 is {64 bits, 32 bits, 32 bits} with d=16 and Q=4 for layer 1 and d=8 and Q=4 for layer 2/3; rank 4 is {32 bits, 32 bits, 32 bits, 32 bits } with d=8 and Q=4 for layer 1/2/3/4. Option 2: Different quantization resolution Q than rank 1, but same d value with rank 1 of the same PC. => E.g., rank 1 is 64 bits with d=16 and Q=4 (SQ); rank 2 is {64 bits, 64 bits} with the same d, Q to rank 1; rank 3 is {64 bits, 32 bits, 32 bits} with d=16 and Q=4 for layer 1 and d=16 and Q=2 for layer 2/3; rank 4 is {32 bits, 32 bits, 32 bits, 32 bits } with d=16 and Q=2 for layer 1/2/3/4. Rank1; Rank2; Rank3; Rank4. PC1; 64; 64, 64; 64, 32, 32; 32, 32, 32, 32. PC2; 128; 128,128; 128,64,64; 64, 64, 64, 64. …. |
| ZTE | The candidate payload sizes for rank 3 and rank 4 have a strong dependency on proposal 3.3, such as the number of candidate payload sizes for rank 1. We can first agree on some high-level design principles like proposal 3.4, or lower layers are given precedence over larger layer for payload allocation. For the down-selection between option 1 and 2, we prefer Option 1, that is, a common quantization codebook is supported regardless of the rank, layer, or latent dimension. |
| Futurewei | We prefer Option 0. |
| MediaTek | Option 0. |
| Mod | @Huawei, option 1 and option 2 introduce new payload parameter configuration compared to rank1, so there would be total N+x payload options where N is the payload options for rank1. So if rank-common and layer-common model is developed, this model has to be trained with all the N+x payload options. @Samsung, let us consider the configuration method as a separate discussion. @vivo, please do not mix UE determination payload here, let us first decide the options. Even if UE decides the payload, UE should follow the agreed options, otherwise how can you ensure the NW is able to interpret your determined payload? Please note that it is NW-first training, what payload can be supported is determined by NW first. |
| CATT | Similar to proposal 3.5, we prefer to have common candidate payload parameters across layers and ranks. For each layer and each rank, the payload option can be selected and configured by NW from the common candidate payload parameters. If companies want to add constraints to limit the payload configuration for different ranks, e.g., rank 2 has the same payload as rank 1, or rank 3 and rank 4 has less payload compare to rank 1 and rank 2, we are OK to further discuss how to limit the choose of subset payload options from the common candidate payload parameters in each rank. |
| Ofinno | Option 0 can be considered as a baseline first |
| LG Electronics | We support Option 1, as the latent dimension has more direct relationship with model design than the quantization resolution Q. Introducing common/fixed Q values is more preferable. |
| Lenovo | We slightly prefer option 1. |
| TCL | Support option 2. |
| Xiaomi | Considering model training complexity, Option 0 is preferred due to its simplicity. |
| HONOR | Prefer option 0 for simplicity |
| Fujitsu | We share the similar view as that of ZTE. |
| ETRI | Our preference is Option 0. |
| CMCC | Support Option 0. And consider Proposal 3.6 for rank = 2, we prefer to keep the payload of layer 1 and 2 is same for rank 3 and rank 4: - For rank 3, consider alternatives of {a, a, a}, {a, a, b}, {a, b, b}. - For rank 4, consider alternatives of {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c}. |
| Transsion | We prefer Option 0. |

##### Proposal 3.6b (closed)

For payload parameter combinations (PC), consider followings

- Support N’ <= N multiple PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent per-layer PCs chosen from the per-layer PC pool.

|  | Layer 0 | Layer 1 | Layer 2 | Layer 3 |
| --- | --- | --- | --- | --- |
| Rank 1 | X |  |  |  |
| Rank 2 | X | X |  |  |
| Rank 3 | X30 | X31 | X32 |  |
| Rank 4 | X40 | X41 | X42 | X43 |

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {X30, X31, X32} = {a, a, a}, {a, a, b}, {a, b, b} for potential downslection.

- For rank 4, consider alternatives of {X40, X41, X42, X43} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c}.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

| Companies | views |
| --- | --- |
| Xiaomi | For rank 3 and 4, we prefer that X30, X31 and X32 are same or X40, X41, X42 and are same. When they are different, what is the relationship of size among a, b, c? e.g., a>b>c or a<b<c? |
| Mod | @Xiaomi, relationship of a, b, c will be discussed and justified by simulation. |
| Samsung | Ok. |

After discussion, the proposal is agreed as follows with minor change on clarifying the pair of {d,Q} or triplet of {d, L, Q},

##### Agreement 3.6b

For payload parameter combinations (PC), consider followings

- Support Y configurable PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent a pair of dl,ν,Ql,ν or a triplet of dl,ν, L,Ql,ν  chosen from the per-layer PC pool.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {Xn_31, Xn_32, Xn_33} = {a, a, a}, {a, a, b}, {a, b, b} for potential down selection.

- For rank 4, consider alternatives of {Xn_41, Xn_42, Xn_43, Xn_44} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c} for potential down selection.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

To facilitate the discussion and down-selection of payload parameter combinations, following guidance is proposed.

##### Proposal 3.7 (closed)

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , average SGCS across layers can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

After discussion, concern on average SGCS across layers is raised as some of the pairs / triplets are applied to some set of layers, but some others may be used to other layers. Thus, per layer SGCS can be considered for down-selection of pairs of {d, Q} or triplet of {d, L, Q}

##### Agreement 3.7

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , SGCS per layer can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

### Payload configurations and UE determinations

FL note on payload configurations:

Regarding payload configuration options, following proposals are observed from companies’ tdoc.

- Indicators:

  - Configure {d,Q}, or {d, Q, L} separately from pairing ID: OPPO, DCM, Fujistu, MTK, NEC

  - Configure payload parameters in form of {d, Q, total payload}: SS

  - NW configures max payload size and allowable rank: Apple

  - Configure d and total budget B: Nokia

  - configure payload size: vivo, Xiaomi

- Across layers / ranks

  - The payload size can be configured directly in inference report configuration: vivo

  - Configure total payload size for each rank or first layer: Xiaomi

  - Directly configure the payload parameters for each layer: ZTE, CMCC

Some companies propose to configure the payload parameters of latent dimension and quantization resolution, while another option is to configure the payload size. For payload configuration of high rank, some companies propose explicit indication of the payloads for each layer. FL thinks this issue has lower priority, we can discuss it after finishing the payload parameters and scalability issue.

FL note on UE determination of payload size:

There are 8 sources proposing a mechanism for UE reporting latent dimension / payload size

- UE selects the maximum payload size to avoid omission: Google

- Payload truncation indication of each layer: vivo

- UE selects proper payload option per channel condition: KT

- Indication of latent dimension: Lenovo

- Reports selected payload option: OPPO

- Payload size indicator: LGE

- UE determines payload size per layer and indicate in CSI part 1: Apple

- UE report the total latent dimension d in CSI part1, and per layer dimension in CSI part 2: DCM

##### Discussion 3.7 (low priority):

Is it feasible for UE to determine and report the latent dimension or quantization resolution?

| Companies | Views |
| --- | --- |
| Mod | From reading of the proposals, the motivation can be categorized as following: - To avoid CSI omission - To further reduce overhead, similar to the NNZC reporting in eT2. For avoiding CSI omission, FL does not think it is feasible. It should be noted that UCI packing and CSI processing are two orthogonal and independent process. When UE measures and compute the CSI, UE does not know how much the available UL resources is as there would be other UCI multiplexed with the CSI reports. For further reducing overhead, it seems to be a different situation than eT2. The angular-delay domain representation in eT2 may be sparse, but it may not be the case for latent space. Furthermore, indicating the location of these non-zero entries also requires some additional overhead. |
| NTT DOCOMO | We don’t think it is feasible because NW should train the models to adapt to all possible latent dimensions and quantization resolutions. They should be NW-configured or specified, and the number of parameter combinations should be limited as well. |
| Samsung | CSI omission is to be discussed separately. For truncation and zero-padding based approach, the network may train a decoder which supports CSI reconstruction on finer payload size granularity than the configurable N payload sizes. In this case, the network may define the segment sizes for the UE to perform dropping (analogous to Group 0, 1 2 of part 2 CSI in eType II) |
| Huawei, HiSilicon | Not needed. |
| ZTE | For legacy eType II, the UE can determine the number of NZC since the selection of non-zero coefficients can be done by UE implementation easily, such as by simply discarding weak paths based on path power. However, for payload determination of AI/ML CSI feedback, it's not clear how to select the actual payload size if there are some levels of flexibility allowed, given that the reconstructed CSI is transparent to the UE side. |
| Futurewei | No. It should be network that configures the latent dimension or quantization resolution |
| MediaTek | We think it is needed if NW configures the total payload size for all layers and UE determines what payload option (d,Q) it can use for each layer. Also, it may be needed if the omission occurs to a part of all layers. |
| CATT | Agree with the FL, not needed |
| Ofinno | At least for quantization resolution, some clarification is needed since exchanged codebook seems to have some flexibility on the resolution. |
| TCL | Agree. |
| HONOR | No. The latent message size and quantization resolution should be configured by NW. |
| Fujitsu | No need to discuss. |
| ETRI | In our view, UE can determine the payload size among the N payload sizes, e.g., by applying different quantization parameters (i.e., Q, L) for same d. |
| CMCC | NW configuration is enough. UE just need to report the RI. |
| Transsion | Agree with MediaTek that UE report the latent dimension if NW configures the total payload size for all layers. |

FL note on association between payload configuration with other configurations:

Three sources, CATT, HONOR and QC, proposed that association between payload configuration with other configurations, e.g., Tx port and subband, can be considered. For example, some payload is too small for a full bandwidth or 128 ports reporting or rank 4 reporting, or some other payload is too high for a small number of ports, or small bandwidth. FL thinks the discussion can be postponed after finishing the payload configurations and scalability.

### UCI mapping and omission

Two-part UCI seems to be common understanding as the actual payload would depend on the RI reported in CSI part 1. The major controversial part lies in the UCI grouping, mapping and omission order. Generally, there are two examples, one is based on layer indices which seem to be simplest approach; another option is based on portions of each layer similar to the eT2 approach. The fist portion of all layers have a higher priority than a second portion of all layers. The former maintains the per-layer quality when CSI omission occurs but rank decreases, while the latter maintain the rank with the sacrifice of per-layer CSI quality. The following is a summary of companies’ position.

UCI mapping / omission / truncation

- Layer-wise priority and omission (2):

  - QC, Apple

- Per-layer grouping / Truncation-based / hierarchy payload scalability (11):

  - Huawei, Xiaomi, SS, HONOR, OPPO, MTK, Vivo, ZTE, KT, Nokia, DCM

For the second approach, a hierarchical structure of the latent message is needed to ensure the omitted CSI is still readable by the NW. Compared to the non-hierarchy latent message, the performance may be degraded as the non-hierarchy latent message may be output by specific output layers. Further the omission may result in new payload options, which further increase the inter-vendor training challenges. FL think it would be more proper to discuss this aspect after finishing the payload parameters and scalability issues.

# Report and resource configuration

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 1: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), support using CSI-ReportConfig for inference measurement and report configuration. Indicate at least the following in CSI-ReportConfig: • An ID for pairing encoder at UE side and decoder at NW side • A new report quantity, e.g., “AIML-cri-RI-PMI-CQI” • Number of TX antenna ports • Number of subbands • CSI-RS resources for channel measurement • Resources for interference measurement |
| Huawei, HiSilocon | Proposal 2: For measurement and report configuration,  Support P/SP/A-CSI-RS as CSI-RS resource types.  Support SP/A-CSI as CSI report types.  Introduce a new CSI report quantity based on latent space. Proposal 12: For AI/ML CSI compression, deprioritize the CBSR discussion. Proposal 13: For Rel-20 CSI feedback via two-sided AIML models, support up to 128 ports, 19 subbands, and rank-4 (with precoding matrix as Target CSI). |
| Google | Proposal 3: The CSI report configuration for AI/ML based CSI compression is based on the Rel-19 eType2 CSI report configuration • The CSI report configuration can additionally provide a list of model IDs corresponding to different CSI payload size • The UE selects the model according to the maximum payload size for the CSI report to avoid the CSI omission Proposal 8: Do not support to specify the quantization scheme. • The quantization operation can be exchanged during inter-vendor collaboration procedure |
| CATT | Proposal 2: For inference report of AI/ML-based CSI compression, support aperiodic and semi-persistent CSI report. Proposal 3: For AI/ML based CSI compression, rank <= 4 can be supported. Proposal 4: For AI/ML based CSI compression, up to 128 Tx ports can be supported. Proposal 5: For AI CSI compression, support the following options for configurable subband sizes:  Option 1: Reuse the legacy mechanism.  Option 2: Select one subband size for each BWP Proposal 6: The reported subband configuration (i.e., csi-ReportingBand) can be restricted for AI/ML-based CSI compression. Proposal 12: For inference report configuration of AI/ML based CSI compression, new reportQuantity or new parameters dedicated to AI CSI compression can be introduced. Proposal 13: The value of N1,N2 as well as RI restriction can be configured for inference for AI CSI compression. Proposal 14: For CSI resource configuration for channel measurement in AI CSI compression, CSI-RS resource configuration for R19 eType II codebook based CSI reporting can be reused.  Support periodic CSI-RS, semi-persistent CSI-RS and aperiodic CSI-RS Proposal 15: For AI CSI compression, reuse the legacy mechanism used for CSI resource configuration in interference measurement for R19 eType II. |
| Vivo | Proposal 2: For CSI compression using UE-side model, periodic, semi-persistent, and aperiodic CSI-RS are supported, and semi-persistent, and aperiodic inference report are supported. Proposal 3: CSI-ReportConfig is used for the configuration of inference results reporting with the following enhancements. • Introduced new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI compression. Proposal 5: Study whether a dataset (direction 4-1), a model parameter direction 3a-1 and/or a specified model in direction C) can be associated with same pairing ID • If yes, one additional ID also needs to be indicated in inference configuration to indicate which method or which reference model is used |
| Xiaomi | Proposal 8: AI model based CSI is reported aperiodically or semi-persistently on PUSCH resources. |
| TCL | Proposal 3: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects: • AI/ML-specific CSI-RS resource configuration for CSI compression • AI/ML-specific fields in CSI-ReportConfig IE • Dedicated report quantities and report configurations for AI |
| ZTE | Proposal 5: For the CSI reporting setting for inference, pairing ID shall be configured to enable the UE to select an encoder compatible with the decoder used by the gNB. Proposal 6: For the CSI reporting setting for inference, support UE to indicate whether the CSI content is generated using the legacy codebook or the deployed AI/ML model. |
| Samsung | Proposal#2: Support at least the following for the CSI measurement and reporting - A single CSI-RS resource  16, 32 ports - Up to rank 4 - FFS: restrictions on the number of sub-bands - FFS: other configuration aspects, e.g., rank restriction, CBSR |
| Lenovo | Proposal 2: Reuse CSI-ReportConfig to configure the CSI report with AI based CSI compression by introducing new reportQuantity for CSI report with AI based CSI compression. Proposal 3: A pair ID is configured with a CSI-ReportConfig used for CSI report with AI based compression. Proposal 4: RAN1 needs to study how to support various subband and/or Tx port configuration with a same pair ID. |
| Panasonic | Proposal 2: At least for inference, legacy CSI measurement and report configuration framework can be reused. Proposal 3: At least for inference, periodic, semi-persistent, and aperiodic CSI-RS are supported as CSI-RS resource type for CMR. Proposal 4: At least for inference, new RRC parameter for CSI report configuration to distinguish CSI report of AI/ML-based report and non-AI/ML-based report. Proposal 5: For inference configuration, the same ID as pairing ID for AI/ML-based CSI compression can be used. |
| OPPO | Proposal 6: Reuse the legacy framework of CSI report configuration and CSI-RS resource configuration for CSI measurement. Proposal 7: Support to indicate whether the CSI report is through AI/ML or legacy codebook, following options can be considered:  Option 1: introduce new reportQuantity, e.g., AI-PMI.  Option 2: introduce new codebookType, e.g., codebookAI-r20  Option 3: introduce a new field, e.g., AI-r20, instead of within codebookType field. |
| Nokia | Proposal 1: For ML-based CSI compression, support semi-persistent and aperiodic reporting for AI/ML-based CSI compression feedback over PUSCH. Proposal 2: For ML-based CSI compression, use legacy (non-AI/ML) eType-II processing timelines and triggering as the starting point for AI/ML-based CSI compression feedback. Proposal 5: Support the possibility to configure hard codebook subset restriction (CBSR) for a CSI reporting setting for inference, based on legacy CBSR higher-layer parameter, such that a bit value of zero indicates that the target precoding matrix for inference is not allowed to correspond to any precoder associated with that bit. |
| LGE | Proposal #2: Consider the methods for association between pairing ID and payload configuration (e.g., via signaling procedure for applicable functionality reporting). Proposal #7: Consider report of both inference result and monitoring result together when monitoring occasion is occurred. |
| Apple | Proposal 2: For AI based CSI compression inference configuration, given the flexible payload size generated by AI model, consider support periodic, semi-persistent and aperiodic reporting. Proposal 3: For AI based CSI compression inference configuration, a paring ID should be configured to identify the matching CSI generation and CSI reconstruction model. Several signaling options can be considered: • Alt 1: Pairing ID is configured together with resourceForChannelMeasurement. • Alt 2: Pairing ID is included within codebookConfig. • Alt 3: Pairing ID is associated directly with reportQuantity. • Alt 4: Pairing ID is introduced as a separate field within CSI-reportConfig. Proposal 4: For AI based CSI compression inference configuration, introduce a new report quantity aiPMI, to distinguish PMI feedback using legacy codebook. Proposal 5: For AI based CSI compression inference configuration, introduce a new IE (e.g., AI-CSICompressionConfig or CodebookConfig-r20) to consolidate configuration parameters relevant to AI based CSI compression. The IE minimum contains rank restriction parameters. |
| NEC | Proposal 2: In CSI spatial/frequency compression, support inference report configuration reuse the CSI-ReportConfig with additional contents of model ID/ pairing ID and quantization parameters. Proposal 3: In CSI spatial/frequency compression, introduce new reportQuantity for compressed PMI in CSI-ReportConfig. |
| Nokia | Proposal 8: Support the possibility to configure hard codebook subset restriction (CBSR) for a CSI reporting setting for inference, based on legacy CBSR higher-layer parameter (hard CBSR only). |
| Transsion Holdings | Proposal 1: The following potential spec impacts related to CSI report configuration of CSI compression should be considered: • Signaling to trigger the AI/ML model CSI compression • Pairing between CSI generation model and CSI reconstruction model • Indicate the CSI feedback payload • CSI-RS resource configuration • CSI reporting configuration |
| Fujistu | Proposal 2:  For CSI compression Case 0, the legacy CSI-RS resource configuration could be reused at the inference stage. |
| ETRI | Proposal 2: For AI/ML-based CSI compression, reuse the measurement resource configuration in the legacy CSI framework. Proposal 3: For AI/ML-based CSI compression, reuse the report configuration in the legacy CSI framework and further study following aspects: • Mechanism to indicate to construct CSI feedback payload using AI/ML-based encoder model • Method to ensure consistency between the NW-side model and the UE-side model • Method to indicate how to derive CQI (Channel Quality Indicator) and RI (Rank Indicator) • Configuration to support multiple CSI payload length using the AI/ML-based encoder model. |
| CMCC | Proposal 3: Periodic CSI report, semi-persisted CSI report on PUCCH, semi-persisted CSI report on PUSCH and aperiodic CSI report can be supported for AI/ML based CSI compression. Proposal 4: CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations can be supported for AI/ML based CSI compression. Proposal 5: In addition to up to 32 ports CSI, 48, 64, 128 ports CSI should also be supported in AI/ML based CSI compression. Proposal 6: For up to 32 ports CSI, only one CSI-RS resource can be configured for one CSI report configuration, each CSI-RS resource is configured with up to 32 ports. Proposal 7: For 48, 64 and 128 ports CSI, up to 4 CSI-RS resources can be configured for one CSI report configuration, each CSI-RS resource is configured with up to 32 ports, and these CSI-RS resources are aggregated to form up to 128 CSI ports. |
| ETRI | Proposal 10: For AI/ML-based CSI compression, investigate the feasibility of introducing codebook subset restriction in the AI/ML-based CSI feedback framework. |
| ITL | Proposal 2 For CSI compression using two-sided model sub-use case, following CSI reporting related configurations can be at least considered for AI/ML based approaches: • The maximum CSI payload size • Set of actual CSI payload size with payload set ID or AI model type ID • Quantization type/ID with parameters • Number of subbands, Number of CSI-RS antenna ports / antenna panel dimensions • Rank restriction • LCM type information Proposal 3 For CSI compression using two-sided model sub use case, it should be considered to introduce new or modified type of CSI part(s) accommodating AI/ML specific CSI contents/fields |
| CEWiT | Proposal-1: For AI/ML based CSI compression, take the current CSI framework as starting point. Proposal-2: For AI/ML based CSI compression, specify the CSI-RS resource type for inference configuration. Proposal-3: For AI/ML based model inference, introduce a new parameter in reportQuantity corresponding to CSI Report. |

## Discussion

FL note on distinguishing the ML CSI feedback from other non-AI CSI report:

Some companies propose to adopt a new RRC parameter, or a new CSI report quantity, or new codebook configuration, to distinguish the ML CSI feedback from the legacy non-AI CSI report. Another possibility is to identify the CSI feedback via two-sided inference by a CSI report configuration with pairing ID.

##### Proposal 4.1 (low priority):

Study, and potentially select from, following options for identifying a CSI report via two-sided inference

- Alt1: by pairing ID

- Alt2: by a new report quantity

- Alt3: by a new codebook configuration or payload indication

- Alt4: by other new RRC parameters

| Companies | Views |
| --- | --- |
| vivo | Alt 2 |
| NTT DOCOMO | Both Alt. 2 and Alt. 3 can work. Alt. 1 should work as well, depending on the progress of the pairing ID discussions. Therefore, at least Alt. 4 is not necessary. |
| Samsung | This can be discussed later after things become clearer on what a CSI report configuration would look like for CSI compression. |
| Huawei, HiSilicon | Alt2 or Alt3 can be a unified solution. For Alt1, not sure whether pairing ID must be mandatory, considering Direction C may specify only one model. |
| ZTE | We prefer Alt2 which is simple and aligned with AI BM and CSI prediction. |
| Futurewei | Prefer Alt2. |
| MediaTek | We are fine with both Alt 2 and Alt 3, but prefer Alt 2. |
| CATT | Alt 2 or Alt 3 can work. |
| Ofinno | Fine for Alt2/3 |
| InterDigital | Support in principle, we can do down-selection in the next meeting. |
| LG Electronics | We have the same comment as Samsung. However, Alt1 is preferred. |
| TCL | Alt 1 or Alt2 is fine. |
| SPRD | Prefer Alt 2. |
| Panasonic | Alt.2 similar to CSI prediction in Rel.19 |
| HONOR | Alt 2 is preferred and can be aligned with AI-CSI reports in R19 |
| Fujitsu | We think Alt1 and Alt3 can be considered. |
| ETRI | Prefer Alt 2. |
| CMCC | At least Alt 2 can distinguish it. |
| Transsion | We prefer Alt2 and Alt3. |
| NEC | We prefer Alt 2. |

FL note one CSI framework:

Regarding on the CSI report configuration and measurement resource configuration associated with the CSI feedback via two-sided inference, following was proposed in last meeting and seemed to be quite stable.

##### Agreement 4.2:

For R20 CSI feedback via two-sided AIML models, reuse legacy CSI reporting framework

- Support CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations.

- Support aperiodic report and semi-persistent CSI report on PUSCH and reuse legacy triggering mechanism.

| Companies | Views |
| --- | --- |
| Mod | There seems no objection from the last meeting. Please comment if you are not okay with this proposal. |
| Samsung | Support |
| Futurewei | Support |
| MediaTek | Support |
| CATT | support |
| Ofinno | Support |
| InterDigital | Support |
| CL | Support |
| SPRD | Support |
| CMCC | Support |
| Transsion | Support |

All companies support reusing legacy CSI report framework, so the original proposal 4.2 is agreed.

FL note on CBSR:

As mentioned in section 2, two sources mentioned the feasibility of supporting CBSR for the CSI feedback via two-sided inference. In the last meeting, the feasibility of legacy CBSR principle were discussed, companies’ comments are copied below.

| Companies | Comments from RAN1 #122 (please DO NOT edit) |
| --- | --- |
| OPPO | In our view, CBSR may cause the mis-alignment between inference and training for the UE-side encoder and NW-side decoder, since this kind of CBSR may not be considered in model training stage. In additional, there is no explicit beam concept in AI/ML CSI feedback, the latent message of encoder output is a feature extraction based on the CSI eigenvector. Therefore, no need to discuss this issue. |
| Xiaomi | We share similar view with OPPO. Currently, we don’t see the necessity to discuss this issue |
| Huawei, HiSilicon | Calculating CBSR may need additional T2 conversion to the angular domain, which increases complexity. We may consider it with lower priority. |
| NTT DOCOMO | CBSR is an important feature for network interference control during the network operation. It should be studied and supported. If it is not supported, the NW has to ignore some directions which reduces the efficiency of the AI/ML CSI and introduces bias on the CQI. The performance gain of AI/ML CSI will be degraded. |
| ZTE | We share similar understanding with OPPO. |
| CMCC | CBSR has clear physical meaning related with DFT based codebook, we think it is not needed here. |
| Ericsson | We believe CBSR is an essential feature that must be considered for coexistence with other services, such as satellite. If the two-sided CSI compression use case cannot even support this basic legacy feature, its overall usefulness is highly questionable. |
| Futurewei | We share similar view with OPPO. |
| MediaTek | Similar view as OPPO |
| CATT | We fail to see the necessity of CBSR. This can be low priority discussion |
| vivo | We see the beneficial of CBSR for network interference control but with additional complexity. |
| Samsung | Ok. |
| LG Electronics | Support |
| Google | OK |
| TCL | Agree with OPPO. |

OPPO, Xiaomi, Huawei, ZTE, CMCC, Futurewei, MTK, CATT, TCL presented concerns on applying CBSR. Given that it is pretty early stage of the release, let us continue discuss and share opinions here. More concrete proposals can be brought up at a later stage based on the views and after finishing the target CSI type discussion.

##### Discussion 4.3 (low priority):

Please share comments on the necessity, feasibility and how to support CBSR for CSI feedback via two-sided inference.

| Companies | views |
| --- | --- |
| NTT DOCOMO | CBSR is an essential feature for practical network operations. It is used for interference management in real-life deployments. If a UE reports CSI with directions on which NW does not want to transmit signals, there will be CQI mismatch that is hard to be compensated by NW. Therefore, the alignment between NW and UE on which directions can be used is essential. The major concern of CBSR is UE complexity. For this issue, we can consider only supporting hard CBSR. UE can remove a limited number of components from CSI using basic matrix operations without applying DFT. |
| Samsung | We believe it is necessary to support CBSR. The configuration could follow the legacy SD basis restriction and at least hard restriction could be supported. How UE applies CBSR is up to implementation. The UE may project the target CSI into the null space of the submatrix from the restricted basis vectors or represent the target CSI with unrestricted basis (e.g., by removing components for the restricted basis vectors.) |
| Huawei, HiSilicon | No need to do it, since AI/ML CSI compression does not convert the to-be-compressed CSI into beam domain. Even in legacy, CBSR is nice to have (to reduce UL overhead), not a must. We do not get any information of commercializing this feature. |
| CATT | This is low priority issue for the moment |
| TCL | Agree with Huawei’s comments. |
| ETRI | We shares a similar view with NTT DOCOMO and SS, as we shares the need of the CBSR. |
| CMCC | Not needed. In addition, Rel-19 MIMO CSI for up to 128 ports introduced another CBSR enhancement. Which kind of CBSR to introduce here will also need to decide. |
| Transsion | Not needed. |

Apart from the proposals elaborated above, there are other proposals on reporting options on top of the inference report, e.g.,

- UE determining whether a fallback performed, and if so, report non-AI PMI.

- UE report inference and monitoring report in the same report.

These can be discussed after all above aspects become stable.

# CQI assumptions

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Ericsson | Proposal 7 Support option 1a as the method for CQI calculation. |
| Futurewei | Proposal 3: For Rel-20 AI/ML-based CSI spatial/frequency compression without temporal aspects (“Case 0”), for CQI determination, if the actual or reference CSI reconstruction model is available at UE, adopt Option 2a to determine CQI at UE: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment. |
| Spreadtrum | Proposal 3: For CQI determination in CSI report, support option 1 as starting point. |
| Huawei, HiSilicon | Proposal 4: For CQI determination, consider Option 1a/1b, i.e., it is determined with realistic channel measurement, and with potential adjustment indicated by gNB if needed. |
| InterDigital | Proposal 3: Support at least one of Option 1-b or Option 2-b for CQI determination. |
| Google | Proposal 4: Support the RI/CQI calculation and report based on precoded CSI-RS. |
| CATT | Proposal 16: For inference report of AI/ML-based CSI compression, if CQI in CSI report is configured, CQI is calculated based on target CSI with realistic channel measurement.  Same quantization scheme can be reused as in legacy CQI reporting Proposal 17: For inference report of AI/ML-based CSI compression, RI is determined by UE with rank adaption. RI restriction is supported. |
| Vivo | Proposal 4: CQI can be calculated based on standardized decoder/nominal from RAN4. • FFS on whether additional specification is needed. |
| Xiaomi | Proposal 10: Support Option 1a, CQI is calculated based on target CSI with realistic channel measurement to determine RI/CQI. |
| ZTE | Proposal 7: For CSI feedback using two-sided model, for CQI determination in a CSI report, support Option 1a: CQI is calculated based on target CSI with realistic channel measurement. Proposal 8: For CSI feedback using two-sided model, for RI determination in a CSI report, legacy RI determination can be reused as a starting point. |
| Samsung | Proposal#9: Defer the discussion on CQI determination schemes for two-sided model-based CSI compression until some progress on PMI feedback is achieved. |
| Lenovo | Proposal 5: For AI based CSI compression report, CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation. Proposal 6: For AI based CSI compression report, RI is calculated based on the realistic channel estimation based on the CSI-RS resource indicated by CRI as legacy. |
| Panasonic | Proposal 9: For CQI determination in CSI report, deprioritize Option 1c and 2b.  Option 1c: CQI is calculated based on legacy codebook  Option 2b: CQI is calculated using two-stage approach. UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder. Proposal 10: For CQI determination in CSI report, Option 1a or 1b should be baseline. The necessity of Option 2a can be further discussed.  Option 1a: CQI is calculated based on target CSI with realistic channel measurement  Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment  Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform construction model inference with potential adjustment.  The CSI reconstruction part for CQI determination at the UE is a nominal or reference model, which is different from the actual CSI reconstruction part at the network. |
| OPPO | Proposal 4: Regarding the RI determination, support the RI report with min⁡(RI_1,RI_2), where RI_1 is calculated with PMI as target CSI and RI_2 is calculated with PMI as legacy codebook. FFS: how to determine the legacy codebook Proposal 5: Regarding the CQI determination, support CQI report with both of CQI_1a and CQI_1c, where CQI_1a is calculated with PMI as target CSI and CQI_1c is calculated with PMI as legacy codebook FFS: how to determine the legacy codebook FFS: CQI quantization method FFS: CSI report configuration |
| HONOR | Proposal 4: For CQI determination, support option 1a, CQI is calculated based on target CSI with realistic channel measurement. |
| Nokia | Proposal 6: For calculating the CQI, support CQI calculation based on target CSI (Options 1a). An adjustment of the CQI may be applied by a UE, based on the receiver implementation in a network-transparent manner. |
| LGE | Proposal #2: For CQI determination in CSI compression using two-sided model, consider to prioritize Option 1b (via realistic channel measurement + potential adjustment) or Option 1c (via legacy codebook). |
| Apple | Proposal 6: For AI based CSI compression, CQI/RI is calculated based on target CSI with realistic channel measurement, with potential adjustment. |
| NEC | Proposal 8: For CQI determination in CSI report, if a proxy CSI reconstruction part at UE side is proven to be feasible, support Option 2a; if not, support Option 1a/1b. Proposal 9: For CQI determination, Options 1c and Option 2b can be deprioritized due to the respective disadvantages in computational efficiency and signaling overhead. |
| Sharp | Proposal 1: For CQI determination, for option 1c, discuss the precoding information type of the output CSI. Proposal 2: For CQI determination, prioritize option 1c and option 2a-2. For UEs with different capabilities, UEs may support different options based on their capabilities. Proposal 3: Support a type of CQI determination based on UE capability. |
| Transsion Holdings | Proposal 2: Regarding CQI determination, support Option 1b: • Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment |
| Fujistu | Proposal 2:  For CSI compression Case 0, prioritize Option 1a and Option 1b for CQI/RI determination. o Option 1a: CQI is calculated based on target CSI with realistic channel measurement. o Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment. |
| MTK | Proposal 2. For CQI determination using two-sided AI/ML models for CSI compression: i) deprioritize sub-options 1c, 2a, and 2b; and ii) Further clarify potential adjustment in option 1b. |
| ETRI | Proposal 4: For AI/ML-based CSI compression, further discuss on the CQI determination method. |
| CMCC | Proposal 8: CQI should be reported in the AI/ML based CSI compression report. Proposal 9: Take Option 1 as the assumption of CQI determination for AI/ML based CSI compression: - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation Proposal 10: Take Option 1a as the starting point of CQI determination for AI/ML based CSI compression: - Option 1a: CQI is calculated based on target CSI with realistic channel measurement Proposal 11: RI should be reported in the AI/ML based CSI compression report, and Rank =1~8 can be supported. |
| Ofinno | Proposal #6: Regarding CQI determination, prioritize Option 1a. Also, consider adopting PMI based approach (Option 1c) as a fallback when the CQI from Option 1a is unreliable. Proposal #7: Consider a method to minimize the number of utilized AI models while maintaining accuracy-overhead performance across the full range of supported ranks with consideration of AI/ML processing unit occupation. |
| Qualcomm | Proposal 12: Precoder assumptions for CQI estimation is upto UE implementation, • RAN4 may define a relaxed testing requirement to accommodate various potential precoder assumptions. • Further discuss whether RAN1 may clarify that UE reports a CQI estimate. |
| DCM | Proposal 4 - RI and CQI are determined based on the same option. - Support Option 1a as the baseline for both RI and CQI determination. - Not support Option 2a due to the UE complexity and inaccuracy of UE reconstructed CSI, which cannot be compensated at the NW side because the NW does not know the UE implementation. - Not support Option 2b due to the additional round-trip latency for the CQI reporting. |
| CEWiT | Proposal-5: Consider Option-1 based methods for CQI determination as a starting point. |

## Discussion

CQI assumptions identified in R18 includes the following.

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

The summary of companies’ proposals is presented as the table below.

|  | Support | deprioritize |
| --- | --- | --- |
| Option 1a (16 vs. 0) Encoder input | Ericsson, Spreadtrum, Huawei, CATT, Xiaomi, ZTE, Panasonic (baseline), OPPO, Nokia (adjustment in spec transparent manner), NEC, Fujistu, DCM, HONOR, CMCC, Ofinno, CEWiT |  |
| Option 1b (11 vs. 0) Encoder input + adjustment | Spreadtrum, Huawei, Panasonic (baseline), LGE, Apple, NEC, Transsion Holdings, Fujistu, MTK (further clarification), IDC, CEWiT |  |
| Option 1c (3 vs. 4) eT2 | Ofinno, OPPO, Sharp (further discuss precoding information) | Panasonic, NEC, MTK, QC |
| Option 2a (4 vs. 3) Proxy decoder, ref decoder | Vivo, Lenovo, NEC, Futurewei, Panasonic (different than actual decoder), Sharp | MTK, QC, DCM |
| Option 2b (2 vs. 4) Precoded RS | Google, IDC | Panasonic, NEC, MTK, DCM |
| Other | Samsung: Defer the discussion after PMI part is stable. Qualcomm: CQI assumption upto implementation, RAN1 clarify UE report a CQI estimate |  |

FL note: From companies’ proposals, compared to the last meeting, the situation remains same with slightly more supporters for option 1a and 1b, and no objection is observed.

Collecting companies’ comments mentioned in the last meeting, following are FL’s response and observations

- Boundary between option 1a and 1b are blur, quite a few companies mentioned that the adjustment can be done by NW, or even by the UE, how is can be detected or tested as it is inside the UE and transparent to the NW. Proponents of option 1b are encouraged to clarify this aspect.

- Regarding option 1c, @OPPO, it seems the benefit is not well acknowledged by the majority and there is strong objection from chipset vendors considering the additional complexity. @ofinno, target CSI calculation is inside UE, the intention of representing in eType II format is to clarify that it is uncompressed and unquantized and facilitate CBSR.

- Regarding 2a, @Lenovo, @Futurewei, even if a decoder is employed by the UE, it would not be the same as the NW’s actual decoder. Thus, the CQI resulted by the reconstructed CSI is an estimate of the actual reconstructed CSI / CQI. With this spirit, I believe it can be absorbed in to option 1a/1b category.

- Regarding 2b, it seems already supported by existing spec.

Following the discussion of option 1a/1b boundary, FL think they both provide an estimate of the CQI, the only matter is how loose or tight it is to the actual CQI. In light of this, some clarification in the spec may be needed to distinguish it from the actual CQI which was in the legacy defined according to the reconstructed precoding matrix.

Given the proposals and situation, let us try to converge to option 1a and 1b a bit by focusing on the difference between them and what are the spec impacts. Some minor updates are made to the observations to improve the clarity.

##### Observation 5.1:

Take following observations:

- All options except 2b provide an CQI estimate rather than the legacy CQI definition which reflects the actual decoder output (i.e., the actual reconstructed CSI).

- Option 2b can be supported with existing specification.

- Option 1c and 2a may be infeasible considering the complexity of calculating eT2 PMI or running a proxy decoder in addition to the CSI encoder.

##### Proposal 5.2 (low priority):

Continue study CQI assumptions focusing on option 1a and 1b.

- Note: The study may involve the fundamental difference between them, and what are the necessary spec impacts for each of them

| Companies | Comments |
| --- | --- |
| Mod | Intention is to capture the observations and proposal together in the chairman note. The observation provides an informative note for the current study outcome and as a guidance for way forward. Let us move forward with a bit more on the thoughts of their spec impacts. |
| vivo | option 1a differs from the legacy CQI definition, which is calculated based on the reported PMI. Using the uncompressed PMI for option 1a's CQI might lead to overestimated CQI values, potentially resulting in higher error rates and the use of higher-order MC |
| NTT DOCOMO | We generally think Option 1a is the best we can do. Of course, the compensation on the CQI is necessary since the Option 1a CQI is optimistic. Because NW knows the implementation and performance of the CSI reconstruction models, the compensation should be conducted by NW implementation instead of UE. |
| Samsung | Ok. |
| Huawei, HiSilicon | No need to specify how to calculate CQI. Even if there overestimation, NW can perform OLLA to correct it. |
| ZTE | We prefer option 1a. For Option 1b, the method for performing CQI adjustment remains unclear and requires further investigation to establish its feasibility. |
| MediaTek | The boundary between 1a and 1b is not clear as the adjustment mechanism already can be applied by NW even option 1a is used. Therefore, option 1a seems more straightforward to agree on without introducing additional ambiguity about where and how adjustment is applied |
| CATT | OK in general. Our preference is Opt 1a. |
| Ofinno | Fine in principle but need more consideration when the performance of Option 1a is degraded. When it happens, Should AI model be updated or fallback at least for CQI determination with using the legacy codebook? |
| InterDigital | Support |
| LG Electronics | Option 1c (34 vs. 4) eT2; Ofinno, OPPO, Sharp (further discuss precoding information), LGE; Panasonic, NEC, MTK, QC. First of all, our preference has not been captured properly. We support not only Option 1b, but also Option 1c. So, the table above would be corrected as below. Although both Option 1c and Option 2a may have additional UE complexity on top of running encoder model alone, Option 1c is much more preferable because Option 1c, basically, does not incur the model mismatch between UE and NW. However, for Option 2a, alignment between proxy/reference decoder at UE and NW-side actual decoder should be guaranteed. Thus, Option 1c is more practical and more feasible than Option 2a. Moreover, Option 1c provides simplicity and backward compatibility to legacy framework. As per Option 1b, we have to further clarify the term “potential adjustment”, which some companies including us interpret as the NW indication of CQI adjustment (e.g., CQI offset). To our understanding, Option 1b is not just UE implementation and fundamentally different than other options. Thus, Option 1b may be unique solution which is able to reflect NW-side decoder’s output. |
| TCL | Agree |
| Xiaomi | We are fine and prefer Option 1a. |
| SPRD | OK with focusing on option 1a and 1b. We prefer option 1b. |
| Panasonic | The proposal to move forward with Option 1a and 1b is acceptable. We think misalignment of the CQI between network and UE may not be a big issue since network may always make some adjustment on UE reported CQI, for example based on outer-loop link adaptation using HARQ-ACK. |
| HONOR | Option 1a is prefered. For Option 1b, for the potential adjustment, there are two potential scenarios: 1) the adjustment is done by UE without any external information, and 2) the adjustment is done by UE with information indicated by NW, e.g., CQI compensation information. For situation 1), it is up to UE’s implementation and there is extra spec impact. For situation 2), since the compensation information is provided by the NW, the CQI adjustment can also be directly completed by the NW, and it seems unnecessary for the UE-side to adjust. |
| CMCC | Support |
| Transsion | We are fine and prefer Option 1b. |

# Processing timeline, criteria, active resource and capability

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| Spreadtrum | Proposal 6: For CSI compression, for inference, for PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,  OAPU= 0 or N is reported by UE  OCPU=0 or M is reported by UE  Note: Detailed values of N and M can be further discussed in UE feature.  Note: Combination of OAPU= 0 and OCPU=0 is not allowed Proposal 7: For occupancy duration of CPU and APU for CSI compression. • Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report • Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report Proposal 8: For the determination of CSI report priority value of a CSI report for inference, the existing Pri_iCSI (y,k,c,s) is reused  k = 1 for the CSI report for inference. |
| Huawei, HiSilcon | Proposal 11: Regarding the occupancy of PU for AI/ML-based CSI compression, for inference, OAPU and/or OCPU are occupied with UE capability report.  Combination of OAPU = 0 and OCPU =0 is not allowed.  For the OAPU > 0 and OCPU > 0 case, if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to be updated, and neither of the PUs are occupied.  CPU/APU occupation starts from the latest RS occasion (for SP-CSI report) / DCI (for A-CSI report) till PUSCH carrying the CSI report. Proposal 12: Regarding the CSI processing timeline for AI/ML-based CSI compression, for inference, extend legacy Z_2/ Z_2^' to Z_2+d/ Z_2^'+d, where additional delay d is reported by UE per SCS. Proposal 13: Regarding the CSI priority rules, reuse existing CSI priority 〖Pri〗_iCSI (y,k,c,s) by considering k=1 for inference CSI report. |
| InterDigital | Proposal 5: Support CPU pool sharing between legacy CSI reporting and AI/ML-based CSI. Proposal 6: Support PU pool sharing by different AI/ML functionalities (e.g., AI/ML based CSI reporting, AI/ML based BM, AI/ML based POS). Proposal 7: Support AI/ML Processing Units (PU) occupancy definition per AI/ML functionality (e.g., AI/ML based CSI reporting, AI/ML based BM, AI/ML based POS). Proposal 8: Support accounting of the AI/ML model readiness status of active models for determination of, the inference timeline. |
| Google | Proposal 5: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report. Proposal 6: For CPU/APU rule, support to reuse the same mechanism of AI/ML based CSI prediction for AI/ML based CSI compression • UE reports the number of CPUs, number of APUs and the APU pool for AI/ML based CSI compression Proposal 7: For minimum processing delay, support to reuse the same mechanism of AI/ML based CSI prediction for AI/ML based CSI compression • UE reports the additional processing delay for AI/ML based CSI compression on top of the processing delay for Rel-19 eType2 CSI report |
| CATT | Proposal 18: For inference report of AI/ML-based CSI compression, support the following for PU occupancy  OAPU= 0 or N is reported by UE  OCPU=0 or M is reported by UE  The values of N and M are reported by UE  Combination of OAPU= 0 and OCPU=0 is not allowed Proposal 19: For inference report of AI/ML-based CSI compression, the legacy mechanism of occupancy duration for R19 eType II codebook can be reused, if associated monitoring report is not configured. Proposal 20: For CSI compression, for inference, in addition to legacy Z/Z’ for R19 eTypeII codebook, UE report the value of t per SCS  The value of t is reported by UE Proposal 21: For inference report of AI/ML-based CSI compression, the priority of the inference report is the same as CSI report not carrying L1-RSRP or L1-SINR (analogous to Rel-19 eType II codebook). |
| Vivo | Proposal 10: For CSI compression using UE-side model, the similar rule that agreed in CSI predication can be reused, e.g., • For PU occupancy, support Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, • the same occupancy duration is used if both CPU and APU are reported non-zero value Proposal 11: For CSI compression using UE-side model, for inference, in addition to legacy Z/Z’ for type II codebook, UE may report the value of t per SCS • Detailed value of t can be discussed in UE feature Proposal 12: For the determination of CSI report priority value of a CSI report for inference, the existing Pri_iCSI (y,k,c,s) is reused • k = 1 for the CSI report for inference Proposal 13: For the determination of CSI report priority value of a CSI report for monitoring, the existing Pri_iCSI (y,k,c,s) is reused • k = 1 for the CSI report for monitoring |
| Xiaomi | Proposal 14: Reuse legacy CSI reporting priority rule definition, and k=1 for two-sided AI model based CSI feedback. Proposal 15: For two-sided AI model based CSI compression, reuse the CSI processing criteria for AI based CSI prediction, and extend legacy Z/Z’ to Z/Z’+t’ for CSI generation part model inference, where value of t’ is reported by UE per SCS. |
| TCL | Proposal 4: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules. Proposal 5: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting. Proposal 5: Consider to reuse the definition of AI/ML PU occupancy in CSI prediction. • FFS if the value of OAPU is the same or not. • FFS if the value of occupancy duration of OAPU is the same or not. Proposal 4: Consider if different values of OCPU should be defined for different target CSI. Proposal 6: Study the selection rule if the AI-related CSI report to be reported is jointly determined by the legacy CPU and AI/ML PU. • Option 1: Selection of CSI report prioritizes the AI/ML PU constraint. • Option 2: Selection of CSI report prioritizes the legacy CPU constraint. • Option 3: Selection of CSI report jointly considers the legacy CPU and AI/ML PU constraint. Proposal 7: The occupation time of CPU and A/ML PU should be considered separately. The duration of the PU occupation time could be configured by the NW. |
| ZTE | Proposal 16: For CSI feedback using two-sided model, dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied for an inference report, where OAPU= 0 or N is reported by UE in UE capability report, and OCPU= 0 or M is reported by UE in UE capability report. Proposal 17: For CSI feedback using two-sided model, for an inference report, Rel-15 CPU occupation time is reused for APU occupation time and/or CPU occupation time. Proposal 18: For CSI feedback using two-sided model, for an inference report, extend legacy Z2 / Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS in UE capability report. Proposal 19: For the determination of CSI report priority, the existing Pri_iCSI (y,k,c,s) is reused, where k = 1 for CSI reports for inference and monitoring. |
| Samsung | Proposal#10: Defer the discussion on CSI processing criteria and priority rule for two-sided model-based CSI compression until some progress on PMI feedback is achieved. |
| Lenovo | Proposal 14: For AI based CSI compression report, for PU occupancy, dedicated AI/ML PU and/or legacy CPU are occupied, and the UE separately report the number of AI/ML PUs and the number of CPUs for a CSI compression report. Proposal 15: For AI based CSI compression report, based on legacy (Z_2,〖Z'〗_2 ), the UE indicates where additional CSI computation is needed. Proposal 16: Specify a few classes for model complexity. The encoder of the AI-based CSI-feedback is then assigned to one of the classes of the model complexity. Proposal 17: Specify a few classes for AI HW capability. The UE reports the HW capability group to the NW. Knowing the active AI model, the gNB is able to determine the required APU. Proposal 18: k=1 is adopted to determine the CSI report priority value for AI based compressed CSI report. |
| Panasonic | Proposal 11: For CSI compression using two-sided model, to calculate the inference report, UE reports the number of dedicated AI/ML PUs and /or legacy CPUs by reusing the mechanism of CSI prediction as the baseline. Proposal 12: For CSI compression using two-sided model, for inference, in addition to legacy processing timeline, UE may report the offset value per SCS. |
| OPPO | Proposal 12: Regarding the aspects of CPU/APU design and occupation time, active resource/port counting, timeline, reuse the related specification outcome for inference in BM and CSI prediction agenda. Proposal 13: Support to distinguish the priority of AI/ML and legacy CSI report for inference, and the priority of CSI report for inference and monitoring. |
| HONOR | Proposal 6: For CPU/APU occupancy, further study whether CPU/APU occupancy is specific for different layers/ranks. |
| LGE | Proposal #10: For CSI compression, study on PU counting rule considering • Different types of CSI reports, e.g., inference or monitoring • Different types of AI/ML models, e.g., encoder, proxy/nominal decoder, intermediate KPI estimator Proposal #11: Study on methods to manage the NW-side computing capability for two-sided model, e.g., NW indication of inference skipping. |
| Apple | Proposal 9: For AI based CSI compression CPU/APU, use R19 CPU/APU framework as a starting point. |
| NEC | Proposal 13: In CSI spatial/frequency compression, to calculate the inference report for compressed CSI, - For PU occupancy, support:  Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,  OAPU= 0 or N is reported by UE  OCPU=0 or M is reported by UE  Note: Detailed values of N and M can be further discussed in UE feature.  Note: Combination of OAPU= 0 and OCPU=0 is not allowed  Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU,  the CPU occupancy time for CSI report with reportQuantity set to 'cri-RI-PMI-CQI' can be reused  the same occupancy duration is used if both CPU and APU are reported non-zero value Proposal 14: In CSI spatial/frequency compression, for a CSI report for inference, extend legacy Z2/Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS for CSI spatial/frequency compression. - Detailed values of d and d’ can be further discussed in UE feature. Proposal 11: In CSI spatial/frequency compression, support to define additional new priority rule for the following collisions: - AI/ML-based CSI report and non-AI/ML-based CSI report. - Inference CSI report and monitoring CSI report. |
| Fujistu | Proposal 8:  The legacy calculation of priority values could be reused for AI/ML-based CSI feedback.  The AI/ML-based CSI report should have a higher priority over the codebook-based CSI report, which could be realized by implementation. |
| MTK | Proposal 3. As priority rules for AI/ML-related CSI reports, assign lowest priority to the data collection reports and equal priority to monitoring and inference reports. Proposal 12. Consider a dedicated AI/ML PU for AI/ML-based CSI Compression which will not be shared with other AI/ML functionalities. Proposal 7. Consider adding additional delay based on UE’s capability report into legacy timeline in order to accommodate delays incurred from loading AI/ML model onto dedicated AI processing unit and its execution. |
| CMCC | Proposal 20: For AI/ML based CSI compression, to calculate the inference report, • For PU occupancy, support o Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied • For occupancy duration of CPU and APU, o the same occupancy duration is used if both CPU and APU are reported non-zero value o if associated monitoring report is not configured, reuse following legacy occupancy duration - For periodic and semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report), occupation starts from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, respective latest CSI-RS/CSI-IM occasion no later than CSI reference resource, until the last symbol of the PUSCH carrying the report - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. Proposal 21: For AI/ML based CSI compression, extend legacy Z/Z’ to Z+d / Z’+d’, where d is reported by UE per SCS • Detailed values of d can be further discussed in UE feature Proposal 22: For AI/ML based CSI compression, for the determination of CSI report priority value of a CSI report for inference, the existing 〖Pri〗_iCSI (y,k,c,s) is reused k = 1 for the CSI report for inference |
| ETRI | Proposal 9: For AI/ML-based CSI compression, adopt the priority rules for the legacy type2-r16 CSI reports. |
| Qualcomm | Proposal 13: Adopt existing framework of CPU and APU counting for CSF via two-sided model, UE may report dedicated CPU and APU cost for this feature. Proposal 14: Adopt existing timeline formulation for CSF via two-sided model, UE may report dedicated additional time requirement t for this use case. |
| DCM | Proposal 10 - Rel-16 eType2 CSI is the baseline of the CSI processing criteria and timeline, with the following enhancements  UE reports APU and CPU occupation numbers, reusing the mechanism of CSI prediction as the baseline.  Relaxation of the processing timeline if necessary. |

## Discussion

FL note: From the proposals, almost all companies propose to reuse the legacy CSI processing unit rules and legacy AI processing unit rules. Two companies propose to adopt dedicated processing units pool for CSI compression. In RAN-plenary #108, it has been agreed to adopt two processing unit pools and it’s upto UE to select which pool to use for each feature. FL think this proposal can be handled within the scope of this proposal, i.e., UE selecting a dedicated pool for CSI compression without sharing with other features.

Similarly, most companies propose that legacy timeline framework can be adopted by allowing UE to report additional processing time for this feature. There is one proposal from InterDigital considering model loading time for inactive models. In Rel-19 study, model loading time has been studied, and the assumption is that it can be absorbed into the specified / reported timeline.

For priority rule, lots of company propose to adopt k=1 for AIML CSI compression, just like other non-AI CSI feedback. Some other companies prefer to considering different priority for AIML and non-AI CSI feedback, while some others think this may be handled by NW implementation by assigning different report IDs.

All above aspects can be discussed in a later stage of the release after the PMI type, CQI, payload, UCI mapping become stable and clear.

FL observation 6.1 (not for online / offline session)

- Majority companies propose to adopt legacy CSI processing unit rule and legacy AIML processing unit rule for CSI compression via two-sided model. Specific CPU and APU occupation values can be reported by UE capability

- Majority companies propose to adopt Rel-19 timeline formulation for CSI compression via two-sided model, allowing UE to report additional time requirement on top of existing {Z, Z’} values.

- Majority companies propose to adopt same priority rule to address CSI collision by setting k=1 for CSI compression via two-sided model.

| Companies | Comments |
| --- | --- |

# Other topics proposed by companies to 10.1.1.1

## Summary of companies’ proposals

| Companies | Proposals |
| --- | --- |
| TCL | Proposal 8: Ran1 should consider whether/how to transmit the raw CSI for monitoring purpose. • FFS: The overhead reduction scheme based on spectral or temporal processing. Proposal 9: Study the triggering mechanism based on UCI loss event for CSI information reset or retransmission. • Option 1: UCI loss event is determined by NW • Option 2: UCI loss event is determined by UE Proposal 10: Investigate the CSI information synchronization for mitigating the impact of UCI loss. • Option 1: Reset the CSI information to pre-defined values. • Option 2: NW configs or UE initials CSI information retransmission. |
| InterDigital | Proposal 2: Support SRS configurations linked to CSI-RS or CSI feedback resources through explicit offsets, dynamic triggering rules. |
| Lenovo | Proposal 1: Investigate how the UE and gNB can coordinate on the active encoder/decoder model in cases that the selection of the active two-sided model depends on the measurement of the channel. Proposal 13: Support reporting of indication of the applied pairing-ID, , used for encoding of the model at the UE. |
| Google | Proposal 2: Support hybrid AI/ML based and non-AI/ML based CSI measurement and report • UE reports the CSI based on AI/ML if it reports a small RI and the UE can report the CSI based on Type1 codebook if it reports a large RI |
| MTK | Proposal 1. For pairing and applicability report of two-sided AI/ML models using AI/ML model ID, the following procedure is proposed as the starting point of discussion: • Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. • Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. • Step 3: NW sends a sets of AI/ML model IDs available at NW-side and ready to be paired and may provide initial configuration of each inquired AI/ML model ID. • Step 4: UE reports applicable AI/ML model IDs corresponding to the AI/ML model IDs inquired by NW during step 3. • Step 5: NW configures a set of applicable AI/ML models based on the report received in step 4 from UE and may provide updated configurations compared to step 3. |

## Discussion

There seems to be no essential issues in the section. Please provide your views on any other essential issues that have not yet been captured in the discussion.

| Companies | Comments |
| --- | --- |

# Proposals for online sessions

## Monday

## Tuesday

## Wednesday

##### Proposal 3.3c (high priority)

Down-selection to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption

- Candidate values for L: L=2, d

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

##### Proposal 3.6b (high priority, merging 3.5 and 3.6)

For payload parameter combinations (PC), consider followings

- Support N’ <= N multiple PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent per-layer PCs chosen from the per-layer PC pool.

|  | Layer 0 | Layer 1 | Layer 2 | Layer 3 |
| --- | --- | --- | --- | --- |
| Rank 1 | X |  |  |  |
| Rank 2 | X | X |  |  |
| Rank 3 | X30 | X31 | X32 |  |
| Rank 4 | X40 | X41 | X42 | X43 |

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {Xn_31, Xn_32, Xn_33} = {a, a, a}, {a, a, b}, {a, b, b} for potential downslection.

- For rank 4, consider alternatives of {Xn_41, Xn_42, Xn_43, Xn_44} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c}.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

## Thursday

## Friday

##### Proposal 3.7 (new, high-priority)

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , average SGCS across layers can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

##### Proposal 4.2 (low priority):

For R20 CSI feedback via two-sided AIML models, reuse legacy CSI reporting framework

- Support CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations.

- Support aperiodic report and semi-persistent CSI report on PUSCH and reuse legacy triggering mechanism.

##### Proposal 5.2 (low priority):

Continue study CQI assumptions focusing on option 1a and 1b.

- Note: The study may involve the fundamental difference between them, and what are the necessary spec impacts for each of them

# FL closing remark

In this meeting, good progress was made regarding the following aspects

- Precoding matrix feedback with port-subband domain representation is agreed, as opposed to beam-delay domain as transformed by SD and FD bases (agreement 2.1a).

- General principles for determining the payload combinations / per-layer parameter pairs / triplets (agreement 3.4a).

- Parameter combination is formulated in terms of per-layer payload parameter pairs (for SQ) / triplets (for VQ). Candidate values for the per-layer payload parameter pairs / triplets are also identified (agreement 3.3c and 3.6b).

For the next meeting, high priority topics include

- Down-selection of SQ and VQ, or converging to a general framework that could accommodate both. In light of this, whether the codebook is exchanged or standardized.

  - Let us continue the discussion along the proposal 3.1 track, considering the identified candidate values of {d, Q} or {d, L, Q} in agreement 3.3c.

- Determining the parameter combinations.

  - Step 1: Down-select to N parameter pairs / triplets based on the evaluation of the candidate values (e.g., per-layer SGCS or spectral efficiency).

  - Step 2: Determine Y parameter combinations by choosing parameter pairs / triplets across layers / ranks) from the N parameter pairs / triplets, based on the spectral efficiency.

- Continue study CQI assumptions with focusing one or two options, e.g., option 1a and 1b.

  - Study includes spec impacts and performance benefit over benchmark of eT2 and option 2b (which is already supported by current spec)

# Previous agreements

## RAN1 #122

Agreement:

For CSI feedback via two-sided model, support at least precoding matrix as target CSI type.

- FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

Agreement

Specify the precoding matrix feedback via two-sided model as follows: For a certain layer l and rank ν,

- A precoding matrix is mapped with a latent message zl=z0l, z1l,⋯,zdzl,ν-1l, with l = 1, ..., ν where l is the layer index.

  - The mapping is subject to [at least] a pairing ID configured by high-layer signalling.

  - FFS dzl,νis common or specific for different layers or ranks

  - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further

- zl is quantized and mapped to bit sequence before reporting.

Agreement

For precoding matrix feedback, for a certain layer l of rank ν, the latent message zl contains dzl,ν real values

- If scalar quantization (SQ) supported, it refers to the quantization of each real value independently,

  - The payload size for layer l and rank ν is determined by dzl,ν×Ql,ν, where Ql,ν is the number of bits per scalar.

- If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized.

  - The payload size for layer l and rank ν is determined by dzl,νL×Ql,ν, where L is the length of the vector segment,  Ql,ν is the number of bits per segment

  - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded.

Agreement:

For precoding matrix feedback, consider the following aspects for payload determination

- Support only SQ, or only VQ or both

- dzl,ν and / or Ql,ν and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting.

- dzl,νand / or Ql,ν is layer-common or layer-specific, rank-common or rank-specific.

- If configured, dzl,νand / or Ql,ν and / or L above are configured separately from pairing ID, or configured by Pairing ID.

  - The configuration may be via other parameters which are used to derive dzl,ν, L and Ql,ν.

Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges.

Note: dzl,ν, Ql,ν and L are independent on subband and port configurations,

- FFS each combination of dzl,ν, Ql,ν and L is applicable to a certain set of subband and port configurations

Agreement:

For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.

Agreement:

For quantization codebook used for quantizing the feedback,

- For inter-vendor collaboration Direction A, select one of the following:

  - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters

  - Alt2: Use a standardized quantization codebook decided by RAN1.

## RAN1 #122bis

Agreement:

Support precoding matrix with port-subband domain representation as target CSI type.

- For a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- Note: Only sub-band PMI reporting is supported.

Agreement:

For determining the payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and ranks, consider following general principles for precoding matrix-based target CSI

- Rank-common and layer-common model with payload-scalable design is considered as the baseline implementation for discussion purpose.

  - Note: layer-specific / rank-specific (if supported) payload parameters are supported by this rank-common and layer-common model with payload-scalable design.

- Limiting the total number of (per-layer) payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and all ranks.

- At least for medium to high per-layer payload regime, the total payload size of rank 2, rank 3 and 4 are comparable to each other.

- Spectral efficiency or system throughput and trade-off with feedback overhead are considered as the performance metric.

Agreement:

When the Target CSI is precoder matrix, in order to down-select to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν based on performance, overhead and complexity trade-off results, consider the following candidate values:

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption for SQ.

- Candidate values for L: L=2, 4,  dl,ν

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

- Note: The above is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: Rank-common and layer-common model with payload scalable design is considered.

- Note: The above does not mean support of both SQ and VQ.

- Note: The number of parameters combinations configurations (Y) does not exceed 8.

Agreement:

For payload parameter combinations (PC), consider followings

- Support Y configurable PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent a pair of dl,ν,Ql,ν or a triplet of dl,ν, L,Ql,ν  chosen from the per-layer PC pool.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {Xn_31, Xn_32, Xn_33} = {a, a, a}, {a, a, b}, {a, b, b} for potential down selection.

- For rank 4, consider alternatives of {Xn_41, Xn_42, Xn_43, Xn_44} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c} for potential down selection.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

Agreement:

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , average SGCS across per layer can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

Agreement:

For CSI feedback via two-sided AIML models, reuse legacy CSI reporting framework

- Support CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations.

- Support aperiodic report and semi-persistent CSI report on PUSCH and reuse legacy triggering mechanism.

# References

- TR 38.843 v19.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” (Release 19), June 2025

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm (Moderator), 3GPP TSG RAN #108, June 2025