3GPP TSG RAN WG1 #122bis	                                   					       												                          R1-25nnnnn

Prague, Czech Republic, October 13th –17th, 2025

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.13

Agenda Item:	9.13

Document for: 	Endorsement

### 9.13 UE features for LTE based 5G broadcast Phase 2

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 3. LTE_terr_bcast_Ph2 | 3-1a | Cyclic shift of PMCH – fixed alpha1 | 1. Support of cyclic shift for the bit sequence in Section 6.3.1 of TS 36.211 for the i^th subframe of the time-interleaved TB by X_i bits | 3-1 | Yes | N/A | UE is not able to support time-interleaving with the cyclic shift and alpha1 | Per band | No | N/A | ∑#CBs-1r=#CBs-SiEr, with Si=(i×α) mod #CBs - Er is the number of bits in the rth codeblock within a subframe (as defined in TS 36.212) α=1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 3. LTE_terr_bcast_Ph2 | 3-1b | Cyclic shift of PMCH – alpha2 | 1. Support of cyclic shift for the bit sequence in Section 6.3.1 of TS 36.211 for the i^th subframe of the time-interleaved TB by X_i bits | 3-1 | Yes | N/A | UE is not able to support time-interleaving with the cyclic shift and alpha2 | Per band | No | N/A | ∑#CBs-1r=#CBs-SiEr, with Si=(I ×α) mod #CBs - Nsymb denotes the number of OFDM symbols within a subframe - #CBs denotes the number of CBs in the time-interleaved (scaled) TB - Er is the number of bits in the rth codeblock within a subframe (as defined in TS 36.212) α= #CBsN×Nsymb | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 3. LTE_terr_bcast_Ph2 | 3-1c | Cyclic shift of PMCH - alpha3 | 1. Support of cyclic shift for the bit sequence in Section 6.3.1 of TS 36.211 for the i^th subframe of the time-interleaved TB by X_i bits | 3-1 | Yes | N/A | UE is not able to support time-interleaving with the cyclic shift and alpha3 | Per band | No | N/A | Xi=Ainf,nsfNscQmN Ainf,nsf=m=07c810nf mod 128+nsf+m2m mod N where - the pseudo-random sequence c(i) is defined by clause 7.2 and shall be initialized with cinit=NIDMBSFN at the beginning of each radio frame for which nf mod 128=0, - Nsc=MbitQmL is the number of subcarriers available in one OFDM symbol for PMCH, - Qm is the modulation order, - i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped, - N is the number of subframes to which the transport block is mapped | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 3. LTE_terr_bcast_Ph2 | 3-1 | Time-interleaving | 1. Support of PMCH transmission pattern, excluding MCCH and MSI, with time interleaving for a set of PMCH numerologies 2. Support of TBS determination for the scaled TB up to a maximum TBS 3. Support of determining the starting point for reading from the circular buffer (k0) for each subframe 4. Support of the extended MSI periodicities | Support of fembmsDedicatedCell | Yes | N/A | UE is not able to support time-interleaving for LTE-based 5G broadcast | Per band | No | N/A | For component 1, the UE indicates a bitmap [b15, b7dot5, b2dot5, b1dot25] where each bit indicates whether the UE supports time-interleaving for the corresponding numerology For component 2, the maximum TBS a UE supports for the scaled TB is derived based on the UE’s unicast capability Note: One TB is mapped to N non-consecutive subframes. Two transmissions of the same TB are separated by (M-1) subframes. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2506915	Discussion on UE features for LTE based 5G broadcast	ZTE Corporation, Sanechips

R1-2506977	UE features for LTE based 5G broadcast	Xiaomi

R1-2507241	UE features for LTE broadcast	Samsung

R1-2507709	UE features for LTE based 5G broadcast Phase 2	Qualcomm Incorporated

R1-2507742	Summary of UE features for LTE based 5G broadcast Phase 2	Moderator (AT&T)