3GPP TSG RAN WG1 #122bis	                                   					       												                          R1-25nnnnn

Prague, Czech Republic, October 13th –17th, 2025

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.2

Agenda Item:	9.2

Document for: 	Endorsement

### 9.2 UE features for NR MIMO Phase 5

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-1 | Enhanced Type-I SP codebook for 64 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources within one slot 2. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 2-35 | yes | n/a | Enhanced Type-I SP codebook is not supported for 64 ports – Scheme-A, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {2,4} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as show

| 59. NR_MIMO_Ph5 | 59-2-1-1a | Enhanced Type-I SP codebook for 48 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources within one slot 2. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-1 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:82,3} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-1b | Enhanced Type-I SP codebook for 128 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources within one slot 2. A list of supported combinations, each combination is Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Support 4 CSI-RS resources in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-1 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {4, 5, 6, 7, 8} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-1c | Enhanced Type-I SP codebook for 64 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources within one slot 2. A list of supported combinations, each combination is Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 2-35 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {2,4} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-1d | Enhanced Type-I SP codebook for 48 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources within one slot 2. A list of supported combinations, each combination is Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-1c | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {2,3} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-1e | Enhanced Type-I SP codebook for 128 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources within one slot 2. A list of supported combinations, each combination is Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Support 4 CSI-RS resources in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-1c | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {4, 5, 6, 7, 8} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-2 | Enhanced Type-I MP codebook for 64 ports | 1. Support of enhanced Type-I MP codebook for 64 ports within 1 slot 2. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum number of panels 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 2-35 | yes | n/a | Enhanced Type-I MP codebook is not supported for 64 ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {2, 4} Component 4 candidate value {2,4} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-2a | Enhanced Type-I MP codebook for 48 ports | 1. Support of enhanced Type-I MP codebook for 48 ports within 1 slot 2. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum number of panels 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-2 | yes | n/a | Enhanced Type-I MP codebook is not supported for 48 ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {2, 4} Component 4 candidate value {2,3} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-2b | Enhanced Type-I MP codebook for 128 ports | 1. Support of enhanced Type-I MP codebook for 128 ports within 1 slot 2. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum number of panels 4. Support 4 CSI-RS resources in a resource set 5. Supported processing capability 6. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-2 | yes | n/a | Enhanced Type-I MP codebook is not supported for 128 ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 2 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 3 candidate value {2, 4} Component 5 candidate value {Capability 1, Capability 2} Component 6 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU =1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-3 | Extended Rel-16 eType-II codebook for 64 Tx ports | 1. Support of extended Rel-16 eType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources within 1 slot 2. Support of parameter combination 1-6 3. Support of rank 1-2 4. Support R=1 5. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 6. supported processing capability 7. Max # of CSI-RS resource in a resource set 8. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 16-3a | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 64 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 5 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 6 candidate value {Capability 1, Capability 2} Component 7 candidate value {2,4} Component 8 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-3a | Extended Rel-16 eType-II codebook for 48 Tx ports | 1. Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources within 1 slot 2. Support of parameter combination 1-6 3. Support of rank 1-2 4. Support R=1 5. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 6. supported processing capability 7. Max # of CSI-RS resource in a resource set 8. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-3 | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 48 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 5 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 6 candidate value {Capability 1, Capability 2} Component 7 candidate value {2,3} Component 8 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-3b | Extended Rel-16 eType-II codebook for 128 Tx ports | 1. Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources within 1 slot 2. Support of parameter combination 1-6 3. Support of rank 1-2 4. Support R=1 5. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 6. supported processing capability 7. Support 4 CSI-RS resources in a resource set 8. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-3 | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 128 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 5 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 6 candidate value {Capability 1, Capability 2} Component 8 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-3-1 | PMI sub-bands with R=2 for extended Rel-16 eType-II codebook for up to 128 ports | 1. Support of PMI sub-bands with R=2 for extended Rel-16 eType-II codebook for up to 128 ports 2. A list of supported combinations, each combination is {Max # of Tx ports in a report, Max # of sets of aggregated resources, and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 | 59-2-1-3 | Yes | n/a | PMI sub-bands with R=2 for extended Rel-16 eType-II codebook for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64, 128, 256} c. {64, …, 256, 512, 768, 1024} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-3-3 | Rank 3,4 for extended Rel-16 eType-II codebook for up to 128 ports | 1. Support of Rank 3,4 for extended Rel-16 eType-II codebook for up to 128 ports 4. Support R=1 5. A list of supported combinations, each combination is {Max # of Tx ports in a report, Max # of sets of aggregated resources, and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 | 59-2-1-3 | Yes | n/a | Rank 3,4 for extended Rel-16 eType-II codebook for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64, 128} b. {1, 2, …, 64} c. {64, …, 256, 512, 768, 1024} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4 | Extended Rel-17 FeType-II codebook with 64 Tx ports | 1. Support of extended Rel-17 FeType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources within 1 slot 2. Support of parameter combinations with M=1 3. Support of rank 1-2 4. Support R=1 5. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 6. Supported processing capability 7. Max # of CSI-RS resource in a resource set 8. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 23-9-1 | yes | n/a | Extended Rel-17 FeType-II codebook is not supported with 64 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 5 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 6 candidate value {Capability 1, Capability 2} Component 7 candidate value {2,4} Component 8 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4a | Extended Rel-17 FeType-II codebook with 48 Tx ports | 1. Support of extended Rel-17 FeType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources within 1 slot 2. Support of parameter combinations with M=1 3. Support of rank 1-2 4. Support R=1 5. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 6. Supported processing capability 7. Max # of CSI-RS resource in a resource set 8. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-4 | yes | n/a | Extended Rel-17 FeType-II codebook is not supported with 48 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 5 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 6 candidate value {Capability 1, Capability 2} Component 7 candidate value {2,3} Component 8 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5 | Extended Rel-18 eType-II Doppler codebook for 64 Tx ports | 1. Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups within 1 slot 2. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 3. Support PMI subband R=1 4. Support parameter combinations with L=2,4 5. Support rank = 1,2 6. Support 64 ports 7. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 8. Supported processing capability 9. Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR 10. Value of Y for CPU occupation when A-CSI-RS is configured for CMR 11. Support for the size of DD-basis, N4=1 12. Scaling factor for active resource counting Kp 13. Max # of CSI-RS resource in a resource group for aperiodic CSI-RS resource set or in a resource set for periodic CSI-RS resource set 14. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 40-3-2-1 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 7 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 8 candidate value {Capability 1, Capability 2} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 3} Component 12 candidate values: {1, 2, 4} Component 13 candidate value {2,4} Component 14 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YxN4xceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Yx KDOPPxceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YxN4, when P/SP-CSI-RS is configured for CMR OCPU = Yx KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5a | Extended Rel-18 eType-II Doppler codebook for 48 Tx ports | 1. Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups within 1 slot 2. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 3. Support PMI subband R=1 4. Support parameter combinations with L=2,4 5. Support rank = 1,2 6. Support 64 ports 7. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 8. Supported processing capability 9. Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR 10. Value of Y for CPU occupation when A-CSI-RS is configured for CMR 11. Support for the size of DD-basis, N4=1 12. Scaling factor for active resource counting Kp 13. Max # of CSI-RS resource in a resource group for aperiodic CSI-RS resource set or in a resource set for periodic CSI-RS resource set 14. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 7 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 8 candidate value {Capability 1, Capability 2} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 3} Component 12 candidate values: {1, 2, 4} Component 13 candidate value {2,3} Component 14 candidate values a. {1, …, 64} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Legacy timeline OCPU = Y x N4 x ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y x KDOPP x ceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = Y x N4, when P/SP-CSI-RS is configured for CMR OCPU = Y x KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5b | Extended Rel-18 eType-II Doppler codebook for 128 Tx ports | 1. Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups within 1 slot 2. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 3. Support of PMI subband R=1 for extended Rel-18 eType II Doppler codebook 4. Support parameter combinations with L=2,4 5. Support for rank = 1,2 6. Support 64 ports 7. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 8. Supported processing capability 9. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 10. Value of Y for CPU occupation (OCPU = Y. KDOPP), when A-CSI-RS is configured for CMR 11. Support for the size of DD-basis, N4=1 12. Scaling factor for active resource counting Kp 13. Support 4 CSI-RS resources in a resource group for aperiodic CSI-RS resource set or in a resource set for periodic CSI-RS resource set 14. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 7 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 8 candidate value {Capability 1, Capability 2} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 3} Component 12 candidate values: {1, 2, 4} Component 14 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Legacy timeline OCPU = Y x N4 x ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y x KDOPP x ceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = Y x N4, when P/SP-CSI-RS is configured for CMR OCPU = Y x KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-2-1 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. 3. The maximum value of KS | 2-36 | yes | n/a | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {1,2,3,4} Component 2 candidate values: a. {2,4,8,12,16, 24, 32} b. {1,2,3,4 … 256} c. {64, …, 256, 512, 768, 1024} Component 3 candidate values: {2,3,4,5,6,7,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-2-2 | Hybrid BF (CRI-based) with Rel-16 eType-II codebook | 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. 3. The maximum value of KS | 16-3a | yes | n/a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {1,2} Component 2 candidate values: a. {2,4,8,12,16, 24, 32} b. {1,2,3,4 … 256} c. {64, …, 256, 512, 768, 1024} Component 3 candidate values: {2,3,4,5,6,7,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-4 | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH | 1. Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in a set of aggregated resources, Max # of sets of aggregated resource, and total # of Tx ports} simultaneously | 2-15 | Yes | n/a | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in in a set of aggregated resources is {48, 64, 128} The candidate value set of the max # of sets of aggregated resource is: {2, …, 64} The candidate value set of total # of ports is: {48, …, 256, 512, 768, 1024} Note: Component 2 is reported per BC | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4b | M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports | 1. Support M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports 2. Support of parameter combinations with M=2 3. A list of supported combinations, each combination is {Max # of Tx ports in a report, Max # of sets of aggregated resources, and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=1 | 59-2-1-4 | Yes | n/a | M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values a. {48, 64} b. {1, 2, …, 64, 128, 256} c. {64, …, 256, 512, 768, 1024} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4c | M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports | 1. Support M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64ports 2. A list of supported combinations, each combination is {Max # of Tx ports in a report, Max # of sets of aggregated resources, and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=2 | 59-2-1-4 | Yes | n/a | M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64} b. {1, 2, …, 64, 128, 256} c. {64, …, 256, 512, 768, 1024} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5c | N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports | 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in a report, Max # of sets of aggregated resources or groups of aggregated resource, and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in a report, Max # of sets of aggregated resources or groups of aggregated resource, and total # of Tx ports} for one CSI report setting 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR | 59-2-1-5 | Yes | n/a | N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {1,2,4,8} b. {48, 64,128} c. {1, 2,3,4 … 64, 128, 256} d. {64, …, 256, 512, 768, 1024} Component 3 Candidate values a. {1,2,4,8} b. {48, 64,128} c. {4,8,12} d. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5f | PMI subband R=2 for extended Rel-18 Type-II Doppler codebook for up to 128 ports | 1. Support PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports 2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in a report, Max # of sets of aggregated resources or groups of aggregated resource, and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 | 59-2-1-5 | Yes | n/a | PMI subband R=2 for extended Rel-18 Type-II Doppler codebook for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {1,2,4,8} b. {48, 64,128} c. {1, 2,3,4 … 64, 128, 256} d. {64, …, 256, 512, 768, 1024} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-9 | NES SD Type1 for Rel-19 Type-I single-panel codebook | 1. Support NES SD Type1 for Rel-19 Type-I single-panel codebook 2. Supported NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook 3 Supported number of ports for CSI report subconfig | 59-2-1-1, 1a, 1b, 1c, 1d, or 1e and 42-1,1a, 1b or 1c | Yes | n/a | NES SD Type1 for Rel-19 Type-I single-panel codebook is not supported | [Per-band and per-BC] | n/a | n/a | n/a | Component 2 candidate values: - Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) - Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources Component 3 candidate values: One or more values from {2, 4, 8, 12, 16, 24, 32, 48, 64, 128} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed conclusion: The following is RAN1’s understanding for Case 1 in Question 1 in the RAN2 LS on per band and per BC capability (R1-2506724):

- When UE indicates both per band and per BC capability, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA)

Proposed conclusion: The following is RAN1’s understanding for Case 2 in Question 1 in the RAN2 LS on per band and per BC capability (R1-2506724):

- RAN2’s understanding is correct

Proposed conclusion: The following is RAN1’s understanding for Case 3 in Question 1 in the RAN2 LS on per band and per BC capability (R1-2506724):

- RAN2’s understanding is correct

Proposed conclusion: The following is RAN1’s understanding for Question 2 in the RAN2 LS on per band and per BC capability (R1-2506724):

- For per band and per BC capabilities, it is expected that UE indicates the same granularity as the pre-requisite

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per SRS resource set (SRS set use is configured as with usage set to ‘non-codebook ’for non-codebook based 3Tx PUSCH transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol |  | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3} Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 3. Maximum number of supported SRS port per resource 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE |  | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} Component 3 candidate values: {1,2,3} Note: When configured according to Component 4, the number of ports supported by UE for transmission in an SRS resource is 3 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL 4. Supported downgrade antenna switching configurations |  | yes | n/a | 3TX 3T6R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} Component 4 candidate values: combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 2T2R, 2T4R, 2T6R, 3T3R, 3T6R} Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch or 59-3-3a to indicate SRS antenna switching downgrading capability. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-1a | PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s) | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for joint DL/UL TCI state(s) | 23-1-1, 23-1-1h | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s) is not supported | Per band | n/a | Applicable only to FR1 | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-1b | PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state(s) | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for separate DL/UL TCI state(s) | 23-10-1, 23-1-1h | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control under separate DL/UL TCI state(s) is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-2a | Path Loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) | Support of applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) | 23-1-1, 23-1-1h, 20-2-4, 20-2-4a | yes | n/a | Applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) is not supported | Per band | n/a | Applicable only to FR1 | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-2b | Path Loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s) | Support of applying path loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s) | 23-10-1, 23-1-1h, 20-2-4, 20-2-4a | yes | n/a | Applying path loss offset on PDCCH-order PRACH under separate DL/UL TCI state(s) is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | 2-52 | yes | n/a | Two separate SRS closed loop indexes is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-4d | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA CFRA for inter-cell without CORESETPoolIndex | Support of PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA CFRA for inter-cell | 59-4-4b | yes | n/a | PDCCH ordered sent by one TRP triggers RACH procedure towards a different TRP based on CRFA CFRA for inter-cell is not supported without CORESETPoolIndex | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-7b | DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes | Support DCI format 2_3 to indicate TPC for one of two separate SRS closed loop indexes. | 59-4-3, 8-6 | yes | n/a | The function of DCI 2_3 indicating TPC command for one of two separate SRS closed loop indexes is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate one or of two separate SRS closed loop index(es) under separate DL/UL TCI state mode | 59-4-8, 59-4-3, 23-10-1 | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate one or of two separate SRS closed loop index(es) under joint TCI state mode | 59-4-8, 59-4-3, 23-1-1 | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under joint TCI state mode is not supported | Per band | n/a | FR1 only | n/a |  | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-4c | RX timing difference larger than CP length for two TAs without restriction of multi-DCI based multi-TRP operation | Support of the RX timing difference between the two DL reference timings is larger than CP length for two TAs without restriction of multi-DCI based multi-TRP | 59-4-4a or 59-4-4b | yes | n/a | RX timing difference between the two DL reference timings is no larger than CP length for two TAs without restriction of multi-DCI based multi-TRP | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-4c | Support two TAs enhancement for sDCI based intra-cell Multi-TRP operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for sDCI based intra-cell Multi-TRP operation | 40-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation sDCI based intra-cell Multi-TRP operation is not supported | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-9a | SSB reception from UL-only TRP for joint DL/UL TCI state(s) | Receiving SSB from UL TRP at least to determine pathloss on PDCCH-order PRACH for joint DL/UL TCI state(s) | 23-1-1 | yes | n/a | Receiving SSB from UL TRP at least to determine pathloss on PDCCH-order PRACH for joint DL/UL TCI state(s) is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9a | SSB reception from UL-only TRP for separate DL/UL TCI state(s) | Receiving SSB from UL TRP at least to determine pathloss on PDCCH-order PRACH for separate DL/UL TCI state(s) | 23-10-1 | yes | n/a | Receiving SSB from UL TRP at least to determine pathloss on PDCCH-order PRACH for separate DL/UL TCI state(s) is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

R1-2506882	UE features for NR MIMO Phase 5	vivo

R1-2506924	UE features for NR MIMO Phase 5	Huawei, HiSilicon

R1-2507038	Discussion on UE features for NR MIMO Phase 5	ZTE Corporation, Sanechips

R1-2507073	NR MIMO Phase 5 UE features	Nokia

R1-2507127	Maintenance on UE features for NR MIMO Phase 5	CATT

R1-2507160	UE features for NR MIMO Phase 5	OPPO

R1-2507237	UE features for NR MIMO Phase 5	Samsung

R1-2507460	Views on UE features for NR MIMO Phase 5	Ofinno

R1-2507705	UE features for NR MIMO phase 5	Qualcomm Incorporated

R1-2507739	Summary of UE features for NR MIMO Phase 5	Moderator (AT&T)

R1-2507796	Discussion on MIMO UE features	NTT DOCOMO, INC.

R1-2507863	Discussion on UE features for NR MIMO Phase 5	Ericsson