3GPP TSG RAN WG1 #122bis	                                   					       												                          R1-25nnnnn

Prague, Czech Republic, October 13th –17th, 2025

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.6

Agenda Item:	9.6

Document for: 	Endorsement

### 9.6 UE features for NR mobility enhancements Phase 4

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 1a. Support of inter-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource | 45-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 1a. Support of inter-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource | 63-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {0,1,2,3,4} Note: For component 4, the UE must support a non-zero value for at least one of aperiodic and semi-persistent | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS and CSI-IM resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | FFS RAN2 FG for LTM in Rel-18 | Yes | No | Intra-frequency Periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-6a | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS and CSI-IM resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic semi-persistent CSI-RS(s) and CSI-IM resources of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | FFS 63-6 | Yes | No | Intra-frequency Semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells | 1. Support of intra-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells 1a. Support of inter-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE | 63-6 | Yes | No | Intra-frequency Periodic CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,…64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-7a | Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells | 1. Support of intra-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells 1a. Support of inter-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE | 63-6a | Yes | No | Intra-frequency Semi-persistent CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,...64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-10 | Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception | 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration | 63-6 or 63-6a or 63-7 or 63-7a or 63-11 | Yes | No | Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-11 | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for a target cell after reception of Handover Command based on periodic CSI-RS resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after Handover Command based on periodic CSI-RS(s) of a target cell 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a target cell 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a target cell 4. Maximum number of ports in one NZP CSI-RS resource 5. Max rank for CSI reporting for a target cell 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a target cell | RAN2 related FG | Yes | No | Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for a target cell after reception of Handover Command is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 4 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 5 candidate values: {1,2,3,4,5,6,7,8} Component 6 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |

R1-2506943	UE features for NR mobility enhancements phase 4	Huawei, HiSilicon

R1-2507039	Discussion on UE features for NR mobility enhancements Phase 4	ZTE Corporation, Sanechips

R1-2507076	NR mobility enhancements Phase 4 UE features	Nokia

R1-2507128	Discussion on UE features for NR mobility enhancements Phase 4	CATT

R1-2507162	Discussion on UE features for NR mobility enhancements	OPPO

R1-2507273	Remaining issues on UE features for Rel-19 LTM	Samsung

R1-2507475	UE features for NR mobility enhancements phase 4	Ericsson

R1-2507741	Summary of UE features for NR mobility enhancements Phase 4	Moderator (AT&T)

R1-2507799	Discussion on UE features for NR mobility enhancemens Phase4	NTT DOCOMO, INC.