3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.1

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session notes for AI 8.1 (Maintenance on AI/ML for NR Air Interface)

Document for:	Discussion, Decision

## Maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

Note: Maximum one contribution. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- Specification support for beam management

- Specification support for positioning accuracy enhancements

- Specification support for CSI prediction

[122bis-R19-AI/ML] Email discussion on AI/ML – Juan (Qualcomm)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2506760	Remaining Issues of Rel-19 AI/ML for NR Air Interface	Ericsson

R1-2506770	Discussion on maintenance of AI for Air Interface	ZTE Corporation, Sanechips

R1-2506871	Maintenance on AI/ML for NR Air Interface	vivo

R1-2506934	Maintenance of Rel-19 AI/ML for air interface	Huawei, HiSilicon

R1-2506961	Maintenance on AI/ML for NR Air Interface	Xiaomi

R1-2507000	Maintenance of specification support for AI/ML for NR Air Interface	CMCC

R1-2507094	Remaining issues on AI/ML for NR air interface	CATT

R1-2507155	Maintenance on Rel-19 AI/ML for NR air interface	OPPO

R1-2507227	Remaining issue on AI/ML for NR Air Interface	Samsung

R1-2507277	Remaining issues on AI/ML for air interface in Rel-19	Fujitsu

R1-2507302	Remaining Issues on AIML for NR Air Interface	NEC

R1-2507374	Discussion on AI/ML maintenance in 5G NR interface	Panasonic

R1-2507383	Maintenance on AI/ML for NR Air Interface	Nokia

R1-2507394	Maintenance on AI/ML for NR Air interface	LG Electronics

R1-2507442	Maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface	Lenovo

R1-2507452	Maintenance of AI beam management	Ofinno

R1-2507515	Maintenance for AI/ML for NR Air Interface	Google

R1-2507582	Maintenance on Rel-19 AI/ML	InterDigital, Inc.

R1-2507648	Remaining issues for R19 AI/ML for NR air interface	Apple

R1-2507694	Maintenance on AI/ML for NR air interface	Qualcomm Incorporated

R1-2507774	Maintenance on AI/ML for NR Air Interface	Sharp

R1-2507832	Remaining issues on AI/ML for air interface in Rel-19	Quectel

R1-2507865	Remaining issues on AI PHY	ASUSTeK

### Specification support for beam management

R1-2507998

Agreement:

The following TP for TS 38.214 Clause 5.2.1.4.3b is endorsed.

Reason for change: The slot of transmission occasion for monitoring is unclear for the linkage of the corresponding CSI report for inference.

Summary of change: Clarify the first slot of transmission occasion for monitoring is used for the linkage.

Consequences if not approved: gNB and UE may have different understanding on the linkage.

##### TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting <omitted texts> - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; <omitted texts>

Agreement:

The following TP for 38.214 Clause 6.2.1.3 is endorsed.

Reason for change: P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is missing in the prioritization rules between a transmission of SRS and transmission of a physical signal/channel.

Summary of change: Add P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP in the prioritization rules.

Consequences if not approved: the prioritization rules between a transmission of SRS and transmission of a physical signal/channel with P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is unclear.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/P-CRI/P-SSBRI/RS-PAI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/P-L1-RSRP, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P/P-L1-RSRP on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol. <omitted text>

Agreement:

The following TP for 38.214 Clause 6.2.1 is endorsed.

Reason for change: P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is missing in the collision handling procedure between a transmission of SRS and transmission of a physical signal/channel.

Summary of change: Add P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP in the collision handling procedure.

Consequences if not approved: the collision handling procedure between a transmission of SRS and transmission of a physical signal/channel P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is unclear.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s), or only RS-PAI report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s), or only RS-PAI report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH. <omitted text>

Agreement:

Regarding the RAN2 LS from R1-2506715, for the UE-side candidate data collection configuration for BM-Case2, include the following IEs along with the already agreed IEs in RAN2:

- TimeGap-r19, i.e., the expected time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of expected predicted time instances.

FFS: ReportConfigType, ReportQuantity

### Specification support for positioning accuracy enhancements

R1-2508009

Agreement:

The following text proposal to TS 38.214 V19.1.0 is endorsed.

| Reason for change | - Capture the agreements related to associated ID for ensuring consistency between training and inference for Rel-19 AI/ML positioning. - For training data provided by PRU, TRP-LocationInfo-Implicit may be provided together with NR-PRU-DL-Info for ensuring consistency between training and inference for Rel-19 AI/ML positioning. |
| --- | --- |
| Summary of change | - Describe the UE behavior on receiving associated ID from the network. - Clarify that the UE may be provided with NR-TRP-LocationInfo-Implicit associated with NR-PRU-DL-Info. |
| Consequences if not approved | For Rel-19 AI/ML positioning, the UE may not be able to ensure consistency between training and inference. |
| Suggested text proposal | TS 38.214 V19.1.0 ##### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID. - If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell; - Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell. If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell. If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. The UE may be configured by the network with TRP-LocationInfo-Implicit-Element [17, TS 37.355]. - If TRP-LocationInfo-Implicit-Element is provided, and if nr-PhysCellID or nr-CellGlobalID is provided with nr-AIML-AssociatedID, for TRP(s) in the given cell, the UE may assume that the geographical coordinates of the TRP(s) are consistent for a same nr-AIML-AssociatedID. < Unchanged parts are omitted > The UE may be provided with NR-PRU-DL-Info which contains measurement(s) performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurement(s), and the location information of the PRU. The UE may be provided with NR-TRP-LocationInfo-Implicit [17, TS 37.355] associated with NR-PRU-DL-Info. < Unchanged parts are omitted > |

Agreement:

Regarding the RAN2 LS R1-2506737, the following response from RAN1 is endorsed. The draft LS response in R1-2508025 will be updated accordingly to reflect the following responses.

Question 1: Does a single Associated ID correspond to the location coordinates of one specific TRP within a cell, or does it represent the location information of a group of TRPs in the same cell, each having distinct location coordinates? Response to Question 1: As stated in RAN1#121 agreement, an associated ID is configured per cell (e.g., NCGI-r15). It is provided for a group of TRP(s) in the same cell, not for one specific TRP within a cell. Note: A group of TRP(s) in the same cell can have only one TRP. Question 2: Is the Associated ID in any way related to the identification of the location of Antenna Reference Points (ARPs) associated with DL-PRS Resource Sets and DL-PRS Resources? Response to Question 2: It is up to network implementation whether the associated ID is related to the identification of the location of ARP(s). Question 3: Are Associated IDs unique across different cells? Can cells belonging to different Positioning Frequency Layers (PFLs) share the same Associated ID? Furthermore, if a single cell is part of multiple PFLs, should it be assigned a distinct Associated ID for each PFL? Response to Question 3: As captured in the RAN1#121 agreement, the associated ID is configured per cell, not per PFL. Specific to RAN2’s Question 3: - Associated ID(s) are not required to be unique across different cells. - As associated ID(s) are unique only per cell, sharing associated ID(s) for cells belonging to different PFLs is irrelevant. - The associated ID is not PFL specific. The associated ID is specific only to geographical coordinates of a group of TRP(s) in a cell. Question 4: RAN2 observes that an Associated ID is configured "per-Cell". However, NR DL-PRS Assistance Data support also PRS-only Transmission Points (TPs), which are not associated to any specific cell (i.e., have no NCGI). Can an Associated ID also implicitly indicate the location coordinates of such PRS-only TPs? Response to Question 4: From RAN1’s perspective, in Rel-19, the associated ID cannot be used to implicitly indicate the consistency of location coordinates of such PRS-only TPs, as the PRS-only TPs are not associated with a cell.

### Specification support for CSI prediction

R1-2508061

Agreement:

The following TP for TS 38.214 is endorsed.

| Reason for change | During non-active periods of cell DTX, UE is not expected to receive P/SP CSI-RS for CSI report associated with report quantity comprising at least RI which means this can be applied to SP inference report. However, whether it is applied for monitoring report is not clear. |
| --- | --- |
| Summary of change | Report quantity ‘csi-pai-r19’ is added. |
| Consequences if not approved | Inconsistent UE behaviour during non-active periods of cell DTX. |
| TS38.214 5.1.6.1 CSI-RS reception procedure << Unchanged parts are omitted >> During non-active periods of cell DTX if cell DTX is activated for a serving cell, the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. If the cell DTX is activated for a serving cell [10, TS 38.321], the most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in active periods of cell DTX for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' or 'cjtc-P' or ‘csi-pai-r19’. << Unchanged parts are omitted >> |  |

Agreement:

Regarding the RAN2 LS from R1-2506715, for the UE-side candidate data collection configuration for CSI prediction, include the following IEs:

- CSI-ResourceConfigId

- FFS: whether to include other parameters