3GPP TSG RAN WG1 #122bis			R1-250abcd

Prague, Czech, Oct. 13th – 17th, 2025

Agenda Item:	10.4.1

Source:	Moderator (China Telecom)

Title:	FL Summary #1 of Coverage Enhancement for NR Phase 3

Document for:	Discussion

- Introduction

In RAN #108 meeting, a new 5G-A Rel-20 WI Coverage Enhancement for NR Phase 3 was approved [1], and was revised in RAN#109 [2], the objectives are listed as follows.

The detailed objectives for coverage enhancement of this work item include: - Specify following PRACH coverage enhancements [RAN1, RAN2] - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure. - UE may receive UL beam information after transmission of MSG1 - UL beam information is to assist the UE decision on Msg3 beam selection. - Note 1: “different Tx beams” is for the purpose of future RAN1 discussions - Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable. - Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable. - Note 4: The PRACH repetitions are transmitted over ROs associated with the same SSB. - Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18 - Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI [RAN1, RAN2] - Specify enhancements to improve PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables [RAN1]

In this document, based on the submitted contributions listed in reference section, a summary of companies’ proposals on “Coverage Enhancement for NR phase 3” is provided to assist the discussion.

Here is the color code used in this summary:

- FL observations

- FL proposals

- FL summary based on the companies’ input

- RAN1 agreements

- Contact information

Companies are welcome to input your contact information below, so that I could be familiar with you as soon as possible, and also for organizing any unofficial discussions if necessary.

| Company | Name | Email |
| --- | --- | --- |
| China Telecom | Hang Yin | yinh6@chinatelecom.cn |

The QR codes of FL’s Wechat/Whatsapp/LINE are attached as follows. Companies are welcomed to reach me out by scanning any of the QR codes so that we can have a stronger connection before we meet.

- Discussion Points on PRACH enhancements

In this section, FL summarized the key items for making progress on the PRACH enhancements with different Tx beams based on companies’ contributions.

Issue#1-1: Understanding of PRACH with Different Tx Beams

Companies’ views on Issue#1-1 can be found as follows.

| Company | Opinions |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 1: Selection of same or different Tx beams in PRACH repetitions is up to UE implementation. |
| ZTE, Sanechips | Proposal 1: Multiple PRACH transmissions mechanism specified in Rel-20 should focus on RACH procedure initiated by UE in RRC non-connected state. . Observation 2: The standardization complexity can be reduced by adopting a single sweeping factor, for example, initial selection and fall back operation among different sweeping factors can be avoided. Proposal 5: The PRACH sweeping factor should be introduced to define the RO group size, which represents the maximum number of ROs that a UE can utilize for PRACH sweeping transmission. And the following two cases should be considered to determine the ROs used in PRACH transmission: - Case-1: ROs used for PRACH sweeping are selected from the RO group according to UE capability by assuming that a different beam is used for each selected RO. - Case-2:All ROs are used by allowing the coupled operation between PRACH sweeping and PRACH repetition in one PRACH transmission, i.e., one beam can be used for multiple ROs. Proposal 6: For PRACH sweeping, a single sweeping factor should be defined for RO group size determination. Proposal 8: RAN1 should consider the following cases for PRACH repetition and PRACH sweeping, - Combined operation of PRACH repetition and PRACH sweeping; Selection/Switching among PRACH repetition and/or PRACH sweeping. |
| Xiaomi | The number of Tx beams is UE-implementation specific and is not specified. |
| Nokia | Proposal 3. PRACH repetitions with same Tx beam is the baseline solution. PRACH repetitions with different Tx beams can be configured only alongside the baseline solution. |
| CATT | Proposal 5: Further study how to determine the transmission method in a RACH procedure if a UE supports Rel-18 PRACH repetition and multiple PRACH transmissions with different Tx beams simultaneously. |
| OPPO | Proposal 1: The PRACH transmission with different Tx beams should also support preamble repetition on each Tx beam. Observation 1: In case that the valid RO set consists of multiple subsets and each subset associated with same beam, the UE transmission and gNB reception can be same as Rel-18 in each valid RO subsets. Proposal 3: Reuse the procedure in Rel-18 to define the valid RO set for multiple PRACH transmissions with different Tx beams. Proposal 4: The valid RO set used for PRACH transmission consists of multiple valid RO subsets. Each subset is associated with same Tx beam. |
| LGE | Observation 1: It is important to consider whether the UE has the capability to transmit using multiple Tx beams, and how many beams it can support. Proposal 1: For multiple PRACH transmission with different beams, support to divide the multiple RACH occasions within a group into sub-groups, with each sub-group associated with a specific UE Tx beam. - If the UE supports fewer UE Tx beams than the number of sub-groups, it may reuse the same Tx beam across multiple sub-groups. |
| Transsion Holdings | Proposal 2: It is recommended to support the combination of multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with the same Tx beam. |
| China Telecom | Observation 2: Using same Tx beam in multiple PRACH transmissions with different Tx beams shouldn’t be forbidden. Proposal 5: When multiple PRACH transmission with different Tx beams, it is up to UE’s implementation to determine which spatial filter is used. |
| KT | Observation 1-1: For Case A, B, G, and H, an additional configuration to define R or K is required. Observation 1-2: For Case C and I, two additional configurations to define two of K, R, or G are required. Observation 1-3: For Case D and E, two additional configurations to define F and R (or K) are required. Observation 1-4: For Case F, three additional configurations to define F and two of K, R, or G are required. Proposal 2: Different preamble indices with predefined relationships should be used for different transmit beams, at least in Cases A, C, D, F, G, and I. |
| Panasonic | Proposal 1: Support that it is up to UE implementation to determine which Tx beam for transmitting each of N PRACHs. Proposal 2: Support that a UE determines a group of N ROs, which are associated with the same SSB, for transmitting N PRACHs based on UE implementation. |
| Qualcomm | Proposal 3: For multiple PRACH transmissions, support allowing both same and different Tx beam transmissions within the same PRACH attempt with multiple PRACH transmissions. Proposal 4: Within any RO group used to support Release 20 multiple PRACH transmission with beam sweeping, allow configuration of transmission modes that combine beam sweeping and repetition, i.e., allow repetitions using each beam. |
| Ericsson | Proposal 2: Adopt the working assumption that Tx beams for multiple PRACH transmissions are up to UE implementation. |
| NTT DOCOMO | Proposal 1: Support multiple PRACH transmissions with different beams for CBRA and CFRA. Proposal 3: UE determines whether to select PRACH resource for multiple PRACH transmissions with different beams based on UE capability of beam correspondence with or without TX beam sweeping. - If UE supports none of Rel-15/16 beam correspondence without TX beam sweeping, UE selects PRACH resource for multiple PRACH transmissions with different beams. - If UE supports at least one beam correspondence without TX beam sweeping out of Rel-15/16 beam correspondence capabilities, UE selects PRACH resource for single PRACH transmission or PRACH resource for PRACH repetitions with same beam, based on RSRP threshold for Msg 1 repetition. |
| Sharp | Observation 1 - When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection, compared to single PRACH transmission. Observation 2 - When UE is not capable of full beam correspondence, it may be beneficial to employ the PRACH beam sweeping at the UE side to improve the detection performance and reduce the access latency, compared to single PRACH transmission. |
| ASUSTek | Proposal 1: RAN1 study following aspects for Msg1 repetition with different TX beams: - Which type RA UE could apply Msg1 repetition with different TX beams (e.g., CFRA, CBRA, or both) - Whether to support RACH partition for Msg1 repetition with different TX beams - In which condition, the UE is allowed to apply Msg1 repetition with different TX beams (e.g., UE determines based on RSRP, or gNB indicates) and how UE determines Rel-18 or Rel-20 Msg1 repetition scheme for RA procedure - After RA procedure is completed, which TX beam is used as default beam for UL transmission before receiving TCI state indication. |
| CEWiT | Proposal 1: Consider Rel-18 PRACH repetitions using the same Tx beam as the baseline Proposal 4: The maximum number of PRACH repetitions should be restricted to the maximum number of supported UL Tx beams. Proposal 5: UE identifies the number of PRACH repetitions based on the available UL Tx beam |

FL Observations:

Aligning the common understanding of extending multiple PRACH transmissions with same Tx beam to different Ts is the prerequisite of achieving a unified design. According to the contributions, the key issue for reaching the consensus is the determination of TX beam utilized for PRACH transmission.

Based on companies’ understanding (SPD, ZTE, Sanechips, Xiaomi, OPPO, LGE, Transsion Holdings, China Telecom, KT, Panasonic, Qualcomm, Ericsson, NTT DOCOMO, Sharp), it should be up to UE implementation to determine which Tx beams should be selected for PRACH transmission, which means same Tx beam can also be used for partial/all of the PRACH transmission in Multiple PRACH transmissions with different Tx beams.

What’s more, some companies (OPPO, LGE, KT) proposed to introduce a sub-RO group, within which the same Tx beam is used, so that gNB can acquire an extra detection gain by combining the measurement of all the PRACHs within the sub-RO group.

Besides, as proposed by companies (ZTE, Sanechips, CEWiT), FL think it is also common understanding that the Repetition times of multiple PRACH transmissions with different Tx beams is the maximum Tx beams can be used by UE but not the number of Tx beams are used.

According to the above observations, the following proposal is proposed.

FL’s Proposal 1-1:

It’s up to UE implementation to determine the Tx beams for multiple PRACH transmissions with different Tx beams.

- UE can use the same Tx beam for multiple PRACH transmissions.

- Note 1: whether/how to introduce sub-RO groups (within which the same Tx beam is used) is a separate discussion.

- Note 2: UE is not required to use X Tx beams for X times multiple PRACH transmissions.

Companies are welcomed to provide your comments on FL’s Proposal 1-1 in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL Observations:

Besides, some companies (Nokia, CEWiT) propose that multiple PRACH transmissions with same Tx beam should be the baseline of that with different Tx beams, which means the support of multiple PRACH transmissions with same Tx beam should be the prerequisite for supporting multiple PRACH transmissions with different Tx beams. However, according to the WID, only when UE can’t fulfil the beam correspondence requirements without Tx beam sweeping, UE will use multiple Tx beams for multiple PRACH transmissions, otherwise UE will use the same Tx beam. Moderator think this issue can be further discussed

Companies are welcomed to provide your comments on whether multiple PRACH transmissions with same Tx beam should be the baseline of multiple PRACH transmissions with different Tx beams in the following table.

| Company | Comments |
| --- | --- |

Issue#1-2: Procedures/definitions in multiple PRACH transmissions with same Tx beam reused for multiple PRACH transmissions with different Tx beams

Companies’ views on Issue#1-2 can be found as follows.

| Company | Opinions |
| --- | --- |
| vivo | Proposal 1: Support Type-B PRACH repetition in the same use cases as Type-A PRACH repetition. |
| ZTE, Sanechips | Proposal 2: The following mechanisms specified for Rel-18 PRACH repetition can be reused for Rel-20 PRACH sweeping, - Separate UE feature definition, - SSB-to-RO mapping rule, - Definition of time period, - Single RAR reception within a single RAR window, - PRACH power control. Observation 1: For RO grouping rule of PRACH sweeping, it is no longer necessary to restrict a SSB to be associated with identical preamble indexes solely for the purpose of simplifying combination reception. Proposal 4: For PRACH sweeping, existing RO grouping rule for PRACH repetition should be revisited for latency reduction, e.g., by allowing ROs within one RO group with different frequency resources, or different preamble indexes associated to the same SSBs. |
| Xiaomi | Proposal 1: For multiple PRACH transmissions with different Tx beams, adopt the same design as defined in Rel-18 for multiple PRACH transmissions with the same Tx beam for the following aspects: - Framework for gNB configuration and PRACH resource selection - RO group determination - PRACH mask index - Preamble index - RAR window |
| Nokia | Proposal 2. Signaling structures of the Rel-18 PRACH repetitions feature should be reused as much as possible. |
| CATT | Proposal 2: RO group size including {2, 4, 8} could be supported for multiple PRACH transmissions with different Tx beams, and different sizes of RO group should be differentiated by following RACH partitioning methods. - Different sizes of RO group are transmitted on separate ROs; - Different sizes of RO group are transmitted with separate preamble on shared ROs. |
| OPPO | Proposal 2: The maximum number of preamble repetitions should be increased for PRACH transmission with different Tx beams. Proposal 3: Reuse the procedure in Rel-18 to define the valid RO set for multiple PRACH transmissions with different Tx beams. Proposal 5: At least following options could be considered for the time period definition. - Option 1: Same definition as Rel-18. - Option 2: Integer number of association pattern periods. - Option 3: Integer number of time period defined in Rel-18. |
| InterDigital | Proposal 4: The UE will transmit Msg1 beam sweep repetitions over an RO group, similar to the design in Rel-18. |
| China Telecom | Observation 1: To minimize the specs impact of multiple PRACH transmissions with different TX beams, the definitions and mechanisms specified in Rel-18 for multiple PRACH transmissions with same beam should be reused as much as possible. Proposal 1: The following definition for multiple PRACH transmissions with same Tx beam in Rel-18 should be reused for that with different Tx beams: - RO group - Time period - Msg1-RepetitionTimeOffsetROGroup (time offset between RO groups) Proposal 2: Reuse the following definition/rules for multiple PRACH transmissions with different Tx beams - SSB-to-RO mapping rule - Starting position of RAR time window Proposal 3: For multiple PRACH transmissions with different Tx beams, candidate values for msg1-RepetitionNum-r18 should be {2, 4, 8}. |
| Transsion Holdings | Proposal 1: It is recommended to clarify whether multiple PRACH transmissions with different Tx beams are associated with the same SSB or with different SSBs. Proposal 3: It is recommended to discuss whether different Tx beams should share the same RO set or be configured with separate RO sets. |
| Panasonic | Proposal 3: For multiple PRACH transmissions with different Tx beams, support to define RAR window at the end of group of N ROs for N PRACH transmissions. - Note: The above is the same as Rel.18 multiple PRACH transmission with the same beam - FFS to define RAR window before the end of last valid RO for early termination. Proposal 4: For multiple PRACH transmissions with different Tx beams, support calculate RA-RNTI as same as for Rel. 18 multiple PRACH transmissions with same Tx beam. |
| Offino | Observation 1. Alignment between CBRA and CFRA applicability would ensure consistency with Rel-18 coverage enhancement. Proposal 1. RAN1 confirms that each PRACH transmission of a multi-beam PRACH set shares the same preamble index, consistent with Rel-18. Proposal 2. RAN1 confirms that RA-RNTI determination for multi-beam PRACH follows the Rel-18 principle of using the last PRACH occasion. |
| ETRI | Proposal 3: PRACH transmissions is supported for CFRA and CBRA. Proposal 4: For RAR monitoring, the RAR window is defined per RO group |
| Lenovo | Proposal 1: Reuse as much as possible the existing solutions specified for PRACH repetition with same beam for PRACH repetition with different beams, including at least repetition number determination, RO determination, time period determination for PRACH repetition. |
| Apple | In other words, dedicated preambles within the shared RO are configured for PRACH transmission with the same Tx beam and with different Tx beams, respectively. Other configuration parameters, such as RO grouping, time period, and repetition levels (i.e., 2, 4, 8), are shared by both features. Proposal 1: For multiple PRACH transmission with different Tx beams, reusing the Rel-18 multiple PRACH transmission with the same Tx beam rules to determine the RO group and time period for RO groups. |
| Qualcomm | Proposal 1: For Release 20 Multiple PRACH transmissions with beam sweeping, each PRACH attempt by a UE is limited to no more than 8 transmissions across all beams regardless of the transmit beam Proposal 2: For Release 20 multiple PRACH transmissions with different transmit beams, reuse the same RO group definition from Release 18, including, the periodicity of the RO groups. Proposal 6: The UE uses the same preamble across all ROs in an RO group during multiple PRACH transmissions regardless of the operating mode defined by the number of beams and number of repetitions per beam. |
| Ericsson | Proposal 4: Discuss whether assigning different preambles to multiple PRACH transmissions with different Tx beams within a single PRACH attempt does not violate Note 5 in the WID, which states that “the procedure for repetitions of a PRACH transmission is as in Rel-18.” Proposal 7: Conclude that the Rel-17 PRACH transmission power computation can be reused to calculate the transmission power for each PRACH occasion in a PRACH attempt comprising multiple PRACH transmissions with different beams, with PREAMBLE_TRANSMISSION_COUNTER tracking the number of PRACH attempts. Proposal 9: Support repetition counts of {2, 4, 8} for multiple PRACH transmissions with different Tx beams. FFS: Need for additional higher repetition count candidates. Proposal 12: Support multiple PRACH transmissions with different Tx beams for CFRA. FFS: Additional enhancements for multiple PRACH transmissions with different Tx beams. |
| NTT DOCOMO | Proposal 4: UE monitors single RAR in RAR window which starts after the last RO. |
| Sharp | Proposal 1 - For multiple PRACH transmissions with different Tx beams - Separate PRACH occasions or shared PRACH occasions with separate PRACH preambles can be configured to differentiate 1) single PRACH transmissions, 2) multiple PRACH transmissions with same Tx beam, 3) multiple PRACH transmission with different Tx beams, using different PRACH repetition levels. - At least repetition level of 2 and 4 is supported. FFS: repetition level of 8. Proposal 2 - For multiple PRACH transmissions with different Tx beams - The same PRACH preamble is applied during multiple PRACH transmissions. - The same SSB to RO group mapping for multiple PRACH transmission with same Tx beams is reused. - The same Tx power is applied during multiple PRACH transmissions. |
| CEWiT | Proposal 3: Adaptation of all remaining features in Rel-18 PRACH repetition framework need to be carefully studied. Proposal 6: The determination of RACH occasions can be adapted from Rel-18 |

FL Observations:

As per Note 5 in WID, multiple PRACH transmission with different Tx beams should reuse the procedure of multiple PRACH transmission with same beam as much as possible. Clarifying what can be reused in Rel-20 is necessary. According to the contributions, the potential procedures and definitions to be reused are listed as follows:

- Scenarios of multiple PRACH transmissions with different Tx beams: i.e., both CBRA and certain case (ReconfigurationWithSync case) of CFRA.

Supporting companies (4): vivo(?), Offino, ETRI, NTT DOCOMO

- RO group definition and determination: For a PRACH transmission with Npreamblerep preamble repetitions, a set consists of Npreamblerep valid PRACH occasions that are consecutive in time, use same frequency resources, and are associated with same one or more SS/PBCH block index(es), and each SS/PBCH block index is associated with same preamble indexes in all valid PRACH occasions within the set.

Supporting companies (10): Xiaomi, OPPO, InterDigital, China Telecom, Offino, Lenovo, Apple, Qualcomm, Sharp, CEWiT

Objecting companies: ZTE, Sanechips (no need to associated with the same SSB)

- Definition of time period:

Supporting companies (5): ZTE, Sanechips, OPPO (one of acceptable options), China Telecom, Lenovo, Apple

- SSB-to-RO mapping rule：as legacy

Supporting companies (3): ZTE, Sanechips, China Telecom, Sharp

- RAR window: start after the last RO in RO group

Supporting companies (6): ZTE, Sanechips, Xiaomi, China Telecom, Panasonic, ETRI, NTT DOCOMO

- PRACH power control

Supporting companies (6): ZTE, Sanechips, Ericsson,

- Candidate values of repetition times: i.e., {2.4,8}

Supporting companies (6): CATT, China Telecom, Qualcomm(no more than 8), Ericsson, Apple, Sharp (FFS:8)

Objecting companies: OPPO (larger number than 8)

- Time offset between RO groups (Msg1-RepetitionTimeOffsetROGroup)

Supporting companies: China Telecom

- RA-RNTI calculation rules:

Supporting companies (2): Panasonic, Offino

- Not applying PRACH mask:

Supporting companies: Xiaomi

For the RA-RNTI calculation rule, it is associated with the indication methods of UL beam, FL think it can be discussed if it is concluded that RA-RNTI won’t be used for UL beam indication. For the other issues, based on the majority’s view, the following proposals are proposed on which definitions/mechanism of multiple PRACH transmission with same Tx beam can be reused for multiple PRACH transmission with multiple Tx beams.

FL’s Proposal 1-2a:

The multiple PRACH transmission with different Tx beams is supported for both CBRA and ReconfigurationWithSync case in CFRA.

Companies are welcomed to provide your comments on FL’s Proposal 1-2a in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL’s Proposal 1-2b:

Reuse the following definitions and mechanisms in multiple PRACH transmissions with same Tx beam for multiple PRACH transmissions with different Tx beams.

- Definition and determination of RO group

- definition of time period

- SSB-to-RO mapping rule

- Definition of RAR window

- PRACH power control rule (power ramping is not included)

- Time offset between RO groups

- Note: the terminology RO group stands for the ROs for multiple PRACH transmissions as specified in TS 38.213, i.e., “set consists of Npreamblerep valid PRACH occasions…”.

Companies are welcomed to provide your comments on FL’s Proposal 1-2b in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL’s Proposal 1-2c

The candidate values of repetition times for multiple PRACH transmissions with different Tx beams is {2, 4, 8}.

Companies are welcomed to provide your comments on FL’s Proposal 1-2c in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL’s Proposal 1-2d:

PRACH mask is not supported for multiple PRACH transmissions with different Tx beams in 5G-A Rel-20.

Companies are welcomed to provide your comments on FL’s Proposal 1-2d in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#1-3: Differentiation of PRACH resources

Companies’ views on Issue#1-3 can be found as follows.

| Company | Proposals |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 3: To differentiate the different Tx beam with the same Tx beam within PRACH repetition and PRACH without repetition, support both of Configuration 1 and Configuration 2 by reusing Rel-17 framework of feature combination (FeatureCombination-r17). |
| vivo | Proposal 2: Reuse the mechanism of PRACH repetition number differentiation for the purpose of differentiating PRACH repetition with same or different TX beams. |
| Huawei, Hisilicon | Proposal 2: Support separate RACH Occasion (RO) solutions and separate preamble solutions to differentiate multiple transmissions with different Tx beams from the case where multiple PRACH transmissions are transmitted using the same Tx beam. |
| Xiaomi | Proposal 2: Configure independent PRACH resources for two features of “same Tx beam” and “different Tx beams”. The UE’s selection between the two features when both are enabled is up to RAN2’s decision. |
| Nokia | Proposal 1. Reserved preambles and/or ROs are configured for PRACH repetitions with different Tx beams Proposal 2. Signaling structures of the Rel-18 PRACH repetitions feature should be reused as much as possible. |
| CATT | Proposal 1: Multiple PRACH transmissions with different Tx beams should be introduced as a new UE feature for early identification. Proposal 2: RO group size including {2, 4, 8} could be supported for multiple PRACH transmissions with different Tx beams, and different sizes of RO group should be differentiated by following RACH partitioning methods. - Different sizes of RO group are transmitted on separate ROs; - Different sizes of RO group are transmitted with separate preamble on shared ROs. |
| InterDigital | Proposal 1: Support network providing the configuration for the RACH partition intended for Msg1 sweep. Proposal 2: Support the RACH partition for Msg1 beam sweep different from the RACH partition for Msg1 repetitions with a single beam. |
| Samsung | Network needs to differentiate the transmission mode (i.e., using the same or different UL Tx beams) that UE uses for the preamble repetitions in order to decide for combined detection or individual detection. Network configures separate RACH resources e.g., ROs and/or preambles for different transmission modes (i.e., same or different Tx beam) UE uses for the preamble repetition. |
| China Telecom | Proposal 4: Adopting feature combination framework to realize the corresponding PRACH resource partitioning for PRACH transmissions with different Tx beams for UE. |
| TCL | Proposal 2: RACH preambles can be used to notify the gNB that the UE used multiple PRACH transmissions with different beams. |
| Lenovo | Proposal 2: Dedicated ROs and/or preambles can be provided to UEs for PRACH repetition with different beams. Proposal 3: To configure dedicated ROs/preambles, RAN1 to discuss whether to introduce a new feature for PRACH repetition with different beams or reuse the legacy feature “msg1-Repetitions” for RACH resource configuration. |
| Denso | Proposal 1: Support differentiation of PRACH resources between PRACH repetition with different beams and PRACH repetition with same beams. - PRACH resources can be differentiated by ROs and/or PRACH preambles |
| Apple | Proposal 2: If both multiple PRACH transmission with same Tx beam and different Tx beams are configured. Only the share RO resources is supported with dedicated configured preambles for each feature. |
| Qualcomm | Proposal 5: Different combinations of beam sweeping and beam repetition within an RO group are separated via preamble partitioning. |
| Ericsson | Proposal 1: Down select between the following Option 1 and Option 2 as the starting point for defining the procedure for multiple PRACH transmissions using different Tx beams: - Option 1: Multiple PRACH transmission with different Tx beams is supported. PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is not supported. - Note: If multiple PRACH transmission with different Tx beams is supported, the mechanism defined for multiple PRACH transmissions with the same beam should be reused as much as possible. - Option 2: Multiple PRACH transmission with different Tx beams is supported. PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is supported. - FFS: whether/how to indicate best UL beam based on multiple PRACH transmissions for the subsequent UL transmissions. - Note: If multiple PRACH transmission with different Tx beams is supported, the mechanism defined for multiple PRACH transmissions with the same beam should be reused as much as possible. Proposal 5: Discuss methods to differentiate multiple PRACH transmissions with different Tx beams and with the same Tx beam, including using different preambles across beams, by allocating distinct PRACH resources (either via separate preambles within shared occasions or via additional occasions), or configuring different PRACH occasions in a valid set of PRACH occasions. |
| NTT DOCOMO | Proposal 2: Support separate PRACH resource configurations for multiple PRACH transmissions with different beams, PRACH repetitions with same beam, and single PRACH transmission. |
| Sharp | Proposal 1 - For multiple PRACH transmissions with different Tx beams - Separate PRACH occasions or shared PRACH occasions with separate PRACH preambles can be configured to differentiate 1) single PRACH transmissions, 2) multiple PRACH transmissions with same Tx beam, 3) multiple PRACH transmission with different Tx beams, using different PRACH repetition levels. - At least repetition level of 2 and 4 is supported. FFS: repetition level of 8. |

FL Observations:

Most companies (Spreadtrum, UNISOC, Huawei, Hisilicon, Xiaomi, Nokia, CATT, InterDigital, Samsung, China Telecom, TCL, Lenovo, Denso, Apple Qualcomm, NTT DOCOMO, Sharp) agree that multiple PRACH transmissions with different Tx beams should be differentiated with multiple PRACH transmissions with same Tx beam using separate preambles/ROs, which can be achieved by PRACH partitioning methods based on feature combination framework as specified in Rel-18.

And proposed by Apple, when both multiple PRACH transmissions with same Tx beam and different Tx beams are configured, to reduce the resources configured for PRACH transmissions, only shared RO resources with separate preambles should be configured. However, FL think it is not an essential issue for specifying the differentiation mechanism and not mentioned by other companies. Thus, FL would like to treat this issue with low priority.

FL’s Proposal 1-3:

Support to use separate ROs or separate preambles on shared RO to differentiate multiple PRACH transmissions with same Tx beam and different Tx beams.

  - Details can be decided by RAN2

Companies are welcomed to provide your comments on FL’s Proposal 1-3 in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#1-4: Criteria of determination of repetition times for Multiple PRACH transmissions with different Tx beams

Companies’ views on Issue#1-4 can be found as follows.

| Company | Proposals |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 2: For RSPR thresholds configuration of PRACH repetition, two alternatives can be considered. - Alt1: Same RSRP thresholds as same Tx beams of PRACH repetitions - Alt2: Separate RSRP thresholds for PRACH repetition with different Tx beams |
| vivo | Observation 9: Better coverage is required for selection Type-B PRACH repetition compared to Type-A PRACH repetition. Proposal 6: The RSRP of downlink pathloss reference should be no less than an RSRP threshold so that a Type-B PRACH repetition can be selected for a given repetition number. |
| ZTE, Sanechips | Proposal 3: An RSRP-threshold can be considered as a factor for triggering PRACH sweeping, - FFS: Whether the existing RSRP-Threshold configured for PRACH repetition can be reused for PRACH sweeping. |
| Huawei, Hisilicon | Proposal 10: For multiple PRACH transmissions with different Tx beam, SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions for the first RACH attempt. - FFS: details on how to determine the values of the SSB-RSRP threshold(s). |
| CATT | Proposal 3: Further study whether to introduce separate RSRP thresholds for PRACH transmissions with different Tx beams to determine the size of RO group and select reference signal. |
| InterDigital | Proposal 3: Introduce conditions for a UE to select the number of repetitions/beams for Msg1 transmission, e.g., based upon received signal strength and degree of misalignment for UE UL beam, etc. |
| Samsung | - Observation2: The instability of the UL Tx beam sweep gain makes the legacy RSRP-threshold-based method unsuitable for determining the preamble repetition number. - Proposal 2: Rather than the legacy RSRP-threshold-based criteria, RAN1 studies a new criterion for preamble repetition number determination, e.g., considering the number of supported UL Tx beams. |
| ETRI | Proposal 2: Discuss whether to introduce additional SSB-RSRP thresholds for PRACH transmission purpose. |
| Lenovo | Proposal 1: Reuse as much as possible the existing solutions specified for PRACH repetition with same beam for PRACH repetition with different beams, including at least repetition number determination, RO determination, time period determination for PRACH repetition. |
| Qualcomm | Observation 1: RSRP thresholds may not serve as an appropriate selection criteria for the number of Tx. Beams. Proposal 9: Multiple PRACH transmission with Tx beam sweeping is not limited to cell-edge UEs. |
| Ericsson | Proposal 10: Support configuring SSB-RSRP thresholds for each candidate repetition count to determine the number of PRACH transmissions during CBRA. |
| CEWiT | Proposal 2: RSRP threshold based PRACH repetitions should not be supported in Rel-20 PRACH repetitions. Proposal 5: UE identifies the number of PRACH repetitions based on the available UL Tx beams |

FL Observations:

In Rel-18, for multiple PRACH transmissions with same Tx beam, RSPR based method is used to determine the number of PRACH transmissions for the first RACH attempt. For the repetition determination of multiple PRACH transmissions with different Tx beams, the opinions of companies are summarized as follows,

1. Reuse the RSRP based method

Supporting companies (11): Spreadtrum, UNISOC, vivo, ZTE, Sanechips, Huawei, Hisilicon, CATT, ETRI, Lenovo, Ericsson

2. Introducing a new determination criterion considering the number of UL beams UE supporting

Supporting companies (4): InterDigital, Samsung, Qualcomm, CEWiT

More companies prefer to reuse the RSRP based method, while most of which it is necessary to discuss whether a new RSRP threshold should be specified, since the gNB may not be able to combine the measurements of multiple PRACH transmissions with Tx beams.

However, on the one hand, the UL Tx beam sweeping gain can be instable due to the different times of using different Tx beams; on the other hand, the cell-center UEs can’t fulfill the correspondence requirement and also want to adopt the multiple PRACH transmissions with different Tx beams, where RSRP is not the key issue to determine the times of repetition. Thus, some companies propose to introduce a new determination criterion at least in addition to RSRP-based criterion.

Based on the above observation, the following proposal is provided for further discussion.

FL’s Proposal 1-4:

For the determination of repetition times for multiple PRACH transmissions with same Tx beam, down-select from the following two options

- Option 1: Reuse the RSRP-based method of multiple PRACH transmissions with same Tx beam in Rel-18.

  - FFS: define new RSRP threshold(s) for different times of repetition.

- Option 2: Introduce a new criterion, e.g., considering the number of Tx beams UE supported.

Companies are welcomed to provide your comments on FL’s Proposal 1-4 in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#1-5: Indication of UL beam information to UE

Companies’ views on Issue#1-5 can be found as follows.

| Company | Proposals |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 4: A fixed bits length in FDRA field in RAR grant is used to indicate application beam of MSG3 PUSCH initial transmission. Proposal 5: For MSG3 retransmission scheduled by DCI format 0_0, separate beam indication of MSG3 PUSCH retransmission in DCI format 0_0 with CRC scrambled by TC-RNTI is supported. Proposal 6: A fixed bits length in FDRA field in DCI format 0_0 is used to indicate application beam of MSG3 PUSCH re-transmission. |
| vivo | Observation 3: Assistant information format depends on the candidate number of TX beams enabled by network independent of the number of PRACH repetitions, as well as the maximum number of TX beams supported by UE. Proposal 3: RAN1 to discuss the candidate number of TX beams in Type-B PRACH repetition. Observation 4: Additional RO timing information indication is needed if the assistant information is implicitly indicated via RA-RNTI. Observation 5: For assistant information indication in RAR, no reserved bits are available in existing MAC PDU of RAR, and repurpose of existing fields of existing RAR or introducing additional fields in a new RAR format is needed. Proposal 4: RAN1 to discuss the pros and cons of assistant information indication via RA-RNTI or explicit signaling in RAR. Proposal 5: RAN1 to discuss whether and how assistant information would be used by UE for UL transmission after successful reception of RAR, and its specification impacts if any. |
| ZTE, Sanechips | Proposal 7: Regarding indication of UL beam information, the following options can be considered, - Option 1: Implicitly indicated by RA-RNTI; - Option 2: Explicitly indicated via MAC RAR, e.g., re-interpretation of existing information field in RAR UL grant. - FFS: Potential impact on combined operation of Msg3 PUSCH repetition and PRACH sweeping. |
| Huawei, Hisilicon | Proposal 1: The gNB indicates to the UE the best beam to be used by the UE for multiple PRACH transmissions based on the previous received PRACH transmissions using multiple beams. FFS: Details on how the best beam is indicated to the UE by the gNB when transmitting PRACH using multiple Tx beams. Proposal 6: The gNB indicates to the UE the best beam to be used by the UE for multiple PRACH transmissions based on the previous received PRACH transmissions using multiple beams. - FFS: Details on how the best beam is indicated to the UE by the gNB when transmitting PRACH using multiple Tx beams. |
| Xiaomi | The gNB could indicate the best one or more UL Tx beams via Msg2, e.g., through at least one of the following solutions: - MAC CE - Reserved bits in the RAR PDU, or - Reinterpretation of existing bits in the RAR UL grant - Other solutions are not to be precluded UL Tx beam determination for Msg3 - In our view, when the UE receives a Msg2 containing an indication of the recommended UL Tx beam, whether the UE follows this gNB-indicated beam for Msg3 PUSCH transmission should be left to UE implementation Proposal 3: For multiple PRACH transmissions with different Tx beams, (re-)consider the following items for further discussion: - Maximum number of PRACH transmissions - UL Tx beam indication mechanism - UL Tx beam determination for subsequent uplink transmissions |
| Tejas | Observation 1: To enhance the performance of subsequent UL transmissions (Msg3 transmission) through beam refinement, the gNB must assess the quality of PRACH transmissions across multiple beams and indicate the most suitable beam to the UEs thereby improving UL coverage. Proposal 1: Multiple PRACH transmissions with different Tx beams should be enabled for 4-step RACH procedure, to assist UE decision on beam selection for subsequent UL transmissions. |
| Nokia | Proposal 4. UL beam information is received by the UE via Msg2. |
| CATT | Proposal 4: UL beam information can be provided by the RO index within a RO group (i.e. Option 1). - FFS indicated by the reserved bits in DCI format 1_0 scrambled by RA-RNTI or carried in RAR. Proposal 7: Network can indicate X candidate beams with superior quality to allow UE select one beam for Msg3 transmission. |
| OPPO | Proposal 6: For the beam information indication, following can be considered. - Explicitly indication, e.g., indicating a specific Tx beam. - Implicitly indication, e.g., indicating an RO resource, which is associated with one Tx beam. Proposal 8: The following options can be considered for carrying the beam indication information. - Option 1: The field in DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI. - Option 2: The RA-RNTI used to scramble the CRC for DCI format 1_0. - Option 3: The field in RAR UL grant. - Option 4: The field in DCI format 0_0 with CRC scrambled by the TC-RNTI. |
| LGE | Observation 2: If gNB indicates the sub-group index of RACH occasion to the UE, the UE can then infer that the Tx beam used for PRACH transmission within that sub-group corresponds to a favorable UL channel condition, and use the same Tx beam for subsequent Msg3 PUSCH transmission. Proposal 3: Indicate the sub-group index of RACH occasion to the UE. Observation 3: In scenarios where the gNB successfully detects PRACH preambles from multiple sub-groups, it is necessary to determine whether the gNB should indicate only one sub-group index or multiple indices to the UE. Indicating a single sub-group index simplifies system design and signaling procedures. Proposal 4: Decide whether gNB should indicate only one sub-group index or multiple indices to the UE. Proposal 5: Discuss following methods to decide how the gNB can convey UE Tx beam-related index information to the UE - Explicit indication via UL grant in RAR/DCI (e.g., MCS, TPC, CSI request field) - Implicit indication via existing UL grant fields (e.g., TDRA) - Indication via RA-RNTI association (e.g., multiple RA-RNTI) Proposal 6: In case multiple PRACH transmission with different beams, for msg3 PUSCH transmission / PUCCH transmission, define UE behavior such that the UE either - Follows the most recently received Tx beam indication from the gNB, or - Uses the Tx beam that was successfully used in the most recent UL transmission. |
| Transsion Holdings | Proposal 4: It is recommended that UL beam indication could be carried by RAR PDSCH, RA-RNTI scrambled PDCCH, or other appropriate methods. |
| InterDigital | Proposal 5: Support beam indication (e.g., among the beams swept for msg1) in msg2 for the subsequent RACH operations and UE transmissions. |
| Samsung | - Observation 4: Providing multiple UL Tx beam info may help UE for MSG3 beam selection but introduce signaling overhead compared to providing single beam info. - Proposal 3: -RAN1 to clarify whether the scope of beam information is limited to single beam or multiple beams. - Proposal 4: The network provides RO (index) within the RO set as beam information to UE to assist the UE decision on UL Tx beam selection. |
| TCL | Proposal 1: The maximum number of Tx beams for multiple PRACH transmissions need to be studied. Proposal 3: A field within the DCI scheduling the RAR can serve as an indicator for the Tx beam. |
| China Telecom | Observation 4: If RA-RNTI is used for indicating UL beam to UE when multiple PRACH transmissions is with different Tx beams, the RA-RNTI should be associated with the valid PRACH occasion selected by gNB with the best measurement results in the set of PRACH occasions. Observation 5: If RA-RNTI is used for indicating UL beam to UE when multiple PRACH transmissions is with different Tx beams, there could be cases that a same RA-RNTI associated with different ROs supposed to using different Tx beams. Observation 6: Introducing a new field in MAC RAR to indicate the UL beam may cause a waste of bits and too much spec. impact. Observation 7: The 1-bit in CSI request field, X LSB bits of TPC command for PUSCH field can be reused for indicating the UL beam for Msg3 beam selection, but there is no enough bits available in a single field in MAC RAR can be repurposed for indicating the UL beam independently. Proposal 8: Using one or more of the following methods to indicate the UL beam to assist UE selecting Msg3 beam. - Opt. 1: RA-RNTI - Opt. 2: Repurpose one or more existing fields in MAC RAR |
| KT | Proposal 3: RA-RNTI for the RAR for the multiple PRACH transmissions with different Tx beams is determined from the RO with the last in time, the topmost in frequency in the RO group for the multiple PRACH transmissions. Proposal 4: PREAMBLE_INDEX of the RAR for the multiple PRACH transmissions with different Tx beams indicates the index of the PRACH preamble with the best beam. |
| Panasonic | Proposal 6: Support to provide UL beam indication for Msg3 PUSCH transmission. - FFS: Details on signalling of the UL beam indication. |
| Offino | Proposal 3. It is proposed that RAN1 consider the option for RAR to indicate one PRACH occasion within the set, enabling the UE to infer the corresponding Tx beam. Proposal 4. It is proposed that RAN1 discusses which uplink transmissions may use the UL beam information indicated after Msg1, e.g., PUSCH/PUCCH transmissions before TCI state activation or default beam selection after BFR. |
| ETRI | Proposal 5: Beam information is included the RAR MAC PDU. Proposal 6: Beam information is included in a partitioned field in RAR UL grant Proposal 7: Addition of new RA-RNTI for multiple PRACH transmissions with different Tx beam outside the legacy range of RA-RNTI and MSGB-RNTI. Proposal 8: Addition of new RA-RNTI which can indicate beam information. |
| Lenovo | Proposal 5: Following two methods could be considered for indicating the Tx beam for Msg 3: - Method 1: Using RA-RNTI to indicate the best beam for Msg 3. Different RA-RNTIs corresponds to different Tx beams for Msg 3 - Method 2: Using the RAR to indicate the best beam for Msg 3, including RAR DCI or RAR itself. |
| DENSO | Proposal 2: For beam indication for Msg3 transmission, the following methods can be considered: - Option 1: Implicit indication using RA-RNTI - Option 2: Explicit indication using a dedicated field in RAR Proposal 3: RAN1 to discuss whether to support beam indication via DCI 0_0 with TC-RNTI for Msg3 retransmission. |
| Apple | Proposal 4: Determine the best UL Tx beam via the RA-RNTI. UE attempts to detect a DCI format 1_0 with CRC scrambled with different RA-RNTIs derived from the ROs in the RO group. |
| Qualcomm | Proposal 7: Discuss the contents of beam indication in MSG2 upon Tx. beam sweeping in MSG1. Proposal 8: For multiple PRACH transmissions with different Tx beams, support beam indication in the MAC-CE of MSG2. - FFS: Use existing MAC-CE or define a new MAC-CE. |
| Ericsson | Proposal 5: Discuss methods for indicating the best UL beam for subsequent UL transmissions, including use of RA-RNTI or the RAPID field in the RAR (when different preambles are used across multiple PRACH transmissions with different Tx beams), where in all cases the method corresponds to the PRACH occasion with the highest SNR. |
| NTT DOCOMO | Proposal 4: One or multiple candidate values for the number of beams are configured by SIB/RRC. UE determines one value based on TX beam sweeping capability. Proposal 5: For multiple PRACH transmissions with different beams, Msg 3 TX beam is explicitly indicated by a new field in RAR. - The presence of the Msg 3 TX beam indication field in RAR is determined based on whether multiple PRACH transmissions with different beams is selected by UE in the RACH attempt. - The field length for the Msg 3 TX beam indication field is determined based on the selected number of PRACH transmission beams. |
| Sharp | To indicate the identified Tx beams for Msg3 transmission, several options can be considered as follows: - Option 1: explicit indication of the selected Tx beams for Msg3 transmission in the RAR - Option 2: implicit indication of the selected Tx beams for Msg3 transmission by extending Type1-PDCCH CSS set Proposal 3 - RAN1 to study the solution to indicate the selected Tx beam for Msg3 transmission. |
| ASUSTek | Proposal 1: RAN1 study following aspects for Msg1 repetition with different TX beams: - How gNB provides the UL beam information (e.g., via PDCCH or RAR) - Study the number of TX beams indicated in UL beam information. |

FL Observations:

Companies (vivo, CATT, LGE, Samsung, TCL, ASUSTek) proposed that more than 1 UL beams can be considered being indicated to UE, so that UE will have more flexibility for Msg3 beam selection. However, such indication will increase the overhead for indication. Besides, companies (vivo, Xiaomi, Offino) proposed that whether/how the indicated UL beam information used by UE should be discussed.

From the perspective of FL, as per the objective in WID, UE may receive UL beam information, which is to assist UE decision of Msg3 beam selection. If no such information indicated to UE, for the UE can’t fulfill the correspondence requirement, there will still be obstacles for successful RA. However, it is up to UE’s implementation whether/how to use the indicated beam information regardless how many UL beams are indicated.

FL’s Proposal 1-5a:

Support indicating up to X(X>=1) UL beam(s) UE for assisting Msg3 beam selection.

- It is up to UE’s implementation on how to use the received UL beam information.

- FFS: value of X

Companies are welcomed to provide your comments on FL’s Proposal 1-5a and preference on whether to indicate more than 1 UL beams to UE in the following table.

| Company | Y/N | Comments and preference |
| --- | --- | --- |

FL Observations:

As for how to indicate the UL beam, following methods are proposed by companies (companies are welcomed to check whether your preference is categorized correctly):

- Option 1: implicitly indicated by RA-RNTI

Supporting companies (11): vivo, ZTE, Sanechips, OPPO, LGE, China Telecom, KT, Lenovo, DENSO, Apple, Ericsson

- Option 2: explicitly indicated by DCI format 1_0 with CRC scrambled by RA-RNTI (e.g., repurposing existing field(s), by reserved bits, etc.)

Supporting companies (6): CATT, OPPO, LGE, TCL, Lenovo, ASUSTek

- Option 3: explicitly indicated by repurposing field(s) of UL Grant (e.g. TDRA, MCS, etc.) or introducing a new field in MAC RAR

Supporting companies (20): Spreadtrum, UNISOC, vivo, ZTE, Sanechips, CATT, Xiaomi, OPPO, LGE, China Telecom, KT, Offino, ETRI, Lenovo, DENSO, Qualcomm, Ericsson, NTT DOCOMO, Sharp, ASUSTek

- Option 4: explicitly by DCI format 0_0 with CRC scrambled by TC-RNTI for Msg3 scheduling

Supporting companies (4): Spreadtrum, UNISOC (for Msg3 retransmission), OPPO, DENSO (for Msg3 retransmission)

- Option 5: explicitly indicated by DCI format 0_0 with CRC scrambled by a new RNTI

Supporting companies: ETRI

- Option 6: implicitly indicated by extending Type1-PDCCH CSS set

Supporting companies: Sharp

From the perspective of FL, all the above options can be considered as long as it is feasible. Companies are welcome to provide your opinions on all of the options in this meeting, and FL will summary the cons & pros as reference for further study.

FL’s Proposal 1-5b:

Indicating the UL beam information via one or more methods from following options:

- Option 1: implicitly indicated by RA-RNTI

- Option 2: explicitly indicated by DCI format 1_0 with CRC scrambled by RA-RNTI (e.g., reserved bits)

- Option 3: explicitly indicated by repurposing field(s) or in MAC RAR

- Option 4: explicitly indicated by introducing new field in MAC RAR

- Option 5: explicitly indicated by DCI format 0_0 with CRC scrambled by TC-RNTI for scheduling Msg3

- Other options are not precluded

Companies are welcomed to provide your comments on FL’s Proposal 1-5b in the following table.

| Company | Y/N | Preference and Comments |
| --- | --- | --- |

Issue#1-6: Power ramping issues

Companies’ views on Issue#1-6 can be found as follows.

| Company | Proposals |
| --- | --- |
| vivo | Observation 7: Events for power ramping counter suspension in case of Type-A PRACH repetition can be reused for Type-B PRACH repetition. |
| LGE | Proposal 2: Discuss conditions for applying power ramping in multiple PRACH transmissions with different Tx beams |
| Samsung | - Observation 5: For the PRACH reattempt, a UE may keep, partially change, or totally change the UL Tx beams used in the previous PRACH transmission. - Observation 6: The legacy power ramping method for PRACH reattempt can cause ambiguity in UE behaviour regarding how to apply power ramping when UE partially changes the UL Tx beams. - Proposal 5: Consider the following power ramping options for PRACH reattempt with multiple UL Tx beams: - Option 1: single power ramping counter, and the power ramping counter suspends if all UL Tx beams are changed for the retransmission - Option 2: single power ramping counter, and the power ramping counter suspends if any of the UL Tx beams is changed for the retransmission - Option 3: multiple power ramping counters, e.g., each beam corresponds to one power ramping counter, and the power ramping counter suspends if the corresponding UL Tx beam is changed for the retransmission |
| China Telecom | Proposal 6: If a new set of spatial domain transmission filters never used before are used by UE in the retransmission, Layer 1 should notify higher to suspend the power ramping counter. |
| Offino | Observation 4. The current specification does not define power ramping behavior for multi-beam PRACH transmissions, which may lead to inconsistent implementation across UEs. Proposal 5. For a power ramping design with multi-beam PRACH transmission, physical layer indicates power ramping suspension in at least one of the following cases: - Option 1) when currently selected beams are fully disjoint with previous transmitted beams, or - Option 2) when current selected beams include any beam that was not included in previously transmitted beams. |
| Apple | Proposal 3: Considering maximum transmission power is used for each PRACH transmission. |
| Ericsson | Proposal 8: Support modifying the legacy rule to suspend the power ramping counter when any of the spatial domain transmission filters changes prior to a retransmission, to accommodate multiple PRACH transmission with different Tx beams. |
| NTT DOCOMO | Proposal 6: No power ramping among multiple PRACH transmissions in one RACH attempt. Legacy power ramping rule is applied among RACH re-attempts. |

FL Observations:

As pointed by many companies, the power ramping rule for multiple PRACH transmissions (shown as follows) are not suitable for multiple PRACH transmissions with different Tx beams.

TS 38.213 clause 7.4 ----<Omitted unrelated part>---- If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher to suspend the power ramping counter as described in [11, TS 38.321]

A common understanding is that for the following case, the power ramping counter shouldn’t suspend, where though the spatial domain filter changes in prior to the retransmission, but the same set of Tx beams are used in the retransmissions.

And for the following case, the power ramping counter is supposed to suspend, where all of the Tx beams in retransmission are changed to new beams.

The situation can be more complicated and the solutions can be diverse in the following cases.

- Case #1: Part of the Tx beams used in retransmission is changed to the Tx beam used in last transmission.

- Case #2: Part of the Tx beams used in retransmission is changed to the Tx beam never used in last transmission.

Companies’ opinions on whether the power ramping counter should suspend in above cases are diverse. Besides, Samsung propose to introduce multiple power ramping counter for different Tx beams, while companies (Apple, NTT DOCOMO) propose to always adopt/not adopt power ramping for Multiple PRACH transmissions with different Tx beams.

Based on the above observation, the following proposal is proposed on power ramping issue.

FL’s Proposal 1-6:

For power ramping for multiple PRACH transmissions with different Tx beams, further discuss the following issues

- Whether to support power ramping.

- Whether/when to suspend power ramping counter.

- Whether to introduce multiple power ramping counter based on each Tx beam

Companies are welcomed to provide your comments on FL’s Proposal 1-6 and preference in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#1-7: Other issues

Companies’ views on Issue#1-7 can be found as follows.

| Company | Opinions |
| --- | --- |
| InterDigital | Proposal 6: Study necessity of binding Msg1 beam sweep (Rel-20) feature with Msg3 repetitions (Rel-17) feature. |
| Panasonic | Proposal 5: Support specify a unified solution for indicating UL beam for Msg3 PUSCH transmission for 2 capabilities of UE, i.e., UE with or without capable of Msg3 PUSCH repetition. |
| Samsung | - Observation 7: If a UE still fails PRACH with UL Tx beam sweep when UE has reached its maximum transmission power, it may suggest that the coverage bottleneck is no longer related to spatial domain beam directions. - Proposal 6: RAN1 considers the fallback mechanism when UE can switch to legacy multiple PRACH transmission with preamble repetitions using singe UL Tx beam. |
| China Telecom | - Proposal 7: In retransmission for multiple PRACH transmissions, fall back between multiple PRACH transmissions with same Tx beam and different Tx beams is not supported. |
| Lenovo | Proposal 6: To enhance Msg3 coverage, Msg1 repetition with different beams can be configured for determining the Tx beam for Msg3 transmission, even the Msg1 coverage is not an issue |
| Denso | Proposal 4: RAN1 to study mechanisms for coupling of Msg3 beam selection and Msg3 repetition. |
| Ericsson | Proposal 3: Discuss mechanisms to account for beam switching time when determining valid PRACH occasions for multiple PRACH transmissions with different Tx beams. Proposal 11: Support incrementing the repetition count to the next available higher value after every predefined number of PRACH attempts for multiple PRACH transmissions with different Tx beams, with the detailed specification left to RAN2. |

FL Observations:

In this section, the issues not mentioned by too much companies are summarized, which can be categorized into 3 issues:

- Fall back issue in retransmission

- Relationship between Msg1 repetition and Msg3 repetition

- Ericsson propose to introduce a time guard in case the time interval between two valid RO may not be enough for beam switching at UE side.

Since the contributions focused on these issues are not many enough, FL would like to invite companies provide your comments (if any) on these issues respectively in this meeting, so that may be a more detailed discussion can be conducted in the future meeting.

Companies are welcomed to provide your comments on Fallback of multiple PRACH transmissions with different Tx beams from any perspective in the following table.

| Company | Comments |
| --- | --- |

Companies are welcomed to provide your comments on Relation of Msg1 Repetition and Msg3 Repetition from any perspective in the following table.

| Company | Comments |
| --- | --- |

Companies are welcomed to provide your comments on Introducing a Time Guard for Ensuring the Time Interval between ROs are Enough for UL beam switching in the following table.

| Company | Comments |
| --- | --- |

Companies are also welcomed to provide your views on Any Other Issues in the following table.

| Company | Comments |
| --- | --- |

- Discussion Points on PUSCH Repetition Scheduled by DCI 0_0 with C-RNTI

In this section, FL summarized the key items for making progress on the PUSCH repetition scheduled by DCI 0_0 with C-RNTI based on companies’ contributions.

Issue#2-1: Clarification on the scenarios of PUSCH scheduled by DCI format 0_0 with C-RNTI

Companies’ views on Issue#2-1 can be found as follows.

| Company | Opinions |
| --- | --- |
| ZTE, Sanechips | Proposal 9: PUSCH repetition scheduled by DCI 0_0 with C-RNTI mechanism specified in Rel-20 should be restricted to the transmission before receiving RRCReconfiguration. |
| Xiaomi | Proposal 4: Clarify whether the scope of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI is limited to Msg5 and post-Msg5 transmissions initiated from RRC_IDLE, or whether it also includes PUSCH with UL-SCH in RRC_CONNECTED states and Msg5/post-Msg5 transmissions from RRC_INACTIVE states. |
| Nokia | Given the above, we suggest avoiding spending time debating on whether Msg5 refers to the first or the second message highlighted in red in . And simply acknowledge that, at the very least, the target payload of the PUSCH scheduled by DCI format 0_0 with C-RNTI is at least 1000 bytes. Proposal 8. For PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI, RAN1 to agree that the target payload is at least 1000 bytes. |
| CATT | Proposal 6: PUSCH repetition can be applied to all PUSCHs scheduled by DCI 0_0 with C-RNTI. |
| OPPO | Proposal 11: Indication of Msg5 PUSCH repetition only apply for Msg 5 scheduling. |
| InterDigital | Observation 4: For UEs under compromised channel environments and completing successful initial access using various coverage enhancement features, Msg5 may become a bottleneck. |
| Samsung | - Observation 8: For discussion purpose in this contribution, Msg5 PUSCH refers to the PUSCH carrying RRC message RRCSetupComplete. - Proposal 7: The repetition enhancement should apply to PUSCH scheduled by DCI 0_0 with C-RNTI transmitted before and after RRCReconfiguration. |
| China Telecom | Observation 8: According to our understanding, there is no typical scenario/case where PUSCH is scheduled by DCI format 0_0 with CRC scrambled by C-RNTI other than Msg5 identified. Proposal 9: The mechanism supporting Msg5 repetition, which is the PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI before RRC connection establishment complete, should be the starting point. |
| Panasonic | Proposal 7: To decide whether an enhancement is to focus on (i) only Msg.5 PUSCH repetition scheduled by DCI format 0-0 with C-RNTI, or (ii) a generic case of PUSCH repetition scheduled by DCI format 0-0 with C-RNTI (including Msg5 PUSCH repetition). |
| Lenovo | Proposal 7：For PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled with C-RNTI, repetition related parameter should be determined separately for: - Case 1: DCI 0_0 with CRC scrambled by C-RNTI is detected in any CSS associated with CORESET 0. - Case 2: DCI 0_0 with CRC scrambled by C-RNTI is detected in any CSS not associated with CORESET 0 or in USS. Proposal 8：For PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled with C-RNTI, - Case 1: DCI 0_0 with CRC scrambled by C-RNTI is detected in any CSS associated with CORESET 0, the repetition related procedure for Msg 3 could be reused. - Case 2: DCI 0_0 with CRC scrambled by C-RNTI is detected in any CSS not associated with CORESET 0 or in USS, the repetition related procedure for DCI format 0_1/2 could be reused.. |
| Ericsson | Proposal 13: Clarify whether the WID scope includes PUSCH repetition scheduled by DCI 0_0 with C-RNTI in the RRC CONNECTED state. Observation 21: Similar to Msg5 PUSCH initial transmission, Msg5 PUSCH retransmissions are scheduled by DCI format 0_0 with C-RNTI. Thus, repetition procedures defined for the initial transmission can be reused for retransmissions, and specification efforts can be first focused on the initial-transmission procedure. |
| Apple | The first case is when PUSCH is scheduled by DCI format 0_0 with C-RNTI before the RRC reconfiguration is completed, i.e., PUSCH after Msg3 and before the completion of RRC reconfiguration, such as Msg5 PUSCH. The second case is when PUSCH is scheduled by DCI format 0_0 after the RRC reconfiguration, where the gNB is already aware of the UE capability |
| NTT DOCOMO | Proposal 7: Support PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, irrespective of scenarios (i.e., TN or NTN) |
| ASUSTek | Proposal 2: RAN1 study following aspect for PUSCH repetition scheduled by DCI 0_0 with C-RNTI: - In which condition, the UE is allowed to apply “PUSCH repetition scheduled by DCI format 0_0 with C-RNTI. |

FL Observations:

PUSCH can be scheduled by DCI 0_0 with C-RNTI before RRC connection is established (Case #1), including Msg5 and subsequent transmissions before RRC connection established; and after the RRC connection is established (Case #2). According to the justification, this objective mainly aims on improving the coverage in Case #1, which is also supported by all the companies (ZTE, Sanechips, Xiaomi, Nokia, CATT, OPPO, InterDigital, China Telecom, Samsung, Panasonic, Lenovo, Ericsson, Apple). There are also companies (CATT, Samsung, Lenovo, Apple) think the PUSCH repetition scheduled by DCI 0_0 with C-RNTI in Case #2 should be supported at the same time. Samsung propose that the RRC reconfiguration can be a typical scenario of Case #2.

For the companies who only support Case #1, on the one hand, it is not likely that the PUSCH repetition scheduled by DCI 0_0 with C-RNTI is needed since the PUSCH repetition scheduled by DCI 0_1/0_2/0_3 has already been supported; on the other hand, supporting both scenarios may result in a very complicated mechanism or two different mechanism, since the signals can be used to request and indicate the repetition are different in these two scenarios.

From perspective of FL, Case #2 can be supported if typical scenarios can be identified, or the same mechanism for Case #1 can be applied in Case #2, otherwise the workload can be too large considering the limited TU for this WI. Thus, the following proposal is provided as a starting point for discussion. And for discussion purpose, Msg5 repetition stands for PUSCH repetition scheduled by DCI 0_0 with C-RNTI before RRC connection established in the following parts of this summary.

FL’s Proposal 2-1:

Specify the mechanism to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI before RRC connection established.

- FFS: whether/how to support PUSCH scheduled by DCI 0_0 with CRC scrambled by C-RNTI after RRC connection established.

- FFS: whether can be applied to NTN scenario

Companies are welcomed to provide your comments on FL’s Proposal 2-1 and preference on whether to support Case #2 in the following table.

| Company | Y/N | Comments and preference |
| --- | --- | --- |

Issue#2-2: Mechanisms of Msg3 repetition can be reused

Companies’ views on Issue#2-2 can be found as follows.

| Company | Proposals |
| --- | --- |
| ZTE, Sanechips | Proposal 11: The following mechanisms specified for PUSCH repetition of Msg3 re-transmission in Rel-17 can be reused for Rel-20 PUSCH repetition scheduled by DCI format 0_0 with C-RNTI, - RV determination, - Available slot determination, - Frequency hopping. |
| Huawei, Hisilicon | Proposal 12: At least the following mechanisms and procedures are studied by RAN1 in Rel-20 for specifying the support of Msg5 PUSCH repetition: - Triggering mechanism for Msg5 PUSCH repetition, indication of number of repetitions and RV pattern. |
| Xiaomi | Proposal 6: For PUSCH transmission scheduled by DCI format 0_0 with C-RNTI during initial access, reuse the same design as for Msg3 repetition in Rel-17 CE for the following aspects: - PUSCH repetition type - Indication of number of repetitions - Candidate values - Resource allocation - Frequency hopping - RV determination - No combination with TBoMS |
| Tejas | Proposal 5: Study of reuse of repetition parameters (like repetition number, RV determination, freq. hopping configurations) indicated via RAR message for repetition of initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C_RNTI. |
| Nokia | Proposal 5. For time and frequency allocation of PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI, the same principle as for PUSCH repetition type A counted on available slots is considered as a starting point. |
| OPPO | Proposal 10: For Msg5 PUSCH repetitions, RV 0 is used for the first repetition of Msg5 PUSCH initial transmission, and a dynamically indicated RV id via DCI 0_0 for the first repetition of Msg5 PUSCH re-transmission. - For the repetition of Msg5 PUSCH initial transmission, the RV sequence is configured by gNB. |
| Denso | Proposal 5: The mechanisms for Msg3 retransmission with repetition can be considered as a baseline for Msg5 repetition in the following aspects: - Indication of number of repetitions and MCS index - Determination of redundancy version and available slots - Frequency hopping |
| Apple | This counting scheme could be extended to PUSCH scheduled by DCI format 0_0. Proposal 8: Consider availabe slot based counting for PUSCH repetition scheduled by DCI format 0_0. |
| Ericsson | Proposal 14: Support Msg5 PUSCH repetition based on Type A repetition principle. Proposal 15: Support applying the available slot determination procedure for Msg5 PUSCH repetition, with the RV pattern {0, 2, 3, 1} cycling across the determined available slots during the initial transmission. |
| Sharp | Proposal 5 - Type A PUSCH repetition is applied for PUSCH scheduled by DCI format 0_0 with C-RNTI. - Available slot counting is applied for PUSCH repetition scheduled by DCI format 0_0 with C-RNTI. |

FL Observations:

Msg3 repetition was introduced in Rel-17, which can be scheduled by DCI format 0_0 with TC-RNTI for initial transmission. Since both procedures are for PUSCH scheduled by DCI format 0_0, some mechanisms can be considered to be reused.

- RV determination:

Supporting companies (8): ZTE, Sanechips, Huawei, Hisilicon, Xiaomi, Tejas, Denso, Ericsson,

Objecting companies: OPPO (?)

- Available slot determination:

Supporting companies (6): ZTE, Sanechips, Xiaomi, Denso, Apple, Sharp

- Frequency hopping configuration:

Supporting companies (5): ZTE, Sanechips, Xiaomi, Tejas, Denso,

- PUSCH repetition type: Type A PUSCH repetition only

Supporting companies (3): Xiaomi, Ericsson, Sharp

- Candidate values:

Supporting companies (2): Xiaomi, Tejas

- No combination with TBoMS:

Supporting companies: Xiaomi

For the candidate values of repetition, FL think it can be discussed as the details of indication of repetition. Based on the view of majority, the following proposal is provided.

FL’s Proposal 2-2:

Reuse the following rules of Msg3 repetition for PUSCH repetition scheduled by DCI 0_0 with C-RNTI before RRC connection established.

- RV determination rule

- Available slot determination rule

- Frequency hopping

- Only applicable to PUSCH Repetition Type A

- Not combined with TBoMS in Rel-17.

Companies are welcomed to provide your comments on FL’s Proposal 2-2 in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#2-3: Request of Msg5 repetition

Companies’ views on Issue#2-3 can be found as follows.

| Company | Opinions |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 7: A new RSRP threshold can be configured for PUSCH repetition scheduled by DCI format 0_0 with C-RNTI. Proposal 8: MSG3 is used for capability/request report of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI. |
| vivo | Proposal 8: LCID of Msg3 can be used for requesting repetition of PUSCH scheduled by fallback DCI with C-RNTI for UEs with RSRP of DL pathloss reference less than an RSRP threshold separately defined. |
| ZTE, Sanechips | Proposal 12: The following two options of triggering mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI can be considered based on the mechanism discussed in section 3.2: - Option 1: An RSRP-based condition can be considered, - Relationship with existing RSRP-Threshold for Msg3 repetition. - Option 2: Reuse the trigger condition for Msg3 repetition. Proposal 10: Regarding early identification of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI, the following options can be considered, - Option 1: Explicitly indicated by, e.g., higher layer signaling or separate PRACH resources; - Option 2: Implicitly indicated via Msg3 repetition. |
| Xiaomi | Proposal 5: For Msg5 and post-Msg5 PUSCH repetitions during initial access, reuse the PUCCH repetition request / early UE capability reporting mechanism defined in Rel-18 NR-NTN for Msg4 HARQ-ACK. - A separate RSRP threshold can be optionally configured by the gNB |
| Tejas | Observation 2: Enabling the repetitions for initial transmission of PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI (Msg5) will yield coverage improvement. Proposal 2: Study the UE requested repetition for the initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, with gNB indicating the number of repetitions. Proposal 4: Study on the request mechanism for repetition of initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, using Msg3-based signalling via DM-RS configurations as per following options : - DMRS sequence partitioning - DMRS resource muting pattern - DMRS symbol position |
| Nokia | For example, the Msg3 repetition request would also be a request for PUSCH repetition scheduled by DCI format 0_0 with C-RNTI, without the need for dedicated RACH resources. Alternatively, the HACK-ACK of Msg4 can be a channel for such capability reporting. Proposal 7. For PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI, RAN1 to discuss details on early UE capability indication. |
| CATT | Proposal 7: PUSCH repetition scheduled by DCI 0_0 with C-RNTI can be early identified in Msg3 PUSCH. |
| InterDigital | Proposal 7: Study whether to specify Msg5 indication design through UE selection of RACH partition or specify the UE indication signalling as part of Msg3. |
| Samsung | UE reports the repetition capability after the successful contention resolution. |
| China Telecom | Proposal 12: Support to introduce a new specific value of eLCID in the MAC header of Msg3 to request the Msg5 repetition. |
| Panasonic | Proposal 8: Support that a UE sends a request for Msg5 PUSCH repetition and/or repetition number to gNB. One of the following options can be considered if UE supports Msg5 PUSCH repetition function. - Option 1: The request is a part of Msg3 PUSCH - Option 2: The request is a part of a PUCCH/UCI carrying HARQ ACK/NACK of Msg4 PDSCH |
| TCL | Proposal 4: The capability of PUSCH repetition scheduled by DCI 0_0 with C-RNTI can be identified through either Msg3 PUSCH transmissions or PRACH transmissions. |
| Denso | Proposal 6: The UE capability for supporting the Msg5 repetition feature can be reported via Msg3 PUSCH transmission. |
| Apple | Proposal 5: For PUSCH repetition before the RRC reconfiguation, UE sends the repetition request and the capability report via the higher layer singalling in Msg3 PUSCH. |
| Qualcomm | Proposal 13: Support early capability indication of repetition of PUSCH scheduled with DCI format 0_0 and scrambled with C-RNTI in MSG3. Proposal 14: In addition to early capability indication of repetition of PUSCH scheduled with DCI format 0_0 and scrambled with C-RNTI, support dedicated capability indication as well. |
| Ericsson | Proposal 18: Support configuring an optional RSRP threshold to allow a UE to request Msg5 PUSCH repetition. Proposal 19: Support that, when the candidates for the repetition count are configured, the UE shall request Msg5 PUSCH repetition only if the RSRP of the DL pathloss reference falls below the configured threshold, or, if no threshold is configured, any UE capable of Msg5 PUSCH repetition shall request the repetition by default. Proposal 20: Discuss methods for requesting Msg5 PUSCH repetition, including via Msg3 PUSCH or Msg4 HARQ-ACK. |
| NTT DOCOMO | Proposal 8: The following details can be considered for specifying PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI: - Repetition factors supported: {2, 4, 8} - For NTN scenario, additional larger value can be considered - NW configuration: SIB1 to configure, e.g., - Turn on/off of the feature in the cell - Candidate repetition factors in the cell - Threshold to choose one of the candidate repetition factors (if needed), e.g., SSB-RSRP for each of the candidate factors in the cell - UE requests: Reuse e.g., Msg3 PUSCH based request - Indication of an actual repetition factor: Add a new field in DCI format 0_0 with CRC scrambled by C-RNTI |
| Sharp | Proposal 4 - UE indicates support of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI via Msg3 transmission. - If UE requests the Msg3 repetition, the UE determines the number of repetitions for PUSCH transmission scheduled by DCI format 0_0 with C-RNTI. |

FL Observations:

According to companies, it is common understanding that the Msg5 repetition should be identified early, which means the request of Msg5 repetition is needed. Most companies (16: Spreadtrum, UNISOC, vivo, Xiaomi, Tejas, CATT, Nokia China Telecom, Panasonic, TCL, Denso, Apple, Qualcomm, Ericsson, NTT DOCOMO, Sharp) support request of Msg5 repetition via Msg3 (e.g., LCID, eLCID, repurpose existing fields). Besides, some companies (Spreadtrum, UNISOC, vivo, Xiaomi) proposed that a new RSRP can be configured for UE to make decision at the same time. Other options are also proposed by companies: e.g., Based on RSRP (ZTE, Sanechips); via HACK-ACK or Msg4 (Nokia, Panasonic, Ericsson); Based on RACH partition (Interdigital)

Since all the companies who propose using other options are also fine with utilizing Msg3 for Msg5 repetition request, FL would like to preclude all the other options. Thus, the following proposal is provided for discussion.

FL’s Proposal 2-3:

Support to request PUSCH repetition scheduled by DCI 0_0 with C-RNTI before RRC connection established via Msg3.

- FFS: whether to configure a separate RSRP of DL reference signal for UE to trigger the request.

- FFS: other details.

Companies are welcomed to provide your comments on FL’s Proposal 2-3 in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#2-4: Indication of Msg5 repetition

Companies’ views on Issue#2-4 can be found as follows.

| Company | Opinions |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 9: A small set of repetition numbers for PUSCH is configured, and one value is dynamically indicated by a new field in padding bits of DCI format 0_0. |
| vivo | Proposal 7: Repetition number of PUSCH scheduled by fallback DCI with C-RNTI can be signalled via the DAI field in the DCI 1_0 with TC-RNTI, with a set of candidate repetition factors configured in SIB1. |
| ZTE, Sanechips | Proposal 13: The following mechanisms can be considered for repetition factor indication of PUSCH repetition scheduled by DCI 0_0 with C-RNTI, - A set with 4 candidate repetition factors can be configured via SIB1, - FFS: Shared with or separate from candidate repetition factors configured for Msg3 repetition; - Default set with repetition factors {1~4} applies if the configuration is absent. - 2 MSBs of the MCS information field in DCI 0_0 with C-RNTI are used for indicating one repetition factor from the configured/default set. Proposal 14: The following mechanisms can be considered for MCS indication of PUSCH repetition scheduled by DCI 0_0 with C-RNTI, - A set with 8 candidate MCS indexes can be configured via SIB1, - FFS: Shared with or separate from candidate MCS indexes configured for Msg3 repetition; - Default set with MCS indexes {0~7} applies if the configuration is absent. 3 LSBs of the MCS information field in DCI 0_0 with C-RNTI are used for indicating one MCS index from the configured/default set. |
| Huawei, Hisilicon | Proposal 11: At least the following mechanisms and procedures are studied by RAN1 in Rel-20 for specifying the support of Msg5 PUSCH repetition: - Triggering mechanism for Msg5 PUSCH repetition, indication of number of repetitions and RV pattern. - Identify which legacy procedures used for Msg3 PUSCH repetition can be reused for Msg5 PUSCH repetition, and which, if any, cannot be. |
| Tejas | Proposal 6: Study of dynamic indication of the number of repetitions for PUSCH (Msg5) retransmission via DCI based on channel conditions. |
| Nokia | Proposal 6. For indicating number of repetitions of PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI, MSBs of MCS field is considered as a starting point (similar to PUSCH repetitions scheduled by DCI format 0_0 with TC-RNTI). RAN1 further discuss details, e.g., the number of MSBs of MCS field. |
| CATT | Proposal 8: PUSCH repetition number can be indicated by 2MSB of MCS field in DCI 0_0 with C-RNTI. - Further study the candidate value of the PUSCH repetition. |
| OPPO | Proposal 9: The following options can be considered for indication of the Msg 5 PUSCH repetition. - Option 1: The TDRA field in DCI format 1_0 and the TDRA entries configured with repetition number. - Option 2: The MSB bits of MCS field in DCI format 0_0. |
| Transsion Holdings | Proposal 5: It is recommended that study a mechanism for indicating the number of repetitions. |
| InterDigital | Proposal 8: Support a UE specific Msg5 repetition indication where the network may indicate a suitable number of Msg5 repetitions. For the signalling to indicate a number of Msg5 repetitions from the network to the UE, the indication may be provided during the RACH procedure itself, e.g., during Msg3/Msg4 or as part of the Msg5 scheduling DCI Proposal 9: Study suitable signaling methods for the network to provide Msg5 repetition indication. |
| Samsung | - Proposal 9: UE determines the number of repetition based on gNB configuration/indication. |
| KT | Proposal 6: For Msg5 PUSCH repetition, the following alternatives can be considered. - Alt 1: A set of repetition factors can be configured via SIB1, and one repetition factor can be indicated via DCI format 0_0 with C-RNTI. - Alt 2: Repetition factor of Msg3 PUSCH can be applied to Msg5 PUSCH by indicating enable or disable of repetition. Observation 2: Frequency hopping is not supported for PUSCH transmission indicated via DCI format 0_0 with CRC scrambled by C-RNTI. Proposal 7: RAN1 to specify frequency hopping for PUSCH transmission indicated via DCI format 0_0 with CRC scrambled by C-RNTI. - Frequency offset can be implicitly configured based on the initial BWP size and parts of FDRA field. - Frequency hopping mode can be implicitly configured based on the frequency hopping flag field and indicated repetition factor. |
| China Telecom | Observation 9: The similar mechanism for indication of Msg3 repetition times can be reused for that of Msg5 repetition if Msg5 repetition times is indicated by DCI format 0_0 with CRC scrambled by C-RNTI. Proposal 10: Adopting the candidate value of Msg3 repetition times, i.e., {1,2,3,4,7,8,16}, as the starting point for that of Msg5 repetition. Observation10: Compared with Msg3, Msg5 has a lower demand for MCS configuration flexibility. Proposal 11: Support using 2 MSB of MCS information field in DCI format 0_0 with CRC scrambled by C-RNTI to indicate Msg5 repetition times from a set of four configured repetition values or from {1, 2, 3, 4} if not configured. |
| Panasonic | Proposal 9: Support that gNB can indicate a repetition number of Msg5 PUSCH to the UE. One of the following options can be considered - Option 1: The indication is a part of Msg4 PDSCH - Option 2: The indication is a part of DCI format 0_0 with CRC scrambled by C-RNTI for scheduling Msg5 PUSCH repetition - Option 3: To mandate certain rule for UE supports Msg5 PUSCH repetition function. |
| TCL | Proposal 5: The Msg5 PUSCH repetition number can be indicated by the two most significant bits of the MCS field within DCI format 0_0 associated with the C-RNTI. |
| Offino | Proposal 6. To enable Msg5 PUSCH repetition scheduled by DCI 0_0 with C-RNTI, consider options of a) explicit DCI fields, b) implicit TDRA-based signaling, or c) semi-persistent configuration. Proposal 7. It is proposed that RAN1 agrees the treatment of Msg5 repetition as non-HARQ repetition of the same TB, keeping HARQ process state unchanged. |
| ETRI | Proposal 1: The maximum number of Tx beams for PRACH transmissions should be configurable via RRC (i.e., SIB1), and support up to 8. Proposal 9: MCS field in the scheduling DCI format may indicate both repetition number and MCS index. Proposal 10: Retransmitting PUSCH can be indicated a repetition number. |
| Apple | Proposal 6: For PUSCH repetition after the RRC reconfiguation, the repetition is enabled by repetition factor confiugration. Proposal 7: Dynamic repetiton factor indication can be consdiered, and re-interpret the bits of existing information field in DCI format 0_0. |
| Qualcomm | Observation 2: In the current specifications, two issues limit the use of the TDRA field to enable repetitions for a PUSCH scheduled by DCI format 0_0 scrambled with C-RNTI: - First, two out of three TDRA tables do not support a configuration for repetition parameter - Second, the clause in TS 38.214 does not allow repetition even when the UE is configured with a dedicated TDRA table with a repetition factor. Proposal 10: Support indication of repetition of PUSCH scheduled with DCI format 0_0 scrambled with C-RNTI via the TDRA field. Proposal 11: For UEs that are not in RRC connected states or not configured with a dedicated TDRA table, support repetition indication of PUSCH scheduled by DCI format 0_0 and scrambled with C-RNTI via a common configuration TDRA table that includes a repetition factor. - FFS: whether to include an extra repetition factor in the existing TDRA table of common configuration or support a new TDRA table in a common configuration with a repetition factor. Proposal 12: Support the following text proposal in TS 38.214 section 6.1.2.1 (Resource allocation in time domain) For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format, 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, or format 0_0 with CRC scrambled with C-RNTI, the number of repetitions K is determined as - if numberofRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberofRepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor, - otherwise K=1. |
| Ericsson | Proposal 16: Support UE-specific dynamic indication of the Msg5 PUSCH repetition count. Proposal 17: Support configuration of more than one candidate for the repetition count for Msg5 PUSCH, with the Msg3 PUSCH repetition candidates included at least. FFS: Extension of the candidates to include {20, 24, 28, 32} Proposal 21: Discuss methods for dynamically indicating the Msg5 PUSCH repetition count, including using the modulation and coding scheme field or the Time Domain Resource Assignment field in DCI format 0_0 with C-RNTI. |
| NTT DOCOMO | Proposal 8: The following details can be considered for specifying PUSCH repetition scheduled by DCI format 0_0 with CRC scrambled by C-RNTI: - Repetition factors supported: {2, 4, 8} - For NTN scenario, additional larger value can be considered - NW configuration: SIB1 to configure, e.g., - Turn on/off of the feature in the cell - Candidate repetition factors in the cell - Threshold to choose one of the candidate repetition factors (if needed), e.g., SSB-RSRP for each of the candidate factors in the cell - UE requests: Reuse e.g., Msg3 PUSCH based request - Indication of an actual repetition factor: Add a new field in DCI format 0_0 with CRC scrambled by C-RNTI |
| Sharp | Proposal 6 - When numberOfRepetitions or pusch-AggregationFactor is configured by higher layers, same mechanism for PUSCH repetition scheduled by DCI format 0_1/0_2/0_3 is reused for that scheduled by DCI format 0_0. - When numberOfRepetitions and pusch-AggregationFactor is not configured by higher layers, MCS field in the DCI format 0_0 with CRC scrambled by C-RNTI is repurposed to indicate the number of repetitions for PUSCH transmission. - 4 number of repetitions can be configured by higher layers. - 2 MSBs of MCS field are used to indicate the number of repetitions. |

FL Observations:

For indicating the Msg5 repetition times, the candidate values can be configured in advanced or using the default values. When the candidate values of repetition time are configured, companies (Spreadtrum, UNISOC, vivo, ZTE, Sanechip, Huawei, Hisilicon, KT, NTT DOCOMO) proposed to configure them in SIB1, reusing the similar procedure in Msg3 repetition.

And companies propose to reuse the repetition times configured for Msg3, FL would like to discuss such mechanism in Issue 2-5.

FL’s Proposal 2-4a:

The repetition times of PUSCH scheduled by DCI 0_0 with C-RNTI before RRC connection established is indicated to UE from a set of (4) candidate values which are predefined or configured in SIB1.

Companies are welcomed to provide your comments on FL’s Proposal 2-4a in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL Observations:

After configuring/predefine a set of repetition values, the indication can be used to dynamically indicate one of values. The companies’ opinions on indication methods are summarized as follows:

- DCI 0_0 with C-RNTI based method (Interdigital, KT, Panasonic)

  - 2 MSB of MCS field (15): ZTE, Sanechip, Huawei, Hisilicon, Tejas, Nokia, CATT, OPPO, CTC, TCL, Ericsson, Offino, ETRI, Apple, Ericsson

  - TDRA based (4): OPPO, Offino, Qualcomm, Ericsson

  - New field: NTT DOCOMO

- DAI field in DCI 1_0 with TC-RNTI: vivo

- Via Msg 4 PDSCH: Panasonic, Interdigital

- Via DCI scheduling Msg3: Interdigital

Nearly all the companies are fine to use the DCI 0_0 with C-RNTI to indicate the Msg5 Repetition, though the detailed indication methods can be different. Only vivo propose to use DAI field in DCI format 1_0 with TC-RNTI. For the detail indication method, most companies propose using the 2 MSB in MCS field for indication or at least as an acceptable option, while Qualcomm only propose use TDRA based indication. Considering the majority’s view, the following proposal is given.

FL’s Proposal 2-4b:

Using the 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI to indicate the repetition times of PUSCH scheduled by DCI 0_0 with C-RNTI before RRC connection established.

Companies are welcomed to provide your comments on FL’s Proposal 2-4b in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL Observations:

The candidate values for Msg5 repetition are not mentioned by too much companies and the current views are diverse. Thus, FL prefer to not making any conclusion on the candidate values of Msg5 repetition times in this meeting. Companies are welcomed to provide your comments on candidate values of Msg5 repetition times in the following table.

| Company | Comments |
| --- | --- |

Issue#2-5: Relation with Msg3 repetition

Companies’ views on Issue#2-5 can be found as follows.

| Company | Opinions |
| --- | --- |
| ZTE, Sanechips | Proposal 10: Regarding early identification of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI, the following options can be considered, - Option 1: Explicitly indicated by, e.g., higher layer signaling or separate PRACH resources; - Option 2: Implicitly indicated via Msg3 repetition. Proposal 12: The following two options of triggering mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI can be considered based on the mechanism discussed in section 3.2: - Option 1: An RSRP-based condition can be considered, - Relationship with existing RSRP-Threshold for Msg3 repetition. - Option 2: Reuse the trigger condition for Msg3 repetition. |
| Tejas | Proposal 3: Study on the default assumption that the UE requests repetitions for the initial transmission of PUSCH (Msg5), scheduled via DCI format 0_0 with CRC scrambled by C-RNTI, whenever repetition was requested for Msg3. |
| Samsung | - Proposal 8: The configuration (e.g., repetition number) of PUSCH repetition should be separated from Msg1 or Msg3 repetition configuration. - Proposal 10: The capability of PUSCH repetition by DCI 0_0 with C-RNTI is not suitable to be tied with Msg1/3 repetition. - Proposal 11: The capability of PUSCH repetition by DCI 0_0 with C-RNTI is separately reported from Msg1/Msg3 repetition request. |
| China Telecom | Proposal 13: Consider to support to trigger the Msg5 repetition when Msg3 repetition is request and adopted. |
| KT | Proposal 6: For Msg5 PUSCH repetition, the following alternatives can be considered. - Alt 1: A set of repetition factors can be configured via SIB1, and one repetition factor can be indicated via DCI format 0_0 with C-RNTI. - Alt 2: Repetition factor of Msg3 PUSCH can be applied to Msg5 PUSCH by indicating enable or disable of repetition. |
| ETRI | Proposal 12: PUSCH repetition feature can be an independent feature of other repetition-related features. |
| Sharp | Proposal 4 - UE indicates support of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI via Msg3 transmission. - If UE requests the Msg3 repetition, the UE determines the number of repetitions for PUSCH transmission scheduled by DCI format 0_0 with C-RNTI. |

FL Observations:

Based on companies’ input, it is pointed out that Msg5 always has a worse performance than Msg3, if Msg3’s coverage is a problem, then Msg5’s coverage will also be a problem. Thus, companies propose the signalling/configuration for Msg3 repetition can be also used for Msg5 repetition at the same time, e.g., implicitly indicate the Msg5 repetition request when Msg3 repetition is requested (ZTE, Sanechip, Tejas, China Telecom, Sharp), the configured repetition time list for Msg3 can be used by Msg5 if not configured (KT).

However, companies (Samsung, ETRI) think that Msg5 repetition should be an independent feature of any other repetition related features (Msg1/Msg3 repetition).

Companies are welcomed to provide your views on if any signalling/configuration for Msg3 repetition can be also applied to Msg5 repetition at the same time (e.g., Msg5 repetition is also requested when Msg3 repetition is requested, the repetition times list of Msg3 repetition can be also applied to Msg5 repetition, etc.) in the following table.

| Company | views |
| --- | --- |

Issue#2-6: Other Issues

Companies are welcomed to provide your comments on any other issues that is essential but not included in the above issues in the following table.

| Company | Comments |
| --- | --- |

- Discussion Points on extending pi/2-BPSK to more MCS entries

In this section, FL summarized the key issues for making progress on the PUSCH repetition scheduled by DCI 0_0 with C-RNTI based on companies’ contributions.

Issue#3-1: Which table(s) to be enhanced

Companies’ views on Issue#3-1 can be found as follows.

| Company | Proposals |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 10: Rel-20 supports to extend pi/2-BPSK to more MCS entries only in Table 6.1.4.1-1 (64QAM MCS table). |
| vivo | Proposal 11: Support pi/2 BPSK for MCS indexes 2 to 5 in MCS Table 6.1.4.1-1, and MCS indexes 6 to 11 in Table 6.1.4.1-2 defined in 3GPP TS38.213. |
| Huawei, Hisilicon | Proposal 13: Extend the MCS entries for pi/2-BPSK in Table 6.1.4.1-1 and Table 6.1.4.1-2 in TS 38.214 to support the maximum achievable SE = 0.8770 up to IMCS = 6 and IMCS = 12, respectively. |
| Xiaomi | Proposal 7: Consider the following aspects on the normative work for pi/2-BPSK to more MCS entries: - MCS table design - Not prefer introducing new MCS table(s) - Consider both 64 QAM table and 64 QAM lowSE table - The scope of application of the feature - Only focus on DFT-s-OFDM PUSCH, as specified in Rel-15 |
| OPPO | Proposal 12: Introducing MCS table with more pi/2-BPSK entries only for DFT-s-OFDM. RAN1 identify the SNR range with coverage limited use case. |
| China Telecom | Proposal 14: Support extending pi/2-BPSK to more MCS entries applying to both MCS tables in current specs, which the evaluation can focus on Table 6.1.4.1-1 as the start point. |
| Panasonic | Proposal 10: Support to extend pi/2-BPSK to MCS indexes 2, 3, and 4 in such as Table 6.1.4.1-1, TS 38.214. - Extending pi/2-BPSK to MCS index 5 can also be considered. Proposal 11: The simultaneous operation of extension of pi/2-BPSK and low-SE MCS table would not be required. |
| Apple | To align with the WID's objective of improving cell-edge UE data rates, the DFT-s-OFDM MCS table for 64QAM should be enhanced |
| Qualcomm | Focusing on Table 6.1.4.1-1, it is clear that pi/2 BPSK can be applicable to MCS entries no higher than MCS 6. Beyond this, the code rate for pi/2 BPSK begins to exceed 1 and is therefore an invalid configuration. |
| Ericsson | Proposal 25: Support a new MCS table equivalent to the legacy Table 6.1.4.1-1, incorporating extended MCS indices to support pi/2-BPSK while retaining the existing values for the other MCS indices, along with a new RRC configuration referring to this table. |
| NTT DOCOMO | Proposal 9: For extension of pi/2-BPSK to more MCS entries, - Follow legacy pi/2-BPSK application approach in Rel-15 to more MCS entries, i.e., support RRC-based switching between pi/2-BPSK and other exiting modulation order(s) - Support to consider entries on Table 6.1.4.1-1 in TS 38.214 - Open to consider Table 6.1.4.1-2 (qam64lowSE) - Open to consider any of the exiting modulation orders for switching with pi/2-BPSK |

FL Observations:

In current specs, there are two MCS tables defined for PUSCH, i.e., Table 6.1.4.1-1 (64QAM table) and Table 6.1.4.1-2 (64QAM lowSE table), in TS 38.214. As per justification, extending pi/2-BPSK targeting to improve the coverage of scenario with high UL date rate, e.g., 5Mbps, extend pi/2 BPSK should at least be applied to ‘64QAM’ table.

Companies (Spreadtrum, UNISOC, vivo, Panasonic, Apple, Qualcomm, Ericsson) prefer to extending pi/2-BPSK to more MCS entries only in 64QAM table. While companies (Huawei, Hisilicon, Xiaomi, China Telecom, NTT DOCOMO) are fine to further considering extending pi/2-BPSK to more MCS entries also in 64QAM lowSE table. Based on companies, the following proposal is provided by FL for discussion.

FL’s Proposal 3-1:

Extending pi/2-BPSK to more MCS entries when Table 6.1.4.1-1 in TS 38.214 is applied.

- FFS: whether extending pi/2-BPSK to more MCS entries when Table 6.1.4.1-2 in TS38.214 is applied.

Companies are welcomed to provide your comments on FL’s Proposal 3-1 and preference on whether extending pi/2-BPSK to more MCS entries in lowSE table in the following table.

| Company | Y/N | Comments and preferences |
| --- | --- | --- |

Issue#3-2: Performance gain derived from power boosting when pi/2-BPSK is adopted

Companies’ views on Issue#3-2 can be found as follows.

| Company | Opinions |
| --- | --- |
| vivo | Observation 14: The overall coverage gain of pi/2 BPSK over QPSK is independent from the PRB allocation. Observation 15: With the assumption of 2.8dB power boosting gain, the overall coverage gain of pi/2 BPSK over QPSK can be achieved when SE is less than 0.877. Observation 16: If different power boosting gains are considered, the overall coverage gain of pi/2 BPSK over QPSK can be obtained at different SE ranges, i.e. different MCS indexes. Proposal 9: pi/2 BPSK extension UE capability should be reported, and the target UEs are of PC3 only. |
| ZTE, Sanechips | Observation 10: QPSK modulation has a higher PAPR value compared to pi/2-BPSK requiring additional power back-off for the UE UL transmissions. |
| Huawei, Hisilicon | Based on our simulations and using as baseline the following Table 2 from TS 38.101-1 which indicates the MPR table for Power Class 3 (PC3) UE, we highlight the following: - The pi/2-BPSK modulation achieves an additional 1dB power gain for the case without power-boosting capability when compared to QPSK modulation values as shown in Table 2. - Using the values of pi/2-BPSK outer RB MPR=0 and QPSK outer RB MPR= 1 given in Table 2 and using the transmit power of 23 dBm for PC3 UEs for both modulations in the non-power-boosting scenario, we have the following: - For pi/2-BPSK: Tx Power – MPR = 23 – 0 = 23 dBm. - For QPSK: Tx Power – MPR = 23 – 1 = 22 dBm. - Additionally, the pi/2-BPSK modulation achieves an additional 2.8dB power gain when power-boosting is enabled for pi/2-BPSK compared to the QPSK modulation values as shown in Table 2. - Using the values of pi/2-BPSK outer RB MPR= 1.2 and QPSK outer RB MPR= 1 given in Table 2 and using as reference the transmit power of 26 dBm for pi/2-BPSK (powerBoosting-pi2BPSK UE capability, see NOTE 1 in Table 2), we have the following: - For pi/2-BPSK: Tx Power – MPR = 26 – 1.2 = 24.8 dBm. - For QPSK: Tx Power – MPR = 23 – 1 = 22 dBm. |
| Xiaomi | Observation 1: The extension of π/2-BPSK to additional MCS entries is only applicable to UEs that: - Support the powerBoosting-pi2BPSK capability, - Operate in bands n40, n41, n77, n78, or n79, and - Belong to Power Class 3 (23 dBm). Under these conditions, such UEs can achieve a power boosting gain of approximately 2.8 dB, primarily due to the significantly reduced MPR. |
| Nokia | For UE indicating power boosting, the situation is more complex since it depends on hypotheses related to whether the UE is a PC3 device and operating in TDD bands for which 26 dBm reference power applies, or whether UE can boost QPSK up to 1 dB above 23 dBm using transparent techniques and so on. |
| CATT | Observation 2：For a power class 3 UE, pi/2 BPSK can have extra 2.8 dB gain than QPSK due to power boosting. |
| Ericsson | Observation 43: When a UE does not support power boosting, the power gain of pi/2-BPSK over QPSK ranges can be up to 1 dB, while with power boosting, the gain can be up to 2.8 dB when the upper bound of MPR values are applied to the maximum transmit power of a PC3 UE. |

FL Observations:

According to Table 6.2.2-1, an extra power gain can be acquired when pi/2-BPSK is applied due to the lower PAPR compared with QPSK for PC3 when DFT-s-OFDM is used. As pointed by companies, the power gain can be up to 2.8dB when power boosting is adopted (vivo, ZTE, SaneChip, Huawei, Hisilicon, Xiaomi, Nokia, CATT, Ercssion), while this power gain will reduce to only 1 dB if there is no power boosting (Huawei, Hisilicon, Ericsson). This power gain is the main source of where the performance gain applying pi/2-BPSK derived from. Therefore, to maximum the performance gain be acquired, the following proposal is provided for discussion.

FL’s Proposal 3-2:

Extending pi/2-BPSK to more MCS entries for PC3 UE using DFT-s-OFDM with power boosting.

- FFS: whether extending pi/2-BPSK can be applied to PC3 UE using DFT-s-OFDM without power boosting.

Companies are welcomed to provide your comments on FL’s Proposal 3-2 and preference on whether to extend pi/2-BPSK to more MCS entries if power boosting is not used in the following table.

| Company | Y/N | Comments and preferences |
| --- | --- | --- |

Issue#3-3: General considerations on extension of pi/2-BPSK to MCS entries

Companies’ views on Issue#3-3 can be found as follows.

| Company | Opinions |
| --- | --- |
| vivo | Proposal 13: pi/2 BPSK extension should be able to be enabled or disabled via separate RRC signaling in PUSCH-Config, which is only needed for PC3 UEs. |
| ZTE, Sanechips | Proposal 15: The extension of pi/2-BPSK to more MCS entries mechanism specified in Rel-20 should focus on PUSCH transmission of UEs in RRC connected state. Proposal 16: A clarification on the target date rate and performance gain should be made before making any specific decision on extending pi/2-BPSK to more MCS entries. |
| Tejas | Proposal 8: Study of the impact of introducing additional MCS entries on spectral efficiency and overall network performance. The study should aim to quantify the associated gain and assess their justification for potential inclusion in the specification. |
| China Telecom | Proposal 15: The MCS entry extended to pi/2-BPSK with maximum MCS index in two tables should share the same spectral efficiency. Proposal 16: When extending the pi/2-BPSK to a specific MCS entry, the target code rate should be modified at the same time to keep the spectral efficiency the same. |
| KT | Proposal 9: An additional RRC parameter to apply additional use of pi-2 BPSK is adopted. |
| Panasonic | Proposal 12: For the indication of the usage of extended MCS entries for pi/2-BPSK, the following approaches should be discussed. - Approach 1: Semi-static indication - Higher layer parameter tp-pi2BPSK-r20 is introduced. - The usage of extended MCS entries for pi/2-BPSK is based on tp-pi2BPSK-r20. - Approach 2: Dynamic indication - The usage of extended MCS entries for pi/2-BPSK is based on RNTI for CRC scrambling PDCCH scheduling corresponding PUSCH. |
| Offino | One issue is the scope of MCS entries. Since π/2-BPSK is a QPSK-based modulation, the natural candidates for extension are QPSK MCS entries with low coding rates. Proposal 8. It is proposed that RAN1 discusses the scope of MCS entries for π/2-BPSK extension. Proposal 9. It is proposed that RAN1 discusses signaling options for π/2-BPSK extension. |
| ETRI | Proposal 13: Re-interpret the current MCS index table by introducing more π/2-BPSK indices among QPSK indices. Proposal 14: Enhanced MCS index table can be applied to RRC idle mode as well. |
| Lenovo | Proposal 9：For a new MCS table could be determined based on the existing MCS table by configuring a number of entries supporting pi/2 BPSK. Proposal 10：UE should determine to use the legacy MCS table or the new MCS table with more entries supporting pi/2 BPSK for transmission based on gNB’s indication. |
| Apple | Proposal 9: The same spectral efficiency is kept for both Pi/2 BPSK and QPSK in the extending pi/2 BPSK entries. Proposal 10: Introduce a new RRC parameter to indicate the extending MCS entries for pi/2 BPSK modulation with transform precoding. The parameter for extending pi/2 BPSK entries is present only if tp-pi2BPSK is enabled. |
| Qualcomm | Proposal 16: The network configures the UE with the maximum MCS index for which Pi/2-BPSK is enabled |
| NTT DOCOMO | Proposal 10: For extension of pi/2-BPSK to more MCS entries, - Support a new RRC parameter to switch between pi/2-BPSK and other existing modulation scheme(s) Only one UE capability signalling (feature group) is necessary |

FL Observations:

Companies (vivo, KT, Panasonic, Offino, Lenovo, Apple, Qualcomm, NTT DOCOMO) propose that a separate RRC parameter should be introduced to indicate UE using the MCS with extended modulation scheme. Based on the opinions, the following proposal is provided for discussion.

FL’s Proposal 3-3a:

Introduce a new RRC parameter to indicate UE the enable of extending pi/2-BPSK to more MCS entries.

Companies are welcomed to provide your comments on FL’s Proposal 3-3a in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

FL Observations:

Besides, according to the examples given in contributions, and as proposed by companies (Spreadtrum, UNISOC, China Telecom, Apple), when extending pi/2-BPSK to more entries, the code rate should be modified at the same time to keep spectral efficiency the same. Thus, the following proposal is given for discussion.

FL’s Proposal 3-3b:

When extending pi/2-BPSK to more MCS entries, the code rate should be modified to keep spectral efficiency the same.

Companies are welcomed to provide your comments on FL’s Proposal 3-3b in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#3-4: Which MCS entries can be applied with the extension of pi/2-BPSK

Companies’ views on Issue#3-4 can be found as follows.

| Company | Proposals |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 11: Pi/2 BPSK is extended to MCS2/3/4/5 entries instead of QPSK with the same spectrum efficiency |
| vivo | Proposal 11: Support pi/2 BPSK for MCS indexes 2 to 5 in MCS Table 6.1.4.1-1, and MCS indexes 6 to 11 in Table 6.1.4.1-2 defined in 3GPP TS38.213. |
| Huawei, Hisilicon | Observation 12: Extending the MCS index value to support the maximum SE of 0.6016 for non-power boost UEs and the maximum SE of 0.877 for power boost UEs achieves the required UL data rate as indicated in the WID, i.e., > 5 Mbps. Proposal 17: Extend the MCS entries for pi/2-BPSK in Table 6.1.4.1-1 and Table 6.1.4.1-2 in TS 38.214 to support the maximum achievable SE = 0.8770 up to IMCS = 6 and IMCS = 12, respectively. |
| Nokia | Proposal 9. RAN1 to extend pi/2-BPSK at least to MCS2 and MCS3. Whether to extend to other MCS entries should depend on results obtained for larger number of Rx antennas. |
| China Telecom | Proposal 17: The maximum MCS index of MCS entry with pi/2 BPSK extended to in Table-1 and Table-2 is 6 and 12 respectively. |
| KT | Proposal 8: The spectral efficiency is preserved from the change of modulation order, and the resulting code rate not exceeds 1. Proposal 10: The range of MCS index to additionally adopt pi-2 BPSK is IMCS 2-4 for Table 6.1.4.1-1 and IMCS 6-10 for Table 6.1.4.1-2. |
| Panasonic | Proposal 10: Support to extend pi/2-BPSK to MCS indexes 2, 3, and 4 in such as Table 6.1.4.1-1, TS 38.214. - Extending pi/2-BPSK to MCS index 5 can also be considered. |
| Qualcomm | Observation 3: When using the MCS Table 6.1.4.1-1, MCS entries higher than MCS 6 are not applicable for pi/2 BPSK. Proposal 15: Rely on UE capability indication to determine the range of MCS entries that are applicable for pi/2 BPSK. Consider one of the following two options: - Option 1: A support of Pi/2-BPSK modulation for higher MCS values with a UE recommendation of the maximum MCS values. - Option 2: A support of Pi/2-BPSK modulation up to a certain MCS value |
| Ericsson | Proposal 24: Discuss whether to support pi/2-BPSK for MCS indices 2-6, or only a subset of these indices, as specified in Table 6.1.4.1-1 of TS 38.214. |

FL Observations:

As pointed by companies (KT, China Telecom, Ericsson, Qualcomm), the code rate shouldn’t be larger than 1, thus the maximum index of MCS entries pi/2-BPSK can be extended to is 6 for 64QAM table, and 12 for lowSE table if supported.

Companies’ view on which MCS entries pi/2-BPSK can be extended to are summarized as follows:

For 64QAM table, if power boosting is applied, companies providing preference (Spreadtrum, UNISOC, vivo, Huawei, Hisilicon, KT, Panasonic, Nokia) agree that indexes 2-5 of MCS entries can be applied; otherwise, only indexes 2/3 of MCS entries can be applied. And whether to support MCS entries with index 6 is controversial since the performance gain is relative limited.

For 64QAM with lowSE table, if supported, companies who provide preference (vivo, Huawei, Hisilicon, KT) agree that MCS entries with index 6-10 can be supported, whether to support MCS entries with index 11 and 12 is controversial since the performance gain is relative limited.

From FL’s point of view, when decide which MCS can apply pi/2-BPSK, the power boosting should be taken into consideration since it is the main source where performance derived from. Thus, the following proposal is provided according to companies’ contributions.

FL’s Proposal 3-4:

For extending pi/2 BPSK to more MCS entries:

- Pi/2-BPSK is extended to MCS indexes 2 to X(5<=X<=6) when MCS Table 6.1.4.1-1 is applied.

- FFS: value of X

- Pi/2-BPSK is extended to MCS indexes 6 to Y(10<=Y<=12) when MCS Table 6.1.4.1-2 is applied, if supported

  - FFS: value of Y

Companies are welcomed to provide your comments on FL’s Proposal 3-4 in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#3-5: LLS methodology and results

Companies’ views on Issue#3-5 can be found as follows.

| Company | Proposals |
| --- | --- |
| ZTE, Sanechips | Proposal 17: Define key simulation assumptions (such as RB numbers, channel types, and target data rate) to evaluate the performance of extending pi/2-BPSK to more MCS entries. |
| vivo | Observation 18: With the assumption of 2.8dB power boosting gain, the overall coverage gain of pi/2 BPSK over QPSK can be achieved when SE is less than 0.877. Observation 19: If different power boosting gains are considered, the overall coverage gain of pi/2 BPSK over QPSK can be obtained at different SE ranges, i.e. different MCS indexes. Figure 1. pi/2BPSK gain over QPSK with different PRB allocations Figure 2. pi/2BPSK gain over QPSK with different power boosting |
| Huawei, Hisilicon | Figure 1: BLER evaluation for pi/2-BPSK and QPSK modulation with the same SE (double code rate for pi/2-BSPK) |
| Nokia | Table 1. Net gain results for a PUSCH according to the considered simulation assumptions Net gain; RB4; RB30; RB150. MCS2; 0.39; 0.4; 0.73. MCS3; 0.08; 0; 0.22. MCS4; -0.23; -0.43; -0.52. MCS5; -0.88; -1.19; -1.14. MCS6; -2.07; -2.51; -2.73. |
| CATT | Figure 2: The performance of pi/2-BPSK and QPSK Observation 3: For the MCS index in the MCS table without low SE, the PUSCH transmission with pi/2 BPSK has a performance loss compared with the PUSCH transmission with QPSK. As the MCS index increases, the performance loss of pi/2-BPSK compared with QPSK gradually increases. - For the MCS index 5, the performance loss is about 2 dB; - For the MCS index 6, the performance loss is about 3.1 dB. Proposal 9：For the MCS table without low SE, the MCS index of pi/2-BPSK could be extended to the index up to 5. - FFS：Whether to extend pi/2-BPSK to more MCS indexes for the MCS table with low SE |
| Ericsson |  |

FL Observations:

The simulation provided by companies are summarized in above table, according to which MCS entries can be extended with pi/2-BPSK are recommended. It is common understanding that the total performance gain including the net performance gain/loss and power gain due to lower RSRP (w/ or w/o power boosting). Though different evaluation methodologies are used, according to the results, for indexes smaller than 5 in Table 6.1.4.1-1, the total performance gain are obvious, especially when power boosting is adopted. However, FL think that more simulation results can be more essential to decide whether the pi/2-BPSK can be extended to MCS 6 in Table 6.1.4.1-1. Thus, the following proposal is provided for discussion.

FL’s Proposal 3-5:

LLS should be conducted to decide which MCS entries pi/2-BPSK can be extended to.

- Both net performance gain and power gain due to lower RSRP should be taken into consideration.

- Note: net performance gain refers to the performance gain/loss when using pi/2-BPSK compared with QPSK; power gain refers to the performance acquired due to the low PAPR of pi/2-BPSK compared with QPSK.

Companies are welcomed to provide your comments on FL’s Proposal 3-5 and opinions on which simulation methodology/assumptions should be aligned in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |

Issue#3-6: Other issues

Companies are welcomed to provide your views on the other issues that you think are essential but not mentioned in the above issues in the following table.

| Company | Comments |
| --- | --- |

- Online proposals

The following proposals are going to be discussed in Monday’s online session.

The following proposals are going to be discussed in Wednesday’s online session.

- Agreements

The following agreements were reached in Monday’s online session.

The following agreements were reached in Wednesday’s online session.

- Reference

- 3GPP RP-251862 New WID: Coverage enhancements for NR Phase 3, Telecom Italia, RAN #108, Prague, Czech, Jun. 2025.

- 3GPP RP-252824 New WID: Coverage enhancements for NR Phase 3, China Telecom, RAN #109, Beijing, China, Sept. 2025.

- R1-2506810, Discussion on NR Coverage enhancement Phase 3, Spreadtrum, UNISOC, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506894, Discussions on NR phase 3 coverage enhancements, vivo, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506916, Discussion on coverage enhancement, ZTE Corporation, Sanechips, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506947, Coverage enhancements for NR Phase 3	Huawei, HiSilicon, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506985, Discussion on coverage enhancement, Xiaomi, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507055, NR coverage enhancement Phase 3, Tejas Network Limited, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507056, Coverage enhancements for NR Phase 3, Nokia, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507115, Discussion on coverage enhancement, CATT, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507172, Discussion on further enhancement for coverage enhancement, OPPO, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507187, Discussion on coverage enhancement, LG Electronics, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507191, Discussion on coverage enhancement, Transsion Holdings, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507216, Coverage enhancement for NR Phase 3, InterDigital, Inc. , RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507249, Discussion on Coverage Enhancement, Samsung, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507289, Discussions on the Rel. 20 coverage enhancement, KT Corp. , RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507329, Discussion on NR coverage enhancements Phase 3	China Telecom, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507375, Discussion on 5G NR coverage enhancement phase 3, Panasonic, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507432, Discussion on coverage enhancement, TCL, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507464, Views on Coverage Enhancement Phase 3, Ofinno, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507502, Discussion on coverage enhancements, ETRI, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507530, Coverage enhancements, Lenovo, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507554, Discussion on coverage enhancement, DENSO CORPORATION, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507673, Discussion on Coverage enhancements for NR Phase 3, Apple, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507717, Coverage enhancement Phase 3, Qualcomm Incorporated, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507783, Discussion on Rel-20 Coverage Enhancement, Ericsson (China), RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507809, Discussions on coverage enhancement, NTT DOCOMO, INC. , RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507822, Discussions on further coverage enhancement for NR, Sharp, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507867, Discussion on coverage enhancement, ASUSTeK, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507900, Discussion on Coverage Enhancements Phase 3, CEWiT, RAN1 #122bis, Prague, Czech, Oct. 2025