3GPP TSG-RAN WG1 Meeting #122bis	R1-250nnnn

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	11.1

Title:	FL summary#1 on overview of 6GR air interface

Source:	Moderator (NTT DOCOMO)

Document for:	Discussion, Decision

1	Introduction

This document summarizes contributions [3] – [56] submitted to agenda item 11.1 (Overview of 6GR air interface).

The following sections are categorized according to the following guidance provided by RAN1 chair:

High level design proposals/principles/target, including scalable 6GR design (e.g., what design is scalable, what design is unscalable), support of minimum spectrum allocation, coverage, MRSS, synchronization signal structure and periodicity, operation of bandwidth/band adaptation, spectrum utilization and aggregation framework, harmonization of TN and NTN, and others (if any). Note: To avoid distributing proposals of a same topic to different sub-agendas, please organize the proposals according to above highlights.

Note: A number of companies provide views on technical details of the following aspects. As per guidance from RAN1 chair, those aspects will be discussed in separate agenda items and/or future RAN1 meetings:

- This RAN1 meeting

  - Evaluation assumptions for 6GR air interface

    - Discussions on models, scenarios, parameters, and methodology, metrics/criteria, as well as traffic model that can be commonly used for evaluating technology proposals.

  - Waveform

    - Including proposals for improving spectrum efficiency, power efficiency, coexistence and coverage, etc.

  - Frame structure

    - Including numerology and frame structure (for all duplex types).

  - Channel coding

    - Including metrics/criteria that can be used for evaluating technology proposals and for down selecting proposals

  - Modulation, joint channel coding and modulation

    - Including metrics/criteria that can be used for evaluating technology proposals and for down selecting proposals

  - Energy efficiency

    - Including evaluation assumptions, proposals for Evaluations, NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving, targeting to categorize proposals by RAN1#123. From RAN1#124, proposals will be distributed to respective related agenda.

  - AI/ML in 6GR interface

    - Collecting AI/ML use cases in all potential components in physical layer design, targeting to select some use cases by RAN1#123. From RAN1#124, selected use cases will be distributed to respective related agenda.

- Future RAN1 meetings

  - Initial access

    - Placeholder only and to be broken down. No contributions before RAN1#124. Including synchronization signal and raster, broadcast signals/channel and physical random access channel, etc.

  - MIMO operation

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - Physical layer control, data scheduling and HARQ operation

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - Duplexing

    - Placeholder only and to be broken down or adapted based on the discussion in AI 11.1. No contributions before RAN1#124.

  - 6GR spectrum utilization and aggregation

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - NTN

    - Placeholder only and to be broken down or adapted based on the discussion in AI 11.1. No contributions before RAN1#124.

  - Other physical layer signals, channels and procedures

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - Sensing

    - Including PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…), aspects of integration with communication services.

    - Placeholder only and to be broken down. No contributions before RAN1#124b.

Similarly, a number of companies provide views on 6G RAN requirements, which is subject to the progress in RANp study for 6G RAN requirements.

2	Proposals for Online Sessions

2.1	Proposals for Monday Online

To be updated

3 Scalable 6GR design

At the RAN1#122 meeting, following agreement was made related to scalable 6GR design and diverse device types:

Agreement​ Study a scalable 6GR design for diverse device types, considering aspects:​ - What should be commonly applicable to all 6G device types​ - FFS: add-on features dedicated to specific device types, if any​

Then, at the RAN#109 meeting, “diverse device types” were discussed and following proposals were agreed. It is concluded that those proposals will be further discussed in RAN while no WG discussion on them.

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type proposal 3 & 4 are endorsed for RAN only (no WG discussion)

Regarding “What should be commonly applicable to all 6G device types​”, according to companies input, there are in general two approaches to consider this aspect as follows:

- Approach 1: Minimum common features which are required to all 6G device types.

  - This approach may be one of the backgrounds of the situation where some companies are proposing “device type” and corresponding features/capabilities in early stage

  - The minimum common features can include, but not limited to

    - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band

    - Idle mode prucedures

    - Initial access prucedures and mobility

    - DL/UL control

    - Basic scheduling/HARQ

    - Basic MIMO

    - Duplexing

    - MRSS

    - 1 TRX chain, smallest maximum supported RF and BB UE BW

- Approach 2: Everything is commonly applicable by default, i.e., maximize applicability of features to all 6G device types beyond Approach 1

  - This approach assumes the discussions on such commonly applicable essential features should be prioritized over “device type” definition discussion

  - The commonly applicable essential features can include, but not limited to

    - The features listed in Approach 1

    - BS/UE EE features

    - Coverage enhancements

    - Data collection

    - Spectrum aggregation

Regarding “FFS: add-on features dedicated to specific device types, if any”, this may require discussion about device type definition to some extent, which should be avoided in RAN1. Some companies assume this can be handled by UE capability signaling while some others propose to discuss how to maximize the applicability of attractive sub-features that have some restriction/difficulty specific to some device type(s) and so on.

Proposal 3.1:

- For scalable 6GR design for diverse device types, RAN1 to consider

  - Approach 1: Minimum common features which are required to all 6G device types

  - Approach 2: Every feature is commonly applicable by default, i.e., maximize applicability of features to all 6G device types

- The features commonly applicable to all 6G device types include, but not limited to

  - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band

  - Idle mode prucedures

  - Initial access prucedures and mobility

  - DL/UL control

  - Basic scheduling/HARQ

  - Basic MIMO

  - MRSS

  - 1 TRX chain, smallest maximum supported RF and BB UE BW

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This issue is controversial and would require some time for mutual understanding among companies 2nd main bullet is candidate minimum set of features applicable to both Approach 1 and 2, for further discussion |

Also, some companies discuss how to improve UE capabilities, including

- Prerequisites can be used to prevent undesired capability combinations

- Define set of mandatory/optional capabilities per use cases / device types, e.g., modular capability

  - Beneficial features for BS and/or UE

- modular and MAC-layer-assisted UE capability and assistance information reporting framework

This can be discussed in later stage of SI or even WI after overall 6GR features become clear.

4 Support of minimum spectrum allocation

At the RAN1#122 meeting, minimum spectrum allocation and smallest maximum supported RF and BB UE BW were discussed and following agreements were made.

Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Then, at the RAN#109 meeting, the above issues were discussed due to the condition of “subject to further discussion and confirmation in RAN” and following proposals were agreed. It is concluded that those proposals will be further discussed in RAN1 while RAN#110 revisit.

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Based on the situation, RAN1 chair provided guidance that RAN1 will focus on how to support the minimum spectrum allocation, especially the case where the minimum spectrum and the smallest maximum UE bandwidth can’t match, considering that it is hard to purely discuss these two elements in RAN1.

Note that following is captured in TR38.914 related to lowest-tier device

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

Although RAN1 chair guidance suggests that RAN1 will not purely discuss the values for the minimum spectrum allocation or smallest maximum UE BW, quite a few companies provide the views especially on the smallest maximum UE BW, for which majority support more than 3MHz BW even for 15kHz SCS, while there are some companies support 3MHz BW for providing services to the low tier market segment, reduce the UE power consumption, SAW-less design for UL, and migration from LTE LPWA. Also, there would be common understanding that it is necessary to consider the tradeoff between device complexity reduction and MBB performance impact when discussing the smallest maximum UE BW. It was also mentioned by companies that the smallest maximum UE BW should be equal to or larger than the common signals/channels BW applicable to all device types for the scalable 6GR design. Hence, following proposal can be considered for further discussion.

Proposal 4.1:

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - tradeoff between device complexity reduction and MBB performance impact

  - common signals/channels BW applicable to all device types

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | As per the guidance from RAN1 chair, RAN1 will not purely discuss the smallest maximum UE BW value. This proposal can be used as starting point to further discuss feasible value from RAN1 perspective |

Regarding the minimum spectrum allocation, some companies mention that RAN1 may not make much progress without considering exact values, which will be discussed in RANp. For now, what RAN1 can discuss is how to handle the case when the minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types, since the smallest maximum UE BW does not impact on the operation on the minimum spectrum allocation as long as UE is capable to operate on the minimum spectrum allocation (e.g. max 20 MHz BW UE can be operate on 5MHz CBW). A number of companies assume similar handling as NR Rel-18 less than 5MHz BW, i.e., puncturing the common signals/channels to fit into the minimum spectrum allocation w/ some performance degradation. Also, some companies consider special handling of such minimum spectrum allocation, e.g., specific design of the common signals/channels (e.g., SSB) for the minimum spectrum allocation.

Proposal 4.2:

- When the minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types (if any), RAN1 to consider following to operate 6GR on the minimum spectrum allocation

  - Opt1: common signals/channels BW are punctured to fit into the minimum spectrum allocation

  - Opt2: specific design of the common signals/channels for the minimum spectrum allocation

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | As per the guidance from RAN1 chair, RAN1 will not purely discuss the minimum spectrum allocation value. This proposal can be used as starting point to further discuss how to handle the case when special handling is necessary for the minimum spectrum allocation value from RAN1 perspective |

5 Overall coverage

At the last RAN1 meeting, overall coverage for 6GR was discussed and the following agreement was made:

Agreement - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

In addition, RAN#109 concluded the following:

Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel

Quite a few companies provide the views on coverage target for 6GR, including not only + 5 to 10 dB enhancement from normal coverage (144dB MCL) for cell-edge performance but also overall DL/UL performance improvements in anywhere within the cell coverage. However, as clarified by RAN1 chair over RAN1 reflector, RAN1 will not discuss coverage target(s) to avoid duplicated work with RAN plenary.

Related to the SID objective “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, some companies mentioned following aspects, which can be discussed in other agenda or WGs

- More antenna elements for BS and/or UE

  - This aspect can be discussed in RANp SI for 6G requirement (especially for deployment scenarios) as well as RAN1 6G study AI11.2 for evaluation assumptions

- More number of TRX

  - This aspect can be discussed in RAN1 6G study AI11.2 for evaluation assumptions

- Incresed UE Tx power

  - Should be led by RAN4. Early RAN4 involvement is necessary

Due to the lack of clear coverage target(s), companies have divergent views which channels need to be improved, and how to do it, including but not limited to

- Which channels need to be improved

  - Ensure targeted coverage for all channels and signals

  - Channels/signals during initial access

  - Mainly for UL coverage

- How to improve coverage

  - Repetitions

    - Including unified solution among different channels

  - Available Slot Counting (ASC)

  - DMRS bundling/Joint Channel Estimation (JCE)

  - TBoMS

  - Cross-slot Tx, including PUSCH and RS

  - DL/UL decoupling (discussed in Section 9)

Some companies also mention this requires early RAN4 involvement. Note that there are some other proposals for coverage enhancements, including waveform, modulation, duplex and so on, which should be discussed in the corresponding agendas.

Give the situation, moderator assume it’s premature to discuss any solutions for coverage enhancement without clear coverage target(s). Following proposal can be considered similar to other discussion points

Proposal 5.1:

- Study and identify the lessons learned from NR coverage enhancement features

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Potential discussion topics are to identify lessons learned from NR CovEnh features, so that better CovEnh features will be considered from 6G Day1 |

6 MRSS

At the last RAN1 meeting, MRSS aspect was discussed and the following agreement was made:

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

Not only the frame structure as stated in the SID, a number of companies mention that 5G compatible design should be ensured for MRSS, including waveform, modulation, numerology, channel coding, duplexing, and so on. All these aspects are discussed in other AIs in RAN1, and hence, moderator suggests considering MRSS aspects when discussing the above topics in other AIs 11.3.1, 11.3.2, 11.4.1, 11.4.2.

Companies provide lessons learned from LTE-NR DSS, including but not limited to

- legacy and practical restrictions due to “always-on” signals like LTE CRS

  - Caused overhead and reduced NR PDCCH capacity

  - But already removed from NR

- The maximum number of rate-matching patterns of PDSCH

  - too limited and thus costs inefficient inter-RAT resource sharing

- The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI

  - costs inefficient inter-RAT resource sharing

- Rate-matching patterns in the first release of NR

  - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell

- overall overhead from operating both RATs on the same carrier

  - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated

- SDM was not considered

  - SDM between 5G and 6G users would allow maximum flexibility for resource allocation

- Interoperability issues between different vendors

  - timing mismatches may cause signal collisions, reduced throughput.

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

Proposed observation 6.1:

- The lessons learned from LTE-NR DSS include, but not limited to

  - legacy and practical restrictions due to “always-on” signals like LTE CRS

    - Caused overhead and reduced NR PDCCH capacity

    - But already removed from NR

  - The maximum number of rate-matching patterns of PDSCH

    - too limited and thus costs inefficient inter-RAT resource sharing

  - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI

    - costs inefficient inter-RAT resource sharing

  - Rate-matching patterns in the first release of NR

    - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell

  - overall overhead from operating both RATs on the same carrier

    - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated

  - SDM was not considered

    - Limited flexibility for resource allocation

  - Interoperability issues between different vendors

    - timing mismatches may cause signal collisions, reduced throughput.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

Regarding the high-level aspects which impact on the NR-6GR MRSS support, following views are provided

- General

  - UE/NW implementation complexity

  - Scheduler coordination

    - Including Multi-vendor (e.g., Inter-DU) operation

  - Traffic pattern

  - Radio resource utilization

    - Including PDCCH capacity

  - No impact on legacy NR UE behavior

  - MRSS should not limit 6GR design, and can be postponed after basic 6GR design is defined

- Operating bands/carriers

  - unified MRSS migration technique across all the bands

  - Single shared carrier MRSS, MRSS + 6G-only multicarrier aggregation, UL-only on MRSS with DL on 6G-only carrier

- Resource split/sharing

  - Study 6GR resource allocation to flexibly utilize resources not occupied by 5G‑NR in an MRSS carrier

    - Including slot and mini-slot based scheduling

  - Opt0: Semi-static TDM/FDM

    - Also for NB-IoT and eMTC

  - Opt1: Signal sharing

    - Pros

      - Reduced resource overhead, including SSB, CORESET

      - Enhancing 6G UE performance by leveraging 5G reference signals received by the UE

    - Cons

      - Limit 6GR signal design, including EE and coverage

      - Complicate UE implementation

  - Opt2: Rate-matching

    - Pros:

      - Similar to LTE-NR DSS

    - Cons

      - (Not identified from contributions)

  - Opt3: SDM

    - Pros

      - SDM between 5G and 6G users would allow maximum flexibility for resource allocation

    - Cons

      - For cross-RAT SDM (assuming same overhead for 5G and 6G DMRS and only time/frequency multiplexing between DMRSs), both 5G and 6G suffer approximately 14% overhead increase

According to the input, following proposals can be considered as starting point

Proposal 6.2:

- High-level aspects which impact on the NR-6GR MRSS support include, but not limited to

  - UE/NW implementation complexity

  - Scheduler coordination

  - Traffic pattern

  - Radio resource utilization

  - No impact on legacy NR UE behavior

  - Operating bands/carriers

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

[Low]Proposal 6.3:

- For the radio resource utilization for NR-6GR MRSS support, RAN1 to study the Pros/Cons of the following options

  - Opt0: Semi-static TDM/FDM between NR and 6GR

  - Opt1: NR signal sharing with 6GR

  - Opt2: Rate matching of 6GR signals/channels around NR signals/channels

  - Opt3: SDM between NR and 6GR

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be discussed as second priority, since the highest priority in this meeting is to identify the high-level aspects which impact on the NR-6GR MRSS support, as agreed in the last RAN1 meeting |

7 Synchronization signal structure and periodicity

At the last RAN1 meeting, SS structure and periodicy aspects were discussed and the following agreement was made:

Agreement - Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Initial access aspects are planned to be discussed from RAN1#124 (Feb. 2026). Therefore, in general, it is better to wait for RAN1#124 to open the discussion on initial access aspects. However, the Interim Milestone states that RAN1 needs to provide interim assessment on the basic sync signal structure and associated periodicity(ies) by June 2026. Therefore, at least on these aspects, RAN1 should start discussion earlier to provide enough assessment.

Regarding the SS structure, a number of companies mentioned that it needs to consider

- Reduced number of sync raster: for NES and UE complexity

- Support of low-tier 6G device: for smallest maximum supported UE BW

- Support of minimum spectrum allocation: punctured SS vs specific design for the spectrum as discussed in Section 4

- Detection performance: If narrower SSB BW is considered, more OFDM symbols would be required to maintain the NR performance

- Ensure orthogonalization against the NR PSS/SSS design: to avoid UE accessing unintended RAT

- Extended coverage: unclear coverage target as discussed in Section 5

- Low complexity/power SS

- decoupling for different RRC states

- multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand)

- NTN aspects (to be discussed in Section 10)

- Scalability to operate on the supported deployments and spectrum, including multi-beam operation

- Compatibility with any duplex modes, e.g., SBFD

Regarding the SS periodicity, quite a few companies propose larger periodicity for NES while considering how to address UE complexity (initial cell search, PSS/SSS detection requirements, access latency). Moderator assumes this aspect can be discussed in AI 11.6:EE since no other aspects were raised for the periodicity

Proposal 7.1:

- High-level aspects which impact on the 6GR sync signal structure include, but not limited to

  - Reduced number of sync raster

  - Support of low-tier 6G device

  - Support of minimum spectrum allocation

  - Detection performance

  - Ensure orthogonalization against the NR PSS/SSS design

  - Extended coverage

  - Low complexity/power SS

  - decoupling for different RRC states

  - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand)

  - Scalability to operate on the supported deployments and spectrum, including multi-beam operation

  - Compatibility with any duplex modes

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

8 Operation of bandwidth/band adaptation

At the last RAN1 meeting, operation of bandwidth/band adaptation was discussed and the following agreement was made:

Agreement - Study and identify the lessons learned from NR BWP framework

Companies provide lessons learned from NR BWP framework, including but not limited to

- A lot of potential uses, including adaptation to traffic demands and energy savings

- A lot of RRC parameters under BWP configuration

  - results in unnecessarily large overhead

- BWP switching delay

  - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides

  - UPT loss and increased UE power consumption

- BWP switching

  - less motivated, for other than CORESET switching

  - will cause misalignment of real active BWP between BS and UE

  - results in unnecessary HARQ-ACK dropping

- SCS switching

  - complicated but less motivated.

- Excessive BWP types

  - including BWP types that have not been effectively used in practical NW, e.g., default BWP, dormant BWP.

- Center frequency of DL/UL BWP

  - unnecessarily common

- lack of RAN4 involvemen

  - leading to large MPR/A-MPR

- Inherent restrictions

  - When a BWP is not covering the sync signal bandwidth, it can lead to different approaches for maintaining sync

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

Proposed observation 8.1:

- The lessons learned from NR BWP framework include, but not limited to

  - A lot of potential uses, including adaptation to traffic demands and energy savings

  - A lot of RRC parameters under BWP configuration

    - results in unnecessarily large overhead

  - BWP switching delay

    - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides

    - UPT loss and increased UE power consumption

  - BWP switching

    - less motivated, for other than CORESET switching

    - will cause misalignment of real active BWP between BS and UE

    - results in unnecessary HARQ-ACK dropping

  - SCS switching

    - complicated but less motivated.

  - Excessive BWP types

    - including BWP types that have not been effectively used in practical NW, e.g., default BWP, dormant BWP.

  - Center frequency of DL/UL BWP

    - unnecessarily common

  - lack of RAN4 involvemen

    - leading to large MPR/A-MPR

  - Inherent restrictions

    - When a BWP is not covering the sync signal bandwidth, it can lead to different approaches for maintaining sync

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

Accroding to the lessons learned from NR BWP framework, companies further propose how to improve BWP framework in 6GR, including but not limited to

- Support simplified BWP framework

  - Only essential/relevant configurations under BWP configurations

  - Single SCS per BWP

  - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP

  - No dynamic BWP switching

  - Minimize the number of BWP types

  - in conjunction with other functionalities related to UE power savings

- Separate DL and UL BWP adaptation

- Inprove robustness, reduced latency and minimize interrupptions

- Target early RAN4 involvement

- Design BWP to support diverse device types in the same band during initial access

- discontinuous frequency resources within one BWP

- improving the performance when BWP location does not coincide with the synchronisation signal frequency

- Combined with TCI framework

- Reduced UE energy consumption

[Low]Proposal 8.2:

- Study how to improve BWP framework, including but not limited to

  - Simplified BWP framework

    - Only essential/relevant configurations under BWP configurations

    - Single SCS per BWP

    - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP

    - No dynamic BWP switching

    - Minimize the number of BWP types

    - in conjunction with other functionalities related to UE power savings

  - Separate DL and UL BWP adaptation

  - Inprove robustness, reduced latency and minimize interrupptions

  - Target early RAN4 involvement

  - Design BWP to support diverse device types in the same band during initial access

  - discontinuous frequency resources within one BWP

  - improving the performance when BWP location does not coincide with the synchronisation signal frequency

  - Combined with TCI framework

  - Reduced UE energy consumption

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be discussed as second priority, since the highest priority in this meeting is to identify the lessons learned from NR BWP framework, as agreed in the last RAN1 meeting |

9 Spectrum utilization and aggregation framework

At the last RAN1 meeting, spectrum utilization and aggregation framework were discussed and the following agreement was made:

Agreement - Study and identify the lessons learned from NR spectrum utilization and aggregation framework - DC is subject to RANP decision in June 2026 - Note: MRSS aspects are separate discussion

In addition, RAN#109 concluded the following:

Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. Proposal 2: 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

Note that following is captured in TR38.914 related to spectrum aggregation

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

Although the technical details on the above aspects can be further discussed in other agenda items to be discussed in RAN1 (e.g., “Initial access”, “Physical layer control, data scheduling and HARQ operation”, and “6GR spectrum utilization and aggregation”), it wold be better to discuss some high-level direction on how to improve the spectrum utilization and operations in this agenda items, because this issue has impact on multiple agenda items.

Companies provide lessons learned from NR spectrum utilization and aggregation framework, including but not limited to

- CA has been a very successful feature in LTE and NR

- Pcell vs Scell

  - Allowing some functionalities only on specific cell like PCell may limit resource utilizations and prevent a NW from entering deep sleep as early as possible on a cell

- Coupling DL and UL carriers for a cell

  - inefficient and ineffective due to different requirements and limitations between DL and UL

  - SUL/SDL, UL Tx switching, LBCA switching operate differently

  - SUL scheme is bound to dedicated SUL bands with UL-only resource

  - ensuring the presence of a corresponding downlink CC used as a reference for measurements

- UL Tx switching

  - did not incorporate all UL transmissions, complicating its use

  - mandates UE to support at least N DL CCs and the N DL CCs are activated, which leads to high DL capabilities requirement and high UE power consumption

- CA applicability

  - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times

  - did not sufficiently facilitate wide variety of deployments and network implementations but was designed to require challenging low latency inter-cell coordination

- SSB adaptation for Scell

  - SSB-less SCell operation

    - limited applicable scenario.

  - On-demand SSB SCell operation

    - limited applicable scenario.

- Activation of additional carrier

  - Slow not only because of signaling protocols and RAN4 requirements, but also because of very relaxed CSI accuracy for the newly activated carrier

  - faces a dilemma of choosing the high service latency caused by SCell activation and high UE power consumption by keeping SCell always activated

  - SCell dormancy

    - impractical as this feature is defined on top of BWP framework, which is unnecessarily flexible and complicated.

  - A-TRS trigger with SCell activation

    - not designed for NES.

- Features (such as HARQ) defined per carrier

  - prevents further improvements on user throughput and latency via cross-carrier operation

  - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving

- Avoid dependencies across carriers

  - such as DAI to simplify implementation and improve performance

- The maximum number of bands in NR multi-band operations

  - actually limited by the maximum UE RF+BB hardware capacity in commercial networks

- Concurrent transmissions of UL-CA/EN-DC

  - only beneficial for UEs who are close to gNB and have redundant UE Tx power and its symbol-by-symbol UL power control requires very tight coordination between PCell gNB and SCell gNBs.

  - need to require a semi-static UL power split for the UE in absence of gNB scheduler coordination.

  - Only supported for connected mode

- Fragmented spectrum

  - not efficiently utilized and latency is unnecessarily increased under NR CA framework

- Signalling overhead and UE processing complexity of PHY channels

  - scale with the number of aggregated carriers rather than the aggregated bandwidth size

- No support of efficient IDLE/INACTIVE modes offloading

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

Proposed observation 9.1:

- The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to

  - CA has been a very successful feature in LTE and NR

  - Pcell vs Scell

    - Allowing some functionalities only on specific cell like PCell may limit resource utilizations and prevent a NW from entering deep sleep as early as possible on a cell

  - Coupling DL and UL carriers for a cell

    - inefficient and ineffective due to different requirements and limitations between DL and UL

    - SUL/SDL, UL Tx switching, LBCA switching operate differently

    - SUL scheme is bound to dedicated SUL bands with UL-only resource

    - ensuring the presence of a corresponding downlink CC used as a reference for measurements

  - UL Tx switching

    - did not incorporate all UL transmissions, complicating its use

    - mandates UE to support at least N DL CCs and the N DL CCs are activated, which leads to high DL capabilities requirement and high UE power consumption

  - CA applicability

    - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times

    - did not sufficiently facilitate wide variety of deployments and network implementations but was designed to require challenging low latency inter-cell coordination

  - SSB adaptation for Scell

    - SSB-less SCell operation

      - limited applicable scenario.

    - On-demand SSB SCell operation

      - limited applicable scenario.

  - Activation of additional carrier

    - Slow not only because of signaling protocols and RAN4 requirements, but also because of very relaxed CSI accuracy for the newly activated carrier

    - faces a dilemma of choosing the high service latency caused by SCell activation and high UE power consumption by keeping SCell always activated

    - SCell dormancy

      - impractical as this feature is defined on top of BWP framework, which is unnecessarily flexible and complicated.

    - A-TRS trigger with SCell activation

      - not designed for NES.

  - Features (such as HARQ) defined per carrier

    - prevents further improvements on user throughput and latency via cross-carrier operation

    - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving

  - Avoid dependencies across carriers

    - such as DAI to simplify implementation and improve performance

  - The maximum number of bands in NR multi-band operations

    - actually limited by the maximum UE RF+BB hardware capacity in commercial networks

  - Concurrent transmissions of UL-CA/EN-DC

    - only beneficial for UEs who are close to gNB and have redundant UE Tx power and its symbol-by-symbol UL power control requires very tight coordination between PCell gNB and SCell gNBs.

    - need to require a semi-static UL power split for the UE in absence of gNB scheduler coordination.

    - Only supported for connected mode

  - Fragmented spectrum

    - not efficiently utilized and latency is unnecessarily increased under NR CA framework

  - Signalling overhead and UE processing complexity of PHY channels

    - scale with the number of aggregated carriers rather than the aggregated bandwidth size

  - No support of efficient IDLE/INACTIVE modes offloading

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

Accroding to the lessons learned from NR spectrum utilization and aggregation framework, companies further propose how to improve spectrum utilization and aggregation framework in 6GR, including but not limited to

- Single framework for 6G spectrum utilization

- CA supporting a wide variety of CA deployments

  - Support for loose NW side coordination, including two PUCCH cell groups

- DL/UL decoupling for a cell

- Native/simplified support for UL Tx switching

- Efficient/effective/practical features of carrier ON/OFF

  - carrier without SSB

  - carrier with on-demand SSB

  - fast carrier activation

- Avoid dependencies across carriers

  - Relax and minimize the need for scheduler interaction across cells in case of CA

- Single cell multi-carriers (SCMC)

  - multiple physical carriers are aggregated into a single logical wideband carrier

- enhanced CA power utilization

- efficient RRC configuration mechanism for CA

- Improve the efficiency, implementation cost and scalability of different cross-carrier scheduling schemes

- Native support for both IDLE/INACTIVE and CONNECTED states

[Low]Proposal 9.2:

- Study how to improve spectrum utilization and aggregation framework, including but not limited to

  - Single framework for 6G spectrum utilization

  - CA supporting a wide variety of CA deployments

    - Support for loose NW side coordination, including two PUCCH cell groups

  - DL/UL decoupling for a cell

  - Native/simplified support for UL Tx switching

  - Efficient/effective/practical features of carrier ON/OFF

    - carrier without SSB

    - carrier with on-demand SSB

    - fast carrier activation

  - Avoid dependencies across carriers

    - Relax and minimize the need for scheduler interaction across cells in case of CA

  - Single cell multicarriers (SCMC)

    - multiple physical carriers are aggregated into a single logical wideband carrier

  - enhanced CA power utilization

  - efficient RRC configuration mechanism for CA

  - Improve the efficiency, implementation cost and scalability of different cross-carrier scheduling schemes

  - Native support for both IDLE/INACTIVE and CONNECTED states

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be discussed as second priority, since the highest priority in this meeting is to identify the lessons learned from NR spectrum utilization and aggregation framework, as agreed in the last RAN1 meeting |

10 Harmonization of TN and NTN

At the last RAN1 meeting, Harmonization of TN and NTN was discussed and the following agreement was made:

Agreement - For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

Since the dedicated agenda item on NTN is planned to be started from RAN1#124, technical details can be discussed there. Howerver, for the harmonized 6GR design for TN and NTN, it would be better to identify which technical areas the NTN aspects need to be considered in early stage. In this sense, this agenda discusses to identify the affected technical areas for the harmonized 6GR design for TN and NTN.

Note that the orbit type and payload type will be discussed in RANp study for 6G requirements.

Companies provide lessons learned from NR/IoT NTN, including but not limited to

- NR NTN was introduced at later releases in a “NBC” fashion

  - Legacy UEs not able to connect, requiring extra development efforts

- Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features

- Achievable data rate was kept low, which limits the applicability of NTN use cases

- GEO satellite is hardly supported due to coverage issues

- Low efficient beam hopping, severe UE power wasting

- High dependency on UE GNSS accuracy

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

Proposed observation 10.1:

- The lessons learned from NR/IoT NTN include, but not limited to

  - NR NTN was introduced at later releases in a “NBC” fashion

    - Legacy UEs not able to connect, requiring extra development efforts

  - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features

  - Achievable data rate was kept low, which limits the applicability of NTN use cases

  - GEO satellite is hardly supported due to coverage issues

  - Low efficient beam hopping, severe UE power wasting

  - High dependency on UE GNSS accuracy

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

Regarding the technical aspects affected by NTN characteristics, following views are provided

- harmonization of TN and NTN should not compromise the design of TN or 6G overall

- Cell search / initial access / SSB periodicity

- GNSS-less/resilient operation

- Coverage enhancements

  - shall not consider any 6G NTN-specific coverage enhancements, i.e., commonly designed with TN

  - Paging in body loss/NLOS/satellite-misaligned scenario

  - both the link and system level, including optimization on initial access

  - 100% coverage ratio in a cell with massive beam footprints

- Positioning

- NTN-TN and NTN-NTN mobility

- DC/CA

  - Note: DC is subject to RANp discussion

- Capacity

  - Including OCC multiplexing

- Large propagation delay

  - Including scheduling/HARQ

- Large doppler shift/drift and timing drifting

  - Including timing and frequency synchronization adjustment

- Duplexing

  - Focus on FDD

  - HD-FDD, including collision handling

  - Support TDD

- Beamforming / beam management

  - Dynamic beam management for (V)LEO constellations with massive satellite beams

  - Robust transmit/receive beamforming (digital, hybrid, or analog) method

  - Refined beam management, enabling more accurate DL beam selection for RACH

- TN-NTN in the same spectrum

  - coexistence mechanism for interference mitigation

- 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

- satellite moving

  - RRC configuration adaptation based on the satellite position

According to the input, following proposals can be considered as starting point

Proposal 10.2:

- The technical aspects affected by NTN characteristics include, but not limited to

  - Cell search / initial access / SSB periodicity

  - GNSS-less/resilient operation

  - Coverage enhancements

  - Positioning

  - NTN-TN and NTN-NTN mobility

  - CA

  - Capacity

  - Large propagation delay

  - Large doppler shift/drift and timing drifting

  - Duplexing

  - Beamforming / beam management

  - TN-NTN in the same spectrum

  - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

  - satellite moving

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |

11 Other aspects

Other than the aspects discussed in the above sections or other agenda items (including those planned in future RAN1 meetings), some companies mention the aspects related to PHY security, NW resilience, and so on. It is moderator’s understanding that neither of other aspects can be discussed in RAN1 without any progress in RANp study on 6G requirements. RAN1 cannot discuss any features without justification on the target/motivation, which need to be clarified in RANp study at first. Following is open question to hear companies’ view.

Question 11.1:

- Companies are invited to provide views on whether to discuss any features, other than those (to be) discussed in other sections in this summary or in other agendas in RAN1. If yes, please elaborate which features need to be studied in this section.

| Company | Y/N | Comments |
| --- | --- | --- |

12	Conclusions

Following agreements were made in this meeting:

To be updated

References

| [1] | RP-252912 | Revised SID: Study on 6G Radio | NTT DOCOMO, CMCC, AT&T, Vodafone |
| --- | --- | --- | --- |
| [2] | R1-2507812 | Workplan for Rel-20 Study of 6GR | NTT DOCOMO, China Mobile, AT&T, Vodafone |
| [3] | R1-2506738 | High level views on 6GR air interface | FUTUREWEI |
| [4] | R1-2506750 | Nokia Views on Selected Aspects of 6G Radio Air Interface | Nokia |
| [5] | R1-2506813 | Overview of 6GR air interface | Spreadtrum, UNISOC |
| [6] | R1-2506841 | Overview of the 6G air interface | Ericsson Telecom S.A. de C.V. |
| [7] | R1-2506843 | Overview of the 6G air interface | TCL |
| [8] | R1-2506897 | Overview of 6GR air interface | vivo |
| [9] | R1-2506918 | High-level views on 6GR | ZTE Corporation, Sanechips |
| [10] | R1-2506988 | 6GR air interface design overview | Xiaomi |
| [11] | R1-2507013 | Overview of 6GR air interface | CMCC |
| [12] | R1-2507057 | Overview of 6GR air interface | Huawei, HiSilicon |
| [13] | R1-2507065 | Enhancements for 6G Fixed Wireless Access | T-Mobile USA Inc. |
| [14] | R1-2507104 | Outline and highlight of 6GR air interface | CATT, CICTCI |
| [15] | R1-2507175 | Overview of 6GR air interface | OPPO |
| [16] | R1-2507201 | Overview on 6G Air interface | Tejas Network Limited |
| [17] | R1-2507212 | Discussion on overview of 6GR air interface | HONOR |
| [18] | R1-2507252 | Design of 6GR air interface | Samsung |
| [19] | R1-2507311 | Overview of 6GR air interface | NEC |
| [20] | R1-2507334 | Overview of 6GR air interface | China Telecom |
| [21] | R1-2507343 | Overview of 6GR air interface | THALES, Airbus, ESA, EchoStar, Eutelsat Group, Novamint, TNO, Fraunhofer IIS, Iridium |
| [22] | R1-2507360 | Views on overall design and techniques for 6GR air interface | LG Electronics |
| [23] | R1-2507366 | Overview of 6GR air interface | NVIDIA |
| [24] | R1-2507373 | Overview proposal of 6GR air interface | Panasonic |
| [25] | R1-2507402 | Discussion on overview of 6GR air interface | Fujitsu |
| [26] | R1-2507407 | Overview of 6GR air interface | SK Telecom |
| [27] | R1-2507466 | Discussion on 6GR Air Interface | Ofinno |
| [28] | R1-2507480 | Overview of 6GR air-interface | Lenovo |
| [29] | R1-2507490 | Design consideration of 6GR air interface | Verizon Sweden |
| [30] | R1-2507505 | Overview of the 6GR air interface | ETRI |
| [31] | R1-2507520 | Overview of 6GR Air Interface | Google |
| [32] | R1-2507538 | Overview of 6GR Air Interface | Fraunhofer IIS, Fraunhofer HHI |
| [33] | R1-2507544 | Discussion on the Overview of 6GR Air Interface | Rakuten Mobile, Inc |
| [34] | R1-2507585 | IMU Views on 6G Radio Air Interface | IMU |
| [35] | R1-2507595 | Overview of 6GR air interface | Sony |
| [36] | R1-2507602 | Positioning, Navigation and Timing (PNT) in 6G NTN-TN harmonization | Airbus, ESA, Fraunhofer IIS, Thales, Iridium |
| [37] | R1-2507606 | Overview of 6GR air interface | MediaTek Inc. |
| [38] | R1-2507629 | Discussion on Overview of 6GR air interface | China Unicom |
| [39] | R1-2507676 | Overview of 6GR air interface | Apple |
| [40] | R1-2507720 | Overview of 6GR air interface | Qualcomm Incorporated |
| [41] | R1-2507730 | Views on device types, min channel BW and MRSS | Intel |
| [42] | R1-2507734 | Overview of 6GR air interface | InterDigital, Inc. |
| [43] | R1-2507745 | Lessons Learned from the 5G NR Air Interface Design | AT&T |
| [44] | R1-2507763 | Views on 6GR air interface | Tiami Networks |
| [45] | R1-2507765 | Overview of 6GR air interface | Sharp |
| [46] | R1-2507768 | Views on 6GR air interface | Fainity Innovation |
| [47] | R1-2507814 | Discussion on overview of 6GR air interface | NTT DOCOMO, INC. |
| [48] | R1-2507823 | Views on 6GR sync signal structure | NICT |
| [49] | R1-2507843 | Overview of 6G Radio air interface | ITL |
| [50] | R1-2507846 | Overview of 6G Radio air interface | WILUS Inc. |
| [51] | R1-2507851 | Views on 6GR air interface | CSCN |
| [52] | R1-2507862 | Overview of 6GR air interface | KDDI Corporation |
| [53] | R1-2507879 | General aspects of 6G IoT and NTN | Nordic Semiconductor ASA |
| [54] | R1-2507884 | Operator considerations on performance gains and migration complexity trade-offs in 6G Radio design | BT plc, AT&T, Bouygues Telecom, Deutsche Telekom, Orange, Vodafone |
| [55] | R1-2507938 | On 6GR Frame Structure and Waveform | Boost Mobile Network |
| [56] | R1-2507941 | IIT Kanpur’s views on 6GR air interface | IIT Kanpur |

RAN1 agreements

RAN1#122

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

- Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Agreement

- Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Agreement

- Identify the high-level aspects which impact on the NR-6GR MRSS support

  - Including the lessons learned from LTE-NR DSS

Agreement

- Study and identify the lessons learned from NR BWP framework

Agreement

- Study and identify the lessons learned from NR spectrum utilization and aggregation framework

  - DC is subject to RANP decision in June 2026

  - Note: MRSS aspects are separate discussion

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

Agreement

- For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

RAN1#122bis