3GPP TSG-RAN WG1 Meeting #122bis	R1-250nnnn

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	11.1

Title:	FL summary#3 on overview of 6GR air interface

Source:	Moderator (NTT DOCOMO)

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes contributions [3] – [56] submitted to agenda item 11.1 (Overview of 6GR air interface).

The following sections are categorized according to the following guidance provided by RAN1 chair:

High level design proposals/principles/target, including scalable 6GR design (e.g., what design is scalable, what design is unscalable), support of minimum spectrum allocation, coverage, MRSS, synchronization signal structure and periodicity, operation of bandwidth/band adaptation, spectrum utilization and aggregation framework, harmonization of TN and NTN, and others (if any). Note: To avoid distributing proposals of a same topic to different sub-agendas, please organize the proposals according to above highlights.

Note: A number of companies provide views on technical details of the following aspects. As per guidance from RAN1 chair, those aspects will be discussed in separate agenda items and/or future RAN1 meetings:

- This RAN1 meeting

  - Evaluation assumptions for 6GR air interface

    - Discussions on models, scenarios, parameters, and methodology, metrics/criteria, as well as traffic model that can be commonly used for evaluating technology proposals.

  - Waveform

    - Including proposals for improving spectrum efficiency, power efficiency, coexistence and coverage, etc.

  - Frame structure

    - Including numerology and frame structure (for all duplex types).

  - Channel coding

    - Including metrics/criteria that can be used for evaluating technology proposals and for down selecting proposals

  - Modulation, joint channel coding and modulation

    - Including metrics/criteria that can be used for evaluating technology proposals and for down selecting proposals

  - Energy efficiency

    - Including evaluation assumptions, proposals for Evaluations, NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving, targeting to categorize proposals by RAN1#123. From RAN1#124, proposals will be distributed to respective related agenda.

  - AI/ML in 6GR interface

    - Collecting AI/ML use cases in all potential components in physical layer design, targeting to select some use cases by RAN1#123. From RAN1#124, selected use cases will be distributed to respective related agenda.

- Future RAN1 meetings

  - Initial access

    - Placeholder only and to be broken down. No contributions before RAN1#124. Including synchronization signal and raster, broadcast signals/channel and physical random access channel, etc.

  - MIMO operation

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - Physical layer control, data scheduling and HARQ operation

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - Duplexing

    - Placeholder only and to be broken down or adapted based on the discussion in AI 11.1. No contributions before RAN1#124.

  - 6GR spectrum utilization and aggregation

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - NTN

    - Placeholder only and to be broken down or adapted based on the discussion in AI 11.1. No contributions before RAN1#124.

  - Other physical layer signals, channels and procedures

    - Placeholder only and to be broken down. No contributions before RAN1#124.

  - Sensing

    - Including PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…), aspects of integration with communication services.

    - Placeholder only and to be broken down. No contributions before RAN1#124b.

Similarly, a number of companies provide views on 6G RAN requirements, which is subject to the progress in RANp study for 6G RAN requirements.

# 2	Proposals for Online Sessions

## 2.1	Proposals for Monday Online

#### Proposal 3.1:

- For scalable 6GR design for diverse device types, RAN1 to consider

  - Approach 1: Minimum common features which are required to all 6G device types

  - Approach 2: Every feature is commonly applicable by default, i.e., maximize applicability of features to all 6G device types

- The features commonly applicable to all 6G device types include, but not limited to

  - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band

  - Idle mode prucedures

  - Initial access prucedures and mobility

  - DL/UL control

  - Basic scheduling/HARQ

  - Basic MIMO

  - MRSS

  - 1 TRX chain, smallest maximum supported RF and BB UE BW

Strive for functionality designs that can be commonly applied to devices with different characteristics

#### Proposal 4.1:

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - tradeoff between device complexity reduction and MBB performance impact

  - common signals/channels BW applicable to all device types

#### Proposal 4.2:

- When the minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types (if any), RAN1 to consider following to operate 6GR on the minimum spectrum allocation

  - Opt1: common signals/channels BW are punctured to fit into the minimum spectrum allocation

  - Opt2: specific design of the common signals/channels for the minimum spectrum allocation

Op1 like NR

Op2 all designed to support 3MHz

Op3 separate design for 3 and 5

#### Proposal 5.1:

- Study and identify the lessons learned from NR coverage enhancement features

#### Proposed observation 6.1:

- The lessons learned from LTE-NR DSS include, but not limited to

  - legacy and practical restrictions due to “always-on” signals like LTE CRS

    - Caused overhead and reduced NR PDCCH capacity

    - But already removed from NR

  - The maximum number of rate-matching patterns of PDSCH

    - too limited and thus costs inefficient inter-RAT resource sharing

  - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI

    - costs inefficient inter-RAT resource sharing

  - Rate-matching patterns in the first release of NR

    - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell

  - overall overhead from operating both RATs on the same carrier

    - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated

  - SDM was not considered

    - Limited flexibility for resource allocation

  - Interoperability issues between different vendors

    - timing mismatches may cause signal collisions, reduced throughput.

#### Proposal 6.2:

- High-level aspects which impact on the NR-6GR MRSS support include, but not limited to

  - UE/NW implementation complexity

  - Scheduler coordination

  - Traffic pattern

  - Radio resource utilization

  - No impact on legacy NR UE behavior

  - Operating bands/carriers

#### Proposal 7.1:

- High-level aspects which impact on the 6GR sync signal structure include, but not limited to

  - Reduced number of sync raster

  - Support of low-tier 6G device

  - Support of minimum spectrum allocation

  - Detection performance

  - Ensure orthogonalization against the NR PSS/SSS design

  - Extended coverage

  - Low complexity/power SS

  - decoupling for different RRC states

  - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand)

  - Scalability to operate on the supported deployments and spectrum, including multi-beam operation

  - Compatibility with any duplex modes

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

#### Proposed observation 8.1:

- The lessons learned from NR BWP framework include, but not limited to

  - A lot of potential uses, including adaptation to traffic demands and energy savings

  - A lot of RRC parameters under BWP configuration

    - results in unnecessarily large overhead

  - BWP switching delay

    - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides

    - UPT loss and increased UE power consumption

  - BWP switching

    - less motivated, for other than CORESET switching

    - will cause misalignment of real active BWP between BS and UE

    - results in unnecessary HARQ-ACK dropping

  - SCS switching

    - complicated but less motivated.

  - Excessive BWP types

    - including BWP types that have not been effectively used in practical NW, e.g., default BWP, dormant BWP.

  - Center frequency of DL/UL BWP

    - unnecessarily common

  - lack of RAN4 involvemen

    - leading to large MPR/A-MPR

  - Inherent restrictions

    - When a BWP is not covering the sync signal bandwidth, it can lead to different approaches for maintaining sync

#### Proposed observation 9.1:

- The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to

  - CA has been a very successful feature in LTE and NR

  - Pcell vs Scell

    - Allowing some functionalities only on specific cell like PCell may limit resource utilizations and prevent a NW from entering deep sleep as early as possible on a cell

  - Coupling DL and UL carriers for a cell

    - inefficient and ineffective due to different requirements and limitations between DL and UL

    - SUL/SDL, UL Tx switching, LBCA switching operate differently

    - SUL scheme is bound to dedicated SUL bands with UL-only resource

    - ensuring the presence of a corresponding downlink CC used as a reference for measurements

  - UL Tx switching

    - did not incorporate all UL transmissions, complicating its use

    - mandates UE to support at least N DL CCs and the N DL CCs are activated, which leads to high DL capabilities requirement and high UE power consumption

  - CA applicability

    - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times

    - did not sufficiently facilitate wide variety of deployments and network implementations but was designed to require challenging low latency inter-cell coordination

  - SSB adaptation for Scell

    - SSB-less SCell operation

      - limited applicable scenario.

    - On-demand SSB SCell operation

      - limited applicable scenario.

  - Activation of additional carrier

    - Slow not only because of signaling protocols and RAN4 requirements, but also because of very relaxed CSI accuracy for the newly activated carrier

    - faces a dilemma of choosing the high service latency caused by SCell activation and high UE power consumption by keeping SCell always activated

    - SCell dormancy

      - impractical as this feature is defined on top of BWP framework, which is unnecessarily flexible and complicated.

    - A-TRS trigger with SCell activation

      - not designed for NES.

  - Features (such as HARQ) defined per carrier

    - prevents further improvements on user throughput and latency via cross-carrier operation

    - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving

  - Avoid dependencies across carriers

    - such as DAI to simplify implementation and improve performance

  - The maximum number of bands in NR multi-band operations

    - actually limited by the maximum UE RF+BB hardware capacity in commercial networks

  - Concurrent transmissions of UL-CA/EN-DC

    - only beneficial for UEs who are close to gNB and have redundant UE Tx power and its symbol-by-symbol UL power control requires very tight coordination between PCell gNB and SCell gNBs.

    - need to require a semi-static UL power split for the UE in absence of gNB scheduler coordination.

    - Only supported for connected mode

  - Fragmented spectrum

    - not efficiently utilized and latency is unnecessarily increased under NR CA framework

  - Signalling overhead and UE processing complexity of PHY channels

    - scale with the number of aggregated carriers rather than the aggregated bandwidth size

  - No support of efficient IDLE/INACTIVE modes offloading

#### Proposed observation 10.1:

- The lessons learned from NR/IoT NTN include, but not limited to

  - NR NTN was introduced at later releases in a “NBC” fashion

    - Legacy UEs not able to connect, requiring extra development efforts

  - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features

  - Achievable data rate was kept low, which limits the applicability of NTN use cases

  - GEO satellite is hardly supported due to coverage issues

  - Low efficient beam hopping, severe UE power wasting

  - High dependency on UE GNSS accuracy

#### Proposal 10.2:

- The technical aspects affected by NTN characteristics include, but not limited to

  - Cell search / initial access / SSB periodicity

  - GNSS-less/resilient operation

  - Coverage enhancements

  - Positioning

  - NTN-TN and NTN-NTN mobility

  - CA

  - Capacity

  - Large propagation delay

  - Large doppler shift/drift and timing drifting

  - Duplexing

  - Beamforming / beam management

  - TN-NTN in the same spectrum

  - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

  - satellite moving

## 2.2	Proposals for Tuesday Online

#### Proposal 4.1a:

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Avoiding potential market fragmentation

  - Common signals/channels applicable to all device types [and minimum spectrum allcation] at least in idle mode and initial access

#### Proposal 5.1a:

- Study and identify the lessons learned from NR coverage enhancement features

- For around 7GHz, the study of 6GR design should aim at continuous coverage with ISD of at least 500m

- RAN1 provides initial analysis of potentially achievable coverage (e.g., MCL for a given data rate) to RAN#110 to determine the coverage target(s)

- All 6GR channels/signals should aim to meet the coverage target(s) from initial release

# 3 Scalable 6GR design

At the RAN1#122 meeting, following agreement was made related to scalable 6GR design and diverse device types:

Agreement​ Study a scalable 6GR design for diverse device types, considering aspects:​ - What should be commonly applicable to all 6G device types​ - FFS: add-on features dedicated to specific device types, if any​

Then, at the RAN#109 meeting, “diverse device types” were discussed and following proposals were agreed. It is concluded that those proposals will be further discussed in RAN while no WG discussion on them.

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type proposal 3 & 4 are endorsed for RAN only (no WG discussion)

Regarding “What should be commonly applicable to all 6G device types​”, according to companies input, there are in general two approaches to consider this aspect as follows:

- Approach 1: Minimum common features which are required to all 6G device types.

  - This approach may be one of the backgrounds of the situation where some companies are proposing “device type” and corresponding features/capabilities in early stage

  - The minimum common features can include, but not limited to

    - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band

    - Idle mode prucedures

    - Initial access prucedures and mobility

    - DL/UL control

    - Basic scheduling/HARQ

    - Basic MIMO

    - Duplexing

    - MRSS

    - 1 TRX chain, smallest maximum supported RF and BB UE BW

- Approach 2: Everything is commonly applicable by default, i.e., maximize applicability of features to all 6G device types beyond Approach 1

  - This approach assumes the discussions on such commonly applicable essential features should be prioritized over “device type” definition discussion

  - The commonly applicable essential features can include, but not limited to

    - The features listed in Approach 1

    - BS/UE EE features

    - Coverage enhancements

    - Data collection

    - Spectrum aggregation

Regarding “FFS: add-on features dedicated to specific device types, if any”, this may require discussion about device type definition to some extent, which should be avoided in RAN1. Some companies assume this can be handled by UE capability signaling while some others propose to discuss how to maximize the applicability of attractive sub-features that have some restriction/difficulty specific to some device type(s) and so on.

#### [Old]Proposal 3.1:

- For scalable 6GR design for diverse device types, RAN1 to consider

  - Approach 1: Minimum common features which are required to all 6G device types

  - Approach 2: Every feature is commonly applicable by default, i.e., maximize applicability of features to all 6G device types

- The features commonly applicable to all 6G device types include, but not limited to

  - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band

  - Idle mode prucedures

  - Initial access prucedures and mobility

  - DL/UL control

  - Basic scheduling/HARQ

  - Basic MIMO

  - MRSS

  - 1 TRX chain, smallest maximum supported RF and BB UE BW

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This issue is controversial and would require some time for mutual understanding among companies 2nd main bullet is candidate minimum set of features applicable to both Approach 1 and 2, for further discussion |
| Panasonic draft |  | On the first bullet, we are not sure whether two approaches are exclusive. After some more clear understanding of device type based on RAN plenary discussion, the characteristics important for the specific device type should be specific, which would be appraoch 1. On the other hand, the other charactoeristic, which is not chracteristics to the specific device type, should be enough to common approach as approach 2. Then instead of two approaches are listed, following one text can be sufficient? - For scalable 6GR design for diverse device types, RAN1 to consider following. - There are minimum common features which are required to all 6G device types. In addition, the feature other than not specific to the specific device type are commonly applicable by default. We support 2nd bullet. To spend the 2nd bullet would be more imporatnt. |
| Spreadtrum |  | For scalable 6GR design for diverse device types, Approach 1 is clear and should be adopted as the baseline for further discussion. For Approach 2, “Every feature is commonly applicable by default” is not clear for us. Does it means that all features (i.e., Every feature) are applicable for all 6G device types and for 6G day 1 network? Is every feature optional or mandatory? In addition, early down-selection is between Approach 1 and Approach 2 is helpful for 6GR overview work as this issue is highly correlated with many other fundamental 6GR design aspects. For lowest-tier device, “Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)” is captured in TR38.914. In our view, whether the feature of MRSS is applicable to lowest-tier device need to futher study. |
| China Telecom |  | We support Apporach 1 in principle with the following changes: - Approach 1: Minimum common features which are supported by required to all 6G device types For Approach 2, it’s also not clear to us, e.g., what is “every feature” means? |
| Google |  | Approach 2 is a bit unclear to us. How to assume every feature is commonly applicable to all types of devices? |
| Lenovo |  | The main problem is that the minimum device bandwidth & MNO carrier bandwidth and coverage has impact on common channel design, which are not quite clear yet. For example, +10dB coverage improvement has impact on the max. common channel (MSI) payload size, while adding bandwidth restriction due to device or MNO carrier bandwidth to it, reduces the data rate of MSI payload size. Also, EE needs to be considered. Hence, in the above option, for common design maybe applicable when all device type supports same coverage, when some of the device type supports extended coverage than others, then the applicability of common design especially common channel design needs further thinking. - For scalable 6GR design for diverse device types, RAN1 to consider for the same coverage case - Approach 1: Minimum common features which are required to all 6G device types - Approach 2: Every feature is commonly applicable by default, i.e., maximize applicability of features to all 6G device types |
| OPPO |  | For the first bullet, we in general support Approach 1. The inter-device-type scalability of 6GR requires a common functionality set as a baseline. But we suggest to replace the “features” to “functionalities”, to avoid mis-understanding to “UE features”. On the other hand, we do not understand how Approach 2 can work for 6G. In our understanding, different 6G device types will anyway have different mandatory/optional features. A device-type-specific mandatory feature may not be applicable for other device types. For example, a sensing feature mandatory for the sensing device type is not applicable for eMBB devices not supporting sensing services. “Every feature is commonly applicable by default” was the wrong way we had in 5G era, which mixed the intra-device-type scalability with inter-device-type scalability. We think Approach 2 does not work in 6G because there will be very diverse device types, such as sensing, NTN, etc. - For scalable 6GR design for diverse device types, RAN1 to consider - Approach 1: Minimum common featuresfunctionalities which are required to all 6G device types - Approach 2: Every feature is commonly applicable by default, i.e., maximize applicability of features to all 6G device types We in general support the second bullet as study scope for minimum common functionalities. Similarly, suggest to replace “features” to “functionalities”: - The featuresfunctionalities commonly applicable to all 6G device types include, but not limited to - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band - Idle mode prucedures - Initial access prucedures and mobility - DL/UL control - Basic scheduling/HARQ - Basic MIMO - MRSS - 1 TRX chain, smallest maximum supported RF and BB UE BW |
| Fujitsu |  | Before we go to the detailed discussion, in our understanding, it might be necessary to confirm one thing that the number of device types should be as small as possible. The basic design of one device type can be used as many as possible. So that the cost of devices can get lower with the increasing number of one device type. Based on this, we think the common features which need to be considered first should be hardwared-relevant features rather than RRC features or logic features/functionalities. For example, the processing capability of devices (such as the peak data rate) and the blind detection calculation capability for PDCCH etc. Besides, being in the earliy stage of 6G, we may need to leave more details up to the further discussions. |
| Fraunhofer |  | We share the concenrns raised above on Approach 2. We support Approach 1. |
| Apple |  | It is not clear who Approach 2 works out considering all different device types. Suggestions below: - The features commonly applicable to all 6G device types include, but not limited to - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band - Basic Idle mode pruocedures - Initial access pruocedures and mobility (not sure if mobilty is applicdable for devices like FWA) - Basic DL/UL control - Basic scheduling/HARQ - Basic MIMO (not clear what basic MIMO feature means given 1TRX chain as low tier device) - MRSS 1 TRX chain, smallest maximum supported RF and BB UE BW |
| Nokia |  | Similarly to Panasonic, we do not see those approaches as mutually exclusive, but they should not be considered as approaches to handle device types. Approach 1 is about setting the minimum baseline for a device to be able to operate in the system, which is something that will be defined one way or another, regardless of device types being considered in 6GR. Approach 2 is about the assumptions for the functionalities/features/capabilities defined in RAN1, which are usually assumed to be applicable to all devices by default, potentially subject to UE capability indication. To some extent this is also independent of device type definitions, it just reflects the normal way of working in RAN1. |
| Samsung |  | Approach 1 seems reasonable e.g., does not make sense to have some eMBB features be applicable for IoT. |
| Ericsson |  | To us, it is unclear what is meant with ‘approach 2’. Clearly, there will be features that are not relevant for the lowest-tier devices. The list in the second main bullet is larely relevant, but we need to go deeper into details before concluding what is supported (and not supported) by the lowest-tier device. For example, what is meant with “basic MIMO” here while listing the 1 Tx/Rx chain separately? The lowest-tier devices are likely not supporting any SU-MIMO. The initial access mechanism needs to be such that also lowest-tier devices can access the network (although reduced performance relative to “eMBB” can be accepted). |
| CEWiT | Y | Support the intention of the proposal. Regarding first bullet: Approach 1 is supported considering the diverse requirements and capabilities under consideration for device types. Regarding second bullet, all device types may have a common phase for initial access procedures, idle mode procedures and control procedures. Also, there can be dedicated phase for initial access procedures, idle mode procedures and control procedures, where this dedicated phase for each device types depend on requirements and capability of each device type. E.g., SSB Tx with larger default periodicity is part of common design and OD-SSB is part of dedicated phase for delay sensitive device types. Based on this suggest the following update - The features commonly applicable to all 6G device types include, but not limited to - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band - part of Idle mode procedures - part of Initial access procedures and mobility - part of DL/UL control - Basic scheduling/HARQ - Basic MIMO - MRSS - 1 TRX chain, smallest maximum supported RF and BB UE BW |
| HONOR | Y | Agree with the proposal in principle. Approach 1 in the first bullet is preferred. |
| LGE |  | 1) Correction of some typo - Idle mode pruocedures - Initial access pruocedures and mobility 2) The last bullet (i.e., 1 TRX chain, smallest maximum supported RF and BB UE BW) seems not clear to include in the list. Depending on UE type, low-tier device type UE may have 1TRX , but normal device type UE may not have 1TRX. Also, depending on the device type, the smallest maximum supported RF and BB UE BW may be different. That is, the 1TRX and the smallest maximum supported RF and BB UE BW seem not common part to all 6G device type. |

#### Proposal 3.1a:

- For scalable 6GR design for diverse device types, RAN1 to consider

  - Strive for functionality designs that can be commonly applied to all 6G device types

- The functionalities commonly applicable to all 6G device types include, but not limited to

  - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band

  - Idle mode prucedures

  - Initial access prucedures and mobility

  - DL/UL control

  - Basic scheduling/HARQ

  - Basic MIMO

  - MRSS

  - 1 TRX chain, smallest maximum supported RF and BB UE BW

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | The proposal is updated based on the discussion in Monday online - Unified approach1/2 as general principle - Replace “features” by “functionalities” - Remove parameters/factors which are included in the following agreement in RANp to avoid potential impact on/from device type discussion Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type |
| Ericsson |  | We should avoid the (currently undefined) term ‘device type’ – whether we group UE capabilities into device types or not is still open. It is better to use terms like ‘devices’ or ‘UE capabilities’ |
| LGE |  | Thank you for providing updated proposal. We are now discussing the SCS and CP length for not only communication but also sensing. So far, it is not concluded to use single numerology per band. Suggest to delete ‘single numerology per band’ in this time. - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band |
| OPPO |  | If it is not proper for RAN1 to use the term “device type”, we can avoid to use it to avoid controversy. Instead, we can use the term “usage scenario” which is defined by ITU-R IMT-2030 framework. And for designing the scalable 6GR, what is essential is to identify the range of scalability, i.e., the lowest capability and highest capability. Suggest to improve the proposal as below: Proposal 4: In terms of diverse device typesFor a given 6G (IMT-2030) usage scenario, study further the range of 6GR scalability, including: - Possible parameters/factors for the usage scenario, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device typethe usage scenario, at least for the highest and lowest capabilities for the usage scenario |
| Samsung |  | Thanks a lot for your efforts, we have comments and revision to correct typos. Regarding modulation, we think Max modulations in RANp is not the case of remove here. #### Proposal 3.1a: - For scalable 6GR design for diverse device types, RAN1 to consider - Identify common Strive for functionality designs that can be commonly applied to all 6G device types - The functionalities commonly applicable to all 6G device types include, but not limited to - Basic PHY features, such as waveform, modulation, coding, frame structure, single numerology per band - Idle mode procedures prucedures - Initial access procedures prucedures and mobility - DL/UL control - Basic scheduling/HARQ - Basic MIMO - MRSS - 1 TRX chain, smallest maximum supported RF and BB UE BW |
| IMU |  | We generally fine with the proposal. As the definition of ‘device types’ is not yet specified, we suggest avoiding the term at this stage. |
| Xiaomi |  | We are fine with the proposal. Note that there are typos in the proposal that purcedures should be procedures. |
| SONY |  | We are OK with the list. We are also OK with including 1 TRX chain, smallest maximum supported RF and BB UE BW. It is common that all devices support 1TX1RX and the smallest maximum RF and BB UE BW, with the understanding that the TRX chain and UE BW will be scaled up with higher classes of devices. |
| CATT |  | Thanks FL. Some suggestions mark in blue: - For scalable 6GR design for diverse device types, RAN1 to consider identity functionality designs that can be commonly applied to all 6G device types - The functionalities commonly applicable to all 6G device types include, but not limited to - Basic PHY features, such as waveform, modulation, , channel coding, frame structure, single numerology per band - Idle mode procedures - Initial access procedures and mobility - DL/UL control - Basic scheduling/HARQ - Basic MIMO - MRSS - Coverage enhancement - Energy saving - 1 TRX chain, smallest maximum supported RF and BB UE BW The reason to add ‘Coverage enhancement’ is that we believe it is common understanding that any CE method should be applicable to all device types. The reason to add ‘Energy saving’ is make it possible to pursue NW and UE ES regardless what UE type is under service in the network. |

Also, some companies discuss how to improve UE capabilities, including

- Prerequisites can be used to prevent undesired capability combinations

- Define set of mandatory/optional capabilities per use cases / device types, e.g., modular capability

  - Beneficial features for BS and/or UE

- modular and MAC-layer-assisted UE capability and assistance information reporting framework

This can be discussed in later stage of SI or even WI after overall 6GR features become clear.

# 4 Support of minimum spectrum allocation

At the RAN1#122 meeting, minimum spectrum allocation and smallest maximum supported RF and BB UE BW were discussed and following agreements were made.

Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Then, at the RAN#109 meeting, the above issues were discussed due to the condition of “subject to further discussion and confirmation in RAN” and following proposals were agreed. It is concluded that those proposals will be further discussed in RAN1 while RAN#110 revisit.

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Based on the situation, RAN1 chair provided guidance that RAN1 will focus on how to support the minimum spectrum allocation, especially the case where the minimum spectrum and the smallest maximum UE bandwidth can’t match, considering that it is hard to purely discuss these two elements in RAN1.

Note that following is captured in TR38.914 related to lowest-tier device

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

Although RAN1 chair guidance suggests that RAN1 will not purely discuss the values for the minimum spectrum allocation or smallest maximum UE BW, quite a few companies provide the views especially on the smallest maximum UE BW, for which majority support more than 3MHz BW even for 15kHz SCS, while there are some companies support 3MHz BW for providing services to the low tier market segment, reduce the UE power consumption, SAW-less design for UL, and migration from LTE LPWA. Also, there would be common understanding that it is necessary to consider the tradeoff between device complexity reduction and MBB performance impact when discussing the smallest maximum UE BW. It was also mentioned by companies that the smallest maximum UE BW should be equal to or larger than the common signals/channels BW applicable to all device types for the scalable 6GR design. Hence, following proposal can be considered for further discussion.

#### [Old]Proposal 4.1:

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - tradeoff between device complexity reduction and MBB performance impact

  - common signals/channels BW applicable to all device types

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | As per the guidance from RAN1 chair, RAN1 will not purely discuss the smallest maximum UE BW value. This proposal can be used as starting point to further discuss feasible value from RAN1 perspective |
| Panasonic | Y |  |
| Spreadtrum | Y with minor modification | For the 1st sub-bullet, the device should be the “low-tier device”. We provide the modification as follows: - tradeoff between low-tier device complexity reduction and eMBB performance impact |
| Google | Y |  |
| Lenovo |  | When low-tier IoT device for example 5MHz has extended coverage(+10~15dB), then the design of common channels needs to handle such extended coverage. Impact of such common channel design on system design needs to be considered. - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least - tradeoff between device complexity reduction and MBB performance impact - common signals/channels BW applicable to all device types and its impact on coverage |
| OPPO | Y in general | In general, we suppor the proposal. But we think the eMBB and IoT are both essential for 6GR design. Suggest to modify the proposal as such: - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least - tradeoff between device complexity reduction and MBB performance impact for eMBB and 6G IoT - common signals/channels BW applicable to all device types |
| Fujitsu | Y |  |
| Fraunhofer | Y |  |
| Apple |  | Looks fine |
| Samsung |  | OK |
| Ericsson |  | It should be clarified that the “common signals/channels BW applicable to all device types” refer to the initial access procedures. Once connected, the network knows the UE capabilities and bandwidths is supports. Furthermore, althgouh the term “at least one low-tier deice type” was agreed last meeting, we think it is clearer to use the term “lowest-tier device” to make the scalability aspect clearer. |
| CEWiT | Y | Support the proposal |
| HONOR | Y | Fine |
| LGE |  | We are fine with the Proposal 4.1. Regarding the smallest maximum supported RF and BB UE BW, we think as follow: - The maximum supported RF and BB UE BW is different depending on the UE type (i.e., low-tier device type, normal device type for eMBB) - The smallest maximum supported RF and BB UE BW should be wider than or same as the minimum CBW / bandwidth for common signals/common channels (e.g., SSB). |

#### [Old]Proposal 4.1a:

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Avoiding potential market fragmentation

  - Common signals/channels applicable to all device types [and minimum spectrum allcation] at least in idle mode and initial access

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Updated proposal after Monday offline - Yellow highlight needs further discussion |
| Samsung |  | We support to remove [ ] in the last bullet. Also, not sure the relevance of the third bullet on energy efficiency, smaller BW is always better energy efficiency. |
| Xiaomi |  | We think the last bullet should be removed. The smallest maximum UE BW should be decoupled from the minimum spectrum allocation and common channel/signal BW. While the BW of common signals/channels should take both minimum spectrum allocation and smallest maximum UE BW into account. |
| SONY |  | We think that this is a good list of criteria to use to study the smallest maximum support RF and BB UE BW. We think that the issue of whether it is possible to have common signals / channels for all devices is an important criterion / consideration when studying the smallest bandwidth, so we think it would be good to capture this aspect. When it comes to device complexity, it is necessary to consider both the impact on device RF domain and BB domain. The UL UE BW may be different to the DL UE BW, so when RAN1 considers smallest bandwidth, it might come to different conclusions for UL and DL. In our view, the device complexity is more severely impacted by the UL RF bandwidth while system performance is less impacted by UL RF bandwidth. RAN1 might conclude on e.g. 10MHz DL bandwidth and 3MHz UL bandwidth. - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider separately for UL and DL at least - Device complexity - Overall system performance impact - Energy efficiency for both BS and UE - Avoiding potential market fragmentation - Common signals/channels applicable to all device types [and minimum spectrum allcation] at least in idle mode and initial access |
| TCL |  | We believe that the last two bullet points should be removed. Perhaps addressing the potential market fragmentation in Proposal 11.2 would be more appropriate. |

#### Proposal 4.1b:

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Avoiding potential market fragmentation

  - Common signals/channels applicable to all device types [and minimum spectrum allocation] at least in idle mode and initial access

  - Aim at a single common signals/channels design in idle mode and initial access, targeting scalable and forward compatible design for diverse device types, as well as meeting mobile broadband service requirements as high priority

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Updated proposal after Tuesday online - Updated the last sub-bullet based on SID text |

Regarding the minimum spectrum allocation, some companies mention that RAN1 may not make much progress without considering exact values, which will be discussed in RANp. For now, what RAN1 can discuss is how to handle the case when the minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types, since the smallest maximum UE BW does not impact on the operation on the minimum spectrum allocation as long as UE is capable to operate on the minimum spectrum allocation (e.g. max 20 MHz BW UE can be operate on 5MHz CBW). A number of companies assume similar handling as NR Rel-18 less than 5MHz BW, i.e., puncturing the common signals/channels to fit into the minimum spectrum allocation w/ some performance degradation. Also, some companies consider special handling of such minimum spectrum allocation, e.g., specific design of the common signals/channels (e.g., SSB) for the minimum spectrum allocation.

#### [Old]Proposal 4.2:

- When the minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types (if any), RAN1 to consider following to operate 6GR on the minimum spectrum allocation

  - Opt1: common signals/channels BW are punctured to fit into the minimum spectrum allocation

  - Opt2: specific design of the common signals/channels for the minimum spectrum allocation

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | As per the guidance from RAN1 chair, RAN1 will not purely discuss the minimum spectrum allocation value. This proposal can be used as starting point to further discuss how to handle the case when special handling is necessary for the minimum spectrum allocation value from RAN1 perspective |
| Panasonic | Y |  |
| Spreadtrum |  | The “specific design” in option 2 should be further clarified. The target device of the specific design is not clear, i.e., for all devices or only for low-tier device. If the specific design targets to all devices, it means the eMBB device will support two sets of common signals/channels. |
| China Telecom |  | We think this kind of minimum spectrum allocation needs to be avoided, since it violates the scalable design principle. |
| Google |  | We think opt2 should be revised as “the common signals/channels are designed based on the minimum spectrum allocation”. Current formulation sounds like two designs. |
| Lenovo | Y | We support Opt1. Since majority of the operators in the last plenary indicated to optimize the common channel for 5MHz carrier, it makes sense to optimize it for 5MHz and support 3MHz by other means, We can still make optimizaiton to improve the performance of 3MHz beyond Rel18. - When the minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types (if any), RAN1 to consider following to operate 6GR on the minimum spectrum allocation - Opt1: common signals/channels BW are punctured to fit into the minimum spectrum allocation. 5G NR Rel18 can be considered as baseline, aim is to improve the performance beyond 5G NR Rel18 - Opt2: specific design of the common signals/channels for the minimum spectrum allocation |
| OPPO |  | In general, in our view both of the options now are sub-optimal approaches, in particular Opt 1 which would lead to up to 5dB PBCH performance loss according to our study. In 6G day 1 the optimal approach should be to design the common signals/channels fitting into the minimum spectrum allocation. If go with separate design and Opt1 is considered, solution for migating performance loss has to be introduced in the mean time. For Opt 2, seems “the common signals/channels” should be “another common signals/channels” We suggest the following changes: - Alt 1: design of the common signals/channels fitting into the minimum spectrum allocation - Alt 2: When tThe minimum spectrum allocation is smaller than the common signals/channels BW applicable to all device types (if any), RAN1 to consider following to operate 6GR on the minimum spectrum allocation - Opt1: common signals/channels BW are punctured to fit into the minimum spectrum allocation with minimized permormance loss - Opt2: specific design of theanother common signals/channels forfitting into the minimum spectrum allocation |
| Fujitsu | Y |  |
| Franunhofer | N | We think this discussion should be postponed until there is a decision on the value of ‘minimum spectrum allocation’. On top of that, this proposal seems to contradict with the previous one (Proposal 4.1), specifically, it seems to imply that we cannot design “common signals/channels BW applicable to all device types” to fit within the ‘minimum spectrum allocation’. That was the case in Rel-18 because of backward compatibility. But of course, as we are designing a new generation, we should explore novel designs to fit common signals/channels within the ‘minimum spectrum allocation’. |
| Apple |  | We think Proposal 4.2 is premature to be discussed without RAN plenary discussion. Whether to support smaller CBW than common signals/channels BW with/without optimization is subject to RAN plenary discussion, in our opinion. |
| Nokia |  | We think the exact way forward for such signals/channel can be discussed in the corresponding AIs that will start early next year. |
| Samsung |  | Depending on the design of common signal/channels. If design is over the minimum spectrum, we should apply Opt 1 but we should try to study whether option 2 is feasible in terms of performance and complexity. |
| Ericsson |  | We interpret the term “common signals/channels BW” as relating to initial access. Both options can be discussed, but we would like to avoid designing special channels and procedures fo the lowest-tier devices as multiple designs may delay the introduction of low-end devices and goes against the “scalable RAT” philosophy. |
| CEWiT |  | We support the intent of the proposal but the opt2 is the preferred way for most of the case. Whereas for the Opt1 should be rigorously examined against future compatibility, scalability and tradeoff between device complexity reduction & MBB performance impact perspective. So we propose that the first release should prioritise the option 2. Regarding the justification of option1 by other companies, handling as NR Rel-18 less than 5MHz BW was optimal only because it was a mid release. However in 6GR we believe going for Opt2 will lead to an optimal design. For e.g., the performance degradation, if any, can be minimized by introducing common phase and dedicated phase for cell common procedures (see our comment for proposal 3.1). Also, it is beneficial from the NES perspective as the operating BW of common signals will be less. |
| HONOR |  | Fine |
| LGE |  | Proposal 4.2 mentions the case when the minimum spectrum allocation is smalle than the common signa/channel BW. But, before discussing the proposal 4.2, we may need to decide whether the common singal/channel BW can be wider than the minimum spectrum allocation or not. |

#### Proposal 4.2a:

- RAN1 to consider following to operate 6GR on the minimum spectrum allocation

  - Opt1: common signals/channels BW for initial access are punctured to fit into the minimum spectrum allocation, if the minimum spectrum allocation is smaller than the common signals/channels BW for initial access

  - Opt2: Separate design of the common signals/channels for initial access for the minimum spectrum allocation from other spectrum allocations , if the minimum spectrum allocation is smaller than the common signals/channels BW for initial access for other spectrum allocations

  - Opt3: A single design of the common signals/channels for initial access which is applicable to any spectrum allocations

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | The proposal is updated based on the discussion in Monday online - Change the overall structure so that the main bulled does not have the condition of “minimum spectrum allocation is smaller than the common signals/channels BW for initial access” - List up all potential solutions according to companies input |
| Ericsson |  | Ok, but detailed discussions are probably better handled in the upcoming initial access agenda item. |
| LGE |  | Revised Proposal 4.2a seems better to cover options. |
| OPPO |  | We in general support this proposal for future study, although we do not support Opt2. But for Opt1, we still think “punctured” is too restrictive for a optimal design. We suggest to use the wording the RAN1 chair used in online session: - Opt1: common signals/channels BW for initial access are punctured to fit into feasible/reusable for the minimum spectrum allocation, if the minimum spectrum allocation is smaller than the common signals/channels BW for initial access |
| Samsung |  | Since the minimum spectrum allocation is not decided yet. We can revise opt 1 as : - Opt1: common signals/channels BW for initial access are punctured to fit into the minimum spectrum allocation, if the case where the minimum spectrum allocation is smaller than the common signals/channels BW for initial access is supported For opt2, this option requires multiple design of SSB depending of the size of spectrum, which is not aligned with SID striving only single option for each feature. So, we suggest to remove - Opt2: dedicated design of the common signals/channels for initial access for the minimum spectrum allocation from other spectrum allocations , if the minimum spectrum allocation is smaller than the common signals/channels BW for initial access for other spectrum allocations Regarding opt3, it is special case for opt1, if the minimum spectrum of allocation is always larger than common signals/channel BW, opt1 will not happen. |
| Fraunhofer |  | We agree with OPPO that Opt1 should be modified to reflect what was discussed in Monday online, which can be seen in the following text copied from the chair notes: Agreement - When the minimum spectrum allocation is smaller than the common signals/channels BW for initial access, RAN1 to consider following to operate 6GR on the minimum spectrum allocation - Opt1: common signals/channels BW for initial access are feasible/reusable for the minimum spectrum allocation - Opt2: specific design of the common signals/channels for the minimum spectrum allocation #### It is better to avoid repeating same arguments in online, and to be more general. We also do not support Opt2. We support keeping Opt3. |
| IMU |  | The listed options (Opt1–3) appear sufficient to cover possible cases at this stage although we don’t fully support option 1, and we support keeping the discussion open while maintaining scalability and flexibility principles. |
| Xiaomi |  | We do not think it is urgent to have the proposal given that the minimum spectrum allocation is not decided and the details would anyway be discussed in the initial access agenda item. But if majority companies support the proposal, it is also fine with us. One clarification on “common signals/channels for initial access”, does it imply that there are common signals/channels not for initial access? If so, how to operate on the minimum spectrum allocation? |
| CMCC |  | The ‘common signals/channels’ design is discussed in different proposals of this summary. We prefer to continue discuss proposal 4.1a on this part, in this proposal all possible directions are listed, which is not much helpful. |
| SONY |  | We think this is a good list of options. RAN1 can study these options. |

# 5 Overall coverage

At the last RAN1 meeting, overall coverage for 6GR was discussed and the following agreement was made:

Agreement - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

In addition, RAN#109 concluded the following:

Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel

Quite a few companies provide the views on coverage target for 6GR, including not only + 5 to 10 dB enhancement from normal coverage (144dB MCL) for cell-edge performance but also overall DL/UL performance improvements in anywhere within the cell coverage. However, as clarified by RAN1 chair over RAN1 reflector, RAN1 will not discuss coverage target(s) to avoid duplicated work with RAN plenary.

Related to the SID objective “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, some companies mentioned following aspects, which can be discussed in other agenda or WGs

- More antenna elements for BS and/or UE

  - This aspect can be discussed in RANp SI for 6G requirement (especially for deployment scenarios) as well as RAN1 6G study AI11.2 for evaluation assumptions

- More number of TRX

  - This aspect can be discussed in RAN1 6G study AI11.2 for evaluation assumptions

- Incresed UE Tx power

  - Should be led by RAN4. Early RAN4 involvement is necessary

Due to the lack of clear coverage target(s), companies have divergent views which channels need to be improved, and how to do it, including but not limited to

- Which channels need to be improved

  - Ensure targeted coverage for all channels and signals

  - Channels/signals during initial access

  - Mainly for UL coverage

- How to improve coverage

  - Repetitions

    - Including unified solution among different channels

  - Available Slot Counting (ASC)

  - DMRS bundling/Joint Channel Estimation (JCE)

  - TBoMS

  - Cross-slot Tx, including PUSCH and RS

  - DL/UL decoupling (discussed in Section 9)

Some companies also mention this requires early RAN4 involvement. Note that there are some other proposals for coverage enhancements, including waveform, modulation, duplex and so on, which should be discussed in the corresponding agendas.

Give the situation, moderator assume it’s premature to discuss any solutions for coverage enhancement without clear coverage target(s). Following proposal can be considered similar to other discussion points

#### [Old]Proposal 5.1:

- Study and identify the lessons learned from NR coverage enhancement features

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Potential discussion topics are to identify lessons learned from NR CovEnh features, so that better CovEnh features will be considered from 6G Day1 |
| Panasonic |  | Repetition consumes the resources compared with other schemes like more antenna, more number of TRX and more Tx power. On the other hand, it could be applicable to rather universally regardless of the situation. Therefore, the repetition should be applicable basically to any channels except one shot reception intended channel of periodic synchronization signals. |
| Spreadtrum | Y |  |
| China Teleocm | Y |  |
| Google | Y |  |
| Lenovo |  | 5G NR introduced coverage enhancement starting from Rel17 which was quite late and coverage enhancement solutions were part of NTN until Rel19. Coverage enhancement for the common channels has an impact on the backward compatilibtly, however other dedicated channels can be enhanced for coverage however its impact on EE should be taken into account. |
| OPPO |  | NR coverage enhancement features are based on the potential bottleneck channels after the evaluation of coverage performance, and NR has introduced some designs to improve the coverage of certain channels, e.g. PRACH, PUSCH, msg3, PUCCH, etc. When we discuss the coverage in 6G, we think the baseline coverage performance of each channel should be evaluated firstly. In order to achieve a better coverage performance, some typical designs in 5G or other potential solutions could be considered as the baseline for the channels, e.g. repetition with available Slot Counting, DMRS bundling, TBoMS, cross-slot TX, etc. After the coverage performance evaluation of each channel and comparision with the target coverage, we can further discuss whether/how to improve coverage for each channel. |
| Fujitsu |  | As we discussed in our contribution, we think ‘Repetition’ is the most important way to support coverage enhancement particularly when the pontentially required enhancement is larger than 10dB. Hence, we need a unifid/clean/extendable repetition solution for all channels as the basis to guranteen the coverage. Other solutions can be considered as well. |
| Apple |  | Okay |
| Nokia | Y | One possibility also is that RAN1 provides input to RAN in December to assist with the decision on coverage target, e.g. based on the evaluation assumptions (hopefully) agreed in 11.2. |
| Samsung |  | OK to discuss. Coverage enhancements are both a UE-specific issue (e.g., number of Rx antennas) and a network specific issue (e.g., varying targets for BLER, latency, false detection/miss, …), and can be different in UL and DL. We agree that it is too early to decide which 6GR channels/signals require a performance improvement (first requires agreed coverage targets, second requires detailed channel/signal design). We also agree that more antenna elements/TRXs can be part of 11.2 Eval assumptions, and HP UEs and their Tx behavior is for RAN4. Its too early to discussed unified/simplified repetition behavior from a system procedure perspective when potential performance bottlenecks of inviditual channels/signals are not yet known (e.g., Initial Access). We think it is meaningful to discuss/decide support for some basic repetition feature with most details FFS at least for the UL channels/signals in Rel-21 6GR. It is clear already that some support for some repetition behavior to meet even the 144-145 dB MCL target is needed, and more so with [6-10] dB improved range for some device classes. It is one of the big lessons learned from NR in Rel-16-20 that piecemeal introduction of repetition features such as PUSCH 32x, ASC, etc. did not allow to benefit from such features in the NR deployments. Support for basic repetition behavior should be part of native HARQ/scheduling operation in Rel-21 6GR and should be discussed early on. |
| Ericsson |  | We think +10 dB of “additional coverage” relative to a baseline 5G Rel-15 system (that does not use repetition) is sufficient, at least in FR1 FDD bands. Clearly, the performance will be lower in this “additional coverage” region, but it should still be possible to connect to the system. Note this does not imply that all channels need to be “enhanced”, but only the ones that are the bottleneck. Technical solutions to achieve this are to be discussed, but most likely repetition, TBoMS, and similar techniques can be useful and should (in a generalized interpretation) be part of the dynamic scheduling framework. |
| CEWiT | Yes | We do agree that RANP should specify the target for us to have focus direction on coverage. We are fine with proposal. But we can be more specific on the lesson learned inputs expected by specifying the categories like coverage of control channel in NR, coverage of data channel (DL and UL) in NR, reference signal coverage in NR, coverage mechanism comparison etc. #### Proposal 5.1: Study and identify the lessons learned from NR coverage enhancement features including coverage of control channel in NR, coverage of data channel (DL and UL) in NR, reference signal coverage in NR, coverage mechanism comparison etc. |
| HONOR | Y | OK |
| ZTE | No | For the coverage, as commented in offline, we prefer to focus on more specific issue, e.g., ensure DL/UL channel to satisfy the requirement/aimi to meet the target, along with potential solution to be considered, similar as the agreement for duplex in last meeting. |
| LGE |  | We are fine with the proposal 5.1 to study and indentify the lessons learnd from NR coverage enhancement features in Agend Item 11.1. From the next year, we think some topics which is potentially listed up will be studied in corresponding agenda items. For example, time domain solutions (e.g., repetition, ACS, JCE, TBoMS, cross-slot Tx) can be discussed in AI for data scheduling (e.g., schemes for CE for UE in CONNECTED state) and/or initial access (e.g., RACH repetition, msg2/4 PDSCH repetition, msg3 PUSCH repetition, PDCCH repetition for IDLE/INACTIVE UE). Spatial domain solutions (e.g., UL only reception point) can be discussed in AI related with MIMO. We need to be clear which is a right Agenda Item to be discussed the schemes for coverage enhancement. |

#### Proposal 5.1a:

- Study and identify the lessons learned from NR coverage enhancement features

- For around 7GHz, the study of 6GR design should aim at continuous coverage with ISD of at least 500m

- RAN1 provides initial analysis of potentially achievable coverage (e.g., MCL for a given data rate) to RAN#110 to determine the coverage target(s)

- All 6GR channels/signals should aim to meet the coverage target(s) from initial release

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | Updated proposal after Monday offline - Yellow highlight needs further discussion - 3rd bullet assumes following timeplan - RAN1 Nov meeting: collect input from companies and provide initial analysis to RANp - RANp Dec meeting: determine tentative target target(s), and task RAN1/4 for confirmation - RAN1/4 Feb meeting: check whether to confirm the tentative target target(s) |
| Ericsson |  | The 3rd bullet is good, having some numbers as input to RAN could be useful. On the 2nd bullet, we think it does not add that much. Aiming for 500 m ISD @ 7GHz could be fine, but without further assumptions (e.g. what environment to assume) it does not add much and it might be better to delete the third second bullet to avoid lengthy online discussions. Simulations done under the third bullet can anyway provide input on the 7 GHz coverage. Note: in the discussions yesterday, two “coverage aspects” were discussed, sometimes a bit mixed – coverage (extension) for the lowest-tiers UEs where the data rate is a couple of kbit/s, as well as coverage in general for significant higher data rates |
| LGE |  | Suggest small modification as follows: - Companies are encouraged to provide RAN1 provides initial analysis of potentially achievable coverage (e.g., MCL for a given data rate) to RAN#110 to determine the coverage target(s) |
| OPPO |  | As analyzed in our contribution, it is too early to determine the metric for coverage. We suggest to consider MIL and MPL together with MCL. And when we discuss the coverage in 6G, we think the baseline coverage performance of each channel should be provided. Suggest to improve the proposal as below: - Study and identify the lessons learned from NR coverage enhancement features - For around 7GHz, the study of 6GR design should aim at continuous coverage with ISD of at least 500m - RAN1 provides initial analysis of potentially achievable coverage (e.g., MCL/MIL/MPL for a given data rate) to RAN#110 to determine the coverage target(s) - All 6GR channels/signals should aim to meet the coverage target(s) from initial release - Companies are encouraged to provide the baseline coverage performance of each channel as benchmark of their analysis. |
| Samsung |  | We suggest to have step-by-step approach as we did for other topics. (1) The first step in RAN1 should be to identify and agree on lessons learned from NR. We can additionally identify some future design principles for improved 6GR coverage. (2) We should then aim to provide an initial link budget analysis to RAN#110. We can successively update and include more evaluation scenarios and refine the MCL/MIL/MPL assumptions based on agreements in 11.2 Eval for RAN#111 and beyond. 6GR coverage should be compared to NR for some essential deployment cases, (1a) FR1 700 MHz, (1b) 2GHz, (1c) 3.5 GHz and (1d) FR1-ext/FR3 7GHz and (2) FR2. We propose to first provide the initial link analysis for (1d) compared to (1c) for the existing SID objective. Several assumptions such as #TRXs, #AEs, Tx power for the 7GHz vs. 3.5 GHz link budget analysis are already agreed in 11.2 (3) Whether/how the 6GR DL/UL channels/signals design can meet the agreed coverage targets will depend on more progress/decisions in other AIs. We suggest to reformulate the FL proposal as shown below. Note that without initial coverage analysis, we are not sure it is agreeable to set the target for 500m for 7 GHz. - For study of 6GR coverage, - Study and identify the lessons learned from NR coverage enhancement features - For around 7GHz, the study of 6GR design should aim at continuous coverage with ISD of at least 500m - RAN1 provides initial analysis of potentially achievable coverage for 3.5 GHz and around 7 Ghz (e.g., MCL/MIL for a given data rate) to RAN#110 to determine the coverage target(s) - Whether and how all 6GR channels/signals meet the coverage target(s) from initial release |
| Xiaomi |  | For the 2nd bullet, isn’t it coverage target that should be discussed in RAN plenary? For the 3rd bullet, we understand the intention to provide inputs to RAN plenary to facilitate the determination of the coverage target. However, we do not think it is practical considering that next RAN1 meeting is just few weeks later and we do not have the aligned evaluation assumptions yet. The proposed updated from LGE is more appropriate. |
| CMCC |  | For the second bullet, it belongs to coverage target in our view. We are not sure whether at least 500m is suitable without analysis on coverage in RAN1. We propose to remove bullet2 #### Proposal 5.1a: - Study and identify the lessons learned from NR coverage enhancement features - For around 7GHz, the study of 6GR design should aim at continuous coverage with ISD of at least 500m - RAN1 provides initial analysis of potentially achievable coverage (e.g., MCL for a given data rate) to RAN#110 to determine the coverage target(s) - All 6GR channels/signals should aim to meet the coverage target(s) from initial release |
| SONY |  | Agree with proposal. This coverage target should apply for all devices (e.g. the coverage should be achieved also for a device with 1RX). |
| CATT |  | At this stage, only the first bullet is realistic to prepare in the next meeting. For 2nd bullet, it is repeating the SID. For the 3rd bullet, we do not think RAN1 can finish all evaluation of all channels without detailed design on any 6G channel in RAN1#123. For the 4th bullet, it is common understanding, otherwise what ‘target’ means. |

# 6 MRSS

At the last RAN1 meeting, MRSS aspect was discussed and the following agreement was made:

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

Not only the frame structure as stated in the SID, a number of companies mention that 5G compatible design should be ensured for MRSS, including waveform, modulation, numerology, channel coding, duplexing, and so on. All these aspects are discussed in other AIs in RAN1, and hence, moderator suggests considering MRSS aspects when discussing the above topics in other AIs 11.3.1, 11.3.2, 11.4.1, 11.4.2.

Companies provide lessons learned from LTE-NR DSS, including but not limited to

- legacy and practical restrictions due to “always-on” signals like LTE CRS

  - Caused overhead and reduced NR PDCCH capacity

  - But already removed from NR

- The maximum number of rate-matching patterns of PDSCH

  - too limited and thus costs inefficient inter-RAT resource sharing

- The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI

  - costs inefficient inter-RAT resource sharing

- Rate-matching patterns in the first release of NR

  - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell

- overall overhead from operating both RATs on the same carrier

  - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated

- SDM was not considered

  - SDM between 5G and 6G users would allow maximum flexibility for resource allocation

- Interoperability issues between different vendors

  - timing mismatches may cause signal collisions, reduced throughput.

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

#### [Old]Proposed observation 6.1:

- The lessons learned from LTE-NR DSS include, but not limited to

  - legacy and practical restrictions due to “always-on” signals like LTE CRS

    - Caused overhead and reduced NR PDCCH capacity

    - But already removed from NR

  - The maximum number of rate-matching patterns of PDSCH

    - too limited and thus costs inefficient inter-RAT resource sharing

  - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI

    - costs inefficient inter-RAT resource sharing

  - Rate-matching patterns in the first release of NR

    - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell

  - overall overhead from operating both RATs on the same carrier

    - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated

  - SDM was not considered

    - Limited flexibility for resource allocation

  - Interoperability issues between different vendors

    - timing mismatches may cause signal collisions, reduced throughput.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic |  | On "the restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI", instead of "cost inefficient", it could be "resource inefficient"? On the bullet point of "Rate-matching patterns in the first release of NR". LTE-CRS of the same cell would not be required to be considered. We would like to know whether it can be applicable also to neighbour cells. |
| Spreadtrum | Y with updates | From our perspective, rate matching patterns is not only used to avoid LTE-CRS, but also used to avoid channels or reference signals of NR itself; thus, the 2nd/3rd bullets are the lesson of rate matching patterns in NR, which is more related PDSCH resource mapping. It should be discussed under agenda items 11.9. So we suggest to delete the 2nd/3rd bullets. For the 4th bullet, rate-matching patterns in the first release of NR is not clear. It should be emphasized as the LTE CRS rate-matching patterns. For the fifth bullet, “overall overhead” is not clear. As the overhead of LTE-CRS is already mentioned in first bullet while NR signaling overhead (e.g., NR SSB) is marginal. So we suggest to delete this bullet as well. - The lessons learned from LTE-NR DSS include, but not limited to - legacy and practical restrictions due to “always-on” signals like LTE CRS - Caused overhead and reduced NR PDCCH capacity - But already removed from NR - The maximum number of rate-matching patterns of PDSCH - too limited and thus costs inefficient inter-RAT resource sharing - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI - costs inefficient inter-RAT resource sharing - LTE-CRS rate-matching patterns in the first release of NR - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell - overall overhead from operating both RATs on the same carrier - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated - SDM was not considered - Limited flexibility for resource allocation - Interoperability issues between different vendors - timing mismatches may cause signal collisions, reduced throughput. |
| Google |  | We think one additional point is that rate matching does not consider the beamforming impact. Different from LTE, in 5G, the SSB is beamformed. |
| Lenovo | N | Divide this proposal between semi-static and dynamic resource sharing. Semi-static resource sharing was implemented in the field which has less impact to the inter-RAT scheduler,, however doesn’t provide the resource efficiency during peak load at each RAT at the same time. Dynamic resource sharing has more impact to the inter-RAT scheduler, offers resource efficiency using rate matching, however no or less field implementation. The market needs to be captured clearly not from the previous specification, The lessons learned from LTE-NR DSS include - Semi-static: Hard resource split between RATs with less impact to the inter-RAT scheduling. Less Resource efficiency during peak load at the same time in both RATs. - Dynamic: increased resource efficiency using rate matching with inter-RAT scheduling coordination. |
| OPPO | Comment | We tend to agree to some of the lessions learnt from LTE-NR DSS as listed, but not all. We suggest to modify the proposed observations as followed. - The lessons learned from LTE-NR DSS include, but not limited to - legacy and practical Scheduling restrictions, signalling overhead and reduced NR PDCCH capacity due to “always-on” signals like LTE CRS and SSB - Caused overhead and reduced NR PDCCH capacity - But already removed from NR - Inefficiency in inter-RAT resource sharing from limited The maximum number of rate-matching patterns of PDSCH and the restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI - too limited and thus costs inefficient inter-RAT resource sharing - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI - costs inefficient inter-RAT resource sharing - Inter-cell interference caused by LTE-CRS of neighbouring cells is not resolved by NR rate-matching patterns in the first release of NR - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell - overall overhead from operating both RATs on the same carrier - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated - SDM was not considered - Limited flexibility for resource allocation - Interoperability issues between different vendors timing mismatches may cause signal collisions, reduced throughput. |
| Apple |  | Okay |
| Samsung |  | The observations from DSS themselves need to be discussed. But problem is that most are not relevant to NR-6GR coexistence (e.g., although AO signals are present in NR, none of such signals incur the large/excessive overhead comparable to LTE CRS). It may be better to directly discuss what NR-6GR MRSS needs. Also, focus should be on FR1. In addition, it is not clear why “SDM is not considered” is a lesson-leaned from LTE-NR DSS. Does this imply that NR-6GR MRSS should by default consider/include SDM as a means of resource sharing (which currently seems opportunistic at best in terms of applicability)? |
| CEWiT | Y with Modification | We support the intent of the proposal. First bullet is not related to the 5G NR but from DSS point of view can be captured in single line as below, “Legacy and practical restrictions due to “always-on” signals like LTE CRS which is removed in NR”. |
| HONOR |  | OK |
| CMCC | Y with comments | From our perspective, this is another lesson learned from LTE-NR DSS. Rate-matching patterns only apply for RRC_CONNECTED UE, which prevents efficient cross-RAT resource sharing: - Idle/inactive UEs cannot decode dynamic rate matching patterns (e.g., DCI-based indications), leaving them unaware of MRSS-reserved resources - To avoid potential collisions, the channels/signals pre-allocated for idle/inactive UE operations cannot be dynamically shared with other RAT, leading to resource waste. So we suggest to add the above observation. - The lessons learned from LTE-NR DSS include, but not limited to - …… - Rate-matching patterns only apply for RRC_CONNECTED UE - Channels/signals pre-allocated for idle/inactive UE operations cannot be dynamically shared with other RAT |

| ZTE |  | The proposal seems a bit trivial from our view. The LTE CRS issues do not exist in 5G-6G MRSS anymore, we don’t need to mention it. However, NR PDCCH, especially located within the first 3 OFDM symbols, shall be considered for 6GR PDCCH, which is similar to the LTE PDCCH region avoided by NR PDCCH. The wording ‘too limited and thus costs inefficient inter-RAT resource sharing’ caused confusion, it is unclear why resource sharing is related to the number of RM patterns. Besides RM for PDSCH, PDCCH/PUSCH RM should be also studied. Also, SDM is unclear, is it MU-MIMO for PDCCH or PDSCH with orthogonal DMRS ports ? Furthermore, we suggest not to have 3 proposals, i.e. combine the proposal 6.1, 6.2 and 6.3 together. Based on that, we have the following suggestion: - Study the following bullets for 5G-6G MRSS, but not limited to - Rate matching design for 6GR PDSCH, including - The maximum number of rate-matching patterns of PDSCH - Whether to support semi-static and/or dynamic RM patterns - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs - Rate matching design for signals/channels other than PDSCH, e.g. PUSCH, PDCCH |
| --- | --- | --- |
| InterDigital |  | OK |
| LGE |  | Several companies have shown interest in addressing inter-vendor scenarios and/or non-co-located cases. One key consideration in these situations is that RATs may be loosely coordinated, which can impose constraints on dynamic spectrum sharing. This issue is closely related to the second bullet point in Proposal 6.2. Therefore, we would like to suggest adding the following bullet points to the proposal. Constraints on using DSS in scenarios with loosely coordinated RATs |
| KDDI |  | OK |
| SONY |  | An additional observation we have is that for 4G-IoT, the legacy control channel region (i.e. LTE PDCCH, PHICH etc) does not need to be reserved in DSS. Hence, DSS with 4G-IoT is more efficient than DSS with 4G-MBB. [4G-IoT covers eMTC and NB-IoT in this context] |
| CATT |  | The following should be removed: - overall overhead from operating both RATs on the same carrier - impacted degraded the overall spectrum efficiency and made DSS less attractive than anticipated - natural consequence when deploying 2 RAT in the same frequency - SDM was not considered - Limited flexibility for resource allocation - Unrealistic to pursue SDM since they are different systems, service flows and even different antenna assumptions - Interoperability issues between different vendors - timing mismatches may cause signal collisions, reduced throughput. - Unclear if this is common to both FDD and TDD systems and why this will happen when we already assume same site deployment |

#### Proposed observation 6.1a:

- The lessons learned from LTE-NR DSS include, but not limited to

  - legacy and practical restrictions due to “always-on” signals like LTE CRS

    - Caused overhead and reduced NR PDCCH capacity

    - But already removed from NR

  - The maximum number of rate-matching patterns of PDSCH

    - too limited and thus caused inefficient inter-RAT resource sharing

  - The restriction of no overlap between rate-matching pattern and PDSCH DMRS REs derived from DCI

    - Caused inefficient inter-RAT resource sharing

  - LTE-CRS rate-matching patterns in the first release of NR

    - cannot resolve any inter-cell interference caused by LTE-CRS of neighbouring cell

  - Rate-matching patterns only apply for RRC_CONNECTED UE

    - Channels/signals pre-allocated for idle/inactive UE operations cannot be dynamically shared with other RAT

  - overall overhead from operating both RATs on the same carrier

    - degraded the overall spectrum efficiency and made DSS less attractive than anticipated

  - SDM was not considered

    - Limited flexibility for resource allocation

  - Interoperability issues between different vendors

    - timing mismatches may cause signal collisions, reduced throughput.

Regarding the high-level aspects which impact on the NR-6GR MRSS support, following views are provided

- General

  - UE/NW implementation complexity

  - Scheduler coordination

    - Including Multi-vendor (e.g., Inter-DU) operation

  - Traffic pattern

  - Radio resource utilization

    - Including PDCCH capacity

  - No impact on legacy NR UE behavior

  - MRSS should not limit 6GR design, and can be postponed after basic 6GR design is defined

- Operating bands/carriers

  - unified MRSS migration technique across all the bands

  - Single shared carrier MRSS, MRSS + 6G-only multicarrier aggregation, UL-only on MRSS with DL on 6G-only carrier

- Resource split/sharing

  - Study 6GR resource allocation to flexibly utilize resources not occupied by 5G-NR in an MRSS carrier

    - Including slot and mini-slot based scheduling

  - Opt0: Semi-static TDM/FDM

    - Also for NB-IoT and eMTC

  - Opt1: Signal sharing

    - Pros

      - Reduced resource overhead, including SSB, CORESET

      - Enhancing 6G UE performance by leveraging 5G reference signals received by the UE

    - Cons

      - Limit 6GR signal design, including EE and coverage

      - Complicate UE implementation

  - Opt2: Rate-matching

    - Pros:

      - Similar to LTE-NR DSS

    - Cons

      - (Not identified from contributions)

  - Opt3: SDM

    - Pros

      - SDM between 5G and 6G users would allow maximum flexibility for resource allocation

    - Cons

      - For cross-RAT SDM (assuming same overhead for 5G and 6G DMRS and only time/frequency multiplexing between DMRSs), both 5G and 6G suffer approximately 14% overhead increase

According to the input, following proposals can be considered as starting point

#### [Old]Proposal 6.2:

- High-level aspects which impact on the NR-6GR MRSS support include, but not limited to

  - UE/NW implementation complexity

  - Scheduler coordination

  - Traffic pattern

  - Radio resource utilization

  - No impact on legacy NR UE behavior

  - Operating bands/carriers

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic |  | We would like to add one bullet on whether NR and 6GR TRP co-location asepcts. |
| Spreadtrum | Y | Support |
| China Telecom | Y |  |
| Google |  | We think the “traffic pattern” in the proposal should be clarified. Some examples could be helpful. |
| Lenovo |  | Kindly add market needs as one bullet, so not all options of DSS was implemented especially the dynamic resource sharing die to the implementation complexity. - UE/NW implementation complexity and market needs |
| OPPO | Comment | Agree with Panasonic’s comment. We are not clear how traffic pattern (e.g., periodic, aperiodic) of a UE impacts the support of NR-6GR MRSS. Furthermore, from reading the contributions submitted to this meeting, a number of companies also mentioned about aligned numerology, aligned frame/slot/symbol boundaries, collocation of NR and 6GR BS/TRP, and NR-6GR MRSS operating bands (in FR1 only). We see these high-level aspects / assumptions should be addressed and concluded from the beginning. In terms of “Single shared carrier MRSS, MRSS + 6G-only multicarrier aggregation, UL-only on MRSS with DL on 6G-only carrier”, this should be categorized as operating scenarios. Lastly, we are not sure how can the issue of multi-vendor (e.g., Inter-DU) operation can be studied and resolved in 3GPP. Therefore, we suggest the following modifications. - High-level aspects which impact on the NR-6GR MRSS support include, but not limited to - Numerology impact/alignment - Frame/slot/symbol boundary impact/alignment - Operating bands (e.g., FR1) - UE/NW implementation complexity - Scheduler coordination - Traffic pattern - Efficiency of radio resource utilization - No impact on legacy NR UE behavior - Operating bands/carriers - NR and 6GR TRP co-location |
| Fujitsu | Y | Numerlogy configuration between NR and 6GR is one of the important issues in the MRSS topic. So, we would like to add it into the last bullet as - Operating bands/carriers including numerology configuration |
| Fraunhofer | Y |  |
| Apple |  | Okay |
| Nokia |  | We agree that there is a need to clarify what is meant by “traffic pattern” here. |
| Samsung |  | OK but suggest slight modification for subbulet - No spec impact on legacy NR UE behavior And ssuggest to add subbullet - Signelling overhead And remove the last subbuleet “Operating bands/carriers” which is not clear what to impact on MRSS |
| HONOR |  | OK |
| CMCC |  | We propose to remove the following bullets which are not clear what needs to be studied in RAN1. - Scheduler coordination - Traffic pattern - Operating bands/carriers |
| ZTE | N | This proposal does not provide much information for study. Suggest to combine the three proposals we suggested above. |
| InterDigital |  | We prefer to include “Energy efficiency of NW” as it is general design consideration for 6GR and it can apply for MRSS operation as well. |
| ETRI | Y |  |
| Fainity |  | Share the same view with Nokia |
| TCL |  | We believe that the “traffic pattern” should be clarified. |
| LGE |  | As we highlighted in the proposed observation 6.1, scheduler coordination has a significant impact on NR-6G MRSS. Therefore, we prefer to retain the term "scheduler coordination" in the proposal. Additionally, we are curious whether performance impacts—such as peak throughput—are being considered in this proposal. Since overhead from 5G can affect 6G throughput and vice versa, aiming for higher throughput should be one of the key objectives to be taken into account. |
| CATT | N | Same view as CMCC. |

#### Proposal 6.2a:

- High-level aspects which impact on the NR-6GR MRSS support include, but not limited to

  - UE/NW implementation complexity

  - Scheduler coordination

  - Traffic pattern

  - Radio resource utilization

  - No impact on legacy NR UE behavior

  - Operating bands/carriers

  - NR and 6GR TRP co-location

  - Signalling overhead

  - NW Energy efficiency

  - Numerology impact/alignment

  - Frame/slot/symbol boundary impact/alignment

#### [Low]Proposal 6.3:

- For the radio resource utilization for NR-6GR MRSS support, RAN1 to study the Pros/Cons of the following options

  - Opt0: Semi-static TDM/FDM between NR and 6GR

  - Opt1: NR signal sharing with 6GR

  - Opt2: Rate matching of 6GR signals/channels around NR signals/channels

  - Opt3: SDM between NR and 6GR

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be discussed as second priority, since the highest priority in this meeting is to identify the high-level aspects which impact on the NR-6GR MRSS support, as agreed in the last RAN1 meeting |
| Spreadtrum |  | For Opt 1, we have some concern the impact of 6GR UE and NR UE, so we suggest only studying tranparent sharing. For Opt2, it means dynamic spectrum sharing, so it should be revised as dynamic TDM/FDM between NR and 6GR, including rate matching of 6GR signals/channels around NR signals/channels. The suggested updates are as below with red. - For the radio resource utilization for NR-6GR MRSS support, RAN1 to study the Pros/Cons of the following options - Opt0: Semi-static TDM/FDM between NR and 6GR - Opt1: Tranparent sharing the signal between NR and 6GR - Opt2: Dynamic TDM/FDM between NR and 6GR, including rate matching of 6GR signals/channels around NR signals/channels - Opt3: SDM between NR and 6GR |
| Chian Telecom |  | We have concern on Opt0, it may result in low resource utilization rate. |
| Google |  | For opt1, we think it could be partly or fully sharing. For example, 6GR may support more ports and 5G signal may support fewer ports. The sharing is only partially sharing. |
| Lenovo |  | Signal sharing is not MRSS, 6GR should aim to design for non-MRSS case first. Another concern, is the RAT identification in case of signal sharing and impacts on UE complexity. Kindly delete signal sharing from MRSS, as MRSS means resource sharing between RATs. |
| OPPO | Comment | Althought the FL has listed this proposal (options) as low priority for this meeting, but we believe at least Opt 0 (Semi-static TDM/FDM between NR and 6GR) will have some impacts to the MRSS study in RAN4. For other options, each company can continue their study until the next meeting. Therefore, we suggest to modify the proposal as follow. - For NR-6GR MRSS, semi-static TDM and FDM between NR and 6GR is feasible from RAN1’s perspective. Further study the feasibility and Pros/Cons of the following options. - Rate matching of 6GR signals/channels around NR signals/channels - NR signal sharing with 6GR SDM between NR and 6GR |
| Fujitsu | Y | We support FL’s proposal |
| Apple |  | Okay |
| Nokia |  | It is unlikely that the final solution would be based on any of the options exclusively, so studies are needed on many directions. The options in the list are not directly comparable, so further discussions are needed. |
| Samsung |  | Opt0 is sufficient (at least to start with). No need to consider Opt3, not meaningful for FR1. Even if FR2 were to be considered, the applicability of SDM for MRSS in general is opportunistic at best. Opt1 should not be supported as a design criterion for 6G, especially no sharing of NR SSB and 6GR SSB. Spec-transparent sharing may be considered after independent design of 6GR signals/channels based on 6GR KPIs. |
| Ericsson |  | The options listed are not necessarily mutually exclusive. It is important to design an efficient 5G-6G MRSS scheme which requires a very dynamic sharing between the two. We also need to discuss how to “hide” 6G signals from 5G UEs (for data transmission, scheduling can be used, but for initial access we need to rely on the mechanisms supported by deployed 5G UEs). |
| HONOR | Y | OK |
| CMCC |  | For option1, we propose to remove it. 6GR design should not be restricted by 5G design. And other options are not mutually exclusive, for option2, we would like to add subbullet as follows. - Opt2: Rate matching of 6GR signals/channels around NR signals/channels - At least consider semi-static rate matching pattern |
| ZTE | N | In option 0, MRSS includes dynamic radio resource sharing. We don’t understand why option 0 is only focusing on semi-static way. Anyway, TDM/FDM is implementation issue. In option1 for 6GR sharing NR signals/channels, this would introduce strong restrictions for 6GR signals/channels design. At this stage, it is better not to consider this general restriction before designing specific 6GR signals/channels. In other words, it should be discussed case by case for each channel / signal, e.g. SSB, PDCCH, CSI-RS and DMRS may have different considerations. In option 2, it has been reflected in proposal 6.1. Option 3 is unclear. |
| InterDigital | Y | OK |
| KT | N | Option1 and Option2 can be merged, such as “Dynamic sharing between NR and 6GR” that aligned with other options in high-level. Additionally, Option 1 and 2 may not be strictly divided at this stage. In our view, both Option 1 and Option 2 can be considered at the same time w.r.t. types of signals/channels. We propose a merged option as following: - For the radio resource utilization for NR-6GR MRSS support, RAN1 to study the Pros/Cons of the following options - Opt0: Semi-static TDM/FDM between NR and 6GR - Opt1: Dynamic sharing between NR and 6GR - NR signal sharing with 6GR - Rate matching of 6GR signals/channels around NR signals/channels - Opt2: SDM between NR and 6GR |
| ETRI |  | We don’t think these options are mutually exclusive. We should also consider possible combinations of the options. |
| Fainity |  | OK with the proposals and further suggest the analysis shall be separated for different channels (e.g., PBCH, PDSCH) |
| LGE |  | In terms of 'sharing', sharing can imply not only the sharing of signals and transmission itself, but also cooperation in using the same Tx/Rx method over the same resources. For example, 5G and 6G RATs may use the same time/frequency resources for the same purpose, such as CORESET, and utilize them for system overhead and collision avoidance. Given that, the scope of 'sharing' should be expanded from signal-level sharing to signal/channel-level sharing, at least for study/discussion purposes. Opt1: NR signal/channel sharing with 6GR |
| KDDI |  | First, as several companies have commented, it should be explicitly stated that these options are not mutually exclusive and that multiple options can be utilized. Regarding Option 1, we understand that this option imposes constraints on the 6G signals/channels design. However, one lesson learned from LTE-NR DSS is overall overhead from operating both RATs on the same carrier. Based on this lesson, we should also consider the option of NR signal sharing with 6GR to the extent possible. Considering these points, we currently share ZTE’s perspective. That is, we believe Option 1 requires discussion on a per-channel/per-signal basis. |
| CATT | N | Opt1 is a strong limitation on 6GR design and fail to make a generation-level improvement Opt3 is unclear. It is even unrealistic due to difference among systems regarding traffic load, RS design, etc. |

# 7 Synchronization signal structure and periodicity

At the last RAN1 meeting, SS structure and periodicy aspects were discussed and the following agreement was made:

Agreement - Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Initial access aspects are planned to be discussed from RAN1#124 (Feb. 2026). Therefore, in general, it is better to wait for RAN1#124 to open the discussion on initial access aspects. However, the Interim Milestone states that RAN1 needs to provide interim assessment on the basic sync signal structure and associated periodicity(ies) by June 2026. Therefore, at least on these aspects, RAN1 should start discussion earlier to provide enough assessment.

Regarding the SS structure, a number of companies mentioned that it needs to consider

- Reduced number of sync raster: for NES and UE complexity

- Support of low-tier 6G device: for smallest maximum supported UE BW

- Support of minimum spectrum allocation: punctured SS vs specific design for the spectrum as discussed in Section 4

- Detection performance: If narrower SSB BW is considered, more OFDM symbols would be required to maintain the NR performance

- Ensure orthogonalization against the NR PSS/SSS design: to avoid UE accessing unintended RAT

- Extended coverage: unclear coverage target as discussed in Section 5

- Low complexity/power SS

- decoupling for different RRC states

- multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand)

- NTN aspects (to be discussed in Section 10)

- Scalability to operate on the supported deployments and spectrum, including multi-beam operation

- Compatibility with any duplex modes, e.g., SBFD

Regarding the SS periodicity, quite a few companies propose larger periodicity for NES while considering how to address UE complexity (initial cell search, PSS/SSS detection requirements, access latency). Moderator assumes this aspect can be discussed in AI 11.6:EE since no other aspects were raised for the periodicity

#### [Old]Proposal 7.1:

- High-level aspects which impact on the 6GR sync signal structure include, but not limited to

  - Reduced number of sync raster

  - Support of low-tier 6G device

  - Support of minimum spectrum allocation

  - Detection performance

  - Ensure orthogonalization against the NR PSS/SSS design

  - Extended coverage

  - Low complexity/power SS

  - decoupling for different RRC states

  - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand)

  - Scalability to operate on the supported deployments and spectrum, including multi-beam operation

  - Compatibility with any duplex modes

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic | Y |  |
| Spreadtrum | Y with updates | We are general fine with the proposal and some updates are needed. Fristly, the content/payload of 6GR sync signal also have significant impacts on the 6GR sync signal structure. Secondly, impacts on 6GR sync signal structure from harmonization of TN and NTN should be considered in this agenda, Finally, some aspects (e.g., Reduced number of sync raster) require guidance from RAN4. Therefore, we suggest to make it as follows: #### Proposal 7.1: - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - The content/payload of 6GR sync signal - Reduced number of sync raster - Support of low-tier 6G device - Harmonization of TN and NTN - Support of minimum spectrum allocation - Detection performance - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam operation - Compatibility with any duplex modes - Note: Aspects impacting on the periodicity is to be discussed under AI11.5 - Note: RAN4 involvement is necessary |
| Google |  | For the sixth bullet “detection performance”, we think the tracking performance (time/frequency tracking performance) should also be considered. |
| Lenovo |  | Kindly add latency - Detection performance and latency |
| OPPO |  | As we are discussing some apests that have to be considered, we suggest to only keep the following: - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster - Support of low-tier 6G device - Support of minimum spectrum allocation - Detection performance - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage target - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam operation - Compatibility with any duplex modes - Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| Fujitsu | Y | This proposal seems a good start. |
| Fraunhofer |  | The following note is unclear to us since 11.5 is supposed to end by next meeting and the aspects impacting periodicity and energy efficiency in general should continue to the initial access discussions from RAN1#124 onwards in 11.7 “Note: Aspects impacting on the periodicity is to be discussed under AI11.5” |
| Apple |  | In general, it is better to let initial access agenda to handle this topic. Some comments Ensure orthogonalization low correlation against the NR PSS/SSS design |
| Nokia |  | The list includes aspects impacting sync signal structure and general requirements, like “decoupling for different RRC states”. It is unclear how all of those relate to the signal structure itself, and how to take such list into account for sync signal design. We would suggest consider also additional aspects e.g. how to enable mobility measurements, facilitate/enable time and frequency tracking, possibility of having different periods for PSS/SSS etc. and also how the timing acquisition is enabled. Thus we propose to add following points: - Relation to mobility measurements - Relation to time and frequency tracking - Same or different periodicities for PSS, SSS, PBCH - PSS, SSS, PBCH relation to timing acquisition (symbol, slot, frame) |
| Samsung |  | OK under the understanding that the list is only for purposes of discussions and is not intented as a direct agreement for study. - “Ensure orthogonalization against the NR PSS/SSS design” is about the sequence design of 6GR, which should not be listed as the aspect impacting the 6GR sync singal structure. “Extended coverage” is confusing – does it mean extended coverage comparing to NR SSB? If so, we think it is premature to determine 6GR sync signal would have extended coverage comparing to NR. |
| Ericsson |  | Although the SSB periodicity (e.g. 160 ms) is discussed under agenda item 11.5, it has an impact on the SSB design that needs to be taken into account. |
| CEWiT |  | We do not agree with all the point especially we do not agree with “Ensure orthogonalization against the NR PSS/SSS design”. As a part of proposal 6.3 it is possible to reuse the initial access structure for NR and 6GR for optimization perspective which will help in reducing sync raster and device complexity too. So we propose to remove this bullet. Further, we would like to add energy efficiency (e.g, to include SSB periodicity etc) as one of the aspect that impact SS design (Add as a sub-bullet), even though it will be discussed under EE agenda. |
| HONOR | Y | OK |
| CMCC |  | We propose some modifications on the proposal. - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster - Support of all devices low-tier 6G device - Support of minimum spectrum allocation and - Detection performance - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam operation - Compatibility with any duplex modes Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| ZTE |  | Thanks for moderator’s nice summary. We tend to agree that the sync singal structure desing for supporting various deployments and spectrum is critical. While reviewing companies’ contribution, a list of majority companies prefers to consider to improve the sync-signal structure for accommodating a cluster of cells under cell-free operation (e.g., to improve cell-edge/average UPT, reduce latency, achieve NW/UE energy saving). Additionally, regarding the periodicity, based on chair’s views in last meeting, only the NES related motivation is discussed in NES, other purposes, e.g., NTN, reduced overhead for common channel, can still be treated here. For the other aspects, e.g., Low complexity/power SS and multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand), i assume that it’s more related to NES agenda. Another point is that we need to check the possibility with more SSB number to support various deployment. Then, regarding 7.1, we have the following suggestions: - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster - Increasd number of SSBs - Extension of SSB periodicity - Support of low-tier 6G device - Support of minimum spectrum allocation - Detection performance - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including intra/inter-cell-cluster multi-beam/multi-TRP operation/mobility - Compatibility with any duplex modes - Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| InterDigital |  | We are ok in general except for the following bullet at this point. - Ensure orthogonalization against the NR PSS/SSS design |
| CMCC2 |  | After reading the comments above, we suggest the following update to consider the scalability to support multi-TRP (cell-free) deployment. - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster - Support of all devices low-tier 6G device - Support of minimum spectrum allocation and - Detection performance - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam/multi-TRP (cell-free) operation - Compatibility with any duplex modes Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| ETRI | Y | Generally OK as a starting point |
| Fainity |  | We share the same view with Ericsson that SSB periodicity and its performance should be investigated and then identify other enhancement/structure is needed. |
| TCL |  | We are ok in general except for the following bullets: - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) |
| LGE |  | We are fine with the proposal 7.1. to indentify the high-level aspected which impact on 6GR sync signal design. Importance thing to should be taken into account for 6GR sync signal design are as follows: - latency to detect cell ID - UE detetion complexity Please add these things on the list. |
| IMU |  | We are generally fine with the current proposal except the following suggestion: Proposal 7.1: - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster and bandwidth - Support of low-tier 6G device - Support of minimum spectrum allocation - Detection performance and latency - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage and increased number of OFDM symbols for PBCH - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam operation - Compatibility with any duplex modes - Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| Xiaomi |  | The bullets are not on the same level. Some of them are requirements, and some of them are highly technique specific. At this stage, we should avoid touching technical design. We propose the following updates: - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster - Support of low-tier 6G device - Support of minimum spectrum allocation - Detection performance - UE cell search latency and complexity - Ensure orthogonalization against the NR PSS/SSS design - Extended coverage - MRSS - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam operation - Compatibility with any duplex modes - Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| CATT |  | We think some of the bullets are lack of consensus: - High-level aspects which impact on the 6GR sync signal structure include, but not limited to - Reduced number of sync raster - Support of low-tier 6G device - Support of minimum spectrum allocation - Detection performance - Ensure orthogonalization against the NR PSS/SSS design - Extended Satisfied coverage - Low complexity/power SS - decoupling for different RRC states - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand) - Scalability to operate on the supported deployments and spectrum, including multi-beam operation - Compatibility with any duplex modes - Note: Aspects impacting on the periodicity is to be discussed under AI11.5 |
| vivo |  | We should add “initial search complexity and time” as one important aspect related to user experience for consideration. |

#### Proposal 7.1a:

- High-level aspects which impact on the 6GR sync signal structure include, but not limited to

  - Reduced number of sync raster

  - Support of low-tier all 6G devices

  - Support of minimum spectrum allocation

  - Detection/tracking performance, latency, and complexity

  - Ensure orthogonalization low correlation against the NR PSS/SSS design

  - Extended coverage target

  - Low complexity/power SS

  - decoupling for different RRC states

  - multi-stage SS structure in 6GR initial access (e.g., always-on + on-demand)

  - Scalability to operate on the supported deployments and spectrum, including multi-beam/TRP operation

  - Compatibility with any duplex modes

  - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

# 8 Operation of bandwidth/band adaptation

At the last RAN1 meeting, operation of bandwidth/band adaptation was discussed and the following agreement was made:

Agreement - Study and identify the lessons learned from NR BWP framework

Companies provide lessons learned from NR BWP framework, including but not limited to

- A lot of potential uses, including adaptation to traffic demands and energy savings

- A lot of RRC parameters under BWP configuration

  - results in unnecessarily large overhead

- BWP switching delay

  - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides

  - UPT loss and increased UE power consumption

- BWP switching

  - less motivated, for other than CORESET switching

  - will cause misalignment of real active BWP between BS and UE

  - results in unnecessary HARQ-ACK dropping

- SCS switching

  - complicated but less motivated.

- Excessive BWP types

  - including BWP types that have not been effectively used in practical NW, e.g., default BWP, dormant BWP.

- Center frequency of DL/UL BWP

  - unnecessarily common

- lack of RAN4 involvemen

  - leading to large MPR/A-MPR

- Inherent restrictions

  - When a BWP is not covering the sync signal bandwidth, it can lead to different approaches for maintaining sync

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

#### Proposed observation 8.1:

- The lessons learned from NR BWP framework include, but not limited to

  - A lot of potential uses, including adaptation to traffic demands and energy savings

  - A lot of RRC parameters under BWP configuration

    - results in unnecessarily large overhead

  - BWP switching delay

    - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides

    - UPT loss and increased UE power consumption

  - BWP switching

    - less motivated, for other than CORESET switching

    - will cause misalignment of real active BWP between BS and UE

    - results in unnecessary HARQ-ACK dropping

  - SCS switching

    - complicated but less motivated.

  - Excessive BWP types

    - including BWP types that have not been effectively used in practical NW, e.g., default BWP, dormant BWP.

  - Center frequency of DL/UL BWP

    - unnecessarily common

  - lack of RAN4 involvemen

    - leading to large MPR/A-MPR

  - Inherent restrictions

    - When a BWP is not covering the sync signal bandwidth, it can lead to different approaches for maintaining sync

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic | Y |  |
| Spreadtrum | Y |  |
| Google |  | We would like to understand the issue of “lack of RAN4 involvement”. Some clarifications would be helpful. |
| OPPO | Y in general | In general, we are fine with the proposal. Some modifications are suggested below. BWP is still helpful for energy saving. Excessive BWP-specific RRC parameters result in not only the large overhead, but also the unnecessary RRC parameter options. Not clear why BWP switching results in misalignment of active BWP between NW and UE. Not clear why BWP switching results in unnecessary HARQ-ACK dropping. SCS switching is not needed in 6G because only a single SCS is supported per FR/sub-FR. - The lessons learned from NR BWP framework include, but not limited to - A lot of potential uses, including adaptation to traffic demands and energy savings - A lot of RRC parameters under BWP configuration - results in unnecessarily large overhead and unnecessary RRC parameter options - BWP switching delay - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides - UPT loss and increased UE power consumption - BWP switching - less motivated, e.g., for other than CORESET switching - will cause misalignment of real active BWP between BS and UE - results in unnecessary HARQ-ACK dropping - SCS switching - complicated but less motivated and not necessary for 6GR. - Excessive BWP types - including BWP types that have not been effectively used in practical NW, e.g., default BWP, dormant BWP. - Center frequency of DL/UL BWP - unnecessarily common - lack of RAN4 involvemen - leading to large MPR/A-MPR - Inherent restrictions - When a BWP is not covering the sync signal bandwidth, it can lead to different approaches for maintaining sync |
| Fujitsu | Y | We support FL’s proposal |
| Apple |  | Okay |
| Nokia |  | The list is a good starting point, but some aspects require further clarification. For example, we do not agree with the statement that “BWP switching less motivated, for other than CORESET switching” |
| Samsung |  | Geneally OK but we do not agree that the same center frequency for DL/UL BWP (in TDD only) is unnecessary. On top of the suggested proposal, we would like to also add excessive and widespread specification impact from DCI-based BWP switching. Also, suggest to remove “will cause misaslignemnt on real active BWP between BS and UE” in bullet of BWP switching, this issue is not BWP’s own problem, it is general issue for all features. |
| HONOR | Y | OK |
| ZTE |  | The motivation of BWP may also include BW adaption. - less motivated, for other than CORESET switching and BW adaption |
| InterDigital |  | “BWP switching” bullet is bit unclear for us. Was it less motivate due to the complexity and latency in 5G-NR or due to not meaningful energy saving from changing UE operating bandwidth as it as one of the motivation of BWP switching. |
| ETRI | Y | Generally OK |
| LGE |  | We have similar view as Nokia and ZTE. We don’t think that BWP switching for the purpose of UE operating BW adaptation itself was less motivated (and can’t assure unnecessary for 6GR), therefore it is better to remove the corresponding bullet. - BWP switching - less motivated, for other than CORESET switching - will cause misalignment of real active BWP between BS and UE - results in unnecessary HARQ-ACK dropping In addition, the following needs to be included in above Observation 8.1. - Bandwidth of DL/UL BWP - unnecessarily contiguous bandwidth |
| Xiaomi |  | From our perspective, we think the most important lessons we learned from NR BWP include: - excessive BWP-specific configurations - long BWP switch latency - DCI-based BWP switching reliability |
| SONY | Y |  |
| CATT |  | Suggestions to make it clear: - BWP switching delay - too large due to the assumption that all RF/BB parameters of new BWP are re-loaded at UE sides - UPT loss and increased UE power consumption - BWP switching - less motivated, for other than CORESET switching and bandwidth adjustment - DCI-based BWP switching will cause misalignment of real active BWP between BS and UE - results in unnecessary HARQ-ACK dropping - Center frequency of DL/UL BWP in TDD - unnecessarily common |
| vivo |  | The current BWP switching framework reuiqres gNB to perform cross-BWP scheduling during BWP swiching, which causes unnecessary complexity and restriction. Suggest to add the following bullet: - Complexity and scheduling restriction due to cross-BWP scheduling for the DL/UL grant indicating BWP switching |

Accroding to the lessons learned from NR BWP framework, companies further propose how to improve BWP framework in 6GR, including but not limited to

- Support simplified BWP framework

  - Only essential/relevant configurations under BWP configurations

  - Single SCS per BWP

  - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP

  - No dynamic BWP switching

  - Minimize the number of BWP types

  - in conjunction with other functionalities related to UE power savings

- Separate DL and UL BWP adaptation

- Inprove robustness, reduced latency and minimize interrupptions

- Target early RAN4 involvement

- Design BWP to support diverse device types in the same band during initial access

- discontinuous frequency resources within one BWP

- improving the performance when BWP location does not coincide with the synchronisation signal frequency

- Combined with TCI framework

- Reduced UE energy consumption

#### [Low]Proposal 8.2:

- Study how to improve BWP framework, including but not limited to

  - Simplified BWP framework

    - Only essential/relevant configurations under BWP configurations

    - Single SCS per BWP

    - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP

    - No dynamic BWP switching

    - Minimize the number of BWP types

    - in conjunction with other functionalities related to UE power savings

  - Separate DL and UL BWP adaptation

  - Inprove robustness, reduced latency and minimize interrupptions

  - Target early RAN4 involvement

  - Design BWP to support diverse device types in the same band during initial access

  - discontinuous frequency resources within one BWP

  - improving the performance when BWP location does not coincide with the synchronisation signal frequency

  - Combined with TCI framework

  - Reduced UE energy consumption

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be discussed as second priority, since the highest priority in this meeting is to identify the lessons learned from NR BWP framework, as agreed in the last RAN1 meeting |
| Panasonic |  | We would like to modify following bullet. - discontinuous frequency resources within one BWP and/or multiple BWPs for discontinuous frequency resources. |
| Spreadtrum |  | Fine with FL’s proposal. This proposal should be low priority for this meeting. Detailed studies can be discussed at later meeting. |
| Google |  | We think the single SCS should be per band not per BWP. It is too early to study the CORSET/SS related configurations. Further, we are uncertain what “BWP type” means. We are wondering what “combined with TCI framework” means. The TCI should be per CC instead of per BWP. |
| OPPO |  | In general, we are fine with the proposal. Some modifications are suggested below. Since only a single SCS is supported per FR/sub-FR, no need to configure SCS in BWP configuration. Whether dynamic BWP switching is supported can be further studied, e.g., in energy efficiency agenda. - Study how to improve BWP framework, including but not limited to - Simplified BWP framework - Only essential/relevant configurations under BWP configurations - SingleNo SCS perin BWP configuration - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP - FFS: whether to supportNo dynamic BWP switching - Minimize the number of BWP types - in conjunction with other functionalities related to UE power savings - Separate DL and UL BWP adaptation - Inmprove robustness, reduced latency and minimize interrupptions for BWP switching - Target early RAN4 involvement - Design BWP to support diverse device types in the same band during initial access - discontinuous frequency resources within one BWP - improving the performance when BWP location does not coincide with the synchronisation signal frequency - Combined with TCI framework - Reduced UE energy consumption |
| Fujitsu |  | Except potential benefits behind the enhancements, we think the cost/drawbacks of the enhancements should be considered/studied as well. We understand that almost every factor to improve for BWP corresponds a lesson learnt from 5G. But, to be frank, the purposes/effects of some potential enhancements may be conflicting. For example, the proposed feature of enabling discontinuous frequency resources within a single BWP, while beneficial for throughput enhancement, may not contribute meaningfully to another proposed enhancement, “Reduced UE energy consumption”. |
| Apple |  | Okay |
| Nokia |  | Regarding “no dynamic switching”, we need to separate what are the issues introduced by DCI-based switching and what level of robustness is needed for dynamic switching, and what level of delay is targeted for switching. Those are different aspects but they tend to get mixed in the discussions. An aspect that requires further clarification is “discontinuous frequency resources within one BWP”, as the motivation and baseline assumptions are not clear. |
| Samsung |  | Should first discuss whether 6GR should have the concept of BWP. The reason for the BWP introduction in Rel-15 was UE power savings. However, 6GR (and NR after Rel-15) considers other mechanisms that can make BWP redundant. |
| Ericsson |  | Before discussing “how to improve BWP framework” we should discuss the problem(s) we want to solve – is it UE energy consumption, is it the need to rapidly switch between RRC configurations, is it something else? Once we have some understanding of the problem to solve, we can discuss the solutions – not the other way around. In our view, there is a need to rapidly switch between RRC configurations in many areas (one of them being bandwidth adaptation for UE power saving) so designing a simple tool for this makes sense. |
| CEWiT | Y | Support |
| HONOR |  | OK |
| CMCC |  | Before discussing the details of how to improve BWP framework, we need to discuss first the purpose and/or motivation of BWP or bandwidth/band adaptation in 6GR design. |
| ZTE |  | SCS should be single across all BWPs of a carrier. Furthermore, we think it is to early to say ‘ no dynamic BWP switching’. Here is our suggestion: - Study how to improve BWP framework, including but not limited to - Simplified BWP framework - Only essential/relevant configurations under BWP configurations - Single SCS across per all BWPs within a carrier - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP - No dynamic BWP switching - Minimize the number of BWP - Minimize the number of BWP types - in conjunction with other functionalities related to UE power savings - Separate DL and UL BWP adaptation - Improve robustness, reduced latency and minimize interrupptions - Target early RAN4 involvement - Design BWP to support diverse device types in the same band during initial access - discontinuous frequency resources within one BWP - improving the performance when BWP location does not coincide with the synchronisation signal frequency - Combined with TCI framework - Reduced UE energy consumption |
| InterDigital |  | Similar question with 8.1. What was the key issue for BWP switching? BWP switching itself doesn’t provide much benefit or BWP switching leads to too much complexity/delay? If it was due to complexity/delay, we can find better way to address the issue in 6GR. |
| ETRI |  | The possibility of having more than one configuration in a single BWP should not be limited to CORESET/Search Space. Other parameters could also have multiple sub-configurations within a single BWP. Therefore, - Simplified BWP framework - Only essential/relevant configurations under BWP configurations - Single SCS per BWP - More than one sub-configurations (e.g., CORESET/Search Space, # RB) with dynamic switching feature in a single BWP - No dynamic BWP switching - Minimize the number of BWP types - in conjunction with other functionalities related to UE power savings |
| Fainity |  | We share the same view with most companies that more study is needed and prefer not to narrow down the potentials in current phase |
| LGE |  | Similar view as Nokia/ZTE also here on the BWP switching part. We don’t think at this early stage that the BWP switching is unnecessary for 6GR scenarios, operations and requirements. In addition, based on the lesson from 5G BWP configuration burden, we need to open on the possibility of decoupling between the BWP (i.e., UE operation BW) configuration and other RRC parameter configurations. For these reasons, the Proposal 8.2 needs to be updated as below. - Study how to improve BWP framework, including but not limited to - Simplified BWP framework - Only essential/relevant configurations under BWP configurations - Single SCS per BWP - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP - No dynamic BWP switching - Minimize the number of BWP types - in conjunction with other functionalities related to UE power savings - Separate DL and UL BWP adaptation - Inprove robustness, reduced latency and minimize interrupptions - Target early RAN4 involvement - Design BWP to support diverse device types in the same band during initial access - discontinuous frequency resources within one BWP - improving the performance when BWP location does not coincide with the synchronisation signal frequency - Combined with TCI framework - Reduced UE energy consumption |
| Xiaomi |  | Based on our comments to Proposed observation 8.1, the improvement of BWP in 6G includes - minimize BWP-specific configurations - reduce BWP switch latency - improve BWP switching reliability |
| SONY |  | A main goal should be simplification of the BWP framework, as per the proposal. Some typos: - Study how to improve BWP framework, including but not limited to - Simplified BWP framework - Only essential/relevant configurations under BWP configurations - Single SCS per BWP - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP - No dynamic BWP switching - Minimize the number of BWP types - in conjunction with other functionalities related to UE power savings - Separate DL and UL BWP adaptation - Improve robustness, reduced latency and minimize interruptions - Target early RAN4 involvement - Design BWP to support diverse device types in the same band during initial access - discontinuous frequency resources within one BWP - improving the performance when BWP location does not coincide with the synchronisation signal frequency - Combined with TCI framework - Reduced UE energy consumption |

# 9 Spectrum utilization and aggregation framework

At the last RAN1 meeting, spectrum utilization and aggregation framework were discussed and the following agreement was made:

Agreement - Study and identify the lessons learned from NR spectrum utilization and aggregation framework - DC is subject to RANP decision in June 2026 - Note: MRSS aspects are separate discussion

In addition, RAN#109 concluded the following:

Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. Proposal 2: 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

Note that following is captured in TR38.914 related to spectrum aggregation

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

Although the technical details on the above aspects can be further discussed in other agenda items to be discussed in RAN1 (e.g., “Initial access”, “Physical layer control, data scheduling and HARQ operation”, and “6GR spectrum utilization and aggregation”), it wold be better to discuss some high-level direction on how to improve the spectrum utilization and operations in this agenda items, because this issue has impact on multiple agenda items.

Companies provide lessons learned from NR spectrum utilization and aggregation framework, including but not limited to

- CA has been a very successful feature in LTE and NR

- Pcell vs Scell

  - Allowing some functionalities only on specific cell like PCell may limit resource utilizations and prevent a NW from entering deep sleep as early as possible on a cell

- Coupling DL and UL carriers for a cell

  - inefficient and ineffective due to different requirements and limitations between DL and UL

  - SUL/SDL, UL Tx switching, LBCA switching operate differently

  - SUL scheme is bound to dedicated SUL bands with UL-only resource

  - ensuring the presence of a corresponding downlink CC used as a reference for measurements

- UL Tx switching

  - did not incorporate all UL transmissions, complicating its use

  - mandates UE to support at least N DL CCs and the N DL CCs are activated, which leads to high DL capabilities requirement and high UE power consumption

- CA applicability

  - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times

  - did not sufficiently facilitate wide variety of deployments and network implementations but was designed to require challenging low latency inter-cell coordination

- SSB adaptation for Scell

  - SSB-less SCell operation

    - limited applicable scenario.

  - On-demand SSB SCell operation

    - limited applicable scenario.

- Activation of additional carrier

  - Slow not only because of signaling protocols and RAN4 requirements, but also because of very relaxed CSI accuracy for the newly activated carrier

  - faces a dilemma of choosing the high service latency caused by SCell activation and high UE power consumption by keeping SCell always activated

  - SCell dormancy

    - impractical as this feature is defined on top of BWP framework, which is unnecessarily flexible and complicated.

  - A-TRS trigger with SCell activation

    - not designed for NES.

- Features (such as HARQ) defined per carrier

  - prevents further improvements on user throughput and latency via cross-carrier operation

  - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving

- Avoid dependencies across carriers

  - such as DAI to simplify implementation and improve performance

- The maximum number of bands in NR multi-band operations

  - actually limited by the maximum UE RF+BB hardware capacity in commercial networks

- Concurrent transmissions of UL-CA/EN-DC

  - only beneficial for UEs who are close to gNB and have redundant UE Tx power and its symbol-by-symbol UL power control requires very tight coordination between PCell gNB and SCell gNBs.

  - need to require a semi-static UL power split for the UE in absence of gNB scheduler coordination.

  - Only supported for connected mode

- Fragmented spectrum

  - not efficiently utilized and latency is unnecessarily increased under NR CA framework

- Signalling overhead and UE processing complexity of PHY channels

  - scale with the number of aggregated carriers rather than the aggregated bandwidth size

- No support of efficient IDLE/INACTIVE modes offloading

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

#### Proposed observation 9.1:

- The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to

  - CA has been a very successful feature in LTE and NR

  - Pcell vs Scell

    - Allowing some functionalities only on specific cell like PCell may limit resource utilizations and prevent a NW from entering deep sleep as early as possible on a cell

  - Coupling DL and UL carriers for a cell

    - inefficient and ineffective due to different requirements and limitations between DL and UL

    - SUL/SDL, UL Tx switching, LBCA switching operate differently

    - SUL scheme is bound to dedicated SUL bands with UL-only resource

    - ensuring the presence of a corresponding downlink CC used as a reference for measurements

  - UL Tx switching

    - did not incorporate all UL transmissions, complicating its use

    - mandates UE to support at least N DL CCs and the N DL CCs are activated, which leads to high DL capabilities requirement and high UE power consumption

  - CA applicability

    - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times

    - did not sufficiently facilitate wide variety of deployments and network implementations but was designed to require challenging low latency inter-cell coordination

  - SSB adaptation for Scell

    - SSB-less SCell operation

      - limited applicable scenario.

    - On-demand SSB SCell operation

      - limited applicable scenario.

  - Activation of additional carrier

    - Slow not only because of signaling protocols and RAN4 requirements, but also because of very relaxed CSI accuracy for the newly activated carrier

    - faces a dilemma of choosing the high service latency caused by SCell activation and high UE power consumption by keeping SCell always activated

    - SCell dormancy

      - impractical as this feature is defined on top of BWP framework, which is unnecessarily flexible and complicated.

    - A-TRS trigger with SCell activation

      - not designed for NES.

  - Features (such as HARQ) defined per carrier

    - prevents further improvements on user throughput and latency via cross-carrier operation

    - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving

  - Avoid dependencies across carriers

    - such as DAI to simplify implementation and improve performance

  - The maximum number of bands in NR multi-band operations

    - actually limited by the maximum UE RF+BB hardware capacity in commercial networks

  - Concurrent transmissions of UL-CA/EN-DC

    - only beneficial for UEs who are close to gNB and have redundant UE Tx power and its symbol-by-symbol UL power control requires very tight coordination between PCell gNB and SCell gNBs.

    - need to require a semi-static UL power split for the UE in absence of gNB scheduler coordination.

    - Only supported for connected mode

  - Fragmented spectrum

    - not efficiently utilized and latency is unnecessarily increased under NR CA framework

  - Signalling overhead and UE processing complexity of PHY channels

    - scale with the number of aggregated carriers rather than the aggregated bandwidth size

  - No support of efficient IDLE/INACTIVE modes offloading

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic | Y |  |
| Spreadtrum | Y with updates | Firstly, some bullets are duplicated and update is needed. The lalency of SCell activation is general issue in NR CA, which is mentioned in “Activation of additional carrier”, so “and latency is unnecessarily increased under NR CA framework” can be removed in “Fragmented spectrum” bullet. Secondly, some bullets are related and update is needed. “Signalling overhead and UE processing complexity of PHY channels” can be sub-bullet of “Features (such as HARQ) defined per carrier”. Lastly, cell management overhead is large in NR CA, especially for fragmented spectrum, which should be included in lessons. The suggested updates are as below with red. - The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - …… - Features (such as HARQ) defined per carrier - prevents further improvements on user throughput and latency via cross-carrier operation - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving - Signalling overhead and UE processing complexity of PHY channels - scale with the number of aggregated carriers rather than the aggregated bandwidth size - …… - Fragmented spectrum - not efficiently utilized and latency is unnecessarily increased under NR CA framework - high cell management overhead - Signalling overhead and UE processing complexity of PHY channels - scale with the number of aggregated carriers rather than the aggregated bandwidth size - No support of efficient IDLE/INACTIVE modes offloading |
| Google |  | We think the first step is to list the potential issues and determine whether they are actually issues at the next meeting. Many sub-bullets below need more discussions. In the main-bullet, we can change “lessons” into “potential lessons”. |
| Apple |  | Okay, but we are not sure why we neeed laudray list of things. The same comment apply to other proposals as well. |
| Samsung |  | OK in principle. The following text is not clear to us, DAI for Type-2 HARQ-ACK codebook conders the values accoss cells, suggest to remove - “Avoid dependencies across carriers - such as DAI to simplify implementation and improve performance” Another confusion is the following bullet since A-TRS may reduce SSB usage and improve NES, - A-TRS trigger with SCell activation - not designed for NES. |
| OPPO |  | We have three comments on the proposed observation: Comment #1: The first is about the structure of the observation: in the proposed observation, we see some lessons/characteristics of one specific mechanism/framework are distributed into multiple sub-bullets, e.g., the lessons of Tx switching appreas in both the third sub-bullet “Coupling DL and UL carriers for a cell” and the fourth sub-bullet “UL Tx switching”, the lessons of CA also are included in many sub-bullets, e.g., the “Pcell vs Scell” sub-bullet, the “CA applicability” sub-bullet and so on. This will obviously lead to complexity for company reading and understanding. Therefore, from our perspective, the proposed observation should be grouped based on the following structure: The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - Single cell spectrum utilization - including lessons on carrier paring, SUL…… - CA - including lessons on PCell/Scell, Scell activation/deactivation, Scell domarcy, UL Tx switching, LBCA switching…. - DC - Including lessons on DC. Comment #2: the first sub-bullet says “CA has been a very successful feature in LTE and NR”. From our perspective, what’s the criteria and how to justify CA is a “successful feature” is hard to be aligned among companies, so it would better to be modified to be “CA has been a very successful beneficial feature in LTE and NR”. Comment #3: There are many bullets with detailed descriptions on many aspects, but we are not sure whether some description (as following) is just a characteristic of the feature to show how it works OR it is leasons from NR that we have learned. It would be appreciated if moderator can further clarify. - CA applicability - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times No support of efficient IDLE/INACTIVE modes offloading |
| HONOR | Y | ok |
| CMCC |  | Firstly, for the 1st bullet “CA has been a very successful feature in LTE and NR” , it should not be listed as a lesson, we should focus on the technical aspects on lessons. We think another lessons should be consider is complex multi-carrier scheduling. During R17, when supporting Scell schedules Pcell, complex splitting of BD/CCE capabilities among carriers are introduced, which should be avoided in 6GR. So we propose to add another sub-bullet, - Complex cross carrier scheduling - At least to avoid complex capability splitting of UE for one scheduled cell being scheduled by multiple scheduling cells |
| ZTE |  | We suggest to prioritize the second proposal 9.2. We are not sure all the listed bullets are lessons. For example, for the bullet CA applicability, we don’t think it is a drawback of the NR CA. In addition, SRS carrier switching is also separately implemented from other features. It should be SCell activation. Also, SCell activation with A-TRS trigger is not based on L1 signaling, this can be a part of reason of slow Scell activation. Last, avoiding dependency for DAI between CCs is to reduce implementation complexity, rather than improving performance. The following modification is suggested: - The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - CA has been a very successful feature in LTE and NR - Pcell vs Scell - Allowing some functionalities only on specific cell like PCell may limit resource utilizations and prevent a NW from entering deep sleep as early as possible on a cell - Coupling DL and UL carriers for a cell - inefficient and ineffective due to different requirements and limitations between DL and UL - SUL/SDL, UL Tx switching, SRS carrier switching, LBCA switching operate differently - SUL scheme is bound to dedicated SUL bands with UL-only resource - ensuring the presence of a corresponding downlink CC used as a reference for measurements - UL Tx switching - did not incorporate all UL transmissions, complicating its use - mandates UE to support at least N DL CCs and the N DL CCs are activated, which leads to high DL capabilities requirement and high UE power consumption - CA applicability - aggregation of non-collocated serving cells and two frequency ranges with different slot durations and processing times - did not sufficiently facilitate wide variety of deployments and network implementations but was designed to require challenging low latency inter-cell coordination - SSB adaptation for Scell - SSB-less SCell operation - limited applicable scenario. - On-demand SSB SCell operation - limited applicable scenario. - Activation of additional carrierSCell - Slow not only because of signaling protocols and RAN4 requirements, but also because of very relaxed CSI accuracy for the newly activated carrierSCell - Fast SCell activation with A-TRS trigger, but still not based on L1 signalling - faces a dilemma of choosing the high service latency caused by SCell activation and high UE power consumption by keeping SCell always activated - SCell dormancy - impractical as this feature is defined on top of BWP framework, which is unnecessarily flexible and complicated. - A-TRS trigger with SCell activation - not designed for NES. - Features (such as HARQ) defined per carrier - prevents further improvements on user throughput and latency via cross-carrier operation - inefficient and ineffective for better frequency utilization, load balancing, NW/UE energy saving - Avoid dependencies across carriers - such as DAI to simplify implementation and improve performance - The maximum number of bands in NR multi-band operations - actually limited by the maximum UE RF+BB hardware capacity in commercial networks - Concurrent transmissions of UL-CA/EN-DC - only beneficial for UEs who are close to gNB and have redundant UE Tx power and its symbol-by-symbol UL power control requires very tight coordination between PCell gNB and SCell gNBs. - need to require a semi-static UL power split for the UE in absence of gNB scheduler coordination. - Only supported for connected mode - Fragmented spectrum - not efficiently utilized and latency is unnecessarily increased under NR CA framework - Signalling overhead and UE processing complexity of PHY channels - scale with the number of aggregated carriers rather than the aggregated bandwidth size - No support of efficient IDLE/INACTIVE modes offloading |
| InterDigital | Y |  |
| ETRI | Y |  |
| Nokia |  | Slightly agree with ZTE, that this is a good list here – but maybe time would be better spent to focus on issues to be studied (in Proposal 9.2). For each of the topics to be studied / considered, we can discuss the reasoning (… which could be a direct consequence of the NR short-comings and could be mentioned there directly, if so wanted). On the details: SCell dormancy: we do agree that the BWP framework is too flexible (as we see from the discussions in Sec. 8) – but this is an issue of the BWP framework and not the SCell dormancy as such. Therefore, maybe not good to mix things here. Features defined by carrier: we think that the statements there are a bit strong and a bit hard for us to see where all the arguments are coming from. As an example, the NW/UE energy saving argument could be also seen the other way around (.. as you can do the saving by deactivation / dormancy / cell DTX/DRX). And a thing that is not considered is the effect on the PHY BB architecture – having features or operation per carrier (e.g. HARQ) allows for separate processing and data handling on a carrier per carrier basis. Signaling overhead and UE processing scale with number of carriers rather than BW: For some of the processing & signaling we agree, but we think this statement is not fully correct when considering e.g. PDSCH/PUSCH processing (e.g. LDPC encoding & decoding is dependent on the totally scheduled bandwidth /aggregated TBS size than the number of carriers the bandwidth is allocated to). Fragmented spectrum: we agree with the proposed changes by Spreadrum above. The main issue is the inefficient spectrum usage – not related to latency to our understanding. Or maybe we just failed to understand what the root cause for the latency argument is coming from (… SCell activation?) Avoid dependencies across carriers: we think it is not just the DAI that requires fast cross-carrier / inter-site coordination, but also the PRI has the same need for coordination at the time of PDSCH scheduling. |
| LGE |  | General - Even though we understand the proposal may try to include all the observations suggested by companies, we think there can be difference in the significance or necessity of improvements in 6GR. In our perspective, we see importance for the following topics within the list: - Pcell vs SCell, Coupling DL and UL carriers for a cell, UL TX switching, SSB adaptation for SCell, Activation of additional carrier, Fragmented spectrum, Signalling overhead and UE processing complexity of PHY channels In details, we have the following questions/comments - No support of efficient IDLE/INACTIVE modes offloading - The meaning seems ambiguous. Does this mean “No support of efficient offloading of signaling overheads in IDLE/INACTIVE modes”? If so, it would be better to modify the text and we are supportive for it. - Features (such as HARQ) defined per carrier - This seems to be necessarily studied for multi-carrier single cell operation. On the other hand, we are not sure if this should be necessary for general multi-cell operation for the moment - The maximum number of bands in NR multi-band operations - This seems to be a second round topic which is highly related to RAN4 work. |
| CATT | Yes | We think this is a good list as starting point. |
| vivo |  | We would like to add the sub-bulle for the following bullet - Signalling overhead and UE processing complexity of PHY channels - UE processing complexity scale with the number of aggregated carriers rather than the aggregated bandwidth size - Not able to share UE processing capability among carriers |

Accroding to the lessons learned from NR spectrum utilization and aggregation framework, companies further propose how to improve spectrum utilization and aggregation framework in 6GR, including but not limited to

- Single framework for 6G spectrum utilization

- CA supporting a wide variety of CA deployments

  - Support for loose NW side coordination, including two PUCCH cell groups

- DL/UL decoupling for a cell

- Native/simplified support for UL Tx switching

- Efficient/effective/practical features of carrier ON/OFF

  - carrier without SSB

  - carrier with on-demand SSB

  - fast carrier activation

- Avoid dependencies across carriers

  - Relax and minimize the need for scheduler interaction across cells in case of CA

- Single cell multi-carriers (SCMC)

  - multiple physical carriers are aggregated into a single logical wideband carrier

- enhanced CA power utilization

- efficient RRC configuration mechanism for CA

- Improve the efficiency, implementation cost and scalability of different cross-carrier scheduling schemes

- Native support for both IDLE/INACTIVE and CONNECTED states

#### [Low]Proposal 9.2:

- Study how to improve spectrum utilization and aggregation framework, including but not limited to

  - Single framework for 6G spectrum utilization

  - CA supporting a wide variety of CA deployments

    - Support for loose NW side coordination, including two PUCCH cell groups

  - DL/UL decoupling for a cell

  - Native/simplified support for UL Tx switching

  - Efficient/effective/practical features of carrier ON/OFF

    - carrier without SSB

    - carrier with on-demand SSB

    - fast carrier activation

  - Avoid dependencies across carriers

    - Relax and minimize the need for scheduler interaction across cells in case of CA

  - Single cell multicarriers (SCMC)

    - multiple physical carriers are aggregated into a single logical wideband carrier

  - enhanced CA power utilization

  - efficient RRC configuration mechanism for CA

  - Improve the efficiency, implementation cost and scalability of different cross-carrier scheduling schemes

  - Native support for both IDLE/INACTIVE and CONNECTED states

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be discussed as second priority, since the highest priority in this meeting is to identify the lessons learned from NR spectrum utilization and aggregation framework, as agreed in the last RAN1 meeting |
| Panasonic | Y |  |
| Spreadtrum |  | We are fine with the low priority arrangement by FL and this proposal can be discussed at later meeting. |
| Google |  | This proposal can be discussed after we agree all the lessons learned from 5G |
| Apple |  | Okay |
| Samsung |  | Would like to first discuss what “loose NW side coordination” is if that intends to be different than the two PUCCH groups in NR. The understanding for “single cell multicarriers” is whether to allow a cell to have fragmented spectrum and the applicable scenarios. It would be simpler to first discuss those aspects. Suggest the following update - Single cell multicarriers (SCMC) - multiple physical carriers are aggregated into a single logical wideband carrier at least for non-contigurous case within a same band. OK to discuss cross-carrier scheduling but the reason it has not been implemented has nothing to do with “improve the efficiency, implementation cost and scalability of different cross-carrier scheduling schemes”. Suggest to add a sub-bullet on “sharing or reuse of SSB or RS across cells for increased NES” under the bullet of “Efficient/effective/practical features of carrier ON/OFF” |
| Ericsson |  | The term “spectrum utilization” has a specific meaning in RAN4 so perhaps we should use a different name here. Furthermore, it is probably good to just say “UL/DL decoupling (in the frequency domain)” or similar and avoid the word “cell”. Also, the term “carrier ON/OFF” can be misleading; a carrier without an SSB is still on. |
| OPPO |  | We also agree this proposal should be of low priority for this meeting. But it is not clear the meaning / intention of the first bullet “Single framework for 6G spectrum utilization”. Does this mean CA/DC/SCMC are considered to be the same framework? |
| CEWiT | Y | Support |
| HONOR |  | OK |
| CMCC |  | We propose a “hyper cell” + a serial of “anchor/non-anchor carriers/cells” framework in our contribution, and most of features have been captured by the above proposals, while there are some missing point in the proposal. For example, the last bullet is too general, to distinguish from current NR scheme. So we propose the following modification, - Native support for both IDLE/INACTIVE and CONNECTED states - Multiple carriers are available for initial access. |
| ZTE |  | More than one PUCCH groups can be studied in our view. As RAN conclusion, we prefer including support CA for both colocated and non-colocated TRPs. For DL/UL decoupling, we have to carefully say they are within a cell. It may impact RAN4 regulation design, e.g. redefine TDD band. Alternatively, a cell with DL only in frequency 1 can be paired with another cell with UL only in frequency 2, then TDD band definition is not changed. In such case, DL only or UL only cell can be supported. We have the following modification - Study how to improve spectrum utilization and aggregation framework, including but not limited to - Single framework for 6G spectrum utilization - CA supporting a wide variety of CA deployments - Support for loose NW side coordination, including more than one two PUCCH cell groups - Support for both colocated and non-colocated TRPs - DL/UL decoupling for a cell, e.g. DL only or UL only cell - Native/simplified support for UL Tx switching, SRS switching and carrier switching. - Efficient/effective/practical features of carrier ON/OFF - carrier without SSB - carrier with on-demand SSB - fast carrier activation - Avoid dependencies across carriers - Relax and minimize the need for scheduler interaction across cells in case of CA - Single cell multicarriers (SCMC) - multiple physical carriers are aggregated into a single logical wideband carrier - enhanced CA power utilization - efficient RRC configuration mechanism for CA - Improve the efficiency, implementation cost and scalability of different cross-carrier scheduling schemes - Native support for both IDLE/INACTIVE and CONNECTED states |
| InterDigital | Y |  |
| ETRI | Y |  |
| Nokia | Y | CA supporting a wide variety of CA deployments: We fully back the “Support for loose NW side coordination” but we think that we should also note that we need to support operation for UEs with and without UL CA capability (note: e.g. two PUCCH groups only useable for UL CA capable UEs) and UE power limitations (for dual PUCCH). So maybe the text could be changed as:  CA supporting a wide variety of CA deployments  Support for loose NW side coordination, including two PUCCH cell groups and considering UE capability or power limitations on UL CA. Single carrier multiple cell: A bit along the lines of Samsung, it would be good to discuss the target scenario / use case (& motivation) that we try to support for 6G – is it similar as the Rel-19 RAN4 discussions (fragmented spectrum in a single band) or anything else. |
| Fainity |  | OK |
| LGE |  | General - Based on our view on the observations in 9.1, we are supportive for the following topics for study: - Single framework for 6G spectrum utilization, DL/UL decoupling for a cell, Native/simplified support for UL Tx switching, Efficient/effective/practical features of carrier ON/OFF, Single cell multicarriers (SCMC), In details, we have the following questions/comments - efficient RRC configuration mechanism for CA - This seems naturally to be combined with multi-carrier single cell operation. On the other hand, we are not sure if this should be necessary for general multi-cell operation for the moment - Native support for both IDLE/INACTIVE and CONNECTED states - Meaning of this proposal seems ambiguous. If the intension is signaling overhead offloading for those modes, it would be better to clarify it, such as, “efficient offloading of signaling overheads in IDLE/INACTIVE modes”. Then we are supportive for it. |
| CATT |  | In general we think these targets are good candidate direction to go with. But not hurry and can be discussed when Proposal 9.1 is stable. |

# 10 Harmonization of TN and NTN

At the last RAN1 meeting, Harmonization of TN and NTN was discussed and the following agreement was made:

Agreement - For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

Since the dedicated agenda item on NTN is planned to be started from RAN1#124, technical details can be discussed there. Howerver, for the harmonized 6GR design for TN and NTN, it would be better to identify which technical areas the NTN aspects need to be considered in early stage. In this sense, this agenda discusses to identify the affected technical areas for the harmonized 6GR design for TN and NTN.

Note that the orbit type and payload type will be discussed in RANp study for 6G requirements.

Companies provide lessons learned from NR/IoT NTN, including but not limited to

- NR NTN was introduced at later releases in a “NBC” fashion

  - Legacy UEs not able to connect, requiring extra development efforts

- Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features

- Achievable data rate was kept low, which limits the applicability of NTN use cases

- GEO satellite is hardly supported due to coverage issues

- Low efficient beam hopping, severe UE power wasting

- High dependency on UE GNSS accuracy

As those lessons are kind of observation, which can be caputred in TR, following proposal is made

#### Proposed observation 10.1:

- The lessons learned from NR/IoT NTN include, but not limited to

  - NR NTN was introduced at later releases in a “NBC” fashion

    - Legacy UEs not able to connect, requiring extra development efforts

  - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features

  - Achievable data rate was kept low, which limits the applicability of NTN use cases

  - GEO satellite is hardly supported due to coverage issues

  - Low efficient beam hopping, severe UE power wasting

  - High dependency on UE GNSS accuracy

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic | Y |  |
| Google |  | We think another potential issue is that one practical scenario of mix earth-fixed and earth-moving as discussed in our Tdoc is missing. This scenario is used in some NTN deployment and can reduce the number of UEs performing handover at the same time. |
| Lenovo |  | One problem is that the coverage of NTN was quite different that of TN and henace many coverage enhancements was done for NTN. |
| Apple |  | Okay |
| Samsung |  | OK to generalize the specifications to both TN and NTN, but TN should be prioritized with the specifications for TN also considering NTN whenever possible in a simple manner without introducing designs that are unnecessary/suboptimal for TN. From spec point of view, the first/second sub-bullets are okay. However, for other sub-bullets, it seems not things observed from real-field deployments. Thus, it should be removed from the list. [Update proposal] - The lessons learned from NR/IoT NTN include, but not limited to - NR NTN was introduced at later releases in a “NBC” fashion - Legacy UEs not able to connect, requiring extra development efforts - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features |
| CEWiT |  | This is not a exhaustive list. So proposal should be open to accept the inputs from future meetings too. |
| HONOR |  | OK |
| ZTE |  | Some of items listed for the lession is not the “pain”. For example, “ Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features“., I assume this is more aligned with the integrated design. For others, e.g., GNSS dependency, we should also understand that this enable the shared design with much less complexity for NR-NTN development. |
| Nokia |  | Similarly to ZTE, we would like to note that some of the aspects on the list are there for a reason. For instance, the reliance on GNSS accuracy is to ensure that UL signals are aligned at the gNB (or rather the satellite antenna). When signals are combined at the satellite antenna, it is crucial that we have proper UE pre-compensation to accommodate the aspect of the fact that a network node (the satellite) is moving at high speed. Additionally, beam hopping is mentioned in the list, but we would rather see a need for beam hopping as a result from deployment, where an operator has deliberately made a selection to have substantially more cells than available transceivers. For the NTN deployment scenarios we may need to have a down selection such that we are only supporting a few of these by default. |
| LGE |  | In NR/IoT NTN, even though the NTN configuration including the ephemeris information is required for UL transmission and the proper TO/FO compensation for DL reception, this information was the part of the other SI. To be specific, the UE first need to detect SSB, and then the UE need to (re)acquire SIB1 for SI scheduling information for the OSI. Considering that the SI scheduling information is a part of the SI or RRC, the actual transmission timing for the SI including the NTN configuration would be located the cross-layer processing time after the SIB1 reception. In this case, the PRACH timing will be delayed as well. Consequently, it will reduce the effective service time of the satellite. In short, we can add one more sub-bullet something like “Due to the separate NTN-specific SI design, the latency for initial access was high, which limits the actual service time of the satellite”. Regarding the TN-NTN, NTN-NTN mobility scenario, due to the large RTT in NTN scenario, the HO will make the huge service interruption. Considering that the flight time or the service time of a satellite is limited, such overhead may need to be further reduced in 6GR. So, we also want to add “The service interruption time for TN-NTN and NTN-NTN mobility was high due to the larger propagation delay”. In case of coverage enhancement, since NR NTN cannot change the waveform, it just tries to increase the repetition number. However, the increased repetition number will not be good solution since the satellite have the limited active beam ratio or number. So, for coverage enhancement, we also need to consider other possibilities to reduce the total number of repetitions in 6GR. So, we can add “Repetition is considered for the coverage enhancement in NR NTN, but its applicable scenario can be limited due to the limited active beam ratio at the satellite.” |
| Xiaomi |  | Although for NR-NTN, the GEO satellite coverage may be challenging especially for UL, the situation for IoT NTN is different with repetition factors and single tone configured at least for VoIP services. Thus we prefer making this bullet clear or removing it. - GEO satellite is hardly supported due to coverage issues for NR-NTN The bullet below is not clear, as listed in our paper, NTN key features such as timing and frequency precompensation, NTN timing based scheduling, HARQ-less transmission and NTN-TN mobility are all NTN specific features. Even for coverage, NTN has Msg4 PUCCH feedback enhancement not applicable to NTN. More clarification is needed for this bullet. - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features |
| SONY |  | This is a good starting non-exhaustive list. We would like to be cautious on the statement on “GEO satellite is hardly supported due to coverage issues”. It is understood that the link budget is always more challenging for GEO system but considering that this is the only commercially deployed 3GPP NTN system right now (IoT NTN), we don’t think it is fair to state that the GEO satellite is hardly supported. Some rewording is needed, e.g. - The lessons learned from NR/IoT NTN include, but not limited to - NR NTN was introduced at later releases in a “NBC” fashion - Legacy UEs not able to connect, requiring extra development efforts - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features - Achievable data rate was kept low, which limits the applicability of NTN use cases - Despite the challenging link budget, GEO satellite is the currently commercially deployed mode hardly supported due to coverage issues - Low efficient beam hopping, severe UE power wasting - High dependency on UE GNSS accuracy We are not sure that the “Achievable data rate was kept low, which limits the applicability of NTN use cases”. Isn’t it the case that the achievable data rate was low due to the challenging propagation conditions (e.g. propagation distance). It would be good to consider higher data rates in 6G-NTN, whether that is supported by new constellation types (depending on deployment), narrower beams / more advanced receivers, MIMO etc. |
| vivo |  | For the 1st sub-bullet, we think some important clarification is needed. Suggest to add the following: - NR NTN was introduced at later releases in a “NBC” fashion - GNSS and ephemeris mechanism are required for basic NTN operation - Legacy UEs not able to connect, requiring extra development efforts |
| OPPO | Y |  |

Regarding the technical aspects affected by NTN characteristics, following views are provided

- harmonization of TN and NTN should not compromise the design of TN or 6G overall

- Cell search / initial access / SSB periodicity

- GNSS-less/resilient operation

- Coverage enhancements

  - shall not consider any 6G NTN-specific coverage enhancements, i.e., commonly designed with TN

  - Paging in body loss/NLOS/satellite-misaligned scenario

  - both the link and system level, including optimization on initial access

  - 100% coverage ratio in a cell with massive beam footprints

- Positioning

- NTN-TN and NTN-NTN mobility

- DC/CA

  - Note: DC is subject to RANp discussion

- Capacity

  - Including OCC multiplexing

- Large propagation delay

  - Including scheduling/HARQ

- Large doppler shift/drift and timing drifting

  - Including timing and frequency synchronization adjustment

- Duplexing

  - Focus on FDD

  - HD-FDD, including collision handling

  - Support TDD

- Beamforming / beam management

  - Dynamic beam management for (V)LEO constellations with massive satellite beams

  - Robust transmit/receive beamforming (digital, hybrid, or analog) method

  - Refined beam management, enabling more accurate DL beam selection for RACH

- TN-NTN in the same spectrum

  - coexistence mechanism for interference mitigation

- 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

- satellite moving

  - RRC configuration adaptation based on the satellite position

According to the input, following proposals can be considered as starting point

#### Proposal 10.2:

- The technical aspects affected by NTN characteristics include, but not limited to

  - Cell search / initial access / SSB periodicity

  - GNSS-less/resilient operation

  - Coverage enhancements

  - Positioning

  - NTN-TN and NTN-NTN mobility

  - CA

  - Capacity

  - Large propagation delay

  - Large doppler shift/drift and timing drifting

  - Duplexing

  - Beamforming / beam management

  - TN-NTN in the same spectrum

  - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

  - satellite moving

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | This proposal can be used as starting point for further discussion, as this is moderator’s initial list and companies would need time to improve the text. |
| Panasonic |  | Instead of "CA", generalized term like " spectrum utilization and aggregation framework" would be more aligned with other discussion. |
| Speradtrum |  | Large propagation delay, large Doppler shift/drift and timing drifting, satellite moving are NTN charactistics but not technical aspects. SSB periodicity and cell search are included in initial access. Both GNSS-less/resilient operation and GNSS operation should be studied in time/frequency synchronization. Due to larger propagation delay, timing relationship should also be studied. We suggest to revise the proposal as follows: - The technical aspects affected by NTN characteristics include, but not limited to - Cell search / initial access / SSB periodicity - Time/frequency synchronization (including GNSS-less/resilient operation and GNSS operation) - Timing relationship - Coverage enhancements - Positioning - NTN-TN and NTN-NTN mobility - CA - Capacity - Large propagation delay - Large doppler shift/drift and timing drifting - Duplexing - Beamforming / beam management - TN-NTN in the same spectrum - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam - satellite moving |
| Google |  | We think another one is positioning. If 6G NTN does not rely on GNSS, the potential way is to consider NTN based poisoning. |
| Fujitsu | Y |  |
| Apple |  | Okay |
| Samsung |  | We are okay to consider the following technical aspects affected by NTN characteristics. - Large propagation delay - Large doppler shift/drift and timing drifting - Coverage enhancements For GNSS-less/resilient operation, it is not a technical aspect, but it is seen as a condition as how to pre-compensate large propagation delay and doppler shift/drift. Thus, it can be further considered depending on R20 5GA discussion. For other bullets, TN performance should not be degraded due to NTN specific features. Thus, it is preferable to consider TN scenario first, then re-visit if there is critical issue from NTN side. |
| CEWiT |  | We should include waveform & PAPR aspects, and reference signals. |
| Airbus |  | Okay. |
| HONOR |  | OK |
| ZTE | No | We prefer to do further categorization to highlight on aspects for harmonized design. |
| ESA | Y | We support the proposal. |
| Fainity |  | OK |
| TCL |  | We support this proposal. |
| Nokia |  | GNSS-less/resilient operation is important, but it does not obliviate the usefulness of GNSS information for efficient operation of NTN system, for example in case of IoT NTN. We do see the main characteristic of NTN operation to be that the network node (the satellite) is moving and may potentially be located far from the devices served in the cell. The fast movement and large distance will impact all the aspects on the list. However, it is important to make a distinction between for instance intra-satellite and inter-satellite operation (for instance for mobility for NTN-NTN and positioning). Further, for positioning it makes some difference whether this is considering RRC connected mode only, or if IDLE like mode is also considered here (e.g. related to pre-compensation of UL signals). Hence, we think some further discussion is needed to categorize the key aspects for harmonized TN/NTN design. |
| LGE |  | In NR NTN, due to the satellite moving, during the satellite switching for re-synch for the same cell ID, service interruption will occur. In this point of view, many satellite services in the market considers the multi-orbit operation. So, we want to add “Multi-orbit/satellite operation”. Moreover, as mentioned in the lesson part, in 6GR, we need to think about the coverage enhancement with the reduced repetitions due to the limited active beam ratio/number at the satellite. In this point of view, “PAPR reduction for DL transmission” need to be added. |
| Xiaomi |  | We would like to seek clarification on the satellite moving aspects, for example, is mobility/cell(spotbeam) reselection impacted technical aspects due to satellite moving? We are OK for CA but we think multi-satellite coordination reflects the technical aspects impacted by NTN more comprehensively since not only frequency coordination including CA but also spatial domain coordination including distributed MIMO operation across satellites can be considered. For the capacity bullet, we think the current formulation is not clear. Depending on companies’ views the capacity improvement can be two-folded, whether it’s for msg3 only, or includes connected state transmission state as well configured grant transmission (CB-CG). Clarificaiton on the capacity bullet is needed. Our suggested change to the list as follows, - Cell search / initial access / SSB periodicity - GNSS-less/resilient operation - Coverage enhancements - Positioning - NTN-TN and NTN-NTN mobility - Multi-satellite coordination, e.g. CA, MIMO - [Capacity] - Large propagation delay - Large doppler shift/drift and timing drifting - Duplexing - Beamforming / beam management - TN-NTN in the same spectrum - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam - [satellite moving] |
| SONY |  | We would like to add “data rate enhancements”, since the low data rate was proposed as a lesson learned in observation 10.1 Support Spreadtrum update to GNSS section. |
| CATT |  | Fine in general, but ‘Large propagation delay’ and ‘ Large doppler shift/drift and timing drifting’ are not technical aspects (not in the same level) compared to other bullets. Also, beam hopping is as important as beam forming in NTN. Finally, it is premature to say TN and NTN share the same spectrum at least for now. Suggestions: - The technical aspects affected by NTN characteristics include, but not limited to - Cell search / initial access / SSB periodicity - GNSS-less/resilient operation - Coverage enhancements - Positioning - NTN-TN and NTN-NTN mobility - CA - Capacity - Scheduling/HARQ to address large propagation delay - Timing and frequency synchronization adjustment to combat large doppler shift/drift and timing drifting - Duplexing - Beamforming / beam hopping/ beam management - TN-NTN in the same spectrum - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam - satellite moving |
| vivo |  | We should focus on areas from RAN1 perspective, the other areas, e.g., mobility, should be handled by RAN2 and thus should be removed. For the spectrum sharing or coexistence, it has not been clear whether they should be supported in 6GR (at least in day-1), thus, they should also be removed. In order to enable GNSS-less operation, the NTN UE should be able to connect to the network even without ephemeris. Suggest to clarify: - The technical aspects affected by NTN characteristics include, but not limited to - Cell search / initial access / SSB periodicity - GNSS/ephemeris-less/resilient operation - Coverage enhancements - Positioning - NTN-TN and NTN-NTN mobility - CA - Capacity - Large propagation delay - Large doppler shift/drift and timing drifting - Duplexing - Beamforming / beam management - TN-NTN in the same spectrum - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam - satellite moving |
| OPPO |  | We have two comments: #1: Capacity related enhancement can be low prioritized, and focus on idle UE scenario to avoid later NBC issue. Capacity boost can be added in later release. #2: Regarding spectrum sharing and coexistence, we are fine with TN/NTN in same spectrum to be handled in 6G NTN topic. But we believe that the study of 6G NTN and NR NTN should be based on the outcome of 6G MRSS. We don’t think 6G SID supports the scope of 6G NTN and IoT-NTN spectrum sharing. |

# 11 Other aspects

Other than the aspects discussed in the above sections or other agenda items (including those planned in future RAN1 meetings), some companies mention the aspects related to PHY security, NW resilience, and so on. It is moderator’s understanding that neither of other aspects can be discussed in RAN1 without any progress in RANp study on 6G requirements. RAN1 cannot discuss any features without justification on the target/motivation, which need to be clarified in RANp study at first. Following is open question to hear companies’ view.

#### Question 11.1:

- Companies are invited to provide views on whether to discuss any features, other than those (to be) discussed in other sections in this summary or in other agendas in RAN1. If yes, please elaborate which features need to be studied in this section.

| Company | Y/N | Comments |
| --- | --- | --- |
| BT |  | As we shape 6G, it's vital to keep operator needs front and centre — especially around 5G-6G migration and Total Cost of Ownership (TCO). The proposals in R1-2507884 are: - During the RAN1 study, any PHY layer innovation must consider the Total Cost of Ownership (TCO), including migration related aspects. - During the RAN1 study, the benefits of any PHY layer innovation (performance, efficiency, use case enablement …) must be carefully assessed against the impact on 5G-6G migration. These principles may be high-level, but overlooking them now could lead to costly challenges later. |
| Nokia |  | Agree with BT that these are important aspects to take into account in the overall design of 6GR. |
| Vodafone |  | Agree with BT. According to the RAN#109 agreement, lower CAPEX/OPEX with respect to current networks is a requirement for the 6G design. “The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:] (…) - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. “ This means that CAPEX/OPEX should be evaluated in the 6G design study, so at least a study on how to address the RAN agreement needs to be done in RAN1 in order to perform this evaluation. |
| CEWiT |  | Positioning is one of the minimum TPR agreed by WP5D. For 3GPP therefore it is important to be introduced in the first release of the specifications. In NR Positioning is matured enough and for 6GR it is important to study how it will be adopted. We propose to take this up in the agenda 11.1 and set some guideline. Proposal could be: Study positioning feature adoption to 6GR, 5GNR positioning framework as baseline. |

#### Proposal 11.2:

- Study how to address the following RAN requirement in 6GR physical layer design, considering, e.g., complexity reduction, energy efficiency, market fragmentation, MRSS migration aspects

  - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

| Company | Y/N | Comments |
| --- | --- | --- |
| Moderator |  | After discussion with companies co-sourcing R1-2507884, this proposal is made to further check how we will consider RANp requirement |
| Bouygues Telecom |  | We agree with BT and Vodafone positions expressed above. Operators are now in a position where their SA deployment is only at its early stages and it is expected to expand gradually by 2030. At the time when 6G will arrive, the migration of freshly deployed spectrum and equipment from legacy 5G to new G proves to be a real challenge. This challenge can only be solved if the gains and the cost of RAN evolution towards 6G are correctly understood by the MNO. In this context, having a certain form of CAPEX/OPEX assessment for 6G in not only in line with the spirit of RAN Plenary agreement cited by Vodafone but also a necessary step to help operators overcome the challenge of 5G to 6G migration. |

# 12	Conclusions

Following agreements were made in this meeting:

To be updated

# References

| [1] | RP-252912 | Revised SID: Study on 6G Radio | NTT DOCOMO, CMCC, AT&T, Vodafone |
| --- | --- | --- | --- |
| [2] | R1-2507812 | Workplan for Rel-20 Study of 6GR | NTT DOCOMO, China Mobile, AT&T, Vodafone |
| [3] | R1-2506738 | High level views on 6GR air interface | FUTUREWEI |
| [4] | R1-2506750 | Nokia Views on Selected Aspects of 6G Radio Air Interface | Nokia |
| [5] | R1-2506813 | Overview of 6GR air interface | Spreadtrum, UNISOC |
| [6] | R1-2506841 | Overview of the 6G air interface | Ericsson Telecom S.A. de C.V. |
| [7] | R1-2506843 | Overview of the 6G air interface | TCL |
| [8] | R1-2506897 | Overview of 6GR air interface | vivo |
| [9] | R1-2506918 | High-level views on 6GR | ZTE Corporation, Sanechips |
| [10] | R1-2506988 | 6GR air interface design overview | Xiaomi |
| [11] | R1-2507013 | Overview of 6GR air interface | CMCC |
| [12] | R1-2507057 | Overview of 6GR air interface | Huawei, HiSilicon |
| [13] | R1-2507065 | Enhancements for 6G Fixed Wireless Access | T-Mobile USA Inc. |
| [14] | R1-2507104 | Outline and highlight of 6GR air interface | CATT, CICTCI |
| [15] | R1-2507175 | Overview of 6GR air interface | OPPO |
| [16] | R1-2507201 | Overview on 6G Air interface | Tejas Network Limited |
| [17] | R1-2507212 | Discussion on overview of 6GR air interface | HONOR |
| [18] | R1-2507252 | Design of 6GR air interface | Samsung |
| [19] | R1-2507311 | Overview of 6GR air interface | NEC |
| [20] | R1-2507334 | Overview of 6GR air interface | China Telecom |
| [21] | R1-2507343 | Overview of 6GR air interface | THALES, Airbus, ESA, EchoStar, Eutelsat Group, Novamint, TNO, Fraunhofer IIS, Iridium |
| [22] | R1-2507360 | Views on overall design and techniques for 6GR air interface | LG Electronics |
| [23] | R1-2507366 | Overview of 6GR air interface | NVIDIA |
| [24] | R1-2507373 | Overview proposal of 6GR air interface | Panasonic |
| [25] | R1-2507402 | Discussion on overview of 6GR air interface | Fujitsu |
| [26] | R1-2507407 | Overview of 6GR air interface | SK Telecom |
| [27] | R1-2507466 | Discussion on 6GR Air Interface | Ofinno |
| [28] | R1-2507480 | Overview of 6GR air-interface | Lenovo |
| [29] | R1-2507490 | Design consideration of 6GR air interface | Verizon Sweden |
| [30] | R1-2507505 | Overview of the 6GR air interface | ETRI |
| [31] | R1-2507520 | Overview of 6GR Air Interface | Google |
| [32] | R1-2507538 | Overview of 6GR Air Interface | Fraunhofer IIS, Fraunhofer HHI |
| [33] | R1-2507544 | Discussion on the Overview of 6GR Air Interface | Rakuten Mobile, Inc |
| [34] | R1-2507585 | IMU Views on 6G Radio Air Interface | IMU |
| [35] | R1-2507595 | Overview of 6GR air interface | Sony |
| [36] | R1-2507602 | Positioning, Navigation and Timing (PNT) in 6G NTN-TN harmonization | Airbus, ESA, Fraunhofer IIS, Thales, Iridium |
| [37] | R1-2507606 | Overview of 6GR air interface | MediaTek Inc. |
| [38] | R1-2507629 | Discussion on Overview of 6GR air interface | China Unicom |
| [39] | R1-2507676 | Overview of 6GR air interface | Apple |
| [40] | R1-2507720 | Overview of 6GR air interface | Qualcomm Incorporated |
| [41] | R1-2507730 | Views on device types, min channel BW and MRSS | Intel |
| [42] | R1-2507734 | Overview of 6GR air interface | InterDigital, Inc. |
| [43] | R1-2507745 | Lessons Learned from the 5G NR Air Interface Design | AT&T |
| [44] | R1-2507763 | Views on 6GR air interface | Tiami Networks |
| [45] | R1-2507765 | Overview of 6GR air interface | Sharp |
| [46] | R1-2507768 | Views on 6GR air interface | Fainity Innovation |
| [47] | R1-2507814 | Discussion on overview of 6GR air interface | NTT DOCOMO, INC. |
| [48] | R1-2507823 | Views on 6GR sync signal structure | NICT |
| [49] | R1-2507843 | Overview of 6G Radio air interface | ITL |
| [50] | R1-2507846 | Overview of 6G Radio air interface | WILUS Inc. |
| [51] | R1-2507851 | Views on 6GR air interface | CSCN |
| [52] | R1-2507862 | Overview of 6GR air interface | KDDI Corporation |
| [53] | R1-2507879 | General aspects of 6G IoT and NTN | Nordic Semiconductor ASA |
| [54] | R1-2507884 | Operator considerations on performance gains and migration complexity trade-offs in 6G Radio design | BT plc, AT&T, Bouygues Telecom, Deutsche Telekom, Orange, Vodafone |
| [55] | R1-2507938 | On 6GR Frame Structure and Waveform | Boost Mobile Network |
| [56] | R1-2507941 | IIT Kanpur’s views on 6GR air interface | IIT Kanpur |

# RAN1 agreements

### RAN1#122

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

- Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Agreement

- Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Agreement

- Identify the high-level aspects which impact on the NR-6GR MRSS support

  - Including the lessons learned from LTE-NR DSS

Agreement

- Study and identify the lessons learned from NR BWP framework

Agreement

- Study and identify the lessons learned from NR spectrum utilization and aggregation framework

  - DC is subject to RANP decision in June 2026

  - Note: MRSS aspects are separate discussion

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

Agreement

- For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

### RAN1#122bis