3GPP TSG RAN WG1 #122bis									 R1-250xxxx

Prague, Czech, October 13th – 17th, 2025

Agenda item:	        11.6

Source: 	Samsung (Moderator)

Title:	Moderator summary #0 on AI/ML for 6GR

Document for: 	Decision

# Introduction

In RAN 1 #122, the following agreement was made for use case identification:

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., no collaboration/interaction UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

This paper summarized the proposed use cases for 6GR study on AI/ML.

# Use cases

## CSI prediction and CSI-RS overhead reduction

#### Proposed observation 2.1:

[24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [23 sources] provided preliminary simulation results and analysis on CSI-RS overhead reduction in frequency and/or spatial domain with AI/ML. Details can be found in Table A.

- [4 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML. Details can be found in Table B.

- [3 sources] provided preliminary simulation results and analysis on CSI prediction cross carrier/frequency block with AI/ML. Details can be found in Table B.

- [2 sources] provided preliminary simulation results and analysis on CSI interpolation across analog beams with AI/ML. Details can be found in Table B.

- [2 sources] provided preliminary simulation results and analysis on CSI prediction with antennas port on/off with AI/ML. Details can be found in Table B.

- Besides, one source provided preliminary simulation results and analysis on LT-CI aided CSI prediction (Huawei), Tokenized CSI prediction (Huawei), and time domain CSI prediction combining CSI-RS and DMRS measurements (MediaTek).

Table A

| Sub-use case | Sub-Case A: sparse CSI-RS in frequency and/or spatial domain with AI/ML |
| --- | --- |
| Supported companies | (23) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, {CATT, CICTCI}, Samsung, Fujitsu, Apple, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, LGE10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas |
| Model input | 1. Estimated channel matrix over sparse CSI-RS 2. Received RS signal7,8,9 3 Eigenvector2,3 4 Reported CSI for NW-sided model5,10 |
| Model output | 1. Full channel matrix 2. PMI 2 |
| Label | 1. Estimated/ideal channel matrix based on full CSI-RS density 2. Ideal precoding matrix with full dimension3 |
| Training types assumption | offline training |
| KPI | NMSE, SGCS, throughput, ratio of CSI-RS overhead |
| Benchmark | 1. non-AI based on full CSI-RS 2. non-AI based on sparse CSI-RS |
| Preliminary result | 1.NMSE/SGCS gain over sparse CSI-RS 2. Reduced CSI-RS overhead with decent estimated CSI 3.Link-level throughput gain1 |
| Model location for inference | UE-sided model, NW-sided model2,3,5,6,10, Two-sided model3,4 |
| Collaboration/interaction between UE and NW | As CSI prediction for UE-sided model in NR As two-sided model for CSI compression4 in NR |
| Potential spec impact | 1.Sparse CSI-RS design 2. signalling/ procedure related to LCM 3. Inter-vendor collaboration for two-sided model, when applicable |

Table B

| Sub-use case | Sub-Case B: CSI time domain prediction | Sub-case C: CSI prediction cross carrier/frequency block | Sub-Case D: CSI interpolation across analog beams | Sub-Case E: antenna port on/off |
| --- | --- | --- | --- | --- |
| Supported companies | (4) Ericsson, BJTU, Samsung, Qualcomm | (3) Samsung, Apple, DoCoMo1 | (2) Samsung, vivo | (2) vivo, {CATT，CICTCI}, |
| Model input | 1. channel matrix over K CSI-RS occasions | Channel matrix of frequency block A | Channel matrix of Set B of beams | 1.Channel matrix of certain antenna pattern 2. Eigenvector of certain antenna pattern |
| Model output | Channel matrix of future instances | Channel matrix of frequency block B | Channel matrix of Set A of beams | 1.Channel matrix of other on/off antenna patterns 2. Eigenvector of other antenna patterns |
| Label | Channel matrix (explicit H) in the angle-delay domain corresponding to the target frequency | Channel matrix of frequency block B | Channel matrix of Set A of beams | 1. Ideal channel matrix of the target on/off pattern 2. Ideal eigenvector matrix of the target on/off pattern |
| Training types assumption | offline training | offline training | offline training | offline training |
| KPI | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS, NMSE, throughput, ratio of CSI-RS overhead | NMSE, SGCS, throughput, CSI feedback overhead |
| Benchmark |  | 1.Ground truth of target frequency block 2. Sample and hold | Ground truth of Set A of beams |  |
| Preliminary result | As CSI prediction in NR | Decent SGCS | Decent SGCS | 1. Decent SGCS 2. CSI feedback overhead reduction |
| Model location for inference | UE-sided model | UE-sided model NW-sided model1 | UE-sided model | UE-sided model |
| Collaboration/interaction between UE and NW | As CSI prediction in NR | As CSI prediction in NR | As CSI prediction in NR | As CSI prediction in NR |
| Potential spec impact | As CSI prediction in NR | 1. Cross carrier/frequency switching procedure enhancement based on predicted CSI 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted beams 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted antenna ports patten 2. signalling/ procedure related to LCM |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Intention is to agree on the above as observation. In addition, I suggest to delete the part marked as grey, because no evaluation results provided. Please share your view and comment |

## DMRS design with AI receiver

#### Proposed observation 2.2:

[23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver.

- [22 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver.

- [8 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver.

- [4 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver.

- Details can be found in Table C.

Table C

| Sub-use case | Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain | Sub-case B: Superimposed pilot | Sub-case C: DMRS free |
| --- | --- | --- | --- |
| Supported companies | (22) Nokia1, Futurewei2, Ericsson3, ZTE4, {Spreadtrum, UNISOC}5, Interdigial6, vivo7, xiaomi8, CMCC9, {CATT, CICTCI}10, Fujitsu11, Apple12, Samsung13, Kyocera14, Lenovo15, Huawei16, Qualcomm 17, Ofinno18, NVIDIA19, MediaTek20, Lekha21, LGE22 | (8) vivo 1, CMCC 2, ZTE3, Lenovo4, Huawei5, OPPO6, NVIDIA7, LGE8 | (4) InterDigital1, Huawei2, NVIDA3, MediaTek4 |
| Model input | 1. Received signal of target REs1,2,3,8,9,12,14,15,16,18 ,20,21[and transmitted target REs9,13,19] 2. Raw estimated channel at DMRS7,11 3. Received signal and raw estimated channel at DMRS10 | 1. Received signal of target RE2,3,4,5,6,7,8 2. Raw estimated channel (in time/doppler domain) 1 | Received signal |
| Model output | 1. Estimated channel at all REs5,8,9, 12, 13,16,17,18,19,21 2. LLRs1,2,3, 13,15,19,20 3. Estimated channel at DMRS RE13 | 1. Estimated channel at all REs1,3,4,5,6,8 2.LLR2,5,6,7 | LLR |
| Label | 1. [Approximately] ideal channel 5,7,8,9,12,13,17,18 2. Reference transmission of known sequence/data1,2,3,13,15 3. Label free6, 13,21 | 1. Reference transmission of known sequence/data 2,3 2. [Approximately] ideal channel 1 | Constellation points of data channel |
| Training types assumption | offline training | offline training | offline training |
| KPI | MSE, BLER, throughput | BLER, throughput | BLER, throughput |
| Benchmark | With ideal channel With conventional receiver with sparse or legacy DMRS | With ideal channel With conventional receiver with legacy DMRS overhead | With ideal channel With conventional receiver with legacy DMRS overhead |
| Preliminary result | Higher MSE BLER gain Throughput gain | BLER gain Throughput gain | BLER gain Throughput gain |
| Model location for inference | UE-sided model or NW-sided model | UE-sided model or NW-sided model | UE-sided model 1 NW-sided model3,4 Two-sided model2 |
| Collaboration/interaction between UE and NW | No | No | No for one-sided model As for two-sided model in NR |
| Potential spec impact | 1. DMRS pattern design 2. RAN 4: Demo requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model | 1. DMRS pattern design 2. RAN 4: Demo requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model | 1. RAN 4: Demo requirement 2. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration) 3. Downloaded modulation constellation 4 |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide the potential spec impact on DMRS free with AI receivers. |

## CSI compression and feedback

#### Proposed observation 2.3:

[13 sources] provided preliminary simulation results and analysis on CSI compression and feedback with AI/ML receiver.

- [10 sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC), and/or modulation (JSCM) with two-sided model.

- [4 sources] provided preliminary simulation results and analysis on CSI compression with linear projection and NW-sided model with NW-sided model.

- [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook with NW-sided model.

- Besides, one source provided preliminary simulation results and analysis on Joint CSI prediction and compression (BJTU), Joint CSI Estimation and Compression with DMRS overhead reduction (BJTU), Multi-beam CSI compression for HBF (vivo), JSCC with legacy modulation constellation with NW-sided model ({Pengcheng, ZGC}), SRS fusion (Qualcomm), Vector quantization codebook enhanced CSI compression (BUPT, ZGC).

- Details can be found in Table D.

Table D

| Sub-use case | Sub-case A: CSI compression with joint source and channel coding (JSCC), and modulation (JSCM) | Sub-case B: CSI compression with linear projection and NW-sided model | Sub-case C: DLable basis/codebook |
| --- | --- | --- | --- |
| Support companies | (10) ZTE1, Samsung2, vivo3, {Pengcheng, ZGC}, Lenovo, OPPO, MediaTek4, CMCC, Fujitsu, Apple | (4) BJTU, Samsung, {BUPT, ZGC}, OPPO | (2) ZTE, Samsung |
| Model input of decoder [or model output of encoder, when applicable] | Compressed CSI bits or complex values, [and estimated channel based on SRS2,3] | Compressed CSI information (complex values) [and estimated channel based on SRS] | Amplitudes and phases obtained by a look up table based on feedback CSI bits |
| Model output of decoder [or model input of encoder, when applicable] | 1.Eigenvectors 2.Explicit H1,2,3,4 | 1.Eigenvectors 2.Explicit H | Eigenvectors Explicit H |
| Label | 1.Eigenvectors 2.Explicit H1,2,3,4 | 1.Eigenvectors 2.Explicit H | Eigenvectors |
| Training types | Offline training | Offline training | Offline training |
| KPI | SGCS, NMSE, SE, UPT | SGCS, NMSE, SE, UPT, UE complexity | UPT vs overhead |
| Benchmark | eType II NR separate source and channel coding | eType II NR separate source and channel coding JSCM with two-sided model | eType II |
| Preliminary result | Better performance than benchmark, especially in low SNR range | 1.Better performance than eType II, especially in low SNR range 2. [Slight] lower performance than JSCM with two-sided model but much lower UE complexity | Better performance than benchmark |
| Model location for inference | Two-sided model | NW-sided model | NW-sided model |
| Collaboration/interaction between UE and NW | As for two-sided model in NR | no collaboration or DLable matrix or generated matrix based on configured parameters | DLable basis |
| Potential specification impact | 1. Necessary signalling/ procedure to support JSCC/JSCM 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration | 1. Projection matrix related signalling/procedure 2. Signalling/ procedure related to LCM with NW-sided model | 1. Downloadable basis related signalling/procedure 2. Signalling/ procedure related to LCM with NW-sided model |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  |  |

## AI for beam management and extension

#### Proposed observation 2.4:

[13 sources +] provided preliminary simulation results (or cite to NR AI/ML for beam management simulation results) and analysis on AI/ML for beam management and extension.

- [5+ sources] provided preliminary simulation results (or cite to NR AI/ML for beam management simulation results) and analysis on inter-cell/TRP beam prediction and management.

- [4+ sources] provided preliminary simulation results and analysis on cross frequency band beam prediction.

- [4+ sources] proposed to support DL Tx beam prediction based on the NR study on AI/ML for beam management and [2 sources] support Tx-Rx beam pair prediction based on the NR study on AI/ML for beam management.

- [3+ sources] provided preliminary simulation results (or cite to NR AI/ML for beam management simulation results) and analysis on beam prediction for initial access.

- Besides, the following sub-use cases are proposed by one source: Pathloss prediction (Nokia), reinforcement learning-based approach beam selection (Nokia), beam prediction for high-speed train: (BJTU), RRM measurement/RLF prediction (BJTU), positioning assisted beam prediction (Huawei), L1/L2 mobility(NAVIDA), Spatial/temporal beam prediction for unified mobility(Qualcomm)

- Details can be found in Table E.

Table E

| Sub-use case | Sub-case A: Inter-Cell/TRP beam prediction and management | Sub-Case B: Cross frequency beam prediction | Sub-Case C: Time/Spatial domain DL Tx beam prediction | Sub-Case D: Beam prediction for initial access |
| --- | --- | --- | --- | --- |
| Supported companies | (5+) Nokia, ZTE, xiaomi, CEWiT, DoCoMo, [LGE] | (4+) Futurewei1, xiaomi2, Apple3, Qualcomm4, [Panasonic, LGE] | (5) [Ericsson1, BJTU, Samsung, Qualcomm, Nokia2] | (3+) Huawei, vivo, Qualcomm, [Samsung, ZTE, LGE, Apple] |
| Model input | Measurements from Set B of multiple TRPs/Cells or of some TRPs/Cells | 1. CSI-RS measurement for one or multiple FR1 cells1,3 2. L1 measurements in one frequency2,4 | Measurements from Set B | Measurements from Set B of SSB |
| Model output | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A of multiple TRPs/Cells or from Set A of each TRP/Cell [of current or future time instance] | 1. FR2 cell/beam related information1,3 2. beam related information of another frequency2,4 [of current or future time instance] | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] |
| Label | Measurements [or Top beams] of Set A | Measurements on the target frequency | As NR study on AI for BM | Measurements [or Top beams] of Set A |
| Training types assumption | offline training | offline training | offline training | offline training |
| KPI | Prediction beam/measurement accuracy Throughput RS overhead reduction, Complexity. | Prediction beam/measurement accuracy RS overhead reduction, | As NR study on AI for BM | SGCS, NMSE, throughput, ratio of CSI-RS overhead |
| Benchmark | Based on Set A Based on Set B | Based on measurements on the target frequency | As NR study on AI for BM | Based on Set A Based on Set B |
| Preliminary result | Higher/good prediction accuracy RS overhead reduction | 1. Decent SGCS 2. CSI feedback overhead reduction | As NR study on AI for BM | Decent prediction accuracy |
| Model location for inference | UE-sided model or NW-sided model | UE-sided model or NW-sided model | UE-sided model or NW-sided model | UE-sided model or NW-sided model |
| Collaboration/interaction between UE and NW | As NR AI for BM | As NR AI for BM | As NR AI for BM | As NR AI for BM |
| Potential spec impact | 1. Inter-Cell/TRP beam management related singling/procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. SCell/frequency range activation procedure(of any) based on prediction beam 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | As NR AI for BM | 1. Initial access related procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | For BM, the proposed use cases that can be derived based on NR AI/ML for beam management study are also considered, if the companies reported detailed information according to the agreement in last meeting. many other companies also support this use case, which can be found in excel. |

## AI based (De-)Modulation

#### Proposed observation 2.5:

[5 sources] provided preliminary simulation results and analysis on (de)modulation related design with help of AI/ML or AI/ML receiver.

- [4 sources] provided preliminary simulation results and analysis on modulation constellation design with the help of AI and with non-AI or AI receiver.

- [2 sources] provided preliminary simulation results and analysis on AI-based modulation and precoding with two-sided model.

- Details can be found in Table F.

Table F

| Sub-use case | Sub-use case A: AI-based (de)modulation | AI-based modulation and precoding |
| --- | --- | --- |
| Support companies | (5)ZTE1, OPPO2, vivo3, Lenovo4, Xiaomi5 | (2)ZTE, OPPO |
| Model input | 1. Information bits1,2,5 2. Received signal3 3. Channel characterization [and modulation order] 3,4 | Encoder: information bits Decoder: Soft LLR |
| Model output | 1. Constellation 1,2,5 2. LLR3 3. Probability 4 | Encoder: modulated symbols after layer mapping Decoder: information bits |
| Label | 1. Information bit1,2,5 2. Label free4 | Information bit |
| Training types | Offline training Online training5 | Offline |
| KPI | BLER | BLER |
| Benchmark | Conventional constellations with uniform spacing | NR modulation and layer maping |
| Preliminary result | 0.11~0.8dB gain with different modulation order in AWGN channel 3,5 up to 1dB gain in AWGN channel 1 > 1dB gain in AWGN channel 2,4 | 1 dB gain Up to 2.3dB gain |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 1,2,5 2.NW-sided model3 | Two-sided |
| Collaboration/interaction between UE and NW | NA | As for two-sided model in NR |
| Potential specification impact | 1. Constellations desgin [and downloading mechanism] 2. Signalling/ procedure related to LCM for NW-sided model | 1. Modulation design and layer mapping design 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## AI for DPoD/DPD/none-linearity handling

#### Proposed observation 2.6:

[5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver.

- [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver.

- [2 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter.

- Details can be found in Table G.

Table G

| Sub-use case | Sub-use case A: AI-based DPoD/None-linearity compensation | Sub-use case B: AI-based DPD |
| --- | --- | --- |
| Support companies | (5) Samsung1, Ericsson2, OPPO3, vivo4, Huawei5 | (2) vivo, Huawei |
| Model input | 1. Information bits1,2,5 2. Received signal3 | Time domain samples before pre-distortion |
| Model output | 1. Compensated signal in time domain1,2,5 2. Soft bits2,3 | Time domain samples after pre-distortion |
| Label | 1. DMRS1 2. Known bit sequence2,3 | Time domain samples |
| Training types | Online training/finetune1 Offline training | Offline |
| KPI | BLER, MPR, EVM, throughput | BLER, EVM, MPR |
| Benchmark | Without compensation | No DPoD |
| Preliminary result | 1~3dB BLER gain in mid/high modulation order1,3,4 ~3-7dB MPR gain5 ~100% link-level throughput gain in mid/high SNR2 | 1~3dB BLER gain in mid/high modulation order EVM gain |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 1,2,5 2.NW-sided model3 | UE-sided |
| Collaboration/interaction between UE and NW | NA | As for two-sided model in NR |
| Potential specification impact | 1. RAN 4 requirements 2. UE capability report 3. DMRS design/selection, Tx power determination 4. Data collection and performance monitoring for offline training. | 1. RAN4 requirements 2. Dedicated signal (RS/data) for training or monitoring data collection for AI/ML DPoD. |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## AI for TMPI

#### Proposed observation 2.7:

[3 sources] provided preliminary simulation results and analysis on AI-based UL TMPI with details in Table H

Table H

| Use case | AI based UL TPMI |
| --- | --- |
| Supported companies | (3)vivo1, Fujitus2, Samsung3 |
| Model input of decoder or model output of encoder | multi-bit payload |
| Model output of decoder or model input of encoder | (Reconstructed) eigenvectors of UL channel |
| Label | Estimated eigenvectors of UL channel based on SRS measurement |
| Training types | offline training online finetune1 |
| KPI | SCGS, BLER |
| Benchmark | NR TPMI codebook |
| Preliminary result | 1~5dB BLER gain1 10%~90% SGCS gain2,3 |
| Model location for inference | N/A [downloadable UL codebooks are used in model-transparent manner] 1,3 Two-sided model1,2 |
| Collaboration/interaction between UE and NW | No collaboration for DLable UL codebook As for NR two-sided model |
| Potential specification impact | 1.The procedure related to the download of UL codebooks 2. LCM procedure to facilitate the training of the downloadable UL codebooks or for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## AI for SRS

#### Proposed observation 2.8:

[3 sources] provided preliminary simulation results and analysis on low overhead SRS with AI/ML with details in Table I.

Besides, one source provided preliminary simulation results and analysis on low PAPR SRS sequence design with help of AI/ML.

Table I

| Use case | Low overhead SRS with AI/ML |
| --- | --- |
| Supported companies | (3) {Spreadtrum, UNISOC}, vivo1, Huawei |
| Model input | Received SRS |
| Model output | Estimated channel |
| Label | Ideal channel |
| Training types | Offline training |
| KPI | SCGS, throughput |
| Benchmark | Legacy SRS Ideal channel |
| Preliminary result | Similar SGCS with more overhead 20%~30% throughput gain in low SNR region1 |
| Model location for inference | NW-sided model |
| Collaboration/interaction between UE and NW | No collaboration |
| Potential specification impact | 1.Sparse SRS pattern design 2. Signalling/ procedure related to LCM for NW-sided model |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## AI for PAPR reduction for waveform

#### Proposed observation 2.9:

[3 sources] provided preliminary simulation results and analysis on AI/ML for PAPR reduction with details in Table J.

Table J

| Use case | AI/ML based transform precoding for PAPR reduction |
| --- | --- |
| Supported companies | (3)vivo1, Samsung2, Huawei3 |
| Model input of decoder or model output of encoder | Biased symbol in frequency domain |
| Model output of decoder or model input of encoder | the modulated symbol in frequency domain |
| Label | Label free2 HPE waveform1 |
| Training types | offline training |
| KPI | SGCS, BLER, CCDF of PAPR(UL), throughput (DL) |
| Benchmark | DFT-s-OFDM HPE waveform |
| Preliminary result | >1dB PAPR gain1 3-4dB PARP gain2,3 |
| Model location for inference | Two-sided model |
| Collaboration/interaction between UE and NW | As for two-sided model in NR |
| Potential specification impact | 1. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## AI for Sequence based HARQ-ACK

#### Proposed observation 2.10:

[2 sources] provided preliminary simulation results and analysis on AI/ML for sequence based HARQ-ACK feedback with details in Table K.

Table K.

| Use case | AI/ML for sequence based HARQ-ACK feedback |
| --- | --- |
| Support companies | (2)Qualcomm1, vivo2 |
| Model input | HARQ ACK/NACK bit sequence |
| Model output | Learn sequences |
| Label | HARQ ACK/NACK bit sequence |
| Training types | Offline training |
| KPI | BLER |
| Benchmark | NR RM code for short block length with Maximum Likelihood (ML) receiver |
| Preliminary result | 3-6 dB SNR gain1 1.1-1.9dB SNR gain 2 |
| Model location for inference | NW sided model or NA |
| Collaboration/interaction between UE and NW | Learned codebook or downloadable codebook for HARQ-ACK |
| Potential specification impact | Learned sequence (HARQ-ACK codebook) design or signaling for downloadable Monitoring related procedure, if needed |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## RAN for token traffic

#### Proposed observation 2.11:

[2 sources] provided preliminary simulation results and analysis on RAN for token traffic with details in Table L. Besides, one resource provided preliminary simulation results and analysis on RAN assisted Token communication considering spatio-temporal correlation.

Table L.

| Use case | RAN for token traffic |
| --- | --- |
| Support companies | (2) Huawei1, OPPO2 |
| Model input | UE side: • Input: Raw data (e.g., image/video/audio, etc.) NW or OTT server side: • Input: Tokens or filtered tokens with stamps by RAN |
| Model output | UE side: • Output: Tokens (e.g., tokenized image/video/audio) NW or OTT server side: • Output: Inference results for downstream tasks/Raw data of multiple modalities |
| Label | Label free |
| Training types | Offline training at OTT, transparent to RAN |
| KPI | Supported number of UEs, achievable throughput |
| Benchmark | NR scheduling/HARQ mechanism |
| Preliminary result | 10~30% number of UEs increasing |
| Model location for inference | The tokenizer model is at UE (e.g., an encoder). The de-tokenizer model is at NW or OTT server side (e.g., a decoder). |
| Collaboration/interaction between UE and NW | NA |
| Potential specification impact | • Service awareness in RAN • Spec impact on token error identification, new scheduling and HARQ |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |

## Other use cases

#### Proposed observation 2.12:

The following use cases were proposed by one source with preliminary simulation results and analysis: pathloss prediction (Nokia), AI based UL closed-loop power control (Nokia), prior-information-aided DCI decoding (CMCC), lossless DCI compression (CMCC), early contention resolution in RACH(Ofinno), sensing based RAN digital twin construction with NW-side AI/ML model (Huawei), AI/ML-enabled RAN digital twin with distributed model (Huawei), interference Prediction(vivo), site specific learning for AI/ML and RAN digital twin (Deepsig)

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Anything I missed. Pls note that, this is only for the one with evaluation results. |

## Overall

#### Proposal 1:

TBD

# Framework and evaluation

## Evaluation and KPIs

Several companies discussed aspect on EVM and KPIs. Several companies proposed for comprehensive evaluation of AI/ML use cases by considering KPIs including system performance, system and model complexity, inter-vendor collaboration complexity, power consumption. In addition to intermediate and system KPIs that were adopted in 5G NR, companies proposed new KPIs such power consumption and inference latency to be considered in 6GR.

#### Proposal 3.1 :

For evaluation of AI/ML use cases in 6GR, consider

- Performance related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead)

  - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training

- AI/ML Model related metrics, including model/computational complexity, inference latency, training latency (when applicable),

  - FFS on whether/how to measure power consumption

- Inter-vendor collaboration when applicable

- Generalization performance under a wide range of conditions

  - FFS on whether and how to consider realistic deployment scenarios

Note: Detailed metrics to be discussed per use case.

| Company | Comment |
| --- | --- |
| FL | Leftover from last meeting. The red part may need to explained by proponent companies |

## LCM framework

Many companies proposed enhancement on NR’s LCM, encompassing aspects such as data and model management, including model transfer, applicability of the associated ID, support for localized models, advanced training methods, e.g., online and federated learning, meta-learning for handling network-side additional conditions. Moreover, a number of companies proposed to 5G NR’s LCM framework including functionality-based LCM as a starting point. Enhancement on AI/ML processing unit framework was proposed by a few companies, e.g., 1 company (Samsung) proposed to introduce AI/ML memory unit (MU) on the concurrently activated AI/ML feature/models

#### Proposal 3.2:

For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications (e.g., TS38.331) as a starting point.

- Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include:

  - Data and model management

  - Handling of additional conditions

  - Enablers for continuous (online) on-device model training/finetuning

  - framework for AI/ML processing and memory

[Note: the study strived to provide a unified LCM across use cases]

| Company | Comment |
| --- | --- |
| FL | Some modification according to Ericsson’s proposal |

## Data collection framework

A number of companies discussed data collection framework in their contribution. The following summarizes the discussion points

- Enhancement in the data collection framework for future-proof and unified (across working groups) design.

- Scope and restrictions, e.g., whether to restrict data collection to use cases or to support generic purpose data collection.

- Whether to introduces a new AI/ML data management plane

Some of the proposals may not be under the realm of RAN1. However, RAN1 may identify requirements which may consequently suggest enhancement in the relevant working group. With this in mind, the RAN1 study may focus in identifying requirements that may lead to data collection framework enhancement.

#### Conclusion 3.3:

For AI/ML study in 6GR, RAN1 to study on the content and format for data collection for each use case.

| Company | Comment |
| --- | --- |

# Proposal for online

TBD

# Appendix

## RAN 1 #122

Agreement

For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

- Definition of each (sub-)use case, including at least AI/ML model input/output

- The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

- Assumption on training types, e.g.,

  - offline training, online training/finetuning

  - Label construction (if applicable), including whether/how to obtain label data for model training

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

- Collaboration/interaction between UE and NW, e.g.,

no collaboration/interaction

UE/Network collaboration targeting at separate or joint ML operation

- High level potential specification impact

## Contact information

| Company | Delegate(s) | Email address |
| --- | --- | --- |
| Moderator | Feifei Sun | Feifei.sun@samsung.com |
| Google | Yushu Zhang | yushuzhang@google.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |
| Sharp | Yinan Zhao | Yinan.zhao@cn.sharp-world.com |
| Fainity | Chia-Hung Lin | chlin@fainnov.com |
| Lenovo | Bingchao Liu Vahid Pourahmadi Srinivas Kothapalli | liubc2@lenovo.com vpourahmadi@lenovo.com vkothapalli@lenovo.com |
| CATT | Yongqiang FEI | feiyongqiang@catt.cn |
| SK Telecom | Hyunho Lee | hho.lee@sk.com |
| CMCC | Xiaodong Shen Yi Zheng Yuhua Cao | shenxiaodong@chinamobile.com zhengyi@chinamobile.com caoyuhua@chinamobile.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| Nokia | Keeth Jayasinghe | keeth.jayasinghe@nokia.com |
| ZTE | Xingguang, Wenfeng, Yunqi | wei.xingguang@zte.com.cn liu.wenfeng@zte.com.cn sun.yunqi@zte.com.cn |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Ericsson | Yufei Blankenship Jingya Li Siva Muruganathan | yufei.blankenship@ericsson.com jingya.li@ericsson.com siva.muruganathan@ericsson.com |
| NEC | Peng Guan Pravjyot Deogun Yi Jiang | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM y-jiang_ct@nec.com |
| Panasonic | Henry Tran Tetsuya Yamamoto Hidetoshi Suzuki | xuantuong.tran@sg.panasonic.com yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com |
| NTT DOCOMO | Kosuke Shima Wang Xin Zhang Zhibo | kousuke.shima.nr@nttdocomo.com wangx@docomolabs-beijing.com.cn zhangzb@docomolabs-beijing.com.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| OPPO | Jeffrey Cao Wendong Liu | caojianfei@oppo.com liuwendong1@oppo.com |
| ETRI | Youngjoon Yoon Minhyun Kim | youngjoon.yoon@etri.re.kr minhyun.kim@etri.re.kr |
| Spreadtrum | Shijia shao Zhe yu Mimi chen | Shijia.shao@unisoc.com Zhe.yu@unisoc.com Mimi.chen@unisoc.com |
| CEWiT | Dhivagar Baskaran Shiv Shankar | dhivagar.b@cewit.org.in shivshankar@cewit.org.in |
| TCL | Pu Yuan Tianqi Wu | pu.yuan@tcl.com tianqi1.wu@tcl.com |
| IITK | Chethan Ranganatha | chethanr@iitk.ac.in |
| Tejas Networks | Pavan Kalyan D | pavankalyand@tejasnetworks.com |
| IIT Madras | Anil Kumar Yerrapragada Jeeva Keshav S | anilkumar@5gtbiitm.in jeevak@5gtbiitm.in |
| LGE | Haewook Park | haewook.park@lge.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| AT&T | Salam Akoum | Salam.akoum@att.com |
| Rakuten | Awn Muhammad Abhinav Kishore Mattewada | awn.muhammad@rakuten.com abhinavkis.mattewada@rakuten.com |
| Samsung | Ameha Abebe Zhe Chen Qi Xiong | amehat.abebe@samsung.com tom.chenzhe@samsung.com q1005.xiong@samsung.com |
| Orange | Raphael Visoz | raphael.visoz@orange.com |
| Vivo | Peng Sun | sunpeng@vivo.com |
| MediaTek | Reubengeorge Stephen | Reubengeorge.Stephen@mediatek.com |
| Deepsig | James Lansford | jameslansford@deepsig.io |
| Lekha | Ipshita Panda | ipshita.panda@Lekhawireless.com Sashiganth M sashiganth.m@Lekhawireless.com Sreenath Ramanath sreenath@lekhawireless.com |

# Reference

- R1-2506742	Discussion on AI/ML in 6GR interface	FUTUREWEI

- R1-2506757	Views on AI/ML Operation and Use Cases for 6G Radio Air Interface	Nokia

- R1-2506762	AI/ML Use Cases for 6GR Air Interface	Ericsson

- R1-2506778	Discussion on AI-based Smart Radio for 6G Air Interface	ZTE Corporation, Sanechips

- R1-2506780	Discussion on AI/ML-driven use cases for 6GR	BJTU

- R1-2506784	Discussion on AI/ML in 6GR air interface	TCL

- R1-2506820	Discussion on AIML in 6GR interface	Spreadtrum, UNISOC

- R1-2506821	AI/ML for 6G Air Interface	InterDigital, Inc.

- R1-2506904	Discussion on AI/ML in 6GR interface	vivo

- R1-2506951	AI/ML in 6GR interface	Kyocera

- R1-2506995	Discussion on AI/ML in 6GR interface	Xiaomi

- R1-2507020	Discussion on AI/ML in 6GR interface	CMCC

- R1-2507029	Discussion on AI/ML in 6GR interface	Lekha Wireless Solutions

- R1-2507064	Views on AI/ML in 6GR air interface	Huawei, HiSilicon

- R1-2507107	Views on AI/ML in 6GR interface	CATT, CICTCI

- R1-2507182	AIML use cases for 6GR air interface	OPPO

- R1-2507203	AI/ML in 6GR interface	Tejas Network Limited

- R1-2507259	AI/ML Use cases and framework for 6GR	Samsung

- R1-2507288	Discussion on AI/ML in 6GR	Fujitsu

- R1-2507305	Discussion on AIML in 6GR interface	NEC

- R1-2507377	Discussion on AI/ML in 6GR interface	Panasonic

- R1-2507378	AI/ML Use Cases for 6G	NTU

- R1-2507400	Discussion on AI/ML in 6GR interface	LG Electronics

- R1-2507409	Views on AI/ML in 6GR air interface	SK Telecom

- R1-2507433	AI and ML in 6GR air interface	NVIDIA

- R1-2507471	Views on AI/ML in 6GR interface	Ofinno

- R1-2507488	Discussion on AI/ML Use-cases in 6GR	Lenovo

- R1-2507492	Use cases for AI/ML in 6GR interface	KT Corp.

- R1-2507512	Discussion on AI/ML in 6GR interface	ETRI

- R1-2507522	AI/ML in 6GR Air Interface	Google

- R1-2507525	New use cases for AI/ML in 6GR interface	Pengcheng Laboratory, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2507547	Discussion on AI/ML in 6GR interface	Continental Automotive

- R1-2507601	Discussion on the potential AI/ML use cases for 6GR interface	Sony

- R1-2507604	Discussion on AI/ML for 6GR interface	Ruijie Networks Co. Ltd

- R1-2507605	Discussion on AI/ML Use Cases for 6GR	DeepSig Inc

- R1-2507613	AI/ML Framework and Use Cases for 6GR Air Interface	MediaTek Inc.

- R1-2507633	Discussion on AI/ML in 6GR interface	Transsion Holdings

- R1-2507683	Discussion on 6G AI/ML use cases	Apple

- R1-2507727	AI/ML in 6GR air interface	Qualcomm Incorporated

- R1-2507752	AI/ML use cases and framework for 6GR Air Interface	AT&T

- R1-2507776	Discussions on AI/ML in 6GR interface	Sharp

- R1-2507779	Discussion on AI/ML in 6GR	Shanghai Jiao Tong University, Toyota

- R1-2507821	Discussion on AI/ML for 6GR air interface	NTT DOCOMO, INC.

- R1-2507826	AI/ML in 6GR air interface	KAIST

- R1-2507827	Discussion on AI/ML-enabled use cases for 6GR	BUPT, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2507842	Discussion on AI/ML in 6GR air interface	Hanbat National University

- R1-2507872	Discussion on AI/ML in 6GR Interface	Indian Institute of Tech (M)

- R1-2507883	On new use cases for AI/ML in 6GR interface	Vodafone, Deutsche Telekom

- R1-2507907	AI/ML in 6GR Interface	CEWiT

- R1-2507910	Discussion on AI/ML in 6GR -Physical Layer	Rakuten Mobile, Inc

- R1-2507940	Discussion on AI/ML in 6GR air interface	IIT Kanpur