3GPP TSG RAN WG1 #122bis			R1-250xxxx

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	8.1

Source:	Samsung (Moderator)

Title:	FL summary #0 for AI/ML in beam management

Document for:	Discussion and Decision

- Introduction

This contribution summarised the remaining issues on AI/ML based beam management.

(FL0) Question 0

Please enter/update contact info below to facilitate discussion.

| Company | Contact | Email address |
| --- | --- | --- |
| OPPO | Jeffrey (Jianfei) Cao | caojianfei@oppo.com |
| Ericsson | Henrik Ryden | Henrik.a.ryden@ericsson.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| CMCC | Yi Zheng Jiazhen Zhang | zhengyi@chinamobile.com zhangjiazhen@chinamobile.com |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| vivo | Hao Wu | hao.wu@vivo.com |
| NEC | Peng Guan Pravjyot Deogun Yi Jiang | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM y-jiang_ct@nec.com |
| ETRI | Yongjin Kwon | yjkwon@etri.re.kr |
| Xiaomi | Mingju | limingju@xiaomi.com |
| ZTE | Wenfeng LIU Xingguang WEI | liu.wenfeng@zte.com.cn wei.xingguang@zte.com.cn |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| Panasonic | Henry Tran | xuantuong.tran@sg.panasonic.com |
| CATT | Min Zhu | zhumin@catt.cn |
| Google | Yushu Zhang | yushuzhang@google.com |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |
| LG | Seongwon Go | sw.go@lge.com |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |
| Fraunhofer HHI | Baris Göktepe | Baris.goektepe@hhi.fraunhofer.de |
| KDDI | Taishi Watanabe | ta-watanabe@kddi.com |
| SONY | Chen Sun Yingshuang Bai | chen.sun@sony.com yingshuang.bai@sony.com |
| Hw/HiSi | Yuan Li | liyuan3@huawei.com |
| Apple | Weidong Yang | wyang23@apple.com |
| Nokia | Keeth Jayasinghe | keeth.jayasinghe@nokia.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |

- Remaining issues for UE-side AI/ML model

2.1 APU/CPU

vivo

Proposal 4: Adopt the following TP for TS 38.214 Clause 5.2.1.6

5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to non-zero OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter [RRC_name-r19], OCPU= OCPU,1 is considered.

For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,2 simultaneous CSI calculations it is said to have NCPU,2 CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,2-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,2-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to non-zero OCPU,2(n), the UE is not required to update the N-M2 requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M2≤N is the largest value such that n=0M2-1OCPU,2(n)≤NCPU,2-L  holds.

Huawei

Table 2 Text proposal for PU occupation rule

#### 5.2.1.6 CSI processing criteria The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to non-zero OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter [RRC_name-r19], OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to non-zero OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report with non-zero OCPU,1 and OCPU,2 is not considered within any of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report.

Proposal 7: For the PU occupation rule, only the CSI reports requiring to occupy both CPU and APU (i.e., non-zero OCPU,1 and non-zero OCPU,x) but not considered within any of M and Mx are not required to update.

- For a CSI report which requires to occupy CPU only (OCPU,x=0) /APU only (OCPU,1=0), it should be exempted from being blocked for its CPU/APU occupation (and corresponding CSI update) if the APU/CPU pool, respectively, cannot satisfy the required occupation of this CSI report.

- Consider the text proposal in Table 2 for the PU occupation part in 38.214.

OPPO

Proposal 3: If any condition of APU or CPU cannot be met

- The CSI is not required to be updated by UE

- The values for OCPU,1  and OCPU,x are considered to be zero

Samsung

Proposal 5: Adopt the following TP for TS 38.214 Clause 5.2.1.6 for CSI report for inference.

Reason for change: The M and Mx for determining whether CSI reporting for inference is required to be updated or not is unclear.

Summary of change: Clarify that the M and Mx are determined prior to any of CSI report with corresponding OCPU,1 and OCPU,x considered to be 0.

Consequences if not approved: gNB and UE may have different understanding on the M and Mx when determining whether CSI reporting for inference is required to be updated or not.

TP for TS 38.214 Clause 5.2.1.6 CSI processing criteria <omitted texts> For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report is not considered within any of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report, where the M and Mx are determined prior to any of CSI report with corresponding OCPU,1 and OCPU,x considered to be 0. <omitted texts>

Google

Proposal 2-1: Clarify the CPU/APU allocation based on a CPU-first operation and endorse the following TP for 38.214.

5.2.1.6 CSI processing criteria The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter [RRC_name-r19], OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. The UE allocates the CPUs for the CSI reports that requires OCPU,1first, and then allocates the CPUs for the remaining CSI reports.

Apple

Proposal 1-1: For simultaneous CSI reports, the determination procedure is conducted on them one by one sequentially according to priority rules for CSI reports (Clause 5.2.5 TS 38.214):

- if any condition for CPU and APU is not met, the CSI report is not required to be updated.

Sharp

Proposal 2: Adopt the following TP#2 in TS 38.214 to clarify that, for a CSI report whose required OCPU,1 and OCPU,2 are both nonzero, if any one of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the values for OCPU,1 and OCPU,2 of the CSI report are considered to be 0.

- Reason for change: The current TS38.214 has not capture the note in the following agreement.

Agreement For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, - For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

- Summary of change: Add descriptions to clarify that, for a CSI report whose required OCPU,1 and OCPU,2 are both nonzero, if any one of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the values for OCPU,1 and OCPU,x of the CSI report are considered to be 0

- Consequences if not approved: The Note in above agreement has not captured in the spec.

TP#2 5.2.1.6 CSI processing criteria <unchanged parts are omitted> The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter [RRC_name-r19], OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report with nonzero reported OCPU,1 and nonzero reported OCPU,x is not considered within any one of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows: - OCPU=0 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured. - OCPU=1 for a CSI report with ltm-CSI-ReportConfig or a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index', 'cli-SRS-RSRP', 'cli-RSSI', 'rs-pai-r19', 'none-bm-r19', 'none-csi-r19' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured) or a CSI report with CSI-ReportConfig with higher layer parameter eventType configured. - OCPU=X for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'csi-pai-r19', where the value of X is reported by UE capability. - OCPU=Y+1⋅X, for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'tdcp' and with number of delays Y configured by higher layer parameter Y, where the value of X∈{1, 2} is reported by UE capability. <unchanged parts are omitted>

Qualcomm

Proposal 2: Adopt the following TP for TS 38.214 Clause 5.2.1.6.

Reason for change: The CSI processing criteria for AI/ML CSI reports may prioritize lower priority CSI reports over higher priority ones which is in contrast to the legacy CSI processing criteria.

Summary of change: Update the CSI processing criteria for AI/ML CSI reports.

Consequences if not approved: AI/ML CSI reports with lower priority may be prioritized over AI/ML CSI reports with higher priority, which contrasts with the definition of priority rules as well as the CSI processing criteria for legacy CSI reports.

TP for TS 38.214, Clause 5.2.1.6, CSI processing criteria <omitted texts> For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report is not considered within any of M and Mx, the values for OCPU,1 and OCPU,x are considered to be 0, for the procedure previously described in this clause, and the UE is not required to update the CSI report. <omitted texts>

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.1.1# The determination of updated CSI reports based on CPU and APU

Several companies [3,4,8,9,17,19,20,21] provided TPs to clarify the procedure for the determination of updated CSI reports based on CPU and APU. Based on the discussion in RAN1#122, three alternatives are identified based on company inputs.

Discussion. 2.1.1

For procedure for the determination of updated CSI reports based on CPU and APU, the following alternatives are identified:

Alt 1: Handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated.

  - Spec impact: Delete the corresponding paragraph or make the change as in QC’s TP (proposal 2)

  - Supported by: QC, Apple(?)

Alt 2: handle the procedure for CPU and APU in sequential, and release the occupation based on the outcome.

  - Alt 2-1: CPU first. If the condition for CPU is not met, the corresponding APU is considered to be zero for subsequent procedure for APU.

  - Spec impact: make the change as in Google’s TP (proposal 2-1)

  - Supported by: Google

Alt 3: handle the procedure for CPU and APU together.

  - Step 1: determinate M and Mx; where M is determined based on CPU and Mx is determined based on APU

  - Step 2: Determine a set of CSI reports that are not within M (i.e., the condition for CPU is not met) or not within Mx (i.e., the condition for APU is not met)

    - The corresponding CPU and APU are considered to be zero for the set of CSI reports

  - Step 3: determined M’ (i.e., the condition for CPU is not met) or not within M’x (i.e., the condition for APU is not met)

    - The determination of M’ and M’x is based on the outcome of step 2

  - More clarification in the spec is needed

  - Supported by: vivo, HW, OPPO, Sharp, Samsung

| Company | Comments |
| --- | --- |
| FL | Please share your view on the preferred alternatives. |
| Google | Alt3 is not quite clear to us. In step 1, is it correct that the M and Mx are determined separately for each pool based on the priority of CSI report? In step 2, is it correct that the determined set of CSI reports will be dropped (not updated)? In step 3 it seems the definition of M’ and Mx’ are missing? It is unclear to us which CSI reports should be dropped (not updated) and which should not. Alt1 does not seem to be completed. In current CPU rule, if number of remaining CPUs is not fit for one CSI report, all the remaining CSI report will be dropped (not updated). For Alt1, what happens to the other CSI report if any condition for CPU and APU is not met? |
| Huawei, Hisilicon | Update Alt 3 as follow. - Step 1: determinate M, where M is the set of CSI reports that require to occupy non-zero CPU (i.e., require CPU only or CPU+APU), and the condition for CPU is met; - other CSI reports requiring to occupy non-zero CPU are not updated - Step 2: determinate Mx, where Mx is the set of CSI reports that require to occupy non-zero APU (i.e., require APU only or CPU+APU), and the condition for APU is met. - other CSI reports requiring to occupy non-zero APU are not updated - Step 3: Determine a set of CSI reports that require to occupy non-zero CPU and non-zero APU, and are not within M (i.e., the condition for CPU is not met) or not within Mx (i.e., the condition for APU is not met) - The corresponding CPU and APU are considered to be zero for the set of CSI reports We think vivo and Sharp have the similar view as above. |
| Xiaomi | In my understanding of Alt3, in step 2, CSI reports with top k priority were within either M or Mx. But e.g., the CSI report with kth priority need to be dropped because of the condition of CPU or APU is not met. Thus, the APU and CPU of the CSI report with kth priority will be released. Then, in step 3, CSI report with the (k+1) priority will be considered whether it can be within both M and Mx, and so on… If my understanding is correct, we support Alt 3. |
| CATT | Prefer to support Alt.1. |
| OPPO | Sorry for misleading of our preference to Alt.3. For the APU and CPU issues, it seems Alt.1 is simpler and clearer to us. That is “if any condition for CPU and APU is not met, the CSI report is not required to be updated.” |
| Ericsson | Alt 1 seems simpler and hence support Alt 1. |
| SPRD | Similar view with Xiaomi. If step 3 is intended to update the results obtained from step 2, then it is necessary to provide a more detailed explanation. Otherwise, we can support simpler Alt 1 instead. |
| CMCC | Prefer Alt1. |
| NEC | Prefer Alt1. |
| Fujitsu | Slightly prefer with Alt 1. |
| Sharp | The updated Alt.3 from Huawei looks very clear and definite and we support it. In our understanding, the motivation of the proposal is to capture the note agreed in the RAN1# 121- Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied. Although Alt.1 looks simple, Alt.1 with deleting the corresponding paragraph will result in one PU still being occupied and contradict with the note we agreed. Alt.2 introduces a problematic consequence that a lower priority CSI report can be updated than a higher priority CIS report. This violates the legacy priority-based CSI report determination. For Alt.3, regarding some companies’ concern above, we believe the released PUs should not be used for updating the M and Mx CSI reports. The updated Alt.3 from Huawei will resolve the concerns. |

2.2 CSI report for model inference

Proposals related to CSI report for model inference are summarized as follows.

Ericsson

Proposal 1. For Rel-19 AI/ML based BM, remove the following configuration restrictions in TS38.214 when the CSI Resource Setting is for Set A in a CSI report configuration for inference

- All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources

- UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within aperiodic resource setting

vivo

Proposal 2: For inference, for UE-side model, Set A or CSI-RS resources in Set A should be indicated as virtual Set or virtual resource,

- CSI-RS Resources in Set A shall not be linked to another CSI resource setting for channel measurement

- Set A shall not be considered for PDSCH rate matching

- Set A is not counted as active resource

- Configurations of Set A in the inference CSI report should be ignored, at least including CSI-RS-resourceMapping and resource type.

Huawei

Proposal 3: For the inference of UE-side model, some of the UE assumptions on the resource configuration in legacy spec needs to be revisited for the resource setting of Set A, including:

- UE does not need to perform rate matching around the RS, if the RS is configured only in Set A of the inference CSI report.

- The following configuration restrictions are not applicable to the CSI Resource Setting for Set A of the inference CSI report:

  - When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs.

    - Change to: For Set A of an inference report, when a RS ID in the Set A is configured in another resource configuration (which is actually transmitted), it does not need to keep the same time domain behavior to the Set A.

  - All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior.

    - Change to: For Set A of an inference report, Set B does not need to keep the same time domain behavior to the Set A configured in the same CSI report configuration.

  - All the CSI-RS resources within one set are configured with the same periodicity.

    - Change to: For Set A of an inference report, when two RS IDs in the Set A are also configured in other resource configurations (which are actually transmitted) with P/SP resource type, these two P/SP RS do not need to keep the same periodicity.

  - UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within aperiodic resource setting.

    - Change to: For aperiodic resource setting (if needed) of Set A of an inference report, the number of CSI-RS resources can be more than 16.

Proposal 4: For the applicability report Option B Step 3, some of the UE assumptions on the resource configuration in legacy spec needs to be revisited for the resource setting of Set A/Set B, including:

- UE does not need to perform rate matching around the RS, if the RS is configured only in Set A (resourcesForSetA-r19) and/or Set B (resourcesForChannelMeasurement) for applicability check.

- The following configuration restrictions are not applicable to CSI Resource Setting for Set A and Set B for applicability check:

  - When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs.

    - Change to: For Set A/Set B for applicability check, when a RS ID in the Set A/Set B is configured in another resource configuration (which is actually transmitted), it does not need to keep the same time domain behavior to the Set A/Set B.

  - All the CSI-RS resources within one set are configured with the same periodicity.

    - Change to: For Set A/Set B for applicability check, when two RS IDs in the Set A/Set B are also configured in other resource configurations (which are actually transmitted) with P/SP resource type, these two P/SP RS do not need to keep the same periodicity.

Proposal 8: For applicability report procedure, more than one A-CSI-RS resource sets can be configured in one CSI-ResourceConfig referred by CSI-ReportConfigId (for Option A) and ApplicabilitySetConfigId (for Option B).

- To resolve the ambiguity, gNB needs to additionally indicate the specific A-CSI-RS resource set referred by CSI-ReportConfigId / CSI-ResourceConfigId for applicability report.

OPPO

Proposal 2: For the virtual resource set, i.e. Set A for inference, RAN1 considers and determines the following aspects

- Remove the restriction of the same time domain behavior of all CSI-RS resources in virtual Set A

- PDSCH doesn’t need to rate match around the REs of CSI-RS resources in virtual Set A

- The maximum number of CSI-RS or SSB resources in virtual Set A

Samsung

Proposal 1: Adopt the following TP for TS 38.214 Clause 5.2.1.4.3a for BM-Case2.

Reason for change: The time domain relation between a reference time and the earliest time instance is unclear.

Summary of change: Clarify that the earliest time instance is after the reference time.

Consequences if not approved: gNB and UE may have different understanding on whether the earliest time instance is before or after the reference time.

TP for TS 38.214 Clause 5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <omitted texts> For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', the UE shall determine the reported P-CRI, P-SSBRI, and/or P-L1-RSRP based on: - L1-RSRP measurements performed by the UE for the configured CSI-RS resources, or SS/PBCH block resources associated with the first Resource Setting given by resourcesForChannelMeasurement; and - prediction is performed by the UE based on the L1-RSRP measurements associated with the first Resource Setting, for the configured CSI-RS resources, or SS/PBCH block resources associated with the second Resource Setting given by resourcesForSetA-r19. For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)), where the earliest time instance is after the reference time. If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. <omitted texts>

NEC

Proposal 1: For P-L1-RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam in Set A. Adopt the following TP to TS 38.214 V19.1.0.

- Reason for change: No Tx Power assumption for P-L1-RSRP determination.

- Summary of change: P-L1-RSRP is determined by assuming the powerControlOffsetSS of the CSI-RS resources corresponding to P-CRI.

- Consequences if not approved: P-L1-RSRP is obtained with no Tx Power assumption.

5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <Unchanged part omitted> For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', the UE shall determine the reported P-CRI, P-SSBRI, and/or P-L1-RSRP based on: - L1-RSRP measurements performed by the UE for the configured CSI-RS resources, or SS/PBCH block resources associated with the first Resource Setting given by resourcesForChannelMeasurement; and - prediction is performed by the UE based on the L1-RSRP measurements associated with the first Resource Setting, for the configured CSI-RS resources, or SS/PBCH block resources associated with the second Resource Setting given by resourcesForSetA-r19. - P-L1-RSRP is determined by assuming ss-PBCH-BlockPower, and if provided, powerControlOffsetSS of the CSI-RS resources corresponding to P-CRI if reportQuantity-r19 set to 'p-cri-RSRP-r19'. <Unchanged part omitted>

Proposal 2: The configured P/SP NZP-CSI-RS resources for Set A (and not for Set B) should be available for other channel/signal (e.g., PDSCH) at least during model inference. Adopt the following TP to TS 38.211 V19.1.0.

- Reason for change: Set A is for prediction report, however, P/SP CSI-RS resources configured for Set A would not be available for PDSCH RE mapping and cause large overhead.

- Summary of change: Clarify that resources configured in Set A and not in Set B will be available for PDSCH.

- Consequences if not approved: resources in Set A always are always occupied.

7.3.1.5 Mapping to virtual resource blocks <Unchanged part omitted> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is an periodic or semi-persistent non-zero-power CSI-RS resource configured in the second Resource Setting (given by higher layer parameter resourcesForSetA-r19) and not configured in the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) of a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19'; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. <Unchanged part omitted>

Proposal 4: Adopt the following TP to TS 38.214 V19.1.0.

- Reason for change: Set A size is restricted to 16 resources for AP report, which is too small for Set A.

- Summary of change: Clarify that the up to 16 resources restriction does apply to Set A.

- Consequences if not approved: Set A size is up to 16 for AP report.

5.2.1.4.2 Report quantity configurations <Unchanged part omitted> If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index', 'cri-SINR- Index, 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', or ‘rs-pai-r19', and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set except the resource set configured by resourcesForSetA contained within the resource setting. <Unchanged part omitted>

Panasonic

Proposal 7: Support mapping/association of beams within Set A and beams within Set B based on QCL relationship.

Proposal 13: Support to define that the predicted Top K beam(s) are the best K beam(s) based on the predicted RSRP or the probability of each beam in Set A to be the Top-1 beam

- The trade-off between the predicted RSRP values or the probability of the correctness of each beam in Set A to be the Top-1 beam are up to UE implementation.

Nokia

Proposal 1: For UE-sided BM Case-1 and 2, for AP CSI reporting for inference, endorse the following text proposal to 38.214 Clause 5.2.1.

Reason for change: It is not clear how the trigger for AP CSI Report can carry the information for two Resource Set IDs.

Summary of change: Support in AP CSI triggering state of an additional indication with Resource Set ID for RS prediction for Set A selection. The associated CSI-ReportConfig corresponding to a trigger state in CSI-AperiodicTriggerStateList shall indicate, in addition to the Resource Set ID for channel measurement, a second Resource Set ID for RS prediction.

Consequence if not approved: If this issue is not addressed, the UE may face ambiguity in determining RS resources to be predicted.

5.2.1 Channel state information framework < Unchanged parts are omitted > For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex, TDCP, L1-SRS-RSRP, L1-CLI-RSSI, SRS-RSRP-MRI, CLI-RSSI-MRI, CSI-PAI, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI, CJTC-Dd, CJTC-F, CJTC-Dd-F, CJTC-P, a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings and/or X≥1 ltm-CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings and/or Y≥1 LTM-CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs or one associated LTM-CSI-ReportConfig indicating the Resource Set IDs for channel and optionally for interference where a Resource Set for interference can only be present for a Report Setting given by a CSI-ReportConfig and a trigger state additionally contains one or more reportSubConfigId if the associated CSI-ReportConfig configured with a list of sub-configurations, as described in Clause 5.2.1.1. For P-CRI, P-SSBRI, and P-L1-RSRP, the associated CSI-ReportConfig corresponding to a trigger state in CSI-AperiodicTriggerStateList shall indicate, in addition to the Resource Set ID for channel measurement, a second Resource Set ID for RS prediction. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig or LTM-CSI-ReportConfig, and a trigger state additionally contain one or more reportSubConfigId if the associated CSI-ReportConfig is configured with a list of sub-configurations, as described in Clause 5.2.1.1. < Unchanged parts are omitted >

Proposal 2: For UE-sided BM Case-1 and 2, for CSI-RS resources in Set A, endorse the following text proposal to 38.214 Clause 5.2.2.3.1.

Reason for change: The same-periodicity requirement restricts the flexibility needed for AI/ML beam management, particularly when dealing with subsets of resource sets that require different transmission periodicities.

Summary of change: Allow different CSI-RS resources within a single set to have varied time-domain transmission behaviors when the resources belong to resourcesForSetA-r19, which are used for prediction.

Consequence if not approved: If this issue is not addressed, there could be ambiguity in determining the periodicity of CSI-RS resources in Set A.

5.2.2.3.1 NZP CSI-RS < Unchanged parts are omitted > The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s). The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration: - nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity. - periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. Except resources in resourcesForSetA-r19 used for predicted RS quantities reporting, aAll the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources. - resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211]. - nrofPorts in resourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. - density in resourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid. - cdm-Type in resourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. < Unchanged parts are omitted >

Lenovo

Proposal: When a periodic or semi-persistent CSI-RS resource set is associated with a CSI report configuration as prediction beam set A and it is not associated with another CSI report configuration for data collection, the resources corresponding to the periodic CSI-RS or activated semi-persistent CSI-RS are available for PDSCH resource mapping.

7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource, or except if the non-zero-power CSI-RS is configured in the second Resource Setting given by resourcesForSetA-r19 in a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19' and the non-zero-power CSI-RS is not configured a Resource Setting associated with a CSI-ReportConfig with reportQuantity-r19 set to 'none-bm-r19'; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l.

Google

Proposal 2-2: Endorse the following TP for 38.214 for collision handling between predicted CRI/SSBRI/RSRP report and SRS for carrier switching.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/P-CRI/P-SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/P-L1-RSRP, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P/P-L1-RSRP on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol.

Proposal 2-3: Endorse the following TP for 38.214 for collision handling between predicted CRI/SSBRI/RSRP report and SRS on the same carrier.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH.

Proposal 2-6: Endorse the following TP for 38.214 to clarify that UE does not report the inference results if it fails to receive K transmission occasions within the DRX active time

5.2.2.5 CSI reference resource definition <omitted text> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability.

Proposal 2-9: Introduce a MAC-CE for AI/ML functionality activation/deactivation (i.e., CSI report configuration activation) with regard to the following issues from the coupling of inference report triggering and functionality activation by one signaling

- Issue #1: Current scheduling offset cannot cover the duration for UE to measure X times of SSB (X=1 for BM case 1 and X can be more than 1 for BM case 2), which is required for to measure the RSRP for the Set B beams after activation of the functionality

- Issue #2: Large model activation delay (especially when APU is occupied) would result in large processing delay for inference report

InterDigital

Proposal 1: Support the following:

- Support indication of transmitted/untransmitted RSs configured for Set A based on presence/absence of physical RS transmission parameters in CSI resource configs (i.e., resourceMapping, and CSI-ResourcePeriodicityAndOffset).

- For virtual resource blocks mapping of PDSCH in TS38.211, rate matching of the resource elements in the corresponding physical resource blocks is performed if NZP CSI-RS configured in Set A for CSI report for inference is transmitted.

- For virtual resource blocks mapping of PDSCH in TS38.211, rate matching of the resource elements in the corresponding physical resource blocks is not performed if NZP CSI-RS configured in Set A for CSI report for inference is not transmitted.

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.2.1# Relaxation of configuration restriction for Set A

It is proposed by several companies [1,3,4,8,11,13,15,18] that legacy configuration restriction for CSI Resource Settings shall be relaxed for Set A. Based on the discussion in RAN1#122, the following configuration restriction in TS38.214 are identified which is not applicable to CSI Resource Setting for Set A for CSI report for inference:

- A) When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs.

  - Supported by: HW, Nokia (Set A used for monitoring)

- B) All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior.

  - Supported by: HW, OPPO

- C) All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources

  - Supported by: Ericsson, HW, Nokia

- D) UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within aperiodic resource setting.

  - Supported by: Ericsson, HW, OPPO, NEC

- E) For virtual resource blocks mapping of PDSCH in TS38.211, rate matching of the resource elements in the corresponding physical resource blocks are not performed around the NZP CSI-RS only configured in Set A for CSI report for inference.

  - Supported by: vivo, HW, OPPO, NEC, Lenovo, InterDigital

Proposal. 2.2.1

The following configuration restriction in TS38.214 is not applicable to CSI Resource Setting for Set A for CSI report for inference:

- All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources

- UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within aperiodic resource setting.

- For virtual resource blocks mapping of PDSCH in TS38.211, rate matching of the resource elements in the corresponding physical resource blocks are not performed around the NZP CSI-RS which is only configured in Set A for CSI report for inference.

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Aspect C, D, E are selected due to the majority support and no strong concern (based on the comments from last meeting). Please share your view. |
| Google |  | For inference, we think the first bullet is not necessary. Others should be fine. The third bullet may not be quite necessary if the set A CSI-RS is aperiodic. Currently aperiodic CSI-RS is not included for rate matching. |
| Huawei, HiSilicon |  | 1, We think A) and B) are needed. Assuming Set A occupies full resources assigned with a specific resource type, then other resource sets cannot choose a different resource type, which is quite restrictive. 2, Set A should be Set A only, i.e., if a CSI-RS resource is also configured in another actually transmitted resource set, then the legacy restrictions are still valid. The following configuration restriction in TS38.214 is not applicable to CSI-RS resources only configured in CSI Resource Setting for Set A for CSI report for inference - When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs. - All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior - All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources - UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within aperiodic resource setting. - For virtual resource blocks mapping of PDSCH in TS38.211, rate matching of the resource elements in the corresponding physical resource blocks are not performed around the NZP CSI-RS which is only configured in Set A for CSI report for inference. |
| Xiaomi |  | Support |
| CATT |  | We support this proposal, since the CSI Resource Setting for Set A is only for inference and doesn’t need to be measured. |
| LGE |  | Need more discussion. It seems that the first bullet and third bullet are mutually contradicting. As far as we understand, the intension of relaxing first bullet for Set A is to allow some resources of Set A can be actually transmitted (not as virtual resource), for example, for performance monitoring RS. Then, relaxing of third bullet (rate-matching) for Set A is not correct since some of Set A is actually transmitted. So, the first bullet is not needed. Please correct me if my understanding is wrong. |
| QC |  | The necessity of first bullet needs further discussion |
| OPPO |  | We are supportive of this direction. There is no reason to apply those restrictions to a virtual resource set, i.e. Set A for inference. But one controversial point is whether those restrictions should be applied to resources not only configured in Set A for inference. One example could be resource #1 configured in Set A for inference and also configured within another resource set as AP CSI-RS. When the AP CSI-RS is not triggered and Set A is scheduled virtually with a PDSCH, should UE do rate matching or not? This needs more discussion in our view. |
| Ericsson |  | Open to discuss this proposal. We note that optimizations should be avoided at this stage since we are in maintenance phase. |
| SPRD |  | Support |
| CMCC |  | Fine with the proposal. |
| NEC |  | All proposals are OK to us in general. |
| Fujitsu |  | For the first bullet, we think it requires more discussion on the time domain behavior for Set A for inference. The second bullet is ok. For the third bullet, agree with Google regarding the aperiodic CSI-RS. |
| Sharp | Y | Support the proposal. |

Issue 2.2.2# The condition for dropping CSI report for inference

In [17], it is proposed by Google that DRX needed to be additional considered on the dropping of CSI report for inference.

Proposal. 2.2.2

Endorse the following TP for 38.214 to clarify that UE does not report the inference results if it fails to receive K transmission occasions within the DRX active time.

5.2.2.5 CSI reference resource definition <omitted text> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability. <omitted text>

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | The TP seems necessary since the same condition is captured in TS38.214 on the dropping conditions of other CSI reporting. Please share your view. |
| Google |  | Support. UE does not perform measurement during DRX inactive time. |
| Xiaomi |  | Support |
| CATT |  | Support |
| QC |  | As discussed in RAN1 #122, due to the nature of temporal beam prediction, it is too restrictive to limit this to “within the same DRX active time”, as the number of measurement occasions could be quite limited, and the observation window may span 10s or 100s of ms. This is different compared to e.g., CSI prediction for which the observation window is considerably smaller, and “within the same DRX active time” makes sense. |
| OPPO |  | Assume this condition is not added, can we rely on NW’s implementation to handle this? 1. NW configures the measurements for BM-Case2 are within the same DRX active time? Or 2. NW configures the measurements for BM-Case2 are across different DRX active times and there is no measurement missing. |
| Ericsson |  | As mentioned by Qualcomm, the length of the observation window may need careful consideration before agreeing to this proposal. |
| SPRD |  | Support |
| NEC |  | OK |
| Fujitsu |  | Support |

Issue 2.2.3# Collision handling of CSI report for inference and other signals/channels

In [17], it is proposed by Google that the collision handling rule for CSI report for inference and other signals/channels shall follows the same rule for legacy beam report.

Proposal. 2.2.3a

Endorse the following TP for 38.214 for collision handling between predicted CRI/SSBRI/RSRP report and SRS for carrier switching.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/P-CRI/P-SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/P-L1-RSRP, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P/P-L1-RSRP on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol.

Proposal. 2.2.3b

Endorse the following TP for 38.214 for collision handling between predicted CRI/SSBRI/RSRP report and SRS for carrier switching.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH.

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | These two proposals are natural extension on the collision handling rule for legacy beam report to the CSI report for inference. Please share your view. |
| Google |  | Support |
| Huawei, HiSilicon |  | OK |
| Xiaomi |  | Support |
| CATT |  | Support |
| LGE |  | OK |
| QC |  | OK |
| OPPO |  | Okay to discuss it. But a minor question would be what is the P-L1-RSRP report(s)? Predicted RSRP only? Or with predicted CRI/SSBRI? |
| Ericsson |  | Support |
| SPRD |  | Support |
| CMCC |  | Ok |
| Fujitsu |  | OK |
| Sharp |  | Support |

Others

HW [1] proposed that gNB needs to additionally indicate one AP CSI-RS resource set out of multiple resource sets referred by CSI-ReportConfigId / CSI-ResourceConfigId for applicability report. However, it is understood that each of the resource sets should be checked for applicability report and there is no RAN1 impact on that.

NEC [11] proposed to use configured powerControlOffsetSS of the resource in Set A as UE assumption for the derivation of predicted L1-RSRP. However, based on the comments from companies, it seems difficult to reach consensus at this stage.

Google [17] proposed to introduce a new MAC-CE for AI/ML functionality activation/deactivation (i.e., CSI report configuration activation). However, it seems to be an optimization of the PUSCH based SP beam reporting.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| Google | To clarify our proposal on activation/deactivation. This is not an optimization. When the set B RS is based on SSB, if UE only activates the functionality after receiving the DCI triggering the AP report (UE will not measure the SSB before receiving the DCI since the functionality is not activated), the scheduling has to be as the figure below. Current scheduling offset does not support such kind of scheduling. So the AP report based on SSB as Set B is broken. But AP inference is the most typical use case. |
| NEC | We also would like to clarify that predicted L1 RSRP without any Tx power assumption is meaningless, as one example in our tdoc, -90dBm P-L1-RSRP assuming 30dBm Tx power and -90dBm P-L1-RSRP assuming 33dBm Tx power are two different prediction results. |

2.3 CSI report for performance monitoring

Ericsson

Proposal 2. For the purpose of linking the measurement result of the nth latest transmission occasion of the CSI-RS/SSB resources for monitoring to an inference report, use the first slot of the CSI-RS/SSB resources of the transmission occasion.

vivo

Proposal 3. Adopt the following TP for TS 38.214 Clause 5.2.1.4.3b

Reason for change: The condition of monitoring beam determination is unclear.

Summary of change: Modify the condition of monitoring beam determination.

Consequences if not approved: gNB and UE may have different understanding on the condition of monitoring beam determination.

5.2.1.4.3b	RS-PAI Reporting

<omitted texts>

-	for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall:

-	perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting;

-	determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on largest L1-RSRP(s) value(s) of measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set;

<omitted texts>

Huawei

Proposal 5: For monitoring of UE-side model, considering the case of A-CSI report for inference and P/SP-CSI-RS for monitoring, duplicated mapping from more than one monitoring RS to one prediction result should be precluded when determining the conditions of linked pairs.

Table 1 Text proposal for the mapping between Set A and monitoring resource set

#### 5.2.1.4.3b RS-PAI Reporting <omitted texts> - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein - if the number of resources in the resource set for the second CSI Reporting Setting is smaller than the number of resources in the resource set given by resourcesForSetA-r19, the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if the number of resources in the resource set for the second CSI Reporting Setting is the same as the number of resources in the resource set given by resourcesForSetA-r19, the n-th resource of the resource set for channel measurement of the second CSI Reporting Setting is mapped to the n-th resource of the resource set given by resourcesForSetA-r19; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; <omitted texts>

Proposal 6: Consider the text proposal in Table 1 for the mapping between Set A and monitoring resource set in 38.214.

Xiaomi

Proposal 1: Define the first slot of the CSI-RS/SSB resources for monitoring as the reference point of the transmission occasion to determine the linked inference report. And support the corresponding TP to 38.214.

Reason for change: The condition associated with CSI reporting for RS-PAI is unclear.

Summary of change: Clarify that the slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting is the first slot for condition check.

< Start of change request >

##### 5.2.1.4.3b	RS-PAI Reporting

<Unchanged part omitted>

-	check a condition :

-	for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting;

<End of change request>

< Start of change request >

##### 5.2.1.4.3b	RS-PAI Reporting

<Unchanged part omitted>

-	check a condition :

-	for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting;

<End of change request>

Consequences if not approved: gNB and UE may have different understanding on the condition associated with CSI reporting for RS-PAI.

Samsung

Proposal 2: Adopt the following TP for TS 38.214 Clause 5.2.1.4.3b for CSI reporting for RS-PAI.

Reason for change: The slot of transmission occasion for monitoring is unclear for the linkage of the corresponding CSI report for inference.

Summary of change: Clarify the first slot of transmission occasion for monitoring is used for the linkage.

Consequences if not approved: gNB and UE may have different understanding on the linkage.

##### TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting <omitted texts> - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; <omitted texts>

Fujitsu

Proposal 1: The following TP on the resource mapping between the resource set for monitoring and Set A is proposed to be captured into TS 38.214.

##### 5.2.1.4.3b RS-PAI Reporting <omitted texts> … - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s) if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein - if the size of the resource set for channel measurement of the second CSI Reporting Setting is smaller than the size of the Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting, the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting, where RSMappingtoSetA is a X-bit bitmap, and the y-th nonzero bit of the bitmap corresponds to the y-th entry of the resource set for channel measurement of the second CSI Reporting Setting, and X is the size of the Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting; - if the size of the resource set for channel measurement of the second CSI Reporting Setting is the same as the size of the Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting, the n-th resource in the resource set for channel measurement of the second CSI Reporting Setting is linked to the n-th resource in the Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; … <omitted texts>

Nokia

Proposal 3: For UE-sided BM Case-2 AP CSI report configuration, endorse the following text proposal to 38.214 Clause 5.2.1.4.3b.

Reason for change: To ensure that monitoring results are properly calculated at the UE, it is to clarify that for AP Report for inference, the number of transmission occasions triggered by AP report for monitoring should be limited to M = 1. With this limitation, AP monitoring of AP inference is not useful, though there may be alternative ways to address this issue.

Summary of change: For AP Report for inference, limit the number of transmission occasions triggered by AP report for monitoring to M = 1.

Consequence if not approved: If this change is not implemented, there could inconsistencies in determining monitoring results at the UE.

##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: < Unchanged parts are omitted > - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a -bit field (M is given by nroftransmissionOccasion-r19). When both the first and the second CSI Reporting Settings are aperiodic, the UE is not expected to configure with a value larger than 1 for nroftransmissionOccasion-r19. < Unchanged parts are omitted >

NEC

Proposal 2: For a monitoring report, only if its linked inference report is activated, the UE reports RS-PAI; otherwise, the UE shall drop the monitoring report. Adopt the following TP to TS 38.214 V19.1.0.

Reason for change: For a monitoring report, if its linked inference report is stopped during performance monitoring, the UE will no longer be able to obtain the inference results linked with the (remaining) monitoring instance(s). At this time, the corresponding monitoring behavior should be defined.

Summary of change: For a monitoring report, if the linked inference report is activated, the UE may report the monitoring report; otherwise, the UE may drop the monitoring report.

Consequences if not approved: wrong RS-PAI report after the linked inference report is stopped.

##### 5.2.1.4.3b RS-PAI Reporting <Unchanged part omitted> - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). - the UE reports a RS-PAI report of the second CSI Reporting Setting only if the report of the first CSI Reporting Setting is activated, and drops the report of the second CSI Reporting Setting otherwise. <Unchanged part omitted>

Panasonic

Proposal 12: For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support aperiodic CSI-RS when N =1 is configured.

Proposal 14: Group-based beam reporting can be enhanced to support performance monitoring for NW-sided model.

LG

Proposal #7: Endorse the following TP for TS38.214.

| Reason for change | In current specification, it is described that UE determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on L1-RSRP(s) measured CSI-RS resources, or SS/PBCH Bock resources of the corresponding Resource Set, for ‘rs-pai-r19’ calculation. However, the meaning of ‘the best’ is not clear in the specification from UE behavior perspective. |
| --- | --- |
| Summary of change | Change “the best” to “with largest L1-RSRP value(s)”. |
| Consequences if not approved | Different UE vendor may have different understanding on “the best” nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources, so ‘rs-pai-r19’ may not be calculated correctly. |

<S5.2.1.4.3b> ##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: - for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall: - perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on largest L1-RSRP(s) value(s) of measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set; - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). ============================= unchanged parts are omitted ============================

Google

Proposal 2-4: Endorse the following TP for 38.214 for collision handling between RS-PAI report and SRS for carrier switching.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/RS-PAI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol.

Proposal 2-5: Endorse the following TP for 38.214 for collision handling between predicted CRI/SSBRI/RSRP report and SRS on the same carrier.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only RS-PAI report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only RS-PAI report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH.

Proposal 2-7: Endorse the following TP for 38.214 to clarify that UE does not report the RS-PAI if it fails to receive the corresponding transmission occasions within the DRX active time

5.2.2.5 CSI reference resource definition <omitted text> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'rs-pai-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise.

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.3.1# Clarification on the reference time for the determination of linked inference report

Ericsson [1], Xiaomi [5] and Samsung [9] proposed to clarify the reference time for the determination of linked inference report. The first slot of the transmission occasion is proposed.

Proposal. 2.3.1

Adopt the following TP for TS 38.214 Clause 5.2.1.4.3b for CSI reporting for RS-PAI.

Reason for change: The slot of transmission occasion for monitoring is unclear for the linkage of the corresponding CSI report for inference.

Summary of change: Clarify the first slot of transmission occasion for monitoring is used for the linkage.

Consequences if not approved: gNB and UE may have different understanding on the linkage.

##### TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting <omitted texts> - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; <omitted texts>

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | It is necessary to clarify the reference time for linkage. Please share your view on the proposal. |
| Google |  | Support |
| Huawei, HiSilicon |  | OK |
| Xiaomi |  | Support |
| CATT |  | Support |
| LGE |  | Support. |
| QC |  | OK |
| Ericsson |  | ok |
| SPRD |  | ok |
| CMCC |  | Ok |
| Fujitsu |  | Support |
| Sharp |  | Support |

Issue 2.3.2# Mapping between Set A and monitoring resource set

Agreement (RAN1#120bis) For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A Agreement (RAN1#121) For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model, when the size of the set for monitoring is smaller than the size of Set A, - support the mapping of the resources in the set for monitoring to resources in Set A is configured via RRC, support - A X-bit bitmap with Y non-zero bits is configured by the RRC in CSI Report Config for monitoring, where X is the size of Set A and Y is the size of the set for monitoring - The x-th MSB of the bitmap corresponds to x-th resource in Set A - The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the set for monitoring, 1≤y≤Y

As pointed out by several companies [4,10,15], the yellow highlighted part of the agreement related to resource mapping is not fully captured. Hence, a corresponding text proposal is provided to address this issue.

Proposal. 2.3.2

Adopt the following TP for mapping between Set A and monitoring resource set.

TP for TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting ##### 5.2.1.4.3b RS-PAI Reporting <omitted texts> - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the first CSI Reporting Setting, wherein - if the number of resources in the resource set for the second CSI Reporting Setting is smaller than the number of resources in the resource set given by resourcesForSetA-r19, the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if the number of resources in the resource set for the second CSI Reporting Setting is the same as the number of resources in the resource set given by resourcesForSetA-r19, the n-th resource of the resource set for channel measurement of the second CSI Reporting Setting is mapped to the n-th resource of the resource set given by resourcesForSetA-r19; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; <omitted texts>

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | The TP can provide a complete description on the resource mapping between monitoring and inference. Please share your view on the TP for mapping between Set A and monitoring resource set. |
| Google |  | OK |
| Huawei, HiSilicon |  | OK |
| Xiaomi |  | Support |
| CATT |  | OK |
| OPPO |  | Not support. Current specification says the mapping is done via the RRC parameters, which includes both cases and leaves the mapping details as NW’s configuration. In RAN1 spec, it is not necessary to capture the configuration details. |
| Ericsson |  | Not support. Similar view as OPPO. |
| SPRD |  | OK |
| CMCC |  | Ok |
| Fujitsu |  | Support. This is essential issue and should be addressed. |
| Sharp |  | Ok |

Issue 2.3.3# Correction on the determination of Top beams in resource set for monitoring

Vivo [3] and LG [12] proposed a TP to correct the description on the determination of Top beams in resource set for monitoring.

Proposal. 2.3.3

Adopt the following TP on the Top beams in resource set for monitoring.

##### 5.2.1.4.3b RS-PAI Reporting <omitted texts> - for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall: - perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on largest L1-RSRP(s) value(s) of measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set; <omitted texts>

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | The intention of the change is to make the description to be consistent with existing specification language. Please share your view. |
| Google |  | We failed to see the necessity. The change is not aligned with the previous agreement. |
| Huawei, HiSilicon |  | Seems not needed. |
| Xiaomi |  | Support |
| CATT |  | We don’t think it’s needed. Current wording aligns with the previews agreement. |
| LGE |  | Support. This is not an optimization and to correct ambiguity of UE behavior for determining M(nrofBestBeamforMonitoring-r19) resources for performance monitoring. Usually “the best” is not a specification wording and different UE vendor can have different understanding on “the best” which brings different implementation by UE vendors for determining M resources for performance monitoring. “the best” can be interpreted as “UE’s preferred” which is not matched with the intension of Top-M actual measured monitoring RS(s) for performance monitoring. |
| QC |  | Not needed |
| OPPO |  | Support. The largest L1-RSRP in values is clearer than the “best”. Any principle specified to define the best, largest RSRP, highest SINR/SNR or what else? |
| Ericsson |  | Not needed. |
| CMCC |  | Support |
| Fujitsu |  | Seems not necessary |
| Sharp |  | Support |

Issue 2.3.4# The condition for dropping CSI report for monitoring

Similar to proposal 2.2.2, it is proposed by Google that DRX needed to be additional considered on the dropping of CSI report for monitoring. In [14], NEC propose to stop the report for monitoring when the associated CSI report for inference is stopped.

Proposal. 2.3.4

Endorse the following TP for 38.214 to clarify that UE does not report the RS-PAI if it fails to receive the corresponding transmission occasions within the DRX active time.

5.2.2.5 CSI reference resource definition <omitted text> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'rs-pai-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. <omitted text>

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Similar to proposal 2.2.2, the TP seems necessary since the same condition is captured in TS38.214 on the dropping conditions of other CSI reporting. Please share your view. |
| Google |  | Support |
| Xiaomi |  | We don’t think this one is necessary. It is for PAI report, the monitoring time window can be longer than one DRX active time. |
| CATT |  | OK |
| QC | N | Similar reasoning as previously commented for Proposal 2.2.2 |
| OPPO |  | Similar comment as for BM-Case2. Assume this condition is not added, can we rely on NW’s implementation to handle this? 1. NW configures the measurements for monitoring are within the same DRX active time? Or 2. NW configures the measurements for monitoring are across different DRX active times and there is no measurement missing. |
| Ericsson |  | Not support. Similar view as Qualcomm. |
| NEC |  | OK |
| Fujitsu |  | Support |

Issue 2.3.5# Collision handling of CSI report for monitoring and other signals/channels

In [2], it is proposed by Google that the collision handling rule for CSI report for monitoring and other signals/channels shall follows the same rule for legacy beam report.

Proposal. 2.3.5a

Endorse the following TP for 38.214 for collision handling between RS-PAI report and SRS for carrier switching.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/RS-PAI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol.

Proposal. 2.3.5b

Endorse the following TP for 38.214 for collision handling between predicted CRI/SSBRI/RSRP report and SRS on the same carrier.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only RS-PAI report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only RS-PAI report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH.

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | These two proposals are natural extension on the collision handling rule for legacy beam report to the CSI report for monitoring. Please share your view. |
| Google |  | Support |
| Huawei, HiSilicon |  | OK. Can be merged with Issue 2.2.3? |
| Xiaomi |  | Support |
| CATT |  | OK |
| LGE |  | Same view as Huawei. |
| QC |  | OK |
| Ericsson |  | OK |
| CMCC |  | Ok |
| Fujitsu |  | Support |
| Sharp |  | Ok |

Others

HW [4] proposed to preclude duplicated mapping for the calculation of RS-PAI calculation. However, it seems to be an optimization and it is not clear on the corresponding specification impact.

Nokia [13] proposed to restrict nroftransmissionOccasion-r19 to 1 in case of AP CSI report for monitoring with the associated AP CSI report for inference. However, based on the discussion in the last meeting, it seems that a number of companies have concern on this.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |

2.4 CSI processing Timeline

Qualcomm

Proposal 1: Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: It was agreed to relax timeline for AI/ML CSI reports, but the relaxed timeline (Z3+d / Z3’+d’) is applicable only to aperiodic CSI reports. At least due to inference latency for P/SP AI/ML CSI reports, relaxed timelines should also be applicable to P/SP AI/ML CSI reports.

Summary of change: Introduce a relaxed timeline for P/SP AI/ML CSI reports.

Consequences if not approved: UE will not be able to meet the legacy P/SP CSI report timelines when generating AI/ML CSI reports.

TP for TS 38.214, Clause 5.2.2.5, timeline for P/SP CSI reports <omitted texts> The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to (4+T1)⋅2µDL, such that it corresponds to a valid downlink slot, wherein T1 is non-zero if CSI-ReportConfig is configured with the higher layer parameter [RRC_name-r19], and T1 may be reported by UE capability, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to (5+T2)⋅2µDL, such that it corresponds to a valid downlink slot., wherein T2 is non-zero if reportquantity-r19 is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and different values for T2 may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report. <omitted texts>

Sharp

Proposal 4: Adopt the following TP#4 in TS 38.214 to clarify that CSI computation time for model training data collection is the same as corresponding model inference report.

Reason for change: For model training, UE shall determine the configured future time instances based on the CSI reference resource of the report. For determination of the CSI reference resource, definition of CSI computation time is needed. In current TS38.214, CSI computation time for model training report has not been specified yet.

Summary of change: Clarify that CSI computation time for model training data collection is the same as corresponding model inference report.

Consequences if not approved: UE is not able to determine the CSI reference resource of the model training report and thus not able to determine the future time instances for BM-Case 2.

TP#4 5.4 UE CSI computation time <unchanged parts are omitted> Z, Z' and µ are defined as: Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z1,Z1') of the table 5.4-1 if max{ µPDCCH, µCSI-RS, µUL} ≤ 3 and if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', 'cri-SINR', 'ssb-Index-SINR- Index ', or 'cri-SINR- Index ', or - (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', 'L1-SRS-RSRP', 'L1-CLI-RSSI' or 'ssb-Index-RSRP- Index ', where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or if the CSI report is configured with ltm-CSI-ReportConfig for L1-RSRP measurement, or if the CSI report is configured with eventType, or - (Z3+d,Z3'+d'), with (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or ‘none-bm-r19’, where d and d' are according to UE reported capability as defined in [13, TS 38.306] and different values for d and d' may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report, or <unchanged parts are omitted>

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.4.1# New definition of CSI reference resource for P/SP CSI report for inference

Qualcomm [20] proposed to revisit the definition of CSI reference resource for P/SP CSI report for inference. This will help to relax the processing timeline for AI/ML based CSI report.

Proposal. 2.4.1

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: It was agreed to relax timeline for AI/ML CSI reports, but the relaxed timeline (Z3+d / Z3’+d’) is applicable only to aperiodic CSI reports. At least due to inference latency for P/SP AI/ML CSI reports, relaxed timelines should also be applicable to P/SP AI/ML CSI reports.

Summary of change: Introduce a relaxed timeline for P/SP AI/ML CSI reports.

Consequences if not approved: UE will not be able to meet the legacy P/SP CSI report timelines when generating AI/ML CSI reports.

TP for TS 38.214, Clause 5.2.2.5, timeline for P/SP CSI reports <omitted texts> The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, - where +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and and are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211] - where for periodic and semi-persistent CSI reporting - if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or - if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to (5+T2)⋅2µDL, such that it corresponds to a valid downlink slot., wherein T2 is non-zero if reportquantity-r19 is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and different values for T2 may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report. <omitted texts>

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Only the part of TP related to beam management is captured. Please share your view on the TP. |
| Google |  | Support |
| Huawei, HiSilicon |  | The SP CSI report timeline is not revisited for R18 CSI prediction which also needs prediction. In our view, 4 or 5 ms is already loose to accommodate the inference delay. |
| QC | Y | Support. We believe this enhancement is essential in order for UE to support P/SP CSI reports. The relaxed timeline agreed in Rel-19 is only applicable to AP CSI reports with the updated vales of Z3 and Z3'. The legacy timeline of 5ms for P/SP does not encapsulate the inference latency which could be higher for both spatial and temporal beam prediction. In particular, if we consider temporal beam prediction, the required timeline may be much higher, given UE may predict for up to 8 future instances, and up to 4 beams per instance. |
| Ericsson |  | We are not sure about the need for the proposal. Need further discussion on if 4 or 5ms isn’t enough. Plus, we think optimizations should be avoided in this stage since we are in maintenance phase. |
| Fujitsu |  | Open for discussion |

Others

Sharp [20] proposed to extend d and d’ to the case of data collection. However, it is not clear why the case of data collection requires timeline check since there is no CSI reporting.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| Sharp | We understand there is no CSI report for the case of data collection, while as we explicitly explained, with the introduction of BM-Case 2 parameters into model training, we have to determine the future time instance(s), which means definition of CSI reference resource is needed for such case, shown as follows: 5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19' or 'none-bm-r19', if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)). If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. Regarding determination of CSI reference resource, Z’ is used which means timeline is needed for the case of data collection, hereby we have the following specs: 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, […] - where for aperiodic CSI reporting and for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeA’, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4. Therefore, the case of data collection requires timeline as well, although there is no CSI report. |

2.5 Associated ID

Xiaomi

Proposal 2: Support the following TP on associated ID to 38.214.

Reason for change: The UE assumption with same associated ID configured to be associated with different resource sets is unclear.

Summary of change: Clarify that the UE may assume similar properties of downlink spatial domain transmission filter for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets with same associated ID.

Consequences if not approved: UE may not know the meaning of the similar properties.

< Start of change request >

##### 5.2.1.4.1	Resource Setting configuration

<Unchanged part omitted>

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE may be configured with one or two Associated ID(s) in CSI-ReportConfig:

-	if only associatedIDforSetA-r19 is configured, it is associated with the resource sets of the second Resource Setting, and the UE expects that all CSI-RS resources or SS/PBCH block resources in the resource set of the first Resource Setting are among the CSI-RS resources and/or SS/PBCH block resources in the resource set of the second Resource Setting.

-	if associatedIDforSetA-r19 and associatedIDforSetB-r19 are configured, they are associated with the resource set of the second Resource Setting and of the first Resource Setting, respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 is set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', if the same Associated ID is configured to be associated with different resource sets, the UE may assume similar properties of downlink spatial domain transmission filter for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released.

<End of change request>

< Start of change request >

##### 5.2.1.4.1	Resource Setting configuration

<Unchanged part omitted>

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE may be configured with one or two Associated ID(s) in CSI-ReportConfig:

-	if only associatedIDforSetA-r19 is configured, it is associated with the resource sets of the second Resource Setting, and the UE expects that all CSI-RS resources or SS/PBCH block resources in the resource set of the first Resource Setting are among the CSI-RS resources and/or SS/PBCH block resources in the resource set of the second Resource Setting.

-	if associatedIDforSetA-r19 and associatedIDforSetB-r19 are configured, they are associated with the resource set of the second Resource Setting and of the first Resource Setting, respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 is set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', if the same Associated ID is configured to be associated with different resource sets, the UE may assume similar properties of downlink spatial domain transmission filter for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released.

<End of change request>

CMCC

Proposal 2: The associated ID can be optionally configured for data collection and inference.

Proposal 3: When the associated ID is not configured for data collection and not configured for inference, the UE can report it is either applicable or not applicable.

Proposal 4: When the associated ID is not configured for data collection and not configured for inference, the UE can report it is applicable and the property for Set A/Set B in ascending order of the entries of the corresponding resource set is similar for training and inference.

Panasonic

Proposal 9: Support to apply concept of “associated IDs” for multiple cells for ensuring consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2.

Proposal 10: Support to determine “associated IDs” within a NW operator (or an MNO) to preserve proprietary information.

Proposal 11: Regarding assumption of UE on associated ID,

- If 2 resource sets A in training and inference are configured with a same associated ID, UE might assume the order of 2 resource sets A are identical as an ascending order of the entries of these 2 resource sets.

- If 2 resource sets B in training and inference are configured with a same associated ID, UE might assume the order of 2 resource sets B are identical as an ascending order of the entries of these 2 resource sets.

LG

Proposal #1: Define similar properties of a DL Tx beam set (for Set A and/or Set B) that at least the same downlink spatial domain transmission filters are maintained for each beam in different transmission instances.

Proposal #2: Endorse the following TP for TS38.214.

| Reason for change | In current specification, it is described that if the same Associated ID is configured to be associated with different resource sets, UE may assume similar properties for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets. However, “similar properties” is still not clear for both UE and NW side. Different NW vendors may have different understanding on the definition of “similarity”. Furthermore, there are so many physical properties such as beam angle, beam width, power of main lobe, and power/angle of side lobe, etc from DL Tx beam perspective. |
| --- | --- |
| Summary of change | Change “similar properties” to “the same properties of downlink spatial domain transmission filters” for clear UE assumption. |
| Consequences if not approved | Different NW vendors may have different understanding on the definition of “similarity”. So, consistency between training phase and inference phase may not be maintained. |

<S5.2.1.4.1> ============================= unchanged parts are omitted ============================ For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE may be configured with one or two Associated ID(s) in CSI-ReportConfig: - if only associatedIDforSetA-r19 is configured, it is associated with the resource sets of the second Resource Setting, and the UE expects that all CSI-RS resources or SS/PBCH block resources in the resource set of the first Resource Setting are among the CSI-RS resources and/or SS/PBCH block resources in the resource set of the second Resource Setting. - if associatedIDforSetA-r19 and associatedIDforSetB-r19 are configured, they are associated with the resource set of the second Resource Setting and of the first Resource Setting, respectively. For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 is set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', if the same Associated ID is configured to be associated with different resource sets, the UE may assume similarthe same properties of downlink spatial domain transmission filters for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released. ============================= unchanged parts are omitted ============================

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.5.1# The clarification of “similar properties”

Xiaomi [5] and LG [14] proposed to further clarified the description of “similar properties” in the current TS38.214. Also, CMCC and Panasonic provide inputs in the clarification of assumption of UE on associated ID.

Proposal. 2.5.1

For UE assumption of associated ID,

- the property for a resource set of Set A/Set B is interpreted with the ascending order of the entries of the corresponding resource set.

- the property is understood as the property of downlink spatial domain transmission filter

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | This proposal is an update of RAN1#122 proposal with the addition of second bullet based on company inputs. Please share your view on the proposal. |
| Google |  | Support. This is aligned with previous RAN1 agreement |
| Huawei, HiSilicon |  | OK with the first bullet. Not support the second bullet – the property includes more than the spatial domain filter, everything (scenario, scatters, etc.) that may impact the received beam strength may be counted into the property. |
| Xiaomi |  | Support |
| CATT |  | The similar properties mean the similar NW additional condition including scenarios, antenna downtilt, etc. And the ‘same’ seams more restrictive than the ‘same’. We prefer to keep the current spec wording which aligns with the agreement. |
| LGE |  | Support in principle and prefer to revise as below: Proposal. 2.5.1 For UE assumption of associated ID, - the same property for a resource set of Set A/Set B is interpreted with the ascending order of the entries of the corresponding resource set. - the same property is understood as the property of downlink spatial domain transmission filter the wording “Similar property” is too vague so that different gNB vendor can have different understanding on “similar property”, which brings that the consistency between training and inference cannot be ensured. So “same” is included in our revision. Regarding Huawei’s comment, it seems that everything (scenario, scatters, etc.) cannot be always ensured by gNB which is not a NW additional condition. While, the current two bullet in the proposal can be surely ensured by gNB. |
| QC |  | Support with the following updates: The indexing can follow the same principle as the ones we have agreed for monitoring resource set and Set A, once the size of Set A is equal to the monitoring resource set (copied below). Proposal. 2.5.1 For UE assumption of associated ID, - the same property for a resource set of Set A/Set B is interpreted with the ascending order same indexing of the entries of the corresponding resource set (the n-th resource of the resource set of the first and second CSI Reporting Setting is mapped to the n-th resource of the resource set given by the first and second CSI Reporting Setting); - the same property is understood as the property of downlink spatial domain transmission filter …, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released. |
| OPPO |  | If the intention is to make it clear on Associated ID, we suggest telling the whole story as For UE assumption of associated ID across training and inference phases - the property for a resource set of Set A/Set B is interpreted with the ascending order of the entries of the corresponding resource set. - the property is understood as the property of downlink spatial domain transmission filter |
| Ericsson |  | This was discussed at length during normative phase. We prefer to keep the current spec without any changes. |
| CMCC |  | OK with the first bullet. |
| Fujitsu |  | Ok with the revision from OPPO. |

Others

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |

2.6 UE-side data collection

Lenovo

Proposal 2: Adopt the following CR for TS38.214V19.0.0

5.2.1.4.2 Report quantity configurations A UE may be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'cri-RI-LI-PMI-CQI', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index', 'tdcp', 'cli-SRS-RSRP', 'cli-RSSI', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', 'rs-pai-r19', 'csi-pai-r19', 'none-csi-r19', 'none-bm-r19', 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P'. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'none-csi-r19' or 'none-bm-r19', then the UE shall not report any quantity for the CSI-ReportConfig. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19', the physical layer in the UE measures the L1-RSRPs of the resources in the first and the second Resource Settings and provides the measurement results to the higher layers.

Google

Proposal 2-8: Support to configure the data collection window

CSI-RS for data collection is transmitted in the data collection window

CSI-RS for data collection is not transmitted outside the data collection window

Sharp

Proposal 3: Adopt the following TP#3 in TS 38.214 to clarify that only Set A is considered for determination of CPU occupation time for the model training CSI report.

Reason for change: For model training, both Set A and Set B are configured for channel measurement. If Set B is configured as a subset of Set A (i.e. only associatedIDforSetA-r19 is configured), CPU occupation corresponding to each transmission occasion of both Set A and Set B would be summed and it leads to over-counting for CPU occupation, i.e. CPU occupation corresponding to each transmission occasion of Set B is counted twice.

Summary of change: Clarify that only Set A is considered for determination of CPU occupation time for the model training CSI report.

Consequences if not approved: CPU occupation corresponding to each transmission occasion of Set B is counted twice.

TP#3 5.2.1.6 CSI processing criteria <unchanged parts are omitted> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-bm-r19' and if only associatedIDforSetA-r19 is configured, the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement of the second Resource Setting, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion of the second Resource Setting. <unchanged parts are omitted>

Based on the summary above, there is no proposal formulated at this stage.

Lenovo [16] proposed a TP for physical layer to pass L1 measurement results of UE-side data collection to higher layer. However, the associated description is typically described in 38.321 which can be discussed in RAN2.

Google [17] proposed to introduce a data collection window for energy saving purpose. However, this seems to be an optimization and the corresponding specification impact is unclear.

Sharp [21] proposed to further clarified the condition of CPU occupation for UE-side data collection. However, the suggested changes is unnecessary since the current wording in TS38.214 already provides enough clarity.

| Company | Comments |
| --- | --- |
| FL | Please suggest any other issue that need to be discussed, if any. |
| Sharp | We don’t think current specs is already clear. Briefly speaking, for channel measurement of data collection, both Set A and Set B are included. When we talk about each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement, it is quite obvious that Set A and Set B transmission occasions are all included, which would lead to interpretation when determining CPU occupation, Set A is counted and Set B is also counted. Then, “double count” occurs and obviously, this is redundant since Set B can even be a sub-set of Set A. Therefore, we propose to only count Set A for CPU occupation since Set A is the super-set. |

- Remaining issues for NW-side AI/ML model

3.1 CSI report for model inference

ZTE

Text Proposal 2: To adopt the following changes in section 5.2.1.4.2, TS 38.214.

Reason for change:

According to the following agreement made in RAN1#117, the UE shall report the Top-M largest RSRP to facilitate NW-side model inference. The critical part is that reported M beams shall be associated with the largest M RSRPs among all measured RSRPs. This is aligned with all our previous simulation settings in Rel-18 study phase and ensures an unified UE behaviour. Otherwise, if different UEs adopt different strategies to obtain the reported M beams, it may cause additional training efforts and unpredictable performance degradation at the NW side.

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

Summary of change:

For NW-sided model, for inference report, capture the agreement made in RAN1#117 that the UE shall report beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set.

Consequence if not approved:

Agreement made in RAN1#117 is not captured. Additionally, unaligned UE behaviour on beam reporting for inference may cause additional training efforts and unpredictable performance degradation at the NW side.

<Unchanged part is omitted> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS or nrofReportedRS-r19 (higher layer configured) different CRI or SSBRI for each report setting where the nrofReportedRS-r19 CRI or SSBRI are associated with the nrofReportedRS-r19 largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set, or single CRI or SSBRI for each report setting when nrofReportedRS-r19 is equal to the size of the resource set for channel measurement where single CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set <Unchanged part is omitted>

CMCC

Proposal 2: Adopt the following Text Proposal in Clause 5.2.1.4.2 of TS 38.214 for inference report quantity:

Summary of change:

Specify when UE reports in a single report nrofReportedRS-r19 CRI or SSBRI for each report setting, nrofReportedRS-r19 CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set.

Consequence if not approved:

UE determination on nrofReportedRS-r19 CRI or SSBRI is up to UE implementation, which causes the misalignment between agreement and spec and the degradation on inference performance at NW side.

--------------------------- Start of Text Proposal for TS 38.214 -----------------------------

5.2.1.4.2	Report quantity configurations

< Unchanged parts are omitted >

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS or nrofReportedRS-r19 (higher layer configured) different CRI or SSBRI for each report setting, or nrofReportedRS-r19 (higher layer configured) different CRI or SSBRI for each report setting when nrofReportedRS-r19 is smaller than the size of the resource set for channel measurement where nrofReportedRS-r19 CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set, or single CRI or SSBRI for each report setting when nrofReportedRS-r19 is equal to the size of the resource set for channel measurement where single CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

Panasonic

Proposal 2: For NW-sided model inference in BM-Case2, support UE to report measurement results of K latest consecutive transmission occasions for each of the RS resources in Set B before a CSI report configuration for the measurement, where K is indicated by a UE capability

Proposal 3: Group-based beam reporting can be enhanced to support to report more than 4 beams in one report for NW-sided model.

Proposal 4: For NW-sided model, for inference report, for BM-Case2, support content per measurement time instance in a beam report in L1 signaling as follows:

  - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

  - If M<the size of measurement resource set, the beam information is CRI/SSBRI

  - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRPof a measurement resource set

  - Note: The purpose, such as above “for NW-sided model, for inference” will not be specified in RAN1 specifications

Sharp

Proposal 1: Adopt the following TP#2 in TS 38.214 to clarify that the UE shall derive the CSI parameters other than CRI conditioned on the configured CSI-RS resources when the value of M configured by nrofReportedRS is equal to the size of the resource set for channel measurement.

Reason for change: For NW side model for inference report, when the value of M configured by nrofReportedRs is equal to the size of the measurement resource set, UE only reports one CRI and it is incorrect to specify that UE derives the CSI parameters other than CRI, i.e. (M-1) differential L1-RSRPs, conditioned on the reported CRI.

Summary of change: Further clarify that UE shall derive the CSI parameters other than CRI conditioned on the configured CSI-RS resources when the value of M configured by nrofReportedRS is equal to the size of the resource set for channel measurement.

Consequences if not approved: UE is unable to derive the (M-1) differential L1-RSRPs.

TP#2 5.2.1.4.2 Report quantity configurations <unchanged parts are omitted> Except for a CSI-ReportConfig configured with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18', if the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-RSRP-Index', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RI-LI-PMI-CQI', 'cri-SINR', or 'cri-SINR- Index ', and resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the nrofReportedRS-r19 CSI-RS resources when nrofReportedRS-r19 is configured and equal to the size of the resource set for channel measurement, otherwise the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured for CSI-ReportConfig with reportQuantity set to 'cri-SINR' or 'cri-SINR- Index ') for interference measurement. If CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports. <unchanged parts are omitted>

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 3.1.1# The clarification of Top M beams in CSI report for inference

CATT [3] and CMCC [18] proposed to clarify the top M beam in CSI report for inference.

Proposal. 3.1.1

Adopt the following Text Proposal in Clause 5.2.1.4.2 of TS 38.214 for inference report quantity.

Summary of change: Specify when UE reports in a single report nrofReportedRS-r19 CRI or SSBRI for each report setting, nrofReportedRS-r19 CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set.

Consequence if not approved: UE determination on nrofReportedRS-r19 CRI or SSBRI is up to UE implementation, which causes the misalignment between agreement and spec and the degradation on inference performance at NW side.

5.2.1.4.2 Report quantity configurations < Unchanged parts are omitted > If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS or nrofReportedRS-r19 (higher layer configured) different CRI or SSBRI for each report setting, or nrofReportedRS-r19 (higher layer configured) different CRI or SSBRI for each report setting when nrofReportedRS-r19 is smaller than the size of the resource set for channel measurement or single CRI or SSBRI for each report setting when nrofReportedRS-r19 is equal to the size of the resource set for channel measurement where single CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set < Unchanged parts are omitted >

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | The proposal is based on the outcome of second round discussion in RAN1#122. Please share your view. |
| Google |  | We failed to see the necessity, and we are open for the discussion for the motivation. |
| Huawei, HiSilicon |  | OK. It makes the spec clear to separate the if/else conditions and UE behaviors. |
| Xiaomi |  | Support |
| CATT |  | To align with the previews agreement, we‘d like to clarify the nrofReportedRS-r19 CRI or SSBRI is the largest nrofReportedRS-r19 L1-RSRPs of the resource set as below TP. --------------------------------------Start of TP for TS38.214--------------------------------------------- 5.2.1.4.2 Report quantity configurations <Unrelated part omitted> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index', - if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS or nrofReportedRS-r19 (higher layer configured) different CRI or SSBRI for each report setting where the nrofReportedRS-r19 CRI or SSBRI are associated with the largest nrofReportedRS-r19 L1-RSRPs of the measured of the measured CSI-RS resources or SS/PBCH block resources of the resource set, or single CRI or SSBRI for each report setting when nrofReportedRS-r19 is equal to the size of the resource set for channel measurement where single CRI or SSBRI is associated with the largest L1-RSRP of the measured CSI-RS resources or SS/PBCH block resources of the resource set. <Unrelated part omitted> -----------------------------------------------End of TP for TS38.214------------------------------------- |
| OPPO |  | Not needed in our reading, current spec has clearly specified the condition. |
| Ericsson |  | Not support. This doesn’t seem like an essential correction. |
| SPRD |  | Support |
| CMCC |  | Ok |
| Fujitsu |  | Seems not necessary |

Others

Sharp [21] proposed a TP for the clarification of CRI in case of the number of reported RS is the same as the size of the resource set for channel measurement. However, the clarification seems unnecessary.

| Company | Comments |
| --- | --- |
| FL | Please suggest any other issue that need to be discussed, if any. |
| Sharp | We don’t think the mentioned issue lies on whether further clarification is needed. It is indeed an issue on whether specs text is correct statement or not. Current specs somehow goes against RAN1 agreements and specs in other places. When M is configured equal to the size of resource set for channel measurement, only one CRI is reported, unlike legacy report and other inference report wherein UE reports all the CRIs. |

3.2 NW-side data collection via higher layer signaling

Huawei

Proposal 2: For NW side data collection via higher layer reporting, regarding the L1-RSRP and/or Beam ID for the logging contents of the RAN2 agreement, support the following two options:

- Opt 1: L1-RSRPs of all measured beams (Set A and Set B).

- Opt 2: L1-RSRPs of all measured beams in Set B, and Top-K beams based on the measured beams (beam index and L1-RSRP) in Set A.

- Note: Set A and Set B are only used for discussion purpose and is not known by UE as agreed by RAN2.

Panasonic

Proposal 1: For content of data collection for NW-sided model training via higher layer signaling, for both BM-Case 1 and BM-Case2, for each per instance, support L1-RSRPs measured based on one resource set (for Set A and Set B) configured to UE

- FFS: Details on reporting configuration

The associated discussion is moved to Section 5.

3.3 Beam indication

Panasonic

Proposal 5: For NW-sided model, support to extend Rel. 17 TCI state activation signaling methods to activate TCI states of K predicted beams for N future time instances in BM-Case 2.

- The TCI states of K predicted beams for N future time instances are included in a separate set of TCI states, compared to that of legacy BM.

Ofinno

Proposal 1: Endorse TP#1 in Annex A for P2 sweeping based on Top-K beams.

5.2.1.5	Triggering/activation of CSI Reports and CSI-RS

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot  where  is the SCS configuration for the PUCCH and is the subcarrier spacing configuration for  with a value of 0 for frequency range 1 and for FR2-NTN, and  is provided by K-Mac or  if K-Mac is not provided..

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in clause 5.1.5, through higher layer signaling of qcl-info or qcl-info2 which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. The UE is expected to be configured with k aperiodic CSI-RS resources in the CSI-RS resource set associated with a CSI triggering state without associated qcl-info and the UE assumes the TCI-State follows the most recently reported top k predicted TCI states. If a remaining aperiodic CSI-RS resource in the CSI-RS resource is configured with one of the TCI state for the top k predicted TCI states, the UE assumes the aperiodic CSI-RS resource is not transmitted. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

There is no enhancement agreed in previous meetings and hence it is not suggested to open the discussion in CR stage which is aiming for essential correction rather than optimization.

- UE Capability related issues

ZTE

Text Proposal 2: To adopt the following changes in section 5.2.1.4.1, TS 38.214.

Summary of change: For both NW-side model and UE-side model, extend the maximum number of NZP CSI-RS resources per resource set to 256.

Consequence if not approved: The base station has to configure multiple resource sets for supporting Set A larger than 64, and the UE has to integrate measurement results from the multiple resource sets, leading to additional configuration overhead and UE power consumption.

##### 5.2.1.4.1 Resource Setting configuration <Unchanged part is omitted> A UE is not expected to be configured with more than one CSI-RS resource in resource set for channel measurement for a CSI-ReportConfig with the higher layer parameter codebookType set to 'typeII', 'typeII-PortSelection', 'typeII-r16', 'typeII-PortSelection-r16', or 'typeII-PortSelection-r17'. A UE is not expected to be configured with more than 64 256 NZP CSI-RS resources and/or SS/PBCH block resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'cri-RI-CQI', 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', 'rs-pai-r19', or 'none-bm-r19'. If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources. <Unchanged part is omitted>

vivo

Proposal 2: For inference, for UE-side model, Set A or CSI-RS resources in Set A should be indicated as virtual Set or virtual resource,

- CSI-RS Resources in Set A shall not be linked to another CSI resource setting for channel measurement

- Set A shall not be considered for PDSCH rate matching

- Set A is not counted as active resource

- Configurations of Set A in the inference CSI report should be ignored, at least including CSI-RS-resourceMapping and resource type.

Huawei

Proposal 1: For data collection for both NW-side model and UE-side model, support to extend the maxNrofNZP-CSI-RS-ResourcesPerSet (i.e., Maximum number of NZP CSI-RS resources per resource set) to 256, subject to UE capability.

- Note: UE support of maxNumberCSI-RS-Resource (max number of configured CSI-RS resources) needs to be enhanced to 256 accordingly.

- Note: this corresponds to NW side data collection capability and does not impact L1 CSI report capability.

Panasonic

Proposal 8: For both NW-side model and UE-side model, support a maximum number of NZP CSI-RS resources per resource set to 256 per CC, subject to UE capability.

Nokia

Summary of change: Increase the maximum number of NZP CSI-RS resources per resource set to 128 per CC.

Consequence if not approved: NW may not be able to configure UE for UE-sided data collection in all potentially supported configurations. One example could be Set B not a subset of Set A with Set A having value 64 for maximum number of resources and Set B having any value for maximum number of resources (e.g. 8, 16, 32, 64).

Proposal 4: To increase the maximum number of NZP CSI-RS resources per resource set per CC, endorse the following text proposal to 38.214 Clause 5.2.1.4.1.

##### 5.2.1.4.1 Resource Setting configuration <Unchanged part is omitted> A UE is not expected to be configured with more than one CSI-RS resource in resource set for channel measurement for a CSI-ReportConfig with the higher layer parameter codebookType set to 'typeII', 'typeII-PortSelection', 'typeII-r16', 'typeII-PortSelection-r16', or 'typeII-PortSelection-r17'. A UE is not expected to be configured with more than 64 128 NZP CSI-RS resources and/or SS/PBCH block resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'cri-RI-CQI', 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', 'rs-pai-r19', or 'none-bm-r19'. If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources. <Unchanged part is omitted>

Sharp

Proposal 5: Adopt the following TP#5 in TS 38.214 to clarify that only Set A is considered for determination of CPU occupation time for the model training CSI report.

Reason for change: For model inference report, UE is not expected to measure the CSI-RS resources configured in Set A and the CSI-RS resources configured in Set A do not need to occupy the memory of the UE for processing CSI-RS resources.

Summary of change: Clarify that only Set A is considered for determination of CPU occupation time for the model training CSI report.

Consequences if not approved: CSI-RS resources configured in Set A occupy the memory for processing CSI-RS resources while UE does not expect to measure these resources.

TP#5 5.2.1.6 CSI processing criteria <unchanged parts are omitted> If a CSI-RS resource is referred N times by one or more CSI Reporting Settings not configured with higher layer parameter csi-ReportSubConfigToAddModList, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times. For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and Resource Pairs, if a CSI-RS resource is referred times by one of the CSI-RS resources, where is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted times. For a CSI-ReportConfig containing a list of L sub-configuration(s) provided by higher layer parameter csi-ReportSubConfigToAddModList, if a CSI-RS resource is referred by M sub-configurations among N triggered sub-configurations for CSI reporting for aperiodic CSI-RS resource, or L configured sub-configurations for CSI reporting for periodic or semi-persistent CSI-RS resource, the CSI-RS resource is counted M times and the CSI-RS ports within the CSI-RS resource are counted , where P is the number of ports configured by nrofPorts and is the number of CSI-RS ports in s-th sub-configuration from M sub-configurations derived from the corresponding antenna port subset indicator portSubsetIndicator according to clause 5.2.1.4.2 if configured, otherwise . For a CSI-ReportConfig with reportQuantity set to 'p-cri-r19', or 'p-cri-RSRP-r19', only CSI-RS resources of the first Resource Setting are referred and counted. <unchanged parts are omitted>

The associated discussion is moved to section 5.

Others

vivo [3] proposed to clarify the counting of resources in Set A. This can be further clarified in UE capability session since it does not have specification impact on TS38.214.

| Company | Comments |
| --- | --- |
| FL | Please suggest any other issue that need to be discussed, if any. |

- RAN2 LS on NW side data collection

R1-2506722 - The resource configuration for logged NW side data collection is provided with CSI-ResourceConfig in CSI-MeasConfig - The resource configuration does not have separate resources for Set A and Set B - RAN2 assumes there is no separate CSI resource capability is introduced specifically for logged NW side data collection, that is, the legacy CSI resource capability (e.g. beamManagementSSB-CSI-RS) would be reused. - The list of logging configurations is provided as a new list configurations within CSI-MeasConfig - the data logging procedure based on receiving measurement quantities from lower layer is captured in RRC spec. - Logging periodicity, which is different from CSI resource periodicity, is configurable for NW side data collection. - The L3 event (i.e. Event A1 or Event A2) configuration is contained in the logging configuration to support the event based data logging. - For event based data logging, UE shall perform and log the L1 measurement if the entering condition of configured L3 event is met; UE should not perform and log the L1 measurement if the leaving condition of configured L3 event is met. - From RAN2 point of view, it is sufficient to collect the L1-RSRP and/or Beam ID for the logging contents. - Only periodic CSI resources are used for NW side data collection.

The following issues are related to the LS send from RAN2 (R1-2506722).

Issue 5.1# Data collection via MDT

For MDT based data collection, several companies [4,12] proposed to define the corresponding content of measurement results in RAN1. Also, in [24, 25, 27, 28, 29], the details of reporting content details on data collection via MDT are discussed and suggested to be part of LS reply to RAN2.

Proposal. 5.1

For BM-Case1 and BM-Case2, for NW-sided data collection (for training) via higher layer signaling consider following,

- RSRPs measured at the UE based on one resource set or two sets (for Set A and Set B) configured to UE

- Support following as data content

  - Opt 1: L1-RSRPs of all measured beams (Set A and Set B)

  - Opt 2: L1-RSRPs of all measured beams in Set B, and Top-K beams based on the measured beams (beam index and L1-RSRP) in Set A.

- Note: Set A and Set B are only used for discussion purpose and does not necessarily known by UE.

- Note: “for NW-sided data collection (for training) via higher layer signaling” is only for discission purpose.

- Send LS reply to RAN2 with the above information

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Please share your view on the proposal based on the discussion in RAN1#122. If agreeable, this can be part of LS reply to RAN2. |
| Google |  | For Opt2, we would like to understand whether K is fixed or configured by the network. It seems Opt2 should be sufficient, since UE is not able to measure all beams accurately. |
| Huawei, HiSilicon |  | Seems not needed – RAN2 already agreed to adopt one resource set for one CSI-LoggedMeasurementConfig-r19 38.331 RAN2 also agreed that only P-CSI resources are supported for NW side data collection. So, the mapping between Set A and Set B seems clear to NW. From current 331, UE reports the beam ID and L1-RSRP to all the CSI-RS/SSB resources, which corresponds to Option 1. We are open to discuss if there is any problem with the current RAN2 agreement/331 spec. |
| Xiaomi |  | According to RAN2’s agreement ‘The resource configuration does not have separate resources for Set A and Set B’, we don’t think Option 2 can work. |
| CATT |  | OK with the proposal. |
| LGE |  | We don’t think two separate options needed for single UE behavior. Support only Option 1, which is aligned with the RAN2’s current understanding. In short, it is not needed. 1. The resource configuration for logged NW side data collection is provided with CSI-ResourceConfig in CSI-MeasConfig - The resource configuration does not have separate resources for Set A and Set B … 5. From RAN2 point of view, it is sufficient to collect the L1-RSRP and/or Beam ID for the logging contents. |
| OPPO |  | Support the proposal. It should be RAN1’s liability to decide the content for MDT-based data collection. Notice that reporting all RSRP and CRI/SSBRI are somehow reductant, only Top-K (up to 4) beam indexes and RSRPs would be sufficient to train a model for beam prediction. |
| Ericsson |  | Not needed. RAN2 agreements and what is captured in 38.331 is sufficient. |
| Fujitsu |  | Regarding the options of data content, Opt 1 is preferred. |

Issue 5.2# Max number of resources within a resource set

Several companies [2,3,4,12,13] proposed to extend the maximum number of NZP CSI-RS resources per resource set to a number larger than 64. Also, in [24, 25, 31], the same issue is discussed and suggested to be part of LS reply to RAN2.

Proposal. 5.2

For NW-side model and UE-side model, for data collection

- support the maximum number of NZP CSI-RS resources per resource set to 256 per CC, subject to UE capability.

  - The number of maximum number of NZP CSI-RS resources per resource set across all CCs are kept the same as the one for non-AI.

- Note: This does not impact on non-AI CSI capability.

- Send LS reply to RAN2 with the above information

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Please share your view on the proposal based on the discussion in RAN1#122. If agreeable, this can be part of LS reply to RAN2. |
| Google |  | We think a first issue is how many data collection report configurations can be configured. If the number can be greater than 1, it seems unnecessary to extend the number of resources. |
| Huawei, HiSilicon |  | Support the direction. But for the note, since RAN2 already created a dedicated IE CSI-LoggedMeasurementConfig-r19 (not CSI-reportConfig) for NW side data collection, we can use that dedicated IE to associate the UE capability. For UE side data collection, we can use CSI-ReportConfig with reportQuantity-r19 set to none-BM-r19 For NW-side model and UE-side model, for data collection - support the maximum number of NZP CSI-RS resources per resource set to 256 per CC, subject to UE capability. - The number of maximum number of NZP CSI-RS resources per resource set across all CCs are kept the same as the one for non-AI. - Note: This capability is only applicable to the data collection based on the configuration of CSI-LoggedMeasurementConfig-r19 or CSI-ReportConfig with reportQuantity-r19 set to none-BM-r19does not impact on non-AI CSI capability. - Send LS reply to RAN2 with the above information |
| CATT |  | OK |
| QC | N | This has been discussed before, but there has not been strong motivation for this direction. If the motivation is the large size associated with Set A, alternative solution is to TDM the transmission of Set A. |
| OPPO |  | Need more discussion. For data collection, should we go to as large as 256 resources (i.e. beams) per resource set? With legacy configuration, would that be possible as Google mentioned to configure multiple resource sets for data collection within a CSI-ResourceConfig? |
| Fujitsu |  | Generally fine with the revision from Huawei. |

Others

Qualcomm [30] proposed to discuss assumptions on CPU occupation for NW-side data collection, including the number of occupied CPUs. However, it seems that the issue is RAN1 only and not relevant to RAN2.

| Company | Comments |
| --- | --- |
| FL | Please suggest any other issue that need to be discussed, if any. |
| Google | We think the CPU should be discussed for NW-side data collection. |
| LGE | We think that this issue is valid but agree with FL that this issue is not related with RAN2 reply LS. Prefer to discuss this issue under maintenance on AI/ML for BM as a separate topic. |

- RAN2 LS on periodic CSI

vivo

Proposal 1: For inference, for UE-side model, UE performs inference of periodic CSI and report only after sending RRCReconfigurationComplete with the corresponding CSI-ReportConfig setting to “applicable”.

OPPO

Proposal 1: For NW-side data collection, it is RAN1’s responsibility to decide the reporting contents for MDT signaling.

NEC

If the applicability status of the corresponding CSI-ReportConfig changes from applicable to inapplicable, UE transmits a CSI report which contains an invalid value. Adopt the following TP to TS 38.214 V19.1.0

- Reason for change: For a P/SP CSI report, when the corresponding CSI-ReportConfig changes from applicable to inapplicable, and the inapplicable CSI-ReportConfig is not released, UE may be unable to perform model inference.

- Summary of change: If prediction is not performed by the UE, UE shall transmit a CSI report which contains an invalid value.

- Consequences if not approved: UE report is not clear after the functionality becomes inapplicable.

5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <Unchanged part omitted> For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19' or 'none-bm-r19', if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)). If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19' or 'none-bm-r19', after applicability reporting of the changes of applicability status of the CSI-ReportConfig from applicable to inapplicable, the UE shall transmit a CSI report which contains an invalid value. <Unchanged part omitted>

LG

Proposal #3: Endorse the following TP for TS38.214.

| Reason for change | For inference report in BM-case1/2, if the configured CSI-ReportConfig is periodic CSI report, UE considers it as un-activated (i.e., UE does not need to report anything with the CSI-ReportConfig) until the UE reports it as ‘applicable’ by RRCReconfigurationComplete and/or UAI reporting via OtherConfig. However, it is not clearly described in the current specification. |
| --- | --- |
| Summary of change | Add relevant descriptions to clarify UE behavior if UE is configured with periodic CSI-ReportConfig for inference. |
| Consequences if not approved | UE may report periodic CSI-ReportConfig for inference, even though it is not applicable from the UE side. |

<S5.2.1.4.2> ============================= unchanged parts are omitted ============================ If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', the following applies: - when periodic Reporting Setting is configured for the CSI-ReportConfig, the UE expects that the CSI-ReportConfig is activated only if applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete and/or UAI reporting via OtherConfig. If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and with nrofreportedpredictedrs-r19 and/or nroftimeinstance-r19, the following applies: - the UE is not required to update measurements for more than 64 CSI-RS or SSB resources given by resourcesForChannelMeasurement,[ and is not expected to predict for more than 64 CSI-RS or SSB resources given by resourcesForSetA-r19], - the UE shall report one of the following in a single report: - nrofreportedpredictedrs-r19 different CRIs or SSBRIs of the second Resource Setting, if nroftimeinstance-r19 is not configured and reportQuantity-r19 is set to 'p-cri-r19' or 'p-ssb-index-r19', - nrofreportedpredictedrs-r19 different CRIs or SSBRIs of the second Resource Setting, with corresponding predicted L1-RSRP(s), if nroftimeinstance-r19 is not configured and reportQuantity-r19 set to 'p-cri-RSRP-r19' or 'p-ssb-index-RSRP-r19', - nrofreportedpredictedrs-r19 different CRIs or SSBRIs of the second Resource Setting for each of nroftimeinstance-r19 time instance(s), if nroftimeinstance-r19 is configured and reportQuantity-r19 set to 'p-cri-r19' or 'p-ssb-index-r19', - nrofreportedpredictedrs-r19 different CRIs or SSBRIs of the second Resource Setting for each of nroftimeinstance-r19 time instance(s), with corresponding predicted L1-RSRP(s), if nroftimeinstance-r19 is configured and reportQuantity-r19 set to 'p-cri-RSRP-r19' or 'p-ssb-index-RSRP-r19'. If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19'. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to reportQuantity-r19 set to 'rs-pai-r19'. - the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources of the Resource Setting given by resourcesForChannelMeasurement. ============================= unchanged parts are omitted ============================

AsusTek

Proposal 1: From RAN1’s perspective, Option 1 approach 2 could work with limited RAN1 spec impact while Option 2 could work with no RAN1 spec impact, and it may depend on RAN2’s decision to apply which option.

R1-2506720 In more details, RAN2 identified the following two options on when RRC layer submits periodic CSI inference configuration (i.e. CSI-ReportConfig) to lower layer (i.e. PHY layer): - Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable. - Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete. From RAN2 point of view, the consequence of the two options can be described as follows: - For Option 1, companies in RAN2 think there may be two different potential approaches in PHY layer how to handle non-applicable periodic CSI-ReportConfig. It is for RAN1 information. - Approach 1: The UE’s PHY layer will immediately perform inference of periodic CSI, even if the inference configuration is non-applicable. Consequently, the UE may report invalid periodic CSI before the corresponding CSI-ReportConfig becomes applicable. - Approach 2: The UE’s PHY layer will ignore the inference configuration of periodic CSI if it is non-applicable. Consequently, the UE will not report periodic CSI before the corresponding CSI-ReportConfig becomes applicable. - For Option 2, as RRC layer holds on submitting the inference configuration to lower layer until reporting as applicable, the UE’s PHY layer will perform inference of periodic CSI and report only after sending RRCReconfigurationComplete with the corresponding CSI-ReportConfig setting to “applicable”.

Based on the summary above and the inputs in [32-51], issues and corresponding proposals related to the LS send from RAN2 (R1-2506720) are provided as follows. Two options are provided from RAN2 and the corresponding supported companies are summarized below:

- Option 1

  - Supported by: LG (App 2), Lenovo (App 1), Google (App 2), Ericsson (App 1),

- Option 2 (RAN2 preference)

  - Supported by: ZTE, vivo, Xiaomi, CMCC, CATT, OPPO, Samsung, NEC, Apple, QC, HW, AsusTek

- Up to UE implementation

  - Supported by: Nokia

Proposal. 6.1

The answer to the question in R1-2506720 is as below:

From RAN1 perspective, Option 2 is best option and can resolve the issue. - No additional RAN 1 impact was identified for Option 2.

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Option 2 is proposed based on the super majority of support. Please share your view on the proposal. |
| Google |  | For option 2, we are wondering the correct behavior when the applicability related is reported by UAI? Since we do not define any timeline for RRC message in RAN1, we think it is also fine to say this is up to UE implementation. |
| Huawei, HiSilicon |  | Support Option 2. |
| Xiaomi |  | Support |
| CATT |  | Support Option 2. |
| LGE |  | Support Option 1 since it is aligned with the previous RAN1 agreements as highlighted. Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion |
| QC |  | OK |
| OPPO |  | Support the proposal. |
| Ericsson |  | Support Option 1 approach 1. |
| CMCC |  | Support Option 2. |
| Fujitsu |  | Support the proposal. |
| Sharp |  | Support option 2 |

- RAN2 LS on candidate data collection

R1-2506715 For AI/ML based beam management and CSI prediction UE-side data collection, before UE is configured with CSI-ReportConfig for UE-side data collection measurement, RAN2 agreed to allow UE to indicate its preferred UE-side data collection configuration(s) based on the candidate UE-side data collection configuration(s) provided by network. RAN2 also agreed that UE is NOT expected to perform measurement(s) solely based on those candidate UE-side data collection configuration(s) and this UE behavior will be captured in RRC specification, i.e., no RAN1 specification impact. For AI/ML based beam management, according to RAN1 agreement on UE-side data collection, for each candidate UE-side data configuration, RAN2 agreed at least the following IEs are included: - An identifier of the candidate configuration - CSI-ResourceConfigId for Set A - CSI-ResourceConfigId for Set B - Related associated IDs In addition, RAN2 agreed the same framework of candidate/preferred UE-side data collection configuration is also applicable to CSI prediction. However, RAN2 wonders what IEs are needed for CSI prediction as part of candidate UE-side data collection configuration.

The following issues are related to the LS send from RAN2 (R1-2506715). For beam management, based on the input from [52-70], two newly agreed RRC parameters (i.e., TimeGap-r19, nroftimeinstance-r19) for BM-Case2 are suggested to be added in candidate UE-side data collection configuration. Hence the following is proposed.

Proposal. 7.1

For the UE-side candidate data collection configuration for BM-Case2, include the following IEs along with the already agreed IEs in RAN2:

- TimeGap-r19, i.e., the expected time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of expected predicted time instances.

- Include the above information as part of LS reply to RAN2

| Company | Y/N | Comments |
| --- | --- | --- |
| FL |  | Please share your view on the proposal. |
| Huawei, HiSilicon |  | Support |
| Xiaomi |  | We would like to share some information from our RAN2 colleague (i.e., the contact person of this LS). First, RAN2 has achieved the agreement on above four IEs and doesn’t expect more discussion in RAN1. And it is about the IEs in candidate configuration, Timegap-r19 and nroftimeinstance-r19 is not essential. After UE report the preferred candidate configuration, NW will reconfigure it by CSIreportconfig in which Timegap-r19 and nroftimeinstance-r19 can be included. Second, reply LS is not expected from RAN2. If there is consensus on additional IE for beam management and IEs for CSI prediction, RAN1 can put the agreed IEs into the RRC parameter list which can be sent to RAN2 at the end of this meeting. |
| CATT |  | Support |
| QC |  | Not support. As we had mentioned in our contribution, we believe the CSI-ReportConfig should be shared by NW in order for UE to determine its preference, and it is not clear why it makes sense to send anything other than that. If the purpose is overhead saving, we believe it is not justified. |
| OPPO |  | Open to discuss this LS. |
| CMCC |  | Not need. UE side trains model based on NW configured Timegap-r19 and nroftimeinstance-r19. |
| Fujitsu |  | Ok |
| Sharp |  | Support |

References

R1-2506760	Remaining Issues of Rel-19 AI/ML for NR Air Interface	Ericsson

R1-2506770	Discussion on maintenance of AI for Air Interface	ZTE Corporation, Sanechips

R1-2506871	Maintenance on AI/ML for NR Air Interface	vivo

R1-2506934	Maintenance of Rel-19 AI/ML for air interface	Huawei, HiSilicon

R1-2506961	Maintenance on AI/ML for NR Air Interface	Xiaomi

R1-2507000	Maintenance of specification support for AI/ML for NR Air Interface	CMCC

R1-2507094	Remaining issues on AI/ML for NR air interface	CATT

R1-2507155	Maintenance on Rel-19 AI/ML for NR air interface	OPPO

R1-2507227	Remaining issue on AI/ML for NR Air Interface	Samsung

R1-2507277	Remaining issues on AI/ML for air interface in Rel-19	Fujitsu

R1-2507302	Remaining Issues on AIML for NR Air Interface	NEC

R1-2507374	Discussion on AI/ML maintenance in 5G NR interface	Panasonic

R1-2507383	Maintenance on AI/ML for NR Air Interface	Nokia

R1-2507394	Maintenance on AI/ML for NR Air interface	LG Electronics

R1-2507442	Maintenance on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface	Lenovo

R1-2507452	Maintenance of AI beam management	Ofinno

R1-2507515	Maintenance for AI/ML for NR Air Interface	Google

R1-2507582	Maintenance on Rel-19 AI/ML	InterDigital, Inc.

R1-2507648	Remaining issues for R19 AI/ML for NR air interface	Apple

R1-2507694	Maintenance on AI/ML for NR air interface	Qualcomm Incorporated

R1-2507774	Maintenance on AI/ML for NR Air Interface	Sharp

R1-2507832	Remaining issues on AI/ML for air interface in Rel-19	Quectel

R1-2507865	Remaining issues on AI PHY		ASUSTeK

R1-2506765	Discussion on NW side data collection	ZTE Corporation, Sanechips

R1-2506766	Draft LS reply on NW side data collection	ZTE Corporation, Sanechips

R1-2506998	Discussion on RAN2’s LS to RAN1 and RAN3 on NW side data collection	CMCC

R1-2507226	Discussion on RAN2 LS to RAN1 and RAN3 on NW side data collection	Samsung

R1-2507420	Discussion on the RAN2 LS on NW side data collection	Ericsson

R1-2507581	Discussion for LS reply on NW side data collection	Nokia

R1-2507690	Discussion for LS reply on NW side data collection	Qualcomm Incorporated

R1-2507930	Discussion on the LS reply to RAN2 on NW side data collection	Huawei, HiSilicon

R1-2506769	Draft LS reply on when RRC layer submits periodic CSI inference configuration to lower layer			ZTE Corporation, Sanechips

R1-2506853	Draft reply LS on when RRC layer submits periodic CSI inference configuration to lower layer			vivo

R1-2506854	Discussion on when RRC layer submits periodic CSI inference configuration to lower layer			vivo

R1-2506956	Draft reply LS on when RRC layer submits periodic CSI inference configuration to lower layer			Xiaomi

R1-2506997	Discussion on LS on when RRC layer submits periodic CSI inference configuration to lower layer			CMCC

R1-2507088	Draft reply LS on when RRC layer submits periodic CSI inference configuration to lower layer			CATT

R1-2507158	Reply on LS on when RRC layer submits periodic CSI inference configuration to lower layer			OPPO

R1-2507221	Discussion on RAN2 LS on periodic CSI inference configuration to lower layer	Samsung

R1-2507222	Draft LS reply on RAN2 LS on periodic CSI inference configuration to lower layer	Samsung

R1-2507307	Discussion on LS on when RRC layer submits periodic CSI inference configuration to lower layer			NEC

R1-2507386	Discussion on when RRC layer submits periodic CSI inference configuration to lower layer			Nokia

R1-2507387	Draft LS reply on when RRC layer submits periodic CSI inference configuration to lower layer			Nokia

R1-2507392	Draft reply LS on when RRC layers submits inference configuration to lower layer	LG Electronics

R1-2507441	Draft Reply LS on when RRC layer submits periodic CSI inference configuration to lower layer			Lenovo

R1-2507524	Draft Reply LS on when RRC layer submits periodic CSI inference configuration to lower layer			Google

R1-2507645	Discussion on RAN2 LS on when RRC layer submits periodic CSI inference configuration to lower layer	Apple

R1-2507646	Draft reply on RAN2 LS on when RRC layer submits periodic CSI inference configuration to lower layer	Apple

R1-2507689	Discussion for LS reply on when RRC layer submits periodic CSI inference configuration to lower layer	Qualcomm Incorporated

R1-2507866	Discussion on LS on when RRC layer submits periodic CSI inference configuration to lower layer			Ericsson

R1-2507929	Discussion on the LS reply to RAN2 on periodic CSI inference configuration to lower layer			Huawei, HiSilicon

R1-2506767	Discussion on candidate data collection	ZTE Corporation, Sanechips

R1-2506768	Draft LS reply on candidate data collection	ZTE Corporation, Sanechips

R1-2506851	Draft reply LS on candidate data collection	vivo

R1-2506852	Discussion on candidate data collection	vivo

R1-2506999	Discussion on LS on candidate data collection	CMCC

R1-2507081	Draft reply LS on candidate data collection	CATT

R1-2507157	Reply on LS on candidate data collection	OPPO

R1-2507220	Discussion on RAN2 LS on candidate data collection		Samsung

R1-2507306	Discussion on LS on candidate data collection	NEC

R1-2507384	Discussion for LS on candidate data collection	Nokia

R1-2507385	Draft LS reply on candidate data collection	Nokia

R1-2507393	Draft reply LS on candidate data collection	LG Electronics

R1-2507431	Discussion on the RAN2 LS on candidate data collection	Ericsson

R1-2507439	Draft Reply LS on candidate data collection	Lenovo

R1-2507523	Draft Reply LS on Candidate Data Collection	Google

R1-2507647	Discussion on RAN2 LS on candidate data collection	Apple

R1-2507691	Discussion for LS reply on candidate data collection	Qualcomm Incorporated

R1-2507773	Discussion on RAN2 LS on candidate data collection		Sharp

R1-2507928	Discussion on the LS reply to RAN2 on candidate data collection	Huawei, HiSilicon

Appendix A. Previous agreements

RAN1#116

Agreement

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling

- Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications

- FFS on the report content for beam related information

- FFS on max number of reported beam related information in one report

Agreement

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

  - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

    - FFS on the quantization method of probability information

    - Probability information is the probability of the beam to be the Top 1 or Top K beam

  - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

    - FFS on definition of reported RSRP

    - FFS on the definition and quantization method of confidence information

  - Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

Agreement

- For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework

- FFS on whether/how potential enhancement is needed

Conclusion

For UE sided model at least for inference, for measurement, the configuration of Set B,

- take the current CSI framework as the starting point

RAN1#116b

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Agreement

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Agreement

For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting

- FFS on the details in the CSI-ReportConfig, at least considering:

  - Alt 1: one CSI-ResourceConfigId is configured for Set B

    - FFS: how UE can determine the information about set A

  - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

    - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

  - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

    - FFS: how to configure/indicate separate resource set(s) for Set A

  - Note: separate CSI-ReportConfig for Set A and Set B are not precluded.

  - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig

  - FFS on the association between Set A and Set B with or without additional IE

  - Other necessary configuration are not precluded.

Agreement

Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B:

- Opt1: Based on associated ID (Referring to AI 9.1.3.3)

  - FFS on what can be assumed by UE with the same associated ID across training and inference

  - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

- Opt 2: Performance monitoring based

  - FFS details

- Other options are not precluded.

RAN1#117

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

Agreement

At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support

- Support differential L1-RSRP reporting with legacy quantization step and range

  - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP

  - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Agreement

Following Working Assumption is confirmed.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

- Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

RAN1#118

Agreement

For UE sided model in beam management, introduce support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Agreement

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement

For UE-sided model at least for BM Case-1, for inference results report

- Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

  - FFS whether support only resource set for Set B is configured

- UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

- The beam information in the inference report refers to the resource set for Set A

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including:

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - Note: resources for Set B for monitoring are not precluded and can be study.

  - Note: this is only applicable when the model can predict RSRP

- Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam

  - Note: this is only applicable when the model can generate probability information

- FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including

  - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference.

- For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Agreement

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

RAN1#118bis

Agreement

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

- FFS other alternatives

Agreement

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

Agreement

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

Agreement

For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- Aperiodic (AP) CSI-RS

For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- FFS: Aperiodic (AP) CSI-RS

Note: above CSI-RS resource refers to that used for beam management.

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring:

- Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

  - FFS: the resource set(s) for monitoring

  - UE measures the resource set(s) for monitoring.

  - FFS how/when to report the monitoring results.

- Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The dedicated report configuration used for monitoring links to an inference report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - UE measures the resource set(s) for monitoring.

  - FFS when to report the monitoring results.

Agreement

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Agreement

For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report.

Agreement

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Agreement

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

RAN1#119

Agreement

- For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Conclusion

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- Two CSI-ResourceConfigId s are configured for Set A and Set B separately

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

RAN1#120

Agreement

For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

Agreement

For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Note: how to capture it in the spec is a separate discussion.

Agreement

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances,

- the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Agreement

For inference, for BM-Case 2 of UE-side model,

- The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC.

  - time gap is [10ms, 20ms, 40ms, 80ms, 160ms]

  - N = [1, 2, 4, 8]

- Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement

  - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement

For UE-sided model, in CSI-ReportConfig for inference

- One or two associated IDs can be configured in CSI-ReportConfig

  - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

  - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

RAN1#120b

Agreement

For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported.

Agreement

For BM-Case1, for the Top K beam(s) report as the inference results

- K is configured in inference report configuration to the UE.

Agreement

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #K

RSRP #1

differential RSRP #2

…

differential RSRP #K

- CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K

- RSRP #1 is absolute predicted RSRP

- Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Agreement

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

Time instance indicator

CRI or SSBRI #1 of time instance #1

CRI or SSBRI #2 of time instance #1

…

CRI or SSBRI #K of time instance #1

CRI or SSBRI #1 of time instance #2

CRI or SSBRI #2 of time instance #2

…

CRI or SSBRI #K of time instance #2

…

CRI or SSBRI #1 of time instance #N

CRI or SSBRI #2 of time instance #N

…

CRI or SSBRI #K of time instance #N

RSRP #1 of time instance #1

Differential RSRP #2 of time instance #1

…

Differential RSRP #K of time instance #1

Differential RSRP #1 of time instance #2

Differential RSRP #2 of time instance #2

…

Differential RSRP #K of time instance #2

…

Differential RSRP #1 of time instance #N

Differential RSRP #2 of time instance #N

…

Differential RSRP #K of time instance #N

  - Time instance indicator exist if N > 1

  - The size of CSI field for time instance indicator is

  - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances

  - Time instance #1 corresponds to the time instance indicated by the time instance indicator

  - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances

    - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s)

  - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K

  - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

Agreement

For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances, and also represents the time gap between the reference time and the first future time instance for prediction.

Agreement

For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

Agreement

For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, .

Note: the occupation duration is a separate discussion.

Agreement

For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as:

- At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

  - Where K is the number of predicted beam(s) in the corresponding inference report per time instance

  - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring

  - M is configured by NW in CSI report configuration for monitoring

    - M= 1, 2

  - FFS: detailed rule or signalling

Agreement

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model:

  - Support the size of a set for monitoring is the same as the size of Set A,

    - The n-th resource in the set for monitoring is linked to the n-th resource in Set A.

  - Support the size of a set for monitoring is smaller than the size of Set A

Working Assumption

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Working Assumption

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

- wherein N (N>=1) is configured in CSI-ReportConfig

- FFS on additional rule for counting N linked pair

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Conclusion

For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

Agreement

- For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection:

  - Option 1: only dedicated AI/ML PU is occupied,  is reported by UE.

    - And

  - Option 2: only legacy CPU is occupied,  it is reported by UE.

  - Option 3: both dedicated AI/ML PU and legacy CPU are occupied,  is reported by UE.

    - And

Note: The supported option by UE is reported by UE capability, if multiple options are supported.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

  - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Working Assumption

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

RAN1#121

Agreement

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model, when the size of the set for monitoring is smaller than the size of Set A,

- support the mapping of the resources in the set for monitoring to resources in Set A is configured via RRC, support

  - A X-bit bitmap with Y non-zero bits is configured by the RRC in CSI Report Config for monitoring, where X is the size of Set A and Y is the size of the set for monitoring

    - The x-th MSB of the bitmap corresponds to x-th resource in Set A

    - The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the set for monitoring, 1≤y≤Y

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring.

  - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

- Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately.

The associated working assumption made in RAN1#120b will not be confirmed.

Agreement

For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19”.

Agreement

For UE-sided model, for inference report for BM-Case2, a time instance for prediction is defined as a slot.

Conclusion

For NW sided model for L1-RSRP report in L1 signaling, legacy quantization steps and range are reused.

Agreement

For the determination of CSI report priority value of a CSI report for inference, the existing  is reused.

- k = 0 for the CSI report for inference.

For the determination of CSI report priority value of a CSI report for monitoring, the existing  is reused.

- k = 0 for the CSI report for monitoring.

Agreement

For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference,

- For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

  - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively.

  - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively.

  - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature.

  - Note: Combination of OAPU= 0 and OCPU=0 is not allowed.

  - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied.

Agreement

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively.

- Detailed values of d and d’ can be further discussed in UE feature.

Agreement

For UE-sided model, regarding a CSI-ReportConfig for data collection,

- Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured.

Agreement

For NW-sided model, for inference, when M<the size of measurement resource set, the beam information is CRI/SSBRI

Note: The purpose, such as above “For NW-sided model, for inference” will not be specified in RAN1 specifications.

Agreement

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1,

- Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report.

- Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report.

- Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report).

Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is (0 ≤≤ N)

- Where  is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig the size of CSI field associated with the RS-PAI is

Agreement

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Agreement

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring

  - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

    - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring.

    - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

  - Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP

The associated working assumption made in RAN1#120b will not be confirmed.

Agreement

For UE-sided model, for BM-Case 1 and BM-Case 2, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information.

Agreement

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case2, for occupancy duration of CPU and APU, same occupation time for AI/ML PU and legacy CPU.

- If the CSI report is aperiodic, for AI/ML PU, and for CPU, Rel-15 CPU occupation time for AP CSI report is reused

- If the CSI report is semi-persistent or periodic,

  - From the 1st symbol of the latest CSI-RS/SSB transmission occasion no later than CSI reference resource, until the last symbol of the PUCCH/PUSCH carrying the report.

Agreement

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused:

- For both BM-Case 1 and BM-Case 2:

  - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19

- For BM-Case 2:

  - TimeGap-r19, nroftimeinstance-r19,

- Note: this doesn’t imply the associated ID is always present

RAN1#122

Agreement:

Adopt the following TP for the clarification of ranking information.

Reason for change: The description of ranking information for P-CRIs or P-SSBRIs is unclear.

Summary of change: Clarify that the ranking information is for P-CRIs or P-SSBRIs.

Consequences if not approved: UE cannot generate the ranking information.

TP for TS 38.214 Clause 5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <omitted texts> For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)). If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. For P-CRI or P-SSBRI reporting without P-L1-RSRP, the ranking information of the nrofreportedpredictedrs-r19 P-CRIs or P-SSBRIs (per time instance, if nroftimeinstance-r19 is configured) is conveyed by the order of the P-CRIs or P-SSBRIs reported in the CSI report, where the first reported P-CRI or P-SSBRI ranks first. <omitted texts>

Agreement:

Adopt the following TP for CSI reporting for monitoring.

Reason for change: The condition associated with CSI reporting for RS-PAI is unclear.

Summary of change: Clarify that the timeinstanceformonitoring-r19-th time instance is used in case of BM-Case2 for condition check. Adding the description on resource mapping methods depending on the size of the set for monitoring is smaller than or the same as the size of Set A.

Consequences if not approved: gNB and UE may have different understanding on the condition associated with CSI reporting for RS-PAI.

TP for TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting ##### 5.2.1.4.3b RS-PAI Reporting <omitted texts> - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a -bit field (M is given by nroftransmissionOccasion-r19). <omitted texts>

Agreement:

Adopt the following TP in section 5.2.1.6, TS 38.214.

Reason for change: The CPU occupation time of the monitoring report is unclear.

Summary of change: For UE-sided model, regarding a CSI report with CSI-ReportConfig for monitoring, Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report for all types of CSI reports (i.e., AP/SP/P CSI report).

Consequence if not approved: The CPU occupation time of the monitoring report is not aligned between the gNB and UE, leading to inefficient CSI report scheduling.

#### 5.2.1.6 CSI processing criteria #### <Unchanged part is omitted> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including and/or , for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' or 'rs-pai-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. #### <Unchanged part is omitted>

Agreement

Adopt the following TP for Clause 6.3.1.1.2 in TS 38.212.

Reason for change: For NW-sided model, for a CSI-ReportConfig with the number of reported RS is the same as the size of the corresponding resource set for channel measurement, the mapping between the reported RSRPs and the resources within the resource set is unclear.

Summary of change: Clarify the mapping between the reported RSRPs and the resources within the resource set when the number of reported RS of a CSI-ReportConfig is the same as the size of the corresponding resource set for channel measurement.

Consequence if not approved: The understanding of resources corresponding to reported RSRPs is not aligned between the gNB and UE.

Table 6.3.1.1.2-8G: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting, if nrofReportedRS is configured CSI report number; CSI fields. CSI report #n; CRI or SSBRI #1 as in Table 6.3.1.1.2-6. CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported. …. CRI or SSBRI # M as in Table 6.3.1.1.2-6, if reported. RSRP #1 as in Table 6.3.1.1.2-6. Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported. …. Differential RSRP # M as in Table 6.3.1.1.2-6, if reported. NOTE: The value of M is configured by the higher layer parameter nrofReportedRS. If M is equal to the number of SSB/CSI-RS resources in the corresponding resource set for channel measurement, Differential RSRP #m, m = 2, …, M, corresponds to the (m-1)-th SSB/CSI-RS resources among the remaining M-1 SSB/CSI-RS resources other than the resource corresponding to CRI/SSBRI#1 in the resource set..

Agreement

For one CSI resource setting for Set A for CSI report for inference for UE-side model

- Only a single resource set can be configured for the CSI resource setting.

Agreement

Support the following on the reporting condition of CSI reporting for inference

- For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

Agreement

Support the following on the reporting condition of CSI reporting for monitoring

- After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for monitoring only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion for each of the RS resources in the resource set for monitoring no later than the corresponding CSI reference resource.

Agreement:

For AP CSI report for inference, regarding the associated ID for aperiodic CSI resource setting with multiple resource sets for Set B

- Associated ID in CSI report configuration is for the resource set(s) for Set B

  - Note: This does not have impact on RAN1 specifications

Agreement:

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: The condition of reporting CSI report for inference for BM-Case2 is unclear.

Summary of change: Adding the condition of reporting CSI report for inference for BM-Case2.

Consequences if not approved: gNB and UE may have different understanding on the condition of reporting CSI report for inference for BM-Case2.

5.2.2.5 CSI reference resource definition <omitted texts> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'etypeII-Doppler-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or 𝐾𝑝 consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability. <omitted texts>

Agreement:

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: The condition of reporting RS-PAI is unclear.

Summary of change: Adding the condition of reporting RS-PAI.

Consequences if not approved: gNB and UE may have different understanding on the condition of reporting RS-PAI.

5.2.2.5 CSI reference resource definition <omitted texts> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'etypeII-Doppler-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or 𝐾𝑝 consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'rs-pai-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. <omitted texts>

Agreement:

Adopt the following TP in section 5.2.1.6, TS 38.214 in principle subject to the outcome of the discussion in Proposal below.

Proposal

- For TS38.214 Section 5.2.1.6, use the original notion of CPU (instead of CPU, 1) to represent the legacy CPU, and replace all the text of “CPU, 2” with “APU” to correctly represent AI/ML-based PU.

Reason for change: To capture the agreement to support two AI/ML PU pools.

Summary of change: Adding the description to support two AI/ML PU pools.

Consequences if not approved: two AI/ML PU pools are not supported.

#### 5.2.1.6 CSI processing criteria <Unchanged part is omitted> For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,2x simultaneous CSI calculations it is said to have NCPU,2x CSI processing units for processing CSI reports. If L Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,2x-LLx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,2x-L Lx CPUs are unoccupied, where each CSI report corresponds to OCPU,2x(n), the UE is not required to update the N-M2x requested CSI reports with lowest priority (according to Clause 5.2.5), where is the largest value such that Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2,3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report is not considered within any of M and M2x, the values for OCPU,1 and OCPU,2x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. <Unchanged part is omitted> - for a CSI report with CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', - if nroftimeinstance-r19 is not configured, OCPU,1=X1 and OCPU,2x=X2x, where the value of X1and X2x are reported by UE capability. - if nroftimeinstance-r19 is configured, OCPU,1=Y1 and OCPU,2x=Y2x, where the value of Y1and Y2x are reported by UE capability. - OCPU,1=X1 and OCPU,2x=X2x, for a CSI report configured with the higher layer parameter [RRC_name-r19], where the values of X1 and X2x are reported by UE capability. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,2x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows: <Unchanged part is omitted>

Agreement:

Support the configuration of following parameters in a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19':

- TimeGap-r19, i.e., the time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of predicted time instances for BM-Case 2.

Appendix B. Topic specific agreements

NW-side AI/ML model

Agreement (RAN1#116)

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling

- Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications

- FFS on the report content for beam related information

- FFS on max number of reported beam related information in one report

Agreement (RAN1#116bis)

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above "For NW-sided model, for BM-Case1 and BM-Case2" and "Set A" and "Set B", will not be specified in RAN 1 specifications

Agreement (RAN1#117)

At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support

- Support differential L1-RSRP reporting with legacy quantization step and range

  - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP

  - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Agreement (RAN1#117)

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

  - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

- Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

Conclusion (RAN1#121)

- For NW sided model for L1-RSRP report in L1 signaling, legacy quantization steps and range are reused.

Agreement (RAN1#121)

For NW-sided model, for inference, when M<the size of measurement resource set, the beam information is CRI/SSBRI

Note: The purpose, such as above “For NW-sided model, for inference” will not be specified in RAN1 specifications.

#### Beam indication

Agreement (RAN1#116)

For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework

- FFS on whether/how potential enhancement is needed

UE-side AI/ML model

#### Data collection

Agreement (RAN1#120)

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement (RAN1#120bis)

For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported.

Agreement (RAN1#121)

For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19”

#### Inference (BM-Case1)

Model consistency (Associated ID)

Agreement (RAN1#116bis)

Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B:

- Opt1: Based on associated ID (Referring to AI 9.1.3.3)

  - FFS on what can be assumed by UE with the same associated ID across training and inference

  - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

- Opt 2: Performance monitoring based

  - FFS details

- Other options are not precluded.

Agreement (RAN1#118)

For UE sided model in beam management, support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Agreement (RAN1#120)

For UE-sided model, in CSI-ReportConfig for inference

- One or two associated IDs can be configured in CSI-ReportConfig

  - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

  - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

Resource/Reporting Configuration

Conclusion (RAN1#116)

For UE sided model at least for inference, for measurement, the configuration of Set B,

- take the current CSI framework as the starting point

Agreement (RAN1#116bis)

For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting

- FFS on the details in the CSI-ReportConfig, at least considering:

  - Alt 1: one CSI-ResourceConfigId is configured for Set B

    - FFS: how UE can determine the information about set A

  - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

    - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

  - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

    - FFS: how to configure/indicate separate resource set(s) for Set A

  - Note: separate CSI-ReportConfig for Set A and Set B are not precluded.

  - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig

  - FFS on the association between Set A and Set B with or without additional IE

  - Other necessary configuration are not precluded.

Agreement (RAN1#118)

For UE-sided model at least for BM Case-1, for inference results report

- Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

  - FFS whether support only resource set for Set B is configured

- UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

- The beam information in the inference report refers to the resource set for Set A

Agreement (RAN1#118bis)

For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- Aperiodic (AP) CSI-RS

For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- FFS: Aperiodic (AP) CSI-RS

Note: above CSI-RS resource refers to that used for beam management.

Agreement (RAN1#118bis)

For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report.

Agreement (RAN1#118bis)

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Conclusion (RAN1#119)

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement (RAN1#119)

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- two CSI-ResourceConfigId s are configured for Set A and Set B separately.

Report contents

Agreement (RAN1#116)

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

  - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

    - FFS on the quantization method of probability information

    - Probability information is the probability of the beam to be the Top 1 or Top K beam

  - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

    - FFS on definition of reported RSRP

    - FFS on the definition and quantization method of confidence information

  - Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

Agreement (RAN1#116bis)

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP.

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

- Where the predicted RSRP is based on AI/ML output.

- Note: Support both Option A and Option B is not precluded.

Working Assumption (RAN1#116bis)

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement (RAN1#117)

Following Working Assumption is confirmed.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement (RAN1#119)

For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Agreement (RAN1#120)

For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

Agreement (RAN1#120)

For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Note: how to capture it in the spec is a separate discussion.

Agreement (RAN1#120bis)

For BM-Case1, for the Top K beam(s) report as the inference results

- K is configured in inference report configuration to the UE.

Agreement (RAN1#120bis)

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #K

RSRP #1

differential RSRP #2

…

differential RSRP #K

- CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K

- RSRP #1 is absolute predicted RSRP

- Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Agreement (RAN1#121)

For UE-sided model, for BM-Case 1 and BM-Case 2, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information.

#### Inference (BM-Case2)

Agreement (RAN1#116bis)

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1.

  - Note: overhead reduction is not precluded.

- FFS on details

Agreement (RAN1#118)

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement (RAN1#118)

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

Agreement (RAN1#118bis)

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

Agreement (RAN1#120)

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances,

- the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Agreement (RAN1#120)

For inference, for BM-Case 2 of UE-side model,

- The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC.

  - time gap is [10ms, 20ms, 40ms, 80ms, 160ms]

  - N = [1, 2, 4, 8]

- Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement

  - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement (RAN1#120bis)

For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances, and also represents the time gap between the reference time and the first future time instance for prediction.

Agreement (RAN1#120bis)

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

Time instance indicator

CRI or SSBRI #1 of time instance #1

CRI or SSBRI #2 of time instance #1

…

CRI or SSBRI #K of time instance #1

CRI or SSBRI #1 of time instance #2

CRI or SSBRI #2 of time instance #2

…

CRI or SSBRI #K of time instance #2

…

CRI or SSBRI #1 of time instance #N

CRI or SSBRI #2 of time instance #N

…

CRI or SSBRI #K of time instance #N

RSRP #1 of time instance #1

Differential RSRP #2 of time instance #1

…

Differential RSRP #K of time instance #1

Differential RSRP #1 of time instance #2

Differential RSRP #2 of time instance #2

…

Differential RSRP #K of time instance #2

…

Differential RSRP #1 of time instance #N

Differential RSRP #2 of time instance #N

…

Differential RSRP #K of time instance #N

  - Time instance indicator exist if N > 1

  - The size of CSI field for time instance indicator is

  - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances

  - Time instance #1 corresponds to the time instance indicated by the time instance indicator

  - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances

    - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s)

  - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K

  - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

Conclusion (RAN1#120bis)

For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

Agreement (RAN1#121)

For UE-sided model, for inference report for BM-Case2, a time instance for prediction is defined as a slot.

#### Performance monitoring

Agreement (RAN1#117)

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

Agreement (RAN1#118)

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including:

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - Note: resources for Set B for monitoring are not precluded and can be study.

  - Note: this is only applicable when the model can predict RSRP

- Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam

  - Note: this is only applicable when the model can generate probability information

- FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including

  - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference.

- For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Agreement (RAN1#118bis)

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

- FFS other alternatives

Agreement (RAN1#118bis)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring:

- Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

  - FFS: the resource set(s) for monitoring

  - UE measures the resource set(s) for monitoring.

  - FFS how/when to report the monitoring results.

- Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The dedicated report configuration used for monitoring links to an inference report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - UE measures the resource set(s) for monitoring.

  - FFS when to report the monitoring results.

Agreement (RAN1#119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Agreement (RAN1#120bis)

For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1#120bis)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

Agreement (RAN1#120bis)

For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, .

Note: the occupation duration is a separate discussion.

Agreement (RAN1#120bis)

For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as:

- At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

  - Where K is the number of predicted beam(s) in the corresponding inference report per time instance

  - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring

  - M is configured by NW in CSI report configuration for monitoring

    - M= 1, 2

  - FFS: detailed rule or signalling

Agreement (RAN1#120bis)

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model:

  - Support the size of a set for monitoring is the same as the size of Set A,

    - The n-th resource in the set for monitoring is linked to the n-th resource in Set A.

  - Support the size of a set for monitoring is smaller than the size of Set A

Working Assumption (RAN1#120bis)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Working Assumption (RAN1#120bis)

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

- wherein N (N>=1) is configured in CSI-ReportConfig

- FFS on additional rule for counting N linked pair

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Working Assumption (RAN1#120bis)

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

Agreement (RAN1#121)

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is (0 ≤≤ N)

- Where  is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is

Agreement (RAN1#121)

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model, when the size of the set for monitoring is smaller than the size of Set A,

- support the mapping of the resources in the set for monitoring to resources in Set A is configured via RRC, support

  - A X-bit bitmap with Y non-zero bits is configured by the RRC in CSI Report Config for monitoring, where X is the size of Set A and Y is the size of the set for monitoring

  - The x-th MSB of the bitmap corresponds to x-th resource in Set A

  - The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the set for monitoring, 1≤y≤Y

Agreement (RAN1#121)

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

- Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately

- The associated working assumption made in RAN1#120b will not be confirmed.

Agreement (RAN1#121)

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring

- The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

- Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP

The associated working assumption made in RAN1#120b will not be confirmed.

Agreement (RAN1#121)

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

#### CPU/APU

CSI report for data collection

Agreement (RAN1#121)

For UE-sided model, regarding a CSI-ReportConfig for data collection,

- Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured

CSI report for inference

Agreement (RAN1#118bis)

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2.

Agreement (RAN1#120bis)

- For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection:

  - Option 1: only dedicated AI/ML PU is occupied,  is reported by UE.

    - And

  - Option 2: only legacy CPU is occupied,  it is reported by UE.

  - Option 3: both dedicated AI/ML PU and legacy CPU are occupied,  is reported by UE.

    - And

Note: The supported option by UE is reported by UE capability, if multiple options are supported.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

  - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement (RAN1#121)

For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference,

- For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

  - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively

  - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively

  - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature.

  - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

  - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

Agreement (RAN1#121)

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1,

  - Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report

  - Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report

  - Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report)

Agreement (RAN1#121)

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case2, for occupancy duration of CPU and APU, same occupation time for AI/ML PU and legacy CPU.

- If the CSI report is aperiodic, for AI/ML PU, and for CPU, Rel-15 CPU occupation time for AP CSI report is reused

- If the CSI report is semi-persistent or periodic,

  - From the 1st symbol of the latest CSI-RS/SSB transmission occasion no later than CSI reference resource, until the last symbol of the PUCCH/PUSCH carrying the report.

CSI report for monitoring

Agreement (RAN1#120bis)

For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, .

Note: the occupation duration is a separate discussion.

#### Z/Z’ timeline

Agreement (RAN1#121)

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively

- Detailed values of d and d’ can be further discussed in UE feature.

#### CSI priority value

Agreement (RAN1#121)

For the determination of CSI report priority value of a CSI report for inference, the existing  is reused

- k = 0 for the CSI report for inference

For the determination of CSI report priority value of a CSI report for monitoring, the existing  is reused

- k = 0 for the CSI report for monitoring

RAN2 LS related

Agreement (RAN1#118bis)

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Agreement (RAN1#118bis)

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Agreement (RAN1#118bis)

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing.

Agreement (RAN1#118bis)

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Agreement (RAN1#119)

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Conclusion (RAN1#119)

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

Agreement (RAN1#119)

Send LS to RAN2 with below information.

RAN1 thanks RAN2 for the LS on applicable functionality reporting for beam management UE-sided model.

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

RAN 1 made the following agreements related to the Questions from RAN 2:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN1 would like to provide replies on the following questions from RAN2 in R2-2407848:

Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Q3: Is NW-side additional condition functionality specific?

Answer to Q3: Please also refer to the answer to Q2 to understand the ongoing discussion about the associated ID for NW-side additional condition. And please refer to the agreements related to the Questions from RAN 2.

Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

Answer to Q4: And please refer to the agreements related to the Questions from RAN 2.

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

Answer to Q4-2: Please refer to the agreements related to the Questions from RAN 2.

Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

Answer to Q4-3: Please refer to the agreements related to the Questions from RAN 2.

Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

Answer to Q4-4: Please refer to the agreements related to the Questions from RAN 2.

Q5: What is the content of applicable functionality reporting in Step 4?

Answer to Q5: Please refer to the agreements related to the Questions from RAN 2.

Q6: What is the content of inference configuration in Step 5?

Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

Answer to Q7: Please refer to the agreements related to the Questions from RAN 2.

Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

Answer to Q8: Please refer to the agreements/conclusion related to the Questions from RAN 2.

Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

Answer to Q9: Please refer to the agreements related to the Questions from RAN 2.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

Answer to Q10:  Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

Final reply LS is approved in R1-2410898.

Agreement (RAN1#121)

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused:

- For both BM-Case 1 and BM-Case 2:

  - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19

- For BM-Case 2:

  - TimeGap-r19, nroftimeinstance-r19,

- Note: this doesn’t imply the associated ID is always present