3GPP TSG RAN WG1 #122bis		R1-250xxxx

Prague, Czech, Oct 13th – 17th, 2025

Source: 	Moderator (Apple)

Title:                     FL Summary #1 of NR Mobility enhancement Phase 4

Agenda item:	8.8

Document for:	Discussion and Decision

1 Introduction

This document summarizes the contributions made under the agenda item 5 and 8.8 related to the Rel-19 work item ‘NR mobility enhancements Phase 4’.

2. Contact people

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

| Name | Company | Email address |
| --- | --- | --- |
| Hong He | Apple (FL) | hhe5@apple.com |

3.	CSI acquisition

## Issue 3-1: Reference Configuration for for CQI/PMI/RI Derivation

In 5G NR, reference configuration was defined for CQI/PMI/RI Derivation for serving cell as follows:

…. If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in clause 5.2.2.3.1. - Assume no REs allocated for NZP CSI-RS and ZP CSI-RS. - Assume the same number of front loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig. - Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition. - Assume the PDSCH symbols are not containing DM-RS. - Assume PRB bundling size of 2 PRBs. …..

Three contributions – from Nokia [5], Ericsson [12] and Apple [6] - addressed the issure concerning the reference configuration assumed by UE when computing early CSI report for a candidate cell, aiming to align its understanding with that of the gNB. According to TS 38.214 cited above, the reference configurations are fixed in the specification except for the highglighted DMRS-related assumptions, which depends on parameters provided by RRC signaling in the DMRS-DownlinkConfig IE within the relevant BWP.

Table 1 listed three options identified .

Table 3-1: reference configuration for early CSI associated with target candidate cell

| Index | Description | Proponents |
| --- | --- | --- |
| Opt.1 | - The parameters are derived from the initial BWP configuration of the target cell. | Support: Ericsson |
| Opt.2 | - Default values definied for ‘maxlength’ IE and ‘dmrs-AdditioinalPosition’ IE in DMRS-DownlinkConfig are used to derive the CQI index for target candicate cell. | Support: Apple |
| Opt.3 | - For each candidate, add new RRC parameters in LTM-Candidate IE, outside of the ltm-CandidateConfig. | Concerned: Ericsson |

Please note that Option 3 involves the addition of two new RRC parameters, which is currently less favorable from FL’s perspective due to the freeze on ASN.1. In addition, FL intends to finalize the preferred option first before initiating discussions on the corresponding TP.

| Moderater Question 3-1: which of Opt.1/2/3 above is preferred to define reference configuration used for early CSI report of target candidate cell? In addition, please also comment whether you think TP is need for the preferred Option. |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate Option) | Comments (If a Option is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, Hisilicon | Option 1 | UE can store the measured SINR and transform them to CQI after receiving CSC. In addition, option 1 can be also used for early CSI acquisition for L3HO |

## Issue 3-2: Early CSI Report on PUSCH Repetition

At the RAN1 #121 meeting [2], it was agreed that early CSI must be transmitted on the initial DG-PUSCH, CG-PUSCH, or PUSCH allocated by RAR or Msg-A. Repetition schemes are supported on all these PUSCH channels, which depends on PUSCH configuration and scheduling DCI.

Agreement For PUSCH to convey the early CSI report, - For RACH-less LTM, the first CG or DG PUSCH after CSC is used - For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A - For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B For the reporting LTM early CSI reporting, Table 6.3.1.1.2-7 in TS 38.212 is used as a UCI report format

One open issue identified is how to transmit the early CSI report for a target candidate cell when PUSCH repetition Type-A or Type-B is configured for CG-PUSCH or DG-PUSCH or enabled for PUSCH scheduled by RAR or Msg-A.

This issue was addressed in two contributions [Ofinno, 11] and [Apple, 6], with a consensus emerging to reuse the legacy approach—specifically, multiplexing CSI in the first actual PUSCH for Type-B. However, further discussion is needed on how to manage Type-A repetition.

| Moderater Proposal 3-2: - In case PUSCH repetition Type-B is applied for first CG-PUSCH or DG-PUSCH, reuse the legacy rule for CSI multiplexing and CSI is transmitted on the first actual occasion. |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes vs. No) | Comments (If Propsoal is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon | Yes |  |

It should be noted taht the above rule does not apply to PUSCH repetition Type-A, as there is only a single occasion within the aperiodic CSI slot. In other words, no special rule was defined in legacy for Type-A repetition and it behaves the same as when PUSCH repetition is not used. In particular, the A-CSI report is not restricted to the first occasion for PUSCH repetition Type-A.

| Moderater Question 3-2: which of options below is preferred for early CSI report on PUSCH repetition Type A: - Opt.1: Reuse legacy rule defined for PUSCH repetition Type A (i.e., leave UE to select an occasion that is overlapped with early CSI report) - Opt.2: Multiplex early CSI on the first occasion of PUSCH repetition Type-A, same handling as Type-B. |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate Option) | Comments (If a Option is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon | Option 2 | Opt 2 is also the legacy behavior for A-CSI mux in PUSCH with slot aggregation according to the following conclusion in RAN1#101. Conclusion Conclusion in RAN1#96 with respect to A-CSI multiplexing in PUSCH with slot aggregation is interpreted as the following: - When PUSCH slot aggregation is enabled, if A-CSI triggered by a DCI that schedules a PUSCH in a slot, the A-CSI is multiplexed only in the PUSCH in the first slot. - A valid A-CSI is multiplexed only if the CSI computation corresponding timeline is met. - The CSI computation timeline is referenced to the first slot of the slots with PUSCH repetition. No changes to the specifications are needed. |

## Issue 3-3: Miscellaneous

A set of proposals from one company is outlined below to gather input.  FL recommends that companies examine the associated contributions to better comprehend the reasoning behind the proposal prior to stating their positions.

Table 3-2:

| Index | Proposal description | Background/Justification based on contribution | FL’s Initial Assessment |
| --- | --- | --- | --- |
| P1 | Regarding early CSI acquisition for LTM, support the UE to send the CSI report at least a time gap after reception of the LTM CSC, where the time gap is provided by the network subject to a corresponding UE’s capability. [Samsung, 8] |  | A UE capability discussion. Additionally, since no specific timeline was defined for the timing of early CSI report, the UE retains full flexibility in deciding when to transmit the eCSI. |
| P2 | Confirm the following UE behavior as RAN1 common understanding: - After UE receives the LTM CSC MAC CE, capable UE can still measure P-CSI-RS and SP CSI-RS resources activated before the LTM CSC MAC-CE, even if these CSI-RS resources are not counted/defined as “active”. [Google, 14] |  |  |
| P3 | Support introducing separate UE capability for active CSI-RS ports/resources for gNB controlled L1-RSRP measurement from legacy UE capability. [Spreadtrum, 1] |  | The following was concluded in RAN1 after debating on how to define L1-RSRP report for candidate cell: Conclusion: For the counting of active CSI-RS ports/resources before CSC, no new UE capability is introduced. The total number of active CSI-RS ports/resources is limited by legacy capability report. The total number of active CSI-RS ports/resources is shared by serving cell and candidate cell(s). Therefore, it is unclear whether this proposal is needed. |
| P4 | - The semi-persistent NZP CSI-RSs and CSI-IM resources associated with the CSI report configuration corresponding to the target cell can be automatically deactivated after UE transmits CSI report. [Huawei, 3] | - As CSI-RS measurement can be performed after CSC, when the semi-persistent NZP CSI-RS resources and CSI-IM resources are deactivated is unclear and should be discussed. - To avoid additional signaling overhead, we think these NZP CSI-RSs and CSI-IM resources associated with the CSI report configuration corresponding to the target cell can be automatically deactivated until CSI report is transmitted by UE. |  |

| Moderater Question 3-2: Which of these proposals are supported? |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate which proposal(s) you support) | Comments (If a particular proposal is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon | P2+P4 | Agree with FL views on P1 and P3. For P2, at least for UE only measure CSI after CSC, it must be used. |

## Issue 3-4:  Others

Companies are invited to highlight any critical issues related to the ‘early CSI report’ and L1-RSRP for the candidate cell that were proposed in contribution, but missed from FL's summary above.

| Company | Comments |
| --- | --- |

4. Conditional LTM

Three issues were raised and discussed by the companies: one is related to the TCI-State deactivation, and the other involves TA determination, and early CSI report on PUCCH repetition. Additionally, TPs were included in the related contribution, and FL intends to address both matters and then initiate TP discussion.

## Issue 4-1: TCI-State De-activation

CLTM-related issue concerning TCI-state determination was raised by [Samsung, 8], [Ofinno, 16], [vivo, 2] and [Apple,6].

The following was agreed in RAN2:

RAN2#129 meeting Agreements on C-LTM: 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

RAN2#130 meeting agreement - If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

RAN2 131 Meeting Open issue MAC-21: Whether indicate the selected RS index to the lower layer for RACH-less CLTM. Proposal 3: (14/14) MAC indicates the selected RS index to the lower layer for RACH-less CLTM, as captured in the current MAC running CR. Agreed.

| Moderater Question 4-1: Is the following proposal acceptable? For C-LTM, after the LTM cell switch is triggered, - For RACH-less LTM, the UE determines the TCI state in CandidateTCI-State or CandidateTCI-UL-State whose QCL RS has the same value as the RS signaled from higher layer to lower layer that meets the C-LTM execution condition. - Support (based on contribution): Samsung, vivo, Apple, Ofinno |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate your support: Yes, No, or specify the preferred option.) | Comments (If a particular scheme is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon |  | It is not clear the RS signaled from the higher layer is RS for BM or SSB in the LTM TCI state. In addition, it is not clear how to choose the TCI state is multiple TCI state associated with same RS. |

| Moderater Question 4-2: Is the following proposal from [Samsung, 8], [vivo, 2] [Ofinno,11] [ZTE, 4] and [Apple,6] acceptable? - ‘P4-2-1’: For RACH-based LTM triggered in C-LTM, the UE determines the TCI state as the one associated with the SSB determined during RACH procedure. - Support (based on contribution): Samsung, Apple, ZTE - ‘P4-2-2’: All activated candidate TCI states are deactivated after RACH-based CLTM is performed [ZTE, 4] |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate your support: Yes, No, for each proposal) | Comments (If a particular proposal is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon |  | For P4-2-1, if there are multiple TCI state associated with same SSB, how to choose among them is not clear. For P4-2-2, support |

| Moderater Question 4-3 Is the following proposal from [vivo, 2] [Ofinno,11] [ZTE, 4] and [Apple,6] acceptable? Please also comment which alternative you prefer for the timeline: - Activated Candidate TCI state(s), other than the indicated TCI state, i.e., CandidateTCI-State/CandidateTCI-UL-State whose reference signal has the same RS ID as the RS corresponding to the selected beam, should be deactivated with the following timeline: - Opt.1: upon RACH-less CLTM procedure being triggered. - Opt.2: Once RACH-less CLTM procedure is completed. FL assessment: Opt.2 might be a more appropriate approach for handling the C-LTM failure scenario, since the TCI states remain active until the procedure is completed, rather than deactivated once C-LTM is triggered. |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate your support: Yes, No, or specify the preferred option.) | Comments (If a particular scheme is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon |  | Depends on the answer in 4-1 |

## Issue 4-2: TA value determination

At the RAN1 122 meeting [17], a concern was raised regarding how the Timing Advance (TA) value should be determined for Conditional LTM (C-LTM), and it was agreed that further investigation was needed. Three companies—[HW,3], [Sharp,14], and [Apple,6]—contributed to the discussion. There appears to be consensus that the UE initiates the process by reading the configuration of the target candidate cell to obtain the subcarrier spacing (SCS), which is then used to interpret the TA value provided by the network for the initial uplink transmission—mirroring the approach used in Rel-18 non-conditional LTM.

Nonetheless, a TP to TS 38.213 remains necessary to address the newly introduced MAC Control Element (MAC CE) specific to C-LTM, as the Cell-switch command MAC CE is no longer applicable in this context.Three draft change requests have been submitted [HW,3], [Sharp,14] and [Apple,6]—with the one from [HW,3] being the most thorough and complete. As such, FL recommends using HW’s TP as starting point and check for acceptance.

Text proposal #4-2:

Summary of change: Define TA application time for CLTM when TA value included in the LTM Candidate Timing Advance Command MAC CE is received by UE.

Consequences if not approved: The TA application time for CLTM when TA value included in the LTM Candidate Timing Advance Command MAC CE is received by UE is unclear.

4.2 Transmission timing adjustments < Unchanged parts are omitted > A timing advance command [11, TS 38.321] in case of random access response or in an absolute timing advance command MAC CE or in a cell switch command or in LTM Candidate Timing Advance Command MAC CE, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTA is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command or the first uplink transmission from UE after RACH-less CLTM cell switch is performed. …… For a timing advance command received on uplink slot n, except for a timing advance command received in a cell switch command or LTM Candidate Timing Advance Command MAC CE, and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot n+k+1+2μ∙Koffset where k=Nslotsubframe, μ∙NT,1+NT,2+NTA,max+0.5Tsf, NT,1 is a time duration in msec of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration in msec of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], NTA,max is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, Nslotsubframe, μ is the number of slots per subframe, Tsf is the subframe duration of 1 msec, and Koffset=Kcell,offset-KUE,offset, where Kcell,offset is provided by cellSpecificKoffset and KUE,offset is provided by a Differential Koffset MAC CE command [11, TS 38.321]; otherwise, if not respectively provided, Kcell,offset=0 or KUE,offset=0. N1 and N2 are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. Slot n and Nslotsubframe, μ are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. NTA,max is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot n is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211]. < Unchanged parts are omitted > 21 L1/L2-triggered mobility procedures < Unchanged parts are omitted > ….. If ltm-UE-MeasuredTA-ID of a candidate cell and ltm-ServingCellUE-MeasuredTA-ID of the serving cell are provided to a UE and have same value, the UE estimates based on the UE implementation a timing advance to apply from a first transmission on the candidate cell that is after the reception of a cell switch command for the candidate cell when the condition defined in clause 5.18.35 of [11, TS 38.321] is satisfied or to apply from a first transmission on the CLTM target cell when the condition defined in clause 5.y.3 of [11, TS 38.321] is satisfied. < Unchanged parts are omitted >

| Moderater Question 4-2-1: Is the TP 4-2 above is acceptable? |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon | Y |  |

5. LS on early CSI acquisition for L3 handover

RAN2 discussed on the intention to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework for LTM. RAN2 discussed the RRC CR [29] agreed that the RRC CR is technically correct. However, RAN2 postponed the CR pending confirmation from RAN1.

In the LS [30], RAN2 provided the following information.

RAN2 discussed on the intention to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework for LTM. RAN2 discussed the RRC CR (R2-2506450) and agreed that the RRC CR is technically correct. However, RAN2 postponed the CR pending confirmation from RAN1.

RAN2 identified at least the following potential impacts to RAN1:

- TS 38.214: Describe CSI reporting for L3 handover which will be similar to the description in clause “5.2.4a CSI Reporting for LTM”.

- TS 38.212: Add a reference to TS 38.214 in clause “6.3.2.1.2 CSI”.

RAN2 assumes there is no RAN4 work.

RAN2 discussed on the intention to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework for LTM. RAN2 discussed the RRC CR (R2-2506450) and agreed that the RRC CR is technically correct. However, RAN2 postponed the CR pending confirmation from RAN1.

RAN2 identified at least the following potential impacts to RAN1:

- TS 38.214: Describe CSI reporting for L3 handover which will be similar to the description in clause “5.2.4a CSI Reporting for LTM”.

- TS 38.212: Add a reference to TS 38.214 in clause “6.3.2.1.2 CSI”.

RAN2 assumes there is no RAN4 work.

Contributions [20–28] examined the potential impact of introducing early CSI reporting to support L3 handover. Among these, five companies—[ZTE, 21], [SPRD, 24], [CATT, 25], [Ericsson, 27], and [Huawei, 28]—expressed support for specifying this feature in Rel-19. Conversely, two companies—[vivo, 20] and [Samsung, 26]—raised concerns citing unclear operations and significant implications for RAN1 specifications. One company [Nokia, 23] propose to defer the LS reply until the relevant issues commonly for LTM and L3 handover are addressed.

In [Samsung, 26], several issues related to early CSI for L3 handover were identified. FL plans to use these as a starting point in RAN1 to determine which early CSI designs in LTM can be reused for L3 handover and what new components need to be introduced.

| Moderater Question 5-1: Early CSI measurement in LTM can be performed before or afte LTM CSC MAC-CE. Two possible options exist for enabling early CSI for L3 handover: - Opt.1: For L3 handover, early CSI measurement is conducted only after receiving ReconfigurationWithSync message. - Opt.2: For L3 handover, early CSI measurement can be performed before and after receiving ReconfigurationWithSync message, same as LTM. |  |  |
| --- | --- | --- |
| Company | View/Positions (Please indicate Option) | Comments |
| FL initial assement |  | FL’s understanding is Opt.1 as CSI measurement resource is provided in the same ReconfigurationWithSync message and not earlier configured. |
| Huawei, HiSilicon | Option1 | The configuration is in ReconfigurationWithSync. UE will not perform measurement before handover command. |

| Moderater Question 5-2: A set of rules has been defined for LTM to identify the first PUSCH resource for transmitting the early CSI report.Can the PUSCH determination rules defined for LTM be applied to L3 handover as well, or is there a need to specify new rules? |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments |
| Huawei, HiSilicon | Yes | RAN2’s intention is to reuse the LTM rule. |

| Moderater Question 5-3: Do we still reuse ‘invalid’ CSI report for L3 handover case? |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments |
| Huawei, Hisilicon | Yes | As the CSI reporting is in UCI, the invalid CSI is still needed for UE can not finish measurement or preparing UCI. |

[ZTE, 22] proposed to replace the current single “NZP-CSI-RS/CSI-IM resource” for CSI measurement associated with a CSI reporting configuration by a “NZP-CSI-RS/CSI-IM resource set” for reliable CQI report.

| Moderater Question 5-4:For L3 handover, do you support to replace the current single “NZP-CSI-RS/CSI-IM resource” for CSI measurement associated with a CSI reporting configuration by a “NZP-CSI-RS/CSI-IM resource set”? |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments |
| Huawei, HiSilicon | Yes | We had similar proposal in our tdoc |

Two TPs are included in the joint contributions from [Huawei/Ericsson, 28] and [ZTE, 22]. Companies are encouraged to review these documents, as they may clarify the specific impact on the RAN1 specification and help assess its feasibility for support at this stage.

| Moderater Question 5-5: Which TP from [Huawei/Ericsson, 28] and [ZTE, 22] is preferred, if early CSI report to be supported for L3 handover? Any additional changes you forseen to support this feature? |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments (if you see any additional changes you forseen to support this feature?) |
| Huawei, HiSilicon |  | The TP in [28] is for 214. There is another joint TP for 212 in R1-2507404 |

6. Others aspects

In last meeting of RAN2, it is agreed that coexistence of SBFD and intra-DU LTM is supported [18] as captured below.

- Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI.

A contribution [Ofinno, 11] examined the potential specification implications arising from the combined operation of the LTM and SBFD features, and subsequently proposed the following TP:

Text proposal # 6-1:

Reason for change: Current specification does not clarify how an SBFD-aware UE shall derive CSI reports for LTM when one of the candidate cells operates SBFD. Symbol type configuration for LTM CSI reports is not specified. Summary of change: Update TS 38.214 to clarify that for LTM CSI reporting, the symbol type shall be explicitly configured, and the UE shall follow the same SBFD-aware behavior as for other CSI reports. Consequences if not approved: ### UE may derive and report CSI using resources with mismatched symbol types, potentially leading to incorrect cell measurements and degraded mobility performance.

==================================unchanged omitted=================================== #### 5.2.1.1 Reporting settings Each Reporting Setting CSI-ReportConfig is associated with a single downlink BWP (indicated by higher layer parameter BWP-Id) given in the associated CSI-ResourceConfig for channel measurement and contains the parameter(s) for one CSI reporting band: codebook configuration including codebook subset restriction, time-domain behavior, frequency granularity for CQI and PMI, measurement restriction configurations, and the CSI-related quantities to be reported by the UE such as the layer indicator (LI), L1-RSRP, L1-SINR, CRI, SSBRI (SSB Resource Indicator), CapabilityIndex, TDCP, L1-SRS-RSRP, L1-CLI-RSSI, SRS-RSRP-MRI, CLI-RSSI-MRI, CSI-PAI, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI, CJTC-Dd, CJTC-F, CJTC-Dd-F and CJTC-P. Each Reporting Setting ltm-CSI-ReportConfig is associated with a LTM-CSI-ResourceConfig for channel measurement and contains the parameters(s) for time-domain behavior provided by ltm-ReportConfigType, the number of cells and the number of reference signals per candidate cell provided by nrOfReportedCells, and nrOfReportedRS-PerCell, respectively, when ltm-ReportConfigType set to ‘periodic’ or ‘semiPersistentOnPUCCH’ or ‘semiPersistentOnPUSCH’ or ‘aperiodic’, comprising L1 measurement results associated with current SpCell if spCellInclusion is configured, and the CSI-related quantities to be reported by the UE provided by reportQuantity, if configured. If a UE is configured with SBFD symbols for a serving cell and a ltm-CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) of the corresponding LTM candidate cell(s) as indicated by symbolType for the CSI derivation for the CSI report. ==================================unchanged omitted===================================

| Moderater Question 5: Is the TP #6-1 above is acceptable? |  |  |
| --- | --- | --- |
| Company | View/Positions (Yes or No) | Comments (If a TP is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| Huawei, HiSilicon | N | It is out of scope of WID |

7.	Other Text Proposals (TPs)

This section contains a few text proposals—either editorial or straightforward technical proposals—intended to improve the clarity of the specification. FL has listed them to gather company feedback.

As noted in earlier sections, certain TPs from company contributions are not included here, as the related proposal must first be reviewed and concluded before those TPs can be addressed.

Additionally, certain TPs were already discussed during the previous RAN1 meeting, where specific concerns were explicitly raised as documented in FL summary of RAN1 122 meetiing [19]. FL recommend not to re-submit them in the future unless concerns were addressed. These include:

- The inclusion of SpCell in the CSI report from [Samsung, 8].

- Determination of CSI-RS resource for measurement after receiving CSC MAC CE [ZTE, 4]

These TP will not be treated unless the proposing companies resolve the concerns offline and update FL on the outcome.

TP #7-1: CQI/RI/PMI calculation for early CSI Acquisition for a Candidate Cell [Nokia, 5]

Reason for change

As per the current specification, for RACH-based LTM cell switch using a contention-based 4-step random access procedure, the UE transmits the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to Msg4. However, in LTM with ReconfigurationWithSync with a 4-step RACH, the UE sends a C-RNTI MAC CE and receives a PDCCH with an UL grant as Msg4. In this case, there is no HARQ-ACK for Msg4. Therefore, the specification should be corrected for this scenario.

Summary of change

For RACH-based LTM cell switch using a contention-based 4-step random access procedure, specify that the CSI report is transmitted using the first PUSCH scheduled by the PDCCH transmission received in response to Msg3.

Consequences if not approved

Incorrect handling of CSI reporting for RACH-based LTM cell switch using a contention-based 4-step random access procedure.

5.2.4a	CSI Reporting for LTM

<omitted Text>

For RACH-less LTM cell switch [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant [6, TS 38.213].

For RACH-based LTM cell switch using a contention-free random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the PUSCH scheduled by the RAR UL grant or MsgA PUSCH.

For RACH-based LTM cell switch using a contention-based random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to Msg4 or MsgB, or the first PUSCH scheduled by a PDCCH transmission addressed to the C-RNTI, received in response to Msg3.

<omitted Text>

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| FL initial assement |  | FL understands that HARQ-ACK is transmitted for Msg4 since it is scheduled via a C-RNTI PDCCH. If this interpretation is correct, then the proposed change request is not valid. |
| Huawei, HiSilicon | Yes | We agree with Nokia that CBRA in HO do not have msg4 and ACK. After receiving msg3, gNB will directly give a UL grant for msg5. |

TP #7-2: ‘Repetition’ Setting for LTM-NZP-CSI-RS-ResourceSet [vivo, 2]

| Reason for change | In the last meeting, RRC parameter of repetition was introduced to LTM-NZP-CSI-RS-ResourceSet. However, in the TS38.214, the parameter is captured as per NZP-CSI-RS resource. It is not aligned with the agreement. Furthermore, the parameter only present for beam management is not captured in the current specification. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.2, clarify when an LTM CSI Report Setting configured with report quantity set to ‘cri-rsrp’, the LTM-NZP-CSI-RS-ResourceSet included in the LTM CSI Resource Setting associated with the LTM CSI Report Setting should be configured with ‘repetition’ set to ‘off’. |
| Consequences if not approved | The specification description is not aligned with agreement. |
| TS38.213 5.2.1.2 Resource settings <unchanged parts are omitted> Each LTM CSI Resource Setting LTM-CSI-ResourceConfig contains either configuration of a ltm-CSI-SSB-ResourceSet or a ltm-NZP-CSI-RS-ResourceSet. - A ltm-CSI-SSB-ResourceSet comprises of a list of Z ≥ 1 SS/PBCH blocks indices (given by ltm-CSI-SSB-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the SS/PBCH block indices. For each candidate cell, the UE determines the time domain behavior of a SS/PBCH block from ssb-Periodicity and ssb-PositionsInBurst and the frequency domain behavior of a SS/PBCH block is determined by the higher layer parameters subcarrierSpacing, ssb-Frequency. - A ltm-NZP-CSI-RS-ResourceSet comprises of a list of Z ≥ 1 NZP CSI-RS resource indices (given by ltm- CSI-RS-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the NZP CSI-RS resource indices. The UE shall expect that the NZP CSI-RS resources in ltm-NZP-CSI-RS-ResourceSet are configured with the higher layer parameter repetition set to ‘off’ when LTM-ReportContent configured within the LTM-CSI-ReportConfig associated with the LTM CSI Resource Setting is set to ‘cri-RSRP’. For a report setting ltm-CSI-ReportConfig configured with ltm-ReportConfigType set to ‘periodic’ or ‘semiPersistentOnPUCCH’ or ‘semiPersistentOnPUSCH’ or ‘aperiodic’, the time domain behavior of the NZP CSI-RS resources within a ltm-NZP-CSI-RS-ResourceSet are indicated by the higher layer parameter resourceType. <unchanged parts are omitted> |  |

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |

TP #7-3:  Active CSI-RS Resource and Ports Counting [Samsung, 8]

Reason for change [from contribution]:

In [1], UE’s assumptions on active resource counting before receiving the LTM CSC were agreed and captured in the TS 38.214. However, corresponding descriptions in the TS 38.214 repeatedly characterize legacy condition(s) for active resource counting, which we do not think are necessary or accurate (though the corresponding agreement made in [1] includes components related to legacy condition(s), we understand from the discussions at that time that these components are just for information and descriptive purposes). For instance, for periodic CSI-RS, when the corresponding configuration is released, the resource counting anyways will end; for semi-persistent CSI-RS, when the corresponding deactivation command is applied, the resource counting ends regardless. We only need to clarify or specify that the resource counting is up to reception of the LTM CSC – whatever in legacy is still kept intact. Based on the above, we provide the following TP#4:

38.214-j10

5.2.1.6	CSI processing criteria

<Unchanged parts are omitted>

For a report setting ltm-CSI-ReportConfig configured with reportQuantity is set to 'cri-RI-PMI-CQI':

-	If the UE is capable of performing CSI measurement for CSI acquisition for candidate cell(s) before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]:

-	For a periodic CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting when the periodic CSI-RS resource is configured by higher layer signaling until up to the reception of the LTM cell switch command MAC CE or a RRC reconfiguration message, whichever occurs first.

-	For a semi-persistent CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting from the end of when the activation command MAC CE is applied until up to the reception of the LTM cell switch command MAC CE or a deactivation command MAC CE, whichever occurs first.

-	If the UE is only capable of performing CSI measurement for CSI acquisition for a candidate cell (given by Target Configuration ID field in the LTM Cell Switch Command MAC CE) after receiving the LTM Cell Switch Command MAC CE [10, TS 38.321], no CSI-RS resource and CSI-RS ports are counted as active before the reception of the LTM Cell Switch Command MAC CE [10, TS 38.321].

<Unchanged parts are omitted>

38.214-j10

5.2.1.6	CSI processing criteria

<Unchanged parts are omitted>

For a report setting ltm-CSI-ReportConfig configured with reportQuantity is set to 'cri-RI-PMI-CQI':

-	If the UE is capable of performing CSI measurement for CSI acquisition for candidate cell(s) before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]:

-	For a periodic CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting when the periodic CSI-RS resource is configured by higher layer signaling until up to the reception of the LTM cell switch command MAC CE or a RRC reconfiguration message, whichever occurs first.

-	For a semi-persistent CSI-RS, the CSI-RS resource and CSI-RS ports within the CSI-RS resource are counted as active in a duration of time starting from the end of when the activation command MAC CE is applied until up to the reception of the LTM cell switch command MAC CE or a deactivation command MAC CE, whichever occurs first.

-	If the UE is only capable of performing CSI measurement for CSI acquisition for a candidate cell (given by Target Configuration ID field in the LTM Cell Switch Command MAC CE) after receiving the LTM Cell Switch Command MAC CE [10, TS 38.321], no CSI-RS resource and CSI-RS ports are counted as active before the reception of the LTM Cell Switch Command MAC CE [10, TS 38.321].

<Unchanged parts are omitted>

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Huawei, HiSilicon | N | The periodic RS or activated SP RS can be released or deactived before CSC. the deleted part is needed. |

TP #7-4:CSI-RS Resources Setting for Early CSI [Samsung, 8]

Reason for change [from contribution]:

It was agreed in RAN1 that for CSI acquisition in LTM, the list of NZP CSI-RS resources for channel/interference measurements are associated with a list of candidate cells, and the list of NZP CSI-RS resources and the list of candidate cells are provided in the same resource set. When two resource settings are configured, the same design should be applied to CSI-IM resources as well. Given the above, we present the following TP:

38.214-j10

5.2.4a 	CSI Reporting for LTM

<Unchanged parts are omitted>

The Resource Setting given by higher layer parameter ltm-ResourcesForInterferenceMeasurement, contains configuration of a ltm-CSI-IM-ResourceSet which comprises of a list of Z ≥ 1 CSI-IM resource indices (given by ltm-CSI-IM-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the CSI-IM resource indices. For CSI acquisition associated with a Reporting Setting, ltm-CSI-ReportConfig, the UE is expected to measure the CSI-IM resources in ltm-CSI-IM-ResourceList associated with the LTM-CandidateId that is equal to the LTM-CandidateId of the LTM-Candidate under which the Reporting Setting is configured.

<Unchanged parts are omitted>

38.214-j10

5.2.4a 	CSI Reporting for LTM

<Unchanged parts are omitted>

The Resource Setting given by higher layer parameter ltm-ResourcesForInterferenceMeasurement, contains configuration of a ltm-CSI-IM-ResourceSet which comprises of a list of Z ≥ 1 CSI-IM resource indices (given by ltm-CSI-IM-ResourceList) and a list of Z LTM-CandidateIds (given by ltm-CandidateIdList) referring to candidate cells associated with the CSI-IM resource indices. For CSI acquisition associated with a Reporting Setting, ltm-CSI-ReportConfig, the UE is expected to measure the CSI-IM resources in ltm-CSI-IM-ResourceList associated with the LTM-CandidateId that is equal to the LTM-CandidateId of the LTM-Candidate under which the Reporting Setting is configured.

<Unchanged parts are omitted>

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Huawei, HiSilicon | N | It will change the RRC parameters. The current CSI-IM do not have a list of candidate ID. |

TP #7-5: CSI report for LTM [CATT, 7]

| Reason for change: | In RAN1 #120 meeting, it has been agreed that the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability. The agreement is as follows: Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability.. This should be captured in TS38.214. |
| --- | --- |
| Summary of change: | Remove the square brackets enclosing “depending on the UE capability” from the following sentence. [Depending on the UE capability] the UE may start measuring corresponding NZP CSI-RS resources before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321]. |
| Consequences if not approved: | The condition of UE measuring CSI based on the configured CSI-RS resource(s) before the reception of LTM Cell Switch Command MAC CE is not aligned with the agreement. |

-------------------------------------------- Start of text proposal to TS 38.214 v19.0.0 ----------------------------

5.2.4a CSI Reporting for LTM

….

For RACH-based LTM cell switch using a contention-based random access procedure [23, TS 38.300], the UE shall transmit the CSI report to the candidate cell using the first PUSCH corresponding to a dynamic grant or a configured grant after the HARQ-ACK transmission corresponding to Msg4 or MsgB.

If a valid CSI is not available, the UE shall transmit a CSI report which contains a CQI corresponding to the lowest CQI index. [Depending on the UE capability], the UE may start measuring corresponding NZP CSI-RS resources before receiving the LTM Cell Switch Command MAC CE [10, TS 38.321].

….

-------------------------------------------- End of text proposal to TS 38.214 v19.0.0 ---------------------------

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Huawei, HiSilicon | N | Already resolved in 19.1.0 |

TP #7-6: CSI report for LTM [Lenovo, 10]

Reason for change: 1) CSI-IM resources measurement is missed for the UE behavior on determining the resources for channel/interference measurement based on the CSC MAC CE. 2) If ltm-eCSI-ReportConfig is used for CSI report configuration for CSI acquisition, priority for CSI acquisition should be specified. 3) Early DL transmission for LTM in FR2 cannot be supported without LI reporting in the early CSI for candidate cell.

Summary of changes: ltm-eCSI-ReportConfig is replaced by ltm-CSI-ReportConfig; 3) reportQuantity for CSI acquisition can be set to ‘cri-RI-PMI-LI-CQI’.

Consequences if not approved: 1) The UE behavior on the case that CSI-IM is configured is not clear. 2) If ltm-eCSI-ReportConfig is used for CSI report configuration for CSI acquisition, priority for CSI acquisition should be specified. 3) Early DL transmission for LTM in FR2 cannot be supported without LI reporting in the early CSI for candidate cell.

<omitted text> 5.2.4a CSI Reporting for LTM A UE configured with LTM-Config can be provided configurations for CSI acquisition, by up to one Reporting Setting, [ltm-CSI-ReportConfig], for a candidate cell. Each Reporting Setting [ltm-CSI-ReportConfig] is associated with either one or two Resource Settings - When one Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) is configured, it provides a list of NZP CSI-RS resources for both channel and interference measurements. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Setting. - When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter ltm-ResourcesForChannelMeasurement) provides a list of NZP CSI-RS resources for channel measurement, and the second Resource Setting (given by higher layer parameter [ltm-ResourceForInterferenceMeasurements]), provides a list of [CSI-IM resources] for interference measurement. The UE is not expected to be configured with more than 128 NZP CSI-RS ports in the CSI-RS resource set contained within the Resource Settings [The UE shall expect the following configuration provided by [ltm-CSI-ReportConfig]: - For the frequency granularity of the CSI report, the CQI format indicator is Wideband CQI. - For the frequency granularity of the CSI report, the PMI format indicator is Wideband PMI. - The codebook type is typeI-SinglePanel. - The reportQuantity is set to ‘cri-RI-PMI-CQI’ or ‘cri-RI-LI-PMI-CQI’.] After a UE receives an LTM Cell Switch Command MAC CE [10, TS 38.321] providing a candidate cell (given by Target Configuration ID field), and a [ltm-eCSI-ReportConfig] is configured for the candidate cell, the UE can measure corresponding NZP CSI-RS resources and/or CSI-IM resources and shall transmit a CSI report to the candidate cell. <omitted text>

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Huawei, HiSilicon | N | ltm-eCSI-ReportConfig is fixed in 19.1.0 change the report quantity is not suitable in maintenance phase. |

TP #7-7: CSI report for LTM [Lenovo, 10]

Reason for change: The time domain behavior for the CSI report for LTM-CSI-ReportConfig is configured by ltm-ReportConfigType other than reportConfigType. But it is not considered for the semi-persistent reporting on PUCCH in Clause 5.2.1.5.2 of TS38.214.

Summary of changes: Involve ltm-ReportConfigType for semi-persistent reporting on PUCCH in TS38.214.

Consequences if not approved: Semi-persistent reporting on PUCCH for CSI report for LTM-CSI-ReportConfig is not captured by TS38.214.

<omitted text> 5.2.1.5.2 Semi-persistent CSI/Semi-persistent CSI-RS … For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType or ltm-ReportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent reporting settings for use by the UE on the PUCCH. For a selected reporting setting for which the CSI-ReportConfig contains a list of sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList, the activation command can also select one or more sub-configurations to use by the UE as described in clause 6.1.3.X of [10, TS 38.321]. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. … <omitted text>

| Company | View/Positions (Please indicate your support: Yes, No in this column) | Comments (If a TP text is generally acceptable but requires adjustments to the specific wording, please suggest revised phrasing in the ‘comments’ column.) |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

7. Proposals for Monday’s Online Discussion

<FFS>

References

R1-2506800	Remaining issues on measurements related enhancements for LTM	Spreadtrum, UNISOC

R1-2506880	Maintenance on NR Mobility Enhancement Phase 4	vivo

R1-2506942	Measurements related enhancements for LTM	Huawei, HiSilicon

R1-2507037	Maintenance on Mobility Phase 4	ZTE Corporation, Sanechips

R1-2507068	Maintenance on measurement related enhancements for LTM	Nokia

R1-2507655	Maintenance on NR mobility enhancements Phase 4	Apple

R1-2507125	Maintenance on measurements related enhancements for LTM	CATT

R1-2507235	Maintenance on other Rel-19 topics		Samsung

R1-2507355	Remaining issues on measurements related enhancements for LTM	LG Electronics

R1-2507437	Maintenance on the measurements for LTM	Lenovo

R1-2507459	Maintenance on mobility enhancement phase 4	Ofinno

R1-2507474	Maintenance on NR mobility enhancements Phase 4	Ericsson

R1-2507578	Maintenance on Mobility Phase 4	Google

R1-2507873	Maintenance on measurements related enhancements for LTM	Sharp

R1-2507656	FL Summary #1 of NR Mobility enhancement Phase 4	Moderator (Apple)

R1-2507657	FL Summary #2 of NR Mobility enhancement Phase 4	Moderator (Apple)

Chairman note of RAN1 122 Meeting

R2-2506202, ”Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others”, RAN2 Vice Chairman (CATT)

R1-2505885	FL Summary #1 of NR Mobility enhancement Phase 4	Moderator (Apple)

R1-2506866	Discussion on early CSI acquisition for L3 handover	vivo

R1-2507043	Draft reply LS to RAN2 on early CSI acquisition for L3 handover	ZTE Corporation, Sanechips

R1-2507044	Discussion on early CSI acquisition for L3 handover	ZTE Corporation, Sanechips

R1-2507069	Discussion on LS on support for early CSI acquisition for L3 handover	Nokia

R1-2507070	Draft reply LS on early CSI acquisition for L3 handover	Spreadtrum, UNISOC

R1-2507082	Draft reply LS on early CSI acquisition for L3 handover	CATT

R1-2507219	Discussion on RAN2 LS on early CSI acquisition for L3 handover		Samsung

R1-2507348	Discussion on LS on early CSI acquisition for L3 handover	Ericsson

R1-2507403	Introduction of Rel-19 early CSI acquisition for L3 handover to TS 38.214 [EarlyCSI_L3HO]			Huawei, HiSilicon, Ericsson

R2-2506450, Support early CSI acquisition for L3 handover [EarlyCSI_L3HO], RAN2#131, August 2025

R1-2506716, LS on early CSI acquisition for L3 handover, RAN2, 3GPP TSG RAN WG1#122bis, October 2025