3GPP TSG RAN WG2 Meeting #131bis         	                             R2-2506775

Prague, CZ, Oct 13th – 17th, 2025

Agenda Item:	10.3.3

Source:	Xiaomi

Title:	Consideration on 6GR data transfer, AI/ML framework and security

Document for:	Discussion and Decision

# Introduction

Over the past releases, data collected and transferred within 3GPP network mainly focuses on measurement data for necessary communication and network operation and management. In recent years, together with the growth of AI/ML, the need of data has been explosively increased. To support AI/ML air use cases, RAN has studied and/or supports data collection and transfer for different use cases, including network-sided model, UE-sided model and two-sided model.

In 6GR, as captured in [2], data becomes one of the most important study components of 3GPP RAN.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

During RAN #109 meeting [2], the following services have been agreed as 6GR requirements of new and existing services, including services such as AI, sensing, etc, which will generate tremendous amount of data. Supporting data collection and transfer for new services becomes critical in 6GR.

It is agreed to capture following into TR 38.914 section 5.4: - Mobile broadband - Immersive Communication - Massive Communication (IoT) - Sensing - AI - Voice - Regulatory services

Additionally, different from AI/ML supported in 5GA, as a new type of service, 6GR should integrate AI natively within the network architecture.

Last but not least, to support existing and new services, security of the network should always be supported.

In this contribution, we discussed three aspects: 1) 6GR data type and data transfer; 2) AI/ML framework; 3) security aspects.

# Discussion

## Data Collection and Transfer

### 6GR Data Collection Principles

During past generations, data collection and transfer is mainly used for the purpose of network management and optimization (e.g., MDT) and optimizing user experience (e.g. QoE). Later on, to support UE-side model training, UE needs to collect and transfer the training data to its training server over-the-top. The visibility of collected data has been extensively discussed during Rel-18 and Rel-19 without reaching any consensus. The main reason of not supporting network visibility of collected data for UE-side model training is due to user privacy.

According to the IDC survey, almost 50ZB data are expected to be generated in 2025 from various services, e.g., autonomous driving, AI agent, low-altitude economy, etc. However, along with the big bang of generated data, data leakage becomes more and more severe in recent years. As reported by [Verizon 2024 Data Breach Investigations Report], there were 10,626 data leakage events reported among 30,458 security events world-wide, with 50% increasement compared with 2023 (5,199 data leakage events among 16,312 security events). To avoid unintentional data leakage from 3GPP network, for any data collected from the end user, user privacy should be respected. To be more specific, to reduce the risk of data leakage, reducing intermediate nodes inside 3GPP network storing and processing the collected data from user for UE-side use cases is necessary.

Last but not least, since UE is involved in all data collection procedures, same as 5GNR, user consent should be respected for different purposes of data collection. RAN2 is expected to further coordinate with SA WGs once RAN2 has a clear understanding of what data types need to be supported in 6GR.

Lesson learnt from 5G (1): Exposing data collected from UE at intermediate nodes inside 3GPP network increases the risk of data leakage, further lead to user privacy concern.

To support minimization of drive test, two MDT modes are supported since LTE, i.e., logged MDT and immediate MDT, where logged MDT is mainly focusing on measurement data collection while UE is in RRC_IDLE/INACTIVE state, and immediate MDT mainly reuses RRM measurement for RRC_CONNECTED mode UEs. As supported in the past generations, SON/MDT related data are collected from UE and further gathered at RAN and/or OAM. There are mainly five types of data collected by SON/MDT for different usages:

Table 1. SON/MDT Use Cases

| Producer | Consumer | Collected Data | Usage |
| --- | --- | --- | --- |
| UE or RAN | RAN and/or OAM | RRM measurements (e.g., RSRP, RSRQ, SINR, etc) | Coverage optimization, mobility management, scheduling optimization |
|  |  | UE location information (GNSS, cell ID, TAI, etc) | Accurate coverage analysis, mobility optimization |
|  |  | Connected status (e.g., RRC state, PLMN, etc) | Network resource management, roaming |
|  |  | Event reporting (e.g., RLF, HOF, CEF, etc) | Coverage, configuration optimization |
|  |  | Service Quality (e.g., throughput, packet delay, BLER, etc) | Optimization for services (e.g., throughput, QoS, reliability, quality, etc) |

On the one hand, SON/MDT data collection are mainly used by network management for optimization (e.g., coverage, resource scheduling, mobility, etc). On the other hand, data collection related to service quality (e.g., throughput, packet delay, etc) can also be beneficial to UE, as network should at least guarantee the QoS requirement for different types of services. Though we understand those data are useful for network optimization and network awareness of service quality, SON/MDT data collection should only collect UE available data whenever possible, instead of requesting UE to perform additional measurement, which may cost further extra power consumption at the UE side.

In Rel-19, network-side data collection for NW-side model is also designed with a minimum UE effort. For example, the UE can optionally perform logging for NW-side data collection, to reach a balance between UE memory cost and power consumption caused by frequent measurement reporting.

Lesson Learnt from 5G (2): Consideration of UE efficient involvement is already supported in 5GNR, e.g., UE reports SON/MDT reports whenever available, UE optionally logging for NW-side data collection, etc.

To support different purposes of data collection, e.g., MDT, AI/ML training, inference and monitoring, RRM, etc, the UE is configured by network with many duplicated configurations for similar measurement and reporting:

- L3 measurement:

  - Logged MDT configuration via LoggedMeasurmentConfiguration and report via UEInformationResponse;

  - Immediate MDT configuration via RRCReconfiguration (reusing RRM measurement framework) and report via MeasResults;

  - Mobility measurement configuration via MeasConfig and report via MeasResults.

- CSI measurement:

  - CSI feedback configuration via CSI-MeasConfig and report via CSI framework;

  - AI/ML inference configuration via CSI-ReportConfig with specific reportQuantity and report via CSI framework;

  - AI/ML network-side data collection configuration via CSI-LoggedMeasurementConfig and report via UEInformationResponse;

  - AI/ML UE-side data collection configuration via CSI-ReportConfig with specific reportQuantity for UE-side data collection, and report via CSI framework.

Moreover, Quality of Experience (QoE) was introduced to collect data for optimizing service quality based on users’ subjective experience from application layer, including video, gaming, etc. In LTE, QoE is only visible in application layer and 3GPP network (including RAN) is only used as the data transfer tunnel. In 5GNR, RAN visible QoE (RVQoE) is introduced to allow RAN to optimize radio resources based on application layer service quality. However, the UE needs to report the same QoE value(s) to both application layer (as container which terminated at RAN node first) and Uu interface (as separate parameters, e.g., appLayerBufferLevel, playoutDelayForMediaStartup).

Of course, if the data consumer is different, it would be difficult to use the same configuration and reporting signalling. However, as long as the data transfer tunnel (termination of data transfer) are the same, the duplicated measurement and reporting not only leads to a waste of air interface resources, but also increases UE complexity and power consumption. In 6GR, such duplicate measurement and reporting should be avoided from Day 1.

Lesson Learnt from 5G (3): Even with the same termination, UE has been requested to send duplicate reports over the air interface for the same measurement in 5GNR.

Till Rel-19, a UE needs to support almost a minimum 256kB memory if the UE wants to support features, including:

- Logged MDT (a minimum 64kB memory);

- QoE (a minimum 64kB memory for CONNECTED, another minimum 64kB for IDLE/INACTIVE);

- AI/ML network-side data collection (a minimum 64kB memory).

However, this minimum requirement significantly increases the cost at UE device when supporting multiple features at the same time. On the other hand, depends on different UE implementations, the UE should be allowed to support larger memory size as an optional feature. Within the supported memory size, it is up to UE implementation about how to use the memory for different purposes.

Lesson Learnt from 5G (4): In 5GNR, UE has been requested to support multiple minimum memory sizes for different use cases, which increases the cost at UE side.

Proposal 1: 6GR data collection and transfer design should follow the below principles:

- Respect user privacy and user consent for different types of data collection. Coordination with SA WGs is expected;

- Collected data should only be visible to the data end consumer. Avoid storing/processing/terminating collected data at intermediate nodes within 3GPP network;

- Respect UE’s memory and power consumption cost, including efficient UE participation (e.g., only collect UE available data whenever possible without additional UE measurement), avoid duplicate measurements (for same producer/consumer), reasonable memory requirement, etc.

### Data Types

With the diverse service growth, explosive data growth brings the requirement for network to collect and transfer data supporting various services in 6GR. Services which need data collection and transfer includes the following cases:

- Data for network operation and management optimization (e.g., SON/MDT);

- Data for user experience improvement (e.g., QoE);

- Data for enabling new service(s) (e.g., AI/ML, sensing, etc).

Existing Services - SON/MDT, QoE

As discussed in earlier sections, SON/MDT and QoE data collection and transfer have been studied in previous releases. For those features, based on 5GNR experience, RAN2 already has a good understanding of data types to be collected for those existing data collection services.

Lesson Learnt from 5G (5): Data types for existing data collection services (e.g., SON/MDT, QoE) has been well-established in 5GNR, which can be considered as starting point for 6GR study.

In 6GR, another key objective is to support data collection and transfer for new types of services (e.g., AI/ML and sensing).

New Service – AI/ML

During 5GNR study on AI/ML air interface, data collection for UE-side model, NW-side model and two-sided model has been studied in RAN1/2. The below table summarizes the key data types and their corresponding producer and consumer based on RAN1 LS in 5GA [3][4][5].

Table 2. Typical Data Types of 5GA AI/ML air use cases

| Use Case(s) | LCM purpose | Producer | Consumer | Typical data size (per data sample) for Transfer over air interface | Typical latency requirement |
| --- | --- | --- | --- | --- | --- |
| UE-sided model (e.g., CSI prediction, AI/ML beam management) | Training | UE | UE-side OTT server | Up to 1.5Mbits assuming float 32 and 10 CSI-RS observation instances; The total size is up to UE implementation how to collect/store the collected data. | Relaxed (to be specified in 5GA R20) |
|  | Inference | UE | UE | N/A | N/A |
|  | Monitoring | UE | gNB | Same as CSI feedback size (~ a few 1000 bits) | Near-real-time (L1 report in 5GA R19) |
| NW-sided model (e.g., AI/ML beam management) | Training | UE | Training entity for NW-side model | Maximum 64kB depends on when NW retrieves logged data from UE | Relaxed (L3 report in 5GA R19) |
|  | Inference | UE | gNB | Size of L1 measurement reports | Time-critical (L1 report in 5GA R19) |
|  | Monitoring | UE | gNB | Size of L1 measurement reports | Near-real-time (L1 report in 5GA R19) |
| Two-sided model (e.g., CSI compression) | Training | gNB | UE-side OTT server | Option 4-1: 225MB Option 3a-1 without target CSI: (36kB – 52MB) | Relaxed |
|  | Inference | UE | gNB | same as CSI feedback size (~ a few 1000 bits) | Time-critical |
|  | Monitoring | UE | gNB | same as CSI feedback size (~ a few 1000 bits) | Near-real-time |

As it comes to 6GR, 5GA AI/ML use cases over the air interface can be considered at least to be supported in Day 1, including AI/ML beam management, CSI compression and CSI prediction, though the final decision is up to RAN1. Based on this assumption, 5GNR study on data size/latency/frequency/producer-consumer for training/inference/monitoring data can be considered as baseline for the study of 6GR AI/ML related data.

New Services – Sensing

The following measurement metrics were proposed and discussed during 5GNR ISAC RAN1 study [6]:

  - Type 1: Raw channel data (e.g., channel matrix) [Data size: kB-GB];

  - Type 2: Spectrum of delay, and/or Doppler, and/or angle, i.e., the powers or the amplitudes/phases are reported for at least a range of samples of delay, and/or Doppler, and/or angle [Data size: hundreds to thousands of kB];

  - Type 3: Intermediate sensing results (e.g., Delay, Doppler, Angle, and Power) per detected path [Data size: tens to hundreds of bits];

  - Type 4: Intermediate sensing results (e.g., (Position, Velocity, and Power) or (Range, Velocity, Angle, and Power)) per detected point [Data size: tens to hundreds of bits];

  - Type 5: Final sensing results per detected object/target [Data size: tens of bits]

    - Option 1: Position and velocity

    - Option 2: Range, velocity, and angle

    - Note: Trajectory report is separate discussion

  - Type 6: SINR, RSRP from perspective of sensing [Data size: tens of bits]

TRP monostatic

TRP-TRP bistatic

UE-TRP bistatic

UE monostatic

TRP-UE bistatic

UE-UE bistatic

Figure 1. Sensing Modes

During RAN #109 meeting, it has been agreed that 6GR should study all sensing modes [7].

- 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. - 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs.

Furthermore, it is also agreed that at least sensing use case ‘detection and/or tracking of passive objects’ need to be supported in 6GR within 6G RAN architecture [7].

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

In this section, we analyse different sensing data collection and transfer scenarios based on different sensing modes. Depends on the use cases, the collected sensing data can further be requested by the following consumers:

Consumer Candidate 1: Non-UE-related Application layer

For the use case ‘detection and tracking of passive objects’, the collected sensing data can be consumed by the application layer which requests the sensing service.

Consumer Candidate 2: UE or UE-related Application layer

For use cases such as intelligent driving and health motion detection, the sensing data is consumed by the UE itself. For this scenario, considering the data security aspects, it is most likely the UE will request itself as the sensing data producer (also sensing Rx). Hence, there’s no need to perform data transfer over the air interface.

Even the UE itself has a low data processing capability and needs further processing on its sensing server which is located in application layer (i.e., UE-owned application layer), similar to AI/ML UE-side data collection, such sensing data transfer should be transparently to 3GPP network.

On the other hand, due to lack of sensing capability at the UE device, UE’s application layer may also request sensing data from the network.

Consumer Candidate 3: Network

Network itself can consume the collected sensing data for ‘sensing for network optimization’ use cases, e.g., sensing assisted beam management, sensing assisted mobility, etc.

Table 3. Sensing Data Producer and Consumer

| Sensing Data Consumer | Use Case(s) | Sensing Modes | Producer | Needs of standardized data transfer | Over the air interface? |
| --- | --- | --- | --- | --- | --- |
| Non-UE-related Application layer | Detection and/or tracking of passive objects | TRP monostatic TRP-TRP bistatic UE-TRP bistatic | TRP | RAN <-> SF/Non-UE-related application layer | No |
|  |  | UE monostatic TRP-UE bistatic UE-UE bistatic | UE | UE <-> SF/Non-UE-related application layer | Yes |
| UE or UE-related Application layer | Intelligent driving and health motion detection | UE monostatic TRP-UE bistatic UE-UE bistatic | UE | No | No |
|  |  | TRP monostatic TRP-TRP bistatic UE-TRP bistatic | TRP | RAN <-> SF/UE-related application layer | No |
| Network (RAN/CN) | Sensing for Network Optimization | TRP monostatic TRP-TRP bistatic UE-TRP bistatic | TRP | RAN <-> RAN RAN <-> CN | No |
|  |  | UE monostatic TRP-UE bistatic UE-UE bistatic | UE | UE <-> RAN UE <-> CN | Yes |

Last but not least, from RAN2 point of view, 6GR can start with the study of data procedure/consumer based on different sensing modes and sensing architecture. As for the exact sensing data metrics, we expect the data size, frequency of reporting, etc should also consider RAN1 study on performance evaluation.

Observation 1: Different use cases may generate different data types that terminates at different entities inside/outside of 3GPP network.

In order to design and select a proper data collection and transfer framework, RAN2 should first study how many data types that 6GR should study from Day1, together with their producer/consumer, use case(s), QoS requirement, data size, data reporting frequency, etc.

Proposal 2: Study data collection and transfer in the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

- For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting;

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

- For AI/ML related data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR AI/ML related data;

- In terms of data transfer framework for sensing related data, RAN2 needs to wait for further input from RAN1 evaluation on measurement metrics (e.g., data size, reporting frequency, QoS, etc) and RAN2 study on sensing architecture/protocol (e.g., producer, consumer);

- NOTE: Coordination with RAN1, RAN3 and SA is expected.

### Data Transfer

Based on the above discussion, the below figure shows that what function entities are involved in different types of data collection/transferring services:

Figure 2. Data Collection and Transfer within Network Architecture

It is obvious that different types of data may terminates at different network entities. Depends on the producer and consumer of each use case, the data transfer tunnel could be different. But of course, if the producer and consumer are the same, the same solution of data transfer tunnel between same producer/consumer should be considered.

Observation 2: There’s no single solution/tunnel is perfect for different types of data, considering different pairs of producer/consumer, use case scenario, etc. But it is still possible to use the same tunnel for data transfer with the same termination points (i.e., same producer and consumer).

In summary, based on use cases and data producer/consumer, the following options between different entities for data transfer can be further considered in 6GR:

Table 4. Data Transfer between Producer/Consumer

| Producer/Consumer | Use Case Example(s) |
| --- | --- |
| Data producer: UE; Data consumer: RAN | - AI/ML NW-side data collection, especially when UE collects/reports huge amount of training data (e.g., measurement results) for NW-side model training. RAN can further forward the received data to OAM. - Huge amount of sensing raw data (if needed) measured by sensing UE is reported to RAN either for communication optimization or further forward to sensing client. RAN can further forward the received data to other entities in CN. |
| Data producer: UE; Data consumer: CN/AF | - AI/ML UE-side data collection, where UE needs to transfer its collected data for UE-side model training to its own OTT server. - Sensing UE reports huge amount of sensing results (if needed) to either SF in CN or to sensing client directly. NOTE: If data is requested by over-the-top server, as baseline, UE can always transfer its own data to application layer transparently to 3GPP. |
| Data producer: RAN, Data consumer: CN/OAM/AF | - To support two-sided model, NW transfers collected/measured information (e.g., dataset/model parameter) to UE-side OTT server for two-sided model training. UE should not be used as a relay for huge amount of data transfer between NW and application layer server. Over-the-air is not supported for data transfer between NW and AF. - Efficient data exchange and computing sharing between RAN and Core for AI/ML RAN use cases. - When TRP-monostatic/TRP-TRP bistatic is used, RAN can report huge amount of sensing results (if needed) to SF in CN. |

Proposal 3: Depends on the data producer/consumer and use case(s), study efficient data transmission framework(s), including data transfer tunnel, protocols, and procedures. Aim to support unified design for data transfer with the same termination points (i.e., same producer and consumer).

## AI/ML Framework

AI/ML techniques has been quickly evolved in recent years. Since Rel-17, 3GPP RAN has started study on AI/ML framework, use cases, data collection, etc, and the following use cases have been evaluated over the air interface:

- AI/ML based beam management

- AI/ML based CSI prediction

- AI/ML based CSI compression

- AI/ML based mobility

Based on 5GNR evaluation on different use cases, we see AI/ML can indeed bring performance improvement in certain area, but there’s also some drawbacks, e.g., power consumption, privacy concern, generalization, etc. Therefore, the following key factors must be evaluated for 6GR AI/ML, including UE side complexity and cost, user experience, and support for new enablers required by identified use cases.

- Minimize Model/Parameter Burden: Avoid scenarios necessitating UEs to maintain an excessive number of AI/ML models or model parameters. This burden consumes significant UE storage and complicates model management. Framework enhancements facilitating data categorization (e.g., improving associated identifiers) to enable the generation of more generalized device-side models should be explored.

- Avoid on-device Training: On-device model training requires dedicated UE hardware and software support. Furthermore, it demands substantial computational resources due to numerous iterative operations and consumes significant storage for training data and intermediate parameters. These factors substantially increase UE complexity and cost.

- Preserve User Privacy: User privacy must be safeguarded. During data collection or reporting of additional UE conditions, sensitive user information (e.g., location, media consumption habits) should not be collected.

Proposal 4: 6GR AI/ML framework should follow the below principles:

  - Mitigate the requirement for UEs to maintain excessive models or parameters;

  - Avoid on-device training.

  - Ensure robust user privacy protection.

### AI/ML Architecture and Protocol Design

Since AI/ML has been introduced in the late release of 5GNR, for backward compatibility, supporting AI/ML for air interface/NG-RAN as a RAN functionality didn’t introduce any impact to network architecture/protocol/radio bearer design, etc. By integrating AI/ML function as a functionality integrated inside of RAN, it requires every RAN node introducing AI/ML functions to update its hardware, e.g., integrating GPU inside of RAN node. This increases CPEX for each RAN node and further increases challenges towards market succeed to support native AI/ML in RAN.

Lesson learnt from 5G (6): 5GNR introduces AI/ML functionalities in late releases and the AI/ML architecture design is limited by existing NG-RAN architecture and protocol, which further constrains market deployment due to the cost.

Starting from 6GR, native AI/ML RAN architecture with flexible deployment options need to be studied. An AI/ML dedicated function entity can be considered for flexible deployment, either inside RAN or outside of RAN. However, supporting such flexible deployment at network side should be transparently to UE, to avoid duplicated configuration/reporting over the air interface for the same functionality.

Hence, from RAN2 point of view, in order to support such flexibly deployment with UE transparency, a separate AI/ML protocol needs to be studied on how to handle all AI/ML related functionalities, including LCM, data collection and transfer, etc.

Table 5. AI/ML Architecture and Protocol Design

| AI/ML Function Entity Inside of RAN | AI/ML Function Entity Outside of RAN |
| --- | --- |
| Unified Protocol Design over the air interface |  |

Proposal 5: Study protocol design for AI/ML functionalities to support UE-transparent flexible network deployment (either inside/outside of RAN) of AI/ML functionality.

### Life Cycle Management

5GA Functional framework of AI/ML air interface is captured in TR 38.843 [8] Figure 4.4-1:

Based on the normative phase discussion, there are several procedures that have no specification impact, including:

- Model storage of all use cases;

- Model transfer/delivery procedure (including request, transfer and update) for both one-sided model and two-sided model. Both RAN1 and RAN2 had spent a lot of time discussing whether and how to support model transfer over air interface, but in the end, it is not specified due to lack of motivation (e.g., UE-side model can be transferred to UE via application layer transparently to 3GPP).

  - Note: dataset and model parameter transfer for two-sided model belongs to the domain of training data transfer, as such information is used for UE-part two-sided model training.

Lesson learnt from 5G (7): RAN2 spent quite some time to study and discuss on procedures (e.g., model transfer over air interface, etc) that was not specified in the end, mainly because supporting such procedures is lack of motivation from standardization point of view.

Based on the above observations, we further propose to update functional framework of AI/ML air interface by only focusing on necessary components and procedures that need to be specified:

- Data related procedures

  - Training data collection request and transfer;

  - Inference results reporting, monitoring results reporting.

- Inference and monitoring related procedures

  - Capability/Applicability;

  - Inference/monitoring configuration.

Furthermore, as supported in 5GNR, AI/ML LCM signalling procedure was limited by existing signalling framework, which increases the complexity and overhead of signalling design:

- Applicability reporting procedure was introduced, mainly due to dynamic UE capability reporting is not supported in 5GNR. Based on this, RAN2 further introduced Option A and Option B applicability reporting procedure due to lack of consensus on how UE performs applicability checking, which is completely up to UE temporary capability;

- Network-side data collection logging signalling design couples L1 CSI framework and L3 event-triggered procedures, mainly due to split design between L1 and L3 measurement reporting. RAN2 spent several meetings to discuss whether to add logging event to L1 CSI framework or design a new L3 signalling framework for logging event.

Lesson learnt from 5G (8): Complex signalling design was specified to support AI/ML features (e.g., applicability reporting, network-side data collection), mainly due to limitation from basic system signalling design in 5GNR Day 1 (e.g., capability framework, L1 and L3 measurement configuration/reporting, etc) .

For AI/ML framework in 6GR, though we believe the established 5GA AI/ML functional framework can serve as the starting point, the signalling of AI/ML framework should be designed on top of 6GR system design and signalling structure (e.g., considering 6GR capability framework, 6GR measurement framework, etc) for simplicity and signalling efficiency.

Proposal 6: Design an efficient and simplified AI/ML air interface LCM for necessary components/procedures (e.g., data-related procedures, inference/monitoring related procedures, etc), based on 6GR system design and signalling framework. 5GA AI/ML functional framework concept can serve as the starting point of 6GR study.

## Security

In RAN WG Study Item on 6G (SID in RP-252912), following objective is related to security:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

SA plenary has approved SID for 6G security in SP-251233 (revision of SA3 endorsed SID S3-252973), with the following work task related to RAN:

WT#2: Study security and privacy aspects of 6G RAN architecture (including all UE – RAN interactions and ref: RP-251881).

In RAN#109 meeting, LS RP-252891 (LS on Early Alignment on lower layer AS security aspects between RAN and SA) was sent to SA (cc RAN2, RAN3, SA3, SA2), indicating the following:

Following further discussions during RAN#109, RAN Plenary believes that the security requirements for the access stratum can significantly influence the overall system design. Therefore, it is crucial to establish when these security requirements will be aligned between RAN and SA and particularly between SA3 and RAN2. TSG RAN believes that June 2026 would be an appropriate date. RAN invites SA to consider adding a June 2026 check point (in SA3’s SID on 6G security S3-252973) for the alignment between SA3 and RAN2 on any security requirements that impact the AS protocol design.

Given that RAN plenary has sent LS to SA plenary regarding alignment between SA3 and RAN2 on any security requirements that impact the AS protocol design, RAN2 needs to wait for SA3’s progress before proceeding any related work.

Proposal 7: RAN2 waits for SA3’s input on requirements for access stratum security before starting any related study.

# Conclusion

Lesson learnt from 5G (1): Exposing data collected from UE at intermediate nodes inside 3GPP network increases the risk of data leakage, further lead to user privacy concern.

Lesson Learnt from 5G (2): Consideration of UE efficient involvement is already supported in 5GNR, e.g., UE reports SON/MDT reports whenever available, UE optionally logging for NW-side data collection, etc.

Lesson Learnt from 5G (3): Even with the same termination, UE has been requested to send duplicate reports over the air interface for the same measurement in 5GNR.

Lesson Learnt from 5G (4): In 5GNR, UE has been requested to support multiple minimum memory sizes for different use cases, which increases the cost at UE side.

Proposal 1: 6GR data collection and transfer design should follow the below principles:

- Respect user privacy and user consent for different types of data collection. Coordination with SA WGs is expected;

- Collected data should only be visible to the data end consumer. Avoid storing/processing/terminating collected data at intermediate nodes within 3GPP network;

- Respect UE’s memory and power consumption cost, including efficient UE participation (e.g., only collect UE available data whenever possible without additional UE measurement), avoid duplicate measurements (for same producer/consumer), reasonable memory requirement, etc.

Lesson Learnt from 5G (5): Data types for existing data collection services (e.g., SON/MDT, QoE) has been well-established in 5GNR, which can be considered as starting point for 6GR study.

Proposal 2: Study data collection and transfer in the following aspects:

- Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc);

- For each type of data, study:

  - Applicable use case(s)

  - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM;

  - Data size in a single reporting;

  - Frequency of data reporting;

  - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

- For SON/MDT, QoE related data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR SON/MDT, QoE related data;

- For AI/ML related data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR AI/ML related data;

- In terms of data transfer framework for sensing related data, RAN2 needs to wait for further input from RAN1 evaluation on measurement metrics (e.g., data size, reporting frequency, QoS, etc) and RAN2 study on sensing architecture/protocol (e.g., producer, consumer);

- NOTE: Coordination with RAN1, RAN3 and SA is expected.

Observation 2: There’s no single solution/tunnel is perfect for different types of data, considering different pairs of producer/consumer, use case scenario, etc. But it is still possible to use the same tunnel for data transfer with the same termination points (i.e., same producer and consumer).

Proposal 3: Depends on the data producer/consumer and use case(s), study efficient data transmission framework(s), including data transfer tunnel, protocols, and procedures. Aim to support unified design for data transfer with the same termination points (i.e., same producer and consumer).

Proposal 4: 6GR AI/ML framework should follow the below principles:

  - Mitigate the requirement for UEs to maintain excessive models or parameters;

  - Avoid on-device training.

  - Ensure robust user privacy protection.

Lesson learnt from 5G (6): 5GNR introduces AI/ML functionalities in late releases and the AI/ML architecture design is limited by existing NG-RAN architecture and protocol, which further constrains market deployment due to the cost.

Proposal 5: Study protocol design for AI/ML functionalities to support UE-transparent flexible network deployment (either inside/outside of RAN) of AI/ML functionality.

Lesson learnt from 5G (7): RAN2 spent quite some time to study and discuss on procedures (e.g., model transfer over air interface, etc) that was not specified in the end, mainly because supporting such procedures is lack of motivation from standardization point of view.

Lesson learnt from 5G (8): Complex signalling design was specified to support AI/ML features (e.g., applicability reporting, network-side data collection), mainly due to limitation from basic system signalling design in 5GNR Day 1 (e.g., capability framework, L1 and L3 measurement configuration/reporting, etc).

Proposal 6: Design an efficient and simplified AI/ML air interface LCM for necessary components/procedures (e.g., data-related procedures, inference/monitoring related procedures, etc), based on 6GR system design and signalling framework. 5GA AI/ML functional framework concept can serve as the starting point of 6GR study.

Proposal 7: RAN2 waits for SA3’s input on requirements for access stratum security before starting any related study.

# Reference

[1] RP-252912	Revised SID: Study on 6G Radio

[2] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services

[3] R1-2308730	Reply LS on Data Collection Requirements and Assumptions

[4] R1-2310681	Reply LS on Data Collection Requirements and Assumptions

[5] R1-2410922	LS on signalling feasibility of dataset and parameter sharing

[6] R1-2506480	Summary #5 on evaluations for NR ISAC

[7] RP-252960	pCR for 38.914 on Requirements of new and existing services for 6G

[8] TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface