3GPP TSG-RAN WG2 Meeting #131bis	R2-2506800

Prague, Czech Republic, 13th – 17th, Oct. 2025

Source:	vivo

Title:	Considerations on 6GR AI framework

Agenda Item:	10.3.3

Document for:	Discussion and Decision

- Introduction

At the RAN#108 meeting, the item “New SID: Study on 6G Radio” [1] was approved. There are the following objectives on the 6G AI:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we will discuss the potential 6G AI/ML framework and share our opinion.

- Discussion

  - General Principles and Requirements

In NR, The following AI/ML use cases were discussed, with standardization work completed for some cases in Rel-19:

- Beam management, including spatial-domain downlink beam prediction for Set A based on measurement results of Set B and temporal downlink beam prediction for Set A based on the historical measurement results of Set B;

- CSI feedback enhancement, including time domain CSI prediction using a UE-side model and spatial-frequency domain CSI compression using a two-sided AI/ML model;

- Positioning accuracy enhancements, including direct AI/ML positioning and AI/ML assisted positioning;

- Mobility enhancement, including RRM measurement prediction and measurement event prediction;

The Rel-19 work item “AI/ML for NR air interface” led by RAN1 covers the first three AI/ML use cases, and the Rel-20 work item “AI/ML for Mobility” led by RAN2 will cover the fourth AI/ML use case. The introduction of AI/ML into 5G networks, particularly within the RAN, has largely followed a use-case-specific design paradigm. This means that for each individual use case (e.g., beam management, positioning, mobility), a separate AI/ML model would be designed, developed, and deployed with its own dedicated data pipeline, computational resources, and lifecycle management. While this approach has successfully demonstrated the value of AI/ML in isolated scenarios, it hinders scalability, efficiency, and innovation. In detail:

- Each AI/ML use case operates as an independent “island” within the larger network ecosystem. Model training for different use cases often requires similar raw data (e.g., Channel State Information, RF measurements, traffic loads). However, in a siloed approach, each AI/ML functionality triggers its own data collection process. This leads to massive data duplication, increased overhead and inefficient use of fronthaul/backhaul bandwidth. Also, there is no unified way to monitor, retrain, or update these models. Network operators must manage countless individual models, each with its own performance metrics (KPIs), update cycles, and failure modes. This creates an operational nightmare (OPEX explosion) as the number of deployed AI/ML solutions grows.

- Multiple models operating independently cannot share computational resources (e.g., GPUs, NPUs) efficiently. A base station might have one model for beam prediction and another for mobility management, both idling or processing at different times, leading to poor hardware utilization and increased energy consumption - a critical concern for sustainability.

- The use-case-specific approach requires 3GPP to re-discuss each new AI/ML use case, consuming significant RAN WGs meeting time.

The AI-native vision for 6G implies that AI/ML will be the default tool for features, not an afterthought. More AI/ML use cases are expected in 6G, ranging from foundational RAN operations (e.g., channel prediction, massive MIMO optimization) to entirely new services (e.g., sensing, digital twins). An extensible framework is essential to allow network operators, vendors, and third-party developers to seamlessly plug in new AI/ML use cases as they emerge, future-proofing the network infrastructure. A unified framework would provide the necessary hooks and interfaces to manage the entire lifecycle of models across the network efficiently.

Observation 1: While 5G introduced AI/ML functions through a use-case-specific approach, its fragmentation and lack of coordination make it unsuitable for the AI-native vision of 6G. highlighting the urgent need for a generic, extensible, and unified framework.

In NR, one-sided models were supported and standardized in Rel-19. In Rel-20, two-sided models for CSI feedback enhancement will be standardized in the “NR_AIML_air_Ph2” work item [3], which includes relevant AI/ML framework work for the support for two-sided models. We think that 6G AI/ML framework should support both of one sided model and two-sided model from the outset.

For one-sided models, NR supports both of UE-sided model and NW-sided models. Depending on operator’s policy or use cases themselves, a UE-sided model may be desirable for some AI use cases, while an NW-sided model may be desirable for others. We think that 6G AI/ML framework should support both of UE-sided model and NW-sided model for one-sided models from the outset.

- RAN2 aims to design a generic and extensible 6G AI/ML framework, enabling new AI/ML use cases to be easily introduced and integrated, and supporting both one-sided models (including UE-sided and NW-sided models) and two-sided models.

  - 6G AI/ML Framework and Functions

    - 6G AI/ML framework Basic Procedures

- UE Capability Exchange

For 5G NR AI/ML-enabled enhancement features, such as beam management and CSI prediction, the network can configure UEs with inference configurations based on their reported capabilities. Additionally, a UE AI/ML Processing Unit (APU) pool was introduced to manage beam management and CSI reporting-related AI/ML features. The concept of managing AI/ML processing capability is also needed for other use cases beyond CSI reporting. In 6G, UE capability reporting and management would be fundamental to enabling AI/ML Lifecycle Management (LCM) system to effectively reflect UE AI/ML supported features and processing capabilities.

- UE Applicable Functionality Reporting

For Rel-18/19 AI/ML, based on the all or part of the configured inference configurations, the UE determines applicable AI/ML functionalities, reports both applicable and inapplicable functionalities, and notifies the network of changes in their applicability status. In 6G, beyond functionality applicability reporting, managing AI/ML processing staus change reporting is also needed. UEs should report not only their AI/ML functionality applicability but also their APUstaus change. Based on (de-)activated functionalities, UEs should also report changes in applicability and APU usage status. As UEs can process more dynamic information, 6G AI/ML should aim for unified and flexible functionality applicability reporting, enabling UEs to dynamically adjust their processing status by reporting applicable and inapplicable functionalities, changes in applicability status, PU usage per functionality, and changes in APU usage.

- UE Inference and Performance Monitoring

In Rel-18/19 AI/ML, inference configuration and reporting are performed for applicable UE functionalities. The process allows the network to configure the UE with necessary inference configuration and performance monitoring configuration. A similar approach should be considered for 6G AI/ML use cases inference and performance monitoring configuration, and reporting.

- Functionality (de-)activation and Fallback/Switching

Similarly to Rel-19, as applicable CSI report can be activated by MAC CE/DCI, 6G AI/ML functionality may require functionality (de-)activation. Additionally, AI/ML functionality fallback/switching mechanism to/from non-AI/ML functionality should also be considered. This would allow the UE to fallback/switch to legacy non-AI/ML functionality, in case of AI/ML functionality becomes inapplicable. Thus, 6G AI/ML LCM should introduce functionality (de-)activation and fallback/switching to non-AI/ML functionality.

- UE-side Data Collection and NW-side Data Collection

Data collection is fundamental to enabling AI/ML functionalities. The basic consideration for data collection is that: for model training (either network-side model training or UE-side model training) the entity performing model training should first gather data. The data gathering process may either be the entity (performing model training) directly collect data or data are collected and transferred to the entity. Thus, data collection for both UE-side data collection and NW-side data collection should be, by default, supported in 6G AI/ML.

- Model Delivery/Transfer

As discussed in Rel-18 [2:TR38.843], with regard to model delivery/transfer scenario/configuration specific models may provide performance benefits in some studied use cases. At least, when UE has limitation to store all related models, model delivery/transfer to UE will be crucial in whether the UE can use AI/ML functionalities. In 6G AI/ML, the same consideration can still be valid, that is: based on some specific scenarios/configurations, model transfer may be beneficial. Thus, 6G AI/ML mechanism should investigate signaling support for model deliver/transfer.

- 6G AI/ML signaling framework should support at least the following functions/procedures:

- UE AI/ML Capabilities Exchange;

- Applicable Functionality Reporting;

- Inference Configuration and Reporting;

- Performance Monitoring Configuration and Reporting;

- Functionality (De-)Activation and Fallback/Switching to the non-AI/ML Functionality;

- Data collection, both UE-side Data Collection and NW-side Data Collection;

- Model Delivery/Transfer.

    - 6G AI/ML Functional Framework

Based on the identified functions/procedures recognized in 5G, we try to further analyze the mutual interactions in between taking the functional framework in TR 38.843 as a start point.

Figure 2.2-1: 5G AI/ML functional framework

It is observed that some AI/ML LCM functions/procedures standardized in 5G are not directly reflected in the block diagram above, the interaction with other functions are also missed, which can cause some misunderstandings as an illuminating design guidance in 6G. To put it clearly, we identify the following improvements for a legible diagram:

- Most importantly, procedures like capability transfer and applicability determination are not indicated between the “management” function and the “inference” function in this diagram. However, capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical from the deployment entity’s perspective, especially for the case of UE-sided model.

Observation 2: UE capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical from the deployment entity’s perspective, especially for the case of UE-sided model.

- Regarding data collection, the training data and data used during inference and monitoring are treated and discussed separately in 5G, considering the different time-efficiency. To distingusish the stage at what types of data is used and the urgency of data requirements for a better discussion in future, we propose further dividing it into “non-real-time” for the “training” function and “real-time” for the “inference” function and the “management (e.g., monitoring)” functions.

Observation 3: In 5G AI/ML discussion, data collection is categorized and analyzed according to the functional phase of data utilization and the requisite latency.

Accordingly, we strongly suggest RAN2 revises the existing functional framework to incorporate the functions and procedures successfully specified during 5G phase as a starting point for 6G AI/ML framework. Figure 2.2-2 illustrates the fundamental enhancement from Figure 2.2-1.

Figure 2.2-2: suggested 6G AI/ML functional framework

- Capture 6G AI/ML functional framework in 6G TR taking the one in TR 38.843 as a starting point, with the following modifications:

- Indicate “capability/applicability reporting” procedure via an arrow from inference to management;

- Differentiate data collection with “real-time” for monitoring and inference, “non-real-time” for training.

  - Unified 6G AI/ML Design

    - Unified and Future-Proof LCM Signalling Procedures

In NR, the LCM procedures for each AI/ML use case are separately discussed, often leading to redundant discussions across use cases. For example, after addressing the above components in Beam management framework, similar discussions are repeated or considered in AI/ML mobility. From a specification perspective, these components would be implemented per use case, and their specification maintenance is typically handled separately. For some AI/ML use cases, such as beam management and AI/ML mobility, LCM-related signalling is exchanged between UE and gNB using RRC messages. For positioning use case, tLCM related signalling is exchanged between UE and LMF using LPP messages, or between gNB and LMF using NRPPa messages. This fragmentation significantly increases specification complexity and leads to redundant RAN2 discussions for each LCM procedure across AI/ML use cases, consuming substantial RAN WGs meeting time. As 6G would introduce more AI/ML use cases, this issue will likely worsen. Therefore, we propose that 6G AI/ML LCM discussions avoid addressing LCM components on a per-use-case basis. Instead, efforts should focus on unifying each LCM component across use cases whenever possible. For instance, rather than having separate Applicable Functionality reporting for Beam Management and CSI Prediction, the UE could report Applicable Functionality for both in a unified manner. This approach would significantly reduce specification complexity and streamline LCM deployment. As noted above, the functions, procedures and functional framework are largely similar across use cases. Therefore, we believe it is feasible to support a unified and future-proof LCM procedures for all the AI/ML use cases. With the unified and future-proof LCM procedures, a new AI/ML use case could be  introduced by extending the configured parameters of the affected component signalling procedures, similar to introducing a new positioning method in the LPP specification. See the following figure for an example of introducing a new AI/ML use case (e.g., use case 1 highlighted in red).

Figure 2.3-1: An example for introducing new AI use case 1

For example, if a new AI use case (e.g., use case 1) is introduced, the common applicability reporting IE in the unified applicability reporting message can be used if there is no use-case-specific elements are required. Otherwise, a new IE can be added to the unified applicability reporting message for use case 1. This approach applies to other LCM procedures as well.

- Support unified and future-proof LCM signalling procedures. The LCM signalling procedures should support new AI/ML use cases by extending only the configured parameters of the affected component signalling procedures, this can avoid the need to re-discuss component signalling procedures on a case-by-case basis.

    - Unified UE AI/ML Processing Capability

In the NR AI/ML-based CSI report framework for UE-side inference, the UE can indicate its AI/ML processing capability with the following components.The network is expected to configure appropriate inference request that does not exceed UE processing capability.

1. Number of APU pools.

2. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously in a CC.

3. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously across all CCs.

It is worth noting that the above capability is only for CSI-related AI/ML use cases, and similar capabilities were not introduced for non-CSI AI use cases, such as AI/ML-based positioning. This is because limited AI/ML use cases are supported in NR and only CSI-related AI/ML use cases require real-time inference.

Observation 4: Due to limited AI/ML use cases are supported in NR and only CSI-related use cases require real-time inference, the UE AI/ML processing capability introduced in NR is designed specifically for CSI-related use cases.

In 6G, it is expected that more use cases will be supported, some of which will also have real-time requirements for inference, such as AI/ML-based RS overhead reduction. In such cases, how to design the UE AI/ML processing capability should be discussed. If, similar to NR, UE AI/ML processing capability is defined per feature or per feature group, the implementation costs for the UE would be highly increased. Furthermore, since inference configurations (e.g., periodicity) for different use cases are up to network requirements, this may lead to low utilization efficiency of the APU corresponding to specific feature/feature group.

Figure 2.3.2-1: AI/ML processing capability for multiple use cases

Therefore, we believe that 6G should consider unified UE AI/ML processing capability. Specifically, the UE AI/ML processing capability should be shared for all the AI/ML-based use cases. With unified UE AI/ML processing capabilities, the network can set more efficient inference configuration based on the priorities of different functionalities.

- UE AI/ML processing capability in 6G should be shared for all the AI/ML use cases.

    - Unified Data Framework

From Rel-18/19 AI/ML data collection discussions, we observe the following:

- Separate discussions on data collection mechanism for UE-side and network-side data collection.

- Non-unified data collection solution discussions across different use cases, such as beam management and AI/ML mobility.

One consequence of non-unified data collection solutions for both UE-side and network-side data collection is that Rel-19 has not standardized UE-side data collection solutions, limiting progress in this area. The 6G AI/ML data collection design should aim to unify data collection solutions for both UE-side and network-side data collection.

In current 5G AI/ML data collection designs, the network may separately configure the UE to collect data for two use cases. Based on the network's configuration, the UE collects and reports data to the network. These two use cases may share common measurement results, such as RSRP. However, when reporting data collection results, the UE separately reports RSRP for each use case. This introduces unnecessary duplicated reporting of the same measurement results. If data collection and reporting were use-case-agnostic, the UE could report measurement results common to multiple use cases only once. Based on the use cases requiring data, the network could copy and forward or use the relevant data for each use case. Reporting each measurement result only once would help reduce UE power consumption caused by redundant measurement reporting.

- 6G AI/ML studies should strive for:

- unified data collection solutions for both UE-side and network-side data collection

- use case agnostic data collection framework.

Figure 2.3.3-1: Example of model transfer

The Rel-18 AI/ML study investigated model transfer solutions, but no specification work was conducted in Rel-19. In 6G, UE computational capabilities are expected to increase, enabling UEs to process multiple AI/ML models simultaneously. This computational improvement would enable UEs to perform tasks such as model fine-tuning. For example, the network could just transfer a reference model to the UE, which could then fine-tune it. However, with a potentially large number of AI/ML use cases and the need to manage multiple models per use case, UE memory capacity may be insufficient to handle multiple models simultaneously. In such scenarios, UEs may need to frequently download models from the network or a server via the network as model transfer. To address various model transfer scenarios, a unified model transfer scheme should be thoroughly studied and supported as a 6G AI/ML mechanism.

Figure 2.3.3-2: Example of data transfer

As shown in Figure 2.3-4, there can be various scenarios of transferring the collected data transfer, e.g., from UE to RAN/CN/OAM or RAN to CN/OAM. From transferring perspective, both data collection and model transfer can be considered in a unique framework. Collected data can be transferred from one entity to another in a manner similar to how models can be transferred between entities. Thus, model/data transfer should be supported within a unified data transfer framework. A potential approach to achieve this unified framework would be to design data transfer within the data plane realm[4].

- Study a unified model/data transfer framework in 6G RAN for data transfer.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Observation 1: While 5G introduced AI/ML functions through a use-case-specific approach, its fragmentation and lack of coordination make it unsuitable for the AI-native vision of 6G. highlighting the urgent need for a generic, extensible, and unified framework.

Observation 2: UE capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical from the deployment entity’s perspective, especially for the case of UE-sided model.

Observation 3: In 5G AI/ML discussion, data collection is categorized and analyzed according to the functional phase of data utilization and the requisite latency.

Observation 4: Due to limited AI/ML use cases are supported in NR and only CSI-related use cases require real-time inference, the UE AI/ML processing capability introduced in NR is designed specifically for CSI-related use cases.

- RAN2 aims to design a generic and extensible 6G AI/ML framework, enabling new AI/ML use cases to be easily introduced and integrated, and supporting both one-sided models (including UE-sided and NW-sided models) and two-sided models.

- 6G AI/ML signaling framework should support at least the following functions/procedures:

- UE AI/ML Capabilities Exchange;

- Applicable Functionality Reporting;

- Inference Configuration and Reporting;

- Performance Monitoring Configuration and Reporting;

- Functionality (De-)Activation and Fallback/Switching to the non-AI/ML Functionality;

- Data collection, both UE-side Data Collection and NW-side Data Collection;

- Model Delivery/Transfer.

- Capture 6G AI/ML functional framework in 6G TR taking the one in TR 38.843 as a starting point, with the following modifications:

- Indicate “capability/applicability reporting” procedure via an arrow from inference to management;

- Differentiate data collection with “real-time” for monitoring and inference, “non-real-time” for training.

- Support unified and future-proof LCM signalling procedures. The LCM signalling procedures should support new AI/ML use cases by extending only the configured parameters of the affected component signalling procedures, this can avoid the need to re-discuss component signalling procedures on a case-by-case basis.

- UE AI/ML processing capability in 6G should be shared for all the AI/ML use cases.

- 6G AI/ML studies should strive for:

- unified data collection solutions for both UE-side and network-side data collection

- use case agnostic data collection framework.

- Study a unified model/data transfer framework in 6G RAN for data transfer.

- Reference

- RP-251881 New SID: Study on 6G Radio

- 3GPP TR 38.843 V18.0.0

- RP- 25187 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- R2-2506801 Discussion on 6G Data collection and Data Transfer