3GPP TSG-RAN WG2 Meeting #131bis	                                                                              R2-2506815

Prague, Czech Republic, Oct. 13th-17th

Source:	CATT

Title:	Discussion on SP CSI-RS and CSI-IM for early CSI acquisition

Agenda Item:    8.6.3

Document for:  Discussion and Decision

Introduction

This paper further provides our views on the understanding of the early CSI acquisition by SP CSI-RS/CSI-IM. The detailed discussion is in Section 2, and followed by Section 3 that summarizes the conclusion of our contribution.

Discussion

For R19 LTM, according to RAN1 conclusion, UE may be configured with early CSI acquisition on candidate cells. It supports to perform the early CSI acquisition based on semi-persistent RS or periodic RS. For the semi-persistent RS, the CSI acquisition could be performed only after the RS is activated by MAC CE. Based on current MAC spec [1], the activation of the semi-persistent RS MAC CE could active one ltm-CSI-ResourceConfigId as Resources For Channel Measurement and one ltm-CSI-ResourceConfigId as Resource For Interference Measurements.

According to the current RRC running CR [2],

LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-SSB-ResourceSet-r18 LTM-SSB-ResourceSet-r18, ..., [[ ltm-NZP-CSI-RS-ResourceSet-r19 LTM-NZP-CSI-RS-ResourceSet-r19 OPTIONAL, -- Need R ltm-CSI-IM-ResourceSet-r19 LTM-CSI-IM-ResourceSet-r19 OPTIONAL, -- Need R resourceType-r19 ENUMERATED {periodic, semiPersistent} OPTIONAL -- Cond NoSSB ]] } LTM-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... } LTM-NZP-CSI-RS-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... } LTM-CSI-IM-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-IM-ResourceSetId-r19 CSI-IM-ResourceSetId, ltm-CandidateId-r19 LTM-CandidateId-r18, ... } LTM-CSI-ResourceConfig field descriptions. ltm-CSI-IM-ResourceSet This field indicates the resource set for LTM interference measurements. This field is absent if ltm-NZP-CSI-RS-ResourceSet is configured. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet..

Each LTM-CSI-ResourceConfig-r18 can configure either the NZP CSI-RS or CSI-IM, but not both. For the LTM-CSI-ResourceConfig-r18 that contains the NZP CSI-RS, an ltm-CandidateIdList-r19 is used to associate the NZP CSI-RS belonging to the corresponding candidate cell as highlighted by green. But for CSI-IM, each LTM-CSI-ResourceConfig-r18 can only provide the CSI-IM RSs for only one candidate cell as highlighted by the yellow.

Observation 1: In SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE, CSI Resource Configuration ID1 can be associated to Resource of multiple candidates and CSI Resource Configuration ID2 can be associated to ResourceSet of only one candidate.

Observation 2: There is a granularity mismatch between CSI Resource Configuration ID1 and CSI Resource Configuration ID2 in SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

Based on the current signaling structure and the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE, we need to align the understanding how to perform the early CSI acquisition on the following case,

One LTM-CSI-ResourceConfig-r18 with ID_1 provides the NZP CSI-RS associated with candidate cell 1 and candidate cell 2. Another one LTM-CSI-ResourceConfig-r18 with ID_2 provides the CSI-IM for candidate cell 1.

Generally, there are following three potential understandings,

| Understanding | Remarks |
| --- | --- |
| Understanding 1: UE uses the LTM-CSI-ResourceConfig-r18 with ID_2 for CSI-IM only for candidate cell 1 to perform early CSI acquisition | By this understanding, UE will utilize the CSI-IM resources configured for candidate cell 1 and the RS of candidate cell 1 which configured in LTM-CSI-ResourceConfig-r18 with ID_1 to acquire early CSI for candidate cell1. UE will not utilize the CSI-IM resources configured for candidate cell 1 to perform early CSI acquisition for candidate cell 2. The field description of ltm-ResourceForInterferenceMeasurements-r19 should specify it could only be configured with the CSI-IM-ResourceSetId of the candidate cell where the ltm-CSI-ReportConfig is configured |
| Understanding 2: UE uses the LTM-CSI-ResourceConfig-r18 with ID_2 for CSI-IM to perform early CSI acquisition on all the candidate cells associated with LTM-CSI-ResourceConfig-r18 with ID_1 (i.e. candidate cell 1 and candidate cell 2), | By this understanding, UE will utilize the CSI-IM resources configured on candidate cell 1 to perform early CSI acquisition on all the candidate cells (candidate cell1 and candidate cell 2). No any other impact on the SP CSI-RS/IM activation MAC CE. |
| Understanding 3: UE uses the IM RS from the same candidate cell to perform early CSI acquisition on the candidate cell. The MAC CE is allowed to include at least one of LTM-CSI-ResourceConfig-r18 for Interference Measurements. That is In addition to the CSI-IM RS for candidate cell 1, the SP CSI-RS/IM activation MAC CE needs to activate the CSI-IM RS for candidate cell 2. | By this understanding, when UE performs early CSI acquisition by measuring the CSI-RS associated with a certain candidate cell, UE is allowed to use the CSI-IM RS associated with the same candidate cell to perform the interference. The MAC CE format needs to be updated to allow activate the CSI-IM for multiple candidate cells in accordance with the candidate cells which have activated CSI-RS by the same MAC CE. The field description of ltm-ResourceForInterferenceMeasurements-r19 should specify it could only be configured with the CSI-IM-ResourceSetId of the candidate cell where the ltm-CSI-ReportConfig is configured |

From our view, this is under RAN1 scope, and we need to confirm which is aligned with RAN1 intention. Therefore, we propose that,

Proposal 1a: Send LS to RAN1 to clarify the granularity mismatch issue as follows,

If there are multiple candidates in the resource config of the SP CSI-RS Resources for early CSI acquisition, whether the SP CSI-IM resources in the multiple resource configs for all the related candidates should be activated together when it is to activate the RSs in the resource config of the SP CSI-RS Resources for early CSI acquisition.

Proposal 1b: RAN2 revisit the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE design once RAN1 confirmation mentioned in P1a is received.

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Proposal 1a: Send LS to RAN1 to clarify the granularity mismatch issue as follows,

If there are multiple candidates in the resource config of the SP CSI-RS Resources for early CSI acquisition, whether the SP CSI-IM resources in the multiple resource configs for all the related candidates should be activated together when it is to activate the RSs in the resource config of the SP CSI-RS Resources for early CSI acquisition.

Proposal 1b: RAN2 revisit the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE design once RAN1 confirmation mentioned in P1a is received.

Reference

[1] R2-2506229_ Introduction of NR mobility enhancements Phase 4 in MAC

[2] R2-2506567 Introduction of RRC changes for mobility enhancements phase 4

- Annex

3GPP TSG-RAN WG2 Meeting #131bis	                                                                              R2-250xxxx

Prague, Czech Republic, Oct. 13th-17th

Title:	LS on SP CSI-RS/CSI-IM activation for early CSI acquisition

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	RAN2

To:	RAN1

Cc:

Contact person:	Rui Zhou

zhourui@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

For early CSI acquisition performed based on the semi-persistent RS, the CSI acquisition could be performed only after the RS is activated by MAC CE. Based on current MAC spec, the activation of the semi-persistent RS MAC CE could active one ltm-CSI-ResourceConfigId as Resources For Channel Measurement and one ltm-CSI-ResourceConfigId as Resource For Interference Measurements.

As per the current RAN2 RRC signaling, each LTM-CSI-ResourceConfig-r18 can configure either the NZP CSI-RS or CSI-IM, but not both. For the LTM-CSI-ResourceConfig-r18 that contains the NZP CSI-RS, an ltm-CandidateIdList-r19 is used to associate the NZP CSI-RS belonging to the corresponding candidate cell. But for CSI-IM, each LTM-CSI-ResourceConfig-r18 can only provide the CSI-IM RSs for only one candidate cell.

RAN2 observes there is a granularity mismatch between CSI Resource Configuration ID1 and CSI Resource Configuration ID2 in SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE. So RAN2 wants to confirm with RAN1:

If there are multiple candidates in the resource config of the SP CSI-RS Resources for early CSI acquisition, whether the SP CSI-IM resources in the multiple resource configs for all the related candidates should be activated together when it is to activate  the RSs in the resource config of the SP CSI-RS Resources for early CSI acquisition?

- 2	Actions

To RAN1 group

ACTION: 	RAN2 respectfully asks RAN1 to provide feedback on above questions.

- 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #132	17th – 21th Nov 2025	Dallas, US

TSG-RAN2 Meeting #133	09th – 13rd Feb 2026	Stor-Göteborg , SE