3GPP TSG-RAN WG2 Meeting #131bis                                                   R2-2506844

Prague, Czech Republic, Oct. 13th-17th

Source:	CATT

Title:	Intra-gNB Service Continuity for Multi-hop U2N Relay

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In this contribution, based on the agreements at RAN2#131 meeting as in [1], one remaining issue on how to restrict inter-gNB D2M or I2M path switch is proposed, in this paper, we will further discuss this issue, our views are given as well.

Discussion

RAN2#131bis agreement on service continuity is as follows:

RAN2#131 Agreement: No enhancement is added now to allow indicating the hop count of a candidate target relay UE to the gNB. It can be discussed in maintenance if something is broken with the case where the target relay UE is in idle/inactive.

In last meeting, RAN2 proposed one case on restricting inter-gNB path switch to a target multi-hop relay. In this case, the remote UE will report the legacy measurement result of the candidate First relay to source gNB, based on the cellIdentity information in measurement report for target relay path, the source gNB will send the path switch request to target gNB.

If the candidate first relay UE is in RRC Connected mode at target gNB, the target gNB could find whether the target UE is in direct/indirect/multi-hop mode, if it is in indirect/multi-hop mode, the target gNB could reject the path switch request from source gNB as inter-gNB I2M or D2M path switch is not supported. If the relay UE is in direct mode, the target gNB could give acknowledgement to the path switch request message since inter-gNB D2I or I2I is supported, there is no issue identified.

However, for the candidate first relay UE in RRC Idle/Inactive mode, the target gNB has no idea which type of path switch it is, the first target relay UE could camp on the network directly/indirectly or by multi-hop relay(s). For UE in RRC Inactive mode, the gNB may try to find the target Relay UE by paging to see the hop count information of the target first relay UE, if the first target relay UE is connected to network directly, the gNB will acknowledge the handover request message for inter-gNB D2I or I2I path switch. If it is connected to network indirectly or by multi-hop relays, the gNB will reject the handover request for inter-gNB D2M or I2M path switch. For the candidate first relay UE in RRC Idle mode, gNB could reject the handover request to avoid the possible inter-gNB I2M or D2M path switch, the risk is that UE will miss the possible D2I or I2I path switch. But we think that the source gNB will try to find another suitable target relay UE and its target gNB, no big issue is identified.

Proposal 1: For inter-gNB path switch to target Relay UE in RRC Idle/Inactive mode, it is the network implementation to exclude the case of inter-gNB I2M or D2M path switch.

Conclusion

According to the analysis in section 2, our proposal is summarized as follows:

Proposal 1: For inter-gNB path switch to target Relay UE in RRC Idle/Inactive mode, it is the network implementation to exclude the case of inter-gNB I2M or D2M path switch.

Reference

- Report from session on positioning and sidelink relay, MediaTek, RAN2#131 meeting.