3GPP TSG-RAN WG2 Meeting #131bis	R2-2506848

Prague, Czech, October 13th – 17th, 2025

Agenda Item:	8.5.2

Source:	OPPO

Title:	Left Issues on On-Demand SSB and SSB adaptation (X200, L201, L202, O006)

Document for:	Discussion, Decision

- Introduction

This is to discuss the remaining issues for R19 NES.

- Discussion

  - Mapping between OD-SSB configuration and SMTC (X200)

In the agreed CR, the description for the mapping between OD-SSB configuration and SMTC is as follows

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first  OD-SSB configuration for the serving cell; the UE shall setup SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on.

From the previous discussion, the intention seems that the first component in smtcxlist-r19 is used for OD-SSB periodicity of ms5, while the second component is used for OD-SSB periodicity of ms10.

MeasObjectNR ::=                    SEQUENCE {

[…]

smtcxlist-r19                       SSB-MTCxList-r19                                                OPTIONAL,   -- Need R

[…]

}

SSB-MTCxList-r19::=                 SEQUENCE (SIZE(1..6)) OF SSB-MTC

OD-SSB-Config-r19 ::= SEQUENCE {

[…]

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL, -- Cond ODssbAOssb

[…]

}

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

In our view, there are two issues:

- If there is a periodicity for which there is no corresponding OD-SSB periodicity, following this rule, the SMTC has to be configured within smtcxlist (e.g., if there are OD-SSB configuration for ms5 and ms20, but there is no OD-SSB configuration for ms10), leading to unnecessary signaling overhead;

- If there are two OD-SSB configurations, of which the periodicity value is the same, but other parameter(s) like od-ssb-PositionsInBurst, they have to share the same SMTC configuration. While technically, od-ssb-PositionsInBurst could lead to different setting of SMTC. E.g., if the change of od-ssb-PositionsInBurst leads to SSB out of the SMTC window (of which the minimum value of duration can be sf1). For sure, network can set the SMTC window to be 5ms to cover all possibility of od-ssb-PositionsInBurst, but a winder SMTC window will impose more power consumption for UE to perform SSB searching / measurement.

For sure, these issues can be solved if the SMTC values are kept in the OD-SSB configuration, but then since R2 has already decide to go with this direction (moving the SMTC list to MO), R2 has to clarify the related aspects for clarity.

- [X200] R2 clarify the configuration of smtcxlist, in terms of 1) how to handle the case where there is no OD-SSB configuration for a specific od-ssb-Periodicity value, 2) how to adapt the SMTC window based on the different setting of od-ssb-PositionsInBurst.

  - SMTC for Measurement on Neighbour Cell (L201, L202)

During 130, R2 concluded that

(modified) The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated.

When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

It is obvious that measurement behavior would adapt to the (de)activation status of OD-SSB, and to the specific activated OD-SSB configuration.

On the other hand, we had a previous conclusion below, which leads to some confusion, since some always assume a same SMTC window for both serving and neighbor cell(s).

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

If we stick to the ‘same-SMTC’ restriction as in legacy, then

- When AO-SSB and OD-SSB are on the same frequency: A more frequent measurement on neighboring cell would be adopted when a denser OD-SSB is activated. But neighboring cell SSB may not be transmitted as frequently as the denser SMTC, which would further lead to not only unnecessary measurement, but also failure to reach the R4 requirement (since now R4 defines the L3 measurement requirement according to SMTC window periodicity)

- When AO-SSB and OD-SSB are on different frequency(ies): E.g., assume AO-SSB is on F1, while OD-SSB is on F2. On the other hand, both F1 and F2 may have neighboring cell deployed and thus SSB to measure, and those SSB (of neighboring cell) are transmitted independent of OD-SSB (de)activation status of serving cell. And in either F1 or F2, if there is MO configured for neighboring cell measurement, there is always a SMTC to refer to (for F1, it is also the SMTC for serving cell measurement), which should be used for neighboring cell measurement. If UE deviate from this SMTC, the same issue as for same-frequency case above also happens.

Therefore, R4 has already reached the following agreement.

R4#116 Agreement:

•	For SCell measurement, UE shall follow the OD-SSB specific SMTC when OD-SSB is activated; For neighbor cell measurement, UE follows legacy SMTC regardless of status of OD-SSB.

- [L201] To align with R4 conclusion, R2 confirm the adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

Same issue for SSB adaptation. During 129, R2 concluded that

=> RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

It is obvious that measurement behavior would adapt to the (de)activation status of SSB adaptation.

On the other hand, some always assume a same SMTC window for both serving and neighbor cell(s), and lead to the confusion that whether the adaptive SMTC would apply to measurement on neighboring cell

If we stick to the ‘same-SMTC’ restriction as in legacy, then a more frequent measurement on neighboring cell would be adopted when a denser SSB is activated. But neighboring cell SSB may not be transmitted as frequently as the denser SMTC, which would further lead to not only unnecessary measurement, but also failure to reach the R4 requirement (since now R4 defines the L3 measurement requirement according to SMTC window periodicity)

Therefore, R4 has already reached the following agreement.

R4#116 Agreement:

RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation.

For neighbour cell L3 measurement, legacy requirements apply.

- [L202] To align with R4 conclusion, R2 confirm the adaptive SMTC usage for SSB adaptation does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

  - MO for OD-SSB in SSB-less SCell (O006)

In the agreed CR, the presence of MO for OD-SSB relies on the condition as follows

ServingCellConfig ::=               SEQUENCE {

[...]

[[

servingCellMO-OD-r19                MeasObjectId                                                            OPTIONAL    -- Cond Diff_Freq

]]

}

Which is defined as

| Diff_Freq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. |
| --- | --- |

One left issue is how to handle the case when absoluteFrequencySSB of the serving cell is absent, i.e., SSB-less cell.

Based on the R2 conclusion on SSB-less SCell, there is a case where ssbFrequency in servingCellMo is configured

1: 	Introduce per UE capability to indicate whether the UE supports the configuration of servingCellMO for SCell that does not transmit SS/PBCH block. A UE supporting this feature shall also support NR intra-frequency measurements on neighbor cells based on servingCellMO associated with SCell that does not transmit SS/PBCH block. NW can optionally configure servingCellMO for SCell that does not transmit SS/PBCH block if UE indicate such capability.

2: Clarify in the procedure text that that SSB-based serving cell measurements are skipped.

3: Clarify in the field description of servingCellMO that the ssbFrequency value is same as absoluteFrequencySSB only when the serving cell is associated with SSB; if the serving cell is not associated with SSB, the carrier frequency indicated by ssbFrequnecy of the corresponding MeasObjectNR, if configured, is within the frequency range indicated by any entry of the scs-SpecificCarrierList.

4: Clarify the presence condition of ssbFrequency in servingCellMO for a SSB-less SCell that “If the measObject is associated to an SSB-less SCell, this field is optionally present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell, otherwise, this field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If ssb-ConfigMobility is not configured and associatedSSB is not configured for any cell, the field is absent, Need R.”

5: Clarify in stage 2 that SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object indicated by servingCellMO and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

While in that case, i.e., SSB-less Cell, as discussed in section 2.5, the easiest solution is to rely on servingCellMO directly.

| absoluteFrequencySSB | od-ssb-absoluteFrequency | servingCellMO-OD-r19 |
| --- | --- | --- |
| Present | Present | Present if inter-frequency, Absent if intra-frequency |
| Present | Absent | Absent (intra-frequency) |
| Absent | Present | Absent (proposal as in section 2.5) |
| Absent | Absent | Absent |

So that it is suggested to keep the definition of Diff_Freq.

- [O006] R2 confirm servingCellMO-OD is absent for a SCell without legacy SSB (i.e., case-1).

  - Condition for Measurement Gap

Based on the current 300, the condition for MG in case of intra-frequency measurement is specified as follows

-	For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case:

-	If the serving cell is associated with SSB, other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement;

-	If the serving cell is not associated with SSB (i.e. SSB-less SCell), if the initial BWP or any of the UE configured BWPs do not contain the SSB frequency configured in the measurement object associated with the serving cell, and are not configured with NCD-SSB for serving cell measurement.

Obviously, the condition can be summarized as SSB absence in the configured BWP (including or except initial BWP, depending on SSB-based or SSB-less SCell case).

In the Post-131 discussion, there are two interpretations, both referring to the R1 conclusion as follows, but end up with different understanding, i.e., either AO-SSB and OD-SSB are located in the same BWP in specific cases, or they are located in the same BWP in all cases.

Based on R1 conclusion, it is clear that the conclusion applies to all cases without restriction.

R1-2501496	Summary #4 of on-demand SSB for NES	Moderator (LG Electronics)

From Friday session

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Send an LS to RAN4 to inform them of the above agreement.

And based on the latest R1 specification

When the first SS/PBCH blocks in a configured DL BWP can be used to obtain SIB1 and the frequency location of the first SS/PBCH blocks corresponds to the GSCN of a synchronization raster entry, the UE expects

-	a frequency location of the second SS/PBCH blocks to be different from the frequency location of the first SS/PBCH blocks and not to correspond to the GSCN of a synchronization raster entry

-	frequency resources of the second SS/PBCH blocks not to overlap with frequency resources of the first SS/PBCH blocks

-	the second SS/PBCH blocks to be within the configured DL BWP as the first SS/PBCH blocks

-	the second SS/PBCH blocks are not used to obtain SIB1

When the first SS/PBCH blocks in a configured DL BWP cannot be used to obtain SIB1, the UE expects

-	a same frequency location for the second SS/PBCH blocks and for the first SS/PBCH blocks

-	a same PBCH payload, other than the SFN index and the half frame index, for a first SS/PBCH block, from the first SS/PBCH blocks, and for a second SS/PBCH block, from the second SS/PBCH blocks, with same SS/PBCH block index as the first SS/PBCH block

I.e., clearly, AO-SSB and OD-SSB are either on the same BWP (without restriction) or on the same frequency.

So, for the SSB-based SCell, the existing condition can be reused.

- (300 Open Issue) R2 not pursue change on the intra-frequency measurement gap provision condition for SSB-based SCell case, considering R1 conclusion on AO/OD-SSB intra-BWP collocation restriction.

For SSB-less SCell, in case the ssbFrequency is configured in servingCellMO, it is suggested to follow the same restriction as for SSB-based SCell, to avoid further complexity.

Considering R1 has ruled out the case where AO-SSB is an NCD-SSB at sync-raster, it seems simpler to restrict to the case where SSB frequency configured in the measurement object and the SSB frequency for OD-SSB are always the same.

- (300 Open Issue) For SSB-less SCell, R2 not pursue the case where servingCellMO-OD is separately configured, i.e., limit to the case where the ssbFrequency in the servingCellMO (within servingCellConfig) and the SSB frequency for OD-SSB (i.e., od-ssb-absoluteFrequency) are the same.

If so, the impact to measurement gap provision condition can be avoided.

- (300 Open Issue) For the intra-frequency measurement gap provision condition for SSB-less SCell, R2 not pursue further change on measurement gap provision condition.

- Conclusion

Proposal 1	[X200] R2 clarify the configuration of smtcxlist, in terms of 1) how to handle the case where there is no OD-SSB configuration for a specific od-ssb-Periodicity value, 2) how to adapt the SMTC window based on the different setting of od-ssb-PositionsInBurst.

Proposal 2	[L201] To align with R4 conclusion, R2 confirm the adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

Proposal 3	[L202] To align with R4 conclusion, R2 confirm the adaptive SMTC usage for SSB adaptation does not affect the SMTC usage for neighbouring cell, which is to be performed as in legacy.

Proposal 4	[O006] R2 confirm servingCellMO-OD is absent for a SCell without legacy SSB (i.e., case-1).

Proposal 5	(300 Open Issue) R2 not pursue change on the intra-frequency measurement gap provision condition for SSB-based SCell case, considering R1 conclusion on AO/OD-SSB intra-BWP collocation restriction.

Proposal 6	(300 Open Issue) For SSB-less SCell, R2 not pursue the case where servingCellMO-OD is separately configured, i.e., limit to the case where the ssbFrequency in the servingCellMO (within servingCellConfig) and the SSB frequency for OD-SSB (i.e., od-ssb-absoluteFrequency) are the same.

Proposal 7	(300 Open Issue) For the intra-frequency measurement gap provision condition for SSB-less SCell, R2 not pursue further change on measurement gap provision condition.