3GPP TSG RAN WG2 Meeting #131bis            	                    R2-2506994

Prague, Czech Oct 13th – 17th, 2025

Agenda Item:	8.13.2

Source:	Xiaomi

Title:	Discussion on Rel-19 SL MH-Relay RILs

Document for:	Discussion and Decision

Introduction

In the post-RAN2#131 meeting discussion on 38.331 ASN.1 review [1], certain RILs are marked as “Todo” and require submission for online decision-making. Besides, some RILs—even those flagged as “PropReject”—need further discussion to avoid confusion and potential misalignment.

This contribution focuses on discussions of RILs related to procedural texts and ASN.1 components.

Discussion

Discussion on RILs

RIL: X501

For this RIL, the current text only specifies the scenario where the UE itself has SIB request information to provide (when it has not stored any valid version of the requested SIB(s)), while excluding the scenario where the UE receives the requested SIB from its child UE. Therefore, we propose to refine this sentence for clarity, as reflected in the first change attached in the TP section.

Proposal 1: (X501) To clarify that the understanding of “UE has SIB request information to provide” also includes the requested SIB(s) from the child UE, RAN2 is requested to agree to adopt the TP (first change) proposed in R2-2506994.

RIL: X502

RIL: X502 aims to clarify the definition of “paging information” when handling actions related to the transmission of the RemoteUEInformationSidelink message. If this RIL is not adopted, two distinct interpretations of “paging information” will arise. As shown in the current 38.331 specifications, “paging information” (highlighted in yellow) refers to paging messages and other paging-related information (e.g., paging cycle) received from the network in the upstream direction. However, the current 38.331 CR indicates that “paging information” is received from the Child UE in the downstream direction—and there is no evidence in the procedural text to confirm that this “paging information” refers to paging ID/paging cycle (i.e., paging monitoring information). Confusion may arise if parties assume that a parent UE can receive paging messages (also referred to as “paging information”) from a remote UE, which is inherently incorrect. Thus, clarification is recommended to avoid such confusion.

Therefore, it is proposed to replace the term “paging information” with “paging UE ID/paging cycle”.

5.3.2.1 General Figure 5.3.2.1-1: Paging The purpose of this procedure is: - to transmit paging information to a UE in RRC_IDLE or RRC_INACTIVE. - to transmit paging information for a L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE to its serving L2 U2N Relay UE in any RRC state.

Proposal 2: (X502) To avoid confusion regarding “paging information” when handling actions related to the transmission of the RemoteUEInformationSidelink message, RAN2 to clarify “paging information” as “paging UE ID/paging cycle” and agree to adopt the TP (second change) proposed in R2-2506994.

RIL: X503

RIL: X503 is proposed because Figure 5.8.9.8.1-1 only specifies that the relay UE triggers the NotificationMessageSidelink transmission procedure exclusively to the remote UE—this is inconsistent with the subsequent procedural text: “This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE or to the connected child UE”. Consequently, a revision of Figure 5.8.9.8.1-1 is necessary.

Proposal 3: (X503) RAN2 to agree to adopt the TP (third change) proposed in R2-2506994, which revises Figure 5.8.9.8.1-1 to support the transmission of the RemoteUEInformationSidelink message to both the remote UE and the child UE.

RIL: X505

RIL: X505 is proposed due to the following: the mh-indicationType IE is not used in any current procedural text. The simplest way to resolve this issue is to remove the prefix of “mh-indicationType-r19” (i.e., revise it to “indicationType-r19”) to align with legacy IE naming conventions. If this adjustment is not made, additional procedural texts will need to be added to distinguish between “mh-indicationType” and “indicationType”. Therefore, the proposal is as follows:

Proposal 4: (X505) RAN2 is requested to agree to adopt the TP (fourth change) proposed in R2-2506994, which removes the prefix of the “mh-indicationType” IE.

Conclusion

In this contribution, we propose:

Proposal 1: (X501) To clarify that the understanding of “UE has SIB request information to provide” also includes the requested SIB(s) from the child UE, RAN2 is requested to agree to adopt the TP (first change) proposed in R2-2506994.

Proposal 2: (X502) To avoid confusion regarding “paging information” when handling actions related to the transmission of the RemoteUEInformationSidelink message, RAN2 to clarify “paging information” as “paging UE ID/paging cycle” and agree to adopt the TP (second change) proposed in R2-2506994.

Proposal 3: (X503) RAN2 to agree to adopt the TP (third change) proposed in R2-2506994, which revises Figure 5.8.9.8.1-1 to support the transmission of the RemoteUEInformationSidelink message to both the remote UE and the child UE.

Proposal 4: (X505) RAN2 is requested to agree to adopt the TP (fourth change) proposed in R2-2506994, which removes the prefix of the “mh-indicationType” IE.

Reference

[1] ASN.1 review of sidelink MH-relay

Annex TP

Start of first Change

5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall: 1> if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 or SIB(s) requested by child UE and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before):

End of first Change

Start of second Change

##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message Omitted... 1> if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows: 2> if the L2 U2N Remote UE is in RRC_IDLE: 3> include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE; 3> if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer; 2> else if the L2 U2N Remote UE is in RRC_INACTIVE: 3> include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE; 3> if the UE specific DRX cycle is configured by upper layer, 4> set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC); 3> else: 4> set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC; 2> if any paging UE ID/paging cycle is received from the Child UE: 3> include the received paging information in the sl-PagingInfo-RemoteUE-List;

End of second Change

Start of third Change

Figure 5.8.9.8.1-1: Notification message in sidelink This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE or to the connected child UE, or used by a L2 U2U Relay UE to send notification to the L2 U2U Remote UE for an end-to-end PC5 connection when condition(s) as specified in 5.8.9.10.2 is met for the hop between the L2 U2U Relay UE and the peer L2 U2U Remote UE.

End of third Change

Start of fourth Change

NotificationMessageSidelink message -- ASN1START -- TAG-NOTIFICATIONMESSAGESIDELINK-START NotificationMessageSidelink-r17 ::= SEQUENCE { criticalExtensions CHOICE { notificationMessageSidelink-r17 NotificationMessageSidelink-r17-IEs, criticalExtensionsFuture SEQUENCE {} } } NotificationMessageSidelink-r17-IEs ::= SEQUENCE { indicationType-r17 ENUMERATED { relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection, relayUE-Uu-RRC-Failure } OPTIONAL, -- Need N lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension NotificationMessageSidelink-v1800-IEs OPTIONAL } NotificationMessageSidelink-v1800-IEs ::= SEQUENCE { sl-IndicationType-r18 ENUMERATED {relayUE-PC5-RLF, spare1} OPTIONAL, -- Need N sl-DestinationIdentityRemoteUE-r18 SL-DestinationIdentity-r16 OPTIONAL, -- Need N nonCriticalExtension NotificationMessageSidelink-v19xy-IEs OPTIONAL } NotificationMessageSidelink-v19xy-IEs ::= SEQUENCE { indicationType-r19 ENUMERATED { relayUE-RelayReselection, relayUE-CellSelection } OPTIONAL, -- Need N nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-NOTIFICATIONMESSAGESIDELINK -STOP -- ASN1STOP

End of fourth Change