3GPP TSG-RAN WG2 Meeting #131bis	R2-2507046

Prague, Czech Republic, Oct. 13th - 17th, 2025

Agenda Item:	8.9.2

Source:	OPPO

Title:	Discussion on RRC open issues for IoT NTN TDD

Document for:	Discussion, Agreement

Introduction

In this contribution, we discuss RRC open issues for IoT NTN and present our view.

Discussion

- V211

| - RIL Id | - WI | - Class | - Title | - Tdoc | - Delegate | - Misc | - File version | - Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - V211 | - IoTNTN | - 1 | - Indicate the regenerative operation mode to NAS | - Yes, R2-250xxxx | - vivo (Stephen) |  | - v003 | - ToDo |

[Description]: As per the current spec, when t-ModeSwitching is not configured for the transition from S&F to regenerative, upon the satellite transitions from S&F to regenerative, no mode indication of regenerative mode is indicated to NAS. The NAS may always consider the cell is in S&F mode. It is incorrect.

[Proposed Change]: We propose that the RRC of an S&F capable UE shall indicate the cell is operating in regenerative mode if sf-OperationMode is not present. Alternatively, t-ModeSwitching shall always be configured by the network when the cell is operating in S&F.

- [Analysis]

- Based on the current running CR, the field description of sf-OperationMode and t-ModeSwitching are as below.

sf-OperationMode - Indicates that the cell is operating in Store and Forward mode. If this field is present, UEs supporting the Store and Forward operation ignores cellBarred-NTN and cellBarred. Value 'barred' means the cell is barred for NTN connectivity with the Store and Forward operation, as defined in TS 36.304 [4]. Value 'notBarred' means the cell allows UEs supporting the Store and Forward operation to access. If this field is absent, the NTN cell is operating in normal mode, i.e., not in the Store and Forward mode and UEs supporting the Store and Forward operation follow cellBarred-NTN.

t-ModeSwitching - If sf-OperationMode is present in SIB1, this field indicates the time information on when an NTN cell is going to switch from the Store and Forward Satellite operation mode to the normal mode; otherwise, this field indicates the time information on when an NTN cell is going to switch from the normal mode to the Store and Forward Satellite operation mode.

- And according to the current running CR, for sf-OperationMode, UE AS shall indicate to NAS that the cell is operating in S&F mode, if sf-OperationMode is present. For t-ModeSwitching, UE AS shall forward the t-ModeSwitching to NAS, if t-ModeSwitching is present. Thus, if network switches from S&F mode to normal mode, and t-ModeSwitching is absent, NAS will not be aware of this as there is no indication from AS to NAS in this case. So we think V211 is a valid issue.

- Regarding the proposed change, we think the latter option, i.e., t-ModeSwitching is always configured by the network when the cell is operating in S&F mode mode, is more simple.

For a NTN cell operating in S&F mode, t-ModeSwitching shall be always configured by network.

- S901

| - RIL Id | - WI | - Class | - Title | - Tdoc | - Delegate | - Misc | - File version | - Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - S901 | - IoTNTN | - 2 | - Clarification on TA report | - Yes, R2-250xxxx | - Samsung (Jonas) |  | - vXXX | - ToDo |

[Description]: TA report is according to the RRC procedures applicable for CB-Msg3-EDT (although the field description seems to indicate that it is not applicable to CB-MSg3-EDT).

The CB-Msg3-EDT being applicable to CP solution is likely not an issue as the MAC procedures will not include the TA report if the CB-Msg3 TBS allocation is too small. However, for UP solution, the TA report has higher priority compared to DCCH, meaning that the MAC CE may use unnecessarily occupy data. This is because the ta-Report applies to all types of access procedures.

Also, the arguing against this because this was not addressed for PUR is not valid, as the usefulness of PUR in NTN is questionable. CB-Msg3-EDT is introduced specifically for NTN, so it is in our interest to introduce protocols and procedures that make these procedures as efficient as possible.

[Proposed Change]: TA report is not triggered for the CB-Msg3-EDT procedures or a ta-Report configuration specifically for CB-Msg3-EDT is introduced.

- [Analysis]

We share the same view as above that if the field ta-Report in SIB is also applied to CB-Msg3 EDT, then for UP solution, UE may not be able to complete the data transmission due to the transmission of TA report MAC CE. As TA reporting is mainly used by network to configure UE with a UE-specific K offset, and CB-MSg3-EDT is usually one-shot transmission, there seems no strong motivation to require TA report via Msg3. Based on this, we think TA report should not be triggered for the CB-Msg3-EDT procedures

RAN2 agrees that ta-Report in SIB is not appliable to CB-Msg3-EDT

- Conclusion

Based on the discussion we propose the following:

- For a NTN cell operating in S&F mode, t-ModeSwitching shall be always configured by network.

- RAN2 agrees that ta-Report in SIB is not appliable to CB-Msg3-EDT