# GPP TSG RAN WG2 Meeting #131bis      	                        R2-2507102
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	9.2.2

Source:	Apple

Title:	Discussion on Topology 2

WID/SID:	Ambient_IoT_Solutions_Ph2 – Release 20

Document for:	Discussion and Decision

# 1 Introduction

TR 38.769 [1] has provided a logical system architecture for Ambient-IoT Topology 2. As shown in Figure 1, the “Common reader function” is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

Figure 1: Logical system architecture for Topology 2

It has also defined the A-IoT Enabled UE as “A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.”

In this paper, we discuss the operation of this “A-IoT Enabled UE” (a. k. a. UE reader) in Topology 2.

# 2 Discussion

## 2.1 RRC State of UE reader

For RRC_CONNECTED state UE reader, it is evident that this is supported. For RRC_INACTIVE state UE reader, MO-SDT and MT-SDT mechanisms introduced for 3GPP Rel-17/Rel-18 for IoT systems can be reused to deliver traffic from UE reader to A-IoT Function via Uu interface when the A-IoT enabled UE is in INACTIVE state. This is because it is assumed to have very light Uu traffic for A-IoT communication in most cases. Also, for RRC_CONNECTED UE reader temporary out of connection scenarios (RLF, HO, etc.), this is also can be considered to enhance the robustness of A-IoT operation. Given that this UE reader will be back in-coverage or back to CONNECTED state soon, it should be no problem for the UE reader to deliver the A-IoT data received during this temporary period to A-IoT Function with some short delay. Hence, it is reasonable to support this temporary out of connection scenario.

Proposal 1 	Support UE reader operation in RRC_CONNECTED, RRC_INACTIVE or temporary out of connection scenarios (e.g., RLF or handover).

However, for RRC_IDLE state UE reader, it has been agreed to be de-prioritized in the study and only consider this case if there is minimum specification impact. We think the problem with IDLE state UE reader is that if the gNB wants to cancel or re-configure the A-IoT radio resource allocated to this UE reader, the gNB need triggers a CN-based paging. This will introduce additional complexity in signaling design. Also, for RRC_IDLE UE reader to deliver the A-IoT data to CN, there is no scheme like SDT to be used for IDLE state UE reader, significant RRC procedure overhead and delay will be added, too. Hence, we think it is better to rule out this scenario in Rel-20.

Proposal 2	UE reader operating in RRC_IDLE state is not supported in Rel-20.

## 2.2 A-IoT resource allocation to UE reader

During the study phase, it has been agreed that the radio resources are only dedicated for a single UE reader in Topology 2 w/o shared resource allocation. And they are only configured to the UE reader via dedicated signaling. It is also agreed that resource allocated in one cell will be never valid in other cell(s), at least in “this release”[1]. Since, we did not complete topology 2 normative work in Rel-19, we think it is better to re-confirm that cross-cell validity of A-IoT resource is not supported in Rel-20.

Proposal 3 	RAN2 confirms that cross-cell A-IoT radio resource allocation validity is not supported in Rel-20.

Then, regarding the options for A-IoT resource semi-static allocations studied in Rel-19, RAN2 need to down-prioritize those options in normative work. First, given that the radio resource validity is already limited to a single cell, there is no strong motivation to further limit the radio resource validity in time domain. Anyway, NW always has the option to cancel the current resource allocation by sending a RRCReconfiguration message. Therefore, we think Option 1 should be used for A-IoT reader in both RRC_CONNECTED state and RRC_INACTIVE state.

Proposal 4 	For A-IoT semi-static resource allocation for IDLE and INACTIVE UE reader, Option 1 is used.

But it is a different situation for the “temporary out of connection scenarios” As the scenario(s) are supposed to be temporary, it is logical to have a time constrain of the resource validity. Also, UE reader in those cases may already in the border area of the cell or even moves out of coverage, so it is not proper to allow the resource to be valid for an indefinitely long time. Hence, the timer-based Option 2 is more appropriate.

Proposal 5 	For A-IoT resource allocation for temporary out of connection scenarios,  Option 2 is used.

Given the above considerations, an RRC_CONNECTED UE reader may get the allocated resource associated with timer (Option 2) and/or resource allocated w/o timer (Option 1). Whether those resource can be configured in the same signaling can be further discussed later, but the Option 2 timer is supposed to be used only when temporary out-of-connection event occurs.

Proposal 6 	For A-IoT resource allocation with a timer-based validity for temporary out of connection scenarios, UE starts the timer only when Uu RLF is detected or HO command is received.

## 2.3 UE reader Assistance information to A-IoT enabled gNB

The following signalling flow diagram for inventory procedure in Topology 2 for NAS/UP based solution is included in section 6.5.3.1.2 of TR 38.769[1], as shown in Figure 2.

Figure 2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

In the above diagram, the CN “inventory request” signalling for inventory procedure will trigger the corresponding AS layer procedure (e.g. Paging and Random Access) in A-IoT UE reader, as shown in Step 3, and the inventory results will be eventually reported back to A-IoT CN by the A-IoT UE reader. For command procedure, there will be similar interaction between CN and UE reader. However, the high-level signalling flow in Figure 2 has omitted necessary messages interaction with gNB, which may occur right ahead of step 3, or even earlier to solicit proper configuration for UE reader’s operation in A-IoT air interface.

Resource Configuration assistance

It is agreed that NW is in control of resources to be used by AIoT air interface. To be more specific, the resources are supposed to be configured by the serving gNB of UE reader. To make this configuration be effective and accurate, some assistance information from UE reader needs to be shared with A-IoT enabled gNB, For example, if gNB allocated dedicated R2D rand D2R resource (periodically) to the UE reader, the UE reader need report the expected A-IoT traffic amount in both directions and the number of intended devices (e.g., to participate in slotted aloha contention), so that the gNB can dimension the resource correspondingly.

CW node assistance

For example, to make step 3 work, the A-IoT UE reader need coordinate CW generation to support backscattering (for device Type 1/Type 2a) if the UE reader itself is not a CW node.  The CW node is usually co-located/managed by gNB (e.g., in-topology or out-of-topology). So, it is necessary for the UE reader to send an indication of the expected A-IoT air interface operation to allow NW to configure CW support. Note that even if UE itself is a CW node (in Topology 2), it may have some internal conflict for UE to generate CW in CW band and transmit/receive A-IoT air interface at the same time due to RF constraint, so NW assistance for CW generation from additional CW nodes may still be desired in this case.

To make the above information available to the gNB, we think it is important for UE to send an UL RRC message, which can be either UEAssistanceInformation or a new RRC message dedicated for A-IoT use case.

Proposal 7	Use UAI or a new RRC message for UE reader to indicate the resource configuration assistance information and/or CW assistance information.

## 2.4 SIB Enhancements for Topology 2

One thing is to consider SIB indication of the support of this function by A-IoT-enable gNB. This will help mobile UE reader choose a suitable cell for the reader operation, as we can’t assume all the gNBs will be upgraded to support A-IoT in Topology 2 scenarios. It is also evident that this indication is not equal to the “reader function” support in Topology 1, because an AIoT-enabled gNB may not be able to act as a reader in Topology 1. So far, there is no obvious need for a SIB change to indicate a “reader gNB”. Furthermore, such indication would be needed to support mobility in RRC_INACTIVE (if supported).

Proposal 8 	Consider SIB indication of Topology 2 support by A-IoT enabled gNB, which does not necessarily support reader function.

## 2.5 Uu MAC Enhancements for Topology 2

Based on the discussion in Section 2.4, NW (A-IoT enabled gNB) may allocate R2D and/or D2R resources for the UE reader operation in Topology 2. Besides using dedicated RRC signalling to configure A-IoT grants, L1/L2 indication of those A-IoT resources in Uu interface are also possible and should be considered.  From the UE reader perspective, BSR-like procedure may be needed for solicit A-IoT grants. This is because the A-IoT traffic initiated by a CN A-IoT function may be transparent to gNB and serving gNB is not aware of actual traffic situation in the UE reader, in general. This is similar to the Sidelink BSR design [4] where the grants of another radio interface are solicited by a BSR procedure.

Proposal 9	Define new BSR MAC CEs in Uu to support A-IoT reporting buffered R2D/D2R data.

## 2.6 RAN2 impact of “NGAP + RRC” approach

RRC-based approach is one of the three options to be considered for the transport of A-IoT data for Topology 2. It is important for RAN2 to have a clear idea on the potential RAN2 impact. It is preferred to limit the RRC impact as much as possible for supporting this approach. For example, the RRC Container shall be used to encapsulate A-IoT upper layer data. However, regarding the visibility information, such as “estimated response message size” or “estimated number of devices” or “service type” information, it is unclear whether they will be encoded as new RRC message in ASN.1 format or can also be included in RRC container. So, we think RAN2 need to clarify this in the SI phase.

Proposal 10 	RAN2 to clarify whether there is ASN.1 impact (e.g., new RRC message or new IE in existing RRC messages) to be introduced to support “NGAP + RRC message” for Topology 2.

# 3 Conclusion

In this contribution, we discuss the Topology 2 support for Ambient IoT, and have the following proposals:

Proposal 1 	Support UE reader operation in RRC_CONNECTED, RRC_INACTIVE or temporary out of connection scenarios (e.g., RLF or handover).

Proposal 2	UE reader operating in RRC_IDLE state is not supported in Rel-20.

Proposal 3 	RAN2 confirms that cross-cell A-IoT radio resource allocation validity is not supported in Rel-20.

Proposal 4 	For A-IoT semi-static resource allocation for IDLE and INACTIVE UE reader, Option 1 is used.

Proposal 5 	For A-IoT resource allocation for temporary out of connection scenarios,  Option 2 is used.

Proposal 6 	For A-IoT resource allocation with a timer-based validity for temporary out of connection scenarios, UE starts the timer only when Uu RLF is detected or HO command is received.

Proposal 7	Use UAI or a new RRC message for UE reader to indicate the resource configuration assistance information and/or CW assistance information.

Proposal 8 	Consider SIB indication of Topology 2 support by A-IoT enabled gNB, which does not necessarily support reader function.

Proposal 9	Define new BSR MAC CEs in Uu to support A-IoT reporting buffered R2D/D2R data.

Proposal 10 	RAN2 to clarify whether there is ASN.1 impact (e.g., new RRC message or new IE in existing RRC messages) to be introduced to support “NGAP + RRC message” for Topology 2.

# 4 References

[1] TR38.769 v19.0.0 Study on solutions for Ambient IoT (Internet of Things) in NR