3GPP TSG-RAN WG2 Meeting #131bis	R2-2507111

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.3.2

Source:	Apple

Title:	Views on Directions of 6G Control Plane Enhancements

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

According to SID (RP-252912), the design of 6G radio protocol should be simple, efficient, and prioritize optimal user experience. The 6G radio designs also need to be implementation-friendly and address issues identified in 5G deployments.

This contribution gives some of our initial views on the directions of RAN2 Control Plane study from these perspectives.

# General principle

6G Control Plane study should be based on NR RRC design and focus on NR RRC functions.

NR RRC layer provides the following RRC functions:

- RRC state

- System information

- Paging

- Access control

- RRC connection management (i.e. establishment, resumption, release, maintenance)

- Security function including key management

- SRB and DRB management (i.e. establishment, configuration, maintenance, release)

- QoS handling

- Mobility

- Measurement

- UE capability

- NAS message transport.

- Radio link detection and recovery

- Spectrum aggregation related management, e.g. CA, DC

Amongst all the RRC functions, SRB/DRB management and QoS handling rely on UP design; NAS message transport relies on NAS design; the security function should be designed together with SA3; and mobility & measurement& link recovery will be discussed in the separate A.I.

From RAN2 perspective, the study of 6G control plane will focus on the remaining RRC functions. Besides these NR RRC functions, the NR RRC configuration model should be also revisited.

Proposal 1: The study of 6GR Control Plane should focus on RRC configuration model, RRC state, System information, Paging, RRC connection management, Access control, AS security management (together with SA3), UE capability, and spectrum aggregation related management.

According to the 6GR SID (RP-252912), the 6G radio design should be designed from the perspectives of simplicity, spectrum efficiency, power efficiency, and user experience improvement. Therefore, the 6GR Control Plane should be also designed from these perspectives.

Proposal 2: The study on 6GR control plane protocol should take NR design as starting point and be designed from the perspectives of simplicity, spectrum efficiency, power efficiency, and improved user experience.

# Control Plane Functions

## RRC configuration model

In NR, all configurations for UE dedicated transmission are provided from RAN nodes to UE via RRC signaling. And For each RRC (re)configuration, network must provide UE with complete configuration information.

Based on the current 5G deployment and implementation, the NR RRC configuration model has the following aspects that need to be improved and can be studied in 6G.

- RRC Configuration Latency

In CU-DU split architecture, the RRC entity is located in CU, so all RRC configurations are always provided by CU, regardless of whether the configuration is generated/decided by CU or DU. Regarding the lower layer RRC configuration (which may include MAC and PHY configuration) which is decided by the serving DU, DU has to forward it to CU and let CU to generate and transmit the RRC (re)configuration to UE, which introduces the additional and unnecessary RRC configuration delay. For the network architecture without CU-DU split, if network implementations adopt the similar approach, the additional latency in the lower layer configuration is unavoidable.

Judging from the components of the complete RRC (re)configuration message, the configuration provided by CU usually does not change frequently, while the configuration provided by DU changes frequently.

Observation 1: In NR CU-DU split like architecture, the lower-layer configuration generated in DU can only be sent to CU first and then sent to UE by CU. This increases the configuration latency for lower layer configuration.

If one complete RRC configuration can be split into multiple parts and RAN node generating the partial configuration can send it directly to the UE, the lower-layer configuration latency issue in CU-DU split like architecture can be addressed.

Figure 1: Illustration of DU directly generating and transmitting L1 configuration

- RRC Configuration Size

In NR, with the introduction of more and more features, the NR RRC (re)configuration message size has increased significantly over time, so later the RRC segmentation method was agreed to be applied to RRC (re)configuration message transmission. The transmission of a large-sized RRC (re)configuration message requires more resources and longer UE RRC processing time.

Because network determines UE specific configuration primarily based on UE capabilities and network deployments, most configuration parameters remain the same for UEs with the same capabilities and in the same deployment. This is indeed the case in actual NR deployments.

If UE can know the unchanged configuration in advance, network only provides the configuration of the changed part, which can greatly reduce the RRC (re)configuration message size and shorten the RRC processing time in UE side.

Observation 2: Most of UE dedicated configurations are same for UEs with the same capabilities in the same deployment.

Observation 3: By providing partial/delta configuration based on the reference configuration, the RRC message size can be reduced, thereby saving the system resources and reducing UE processing time.

Figure 2: Illustration of RRC configuration size minimization based on reference configuration

- RRC Configuration Validity

In NR LTM mechanism, network can provide multiple LTM candidate cell configurations with complete configuration so that subsequent cell switch procedure does not involve any RRC message. Here, since the interval between the candidate cell configuration provision and actual cell switching may be long, for some lower-layer resource related configurations, the pre-provided configuration may not remain valid until the cell switch occurs. In this case, the RRC involvement during LTM procedure is inevitable (contrary to the LTM design philosophy), and storing a large number of outdated configurations is meaningless and wastes UE memory.

To address this issue, we can consider having network provide only the unchanged part of the candidate cell configuration (e.g. L1 configuration) in advance, and the potential changed part can be given directly in cell switch command.

Observation 4: During LTM/HO procedure, it is meaningless to provide the parts of the candidate cell configuration that may change in advance.

Figure 3: Illustration of the 2-step candidate cell config provision to ensure configuration validity during LTM

Proposal 3: RAN2 study on 6G RRC configuration model focuses on the following aspects:

- 1) Reducing configuration latency (e.g. RAN node that decides the config can directly send it.)

- 2) Reducing RRC configuration size (e.g. based on reference configuration)

- 3) Ensuring configuration validity (e.g. Do not provide configurations that may change in advance)

## RRC state

For NR RRC states, the key characteristics and difference are summarized as follows:

In 6G, the RRC state design needs to support the following 3 scenarios:

For scenarios 1 and 2, RRC states corresponding to NR RRC_IDLE (1) and RRC_CONNECTED states are required.

For scenario 3, the RRC state can be designed like either NR RRC_IDLE (2) or RRC_INACTIVE state. Regarding the CP latency requirement defined in TR38.914(as below), only the NR RRC_INACTIVE state like design can meet the requirement, in which only AS connection resumption is required when transitioning from data inactive state to data active state. From simplification perspective, we donot need to have two RRC states to address the same scenario. Therefore, for scenario 3, we should focus on using only one state to support scenario 3 while meeting the 6G CP latency requirement, which can be designed based on RRC_INACTIVE state.

Proposal 4: RAN2 study on 6G RRC state focus on supporting only one state for each of the following scenarios:

- 1) When network/CN has no UE context, the study is based on NR RRC_IDLE.

- 2) When network/RAN has UE context and UE has data activity, the study is based on NR RRC_CONNECTED.

- 3) When network has UE context and UE has no data activity, the study is based on NR RRC_INACTIVE.

Figure 4: Illustration of the scenario and RRC state (transitions)

## System information

In NR, system information is divided into minimum SI and other SI. Their main characteristics are shown in the table below.

From the perspective of power/energy saving and resource efficiency improvement, the most significant features of NR SI design is the introduction of two features: 1) on-demand broadcast and 2) area specific SI (or stored SI).

- By using on-demand broadcasting design, network does not need to transmit SI all the time when there is no UE need, which can save system resources and network energy.

- By using area specific SI (i.e. stored SI) design, UE does not need to reacquire SI every time it performs cell reselection, which can save UE power.

However, these two features cannot be applied to Minimum SI in NR, because Minimum SI provides information related to initial access, and this configuration will be different in each cell. And later from network energy saving perspective, the on-demand SIB1 is introduced in NES specific cell in R19 NES WI.

In 6G, considering the benefits of these two features, we should inherit the two features in 6G design and study the possibility of applying them to Minimum SI. Regarding the Minimum SI, MIB is transmitted on PBCH and the design depends on RAN1 study, so RAN2 study can first focus on SIB1.

- Regarding on-demand SIB1 broadcast direction, since NR already supports it in NES cells for NES UE in R19, supporting it in 6G cells is straightforward.  Supporting it from 6G Day1 ensures that all UEs will support it without any backward compatibility issues. This will also make the design much simpler.

- Regarding the area specific SIB1 direction, according to the configuration in the field deployment, it can be found that the configuration in SIB1 that are not related to the initial access is basically the same within a certain area.

- If the configuration in NR SIB1 can be further divided into a cross-cell part (e.g. cellAccessRelatedInfo cellSelectionInfo, si-SchedulingInfo) and a cell-specific part (e.g. RACH-ConfigCommon), the cross-cell part can be configured at the area level. When UEs move within the area but do not trigger initial access, the SIB1 acquisition per cell reselection can be skipped, thereby saving UE power and shortening cell reselection delay.

Figure 5: Example of the UE operation on SIB1 acquisition with area based SIB1

Proposal 5: RAN2 study on 6G system information focuses on the feasibility to support the on-demand SIB1 acquisition and the area based SIB1 (i.e. stored SIB1).

## Paging

In NR, paging allows network to reach RRC_IDLE and in RRC_INACTIVE UEs through Paging messages, and to notify UEs of system information change and ETWS/CMAS indications through Short Messages (transmited on the DCI used to scheduling the paging messages). The paging (short) messages are scheduled and transmitted in paging occasions (i.e. PO).

From system point of view, NR only supports uniform PO distribution to ensure load balancing, but it’s not good for network energy saving. In later NR NES discussion, the non-uniform PO distribution was proposed but was not agreed mainly due to backward compatibility issues. Therefore, since the 6G Day1 design does not need to consider backward compatibility issues, the 6G paging study from the network energy efficiency perspective can consider the non-uniform PO distribution direction.

Figure 6: Illustration of the benefits of non-uniform PO distribution on network energy saving

From UE point of view, the following features of the NR paging design are introduced for UE implementation simplification and for UE power saving, which should be inherited and enhanced in 6G paging design.

- 1) Only one PO set for UE specific paging monitoring

In NR, UE monitors the CN and/or RAN initiated UE specific paging message on the UE specific PO which is calculated based on UE ID and the paging DRX cycle. CN-initiated paging and RAN-initiated paging use different paging DRX cycles, which results in the INACTIVE UE possibly using different, non-overlapping POs to monitor different types of paging, thereby increasing the UE complexity.

This issue was acknowledged and addressed in NR later release to ensure that UEs only use one PO set to monitor UE-specific paging. In 6G design, we should also ensure that only one PO set is used for UE to monitor the UE specific paging.

- 2) Subgroup and WUS based paging mechanism

To reduce UE power consumption due to false paging alarms, the group of UE monitoring the same PO can be further divided into multiple subgroups. With subgrouping, UE shall monitor PDCCH in its PO for paging if the subgroup to which the UE belongs is paged as indicated via associated WUS signaling. The paging subgroup of one UE can be either allocated by CN via NAS registration procedure or calculated based on cell specific subgrouping configuration.

NR introduces two WUS based paging features. In R17, PEI based paging mechanism was introduced, where PEI was used as the WUS signal and the design is based on the PDCCH DCI. In R19, LP-WUS based paging mechanism was introduced, where LP-WUS was used as the WUS signal and the design is based on new signal.

Because the paging frameworks based on PEI and LP-WUS are very similar and are mainly used for different UE device types (different HW requirements), in 6G design, we should also consider the unified subgroup and WUS based paging mechanism, and the study on WUS signal design can be left to RAN1.

Figure 7: Illustration of Subgroup and WUS based paging framework

- 3) Paging cause/voice indication in UE specific paging message

For MUSIM UE, the voice indication in paging message can let UE know if the incoming MT page is for voice call or data, thus helping UE decide whether to consider the network switch between two SIM instances.

MUSIM UE will still exist in 6G, so this voice indication will still be necessary for the same purpose. Furthermore, if paging may provide richer information about MT paging, it will provide more assistance information for subsequent UE paging processing which can improve the user experience.

Proposal 6: RAN2 study on 6G paging focuses on network energy saving (e.g. non-uniformed PO distribution), UE power saving (e.g. subgroup and WUS based paging), UE implementation simplification (e.g. single PO set to monitor paging), and user experience improvement (e.g. carrying richer MT paging info).

## Spectrum aggregation (multi-CC) related management

<Initial access>

In field deployments, operators may deploy multiple CCs in the same area for different purposes. A typical multi-CC deployment is an overlapping deployment of coverage frequency layer and capacity frequency layer:

- The coverage frequency layer is in lower frequency band and used for IDLE/INACTIVE UE to camp on.

- The capacity frequency layer is in higher frequency band and mainly used to for UE dedicated transmission.

- NOTE: The cells on capacity layer have larger bandwidth and smaller coverage than the cells on coverage layer.

Figure 8: Example of the coverage layer + capacity layer multi-CC deployments

In this deployment, from UE point of view, UEs are expected to camp on cells on the coverage layer to avoid frequency cell reselection and performs the data transmission via the cells on the capacity layer to have the better transmission service. From network point of view, all IDLE/INACTIVE UEs camping on the same frequency layer can help network reduce the number of CC used to paging transmission, thereby saving network resources and energy used for paging.

According to current NR design, the IDLE/INACTIVE UE performs the initial access on the camping cell. Assuming all IDLE/INACTIVE UEs camp on coverage frequency layer (F1), if network would like to provide the service to CONNECTED UE via capacity layer (i.e. F2), UE will first perform the initial access procedure and establish the RRC connection on F1 and then network provides the configuration to 1) handover the UE from F1 cell to F2 cell or 2) add F2 as SCell, as shown in Figure 9.

Throughout the entire procedure from initial access to switching CC for connected data transmission, the following issues can be identified:

- The initial access congestion is likely to occur in the coverage layer, as all initial access procedures are performed there.

- The procedure of switching to the cell on capacity layer for UE dedicated transmission has a long delay and requires high signaling overhead.

Figure 9: NR procedures for adding Cell#2 (on capacity layer) as a serving cell for data transmission

In 6G design, to address the above issues in the "coverage layer + capacity layer" multi-CC deployment, RAN2 should study the cross-CC initial access procedure to shorten the latency of starting the capacity layer to provide services to UEs, as shown in Figure 10. That is, a UE not in CONNECTED state can camp on the coverage CC and when entering CONNECTED state, switch to the capacity CC during the initial access procedure.

Figure 10: Illustration of the UE operation during the cross-CC initial access procedure

For 6G cross-CC initial access design, RAN2 study can focus on the following aspects:

- The selection of CC and cell on capacity layer for initial access and UE transmission in CONNECTED state.

- Potential impact on the access control and paging message.

- Method for providing system information related to the cross-CC initial access on capacity layer.

- The specific time point of CC switching during the initial access, e.g. before initiating access, during initial access (at which step?), or immediately after the initial access.

Proposal 7: RAN2 study on 6G initial access procedure focuses on supporting the cross-CC initial access procedure (including paging, SIB provision, access control, RACH procedure, etc.).

<6G CA>

Regarding the spectrum aggregation in CONNECTED state, according to the following RANP#109 agreements (RP-252909), 6G RAN will focus on MRSS+CA deployment.

- The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

In NR CA, PCell is typically deployed on coverage layer and undertakes many special functions in CA connection, e.g. management and recovery of RRC connection, and transmission of important control signaling, always activated. The main functions supported on PCell and SCell are briefly summarized in the following Table.

Table. The functions supported on PCell and SCell in NR CA

In MRSS+ CA deployment, on coverage frequency layer, the frequency resources are shared between 5G RAT and 6G RAT, and there are not many resources available for UE dedicated data transmission.

For the case that PCell is configured on coverage layer, PCell can only be used to provide the coverage and basic connection management (e.g. link/connection recovery), and the SCell/SCC on the capacity layer should play a more important role in connection management related to data transmission.

For the case that PCell is configured on capacity layer and SCell is configured on coverage layer, PCell has the smaller coverage than SCell. so that it is very likely that PCell has radio problem but SCell ​​works well. If NR CA design is followed, UE will initiate RRC connection re-establishment procedure, which will seriously affect the current UE RRC connection.

Since the CA situation in MRSS+CA deployment is different from the traditional NR CA deployment and is more dependent on SCell/SCC, it is necessary to study whether SCC/SCell can support more connection management functions.

Proposal 8: RAN2 study on 6G CA focuses on how to better support the MRSS+CA deployment (i.e. different CCs for coverage and main data transmission).

# Conclusions

This contribution presents some of our initial views about the directions that RAN2 should study for 6G Control Plane. We have the following proposals:

Proposal 1: The study of 6GR Control Plane should focus on RRC configuration model, RRC state, System information, Paging, RRC connection management, Access control, AS security management (together with SA3), UE capability, and spectrum aggregation related management.

Proposal 2: The study on 6GR control plane protocol should take NR design as starting point and be designed from the perspectives of simplicity, spectrum efficiency, power efficiency, and improved user experience.

Observation 1: In NR CU-DU split like architecture, the lower-layer configuration generated in DU can only be sent to CU first and then sent to UE by CU. This increases the configuration latency for lower layer configuration.

Observation 2: Most of UE dedicated configurations are same for UEs with the same capabilities in the same deployment.

Observation 3: By providing partial/delta configuration based on the reference configuration, the RRC message size can be reduced, thereby saving the system resources and reducing UE processing time.

Observation 4: During LTM/HO procedure, it is meaningless to provide the parts of the candidate cell configuration that may change in advance.

Proposal 3: RAN2 study on 6G RRC configuration model focuses on the following aspects:

- 1) Reducing configuration latency (e.g. RAN node that decides the config can directly send it.)

- 2) Reducing RRC configuration size (e.g. based on reference configuration)

- 3) Ensuring configuration validity (e.g. Do not provide configurations that may change in advance)

Proposal 4: RAN2 study on 6G RRC state focus on supporting only one state for each of the following scenarios:

- 1) When network/CN has no UE context, the study is based on NR RRC_IDLE.

- 2) When network/RAN has UE context and UE has data activity, the study is based on NR RRC_CONNECTED.

- 3) When network has UE context and UE has no data activity, the study is based on NR RRC_INACTIVE.

Proposal 5: RAN2 study on 6G system information focuses on the feasibility to support the on-demand SIB1 acquisition and the area based SIB1 (i.e. stored SIB1).

Proposal 6: RAN2 study on 6G paging focuses on network energy saving (e.g. non-uniformed PO distribution), UE power saving (e.g. subgroup and WUS based paging), UE implementation simplification (e.g. single PO set to monitor paging), and user experience improvement (e.g. carrying richer MT paging info).

Proposal 7: RAN2 study on 6G initial access procedure focuses on supporting the cross-CC initial access procedure (including paging, SIB provision, access control, RACH procedure, etc.).

Proposal 8: RAN2 study on 6G CA focuses on how to better support the MRSS+CA deployment (i.e. different CCs for coverage and main data transmission).

# References

- RP-252912	Study on 6G Radi