3GPP TSG-RAN WG2 Meeting #131bis	R2-2507112

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.7.3.2

Source:	Apple

Title:	Open Issues of RLC CR for Rel-19 XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

In the email discussion [POST131][507][XR] for open issues in RLC CR, the rapporteur has suggested to discuss some of identified issues, including: RLC-X01, RLC-V01, RLC-V02, RLC-N01, and RLC-N02.

- RLC-X01: DSR data volume indication with inconsistent packet placement between PDCP and RLC.

- RLC-V01: Optimization of PDCP t-Reordering based on interaction with RLC t-RxDiscard.

- RLC-V02: How to capture the agreement of “Cancelling Autonomous ReTX based on positive Ack”.

- RLC-N01: Update of definition for RX_Next to reflect t-RxDiscard operations.

- RLC-N02: Further clarifications of conditions to stop and reset t-PollRetransmit based on discarding.

This paper presents some of our views on these open issues.

# Discussions

## RLC-X01

During RAN2 #131, we have agreed that PDCP should inform RLC about the data volume associated to each DSR reporting threshold, such that RLC does not need to calculate the volume of non-delay-reporting data [2].  The agreements are:

- [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

Some concerns have been raised about the situations where PDCP and RLC have different packet placement, and PDCP may not be aware how RLC buffer is arranged, which leads to inappropriate indication for RLC data volume calculation in each DSR reporting threshold. In our understanding, if a UE is able to order the RLC packets in a smart way, the functionality of “non-delay reporting” is not needed in the first place, because anyway by implementation the UE is able to make sure that most urgent packets can be submitted to the lower layer for transmission first. Conversely, if the UE needs to support “non-delay reporting” in DSR, potentially it means the UE will always following the same order between PDCP and RLC, and so we tend to think the concerned scenario is very unlikely to happen in practice. Hence, in our views there is no need to pursue any further optimization.

Proposal 1: (RLC-X01) There is no issue related to inconsistent packet placement between PDCP and RLC for the indication of data volume for each DSR reporting threshold.

## RLC-V01

It has been argued that, even if the RLC RX can detect that some packets are already discarded by the TX based on the expiry of t-RxDiscard, the PDCP RX may continue to operate the t-Reordering to wait for the missing packet that may never come, which results in unnecessary delay.

We think that if reordering timer delay is a concern, the SN gap reporting defined in Rel-18 should be configured. Thus, the PDCP TX can directly inform the PDCP RX which packets are already discarded with a SN gap report, and the PDCP RX can stop and reset t-Reordering accordingly. With this in mind, it seems SN gap reporting is already sufficient for this purpose, so we do not see the need of further optimization for the interplay between t-RxDiscard and t-Reordering. Also, there may be some additional complexity if the more cross-layer interaction is required between RLC and PDCP when t-RxDiscard expires.

Proposal 2: (RLC-V01) There is no need to optimize t-Reordering operation based on expiry of t-RxDiscard.

## RLC-V02

In RAN2 #131, we have made the following agreements [2]:

- Try to clarify in RLC specs that “RLC SDU segment(s) for transmission or retransmission” covers RLC SDU segment(s) which have been already considered or pending for transmission/retransmission. - Can try to clarify together with the previous agreement. [That: “The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU”]

However, currently the highlighted aspect has not been captured in the CR as RAN2 did not reach a consensus about how to implement it in the specifications. Some companies have suggested that nothing is needed at all.

In our views, since we have already agreed to clarify the behavior, it is the best RAN2 should honor the agreement. Besides, even though it may sound obvious, the agreed behavior is technically correct. We think we can simply capture the following sentence either in the normative text or as a NOTE: The pending retransmission of a RLC SDU should be cancelled when it is positively acknowledged.

Proposal 3: (RLC-V02) Capture the following sentence in either the normative text or as a NOTE in TS 38.322: ”The pending retransmission of a RLC SDU should be cancelled when it is positively acknowledged”.

## RLC-N01

It has been pointed out that the definition of the state variable RX_Next should be updated, in order to reflect the expiry of t-RxDiscard. The rapporteur has also suggested that we could have a conditional branch based on whether t-RxDiscard is configured or not. We tend to think what has been suggested by the rapporteur is more clear and can avoid some potential issues with the legacy operations. We prefer to minimize the changes in the existing text.

Proposal 4: (RLC-N01) Update the definition of RX_Next in TS 38.322 by adding a description for conditions wherein t-RxDiscard is configured, without impacting the existing text.

## RLC-N02

Currently, we have the following NOTE based on the agreement made in RAN2 #131 [2]:

NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

During the email discussion, some concerns have been raised:

- This is only applicable when stopReTxDiscadedSDU is configured.

- Polling should be continued if there are SDUs that have been “NACKed”

We think (1) is correct, indeed the UE should only stop the t-PollRetransmit in this case if stopReTxDiscadedSDU is configured. When stopReTxDiscadedSDU is not configured, the UE still need to retransmit these SDUs, so there is a desire to obtain the status report via polling. Thus, the NOTE can be modified by adding “If stopReTxDiscadedSDU is configured”.

On the other hand, we do not agree with (2). The TX sends the polling in order to get a status report, which allows the TX to know if retransmission of a RLC SDU is needed. If a RLC SDU is already NACKed, clearly the TX knows it has to consider retransmission for this RLC SDU, so polling for this RLC SDU can be stopped.

Proposal 5: (RLC-N02) Update the NOTE in TS 38.322 as:

- NOTE:	If stopReTxDiscadedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

# Conclusions

This paper presented our views on some open issues of RLC CR for Rel-19 XR, with the following proposals:

Proposal 1: (RLC-X01) There is no issue related to inconsistent packet placement between PDCP and RLC for the indication of data volume for each DSR reporting threshold.

Proposal 2: (RLC-V01) There is no need to optimize t-Reordering operation based on expiry of t-RxDiscard.

Proposal 3: (RLC-V02) Capture the following sentence in either the normative text or as a NOTE in TS 38.322: ”The pending retransmission of a RLC SDU should be cancelled when it is positively acknowledged”.

Proposal 4: (RLC-N01) Update the definition of RX_Next in TS 38.322 by adding a description for conditions wherein t-RxDiscard is configured, without impacting the existing text.

Proposal 5: (RLC-N02) Update the NOTE in TS 38.322 as:

- NOTE:	If stopReTxDiscadedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

# References

- Rapporteur summary of discussion [POST131][507][XR] Final 38.322 CR (vivo)

- 3GPP RAN2 #131 Chair’s Notes, Bengaluru, India, August 2025.