# 3GPP TSG RAN WG2 Meeting #131b      	                           R2-2507115
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Control plane open issues on Rel-19 NES (including RIL E204/E205/A103/X200/O005)

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 NES. In this contribution, we discuss the following control plane open issues on Rel-19 NES:

- Issue 1: Differentiation between SSB-less and OD-SSB case 1 (E024/E025)

- Issue 2: Simplification of SMTC handling procedure text for OD-SSB and SSB adaptation (A103/X200)

- Issue 3: Applicability of PO bundling to LP-SS (O005)

- Issue 4: UE capability on paging adaptation in initialDownlinkBWP-RedCap

- Issue 5: 38.300 specification impacts on measurement gap

# 2 Discussion

## 2.1 Issue 1: Differentiation between SSB-less and OD-SSB case 1 (E024/E025)

This issue was raised by E024/E025 in ASN.1 review on NES RRC CR [2]. In details, the RRC specification needs to differentiate two scenarios: SSB-less SCell and OD-SSB case 1.

- Scenario 1: SSB-less SCell

  - As agreed in RAN2#130 [3], NW may or may not configure ServingCellMO for one SSB-less SCell, depending on UE capability intraF-NeighMeasForSCellWithoutSSB [4].

  - In this scenario, absoluteFrequencySSB is absent in the concerned SSB-less SCell, the UE obtains its timing reference from the SpCell or a SCell in the same or different frequency band, as highlighted in field description of absoluteFrequencySSB.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

- Scenario 2: OD-SSB case 1 (i.e. no always-on SSB associated with the SCell)

  - In this scenario, only OD-SSB is associated with the concerned SCell (i.e. AO-SSB is absent). The corresponding configuration can be: absoluteFrequencySSB is absent but od-ssb-absoluteFrequency-r19 is configured in SCellConfig.

Observation 1: Specifications changes to TS 38.331 are needed to differentiate the following two scenarios:

- Scenario 1 (SSB-less SCell): absoluteFrequencySSB is absent in the concerned SCell, and the UE obtains its timing reference from the SpCell or a SCell in the same or different frequency band.

- Scenario 2 (OD-SSB case 1): absoluteFrequencySSB is absent in the concerned SCell, but od-ssb-absoluteFrequency-r19 is configured in SCellConfig.

Observation 2: As agreed in RAN2#130, NW may configure ServingCellMO for SSB-less SCell if the UE reports it supports capability intraF-NeighMeasForSCellWithoutSSB. Otherwise, NW doesn’t configure ServingCellMO.

To resolve the issue, we think it is necessary for RAN2 to confirm the understanding illustrated in Table.1.

|  | absoluteFrequencySSB in ServingCellConfigCommon | OD-SSB-r19 in SCellConfig | ssbFrequency in ServingCellMO | ServingCellMO-OD |
| --- | --- | --- | --- | --- |
| Scenario 0: SCell associated with AO-SSB only | Present | Absent | Present | Absent |
| Scenario 1-a: SSB-less SCell with ServingCellMO configured | Absent | Absent | Present | Absent |
| Scenario 1-b: SSB-less SCell with ServingCellMO not configured | Absent | Absent | Absent | Absent |
| Scenario 2: OD-SSB case 1 (i.e. only associated with OD-SSB) | Absent | Present, and its od-ssb-absoluteFrequency-r19 is present | Present (same as od-ssb-absoluteFrequency-r19) | Absent |
| Scenario 3: AO-SSB and OD-SSB in same frequency | Present | Present, but its od-ssb-absoluteFrequency-r19 is absent. | Present (same as absoluteFrequencySSB) | Absent |
| Scenario 4: AO-SSB and OD-SSB in different frequency | Present | Present, and its od-ssb-absoluteFrequency-r19 is present and different from absoluteFrequencySSB. | Present (same as absoluteFrequencySSB) | Present, and its ssbFrequency is same as od-ssb-absoluteFrequency-r19) |

Table.1: Illustration of how to configure different scenarios with SSB-less SCell and OD-SSB SCells

To align understanding, we suggest RAN2 to agree Table.1:

Proposal 1 (E024/E025): RAN2 confirm Table.1 on how to configure different scenarios with SSB-less SCell and OD-SSB SCells.

Next, we think the following specifications changes are needed:

- Update the field description of absoluteFrequencySSB to preclude OD-SSB 1 case1 from SSB-less SCell. An example is illustrated as follows:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-absoluteFrequency-r19 is absent in SCellConfig, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

- In field description of ServingCellMO, clarify that ServingCellMO is configured for OD-SSB case 1. An example is illustrated as follows:

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. If the serving cell is associated with either SSB or OD-SSB, the following relationship applies between the corresponding MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB or od-ssb-absoluteFrequency-r19 in SCellConfig and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList. If the serving cell is not associated with neither SSB nor OD-SSB (i.e. SSB-less SCell), the carrier frequency indicated by ssbFrequency of the corresponding MeasObjectNR, if configured, is within the frequency range indicated by any entry of the scs-SpecificCarrierList.

- In presence condition of ssbFrequency (i.e. SSBorAssociatedSSB2) under MeasObjectNR, clarify that ssbFrequency is mandatory present in OD-SSB case 1. An example is illustrated as follows:

| SSBorAssociatedSSB2 | If the measObject is associated to an SCell with either AO-SSB or OD-SSB, this field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If the measObject is associated to an SSB-less SCell (i.e. associated with neither AO-SSB nor OD-SSB), this field is optionally present, Need R, if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If ssb-ConfigMobility is not configured and associatedSSB is not configured for any cell, the field is absent, Need R. |
| --- | --- |

Thus, we propose:

Proposal 2 (E024/E025): To differentiate SSB-less SCell and OD-SSB case 1, adopt the following spec changes:

- Update the field description of absoluteFrequencySSB to preclude OD-SSB 1 case1 from SSB-less SCell.

- In field description of ServingCellMO, clarify that ServingCellMO is configured for OD-SSB case 1.

- In presence condition of ssbFrequency (i.e. SSBorAssociatedSSB2) under MeasObjectNR, clarify that ssbFrequency is mandatory present in OD-SSB case 1.

The TP can be found in Appendix 1.

We notice that one related issue is being discussed in RAN1#121 [7]: in OD-SSB case 1, whether the UE can perform SSB-less operation (i.e. obtain timing from SpCell or anther SCell) when OD-SSB is not activated. The related Proposal can be found in the following copy from FL summary [8], where we also copy the original proposal from Proponent (Huawei) to avoid misunderstanding in RAN2.

### Proposal #2-2 (SSB-less SCell): · SSB-less SCell supporting on-demand SSB SCell operation is considered as Case #1. o If UE reports the capability that both SSB-less SCell operation and on-demand SSB SCell operation are supported, gNB can configure it with an SSB-less SCell on which OD-SSB can be indicated and the relative behaviours follow previous agreements in Case#1.

We think RAN2 can wait RAN1 conclusion, and then determine whether RAN2 spec change is needed:

- If the proposal is agreed in RAN1, RAN2 just need to add some text in field description of absoluteFrequencySSB (i.e. no need to impact ASN.1). For example:

“If the field is absent and od-ssb-absoluteFrequency-r19 is absent in SCellConfig, or if the field is absent and od-ssb-absoluteFrequency-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14].”

- If the proposal is NOT agreed in RAN1, RAN2 don’t need further spec impact.

Observation 3: RAN1 is discussing whether the UE can perform SSB-less operation (i.e. obtain timing from SpCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated. However, in case it is agreed in RAN1, RAN2 just need to add some text in field description of absoluteFrequencySSB (i.e. no need to impact ASN.1).

According to above analysis, we think RAN2 can handle such case, irrespectively of what the final RAN1 conclusion is. There is no reason to delay the correction on Proposal 2. Thus, we propose to agree Proposal 2 and discuss RAN2 impact when RAN1 makes conclusion.

Proposal 3 (E024/E025): RAN2 wait RAN1 conclusion whether the UE can perform SSB-less operation (i.e. obtain timing from PCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated.

## 2.2 Issue 2: Simplification of SMTC handling procedure text for OD-SSB and SSB adaptation (A103/X200)

In ASN.1 review discussion [2], there are some concerns about procedure text on handling smtcxlist (for OD-SSB) and smtc-SSBAdapt (for SSB adaptation):

- Need to reduce signaling overhead to allow not configure all possible values of SSB periodicities (e.g. 6 values for OD-SSB and 2 values for SSB adaptation).

- Need to simplify the procedure text on SMTC setup and align the wording between OD-SSB and SSB adaptation (e.g. suggestion from N001).

To address above two issues, we provide the following suggestion on how to simplify the procedure text and align OD-SSB and SSB adaptation:

- In ASN.1, modify SSB-MTCxList-r19 and SSB-MTC-SSBAdapt-r19 to use same IE type (e.g. SSB-MTCx-r19) which explicitly includes the mapped SSB periodicity.

- In Section 5.5.2.10, modify the procedure of OD-SSB and SSB adaptation with aligned wording that the SMTC with same mapped SSB periodicity is selected.

Below is an example of the suggested spec changes:

ASN.1:

SSB-MTCxList-r19 ::=           SEQUENCE (SIZE(1..6maxAdditionalSMTC-ODSSB-r19)) OF SSB-MTCSSB-MTCx-r19

SSB-MTC-SSBAdapt-r19 ::=  SEQUENCE (SIZE(1..2maxAdditionalSMTC-SSBAdapt-r19)) OF SSB-MTCSSB-MTCx-r19

maxAdditionalSMTC-ODSSB-r19      INTEGER ::= 6  -- Maximum number of additiona SMTC for OD-SSB.

maxAdditionalSMTC-SSBAdapt-r19  INTEGER ::= 2-- Maximum number of additiona SMTC for SSB adaptation.

SSB-MTCx-r19 ::=             SEQUENCE {

ssb-Periodicity-r19                 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

SMTC-r19 						      SSB-MTC

}

Section 5.5.2.10:

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field an SMTC in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first OD-SSB configuration for the serving cell which has the same SS/PBCH block reception periodicity (ssb-Periodicity within SSB-MTCx) as the one in the configuration of activated OD-SSB of the serving cell; the UE shall setup SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on.

If smtcy-SSBAdapt is present, the UE shall setup SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI is not received or the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; Otherwise, the UE shall setup SMTC according to the first an SMTC in smtcy-SSBAdapt for serving cell measurements on the corresponding MeasObjectNR, which has the same SS/PBCH block reception periodicity (ssb-Periodicity within SSB-MTCx) as the one indicated by if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the first adaptive SSB periodicity in adap-SSB-BurstPeriodicityList; the UE shall setup SMTC according to the second SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the second adaptive SSB periodicity in adap-SSB-BurstPeriodicityList.

Thus, we propose:

Proposal 4 (A103/X200): RAN2 adopt the following spec changes to simplify and align the procedure text of handling SMTC for OD-SSB and SSB adaptation:

- In ASN.1, modify SSB-MTCxList-r19 and SSB-MTC-SSBAdapt-r19 to use same IE type (e.g. SSB-MTCx-r19) which explicitly includes the mapped SSB periodicity.

- In Section 5.5.2.10, modify the procedure of OD-SSB and SSB adaptation with aligned wording that the SMTC with same mapped SSB periodicity is selected.

The TP can be found in Appendix 2.

## 2.3 Issue 3: Applicability of PO bundling to LP-SS (O005)

This issue can be described as follows:

[Description]: It is not clear whether the paging adapation (i.e., PO bundling) feature can be enabled together with LP-SS feature or not. [Proposed Change]: R2 discuss and conclude the applicability of co-configuring paging adaptation and LP-SS feature. [Comments]: [Apple] As paging adaptation and LP-WUS are both specified in Rel-19, we generally don’t pursue their feature combination in same release. Prefer to leave it to 6G. [Rapporteur] This requires further discussion in the next meeting. [Huawei] We think there are several possible solutions: Option 1: gNB can configure both R19 LP-WUS and R19 paging adaptation, and when they are configured, R19 LP-WUS applies to legacy POs only, it does not apply to R19 POs. Option 2: gNB can configure both R19 LP-WUS and R19 paging adaptation, and when they are configured, two LP-WUS configurations are provided, where one is for legacy POs, the other one is for R19 POs Option 3: Do not support to configure R19 LP-WUS and R19 paging adaptation simultaneously in SIB1 The issue is also being discussed in RAN1, we prefer to wait for RAN1 conclusion.

We are also informed by our RAN1 colleague that this issue is being discussed in RAN1 (under LP-WUS WI) without conclusion now. To avoid duplicated discussion between RAN1 and RAN2, we propose to just wait RAN1 conclusion.

Proposal 5 (O005): Wait RAN1 conclusion on whether paging adaptation can be applied to LP-WUS.

## 2.4 Issue 4: UE capability on paging adaptation in ReinitialDownlinkBWP-RedCap

This issue was discussed in post-meeting email discussion on 38.306 CR [5]. In details, we think there are two way-forward:

- WF-1: introduce one separate capability of paging in initialDownlinkBWP-RedCap-r17 and one separate capability for PEI in initialDownlinkBWP-RedCap-r17 for (e)redcap UE.

- WF-2: integrate the capability of paging adaptation/PEI in initialDownlinkBWP-RedCap-r17 for (e)redcap UE into existing capability (i.e. pagingAdaptation-r19/pagingAdaptionPEI-SupportBandList-r19).

We don’t have strong view between WF-1 and WF-2. However, WF-1 needs to introduce two new UE capabilities for paging adaptation and PEI in initialDownlinkBWP-RedCap-r17. As Rel-19 NES was completed, the extra spec work should be introduced only when really necessary. Thus, we think maybe the simplest way is to go WF-2 at this stage.

Proposal 6: Integrate the capability of paging adaptation/PEI in initialDownlinkBWP-RedCap-r17 for (e)redcap UE into existing paging adaptation capability (i.e. pagingAdaptation-r19/pagingAdaptionPEI-SupportBandList-r19).

## 2.5 Issue 5: 38.300 specification impacts on measurement gap

    - This issue was discussed in post-meeting email discussion on 38.300 CR [6]. In details, the issue can be described as follows:

Issue description: Capturing measurement gap behaviour for OD-SSB, either in a Note, or by normative text. Clarification what is the meaning of “SSB associated to the initial DL BWP” in the measurement gap section in 9.2.4, whether it refers to AO-SSB or OD-SSB. Possible options to resolve the issue: - Normative text as e.g. provided by rapp in CR v04 (Appendix TP1) - Note as e.g. provided by in the collection of comments in Apple001 (Appendix TP2) - No modification of the measurement gap paragraph

    - According to the discussion in [6], there are two different understanding on how to understand RAN1 agreement “AO-SSB and OD-SSB are on the same BWP”:

- Understanding 1: only the active BWP to receive SIB1 needs to contain both AO-SSB and OD-SSB.

  - Consequence: measurement gap may still be needed when the NW indicates the UE to switch to another BWP (not reception of SIB1) for DL/UL transmission.

- Understanding 2: all the BWPs (active BWP and non-active BWPs) need to contain both AO-SSB and OD-SSB

  - Consequence: it will mandate NW to always configure OD-SSB for all BWPs with AO-SSB (even in non-active BWPs), which has NW configuration restriction and extra signaling overhead (esp. if 4 BWPs are configured).

    - Since RAN1 has captured the agreement in TS 38.213, we think that RAN2 don’t need to further clarify RAN1 agreement at late stage. However, no matter how to understand RAN1 agreement, we think there is ambiguity on what is “SSB associated to the initial DL BWP” in the measurement gap section in 9.2.4. Even if it is Understanding 2, it still implies that the UE can use the frequency resource of either AO-SSB or OD-SSB to determine whether measurement gap is needed. Thus, we think a NOTE for clarification is necessary and has no harm to be captured. Thus, we propose:

Proposal 7: RAN2 adopt the following NOTE to clarify what is “SSB associated to the initial DL BWP” in the measurement gap section in 9.2.4 of TS 38.300.

NOTE 4:	when the serving cell is associated with both SSB and OD-SSB in different frequency, “the SSB associated to the initial DL BWP” includes both SSB and OD-SSB.

# 3 Conclusion

In this contribution, we discuss remaining control plane open issues on Rel-19 NES. Our observations are:

Observation 1: Specifications changes to TS 38.331 are needed to differentiate the following two scenarios:

- Scenario 1 (SSB-less SCell): absoluteFrequencySSB is absent in the concerned SCell, and the UE obtains its timing reference from the SpCell or a SCell in the same or different frequency band.

- Scenario 2 (OD-SSB case 1): absoluteFrequencySSB is absent in the concerned SCell, but od-ssb-absoluteFrequency-r19 is configured in SCellConfig.

Observation 2: As agreed in RAN2#130, NW may configure ServingCellMO for SSB-less SCell if the UE reports it supports capability intraF-NeighMeasForSCellWithoutSSB. Otherwise, NW doesn’t configure ServingCellMO.

Observation 3: RAN1 is discussing whether the UE can perform SSB-less operation (i.e. obtain timing from SpCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated. However, in case it is agreed in RAN1, RAN2 just need to add some text in field description of absoluteFrequencySSB (i.e. no need to impact ASN.1).

Based on above observations, our proposals are:

Proposal 1 (E024/E025): RAN2 confirm Table.1 on how to configure different scenarios with SSB-less SCell and OD-SSB SCells.

|  | absoluteFrequencySSB in ServingCellConfigCommon | OD-SSB-r19 in SCellConfig | ssbFrequency in ServingCellMO | ServingCellMO-OD |
| --- | --- | --- | --- | --- |
| Scenario 0: SCell associated with AO-SSB only | Present | Absent | Present | Absent |
| Scenario 1-a: SSB-less SCell with ServingCellMO configured | Absent | Absent | Present | Absent |
| Scenario 1-b: SSB-less SCell with ServingCellMO not configured | Absent | Absent | Absent | Absent |
| Scenario 2: OD-SSB case 1 (i.e. only associated with OD-SSB) | Absent | Present, and its od-ssb-absoluteFrequency-r19 is present | Present (same as od-ssb-absoluteFrequency-r19) | Absent |
| Scenario 3: AO-SSB and OD-SSB in same frequency | Present | Present, but its od-ssb-absoluteFrequency-r19 is absent. | Present (same as absoluteFrequencySSB) | Absent |
| Scenario 4: AO-SSB and OD-SSB in different frequency | Present | Present, and its od-ssb-absoluteFrequency-r19 is present and different from absoluteFrequencySSB. | Present (same as absoluteFrequencySSB) | Present, and its ssbFrequency is same as od-ssb-absoluteFrequency-r19 |

Proposal 2 (E024/E025): To differentiate SSB-less SCell and OD-SSB case 1, adopt the following spec changes:

- Update the field description of absoluteFrequencySSB to preclude OD-SSB 1 case1 from SSB-less SCell.

- In field description of ServingCellMO, clarify that ServingCellMO is configured for OD-SSB case 1.

- In presence condition of ssbFrequency (i.e. SSBorAssociatedSSB2) under MeasObjectNR, clarify that ssbFrequency is mandatory present in OD-SSB case 1.

The TP can be found in Appendix 1.

Proposal 3 (E024/E025): RAN2 wait RAN1 conclusion whether the UE can perform SSB-less operation (i.e. obtain timing from PCell or anther SCell) in OD-SSB case 1, when OD-SSB is not activated.

Proposal 4 (A103/X200): RAN2 adopt the following spec changes to simplify and align the procedure text of handling SMTC for OD-SSB and SSB adaptation:

- In ASN.1, modify SSB-MTCxList-r19 and SSB-MTC-SSBAdapt-r19 to use same IE type (e.g. SSB-MTCx-r19) which explicitly includes the mapped SSB periodicity.

- In Section 5.5.2.10, modify the procedure of OD-SSB and SSB adaptation with aligned wording that the SMTC with same mapped SSB periodicity is selected.

The TP can be found in Appendix 2.

Proposal 5 (O005): Wait RAN1 conclusion on whether paging adaptation can be applied to LP-WUS.

Proposal 6: Integrate the capability of paging adaptation/PEI in initialDownlinkBWP-RedCap-r17 for (e)redcap UE into existing paging adaptation capability (i.e. pagingAdaptation-r19/pagingAdaptionPEI-SupportBandList-r19).

Proposal 7: RAN2 adopt the following NOTE to clarify what is “SSB associated to the initial DL BWP” in the measurement gap section in 9.2.4 of TS 38.300.

NOTE 4:	when the serving cell is associated with both SSB and OD-SSB in different frequency, “the SSB associated to the initial DL BWP” includes both SSB and OD-SSB.

# 4 References

[1] RAN2#131, Chair Notes.

[2] R2-250xxxx, NES Comments file, Ericsson.

[3] R2-2504986, Correction on servingCellMO configuration for SSB-less SCell	Huawei, HiSilicon, Apple, Samsung, Qualcomm Incorporated, MediaTek Inc, ZTE Corporation, Sanechips, Ericsson, Nokia.

[4] R2-2503762	Correction on servingCellMO configuration - 38.306, ZTE Corporation, Sanechips, Ericsson, Nokia.

[5] R2-250xxxx, Open issues for Rel-19 NES UE capability, ZTE.

[6] R2-250xxxx, Remaining stage-2 open issues for NES enh, Huawei, HiSilicon.

[7] RAN1#122, Chair Notes.

[8] R1-2506596, Summary #2 of on-demand SSB for NES, Moderator (LG Electronics).

# Appendix 1 (E024/E025)

- Update the field description of absoluteFrequencySSB to preclude OD-SSB 1 case1 from SSB-less SCell:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-absoluteFrequency-r19 is absent in SCellConfig, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

- In field description of ServingCellMO, clarify that ServingCellMO is configured for OD-SSB case 1:

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. If the serving cell is associated with either SSB or OD-SSB, the following relationship applies between the corresponding MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB or od-ssb-absoluteFrequency-r19 in SCellConfig and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList. If the serving cell is not associated with neither SSB nor OD-SSB (i.e. SSB-less SCell), the carrier frequency indicated by ssbFrequency of the corresponding MeasObjectNR, if configured, is within the frequency range indicated by any entry of the scs-SpecificCarrierList.

- In presence condition of ssbFrequency (i.e. SSBorAssociatedSSB2) under MeasObjectNR, clarify that ssbFrequency is mandatory present in OD-SSB case 1.

| SSBorAssociatedSSB2 | If the measObject is associated to an SCell with either SSB or OD-SSB, this field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If the measObject is associated to an SSB-less SCell (i.e. associated with neither SSB nor OD-SSB), this field is optionally present, Need R, if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If ssb-ConfigMobility is not configured and associatedSSB is not configured for any cell, the field is absent, Need R. |
| --- | --- |

# Appendix 2 (A103/X200)

ASN.1:

SSB-MTCxList-r19 ::=           SEQUENCE (SIZE(1..6maxAdditionalSMTC-ODSSB-r19)) OF SSB-MTCSSB-MTCx-r19

SSB-MTC-SSBAdapt-r19 ::=  SEQUENCE (SIZE(1..2maxAdditionalSMTC-SSBAdapt-r19)) OF SSB-MTCSSB-MTCx-r19

maxAdditionalSMTC-ODSSB-r19      INTEGER ::= 6  -- Maximum number of additiona SMTC for OD-SSB.

maxAdditionalSMTC-SSBAdapt-r19  INTEGER ::= 2-- Maximum number of additiona SMTC for SSB adaptation.

SSB-MTCx-r19 ::=             SEQUENCE {

ssb-Periodicity-r19                 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

SMTC-r19 						      SSB-MTC

}

Section 5.5.2.10:

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field an SMTC in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first OD-SSB configuration for the serving cell which has the same SS/PBCH block reception periodicity (ssb-Periodicity within SSB-MTCx) as the one in the configuration of activated OD-SSB of the serving cell; the UE shall setup SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on.

If smtcy-SSBAdapt is present, the UE shall setup SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI is not received or the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; Otherwise, the UE shall setup SMTC according to the first an SMTC in smtcy-SSBAdapt for serving cell measurements on the corresponding MeasObjectNR, which has the same SS/PBCH block reception periodicity (ssb-Periodicity within SSB-MTCx) as the one indicated by if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the first adaptive SSB periodicity in adap-SSB-BurstPeriodicityList; the UE shall setup SMTC according to the second SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC