# 3GPP TSG RAN WG2 Meeting #131b      	                        R2-2507118
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.1.2.2

Source:	Apple

Title:	Open issues on LCM of AI/ML based beam management (including E041/E042/X003/H010/E040/Z001/Z002)

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 AI/ML for air interface. In this contribution, we discuss the following open issues related to LCM procedure of AI/ML based beam management:

- Whether/how to differentiate cell-specific or multi-cell associated ID

- How to handle previously configured periodic CSI config that becomes applicable

- Discussion of important RIL

  - E041: Associated ID validity

  - E042: Dedicated flag to configure applicability reporting via UAI

  - X003: Whether to remove the dataCollectionStart

  - H010: Remove detailed reporting parameters in ApplicabilitySetConfig-r19

  - E040: How to ensure that UE does not request a candidate UE-side data collection configuration which is working

  - Z001/Z002: Whether to allow indicating “release” for inference configuration in OtherConfig in RRCReconfigurationComplete/UAI

# 2 Discussion

## 2.1 Differentiation of cell-specific or multi-cell associated ID

The issue is one FFS of RAN2#131 [1].

Agreements on associated ID

- Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

- Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

- We will not define areas.  The Associated ID is 24 bits.

4	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

RAN2 has agreed that the associated ID can be either cell specific or multi-cell specific, and its generation is left to NW implementation. However, if without further indication, the UE can’t differentiate whether it is cell specific or multi-cell specific. For example, if two cells have the same ID, the UE will not be able to determine whether the two cells have same / similar beam deployment. Thus, NW needs to also indicate whether the associated ID is cell specific or multi-cell specific.

Observation 1: Although generation of associated ID is left to NW implementation, the UE can’t differentiate whether it is cell specific or multi-cell specific and thereby can’t determine whether two cells with same associated ID have same / similar beam deployment.

Regarding to the detailed signalling of indication, we prefer to introduce 1-bit indication on top of 24-bit associated ID. Otherwise, RAN2 need to further discuss the length of cell specific associated ID, which should be avoided because RAN2 has completed Rel-19 AI/ML WI. We can leave the signalling overhead reduction for cell specific associated ID to next release. Thus, we propose:

Proposal 1 (Remaining FFS): On top of 24-bit associated ID, introduce 1-bit indication on whether it is applied to single cell or multiple cells.

## How to handles periodic CSI config that becomes applicable

This is one CB which was missed to discuss in RAN2#131 [1].

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

2	Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

We think no new UE behaviour needs to be introduced. In details, for a periodic CSI configuration which was reported as non-applicable, the UE may send UAI when becoming applicable and needs to wait NW to send a followed RRCReconfiguration with inference configuration.

Proposal 2 (missed CB): For a periodic CSI configuration which was reported as non-applicable, the UE may send UAI when becoming applicable and then needs to wait NW to send a followed RRCReconfiguration with inference configuration (i.e., the UE can’t autonomously start inference of periodic CSI after sending UAI).

## Discussion of important RIL

In this section, we discuss some important RIL(s) raised by companies in ASN.1 review.

### 2.3.1 E041: Associated ID validity

This RIL is described as follows:

[Description]: According to current specifications, once an associated ID value is signaled from the network to the UE, the meaning of that value at the UE (or UE-side entity) corresponds to the same beam property/deployment forever. There is no way for the network to reset/reassign that value to other beam properties and inform the UE that should forget the old meaning of that value. We think it is unreasonable to assume that a given meaning holds for a given associated ID forever. It may happen that e.g. some hardware is replaced and the old hardware is not used again anywhere else in the network, or there is a failure/reset in the network entity storing/assigning associated IDs, after which the legacy associated ID meanings (assigned to deployment characteristics) cannot be recovered. Although such events may be very unlikely, they may still happen at a certain point in the future. Thus, there should be a way for the network and UE to make sure that they have the same understanding of an associated ID value. This is important for the UE (or UE-side entity) to be able to understand if a model exists and/or is trained for the same deployment assumed by the network for an associated ID.

It seems the intention is to introduce some time validity on associated ID. Although it seems technically reasonable, we think it is too late for Rel-19 because the time validity mechanism needs non-trivial specification work. In Rel-19, we prefer to leave it to NW implementation to maintain its validity, e.g. update or de-configure the associated ID when it is out of date.

Proposal 3 (RIL E041): It is left to NW implementation to maintain validity of associated ID in Rel-19, e.g. NW updates or de-configures the associated ID when it is out of date.

### 2.3.2 E042: Dedicated flag to configure applicability reporting via UAI

This RIL is described as follows:

[Description]: RAN2#130 agreed the following: “2 Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)” After adding the sets of inference related parameters for option B in otherConfig, it was not discussed how to combine this flag in ASN.1 with the configurations for option B. [Proposed Change]: We will provide a more detailed discussion and solutions in a Tdoc. Some possible solutions are: - Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig. - Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is. - Move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

Among the 3 solutions listed by RRC CR Rapporteur, we prefer solution 1):

- Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

The reasoning is that we believe that updating applicability with UAI is an essential feature. If the UE is not allowed to use UAI to update applicability/non-applicability status, the system will be quite inefficient because the UE has only 16ms (RRC processing delay to send RRCReconfigurationComplete) to determine whether to perform inference. In case of initial reporting non-applicable via RRCReconfigurationComplete, NW can only guess when becoming applicable.

Observation 2: If the UE is not allowed to use UAI to update applicability/non-applicability status, the system will be quite inefficient because the UE has only 16ms (RRC processing delay to send RRCReconfigurationComplete) to determine whether to perform inference. In case of initial reporting non-applicable via RRCReconfigurationComplete, NW can only guess when becoming applicable.

Thus, we believe solution 1) should be pursued.

Proposal 4 (RIL E042): On RIL E042, adopt the following solution 1):

- Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

### 2.3.3 X003: Whether to remove the dataCollectionStart

This RIL is described as follows:

[[Description]: The presence of dataCollectionCandidateConfigList can implicitly indicate UE is about to collect data. There is no need to intorduce separate indication. On the other hand, start indication is useless without dataCollectionCandidateConfigList, since NW is not aware of the required configuration. If UE has already obtained the required configuration for data collection, UE doesn’t need to indicate start indication alone. We propose to remove the dataCollectionStart.

We disagree with this RIL due to following two reasonings:

- RAN2#131 has clearly agreed that the start indication can be sent without a preferred candidate list (i.e., highlighted case a). This RIL will revert the agreement.

- Candidate configuration list was agreed to optionally configured. We understand that the intention of sending start indication alone is to support the case that NW doesn’t provide any candidate configuration. In this case, it is up to NW implementation how to configure data collection configuration.

Agreements on UE candidate data collection 1 Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI. 2 No prohibit timer is needed for UE indicating its preferred data collection configuration. 3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2.

Observation 3: RAN2#131 has clearly agreed that the start indication can be sent without a preferred candidate list. It is intended to support the case that NW doesn’t provide any candidate configuration.

But one valid issue is that the 1-bit start indication can’t differentiate whether the request is for beam management or CSI prediction, whose data collection configuration may be different. To make it future proof (e.g. CSI compression in Rel-20), we prefer to change the 1-bit start indication to an ENUMERATED structure with CSI prediction and BM, where one example can be found as follows:

dataCollectionStart-r19     ENUMERATED {start} ENUMERATED {CSIprediction, BM, spare2, spare1}                OPTIONAL

Thus, we propose:

Proposal 5 (RIL X003): Keep dataCollectionStart-r19. And to differentiate the request for beam management or CSI prediction, change its type to ENUMERATED {CSIprediction, BM, spare2, spare1}.

### H010: Detailed reporting parameters in ApplicabilitySetConfig-r19

This RIL is described as follows:

[Description]: In our understanding, reportConfigType in ApplicabilitySetConfig-r19 needs to only indicate the type of the report, i.e. periodic, semi persistent on PUCCH/PUSCH or aperiodic. Additional information such as reportSlotConfig etc. is not required for the UE to determine applicability, so is not needed in this configuration. [Proposed Change]: ApplicabilitySetConfig-r19 ::= SEQUENCE { applicabilitySetConfigId-r19 ApplicabilitySetConfigId-r19 OPTIONAL, -- Need R resourcesForChannelMeasurement CSI-ResourceConfigId OPTIONAL, -- Need R resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R reportQuantity-r19 ReportQuantity-r19 OPTIONAL, -- Need R reportConfigType CHOICE { periodic SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH SEQUENCE { reportSlotConfig ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320}, reportSlotOffsetList SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32), p0alpha P0-PUSCH-AlphaSetId }, aperiodic SEQUENCE { reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32) } } OPTIONAL, -- Need R reportConfigType-r19 ENUMERATED {periodic, semiPersistentOnPUCCH, semiPersistentOnPUSCH, aperiodic } OPTIONAL, -- Need R nrofReportedPredictedRS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... }

We disagree with this RIL. In our understanding, the reporting resource configuration (reportSlotConfig / reportSLotOffsetList) is useful for the UE to determine applicability of one set of inference configuration because it is needed in CPU occupation time calculation. In details, RAN1#121 has agreed to reuse legacy CPU occupation time for CSI inference in UE-sided model.

Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1, - Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report - Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report - Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report) Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case2, for occupancy duration of CPU and APU, same occupation time for AI/ML PU and legacy CPU. - If the CSI report is aperiodic, for AI/ML PU, and for CPU, Rel-15 CPU occupation time for AP CSI report is reused - If the CSI report is semi-persistent or periodic, - From the 1st symbol of the latest CSI-RS/SSB transmission occasion no later than CSI reference resource, until the last symbol of the PUCCH/PUSCH carrying the report.

And according to clause 5.2.1.6 of latest 38.214 [2], calculation of CPU occupation time of inference of periodic/SP/AP CSI needs to consider the reporting resource. We provide an example of inference of aperiodic CSI-RS. Please note that highlighted part.

For aperiodic CSI-RS, when the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19', NZP CSI-RS resource is active in a duration starting from the end of the PDCCH containing the request for the report of the corresponding CSI Reporting Setting and ending at the end of the scheduled PUSCH containing the report associated with the report of the linked CSI Reporting Setting.

Observation 4: According to clause 5.2.1.6 of latest 38.214, calculation of CPU occupation time of BM inference needs to consider reporting resource. Thus, reportSlotConfig/reportSLotOffsetList are needed for the UE to determine applicability of one set of inference parameter (i.e. option B).

Thus, we propose:

Proposal 6 (RIL H010): Reject H010 because reportSlotConfig/reportSLotOffsetList are needed for CPU occupation calculation and thereby necessary for the UE to determine applicability of ApplicabilitySetConfig-r19 (i.e. option B).

### 2.3.5 E040: How to ensure that UE does not request a candidate UE-side data collection configuration which is working

This RIL is described as follows:

[Description]: According to the current procedural text in 5.7.4.3, the UE may request a candidate UE-side data collection configuration in UAI (received in otherConfig), even if it already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig) for that candidate configuration. This is not a desired behaviour and it should be fixed. [Proposed Change]: The issue and some possible solutions will be discussed in more detail in a Tdoc. Some possible solutions are: - change only the procedural text to make it clear that the UE does not request candidate configurations for which it already has a corresponding UE-side data collection configuration; - change the ASN.1 (and corresponding procedures) so the network can indicate which candidate configuration a real UE-side data collection configuration corresponds to.

We agree that the UE is not expected to request a candidate UE-side data collection configuration which already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig). However, we think it is intended to specify solution for a bad UE implementation, which should be avoided. Otherwise, we need a lot of similar specifications in TS 38.331 if opening the door.

Observation 5: TS 38.331 should avoid specification for a bad UE implementation. Otherwise, we need a lot of similar specifications in TS 38.331 if opening the door.

Thus, we propose to leave it UE implementation to resolve the issue.

Proposal 7 (RIL E040): No specification change is needed for E040. Leave it to UE implementation to avoid requesting a candidate UE-side data collection configuration which already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig).

### Z001/X002: Whether to allow indicating “release” for inference configuration in OtherConfig

This RIL is described as follows (take example of Z001):

[Description]: Currently, for the applicability reporting via RRCReconfigurationComplete, it has been defined as below: 4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference; For the functionality that is provided in the OtherConfig whose applicability has been changed, there is no need for UE to provide the information of the releaseConfigurationPrefernece in the RRCReconfigurationComplete as there is no hurt to keep it still in the otherConfig. [Proposed Change]: [Comments]: [WI CR rapporteur-v022]: We think this issue should be discussed based on Tdocs, since there are changes in the UE behaviour for applicability reporting in RRCReconfigurationComplete versus UAI.

We agree with the intention that there is no harm to keep inference configuration in OtherConfig. However, we are not sure whether some NW implementation implicitly links one set of inference related parameters in OtherConfig with one inference configuration (CSI-ReportConfig). If that is the case, we think that NW needs to know whether the UE prefers “release” for one set of inference related parameters in OtherConfig.

Observation 6: Some NW implementation may implicitly link one set of inference related parameters in OtherConfig with one inference configuration (CSI-ReportConfig). In this case, NW needs to know whether the UE prefers “release” for one set of inference related parameters in OtherConfig.

To allow the above NW implementation, we prefer to allow the UE to indicate “release” for one set of inference related parameters in OtherConfig.

Proposal 8 (RIL Z001/Z002): Reject Z001/Z002, i.e., allow the UE to indicate “release” for one set of inference related parameters in OtherConfig in applicability report (via RRCReconfigurationComplete/UAI).

# 3 Conclusion

In this contribution, we discuss the remaining open issues and some important RILs of Rel-19 AI/ML for PHY. Our observations are:

Observation 1: Although generation of associated ID is left to NW implementation, the UE can’t differentiate whether it is cell specific or multi-cell specific and thereby can’t determine whether two cells with same associated ID have same / similar beam deployment.

Observation 2: If the UE is not allowed to use UAI to update applicability/non-applicability status, the system will be quite inefficient because the UE has only 16ms (RRC processing delay to send RRCReconfigurationComplete) to determine whether to perform inference. In case of initial reporting non-applicable via RRCReconfigurationComplete, NW can only guess when becoming applicable.

Observation 3: RAN2#131 has clearly agreed that the start indication can be sent without a preferred candidate list. It is intended to support the case that NW doesn’t provide any candidate configuration.

Observation 4: According to clause 5.2.1.6 of latest 38.214, calculation of CPU occupation time of BM inference needs to consider reporting resource. Thus, reportSlotConfig/reportSLotOffsetList are needed for the UE to determine applicability of one set of inference parameter (i.e. option B).

Observation 5: TS 38.331 should avoid specification for a bad UE implementation. Otherwise, we need a lot of similar specifications in TS 38.331 if opening the door.

Observation 6: Some NW implementation may implicitly link one set of inference related parameters in OtherConfig with one inference configuration (CSI-ReportConfig). In this case, NW needs to know whether the UE prefers “release” for one set of inference related parameters in OtherConfig.

Based on Observations, we have the following proposals:

Proposal 1 (Remaining FFS): On top of 24-bit associated ID, introduce 1-bit indication on whether it is applied to single cell or multiple cells.

Proposal 2 (missed CB): For a periodic CSI configuration which was reported as non-applicable, the UE may send UAI when becoming applicable and then needs to wait NW to send a followed RRCReconfiguration with inference configuration (i.e., the UE can’t autonomously start inference of periodic CSI after sending UAI).

Proposal 3 (RIL E041): It is left to NW implementation to maintain validity of associated ID in Rel-19, e.g. NW updates or de-configures the associated ID when it is out of date.

Proposal 4 (RIL E042): On RIL E042, adopt the following solution 1):

- Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

Proposal 5 (RIL X003): Keep dataCollectionStart-r19. And to differentiate the request for beam management or CSI prediction, change its type to ENUMERATED {CSIprediction, BM, spare2, spare1}.

Proposal 6 (RIL H010): Reject H010 because reportSlotConfig/reportSLotOffsetList are needed for CPU occupation calculation and thereby necessary for the UE to determine applicability of ApplicabilitySetConfig-r19 (i.e. option B).

Proposal 7 (RIL E040): No specification change is needed for E040. Leave it to UE implementation to avoid requesting a candidate UE-side data collection configuration which already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig).

Proposal 8 (RIL Z001/Z002): Reject Z001/Z002, i.e., allow the UE to indicate “release” for one set of inference related parameters in OtherConfig in applicability report (via RRCReconfigurationComplete/UAI).

# 4 References

[1] RAN2#131, Chair Notes.

[2] R1-2504992, Introduction of AI/ML beam prediction, TS 38.214 CR, Nokia.