3GPP TSG-RAN2 #131bis	R2-2507127

# Prague, Czech Republic, October 13 – October 17, 2025

Agenda item:	10.3.1 (FS_6G_Radio)

Source:	LG Electronics

Title: 	Considerations on User plane for 6G

Document for:	Discussion and decision

1.	Introduction

This document states our views on 6G user plane protocols.

2.	User plane for 6G

2.1	Simplified L2 protocol architecture

In 5G NR, the basic user plane protocol stack is inherited from the LTE, which is also inherited from the UMTS. The well-known MAC/RLC/PDCP stack has been used over +25 years, but it is the time to think about fundamental change in user plane protocol stack.

When evolved from 4G LTE to 5G NR, the main function of RLC was shrunk and simplified to support higher data rate. More specifically, the reordering function is moved to the PDCP, and the concatenation function is moved to the MAC. The following two main functions have remained for the 5G RLC, but we doubt whether keeping these functions in the RLC is valuable for 6G.

- Error correction through ARQ;

- Segmentation and reassembly of RLC SDUs.

The ARQ was introduced in UMTS Rel-99. Later on, HARQ was introduced in HSDPA in Rel-5, and two similar functions have worked together from then on. However, the HARQ performance keeps going on, and now the ARQ becomes a third wheel. With the increased performance of HARQ in 6G, we think ARQ is not needed in most cases, and very limited use of ARQ can be incorporated into PDCP. In this situation, keeping segmentation and reassembly in RLC would not bring meaningful gain, rather it causes large overhead since the RLC should have its own sequence number and window only for segmentation and reassembly. Considering that the RLC is not the only place to have segmentation and reassembly, it would be better to merge segmentation and reassembly functions into PDCP together with ARQ from overhead reduction perspective. This can also remove duplicated functions between RLC and PDCP such as SN allocation and window management. Then, we can remove the RLC in 6G.

In addition, we think SDAP, which was introduced in 5G NR, can also be removed in 6G. The main function of SDAP is mapping between QoS flow and DRB, and this is almost the only function of SDAP. We think such a simple function can also be incorporated into PDCP. Thus, the RLC and SDAP can be removed for 6G, and the necessary functions in those layers can be integrated into PDCP as shown in Figure 2.1.1 below.

Remove RLC and SDAP, and the necessary functions are integrated into PDCP.

Fig. 2.1.1. 6G L2 protocol simplification

2.2	Separate path for control signal and user data

In 5G NR, L2 control signal (i.e. PDCP Control PDU, RLC Control PDU, MAC CE) and L2 user data are transmitted through a same logical channel or a same MAC PDU, and this causes the HOL (Head-of-Line) blocking issue between control signal and user data. On the transmitting side, PDCP Control PDUs are handled in the same DRB with PDCP Data PDUs. A PDCP status report is submitted to lower layers as the first PDCP PDU for transmission, still it is processed in the same DRB. RLC sublayer operates in a similar manner. RLC Control PDUs are prioritized over data PDUs, but they are processed in the same logical channel with the data PDUs. Therefore, urgent user data may be blocked by control PDUs belong to the same DRB or the same logical channel or vice versa. It is a kind of HOL blocking issue on a DRB or a logical channel.

At the MAC sublayer, multiple logical channels are merged into a single UL-SCH transport channel for a serving cell, and UL grant to transmit on the UL-SCH are allocated to one or more logical channels based on PBR and logical channel priorities. Therefore, a PDCP Control PDU and an RLC Control PDU may be blocked by user data belonging to another logical channel. Conversely, user data may be blocked by control PDUs belonging to anther logical channel. In addition, user data may be blocked by MAC CEs because most MAC CEs are prioritized over user data by uplink MAC scheduling, and the uplink resources are allocated to MAC CEs first. These are a kind of HOL blocking issues on the transport channel.

The above-mentioned HOL blocking issues between control signal and user data make it difficult to provide user data with QoS scheduling and to deliver control signal to the network in time.

To resolve the HOL blocking issue on L2 transmission structurally, it needs to introduce separate L2 transmission path for L2 control signal and L2 user data as shown in Fig. 2.2.1. L2 control signals such as PDCP Control PDUs and MAC CEs are transmitted using a MAC PDU which are separated from ones for user data. A HARQ process dedicated to the separate MAC PDU for control signal is also considered, and distinct handling for control signal and user data can be provided at the level of HARQ processing. To summarize, the following can be considered in 6G.

- Separate L2 transmission path for L2 control signal and L2 user data, e.g.

  - Separate HARQ process

  - Separate MAC PDU

Fig. 2.2.1. Separate transmission path for L2 control signal and user data

In addition, the separation of control signal and user data can also be considered for UL grant. If UL grants dedicated to control signal or user data are provided, this will offer a knob to schedule either urgent control signal or time-critical user data on the transport channel and the physical layer. In short, the following can be considered in 6G.

- Separate UL grant for L2 control signal and L2 user data, e.g.

  - UL grant dedicated to MAC CE

  - UL grant dedicated to user data

Design separate path for L2 control signal and user data.

2.3	Per-packet transmission mode

In 5G NR, three RLC operation modes are defined, i.e. AM (Acknowledged Mode), UM (Unacknowledged Mode), and TM (Transparent Mode). Among them, AM and UM are used for user data transmission, and TM is used for control signal transmission. These modes are inherited from UMTS and have been used for more than 25 years.

- AM is typically used for traffic requiring high reliability through retransmissions.

- UM is typically used for traffic requiring low latency without retransmissions.

When a DRB is setup, the transmission mode i.e. either AM or UM, is determined based on QoS requirements of a traffic flow. The transmission mode is maintained throughout the DRB lifetime, and the only way to change the transmission mode for the traffic flow is DRB release and addition. Once DRB is established, all the packets belonging to the DRB is transmitted with the same transmission mode.

However, as new traffics for XR and AI have emerged, we are wondering whether the current transmission mechanism, i.e., per DRB transmission mode, is appropriate to support this new traffic flow. It is identified that AI and XR-like traffic flow has a mixed characteristic within the same traffic flow. We think that this fixed per DRB transmission mode would be inefficient for such mixed‑type packet flows since required characteristic for each packet is different, as explained below.

- XR: I-frame requires high reliability while P/B-frame does not require high reliability.

- AI: inference data requires low latency, whereas training data does not require low latency.

Considering that both AI and XR are killer applications in 6G, we expect that AI and XR-like traffic would be dominant and transmission of such mixed‑type packet flows should be carefully considered and studied for better user experience.

One solution to support mixed-type packet flows is introducing per-packet transmission mode instead of per DRB transmission mode. In the per-packet transmission mode, whether the retransmission is required or not is determined per packet basis, and the need for ACK feedback is indicated to the receiver by in-band signaling. The receiver sends ACK feedback only when the packet requiring ACK feedback is received, and the transmitter retransmits the packet if the ACK feedback is not received until a timer expires. A simple example of per-packet transmission mode is shown in Figure 2.3.1.

Fig. 2.3.1. Example of per-packet transmission mode operation

With this per-packet transmission mode, there is no need to differentiate between DRB types, i.e. AM DRB or UM DRB. A single DRB can serve both lossless and lossy transmission per packet basis.

As the per-packet transmission mode is suitable for mixed-type packet flows, we think it is worthwhile to reconsider the L2 transmission mode.

Introduce per-packet transmission mode suitable for mixed type packet flow

- a. Whether to perform retransmission is determined per packet basis

- b. Single DRB supports both lossless and lossy transmission

2.4	Fine-grained L2 scheduling

In 5G NR, when the MAC entity receives an UL grant, the LCP procedure is performed among all the selected logical channels in the MAC entity. In the LCP procedure in NR, the MAC entity allocates the UL resources of the received UL grant to the selected logical channels, ensuring the PBR for each logical channel. In order to provide the information of UL data volume in the MAC entity, BSR is transmitted including the amount of available data for all logical channels belonging to each LCG. Further, for XR traffic with stringent delay requirements, Delay Status Report (DSR) and LCP enhancement are introduced to support the scheduling of delay critical data.

However, despite the DSR and LCP enhancements for XR, LCP and BSR/DSR procedure in 5G NR cannot sufficiently handle the various characteristics of XR traffics. Specifically, an XR service may include different types of QoS flows (e.g., pose/control, scene, video, audio, and other data), but the network cannot schedule the uplink transmission properly to meet the QoS requirement for each QoS flow due to following reasons:

- First, the network cannot know the amount of data or the amount of delay-critical data for the specific QoS flow since the BSR and DSR in 5G NR reports the amount of data for all logical channels in the LCG.

- Secondly, even though the network knows the amount of data for the specific LCH/LCG and allocates an UL grant to meet the QoS requirement for a specific stream, the allocated UL grant would be used to transmit the data of other traffic streams as well, since the LCP procedure is processed for all selected logical channels in the MAC entity.

Given that XR service is one of killer applications in 6G, the various characteristics of the XR traffic should be carefully considered in 6G L2 scheduling. Therefore, in order to satisfy QoS requirements of the various traffic streams in an XR service, the BSR should be reported with finer granularity than LCG to allow the UL grant scheduling based on the knowledge of the amount of data of each QoS flow or each logical channel.

Further, the 6G L2 scheduling should have more flexibility to allocate an UL grant with fine control of each stream, e.g., per-LCG UL grant or per-LCH UL grant.

Fig. 2.4.1. Example of fine-grained scheduling; per-LCG UL grant

In addition, since the buffer size index of the BSR MAC CE in 5G NR is encoded using the exponential function, large quantization error occurs for the large-sized XR data. Even though the new BS table is introduced for XR traffic, the maximum size for the additional BS table is 750KB, based on the ratio between maximum link rate (60Mbps) and minimum frame rate (15 fps). However, given that higher display resolution is expected in the future XR applications, BS table defined in 5G NR would not be sufficient to cover all the XR traffic expected in 6G.

In our understanding, defining fixed additional BS table(s) does not ensure to report all the possible amount of data of XR traffic with small quantization error, since the XR data rate will be further increased based on the display resolution and frame rate, depending on XR applications. Therefore, in order to support the reporting of the larger data volume for XR traffic with reasonable quantization error, the additional mechanism to indicate the buffer size in a BS table should be studied considering the various data rates in different XR scenarios.

Study L2 scheduling enhancements for XR traffic, e.g.,

- a. Fine-grained BSR, e.g. per-QoS flow BSR, per-LCH BSR

- b.	Fine-grained UL grant, e.g., per-LCG UL grant, per-LCH UL grant

- c. A mechanism to indicate buffer size with small quantization error

## 2.5	Fast MAC PDU generation/processing

In 5G NR, when the UE receives a UL grant from the network, the UE performs the LCP procedure to construct a MAC PDU to fit the size of the UL grant. During the LCP procedure, the UE allocates UL resources to logical channels and MAC CE based on a predefined priority. Based on the allocated UL resource for each component, the UE constructs a MAC subPDU for each component, and each MAC subPDU consists of a MAC subheader and a MAC SDU, MAC CE, or padding.

As explained above, since the UE constructs a MAC PDU after receiving the UL grant, it takes much time to allocate UL resources to each logical channel and construct a MAC PDU according to the allocated UL resource due to the various restrictions and priorities. It means that the network should schedule the UL grant considering this significant processing delay. In addition, if the MAC CE security is introduced in 6G, the processing delay would be even more deteriorated.

In 6G, one of the key requirements is HRLLC (Hyper-Reliable and Low-Latency Communication), and the HRLLC traffic may have a latency requirement up to 0.1ms. However, even in 6G, to support the fairness and prioritization among the logical channels, the LCP procedure would need to be supported. Thus, since LCP is a real-time operation with very tight processing timeline, the latency requirement for HRLLC may not be satisfied.

In order to support the HRLLC traffic in 6G, a possible way is that the UE pre-generates multiple MAC PDUs based on the expectation, and indicates the list of expected MAC PDU size to the network. When the network receives the list of expected MAC PDU size, the network selects one of the expected MAC PDU size based on the network resource and radio condition, and provides UL grant to the UE where the size of UL grant is fit to the selected MAC PDU size.

Fig. 2.5.1. Example of fast MAC PDU generation; based on UE expectation

In other way, the UE can pre-generate a MAC PDU based on the network assistance information. For example, the UE receives a UL grant containing the next UL grant size information. In this case, the UE generates a first MAC PDU transmitted on the UL grant, and the UE pre-generates a second MAC PDU based on the next UL grant size information.

Fig. 2.5.2. Example of fast MAC PDU generation; based on network assistance information

Study a mechanism to pre-generate a MAC PDU before receiving the UL grant based on the UE expectation or the network assistance information.

2.6	L2 Support for L4S

The L4S (Low Latency, Low Loss, Scalable throughput) enables real-time applications to always enjoy a low latency, no matter how busy the network is. It is expected that the L4S is widely used to serve time critical services such as XR services. For a radio network targeting to time critical services, it seems important to support L4S.

The L4S is intended to keep queuing delay consistently low while fully utilizing available capacity. To achieve the goal, the L4S sender reduces TCP congestion window proportionally to the level of congestion. The level of congestion is estimated from the fraction of bytes which encountered congestion. To indicate packets that encountered congestion, each network node on the transmission path detects congestion and sets the congestion encountered (CE) codepoint in the IP header. In addition, each node needs a separate buffer for L4S traffic to distinguish an L4S packet from a normal packet and to protect L4S traffic from being delayed due to normal traffic (e.g., file download).

Some L4S aspects are already included in 3GPP network, e.g. ECN marking, but there has been no consideration in radio protocol. The congestion and queuing delay at the radio interface between a gNB and a UE is affected not only by data traffic but also by radio environment. Therefore, it is likely that the congestion situation varies more rapidly on a radio interface than on a network interface in the network.

In 6G, it is important to handle TCP traffic efficiently to make L4S work well and provide high data rate with low latency. Then, it is worth to consider efficient mechanism to support L4S even in radio protocol.

For example, the congestion information is echoed back to the sender, using the ECN-Echo (ECE) flag in the TCP ACK packet, by the receiver. If the ECE flag is delayed, the rate control in the sender is delayed, which may lead to packet loss and it will degrade overall TCP throughput. Thus, it needs to prioritize TCP ACK packets including ECE flag as shown in Fig. 2.6.1.

Fig. 2.6.1 TCP ACK prioritization for L4S

In addition, it also needs to be considered to have a separate L2 queue for L4S traffic and non-L4S traffic. The dual-Q is essential element for L4S, because, if they are mixed together into a single queue, the L4S operates as classic TCP, and the L4S gain disappears. Thus, RFC9332 suggests to have a separate queue as shown in Fig. 2.6.2.

In the network side, e.g. in UPF-gNB interface, the separation of L4S traffic and non-L4S traffic is possible by allocating them into different QoS flows. However, in radio interface, even if they are mapped to different DRBs, they can be multiplexed into a single MAC PDU by LCP procedure, and separation of two flows cannot be achieved, which may lead to decrease of L4S gain. Therefore, it should also be considered that L4S traffic and non-L4S traffic are separated in whole L2 protocols, e.g. separate logical channel, separate MAC PDU, and/or separate HARQ process.

Fig. 2.6.2 Dual-Q AQM for L4S (RFC9332)

Lastly, Active Queue Management (AQM) may also be considered in L2 protocol. Up to 5G, L2 buffer does not have a special mechanism to handle buffer overflow other than “tail drop”. The tail drop mechanism is nothing but discarding a newly received packet if the buffer is full. Although PDCP has another mechanism to discard PDCP SDU by discard timer, it does not smartly control the L2 queue, and the “tail drop” situation would occur if large amount of data is received at the same time.

There are well-known problems in “tail drop” mechanism, e.g. bufferbloat, TCP global synchronization, and frequent RTO (Retransmission Time Out) in TCP application. To overcome such problems, sophisticated queue management mechanisms are developed, e.g. RED (Random Early Detection), CoDel (Controlled Delay) in IETF.

To efficiently support L4S in 6G, a new mechanism similar to RED or CoDel can be considered in L2 buffer. In addition, ECN marking may also be considered in L2 header to correctly reflect the radio interface situation.

Study efficient mechanisms to support L4S in radio protocol, e.g.

- a. Fast transmission of L4S feedback (TCP ACK prioritization)

- b. Separate L2 queue for L4S traffic and classic TCP traffic

- c. AQM for L2 buffer

2.7	Random Access

The 5G NR supports two types of RACH procedures: the 4-step RACH designed initially for robust access, and the 2-step RACH introduced in NR Rel-16 for low-latency and small data transmissions. These procedures use distinct PRACH resources or RACH occasions to enable separation and efficient operation. In 6G, both procedures are expected to be relevant — e.g. 4-step RACH for extended uplink coverage and 2-step RACH for fast access in small cells. Both 4-step and 2-step RACH procedures can be studied in the early phase of 6G study.

In the meantime, after adoption of 2-step RA in NR Rel-16, RA procedure becomes complicated due to selection of RA type and fallback from 2-step RA to 4-step RA. The situation becomes even worse after the early indication in the RA preamble is introduced by various features. To support early indication of various features, RA partitioning is introduced in NR Rel-17, and this makes the RA procedure even more complicated. We think such complexity in RA procedure should be avoided in 6G, because the RA procedure is the most essential function in mobile system. A neat design of RA procedure is pre-requisite of success of mobile system.

Further, since multiple features for RA partitioning framework are supported in 5G NR, the number of resource partitions increases exponentially. As a result, the RA resource congestion problem becomes worse due to reduced number of allocated preambles and RACH occasions for each partition. Even though the backoff procedure is defined to alleviate the RA resource congestion, it is not sufficient to handle the RA congestion problem with RA partitioning, since there would be the case that some of partitions are highly overloaded while the other partitions are not crowded. Therefore, further mechanisms to handle the RA congestion problem should be studied in 6G, in order to efficiently use the RA resource with the RA partitioning for different purposes. For example, the network may indicate to retransmit the Random Access preamble using different set of Random Access resources in RAR, if the network detects a congestion in the selected set of Random Access resource.

The NR Rel-19 NES introduced PRACH adaptation which will be generally beneficial for 6G network saving energy and efficient resource management. Unlike NR Rel-19 where PRACH resources configured for legacy NR UEs can coexist with PRACH resources configured for NR Rel-19 NES UEs, PRACH adaptation for 6G can be applied to PRACH resources provided in a single configuration, assuming 6G UEs can support PRACH adaptation feature from 6G Day-1. Moreover, considering resource overhead for pre-allocated PUSCH resources and collision risk in dense UE environments for 2-step RACH, adaptation of msgA PUSCH part can be taken into account in 6G. For example, MsgA PUSCH part can be deactivated in case RACH occasions are shared by both 4-step RACH and 2-step RACH

Study support of both 2-step RACH and 4-step RACH with the following considerations:

- a. Simplification of feature combinations

- b. Mitigation of RA overload situations

- c. Adaptation of PRACH and MsgA PUSCH resources

3. Conclusion

Based on the above discussions, we present the following proposals:

- Remove RLC and SDAP, and the necessary functions are integrated into PDCP.

Design separate path for L2 control signal and user data.

Introduce per-packet transmission mode suitable for mixed type packet flow

- a. Whether to perform retransmission is determined per packet basis

- b. Single DRB supports both lossless and lossy transmission

Study L2 scheduling enhancements for XR traffic, e.g.,

- a. Fine-grained BSR, e.g. per-QoS flow BSR, per-LCH BSR

- b.	Fine-grained UL grant, e.g., per-LCG UL grant, per-LCH UL grant

- c. A mechanism to indicate buffer size with small quantization error

Study a mechanism to pre-generate a MAC PDU before receiving the UL grant based on the UE expectation or the network assistance information.

Study efficient mechanisms to support L4S in radio protocol, e.g.

- a. Fast transmission of L4S feedback (TCP ACK prioritization)

- b. Separate L2 queue for L4S traffic and classic TCP traffic

- c. AQM for L2 buffer

Study support of both 2-step RACH and 4-step RACH with the following considerations:

- a. Simplification of feature combinations

- b. Mitigation of RA overload situations

- c. Adaptation of PRACH and MsgA PUSCH resources