3GPP TSG-RAN WG2 Meeting#131bis   		                                                        R2-2507240

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	9.2.2

Source:	ETRI

Title:	Discussion on Ambient IoT Topology 2

Document for:	Discussion and Decision

# Introduction

In last RAN#108 meeting, Rel-20 Ambient IoT work item was approved [1]. Accordingly, this contribution discusses some considerations for topology 2 in Rel-20 Ambient IoT.

# Discussions

Architectecture

In RAN2#127 meeting, RAN2 discussed architecture options and reached following agreement [2].

4. RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

In SA2 TR 23.700-13 v0.4.0, several Topology 2 solutions are captured, and the system architecture and end-to-end protocol stack are specified. Among these solutions, the RRC-based architecture was selected for Topology 2 by SA2 and subsequently approved in the SA plenary meeting [3]. In line with the SA plenary decision, RAN revised the SID related to the AIoT Topology 2 architecture at the last RAN plenary meeting (RP#109) [4]. Therefore, RAN2 should study the technical issues based on the RRC-based architecture.

Proposal 1. Study technical issues of Topology 2 based on RRC-based architecture.

Figure 1. RRC-based architecture for Topology 2 in TR 23.700-13 [5]

Operational procedure in Topology 2

As described in the revised SID, RAN2 plans to study DO-A traffic for Topology 2 after sufficient progress has been made on DO-A for Topology 1. Therefore, DO-DTT and DT traffic should be addressed in advance.

In RRC-based architecture, the following procedure can be considered for DO-DTT and DT traffic:

The gNB selects the appropriate UE reader as intermediate node among candidate UEs and performs configurations.

The gNB receives the service request from CN and transmits it to the UE reader via RRC message over Uu interface.

Based on service request contained in the RRC message, the UE reader performs an Access Stratum (AS) procedure such as paging, random access and data transmission with A-IoT device(s).

After AS procedure between UE reader and device, UE reader transmits A-IoT data to gNB via an RRC message.

Proposal 2. From RAN2 perspective, the following procedure can be considered for DO-DTT and DT traffic in topology 2.

- The gNB configures radio resource and operational parameters for the UE reader.

- The gNB receives the service request from CN and transmits it UE reader via an RRC message.

A UE reader performs an AS procedure with A-IoT device(s).

After AS procedure. UE reader transmits A-IoT data to gNB via an RRC message.

A gNB should be able to dynamically select the most suitable UE reader among multiple available intermediate UEs in proximity to the target AIoT device. This selection process may consider the UE’s availability, location, load, and channel conditions to the AIoT device. To achieve this, if a cell is configured as an AIoT service-capable cell with Topology 2, the gNB would provide an indication to all UEs in the cell via SIB. Based on the SIB, a UE capable of the A-IoT reader role can transmit its capabilities to the gNB during access procedures.

Proposal 3. The gNB broadcasts the system information indicating that the cell is configured as an AIoT service-capable cell with Topology 2. A UE capable of acting as an intermediate node transmits its capabilities to the gNB during access procedures.

Regarding resource configuration, there may be two different configuration mechanisms, namely dedicated resource configuration and shared resource configuration. In the RAN2#127bis meeting, RAN2 discussed this issue and reached the following agreement [6].

1. Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

According to the above agreement, it is evident that dedicated configuration is used as the resource configuration mechanism for the UE reader in Rel-19 AIoT. However, it remains unclear whether a shared resource pool can be considered as a resource configuration mechanism in Rel-20 AIoT. Therefore, RAN2 is requested to discuss this issue.

Proposal 4. Discuss whether shared resource pool can be considered as resource configuration mechanism in rel-20 A-IoT.

In RRC-based architecture, it is necessary to discuss which RRC message should be used for AIoT data transmission between the UE reader and the gNB. For this issue, two candidate solutions may be considered: (i) extending a legacy RRC message, or (ii) designing a new RRC message. Both options may have specification impacts. Therefore, RAN2 is requested to evaluate which solution is more feasible for AIoT data transmission in this architecture while minimizing specification impacts.

Proposal 5. Discuss the RRC message options for AIoT data transmission in RRC-based architecture.

# Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1. Study technical issues of Topology 2 based on RRC-based architecture.

Proposal 2. From RAN2 perspective, the following procedure can be considered for DO-DTT and DT traffic in topology 2.

1)  The gNB configures radio resource and operational parameters for the UE reader.

2)  The gNB receives the service request from CN and transmits it UE reader via an RRC message.

3)  A UE reader performs an AS procedure with A-IoT device(s).

4)  After AS procedure. UE reader transmits A-IoT data to gNB via an RRC message.

Proposal 3. The gNB broadcasts the system information indicating that the cell is configured as an AIoT service-capable cell with Topology 2. A UE capable of acting as an intermediate node transmits its capabilities to the gNB during access procedures.

Proposal 4. Discuss whether shared resource pool can be considered as resource configuration mechanism in rel-20 A-IoT.

Proposal 5. Discuss the RRC message options for AIoT data transmission in RRC-based architecture.

# References

- RP-251885, “New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2,” June, 2025.

- Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, Aug., 2024.

- SP-250834,	“NEW SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2”, June, 2025

- RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2,” Sep., 2025.

- 3GPP TR 23.700-13, “Study on Architecture support of Ambient power-enabled Internet of Things (Release 19),” March, 2025.

- Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, Oct., 2024.