3GPP TSG-RAN WG2 Meeting #131bis	R2-2507257

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on discovery and relay reslection

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the discovery response message sending condition at the intermediate Relay UE. Also, we investigate how to build up a tree-like topology and whether to include RRC state information in the discovery message.

2.	Discussion

2.1 Whether the intermediate Relay UE can send response message without forwarding solicitation message

According to the SA2 spec (TS 23.304), an intermediate Relay UE can send the response message without forwarding the received solicitation message if the intermediate Relay UE already has a PC5 link. The relevant SA2 spec is as follows:

##### 6.3.2.5.3 Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B <omitted> 3a. If an indication that multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe Intermediate UE-to-Network Relay shall drop the received Solicitation message if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit of the received Solicitation message or the (pre)configured maximum number of hops associated with the RSC. The 5G ProSe Intermediate UE-to-Network Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), without sending Solicitation message. i.e. steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate UE-to-Network Relay(s). If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to-Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.). <omitted>

RAN2 can follow the same procedure as the SA2 spec for the discovery response message delivery. So, RAN2 confirms whether the discovery response message can be generated at the intermediate Relay UE, not the last Relay UE, without forwarding the solicitation message toward the last Relay UE, if the intermediate Relay UE already has a PC5 link with its parent UE. When the intermediate Relay UE sends the response message without forwarding the solicitation message, the intermediate Relay UE has to follow the PC5 AS layer condition (e.g., PC5-RSRP) in the same way as the last Relay UE.

Proposal 1: RAN2 confirms the SA2’s understanding such as:

- For discovery model B, if an intermediate Relay UE already has an PC5 connection with a parent UE, the intermediate Relay UE can send a response message toward the Remote UE without forwarding the solicitation message to its parent UE.

Proposal 2: For the discovery model B, if the intermediate Relay UE can generate the response message without forwarding the received solicitation message, the intermediate Relay UE needs to check the PC5 AS condition (e.g., PC5-RSRP) before sending a discovery response message towards the Remote UE.

3.	Conclusion

Proposal 1: RAN2 confirms the SA2’s understanding such as:

- For the discovery model B, if an intermediate Relay UE already has an PC5 connection with a parent UE, the intermediate Relay UE can send a response message toward the Remote UE without forwarding the solicitation message to its parent UE.

Proposal 2: For the discovery model B, if the intermediate Relay UE can generate the response message without forwarding the received solicitation message, the intermediate Relay UE needs to check the PC5 AS condition (e.g., PC5-RSRP) before sending a discovery response message towards the Remote UE.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNe..