3GPP TSG-RAN WG2 Meeting #131bis	R2-2507267

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.11.3

Source:	Nokia

Title:	Other Aspects  of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1	SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. The WID was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

# 2	RRC Open issues

## 2.1	RIL C100, and C104

The CFRA procedure incorporates an RSRP check to verify that the beams assigned with dedicated PRACH preambles to the UE have sufficient signal strength. If none of these beams exceed the RSRP threshold, the UE falls back to the CBRA procedure. When selecting an SS/PBCH Block associated with a dedicated preamble, the RSRP threshold is defined by rsrp-ThresholdSSB. This information element can be provided through either BeamFailureRecoveryConfig or RACH-ConfigCommon. Similarly, when selecting a CSI Reference Signal linked to a dedicated preamble, the RSRP threshold is defined by rsrp-ThresholdCSI-RS, which can be provided through RACH-ConfigDedicated.

In the field description of ra-OccasionType-r19 within BeamFailureRecoveryConfig and RACH-ConfigDedicated, CFRA is explicitly included to indicate that the SBFD RO type can be used by SBFD-aware UEs. Since CFRA may fall back to CBRA when the threshold condition is not met, and given that RAN2 has agreed that SBFD-aware UEs use CBRA resources with the same RO type as indicated in the CFRA resource during fallback, we propose updating the RRC field description to “Indicates the second PRACH occasions of CFRA or of the fallback CBRA to be used by an SBFD-aware UE.”

Proposal 1: Update the field description of ra-OccasionType-r19  to ‘Indicates the second PRACH occasions of CFRA or of the fallback CBRA  to be used by a SBFD aware UE’

# 3	Open issues on Rel-19 Evolution of NR duplex operation (SBFD) Stage 2

## 3.1		Support of L1 CLI measurements

RAN2 CR on TS 38.300 specifications captures the newly introduced L1-based CLI measurements specified in Rel-19 for SBFD operation [1]. Current version of Section 17.2 Cross-Link Interference Management describes the L1-based UE-to-UE CLI reporting as an SBFD specific feature. However, according to the RAN1 agreement in RAN1#121, the L1-based reporting is also supported for UEs not configured with SBFD symbols, i.e., it is supported for legacy UEs:

Agreement For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

Therefore, we suggest updating the current description in Section 17.2 as follows:

Two types of UE-to-UE CLI reporting are supported in case of Sub-Band Full Duplex (SBFD) operation: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

Proposal 2: Update the Section 17.2 of TS 38.300 to capture the support of L1-based reporting for TDD and SBFD: Two types of UE-to-UE CLI reporting are supported in case of Sub-Band Full Duplex (SBFD) operation: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

# 4	Conclusion

Proposal 1: Update the field description of ra-OccasionType-r19  to ‘Indicates the second PRACH occasions of CFRA or of the fallback CBRA  to be used by a SBFD aware UE’.

Proposal 2: Update the Section 17.2 of TS 38.300 to capture the support of L1-based reporting for TDD and SBFD: Two types of UE-to-UE CLI reporting are supported in case of Sub-Band Full Duplex (SBFD) operation: L1-based reporting and L3-based reporting. A UE is not expected to be configured with both L1 CLI measurement and reporting and L3 CLI measurement and reporting simultaneously.

# 5	Reference

[1]  R2-2506604: Introduction of Rel-19 Evolution of NR duplex operation (SBFD)